IEEE P802.11
Wireless LANs

802.11 TGn FRCC Special Committee:

Point-to-Point Simulation Scenario

Date:	January 12, 2004

Author:	Bjorn A. Bjerke and Sanjiv Nanda
	Qualcomm, Inc.
	9 Damonmill Sq. Suite 2A, Concord, MA 01742, USA
	Phone: 781-276-0912
	Fax: 781-276-0901
	e-Mail: bbjerke@qualcomm.com, snanda@qualcomm.com

Abstract

This document provides the missing usage model no. 16 and corresponding simulation scenario needed for the Functional Requirements and Comparison Criteria. The proposed additions to the Usage Models document are highlighted below in red.

# Usage Models

Table 1 defines the usage models defined by this document.

The purpose of these models is to merge representative use cases to create a small number of credible worst-case mixtures of applications.  The usage models have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some subset of the use cases that are identified to be priorities and capable of implementation in proposed 802.11n technology.

The number of usage models needs to be limited because each usage model adds simulation time to the preparation of results for submission against the 802.11n comparison criteria criteria.

Table 1 - Usage Model Definitions

| Model # | Usage Model | Covers Use Cases | Status (Mandatory / Optional) | Score (ave/devn) | Application mix | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Residential | 3, 8, 10, 33, 1, 32, 11, 29, 7, 21 | Mandatory | 2.8/0.3 | 1 x AP STA 1: 19.2 Mbps HDTV (A4) , VoD control channel (A2) STA 3: 24 Mbps HDTV (A4) , VoD control channel (A2) STA 4: SDTV (A3), internet file transfer (A9), local file transfer A(15) STA 5 & 6: Video Phone (A23) STA 7,8 & 9: VoIP (A9) STA 10: Internet streaming video (7), MP3 Audio (A12), Video gaming, controller to internet (A18), local file transfer A(15) STA 11: Video gaming, controller to console (A16) | This scenario should be room to room or indoor/outdoor. The exact spatial distribution and mobility as well as the desired number of simultaneous connections can be referred to [2]. 20m range. |
| 2 | Residential IBSS | 4, 33, 11, 34, 15, 21 | Optional | 2.3/0.6 | 0 AP STA 1: peer-peer DV Audio/Video (A1) sink STA 2,3: local file transfer (A15) sink, local file transfer (A15) source STA 4,5,6,7: Video Phone (A23) sink, Video Phone (A23) source STA 8: 4 x Video gaming, controller to console (A16), sink STA 9: content download (A13) sink STA 10: peer-peer DV Audio/Video (A1) (rate, range combination > current technology) source STA 11,12,13,14: Video gaming, controller to console (A16) source STA 15: content download (A13) source | Note, all these devices are operating on the same channel. |
| 4 | Large Enterprise | 10, 31, 33, 1, 29 | Mandatory Adopted 12 Nov | 2.7/0.4 | 1 AP, cellular frequency re-use. 6 STA (1-6): internet file transfer (A14), 2 STA (7-8): Video Conferencing (A6) 2 STA (9-10): internet streaming video/audio (A7), 14 STA (11-24): local file transfer (A15) (10 downlink (STA 11-20), 4 uplink (STA 21-24)) 6 STA (25-30): VoIP (A9) | Minimum 15 – 20 meter range Aggregate throughput of the following cell should be reduced according to the CCI capacity reduction factor. See Appendix 1 steps 1 to 3. |
| 5 | Conference room | 9, 36, 1, 18, 30 | Optional | 2.4/0.5 | 1 AP 30 STA (1-30): netmeeting, desktop sharing (A19), Local File Transfer (A15) 10 STA (31-40): VoIP (A9) 1 STA: SDTV (A3) | LOS small enterprise 15m |
| 6 | Hot spot | 1, 13, 5 | Mandatory Adopted 12 Nov | 2.8/0.4 | 1 AP 20 STA: Internet file transfer (A14) 10 STA: internet streaming video/audio (A7) 2 STA: SDTV (A3) 20 STA: VoIP | Indoors: Large Enterprise channel model In hot spot area, most of traffic goes through internet, and the duration of the session is limited (less than 2 hours). |
| 9 | Mixed-mode (containing Legacy + HT STAs) BSS | 1 | Mandatory | 2.5/0.6 | 1 HT AP legacy STA 1-3 associated with the HT AP: internet file transfer (A14) 1 HT STA (STA4): SDTV (A3) 2 HT STA (STA5,6): internet file transfer (A14) Note, legacy system means 802.11a in the 5GHz bands and .11g in the 2.4GHz band. | Will apply to large enterprise 20m range |
| 11 | Co-channel legacy BSS | 26 | Mandatory | 2.6/0.6 | 1 legacy AP with 6 legacy STA. 1 HT AP and 6 STA. Co-channel with legacy AP. All STA doing internet file transfer (A14). 2 HT STA: internet streaming audio (A8). 1 HT STA: SDTV (A3) Note, the BSSs are overlapping, without any STA/AP being exactly on top of another STA/AP. | Will apply to large enterprise 20m range |
| 16 | Point-point link rate test | N/A | Mandatory (for use with FR #1 and CC #27) | N/A | 1 HT AP 1 HT STA: SDTV (A3) and HDTV (A4) (1500B MSDUs from fully backlogged AP. Offered load: 4 Mbps increasing to 250 Mbps) | Channel models: - B-NLOS - E-NLOS Channelization: 20 MHz Range: 0 – 200 m |
| 3 | (Reserved) |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |

## Scenario 16 (Point-to-Point Link Rate Test)

The channelization for this scenario is 20 MHz. Goodput vs. range is measured.

| STA Name: AP |  | Role: SDTV/HDTV Source |  |  | Location: (0, 0) |  | Channel Models: B-NLOS E-NLOS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA | 4 - 250 Mbps offered load |  | UDP | 1500 |  | 200 ms |  |  |

| STA Name: STA |  | Role: SDTV/HDTV Sink |  |  | Location: (0, X), where X varies from 0 to 200 |  | Channel Models: B-NLOS E-NLOS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| Note: this STA is a sink only, no apps generate data from this STA |  |  |  |  |  |  |  |  |