IEEE P802.11
Wireless LANs

| LB73 Azimuth draft text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-09-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., MS SJ-10-5, San Jose, CA 95134-1706 | 408-527-0815 | petere@cisco.com |

Abstract

LB73 Azimuth draft text

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 3. Definitions

Insert the following new definition in alphabetical order, renumbering as necessary:

Azimuth: The horizontal orientation of the front face of a station or of a radio beam measured clockwise from True North. The reference datum is also included.

# 7. Frame formats

## 7.3 Management frame body components

### 7.3.2 Information elements

#### 7.3.2.21 Measurement Request element

Change Table 20b, inserting Azimuth request after LCI request and renumbering as shown:

| Name | Measurement Type |
| --- | --- |
| LCI request | 10 |
| Azimuth request | 11 |
| Measurement Pause request | 1112 |

Change second paragraph, fourth sentence (after Table 20b) as shown:

Measurement Types 3 through 1011 and 255 are defined for radio measurement and shall only be included in Radio Resource Measurement Request frames.

Insert the following new subsection after 7.3.2.21.11 as shown and renumber tables accordingly:

##### 7.3.2.21.12 Azimuth request

The Measurement Request field of an Azimuth request is a single octet as defined in Table k9.

Table k9 — Azimuth Request definition

| Azimuth Request | Azimuth Measurement Subject |
| --- | --- |
| Bits 7, 6, 5 | Reserved |
| Bit 4 = 0 | Azimuth of front face of STA |
| Bit 4 = 1 | Azimuth of radio beam |
| Bits 3-0 | Azimuth Accuracy |

‘Azimuth of front face of STA’ refers to the Azimuth of the front face of the station, and ‘Azimuth of radio beam’ refers to the Azimuth of radio reception or transmission.

NOTE— A geographic feature is "an abstraction of a real world phenomenon; it is a geographic feature

if it is associated with a location relative to the Earth”. The designation of a horizontal plane is relative to the Earth. The designation of the ‘front face’ of a station is arbitrary, but refers to an orientable surface (possessing a centerline) of the station. Non-orientable shapes like the Mobius band and Klein bottle are non-intuitive for the purpose of making an Azimuth report.

Azimuth accuracy is the number of valid bits requested in the fixed-point value of Azimuth. Values above nine are undefined and reserved.

#### 7.3.2.22 Measurement Report element

Change Table 20c, inserting Azimuth report after LCI report and renumber accordingly as shown:

| Name | Measurement Type |
| --- | --- |
| LCI report | 10 |
| Azimuth report | 11 |
| Reserved | 1112 |

Insert the following new subsection (7.3.2.22.12) after subsection 7.3.2.22.11, adjusting the subsection numbers, figure numbers and table numbers as necessary:

7.3.2.22.12 Azimuth Report element

An Azimuth report includes Azimuth type, accuracy and datum information, and the length shall be three octets.

The Azimuth format is shown in Figure k28a.

Figure k28a—Azimuth Report field format

Azimuth type is one bit encoding the type of azimuth:  Values defined are:

0: Azimuth of front face of STA

1: Azimuth of radio beam

Azimuth resolution is 4 bits, indicating the number of valid bits in the Azimuth. An Azimuth resolution set to zero shall indicate that the Azimuth cannot be reported with the requested accuracy.

Azimuth is a 9-bit unsigned integer value in degrees from True North, defined by the Azimuth type field.

The Datum value shall be 1 (WGS 84), unless another datum is required for operation in the regulatory domain. The datum is based on that described in IETF RFC 3825, “Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information.”

# 11 MLME

## 11.11 Radio measurement procedures

### 11.11.8 Specific measurement usage

Insert the following new subsection (11.11.8.9) after subsection 11.11.8.8 and renumber accordingly:

#### 11.11.8.9 Azimuth report

A STA receiving an Azimuth request shall respond with a Radio Measurement Report frame including one Azimuth element (Azimuth report).

If the STA receiving an azimuth request lacks the means to report the requested azimuth type to the requested accuracy, then the Azimuth report shall have the Azimuth resolution set to zero. If the STA has no information about Azimuth from True North, then it shall set the Incapable bit in the Measurement Report Mode field. The method by which the azimuth information is generated is not specified, to allow the accuracy of the reported azimuth to be best effort.

NOTE—The value of the azimuth of the front face of the STA or of a directional radio antenna may be set by administrative means.

# Annex A - PICS

## A.4 PICS proforma-IEEE Std 802.11, 1999 Edition9

Insert new Items in Section A.4.13 after the RRM11 items, adjusting the subsequent table numbers as necessary:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM12 | Azimuth Measurement Type | 11.11, 11.11.8.9 | CFk:M | Yes □ No □ N/A □ |
| RRM12.1 | Azimuth Request | 7.3.2.21.12 | CFk:M | Yes □ No □ N/A □ |
| RRM12.2 | Azimuth Response | 7.3.2.22.12 | CFk:M | Yes □ No □ N/A □ |

# Annex D

In SMT MIB of Annex D, in Dot11RRMRequestEntry SEQUENCE, insert new Item after dot11RRMRqstLCIRqstOctet:

Dot11RRMRqstAzimuthRqstOctet INTEGER,

In SMT MIB of Annex D, in dot11RRMRequest TABLE, insert new Item after dot11RRMRqstLCIRqstOctet and renumber accordingly :

dot11RRMRqstAzimuthRqstOctet OBJECT-TYPE

SYNTAX INTEGER {

front face of STA(0),

radio beam(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute corresponds to the subject of the Azimuth measurement request."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 30 }

In SMT MIB of Annex D, insert new Item after dot11LCIReport TABLE and renumber accordingly :

-- ********************************************************************

-- * dot11AzimuthReport TABLE

-- ********************************************************************

dot11AzimuthReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11AzimuthReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Azimuth reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 9 }

dot11AzimuthReportEntry OBJECT-TYPE

SYNTAX Dot11AzimuthReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11AzimuthReportTable

Indexed by dot11AzimuthReportIndex."

INDEX { dot11AzimuthReportIndex }

::= { dot11AzimuthReportTable 1 }

Dot11AzimuthReportEntry ::=

SEQUENCE {

dot11AzimuthReportIndex Unsigned32,

dot11AzimuthReportToken OCTET STRING,

dot11AzimuthIfIndex InterfaceIndex,

dot11AzimuthSTAAddress MacAddress,

dot11AzimuthType INTEGER,

dot11AzimuthResolution INTEGER,

dot11Azimuth Integer32,

dot11AzimuthDatum INTEGER }

dot11AzimuthReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Azimuth Report elements in dot11AzimuthReportTable,

greater than 0."

::= { dot11AzimuthReportEntry 1 }

dot11AzimuthReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the token that was specified in the measurement

request that generated this measurement report. This should be an exact

match to the original dot11RRMRqstToken attribute. Note that there may be

multiple entries in the table that match this value since a single request

may generate multiple measurement reports."

::= { dot11AzimuthReportEntry 2 }

dot11AzimuthIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of Azimuth Report has been

received on"

::= { dot11AzimuthReportEntry 3 }

dot11AzimuthSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this Azimuth report"

::= { dot11AzimuthReportEntry 4 }

dot11AzimuthType OBJECT-TYPE

SYNTAX INTEGER {

front face of STA(0),

radio beam(1) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Azimuth Type is one bit encoding the type of azimuth.

Codes defined are:

front face of STA : in 2s-complement fixed-point 9-bit integer

radio beam : in 2s-complement fixed-point 9-bit integer"

::= { dot11AzimuthReportEntry 5 }

dot11AzimuthResolution OBJECT-TYPE

SYNTAX INTEGER (0..15)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Azimuth resolution is 4 bits indicating the number of valid

bits in the azimuth."

::= { dot11AzimuthReportEntry 6 }

dot11AzimuthInteger OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Azimuth is a 9 bit value defined by the Azimuth type field.

The field is encoded as a 2s-complement fixed-point 9-bit integer

horizontal angle in degrees from True North."

::= { dot11AzimuthReportEntry 7 }

dot11AzimuthDatum OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Datum is an eight-bit value encoding the True North

reference used for the angle given in this Azimuth."

::= { dot11AzimuthReportEntry 8 }

-- ********************************************************************

-- * End of dot11AzimuthReport TABLE

-- ********************************************************************