IEEE P802.11
Wireless LANs

| LB78 Clause 11.1.3.2.1 Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-NOV-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Floyd Simpson | Motorola, Inc. | 8000 W. Sunrise Blvd. Plantation, FL 33467 | 954-723-5269 | Floyd.Simpson@Motorola.com |

Abstract

This document contains normative text changes for letter ballot 78 comments for clause 11.1.3.2.1.  This document resolves comments: 775,778,1558,1559

The submission is referenced to 11k draft D3.0 and replaces all editor instructions as well as actual base draft modification.  This text should replace all draft D3.0 text from page 58 line 12 to page 59 lines 11.

All new text relative to the base draft is identified in blue color and underlined and all deleted text is identified by strikethrough.

Change clause 11.1.3.2.1 as follows:

##### 11.1.3.2.1 Sending a probe response

Change the first two paragraph as shown below:

STAs, subject to criteria below, receiving Probe Request frames shall respond with a probe response only if the SSID in the probe request is the broadcast SSID or matches the specific SSID of the STA. If the DS Parameter Set information element is present in the probe request, a STA where dot11RadioMeasurementEnabled is true shall respond only if the channel number from the DS Parameter Set element matches the channel in use by the STA. If the DS Parameter Set information element is present in the probe request, a STA where dot11RadioMeasurementEnabled is false may respond only if the channel number from the DS Parameter Set element matches the channel in use by the STA. Probe Response frames shall be sent as directed frames to the address of the STA that generated the probe request. The probe response shall be sent using normal frame transmission rules. An AP shall respond to all probe requests meeting the above criteria. In an IBSS, the STA that generated the last beacon shall be the STA that responds to a probe request.

In each BSS there shall be at least one STA that is awake at any given time to receive and respond to probe requests. A STA that sent a beacon shall remain in the Awake state and shall respond to probe requests, subject to criteria in the next paragraph, until a Beacon frame with the current BSS ID is received. If the STA is an AP, it shall always remain in the Awake state and always respond to probe requests, subject to criteria in the next paragraph. There may be more than one STA in an IBSS that responds to any given probe request, particularly in cases where more than one STA transmitted a Beacon frame following the most recent TBTT, either due to not receiving successfully a previous beacon or due to collisions between beacon transmissions.

STAs receiving Probe Request frames shall respond with a probe response only if the SSID in the probe request is the broadcast SSID or matches the specific SSID of the STA. Furthermore, when the DS Parameter Set information element is present in the probe request and if dot11RadioMeasurementEnabled is true at the STA receiving the probe request, it shall respond only if the Current Channel field from the DS Parameter Set element matches the serving channel of the STA receiving the probe request.  Probe Response frames shall be sent as directed frames to the address of the STA that generated the probe request. The probe response shall be sent using normal frame transmission rules. An AP shall respond to all probe requests meeting the above criteria. In an IBSS, the STA that generated the last beacon shall be the STA that responds to a probe request.