IEEE P802.11
Wireless LANs

| LB78 Sections 7.13 Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-JAN-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111NE 25th Ave Hillsboro, OR 97124 | 503-264-0637 | ganesh.venkatesan@intel.com |

Abstract

This document contains normative text changes for letter ballot 78 comments for sections 7.13.  This document resolves comments: 238, 957, 1092, 1104 and 1105.

The submission is referenced to draft D3.0 and replaces all editor instructions as well as actual base draft modification.  This text should replace all draft D3.0 text from page 72 lines 10 through 18.

All new text relative to the base draft is identified in blue color and underlined and all deleted text is identified by strikethrough.

11.13 Link Measurement

A STA may use a Link Measurement Request frame to request another STA to respond with a Link

Measurement Report frame containing link margin and transmit power information a TPC Report element. A STA receiving a Link Measurement Request frame shall respond with a Link Measurement Report frame containing the

power used to transmit the response and the corresponding link margin, using a TPC Report element the Transmit Power field and the estimated link margin in the Link Margin field.

An AP in a BSS or a STA in an IBSS shall include a TPC Report element with the Link Margin field set

equal to zero and containing transmit power information in the Transmit Power field in any Beacon or

Probe Response it transmits.