IEEE P802.11
Wireless LANs

| Use of Authentication frames to initiate over-the-DS Fast BSS Transitions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-02-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

Comment #105 requested that we use Authentication frames exclusively to perform Fast BSS Transitions, rather than the D1.0 technique of using Authentication frames for over-the-air and Action frames for over-the-DS.

This submission discusses the changes needed in D1.0 to make this change.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Overview of current situation

Current (D1.0) handling at the MLME of an Action frame sent over-the-DS to the target AP.

Handling at the MLME of an Authentication frame sent over-the-air to a target AP.

Use of Authentication frames for both over-the-air and over-the-DS

Alternative #1 of handling at the MLME of an Authentication frame sent over-the-DS to the target AP.

Aspects of 802.11 Authentication frame handling that need to be fixed:

- Handling of the sending of management frames of type Authentication in the STA MAC needs to be made conditional on Authentication algorithm and on presence of the IE giving the Target AP. (Clause 8.2.2.1 seems OK, but 11.3.2 isn’t)

- Handling of the receiving of management frames of type Authentication in the AP MAC needs to be made conditional on Authentication algorithm and on presence of the IE giving the Target AP. (Clause 8.2.2.3.5)

- Handling in the MLME of management frames of type Authenticaiton needs to be made conditional on presence of an IE giving the Target AP, so that the MAC generates a MLME-Remote-Request instead of MLME-Authenticate. (Clause 10.3.4.3.3)

- A way needs to be defined for the RRB/SME at the Current AP to send an Authentication Response frame back to the STA.

- Handling of the receiving of management frames of type Authentication in the STA MAC needs to be made conditional on Authentication algorithm (not just on Transaction sequence number (e.g. clause 8.2.2.3.5)) and on presence of the IE giving the Target AP.

- SME procedures invoked on receipt of Authentication frames need to be made conditional on the Target AP  (clause 11.3.2)

Alternative #2 of handling at the MLME of an Authentication frame sent over-the-DS to the target AP.

Aspects of 802.11 Authentication frame handling that need to be fixed:

- Handling of the sending of management frames of type Authentication in the STA MAC needs to be made conditional on Authentication algorithm and on presence of the IE giving the Target AP. (Clause 8.2.2.1 seems OK, but 11.3.2 isn’t)

- Handling of the receiving of management frames of type Authentication in the AP MAC needs to be made conditional on Authentication algorithm and on presence of the IE giving the Target AP. (Clause 8.2.2.3.5)

- SME procedures invoked on MLME-Authenticate.indication need to be made conditional on the presence of the IE giving the Target AP. (Clause 11.3.2)

- Handling of the sending of management frames of type Authentication in the AP MAC needs to be made conditional on Authentication algorithm and on presence of the IE giving the Target AP. (Clause 8.2.2.1 seems OK, but 11.3.2 isn’t)

- Handling of the receiving of management frames of type Authentication in the STA MAC needs to be made conditional on Authentication algorithm (not just on Transaction sequence number (e.g. clause 8.2.2.3.5)) and on presence of the IE giving the Target AP.

- SME procedures invoked on receipt of Authentication frames need to be made conditional on the Target AP  (clause 11.3.2)