IEEE P802.11
Wireless LANs

| Liaison Response – Review of IETF PANA Framework Document |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-04-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains the IEEE 802.11 Working Group review comments on the IETF PANA framework document, draft-ietf-pana-framework-06.txt.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- From: 	Stuart J. Kerry (stuart.kerry@philips.com ), Chair IEEE 802.11 Working Group

To: Mark Townsley (townsley@cisco.com ), Jari Arrko (jari.arkko@piuha.net ) IETF Internet Area Directors

cc: Bernard Aboba (aboba@internaut.com ), Brian Carpenter (brc@zurich.ibm.com )

- Title: IEEE 802.11 Review Comments on IETF PANA Framework Document

Purpose: To provide comments from IEEE 802.11 Working Group on the PANA Framework document, draft-ietf-pana-framework-06.txt

Dear Mark Townsley,

Thank you for the request and opportunity to review draft-ietf-pana-framework-06.txt. The IEEE 802.11 Working Group has reviewed draft-ietf-pana-framework-06.txt and has the following comments:

- The description of PANA operation in section 10.2.1, “PANA with Bootstrapping IPsec”, assumes that an IEEE 802.11 link layer association is required, and that no link layer security mechanisms (TKIP, CCMP) are used. Based on this assumption, the description of PANA operation with IEEE 802.11 systems seems correct.

- The description of PANA operation in section 10.2.2, “PANA with Bootstrapping WPA/IEEE 802.11i” describes a mechanism in which an IEEE 802.11 link layer association is required, IEEE 802.11 Pre-shared Key (PSK) security is used, and the PSK is generated as a result of PANA EAP authentication. There are several issues with this proposed mechanism.

  - In an IEEE 802.11 RSN, only EAPOL data frames pass through the uncontrolled IEEE 802.1X port. General data packets, such as ARP, DHCP and PANA protocol packets are dropped at the uncontrolled port, and blocked at the controlled port until successful completion of IEEE 802.1X encapsulated EAP authentication and successful completion of the 4-Way Handshake.  The PANA protocol operation described will not work with deployed IEEE 802.11 RSN compliant systems, as all PANA traffic will be silently discarded.

>

  - In IEEE 802.11 systems, data frames are sent only in State 3, not in any state, as noted in Section 10.2.2, paragraph 5. See IEEE Std. 802.11-1999, Clause 5.5.

  - The IEEE 802.11 RSN framework is extensible, allowing for multiple Authentication and Key Management (AKM) mechanisms to be defined. It seems that a new AKM, in which select data frames such as ARP and DHCP are allowed on the uncontrolled port, would be required to support PANA authentication as described in section 10.2.2. Changes are required to the IEEE 802.11 specification to support the “PANA with Bootstrapping WPA/IEEE 802.11i” solution that is described, contrary to the statement in paragraph 4 of section 10.2.2.

  - PANA operation seems limited to generating a PSK, “Upon successful authentication, the PaC obtains a separate PSK for each AP controlled by the PAA …” In most IEEE 802.11 WLAN deployments, PSKs are shared among the clients associated to an AP.  The IEEE 802.11 standard does not require use of shared PSKs; however this is the typical PSK implementation/deployment.  IEEE 802.1X/EAP is used in practice to generate per-station keys. Since EAP authentication is supported by PANA, PANA should make use of per-station unique keys, and the additional security that per-station keys provide.

  - An alternative using a “vitual open-access AP” is described in section 10.2.2, in which “upon successful PANA authentication over this AP, the PaC will switch over to another virtual AP to utilize the PANA-derived PSK”. In IEEE 802.11 RSN operaton, the PMK that is derived is part of a PMK Security Association (PMKSA) which includes specific authorizations, and the authorizations used with one AP are not allowed to be used with another AP. In this case, the PANA protocol appears to be delivering the PSKs to use with other APs, providing a configuration function, and configuring the client with the PSKs, rather than establishing a security context that is re-used. This should be clarified in the document.

- Section 10.2.3, “Capability Discovery” describes how a PaC discovers the RSN capabilities of an IEEE 802.11 WLAN. Today, there is no way to indicate the presence of the “PSK mode bootstrapped from PANA” capability in the RSN Information Element.

For your reference, ANSI/IEEE Std. 802.11-1999, as amended by IEEE Std. 802.11a, IEEE Std. 802.11b, IEEE Std. 802.11b-COR1, IEEE Std. 802.11d, IEEE Std. 802.11g-2003, IEEE Std. 802.11h-2003, IEEE Std. 802.11i-2004, IEEE Std. 802.11j-2004 is the current version of the IEEE 802.11 Standard.

Please contact Stuart J. Kerry, IEEE 802.11 Working Group chair, Dorothy Stanley IETF Liaison with any questions. We are happy to discuss these comments with you.

Best Regards,

- Stuart J. Kerry

Contact information:

Stuart J. Kerry

stuart.kerry@philips.com

+1 408 474 7356

Dorothy Stanley

dstanley@arubanetworks.com

+1 630 363-1389

Motion:

Recommend that the IEEE 802.11 Working Group

Adopt the comments in document 06/577r0 as the IEEE 802.11 Working Group Comments on draft-ietf-pana-framework-06.txt.

References: