IEEE P802.11
Wireless LANs

| January 2009 Interpretation Response |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 USA | +1 925 474 2273 | msg@trapezenetworks.com |

Abstract

This document is a response to the interpretation request made by Christian Skupin on December 9, 2008.

Designation Number: 16

Document: IEEE Std. 802.11-2007

Classification: Unambiguous

Relevant Clauses: 7.1.3.7 and Annex G

Relevant Tables: G.1 and G.14

Topic: FCS sample calculation

Interpretation Request #16:

During the evaluation process of my 802.11 PHY/MAC implementation I got in doubt with the accuracy of the given FCS (bytes 97-100) in Table G.1. This FCS in Table G.1 corresponds to bits 784-815 in Table G.14.

With respect to the FCS calculation instructions in chapter 7.1.3.7 and the given message (bytes 1-96) in Table G.1 my calculations result in the FCS {1110 0110 1100 1100 1000 0100 0110 1101} (msb-left), whereas the given FCS is {0101 1011 1110 1010 1001 1001 1011 0111} (bits 784-815 in Table G.14).

In consideration of the fact that my FCS-implementation works with other reference data which I collected by recording 802.11a signals, I assume that the given FCS in Table G.1 is not appropriate.

Interpretation Response #16

IEEE Std. 802.11-2007 contains an error in the FCS calculation of the sample encoding in Annex G.  This issue will be forwarded to the 802.11 Working Group for consideration in the next revision of the P802.11 standard.