IEEE P802.11
Wireless LANs

| 802.11 Some LB199 proposed resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This document contains some proposed resolutions to comments from LB199.

R1:  updated during TGmc meeting.

R2: updated during 2013-11-12 TGmc meeting

R3: resolutions for CID 2114 and 2190 added

R4: remaining editorials added

R5: discussion and updated resolutions to “deprecation” comments.

R6: updated some resolutions. Added two comments assigned to the author. Removed comments assigned to others. Comments ready to discuss: 2000, 2213, 2131, 2145, 2115, 2190, 2436, 2015.

R7: updated during 2013-11-14 TGm meeting

R8: some comments removed (assigned to Brian). Additional comment resolutions written.  Comments ready to discuss: 2329, 2334, 2330, 2335, 2197, 2015, 2045, 2028, 2447, 2448, 2030, 2446, 2469, 2032, 2033, 2034, 2338, 2339, 2035, 2036, 2037, 2041, 2042, 2044, 2046, 2095.

R9: updated some comment resolutions.

R10: Started working on Clause 10 resolutions.  Discussed and updated some of the non-clause-10 resolutions during the 2013-12-06 TGmc telecon.

R11: Following the 2013-12-13 TGmc telecon.

R12: Resolution for CID 2172 updated and approved by straw poll.

R13: Actions from TGm telecons.

R14: Discussed during 2014-01-20 TGmc session.

R15: Reorganized to move completed comments to end.  Pending comments: 2189, 2099 (Gen); 2125, 2126, 2143 (MAC).   Resolution of CID 2189 re-written.

# Unresolved Comments

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2189 |  |  |  | Many of the definitions are inherently specific to 802.11. For example, it is unlikely that some other standard will ever want to re-use the term FT 4-Way handshake (12.18). | Review all definitions claimed to be non-specific. Determine criteria for "non-specific" and then apply these criteria to retaining or moving definitions to 3.2. |  | GEN |

Discussion:

Definitions in 3.1 are merged into an IEEE “dictionary of standards” – document 100.   This is now available online for subscription.  Some time after a standard is released, IEEE staff and volunteers update this dictionary based on definitions in that standard.

The purpose of this dictionary is to support re-use of terms between different standards, and thereby encourage the industry to adopt uniform set of terms.

If the section 3.1 definitions contain definitions that are specific to 802.11, then the only people interested in reading them will already have access to the definition (in the standard).  Furthermore, this creates needless work for all concerned.

In moving definitions to 3.2, we have to ensure that no remaining 3.1 definitions are dependent on 3.2.  Otherwise the IEEE dictionary of standards will have “dangling references”.

R15 – based on the discussion leading to R14, A semi-automated dependency check has been performed between definitions in 3.1 and 3.2.  Definitions marked as “specific” are those identified in R14 by comments and yellow highlights.   Definitions were then marked as “dependent” that are dependent on either definitions in 3.2, definitions marked as “specific”, or definitions that are marked “dependent”.  The the “specific” marking was removed from comments that were marked dependent.

Dependency matching was based on matching the full text of the term against the candidate term and definition, but certain terms used both in specific generic contexts were excluded (e.g., “link”, “user”). The depth of the chain of definitions to one that is specific or in 3.2 was recorded and is shown below.

This leaves a list of definitions to move.    The changes to definitions identified in R14 also need to be made,  as the matching was based on the text of the definitions after these changes were applied.

Proposed Resolution:

Revised.  Make changes as shown in <this-document> under CID2189.

Changes:

Move the following definitions from 3.1 to 3.2:

| term | isSpecific | isDependent | dependentOn | depth |
| --- | --- | --- | --- | --- |
| access category (AC) | False | True | enhanced distributed channel access (EDCA) | 3 |
| access period | False | True | directional multi-gigabit (DMG) | 1 |
| access point (AP) path | False | True | tunneled direct-link setup (TDLS) | 2 |
| active mode | False | True | mesh power mode | 1 |
| aggregated schedule | False | True | service period (SP) | 2 |
| authentication and key management (AKM) suite | True | False |  | 0 |
| average noise power indicator (ANPI) | True | False |  | 0 |
| base channel | False | True | tunneled direct-link setup (TDLS) | 2 |
| basic service set (BSS) max idle period | True | False |  | 0 |
| beamformee | False | True | beamforming steering matrix | 1 |
| beamformer | False | True | beamforming steering matrix | 1 |
| contention based access period (CBAP) | False | True | directional multi-gigabit (DMG) | 1 |
| contention free (CF) pollable | True | False |  | 0 |
| contention free period (CFP) | False | True | point coordinator (PC) | 1 |
| contention period (CP) | False | True | point coordinator (PC) | 1 |
| controlled access phase (CAP) | False | True | point coordination function (PCF) | 1 |
| deep sleep mode | False | True | mesh power mode | 1 |
| delivery-enabled access category (AC) | False | True | scheduled service period (SP) | 3 |
| directional multi-gigabit (DMG) | True | False |  | 0 |
| downlink multi-user multiple input, multiple output (DL-MU-MIMO) | False | True | downlink | 1 |
| dynamic frequency selection (DFS) owner | False | True | off-channel | 4 |
| enhanced distributed channel access (EDCA) | False | True | hybrid coordination function (HCF) | 2 |
| enhanced distributed channel access function (EDCAF) | False | True | enhanced distributed channel access (EDCA) | 3 |
| fast basic service set (BSS) transition (FT) 4-Way Handshake | False | True | 4-Way Handshake | 1 |
| fast basic service set (BSS) transition (FT) initial mobility domain association | True | False |  | 0 |
| flexible multicast service (FMS) | False | True | delivery traffic indication map (DTIM) interval | 1 |
| flexible multicast service (FMS) stream | False | True | flexible multicast stream identifier (FMSID) | 1 |
| flexible multicast stream identifier (FMSID) | True | False |  | 0 |
| group addressed quality-of-service management frame (GQMF) | False | True | quality-of-service management frame (QMF) | 1 |
| Group Key Handshake | True | False |  | 0 |
| hybrid coordination function (HCF) | False | True | point coordination function (PCF) | 1 |
| hybrid coordination function (HCF) controlled channel access (HCCA) | False | True | downlink | 1 |
| hybrid coordinator (HC) | False | True | contention free period (CFP) | 2 |
| individually addressed quality-of-service management frame (IQMF) | False | True | quality-of-service management frame (QMF) | 1 |
| light sleep mode | False | True | mesh power mode | 1 |
| link | True | False |  | 0 |
| location configuration information (LCI) | True | False |  | 0 |
| mesh peer service period (MPSP) | False | True | deep sleep mode | 2 |
| mesh peer service period (MPSP) owner | False | True | mesh peer service period (MPSP) | 3 |
| mesh peering management | True | False |  | 0 |
| mesh power mode | True | False |  | 0 |
| mesh power mode tracking | False | True | mesh power mode | 1 |
| multiple BSSID capability | True | False |  | 0 |
| multi-user (MU) physical layer (PHY) protocol data unit (PPDU) | False | True | downlink | 1 |
| nonpeer mesh power mode | False | True | mesh power mode | 1 |
| non-personal basic service set control point (non-PCP) station (STA) | True | False |  | 0 |
| non-quality-of-service management frame (non-QMF) access point (AP) | False | True | quality-of-service management frame (QMF) | 1 |
| non-quality-of-service management frame (non-QMF) station (STA) | False | True | quality-of-service management frame (QMF) | 1 |
| nontransmitted BSSID | False | True | multiple BSSID capability | 1 |
| null data packet (NDP) | False | True | physical layer (PHY) protocol data unit (PPDU) | 1 |
| off-channel | False | True | base channel | 3 |
| pairwise master key (PMK) | True | False |  | 0 |
| pairwise master key (PMK) R0 key holder identifier (R0KH-ID) | False | True | pairwise master key (PMK) | 1 |
| pairwise master key (PMK) R0 name (PMKR0Name) | False | True | pairwise master key (PMK) | 1 |
| pairwise master key (PMK) R1 key holder identifier (R1KH-ID) | False | True | pairwise master key (PMK) | 1 |
| pairwise master key (PMK) R1 name (PMKR1Name) | False | True | pairwise master key (PMK) | 1 |
| pairwise master key (PMK) S0 key holder identifier (S0KH-ID) | False | True | pairwise master key (PMK) | 1 |
| pairwise master key (PMK) S1 key holder identifier (S1KH-ID) | False | True | pairwise master key (PMK) | 1 |
| pairwise master key R0 (PMK-R0) | True | False |  | 0 |
| pairwise master key R1 (PMK-R1) | True | False |  | 0 |
| pairwise transient key (PTK) | False | True | pairwise master key (PMK) | 1 |
| pairwise transient key (PTK) name (PTKName) | False | True | pairwise transient key (PTK) | 2 |
| parameterized quality of service (QoS) | False | True | hybrid coordination function (HCF) | 2 |
| peer trigger frame | True | False |  | 0 |
| PeerKey Handshake | False | True | station-to-station link (STSL) | 1 |
| peer-specific mesh power mode | False | True | mesh power mode | 1 |
| personal basic service set (PBSS) | False | True | directional multi-gigabit (DMG) | 1 |
| personal basic service set (PBSS) control point (PCP) | False | True | personal basic service set (PBSS) | 2 |
| point coordination function (PCF) | True | False |  | 0 |
| point coordinator (PC) | True | False |  | 0 |
| protection mechanism frame | True | False |  | 0 |
| quality-of-service management frame (QMF) | True | False |  | 0 |
| quality-of-service management frame (QMF) access point (AP) | False | True | quality-of-service management frame (QMF) | 1 |
| quality-of-service management frame (QMF) policy | False | True | quality-of-service management frame (QMF) | 1 |
| quality-of-service management frame (QMF) service | False | True | quality-of-service management frame (QMF) | 1 |
| quality-of-service management frame (QMF) station (STA) | False | True | quality-of-service management frame (QMF) | 1 |
| received signal to noise indicator (RSNI) | True | False |  | 0 |
| resource information container (RIC) | True | False |  | 0 |
| robust Management frame | True | False |  | 0 |
| scheduled service period (SP) | False | True | personal basic service set (PBSS) | 2 |
| sector | True | False |  | 0 |
| service interval (SI) | True | False |  | 0 |
| service period (SP) | False | True | downlink | 1 |
| single-user (SU) physical layer (PHY) protocol data unit (PPDU) | False | True | physical layer (PHY) protocol data unit (PPDU) | 1 |
| sounding | False | True | physical layer (PHY) protocol data unit (PPDU) | 1 |
| spatial sharing (SPSH) | False | True | directional transmission | 1 |
| temporal encryption key | False | True | pairwise transient key (PTK) | 2 |
| temporal key (TK) | False | True | temporal encryption key | 3 |
| temporal message integrity code (MIC) key | True | False |  | 0 |
| traffic indication map (TIM) broadcast | True | False |  | 0 |
| traffic stream identifier (TSID) | False | True | parameterized quality of service (QoS) | 3 |
| transmission opportunity (TXOP) holder | False | True | hybrid coordinator (HC) | 3 |
| transmission opportunity (TXOP) responder | True | False |  | 0 |
| transmitted basic service set identifier (BSSID) | False | True | multiple BSSID capability | 1 |
| tunneled direct-link setup (TDLS) | True | False | link | 1 |
| tunneled direct-link setup (TDLS) direct link | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) initiator station (STA) | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) peer power save mode (PSM) | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) peer power save mode (PSM) initiator | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) peer power save mode (PSM) responder | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) peer station (STA) | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) peer unscheduled automatic power save delivery (U-APSD) [TDLS peer U-APSD (TPU)] | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) peer unscheduled automatic power save delivery (U-APSD) [TDLS peer U-APSD (TPU)] buffer station (STA) | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) peer unscheduled automatic power save delivery (U-APSD) [TDLS peer U-APSD (TPU)] sleep station (STA) | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) power save mode (PSM) | False | True | tunneled direct-link setup (TDLS) | 2 |
| tunneled direct-link setup (TDLS) responder station (STA) | False | True | tunneled direct-link setup (TDLS) | 2 |
| unscheduled service period (SP) | False | True | scheduled service period (SP) | 3 |

Change the following definitions in 3.1:

| (11ad) - The following definition contains too many 802.11-specific terms. coordination function: The logical function that determines when a station (STA) operating within a basic service set (BSS) is permitted to transmit protocol data units (PDUs) via the wireless medium (WM). NOTE-The coordination function within a BSS might have one hybrid coordination function (HCF), or it might have one HCF and one point coordination function (PCF) and has one distributed coordination function (DCF). A quality-of-service (QoS) BSS has one DCF and one HCF. In addition, a directional multi-gigabit (DMG) STA has a DMG channel access function that includes the beacon transmission interval (BTI), the association beamforming training (A-BFT), the announcement transmission interval (ATI), and the service period channel access (SPCA).(11ad) distribution system service (DSS): The set of services provided by the distribution system (DS) that enable the medium access control (MAC) to transport MAC service data units (MSDUs) between stations (STAs) that are not in direct communication with each other over a single instance of the wireless medium (WM). NOTE--These services include transport of MSDUs between the access points (APs) of basic service sets (BSSs) within an extended service set (ESS), transport of MSDUs between portals and BSSs within an ESS, transport of MSDUs between mesh gates in the same or different mesh basic service sets (MBSSs), transport of MSDUs between mesh gates and APs, transport of MSDUs between mesh gates and portals, and transport of MSDUs between STAs in the same BSS in cases where the MSDU has a group destination address or where the destination is an individual address and the STA is associated with an AP. group addressed: When applied to a medium access control (MAC) service data unit (MSDU), it is an MSDU with a group address as the destination address (DA). NOTE-When applied to a MAC protocol data unit (MPDU), it is an MPDU with a group address in the Address 1 field. Syn: multicast. individually addressed: When applied to a medium access control (MAC) service data unit (MSDU), it is an MSDU with an individual address as the destination address (DA). NOTE--When applied to a MAC protocol data unit (MPDU), it is an MPDU with an individual address in the Address 1 field. Syn: directed, unicast. mesh profile: A set of values of parameters that identifies the attributes of the mesh basic service set (MBSS) and that is used in a single (#1180)MBSS. NOTE-The mesh profile consists of the identifiers that are the values for the parameters: mesh ID, active path selection protocol, active path selection metric, congestion control mode, synchronization method, and authentication protocol. operating channel: The operating channel is the channel in which beacons are transmited. NOTE—In IEEE Std 802.11, the the serving AP ofan infrastructure BSS or the DFS owner of an IBSS transmits beacons. piggyback: The overloading of (#100)data with an acknowledgment of a previously received frame to the station (STA) to which the data is directed. prioritized quality of service (QoS): The provisioning of service in which the medium access control (MAC) protocol data units (MPDUs) with higher priority are given a preferential treatment over MPDUs with a lower priority. NOTE--Prioritized QoS is provided through the enhanced distributed channel access (EDCA) mechanism. quality-of-service (QoS) access point (AP): An AP that supports the QoS facility. NOTE- In IEEE Std 802.11, the functions of a QoS AP are a superset of the functions of a non-QoS AP, and thus a QoS AP is able to function as a non-QoS AP to non-QoS stations (STAs). (11ae) quality-of-service (QoS) station (STA): A STA that implements the QoS facility. NOTE-A QoS STA acts as a non-QoS STA when associated in a non-QoS basic service set (BSS). traffic identifier (TID): Any of the identifiers usable by higher layer entities to distinguish medium access control (MAC) service data units (MSDUs) to MAC entities that support quality of service (QoS) within the MAC data service. NOTE--There are 16 possible TID values; eight identify TCs, and the other eight identify parameterized TSs. The TID is assigned to an MSDU in the layers above the MAC. transmission opportunity (TXOP): An interval of time when a particular quality-of-service (QoS) station (STA) has the right to initiate frame exchange sequences onto the wireless medium (WM). NOTE--A TXOP is defined by a starting time and a maximum duration. The TXOP is either obtained by the STA by successfully contending for the channel or assigned by the hybrid coordinator (HC). wildcard BSSID: A BSSID value used to represent all BSSIDs. NOTE—In IEEE Std. 802.11, this is represented by all binary 1s. wildcard SSID: A SSID value used to represent all SSIDs. NOTE—In IEEE Std. 802.11, this is represented by the value “null”. |  |
| --- | --- |

Reference material  (i.e., not part of the changes for this comment)

Definitions from 3.1 not moved:

| term | definition |
| --- | --- |
| access control | The prevention of unauthorized usage of resources. |
| access point (AP) | An entity that contains one station (STA) and provides access to the distribution services, via the wireless medium (WM) for associated STAs. |
| access point (AP) reachability | An AP is reachable by a station (STA) if preauthentication messages can be exchanged between the STA and the target AP via the distribution system (DS). |
| additional authentication data (AAD) | Data that are not encrypted, but are cryptographically protected. |
| admission control | An algorithm to ensure that admittance of a new flow into a resource constrained network does not violate parameterized service commitments made by the network to admitted flows. |
| aggregate medium access control (MAC) protocol data unit (A-MPDU) | A structure containing one or more MPDUs, transported as a single physical layer (PHY) service data unit (PSDU) by the PHY. |
| aggregate medium access control (MAC) protocol data unit (A-MPDU) subframe | A portion of an A-MPDU containing a delimiter and optionally containing an MPDU plus any necessary padding. |
| aggregate medium access control (MAC) service data unit (A-MSDU) | A structure containing one or more MSDUs, transported within a single (unfragmented) data medium access control (MAC) protocol data unit (MPDU). |
| aggregate medium access control (MAC) service data unit (A-MSDU) subframe | A portion of an A-MSDU containing a header and associated MSDU. |
| antenna connector | The measurement point of reference for radio frequency (RF) measurements in a station (STA). The antenna connector is the point in the STA architecture representing the input of the receiver (output of the antenna) for radio reception and the input of the antenna (output of the transmitter) for radio transmission. In systems using multiple antennas or antenna arrays, the antenna connector is a virtual point representing the aggregate output of (or input to) the multiple antennas. In systems using active antenna arrays with processing, the antenna connector is the output of the active array, which includes any processing gain of the active antenna subsystem. |
| antenna selection (ASEL) receiver | A station (STA) that performs receive ASEL. |
| antenna selection (ASEL) transmitter | A station (STA) that performs transmit ASEL. |
| antenna weight vector (AWV) | A vector of weights describing the excitation (amplitude and phase) for each element of an antenna array. |
| association | The service used to establish access point/station (AP/STA) mapping and enable STA invocation of the distribution system services (DSSs). |
| authentication | The service used to establish the identity of one station (STA) as a member of the set of STAs authorized to associate with another STA. |
| Authentication Server (AS) | An entity that provides an authentication service to an Authenticator. This service determines, from the credentials provided by the Supplicant, whether the Supplicant is authorized to access the services provided by the Authenticator. (IEEE Std 802.1X-2010) |
| Authenticator | An entity at one end of a point-to-point LAN segment that facilitates authentication of the entity attached to the other end of that link. (IEEE Std 802.1X-2010) |
| Authenticator address (AA) | The medium access control (MAC) address of the IEEE Std 802.1X Authenticator. |
| authorization | The act of determining whether a particular right, such as access to a resource, is granted to an entity. |
| authorized | To be explicitly allowed. |
| average noise plus interference power indicator (ANIPI) | A medium access control (MAC) indication of the average noise plus interference power measured on a channel that meets the two simultaneous conditions: 1) the station (STA) is not transmitting a frame, and 2) the station (STA) is not receiving a frame addressed to itself. |
| azimuth | The horizontal orientation of the front surface of a station or of a radio antenna system's main lobe measured clockwise from true north. |
| basic service area (BSA) | The area containing the members of a basic service set (BSS). It might contain members of other BSSs. |
| basic service set (BSS) | A set of stations (STAs) that have successfully synchronized using the JOIN service primitives and one STA that has used the START primitive. Alternatively, a set of STAs that have used the START primitive specifying matching mesh profiles where the match of the mesh profiles has been verified via the scanning procedure. Membership in a BSS does not imply that wireless communication with all other members of the BSS is possible. |
| basic service set (BSS) transition | A station (STA) movement from one BSS to another BSS in the same extended service set (ESS). |
| beamforming | A spatial filtering mechanism used at a transmitter to improve the received signal power or signal-to-noise ratio (SNR) at an intended receiver. Syn: beam steering. |
| big endian | The concept that, for a given multi-octet numeric representation, the most significant octet has the lowest address. |
| broadcast address | A unique group address that specifies all stations (STAs). |
| calibration initiator | A station (STA) that initiates a calibration sequence. |
| calibration responder | A station (STA) that transmits during a calibration sequence in response to a transmission by a calibration initiator. |
| candidate peer mesh station (STA) | A neighbor mesh STA to which a mesh peering has not been established but meets eligibility requirements to become a peer mesh STA. |
| channel | An instance of communications medium use for the purpose of passing protocol data units (PDUs) between two or more stations (STAs). |
| channel spacing | The difference between the center frequencies of two nonoverlapping and adjacent channels of the radio transmitter. |
| cipher suite | A set of one or more algorithms, designed to provide data confidentiality, data authenticity or integrity, and/or replay protection. |
| clear channel assessment (CCA) function | That logical function in the physical layer (PHY) that determines the current state of use of the wireless medium (WM). |
| collocated interference | Interference that is caused by another radio or station (STA) emitting radio energy located in the same physical device as the reporting STA, where the reported characteristics of the interference are known a priori without interference detection, measurement, or characterization by the reporting STA. |
| collocated radio | A radio capable of emitting radio-frequency energy located in the same physical device as the reporting station (STA), where the radio's type and some link characteristics are known without signal detection or measurement by the reporting STA. |
| configuration profile | A collection of parameters identified by a profile identifier (ID) that represent a current or available configuration of a station (STA). |
| contiguous transmission | A transmission that uses only one frequency segment. |
| coordination function | The logical function that determines when a station (STA) operating within a basic service set (BSS) is permitted to transmit protocol data units (PDUs) via the wireless medium (WM). |
| Counter mode with Cipher-block chaining Message authentication code (CCM) | A symmetric key block cipher mode providing confidentiality using counter mode (CTR) and data origin authenticity using cipher-block chaining message authentication code (CBC-MAC). |
| cryptographic encapsulation | The process of generating the cryptographic payload from the plaintext data. This comprises the cipher text as well as any associated cryptographic state required by the receiver of the data, e.g., initialization vectors (IVs), sequence numbers, message integrity codes (MICs), key identifiers. |
| data confidentiality | A property of information that prevents disclosure to unauthorized individuals, entities, or processes. |
| deauthentication service | The service that voids an existing authentication relationship. |
| decapsulate | To recover an unprotected frame from a protected one. |
| decapsulation | The process of generating plaintext data by decapsulating an encapsulated frame. |
| dependent station (STA) | A STA that is not registered and whose operational parameters are dictated by messages it receives from an enabling STA. Once enabled by the dynamic STA enablement (DSE) process, a dependent STA's continued operation becomes contingent upon being able to receive messages from its enabling STA over the air. |
| destination mesh station (STA) | A mesh STA that is the final destination of a MAC service data unit (MSDU). This mesh STA might reside in a proxy mesh gate that might forward the MSDU to a STA outside of the mesh basic service set (MBSS). A destination mesh STA might be an end station as defined in IEEE Std 802.1. |
| direct link | A bidirectional link from one quality-of-service (QoS) station (STA) to another QoS STA operating in the same infrastructure QoS basic service set (BSS) that does not pass through a QoS access point (AP). Once a direct link has been set up, all frames between the two QoS STAs are exchanged directly. |
| directed frame | See: individually addressed. |
| directed multicast service (DMS) | A service in which the access point (AP) transmits group addressed frames as individually addressed frames to the requesting non-AP station (STA). |
| disassociation service | The service that removes an existing association. |
| distributed coordination function (DCF) | A class of coordination function where the same coordination function logic is active in every station (STA) in the basic service set (BSS) whenever the network is in operation. |
| distribution service | The service that, by using association information, delivers medium access control (MAC) service data units (MSDUs) within the distribution system (DS). |
| distribution system (DS) | A system used to interconnect a set of basic service sets (BSSs) and integrated local area networks (LANs) to create an extended service set (ESS). |
| distribution system medium (DSM) | The medium or set of media used by a distribution system (DS) for communications between access points (APs), mesh gates, and portals of an extended service set (ESS). |
| distribution system service (DSS) | The set of services provided by the distribution system (DS) that enable the medium access control (MAC) to transport MAC service data units (MSDUs) between stations (STAs) that are not in direct communication with each other over a single instance of the wireless medium (WM). These services include transport of MSDUs between the access points (APs) of basic service sets (BSSs) within an extended service set (ESS), transport of MSDUs between portals and BSSs within an ESS, transport of MSDUs between mesh gates in the same or different mesh basic service sets (MBSSs), transport of MSDUs between mesh gates and APs, transport of MSDUs between mesh gates and portals, and transport of MSDUs between STAs in the same BSS in cases where the MSDU has a group destination address or where the destination is an individual address and the STA is associated with an AP. |
| dynamic frequency selection (DFS) | Facilities mandated to satisfy requirements in some regulatory domains for radar detection and uniform channel spreading in the 5 GHz band. These facilities might also be used for other purposes, such as automatic frequency planning. |
| dynamic station (STA) enablement (DSE) | The process by which an enabling STA grants permission and dictates operational procedures to STAs that are subject to its control. |
| effective isotropic radiated power (EIRP) | The equivalent power of a transmitted signal in terms of an isotropic (omnidirectional) radiator. The EIRP equals the product of the transmitter power and the antenna gain (reduced by any coupling losses between the transmitter and antenna). |
| emergency alert system (EAS) | A U.S. national public warning system. |
| enabling station (STA) | A registered STA that has the authority to control when and how a dependent STA can operate. An enabling STA communicates an enabling signal to its dependents over the air. An enabling STA chooses whether other dynamic STA enablement (DSE) messages are exchanged over the air, over the distribution system (DS), or by mechanisms that rely on transport via higher layers. |
| encapsulate | To construct a protected frame from an unprotected frame. |
| encapsulation | The process of generating a protected frame by encapsulating plaintext data. |
| extended service area (ESA) | The area within which members of an extended service set (ESS) can communicate. An ESA is larger than or equal to a basic service area (BSA) and might involve several basic service sets (BSSs) in overlapping, disjointed, or both configurations. |
| extended service set (ESS) | A set of one or more interconnected basic service sets (BSSs) that appears as a single BSS to the logical link control (LLC) layer at any station (STA) associated with one of those BSSs. |
| extended service set (ESS) transition | A station (STA) movement from one basic service set (BSS) in one ESS to another BSS in a different ESS. |
| fast basic service set (BSS) transition | A station (STA) movement that is from one BSS in one extended service set (ESS) to another BSS within the same ESS and that minimizes the amount of time that data connectivity is lost between the STA and the distribution system (DS). |
| fast session transfer (FST) | The transfer of a session from a channel to another channel, in the same or different frequency bands. The term session refers to non-physical layer state information kept by a pair of stations (STAs) that communicate directly (i.e., excludes forwarding). |
| fixed station (STA) | A STA that is physically attached to a specific location. In licensed bands, a fixed STA might be authorized to operate only at a specific location. |
| forwarding information | The information maintained by a mesh station (STA) that allows the mesh STA to perform its path selection and forwarding functions. |
| fragmentation | The process of segmenting a medium access control (MAC) service data unit (MSDU) or MAC management protocol data unit (MMPDU) into a sequence of smaller MAC protocol data units (MPDUs) prior to transmission. The process of recombining a set of fragment MPDUs into an MSDU or MMPDU is known as defragmentation. These processes are described in 5.8.1.9 of ISO/IEC 7498-1:1994. |
| frame | A unit of data exchanged between peer protocol entities. |
| frequency segment | A contiguous block of spectrum used by a transmission. |
| Gaussian frequency shift keying (GFSK) | A modulation scheme in which the data are first filtered by a Gaussian filter in the baseband and then modulated with a simple frequency modulation. |
| group | The entities in a wireless network, e.g., an access point (AP) and its associated stations (STAs), or all the STAs in an independent basic service set (IBSS) network. |
| group address | A medium access control (MAC) address that has the group bit equal to 1. Syn: multicast address. |
| group addressed | When applied to a medium access control (MAC) service data unit (MSDU), it is an MSDU with a group address as the destination address (DA). |
| group master key (GMK) | An auxiliary key that might be used to derive a group temporal key (GTK). |
| group temporal key (GTK) | A random value, assigned by the group source, which is used to protect group addressed medium access control (MAC) protocol data units (MPDUs) from that source. The GTK might be derived from a group master key (GMK). |
| hidden station (STA) | A STA whose transmissions are not detected using carrier sense (CS) by a second STA, but whose transmissions interfere with transmissions from the second STA to a third STA |
| homogenous extended service set (ESS) | A collection of basic service sets (BSSs), within the same extended service set (ESS), in which every subscription service provider network (SSPN) or other external network reachable at one BSS is reachable at all of them. |
| idle power indicator (IPI) | A physical layer (PHY) indication of the total channel power (noise and interference) as measured in the channel at the receiving antenna connector while the station (STA) is idle, i.e., neither transmitting nor receiving a frame. |
| IEEE Std 802.1X authentication | Extensible Authentication Protocol (EAP) authentication transported by the IEEE Std 802.1X protocol. |
| independent basic service set (IBSS) | A basic service set (BSS) that forms a self-contained network, and in which no access to a distribution system (DS) is available. |
| individual address | A medium access control (MAC) address in which the group bit is 0. Syn: directed address, unicast address. |
| individually addressed | When applied to a medium access control (MAC) service data unit (MSDU), it is an MSDU with an individual address as the destination address (DA). |
| infrastructure | The infrastructure includes the distribution system medium (DSM), access point (AP), and portal entities. It is also the logical location of distribution and integration service functions of an extended service set (ESS). An infrastructure contains one or more APs and zero or more portals in addition to the distribution system (DS). |
| infrastructure authorization information | The information that specifies the access rights of the user of a non-access-point (non-AP) station (STA). This information might include the rules for routing the user traffic, a set of permissions about services that a user is allowed to access, quality-of-service (QoS) configuration information, or the accounting policy to be applied by the infrastructure. |
| integration service | The service that enables delivery of medium access control (MAC) service data units (MSDUs) between the distribution system (DS) and a local area network (LAN) (via a portal). |
| integrity GTK (IGTK) | A random value, assigned by the broadcast/multicast source STA, which is used to protect group addressed medium access control (MAC) management protocol data units (MMPDUs) from that source STA. |
| link margin | Ratio of the received signal power to the minimum required by the station (STA). The STA might incorporate rate information and channel conditions, including interference, into its computation of link margin. The specific algorithm for computing the link margin is implementation dependent. |
| link metric | A criterion used to characterize the performance, quality, and eligibility of a link. |
| little endian | The concept that, for a given multi-octet numeric representation, the least significant octet has the lowest address. |
| liveness | A demonstration that the peer is actually participating in this instance of communication. |
| location subject local | The term used when a location request is for the location of the requesting STA, i.e., when the requesting STA asks, Where am I? |
| location subject remote | The term used when a location request is for the location of the reporting STA, i.e., when the requesting STA asks, Where are you? |
| location subject third party | The term used when the location request is for the location of a station (STA) other than the requesting STA or the requested STA, (i.e., when the requesting STA asks, Where is he/she?) |
| master session key (MSK) | Keying material that is derived between the Extensible Authentication Protocol (EAP) peer and exported by the EAP method to the Authentication Server (AS). |
| medium access control (MAC) frame | The unit of data exchanged between MAC entities. Syn: MPDU. |
| medium access control (MAC) protocol data unit (MPDU) | The unit of data exchanged between two peer MAC entities using the services of the physical layer (PHY). Syn: MAC frame. |
| medium access control (MAC) service data unit (MSDU) | Information that is delivered as a unit between MAC service access points (SAPs). |
| mesh basic service set (MBSS) | A basic service set (BSS) that forms a self-contained network of mesh stations (STAs) that use the same mesh profile. An MBSS contains zero or more mesh gates, and can be formed from mesh STAs that are not in direct communication. |
| mesh facility | The set of enhanced functions, channel access rules, frame formats, mutual authentication methods, and managed objects used to provide data transfer among autonomously operating stations (STAs) that might not be in direct communication with each other over a single instance of the wireless medium. Communication between STAs using the mesh facility takes place using only the wireless medium. The mesh facility transports an MSDU between source and destination STAs over potentially multiple hops of the wireless medium without transiting the MAC_SAP at intermediate STAs. |
| mesh gate | Any entity that has mesh station (STA) functionality and provides access to one or more distribution systems, via the wireless medium (WM) for the mesh basic service set (MBSS). |
| mesh link | A link from one mesh station (STA) to a neighbor mesh STA that have a mesh peering with each other. |
| mesh neighborhood | The set of all neighbor mesh stations (STAs) of a particular mesh STA. |
| mesh path | A concatenated set of mesh links from a source mesh station (STA) to a destination mesh STA. |
| mesh path selection | The process of selecting a mesh path. |
| mesh peering | A relationship between two mesh stations (STAs) that is required for direct communication over a single instance of the wireless medium (WM). A mesh peering is established with a mesh peering protocol. |
| mesh profile | A set of values of parameters that identifies the attributes of the mesh basic service set (MBSS) and that is used in a single MBSS. |
| mesh services | The set of services that enable the creation and operation of a mesh basic service set (MBSS). |
| mesh station (STA) | A quality-of-service (QoS) STA that implements the mesh facility. |
| message integrity code (MIC) | A value generated by a cryptographic function. If the input data are changed, a new value cannot be correctly computed without knowledge of the cryptographic key(s) used by the cryptographic function. |
| mobile station (STA) | A type of STA that uses network communications while in motion. |
| mobility domain | A set of basic service sets (BSSs), within the same extended service set (ESS), that support fast BSS transitions between themselves and that are identified by the set's mobility domain identifier (MDID). |
| mobility domain identifier (MDID) | An identifier that names a mobility domain. |
| multicast | See: group addressed. |
| multicast address | See: group address. |
| multicast-group address | A medium access control (MAC) address associated by higher level convention with a group of logically related stations (STAs). |
| multi-level precedence and preemption (MLPP) | A framework used with admission control for the treatment of traffic streams based on precedence, which supports the preemption of an active traffic stream by a higher precedence traffic stream when resources are limited. Preemption is the act of forcibly removing a traffic stream in progress in order to free up resources for another higher precedence traffic stream. |
| multiple input, multiple output (MIMO) | A physical layer (PHY) configuration in which both transmitter and receiver use multiple antennas. |
| multiple medium access control (MAC) station management entity (SME) (MM-SME) | Component of station management that manages multiple cooperating stations (STAs). |
| multi-user multiple input, multiple output (MU-MIMO) | A technique by which multiple stations (STAs), each with one or more antennas, either simultaneously transmit to a single STA or simultaneously receive from a single STA independent data streams over the same radio frequencies. |
| neighbor access point (AP) | Any AP that is a potential service set transition candidate. |
| neighbor station (STA) | A STA in the following relationship: STA A is a neighbor to STA B if STA A can both directly transmit to and receive from STA B over the wireless medium. |
| network access identifier (NAI) | The user identity submitted by the Supplicant during IEEE Std 802.1X authentication. |
| network access server (NAS) client | The client component of a NAS that communicates with the Authentication Server (AS). |
| network allocation vector (NAV) | An indicator, maintained by each station (STA), of time periods when transmission onto the wireless medium (WM) is not initiated by the STA regardless of whether the STA's clear channel assessment (CCA) function senses that the WM is busy. |
| next-hop mesh station (STA) | The next peer mesh STA on the mesh path to the destination mesh STA. |
| non-access-point (non-AP) station (STA) | A STA that is not contained within an AP. |
| nonce | A numerical value, used in cryptographic operations associated with a given cryptographic key, that is not to be reused with that key, including over all reinitializations of the system through all time. |
| noncontiguous transmission | A transmission that uses nonadjacent frequency segments. |
| nonoperating channel | A channel that is not the operating channel of the basic service set (BSS) of which the station (STA) is a member. |
| non-quality-of-service (non-QoS) access point (AP) | An AP that does not support the quality-of-service (QoS) facility. |
| non-quality-of-service (non-QoS) basic service set (BSS) | A BSS that does not support the quality-of-service (QoS) facility. |
| non-quality-of-service (non-QoS) station (STA) | A STA that does not support the quality-of-service (QoS) facility. |
| operating channel | The operating channel is the channel in which beacons are transmited. |
| operating channel width | The channel width in which the station (STA) is currently able to receive. |
| overlapping basic service set (OBSS) | A basic service set (BSS) operating on the same channel as the station's (STA's) BSS and within (either partly or wholly) its basic service area (BSA). |
| over-the-air fast basic service set (BSS) transition (FT) | An FT method in which the station (STA) communicates over a WM link to the target access point (AP). |
| over-the-DS (distribution system) fast basic service set (BSS) transition (FT) | An FT method in which the station (STA) communicates with the target access point (AP) via the current AP. |
| pairwise | Referring to, or an attribute of, two entities that are associated with each other, e.g., an access point (AP) and an associated station (STA), or two STAs in an independent basic service set (IBSS) network. This term is used to refer to a type of encryption key hierarchy pertaining to keys shared by only two entities. |
| pass-phrase | A secret text string employed to corroborate the user's identity. |
| password | A shared, secret, and potentially low-entropy word, phrase, code, or key used as a credential for authentication purposes. |
| path metric | An aggregate multi-hop criterion used to characterize the performance, quality, and eligibility of a mesh path. |
| peer mesh station (STA) | A mesh STA to which a mesh peering has been established. |
| peer-to-peer link | A direct link within a quality-of-service (QoS) basic service set (BSS), a tunnelled direct-link setup (TDLS) link, or a STA-to-STA communication in an independent basic service set (IBSS). |
| peer-to-peer traffic specification (PTP TSPEC) | The quality-of-service (QoS) characteristics of a data flow between non-access point (non-AP) QoS stations (STAs). |
| per-frame encryption key | A unique encryption key constructed for each medium access control (MAC) protocol data unit (MPDU). |
| physical layer (PHY) frame | The unit of data exchanged between PHY entities. Syn: PPDU. |
| piggyback | The overloading of data with an acknowledgment of a previously received frame to the station (STA) to which the data is directed. |
| portable station (STA) | A type of station (STA) that might be moved from location to location, but that only uses network communications while at a fixed location. |
| portal | The logical point at which the integration service is provided. |
| precursor mesh station (STA) | A neighbor peer mesh STA on the mesh path to the destination mesh STA, that identifies the mesh STA as the next-hop mesh STA. |
| preshared key (PSK) | A static key that is distributed to the units in the system by some out-of-band means. |
| primary channel | The common channel of operation for all stations (STAs) that are members of the basic service set (BSS). |
| prioritized quality of service (QoS) | The provisioning of service in which the medium access control (MAC) protocol data units (MPDUs) with higher priority are given a preferential treatment over MPDUs with a lower priority. |
| protection mechanism | Any procedure that attempts to update the network allocation vector (NAV) of all receiving stations (STAs) prior to the transmission of a frame that might or might not be detected as valid network activity by the physical layer (PHY) entities at those receiving STAs. |
| protocol instance | An execution of a particular protocol that consists of the state of the communicating parties as well as the messages exchanged. |
| proxy mesh gate | A mesh gate acting as an intermediary for IEEE 802 stations (STAs) outside the mesh basic service set (MBSS). |
| pseudorandom function (PRF) | A function that hashes various inputs to derive a pseudorandom value. In order to ensure liveness of a communication in which a pseudorandom value is used, a nonce is used as one of the inputs to the function. |
| public safety answering point (PSAP) | A physical location where emergency calls are received and routed to the appropriate emergency service dispatch center. |
| quadrature binary phase shift keying (QBPSK) | A binary phase shift keying modulation in which the binary data is mapped onto the imaginary (Q) axis. |
| quality-of-service (QoS) access point (AP) | An AP that supports the QoS facility. |
| quality-of-service (QoS) basic service set (BSS) | A BSS that provides the QoS facility. An infrastructure QoS BSS contains a QoS access point (AP). |
| quality-of-service (QoS) facility | The set of enhanced functions, channel access rules, frame formats, frame exchange sequences and managed objects used to provide QoS. |
| quality-of-service (QoS) independent basic service set (IBSS) | An IBSS in which one or more of its stations (STAs) support the QoS facility. |
| quality-of-service (QoS) station (STA) | A STA that implements the QoS facility. |
| reassociation service | The service that enables an established association [between access point (AP) and station (STA)] to be transferred from one AP to another (or the same) AP. |
| receive chain | The physical entity that implements any necessary signal processing to provide the received signal to the digital baseband. Such signal processing includes filtering, amplification, down-conversion, and sampling. |
| receive power | Mean power measured at the antenna connector. |
| received channel power indicator (RCPI) | An indication of the total channel power (signal, noise, and interference) of a received frame measured on the channel and at the antenna connector used to receive the frame. |
| received power indicator (RPI) | A quantized measure of the received power level as seen at the antenna connector. |
| registered station (STA) | A STA for which information needs to be submitted to an appropriate regulatory or coordination authority before it is allowed to transmit. |
| remote request broker (RRB) | The component of the station management entity (SME) of an access point (AP) that supports fast basic service set (BSS) transitions over the distribution system (DS). |
| restricted channel | A radio channel in which transmission is restricted to stations (STAs) that operate under the control of licensed operators. |
| roaming consortium | A group of subscription service providers (SSPs) having inter-SSP roaming agreements. |
| service set transition | A STA movement from one BSS to another BSS, i.e., either a BSS transition or an ESS transition. |
| serving AP | The AP to which the STA is associated. |
| source mesh station (STA) | A mesh STA from which a MAC service data unit (MSDU) enters the mesh basic service set (MBSS). A source mesh STA is either a mesh STA that is the source of an MSDU or a proxy mesh gate that receives an MSDU from a STA outside of the MBSS and forwards the MSDU on a mesh path. |
| space-time block coding/spatial multiplexing (STBC/SM) | A combination of STBC and SM where one spatial stream is transmitted using STBC and one or two additional spatial streams are transmitted using SM. |
| space-time streams | Streams of modulation symbols created by applying a combination of spatial and temporal processing to one or more spatial streams of modulation symbols. |
| spatial multiplexing (SM) | A transmission technique in which data streams are transmitted on multiple spatial channels that are provided through the use of multiple antennas at the transmitter and the receiver. |
| spatial stream | One of several streams of bits or modulation symbols that might be transmitted over multiple spatial dimensions that are created by the use of multiple antennas at both ends of a communications link. |
| station (STA) | A logical entity that is a singly addressable instance of a medium access control (MAC) and physical layer (PHY) interface to the wireless medium (WM). |
| station service (SS) | The set of services that support transport of medium access control (MAC) service data units (MSDUs) between stations (STAs) within a basic service set (BSS). |
| subscription service provider (SSP) | An organization (operator) offering connection to network services, perhaps for a fee. |
| subscription service provider network (SSPN) | The network controlled by a subscription service provider (SSP). The network maintains user subscription information. |
| Supplicant | An entity at one end of a point-to-point LAN segment that is being authenticated by an Authenticator attached to the other end of that link. (IEEE Std 802.1X-2010) |
| Supplicant address (SPA) | The medium access control (MAC) address of the IEEE Std 802.1X Supplicant. |
| time unit (TU) | A measurement of time equal to 1024 ?s. |
| traffic category (TC) | A label for medium access control (MAC) service data units (MSDUs) that have a distinct user priority (UP), as viewed by higher layer entities, relative to other MSDUs provided for delivery over the same link. Traffic categories are meaningful only to MAC entities that support quality of service (QoS) within the MAC data service. These MAC entities determine the UP for MSDUs belonging to a particular traffic category using the priority value provided with those MSDUs at the MAC service access point (MAC_SAP). |
| traffic classification (TCLAS) | The specification of certain parameter values to identify a protocol data unit (PDU) or a medium access control (MAC) service data unit (MSDU). The classification process may be performed above the MAC service access point (MAC_SAP), within the MLME, or within the MAC, based on the type of classification. |
| traffic filter | A set of traffic specifications defined by the use of traffic classification (TCLAS) elements that are utilized by the traffic filtering service (TFS) to identify specific allowed frames. |
| traffic filtering service (TFS) | A service provided by an access point (AP) to a non-AP station (STA) to reduce the number of frames sent to the non-AP STA by not forwarding individually addressed frames addressed to the non-AP STA that do not match traffic filters specified by the non-AP STA. |
| traffic identifier (TID) | Any of the identifiers usable by higher layer entities to distinguish medium access control (MAC) service data units (MSDUs) to MAC entities that support quality of service (QoS) within the MAC data service. |
| traffic specification (TSPEC) | The quality-of-service (QoS) characteristics of a data flow to and from a QoS station (STA). |
| traffic stream (TS) | A set of medium access control (MAC) service data units (MSDUs) to be delivered subject to the quality-of-service (QoS) parameter values provided to the MAC in a particular traffic specification (TSPEC). TSs are meaningful only to MAC entities that support QoS within the MAC data service. These MAC entities determine the TSPEC applicable for delivery of MSDUs belonging to a particular TS using the priority parameter provided with those MSDUs at the MAC service access point (MAC_SAP). |
| transmission opportunity (TXOP) | An interval of time when a particular quality-of-service (QoS) station (STA) has the right to initiate frame exchange sequences onto the wireless medium (WM). |
| unauthorized disclosure | The process of making information available to unauthorized individuals, entities, or processes. |
| unauthorized resource use | Use of a resource not consistent with the defined security policy. |
| unicast | See: individually addressed. |
| unicast address | See: individual address. |
| uniform spreading | A regulatory requirement for a channel selection mechanism that provides uniform usage across a minimum set of channels in the regulatory domain. |
| unreachable destination | A destination mesh station (STA) for which the link to the next hop of the mesh path to this destination mesh STA is no longer usable. |
| user priority (UP) | A value associated with an medium access control (MAC) service data unit (MSDU) that indicates how the MSDU is to be handled. The UP is assigned to an MSDU in the layers above the MAC. |
| validated AP | An AP that has either been explicitly configured as a neighbor or learned through a mechanism such as the Beacon report. |
| wildcard BSSID | A BSSID value used to represent all BSSIDs. |
| wildcard SSID | A SSID value (null) used to represent all SSIDs. |
| wireless distribution system (WDS) | Often used as a vernacular term for a mechanism for wireless communication among nonmesh stations (STAs) using a four address frame format. |
| wireless local area network (WLAN) system | A system that includes the distribution system (DS), access points (APs), and portal entities. It is also the logical location of distribution and integration service functions of an extended service set (ESS). A WLAN system contains one or more APs and zero or more portals in addition to the DS. |
| wireless medium (WM) | The medium used to implement the transfer of protocol data units (PDUs) between peer physical layer (PHY) entities of a wireless local area network (LAN). |

R14 Markup is shown in the box below:

access category (AC): A label for the common set of enhanced distributed channel access (EDCA) parameters that are used by a quality-of-service (QoS) station (STA) to contend for the channel in order to transmit medium access control (MAC) service data units (MSDUs) with certain priorities. access control: The prevention of unauthorized usage of resources. access period: A time period during a beacon interval established in a directional multi-gigabit (DMG) basic service set (BSS) that has associated channel access rules. (11ad) access point (AP): An entity that contains one station (STA) and provides access to the distribution services, via the wireless medium (WM) for associated STAs. access point (AP) path: Path between two tunneled direct-link setup (TDLS) peer stations (STAs) via the AP with which the STAs are currently associated. access point (AP) reachability: An AP is reachable by a station (STA) if preauthentication messages can be exchanged between the STA and the target AP via the distribution system (DS). NOTE 1(#1101)—Preauthentication is defined in 11.5.10.2 (Preauthentication and RSNA key management). active mode: A mesh power mode in which the mesh station (STA) operates in the Awake state towards a neighbor mesh STA. additional authentication data (AAD): Data that are not encrypted, but are cryptographically -protected.(#1121) admission control: An algorithm to ensure that admittance of a new flow into a resource constrained network does not violate parameterized service commitments made by the network to admitted flows. aggregate medium access control (MAC) protocol data unit (A-MPDU): A structure containing one or more(#146) MPDUs, transported as a single physical layer (#61)(PHY) service data unit (PSDU) by the PHY. aggregate medium access control (MAC) service data unit (A-MSDU): A structure containing one or more(#146) MSDUs, transported within a single (unfragmented) data medium access control (MAC) protocol data unit (MPDU). aggregate medium access control (MAC) service data unit (A-MSDU) subframe: A portion of an A-MSDU containing a header and associated MSDU. aggregated schedule: The aggregation of delivery and/or poll schedules by the quality-of-service (QoS) access point (AP) for a particular QoS station (STA) into a single service period (SP). antenna connector: The measurement point of reference for radio frequency (RF) measurements in a station (STA). The antenna connector is the point in the STA architecture representing the input of the receiver (output of the antenna) for radio reception and the input of the antenna (output of the transmitter) for radio transmission. In systems using multiple antennas or antenna arrays, the antenna connector is a virtual point representing the aggregate output of (or input to) the multiple antennas. In systems using active antenna arrays with processing, the antenna connector is the output of the active array, which includes any processing gain of the active antenna subsystem. antenna selection (ASEL) receiver: A station (STA) that performs receive ASEL. antenna selection (ASEL) transmitter: A station (STA) that performs transmit ASEL. antenna weight vector (AWV): A vector of weights describing the excitation (amplitude and phase) for each element of an antenna array. (11ad) association: The service used to establish access point/station (AP/STA) mapping and enable STA invocation of the distribution system services (DSSs). authentication: The service used to establish the identity of one station (STA) as a member of the set of STAs authorized to associate with another STA. authentication and key management (AKM) suite: A set of one or more algorithms designed to provide authentication and key management, either individually or in combination with higher layer authentication and key management algorithms outside the scope of this standard. Authentication Server (AS): An entity that provides an authentication service to an Authenticator. This service determines, from the credentials provided by the Supplicant, whether the Supplicant is authorized to access the services provided by the Authenticator. (IEEE Std 802.1X-2010(M14)) Authenticator: An entity at one end of a point-to-point LAN segment that facilitates authentication of the entity attached to the other end of that link. (IEEE Std 802.1X-2010(M14)) Authenticator address (AA): The medium access control (MAC) address of the IEEE Std(#130) 802.1X Authenticator. authorization: The act of determining whether a particular right, such as access to a resource, is granted to an entity. NOTE 2(#1101)—See IETF RFC 2903 [B32]. authorized: To be explicitly allowed. average noise plus interference power indicator (ANIPI): A medium access control (MAC) indication of the average noise plus interference power measured on a channel that meets the two simultaneous conditions: 1) the station (STA) is not transmitting a frame, and 2) the station (STA) is not receiving a frame addressed to itself.(11ad) average noise power indicator (ANPI): A medium access control (MAC) indication of the average noise plus interference power measured when the channel is idle as defined by three simultaneous conditions: 1) the Virtual Carrier Sense (CS) mechanism indicates idle channel, 2) the station (STA) is not transmitting a frame, and 3) the STA is not receiving a frame. azimuth: The horizontal orientation of the front surface of a station or of a radio antenna system’s main lobe measured clockwise from true north. base channel: Channel on which the tunneled direct-link setup (TDLS) peer station (STA) is associated with an access point (AP). basic service area (BSA): The area containing the members of a basic service set (BSS). It might contain members of other BSSs. basic service set (BSS): A set of stations (STAs) that have successfully synchronized using the JOIN service primitives and one STA that has used the START primitive. Alternatively, a set of STAs that have used the START primitive specifying matching mesh profiles where the match of the mesh profiles has been verified via the scanning procedure. Membership in a BSS does not imply that wireless communication with all other members of the BSS is possible. basic service set (BSS) transition: A station (STA) movement from one BSS to another BSS in the same extended service set (ESS). beamformee: A station (STA) that receives a physical layer (#61)(PHY) protocol data unit (PPDU) that was transmitted using a beamforming steering matrix. beamformer: A station (STA) that transmits a physical layer (#61)(PHY) protocol data unit (PPDU) using a beamforming steering matrix. beamforming: A spatial filtering mechanism used at a transmitter to improve the received signal power or signal-to-noise ratio (SNR) at an intended receiver. Syn: beam steering. big endian: The concept that, for a given multi-octet numeric representation, the most significant octet has the lowest address. broadcast address: A unique group address that specifies all stations (STAs). basic service set (BSS)(#1177) max idle period: A time period during which the access point (AP) does not disassociate a station (STA) due to nonreceipt of frames from that STA. calibration initiator: A station (STA) that initiates a calibration sequence. calibration responder: A station (STA) that transmits during a calibration sequence in response to a transmission by a calibration initiator. candidate peer mesh station (STA): A neighbor mesh STA to which a mesh peering has not been established but meets eligibility requirements to become a peer mesh STA. channel: An instance of communications medium use for the purpose of passing protocol data units (PDUs) between two or more stations (STAs). channel spacing: The difference between the center frequencies of two nonoverlapping and adjacent channels of the radio transmitter. cipher suite: A set of one or more algorithms, designed to provide data confidentiality, data authenticity or integrity, and/or replay protection. clear channel assessment (CCA) function: That logical function in the physical layer (PHY) that determines the current state of use of the wireless medium (WM). collocated interference: Interference that is caused by another radio or station (STA) emitting radio energy located in the same physical device as the reporting STA, where the reported characteristics of the interference are known a priori without interference detection, measurement, or characterization by the reporting STA. collocated radio: A radio capable of emitting radio-frequency energy located in the same physical device as the reporting station (STA), where the radio’s type and some link characteristics are known without signal detection or measurement by the reporting STA. configuration profile: A collection of parameters identified by a profile identifier (ID) that represent a current or available configuration of a station (STA). contention-free period (CFP): The time period during the operation of a point coordination function (PCF) when the right to transmit is assigned to stations (STAs) solely by a point coordinator (PC), allowing frame exchanges to occur between members of the basic service set (BSS) without contention for the wireless medium (WM). contention period (CP): The time period outside of the contention-free period (CFP) in a point--coordinated basic service set (BSS). In a BSS where there is no point coordinator (PC), this corresponds to the entire time of operation of the BSS. controlled access phase (CAP): A time period during which(#1178) the hybrid coordinator (HC) maintains control of the medium, after gaining medium access by sensing the channel to be idle for a point coordination -function (PCF) interframe space (PIFS) duration. It might span multiple consecutive transmission opportunities (TXOPs) and can contain polled TXOPs. contention-based access period (CBAP): The time period within the data transfer interval (DTI) of a directional multi-gigabit (DMG) basic service set (BSS) where enhanced distributed channel access (EDCA) is used. (11ad) - The following definition contains too many 802.11-specific terms. coordination function: The logical function that determines when a station (STA) operating within a basic service set (BSS) is permitted to transmit protocol data units (PDUs) via the wireless medium (WM). NOTE-The coordination function within a BSS might have one hybrid coordination function (HCF), or it might have one HCF and one point coordination function (PCF) and has one distributed coordination function (DCF). A quality-of-service (QoS) BSS has one DCF and one HCF. In addition, a directional multi-gigabit (DMG) STA has a DMG channel access function that includes the beacon transmission interval (BTI), the association beamforming training (A-BFT), the announcement transmission interval (ATI), and the service period channel access (SPCA).(11ad) contention-free (CF) pollable: A station (STA) that is able to respond to a CF poll with a (#100)Data frame if such a frame is queued and able to be generated. Counter mode with Cipher-block chaining Message authentication code (CCM): A symmetric key block cipher mode providing confidentiality using counter mode (CTR) and data origin authenticity using cipher-block chaining message authentication code (CBC-MAC). NOTE 3(#1101)—See IETF RFC 3610. cryptographic encapsulation: The process of generating the cryptographic payload from the plaintext data. This comprises the cipher text as well as any associated cryptographic state required by the receiver of the data, e.g., initialization vectors (IVs), sequence numbers, message integrity codes (MICs), key identifiers. data confidentiality: A property of information that prevents disclosure to unauthorized individuals, entities, or processes. deauthentication service: The service that voids an existing authentication relationship. decapsulate: To recover an unprotected frame from a protected one. decapsulation: The process of generating plaintext data by decapsulating an encapsulated frame. deep sleep mode: A mesh power mode in which the mesh station (STA) operates either in the Awake state or in the Doze state towards a neighbor mesh STA, and is not expected to receive beacons from this neighbor mesh STA. delivery-enabled access category (AC): A quality-of-service (QoS) access point (AP) AC where the AP is allowed to use enhanced distributed channel access (EDCA) to deliver traffic from the AC to a QoS station (STA) in an unscheduled service period (SP) triggered by the STA. dependent station (STA): A STA that is not registered and whose operational parameters are dictated by messages it receives from an enabling STA. Once enabled by the dynamic STA enablement (DSE) process, a dependent STA’s continued operation becomes contingent upon being able to receive messages from its enabling STA over the air. destination mesh station (STA): A mesh STA that is the final destination of a MAC service data unit (MSDU). This mesh STA might reside in a proxy mesh gate that might forward the MSDU to a STA outside of the MBSS. A destination mesh STA might be an end station as defined in IEEE Std 802.1. directed frame: See: individually addressed. direct link: A bidirectional link from one quality-of-service (QoS) station (STA) to another QoS STA operating in the same infrastructure QoS basic service set (BSS) that does not pass through a QoS access point (AP). Once a direct link has been set up, all frames between the two QoS STAs are exchanged directly. directed multicast service (DMS): A service in which the access point (AP) transmits group addressed frames as individually addressed frames to the requesting non-AP station (STA). directional multi-gigabit (DMG): Pertaining to operation in a frequency band containing a channel with the Channel starting frequency above 45 GHz. NOTE 4(#1101)—The Channel starting frequency for 802.11 stations (STAs) is defined in Annex E disassociation service: The service that removes an existing association. distributed coordination function (DCF): A class of coordination function where the same coordination function logic is active in every station (STA) in the basic service set (BSS) whenever the network is in operation. distribution service: The service that, by using association information, delivers medium access control (MAC) service data units (MSDUs) within the distribution system (DS). distribution system (DS): A system used to interconnect a set of basic service sets (BSSs) and integrated local area networks (LANs) to create an extended service set (ESS). distribution system medium (DSM): The medium or set of media used by a distribution system (DS) for communications between access points (APs), mesh gates, and portals of an extended service set (ESS). distribution system service (DSS): The set of services provided by the distribution system (DS) that enable the medium access control (MAC) to transport MAC service data units (MSDUs) between stations (STAs) that are not in direct communication with each other over a single instance of the wireless medium (WM). NOTE--These services include transport of MSDUs between the access points (APs) of basic service sets (BSSs) within an extended service set (ESS), transport of MSDUs between portals and BSSs within an ESS, transport of MSDUs between mesh gates in the same or different mesh basic service sets (MBSSs), transport of MSDUs between mesh gates and APs, transport of MSDUs between mesh gates and portals, and transport of MSDUs between STAs in the same BSS in cases where the MSDU has a group destination address or where the destination is an individual address and the STA is associated with an AP. NOTE 5(#1101)—DSSs are provided between pairs of MACs not on the same instance of the WM. dynamic frequency selection (DFS): Facilities mandated to satisfy requirements in some regulatory domains for radar detection and uniform channel spreading in the 5 GHz band. These facilities might also be used for other purposes, such as automatic frequency planning. dynamic frequency selection (DFS) owner: A station (STA) in an independent basic service set (IBSS) or off-channel TDLS direct link that takes responsibility for selecting the next channel after radar is detected operating in a channel. Due to the nature of IBSSs, it cannot be guaranteed that there is a single DFS owner at any particular time and the protocol is robust to this situation. dynamic station (STA) enablement (DSE): The process by which an enabling STA grants permission and dictates operational procedures to STAs that are subject to its control. effective isotropic radiated power (EIRP): The equivalent power of a transmitted signal in terms of an isotropic (omnidirectional) radiator. The EIRP equals the product of the transmitter power and the antenna gain (reduced by any coupling losses between the transmitter and antenna). emergency alert system (EAS): A U.S. national public warning system. enabling station (STA): A registered STA that has the authority to control when and how a dependent STA can operate. An enabling STA communicates an enabling signal to its dependents over the air. An enabling STA chooses whether other dynamic STA enablement (DSE) messages are exchanged over the air, over the distribution system (DS), or by mechanisms that rely on transport via higher layers. encapsulate: To construct a protected frame from an unprotected frame. encapsulation: The process of generating a protected frame by encapsulating plaintext data. enhanced distributed channel access (EDCA): The prioritized carrier sense multiple access with collision avoidance (CSMA/CA) access mechanism used by quality-of-service (QoS) stations (STAs) in a QoS basic service set (BSS). This access mechanism is also used by the QoS access point (AP) and operates concurrently with hybrid coordination function (HCF) controlled channel access (HCCA). enhanced distributed channel access function (EDCAF): A logical function in a quality-of-service (QoS) station (STA) that determines, using enhanced distributed channel access (EDCA), when a frame in the transmit queue with the associated access category (AC) is permitted to be transmitted via the wireless medium (WM). There is one EDCAF per AC. extended service area (ESA): The area within which members of an extended service set (ESS) can communicate. An ESA is larger than or equal to a basic service area (BSA) and might involve several basic service sets (BSSs) in overlapping, disjointed, or both configurations. extended service set (ESS): A set of one or more interconnected basic service sets (BSSs) that appears as a single BSS to the logical link control (LLC) layer at any station (STA) associated with one of those BSSs. extended service set (ESS) transition: A station (STA) movement from one basic service set (BSS) in one ESS to another BSS in a different ESS. fast basic service set (BSS) transition: A station (STA) movement that is from one BSS in one extended service set (ESS) to another BSS within the same ESS and that minimizes the amount of time that data connectivity is lost between the STA and the distribution system (DS). fast basic service set (BSS) transition (FT) 4-Way Handshake: A pairwise key management protocol used during FT initial mobility domain association. This handshake confirms mutual possession of a pairwise master key, the PMK-R1, by two parties and distributes a group temporal key (GTK). fast basic service set (BSS) transition (FT) initial mobility domain association: The first association or first reassociation procedure within a mobility domain, during which a station (STA) indicates its intention to use the FT procedures. fast session transfer (FST): The transfer of a session from a channel to another channel, in the same or different frequency bands. The term “session” refers to non-physical layer state information kept by a pair of stations (STAs) that communicate directly (i.e., excludes forwarding). (11ad) fixed station (STA): A STA that is physically attached to a specific location. In licensed bands, a fixed STA might be authorized to operate only at a specific location. flexible multicast service (FMS): A service that enables a non-access-point (non-AP) station (STA) to request a multicast delivery interval longer than the delivery traffic indication map (DTIM) interval for the purposes of lengthening the period of time a STA can be in a power save state. flexible multicast stream identifier (FMSID): An identifier assigned by the access point (AP) to a particular group addressed stream subsequent to a successful FMS Request. flexible multicast service (FMS) stream: A succession of frames transmitted by the access point (AP) that correspond to a single flexible multicast stream identifier (FMSID). flexible multicast service (FMS) stream set: A collection of FMS streams identified by the value of the FMS Token field,(#1122) used during the FMS Request procedure. forwarding information: The information maintained by a mesh station (STA) that allows the mesh STA to perform its path selection and forwarding functions. fragmentation: The process of segmenting a medium access control (MAC) service data unit (MSDU) or MAC management protocol data unit (MMPDU) into a sequence of smaller MAC protocol data units (MPDUs) prior to transmission. The process of recombining a set of fragment MPDUs into an MSDU or MMPDU is known as defragmentation. These processes are described in 5.8.1.9 of ISO/IEC 7498-1:1994. frame: A unit of data exchanged between peer protocol entities.(#1179) Gaussian frequency shift keying (GFSK): A modulation scheme in which the data are first filtered by a Gaussian filter in the baseband and then modulated with a simple frequency modulation. group: The entities in a wireless network, e.g., an access point (AP) and its associated stations (STAs), or all the STAs in an independent basic service set (IBSS) network. group address: A medium access control (MAC) address that has the group bit equal to 1. Syn: multicast address. group addressed: When applied to a medium access control (MAC) service data unit (MSDU), it is an MSDU with a group address as the destination address (DA). NOTE-When applied to a MAC protocol data unit (MPDU), it is an MPDU with a group address in the Address 1 field. Syn: multicast. group (#1012)addressed quality-of-service management frame (GQMF): A group (#1012)addressed (#100)Management frame that is transmitted using the quality-of-service management frame (QMF) service.(11ae) Group Key Handshake: A group key management protocol defined by this standard. It is used only to issue a new group temporal key (GTK) to peers with whom the local station (STA) has already formed security associations. group master key (GMK): An auxiliary key that might be used to derive a group temporal key (GTK). group temporal key (GTK): A random value, assigned by the group source, which is used to protect group addressed medium access control (MAC) protocol data units (MPDUs) from that source. The GTK might be derived from a group master key (GMK). hidden station (STA): A STA whose transmissions are not detected using carrier sense (CS) by a second STA, but whose transmissions interfere with transmissions from the second STA to a third STA homogenous extended service set (ESS): A collection of basic service sets (BSSs), within the same extended service set (ESS), in which every subscription service provider network (SSPN) or other external network reachable at one BSS is reachable at all of them. hybrid coordination function (HCF): A coordination function that combines and enhances aspects of the contention-based and contention-free access methods to provide quality-of-service (QoS) stations (STAs) with prioritized and parameterized QoS access to the wireless medium (WM), while continuing to support non-QoS STAs for best-effort transfer. The HCF includes the functionality provided by both enhanced distributed channel access (EDCA) and HCF controlled channel access (HCCA). The HCF is compatible with the distributed coordination function (DCF) and the point coordination function (PCF). It supports a uniform set of frame formats and exchange sequences that STAs might use during both the contention period (CP) and the contention-free period (CFP). hybrid coordinator (HC): A type of coordinator, defined as part of the quality-of-service (QoS) facility, that implements the frame exchange sequences and medium access control (MAC) service data unit (MSDU) handling rules defined by the hybrid coordination function (HCF). The HC operates during both the contention period (CP) and contention-free period (CFP). The HC performs bandwidth management including the allocation of transmission opportunities (TXOPs) to QoS stations (STAs). The HC is collocated with a QoS access point (AP). hybrid coordination function (HCF) controlled channel access (HCCA): The channel access mechanism utilized by the hybrid coordinator (HC) to coordinate contention-free media use by quality-of-service (QoS) stations (STAs) for downlink individually addressed, uplink, and direct-link transmissions. idle power indicator (IPI): A physical layer (PHY) indication of the total channel power (noise and interference) as measured in the channel at the receiving antenna connector while the station (STA) is idle, i.e., neither transmitting nor receiving a frame. IEEE Std(#130) 802.1X authentication: Extensible Authentication Protocol (EAP) authentication transported by the IEEE Std(#130) 802.1X protocol. independent basic service set (IBSS): A basic service set (BSS) that forms a self-contained network, and in which no access to a distribution system (DS) is available. individual address: A medium access control (MAC) address in which the group bit is 0. Syn: directed address, unicast address. individually addressed: When applied to a medium access control (MAC) service data unit (MSDU), it is an MSDU with an individual address as the destination address (DA). NOTE--When applied to a MAC protocol data unit (MPDU), it is an MPDU with an individual address in the Address 1 field. Syn: directed, unicast. individually addressed quality-of-service management frame (IQMF): An individually addressed (#100)Management frame that is transmitted using the quality-of-service management frame (QMF) service.(11ae) infrastructure: The infrastructure includes the distribution system medium (DSM), access point (AP), and portal entities. It is also the logical location of distribution and integration service functions of an extended service set (ESS). An infrastructure contains one or more APs and zero or more portals in addition to the distribution system (DS). infrastructure authorization information: The information that specifies the access rights of the user of a non-access-point (non-AP) station (STA). This information might include the rules for routing the user traffic, a set of permissions about services that a user is allowed to access, quality-of-service (QoS) configuration information, or the accounting policy to be applied by the infrastructure. integration service: The service that enables delivery of medium access control (MAC) service data units (MSDUs) between the distribution system (DS) and a local area network (LAN) (via a portal). integrity GTK (IGTK): A random value, assigned by the broadcast/multicast source STA, which is used to protect group addressed medium access control (MAC) management protocol data units (MMPDUs) from that source STA. light sleep mode: A mesh power mode in which the mesh station (STA) operates either in the Awake state or in the Doze state towards a neighbor mesh STA, and is expected to receive beacons from this neighbor peer mesh STA. link: In the context of an IEEE Std(#130) 802.11 medium access control (MAC) entity, a physical path consisting of exactly one traversal of the wireless medium (WM) that is used to transfer an MAC service data unit (MSDU) between two stations (STAs). link margin: Ratio of the received signal power to the minimum required by the station (STA). The STA might incorporate rate information and channel conditions, including interference, into its computation of link margin. The specific algorithm for computing the link margin is implementation dependent. link metric: A criterion used to characterize the performance, quality, and eligibility of a link. little endian: The concept that, for a given multi-octet numeric representation, the least significant octet has the lowest address. liveness: A demonstration that the peer is actually participating in this instance of communication. location configuration information (LCI): As defined in IETF RFC 6225(#1692): includes latitude, longitude, and altitude, with resolution indicators for each. location subject local: The term used when a location request is for the location of the requesting STA, i.e., when the requesting STA asks, “Where am I?” location subject remote: The term used when a location request is for the location of the reporting STA, i.e., when the requesting STA asks, “Where are you?” location subject third party: The term used when the location request is for the location of a station (STA) other than the requesting STA or the requested STA, (i.e., when the requesting STA asks, “Where is he/she?”) master session key (MSK): Keying material that is derived between the Extensible Authentication Protocol (EAP) peer and exported by the EAP method to the Authentication Server (AS). NOTE 6(#1101)—In this standard, this key is at least 64 octets in length. medium access control (MAC) frame: The unit of data exchanged between MAC entities. Syn: MPDU. NOTE-References to a “frame” from within the clauses describing the MAC are implicitly references to a MAC frame, unless otherwise qualified.(#1179) medium access control (MAC) management protocol data unit (MMPDU): The unit of data exchanged between two peer MAC entities, using services of the physical layer (PHY), to implement the MAC management protocol. medium access control (MAC) protocol data unit (MPDU): The unit of data exchanged between two peer MAC entities using the services of the physical layer (PHY). Syn: MAC(#1179) frame. medium access control (MAC) service data unit (MSDU): Information that is delivered as a unit between MAC service access points (SAPs). mesh basic service set (MBSS): A basic service set (BSS) that forms a self-contained network of mesh stations (STAs) that use the same mesh profile. An MBSS contains zero or more mesh gates, and can be formed from mesh STAs that are not in direct communication. mesh facility: The set of enhanced functions, channel access rules, frame formats, mutual authentication methods, and managed objects used to provide data transfer among autonomously operating stations (STAs) that might not be in direct communication with each other over a single instance of the wireless medium. Communication between STAs using the mesh facility takes place using only the wireless medium. The mesh facility transports an MSDU between source and destination STAs over potentially multiple hops of the wireless medium without transiting the MAC_SAP at intermediate STAs. mesh gate: Any entity that has mesh station (STA) functionality and provides access to one or more distribution systems, via the wireless medium (WM) for the mesh basic service set (MBSS). mesh link: A link from one mesh station (STA) to a neighbor mesh STA that have a mesh peering with each other. mesh neighborhood: The set of all neighbor mesh stations (STAs) relative to a particular mesh STA. mesh path: A concatenated set of mesh links from a source mesh station (STA) to a destination mesh STA. mesh path selection: The process of selecting a mesh path. mesh peer service period (MPSP): A contiguous period of time during which one or more individually addressed frames are transmitted between two peer mesh stations (STAs) with at least one of those mesh STAs operating in light sleep or deep sleep mode. A mesh peer service period is directional and may contain one or more transmission opportunities (TXOPs). A mesh STA may have multiple mesh peer service periods ongoing in parallel. No more than one mesh peer service period may be set up in each direction with each peer mesh STA. mesh peer service period (MPSP) owner: A mesh station (STA) that obtains transmission opportunities (TXOPs), transmits individually addressed frames to the recipient mesh STA in the mesh peer service period, and terminates the mesh peer service period. mesh peering: A relationship between two mesh stations (STAs) that is required for direct communication over a single instance of the wireless medium (WM). A mesh peering is established with a mesh peering protocol. mesh peering management: A group of protocols to facilitate the mesh peering establishment and closure of the mesh peerings. mesh power mode: The activity level identifier of a mesh station (STA) set per mesh peering or for nonpeer neighbor STAs. A lower activity level enables a mesh STA to reduce its power consumption. mesh power mode tracking: Operation to observe the peering-specific mesh power modes from the peer mesh STAs and to maintain the peering-specific mesh power modes for each peer mesh STA. mesh profile: A set of values of parameters that identifies the attributes of the mesh basic service set (MBSS) and that is used in a single (#1180)MBSS. NOTE-The mesh profile consists of the identifiers that are the values for the parameters: mesh ID, active path selection protocol, active path selection metric, congestion control mode, synchronization method, and authentication protocol. mesh services: The set of services that enable the creation and operation of a mesh basic service set (MBSS). mesh station (STA): A quality-of-service (QoS) STA that implements the mesh facility. message integrity code (MIC): A value generated by a cryptographic function. If the input data are changed, a new value cannot be correctly computed without knowledge of the cryptographic key(s) used by the cryptographic function. NOTE 7(#1101)—This is traditionally called a -message authentication code (MAC), but the acronym MAC is already reserved for another meaning in this standard. mobile station (STA): A type of STA that uses network communications while in motion. mobility domain: A set of basic service sets (BSSs), within the same extended service set (ESS), that support fast BSS transitions between themselves and that are identified by the set’s mobility domain identifier (MDID). mobility domain identifier (MDID): An identifier that names a mobility domain. multi-level precedence and preemption (MLPP): A framework used with admission control for the treatment of traffic streams based on precedence, which supports the preemption of an active traffic stream by a higher precedence traffic stream when resources are limited. Preemption is the act of forcibly removing a traffic stream in progress in order to free up resources for another higher precedence traffic stream. multicast: See: group addressed. multicast address: See: group address. multicast-group address: A medium access control (MAC) address associated by higher level convention with a group of logically related stations (STAs). multiple BSSID capability: The capability to advertise information for multiple basic service set identifiers (BSSIDs) using a single Beacon or Probe Response frame instead of using multiple Beacon or Probe Response frames, each corresponding to a single BSSID, and the capability to indicate buffered frames for these multiple BSSIDs using a single traffic indication map (TIM) element in a single Beacon. multiple input, multiple output (MIMO): A physical layer (PHY) configuration in which both transmitter and receiver use multiple antennas. multiple medium access control (MAC) station management entity (SME) (MM-SME): Component of station management that manages multiple cooperating stations (STAs). (11ad) neighbor access point (AP): Any AP that is a potential service set transition candidate. neighbor station (STA): A STA in the following relationship: STA A is a neighbor to STA B if STA A can both directly transmit to and receive from STA B over the wireless medium. network access identifier (NAI): The user identity submitted by the Supplicant during IEEE Std(#130) 802.1X authentication. NOTE 8(#1101)—See IETF RFC 4282. network access server (NAS) client: The client component of a NAS that communicates with the Authentication Server (AS). network allocation vector (NAV): An indicator, maintained by each station (STA), of time periods when transmission onto the wireless medium (WM) is not initiated by the STA regardless of whether the STA’s clear channel assessment (CCA) function senses that the WM is busy. next-hop mesh station (STA): The next peer mesh STA on the mesh path to the destination mesh STA. nonce: A numerical value, used in cryptographic operations associated with a given cryptographic key, that is not to be reused with that key, including over all reinitializations of the system through all time. non-access-point (non-AP) station (STA): A STA that is not contained within an AP. nonoperating channel: A channel that is not the operating channel of the basic service set (BSS) of which the station (STA) is a member. nonpeer mesh power mode: The activity level identifier of a mesh station (STA) towards nonpeer neighbor mesh STAs. Two nonpeer mesh power modes are defined: active mode and deep sleep mode. non-personal basic service set control point (non-PCP) station (STA): A STA that is not a PCP. (11ad) non-personal basic service set control point (non-PCP)/non-access point (non-AP) station (STA): A STA that is not a PCP and that is not an AP. (11ad) non-quality-of-service (non-QoS) access point (AP): An AP that does not support the quality-of-service (QoS) facility. non-quality-of-service (non-QoS) basic service set (BSS): A BSS that does not support the quality-of-service (QoS) facility. non-quality-of-service management frame (non-QMF) access point (AP): An AP that does not implement the quality-of-service management frame (QMF) service.(11ae) non-quality-of-service management frame (non-QMF) station (STA): A STA that does not implement the quality-of-service management frame (QMF) service.(11ae) non-quality-of-service (non-QoS) station (STA): A STA that does not support the quality-of-service (QoS) facility. nontransmitted BSSID: A basic service set identifier (BSSID) corresponding to one of the basic service sets (BSSs) when the multiple BSSID capability is supported, where the BSSID is not announced explicitly but can be derived from the information encoded in the transmitted (#1565)Beacon frames. null data packet (NDP): A physical layer (#61)(PHY) protocol data unit (PPDU) that carries no Data field. off-channel: Channel that is not the base channel. operating channel: The operating channel is the channel in which beacons are transmited. NOTE—In IEEE Std 802.11, the the serving AP ofan infrastructure BSS or the DFS owner of an IBSS transmits beacons. operating channel width: The channel width in which the station (STA) is currently able to receive. overlapping basic service set (OBSS): A basic service set (BSS) operating on the same channel as the station’s (STA’s) BSS and within (either partly or wholly) its basic service area (BSA). over-the-air fast basic service set (BSS) transition (FT): An FT method in which the station (STA) communicates over a WM link to the target access point (AP). over-the-DS (distribution system) fast basic service set (BSS) transition (FT): An FT method in which the station (STA) communicates with the target access point (AP) via the current AP. pairwise: Referring to, or an attribute of, two entities that are associated with each other, e.g., an access point (AP) and an associated station (STA), or two STAs in an independent basic service set (IBSS) network. This term is used to refer to a type of encryption key hierarchy pertaining to keys shared by only two entities. pairwise master key (PMK): The key derived from a key generated by an Extensible Authentication Protocol (EAP) method or obtained directly from a preshared key (PSK). pairwise master key R0 (PMK-R0): The key at the first level of the fast basic service set (BSS) transition (FT) key hierarchy. pairwise master key (PMK) R0 name (PMKR0Name): An identifier that names the PMK-R0. pairwise master key (PMK) R0 key holder identifier (R0KH-ID): An identifier that names the holder of the PMK-R0 in the Authenticator. pairwise master key R1 (PMK-R1): A key at the second level of the fast basic service set (BSS) transition (FT) key hierarchy. pairwise master key (PMK) R1 name (PMKR1Name): An identifier that names a PMK-R1. pairwise master key (PMK) R1 key holder identifier (R1KH-ID): An identifier that names the holder of a PMK-R1 in the Authenticator. pairwise master key (PMK) S0 key holder identifier (S0KH-ID): An identifier that names the holder of the PMK-R0 in the Supplicant. pairwise master key (PMK) S1 key holder identifier (S1KH-ID): An identifier that names the holder of the PMK-R1 in the Supplicant. pairwise transient key (PTK): A concatenation of session keys derived from the pairwise master key (PMK) or from the PMK-R1. Its components include a key confirmation key (KCK), a key encryption key (KEK), and one or more temporal keys that are used to protect information exchanged over the link. pairwise transient key (PTK) name (PTKName): An identifier that names the PTK. parameterized quality of service (QoS): The treatment of the medium access control (MAC) protocol data units (MPDUs) depends on the parameters associated with the MPDU. Parameterized QoS is primarily provided through the hybrid coordination function (HCF) controlled channel access (HCCA) mechanism, but is also provided by the enhanced distributed channel access (EDCA) mechanism when used with a traffic specification (TSPEC) for admission control. pass-phrase: A secret text string employed to corroborate the user’s identity. password: A shared, secret, and potentially low-entropy word, phrase, code, or key used as a credential for authentication purposes. NOTE 9(#1101)—The method of distribution of a password to the units in the system is outside the scope of this standard. path metric: An aggregate multi-hop criterion used to characterize the performance, quality, and eligibility of a mesh path. peer mesh station (STA): A mesh STA to which a mesh peering has been established. peer trigger frame: A Mesh Data or quality-of-service (QoS) Null frame that initiates a mesh peer service period. peer-specific mesh power mode: The activity level identifier of a mesh station (STA) set per mesh peering. Three peer-specific mesh power modes are defined: active mode, light sleep mode, and deep sleep mode. peer-to-peer link: A direct link within a quality-of-service (QoS) basic service set (BSS), a tunnelled direct-link setup (TDLS) link, or a STA-to-STA communication in an independent basic service set (IBSS). peer-to-peer traffic specification (PTP TSPEC): The quality-of-service (QoS) characteristics of a data flow between non-access point (non-AP) QoS stations (STAs). (11ad) PeerKey Handshake: A key management protocol composed of the station-to-station link (STSL) master key (SMK) Handshake and the 4-Way STSL transient key (STK) Handshake. This is used to create new SMK security associations (SMKSAs) and STK security associations (STKSAs) to secure the STSLs. per-frame encryption key: A unique encryption key constructed for each medium access control (MAC) protocol data unit (MPDU). NOTE 10(#1101)—A per-frame encryption key is employed by some security protocols defined in this standard. personal basic service set (PBSS): A directional multi-gigabit (DMG) basic service set (BSS) that includes one PBSS control point (PCP), and in which access to a distribution system (DS) is not present but an intra-PBSS forwarding service is optionally present. (11ad) personal basic service set (PBSS) control point (PCP): An entity that contains one station (STA) and coordinates access to the wireless medium (WM) by STAs that are members of a PBSS. (11ad) personal basic service set (PBSS) control point (PCP)/access point (AP): A station (STA) that is at least one of a PCP or an AP.(11ad) physical layer (PHY) frame: The unit of data exchanged between PHY entities. Syn: PPDU.(#1179) NOTE-References to a "frame" from within the clauses describing the PHYs are implicitly references to a PHY frame, unless otherwise qualified. piggyback: The overloading of (#100)data with an acknowledgment of a previously received frame to the station (STA) to which the data is directed. point coordinator (PC): The entity within the STA in an AP that performs the point coordination function. point coordination function (PCF): A class of possible coordination functions in which the coordination function logic is active in only one station (STA) in a basic service set (BSS) at any given time that the network is in operation. portable station (STA): A type of station (STA) that might be moved from location to location, but that only uses network communications while at a fixed location. portal: The logical point at which the integration service is provided. precursor mesh station (STA): A neighbor peer mesh STA on the mesh path to the destination mesh STA, that identifies the mesh STA as the next-hop mesh STA. preshared key (PSK): A static key that is distributed to the units in the system by some out-of-band means. primary channel: The common channel of operation for all stations (STAs) that are members of the basic service set (BSS). prioritized quality of service (QoS): The provisioning of service in which the medium access control (MAC) protocol data units (MPDUs) with higher priority are given a preferential treatment over MPDUs with a lower priority. NOTE--Prioritized QoS is provided through the enhanced distributed channel access (EDCA) mechanism. protection mechanism: Any procedure that attempts to update the network allocation vector (NAV) of all receiving stations (STAs) prior to the transmission of a frame that might or might not be detected as valid network activity by the physical layer (PHY)(#1187) entities at those receiving STAs. protection mechanism frame: Any frame that is sent as part of a protection mechanism -procedure. protocol instance: An execution of a particular protocol that consists of the state of the communicating parties as well as the messages exchanged. proxy mesh gate: A mesh gate acting as an intermediary for IEEE 802 stations (STAs) outside the mesh basic service set (MBSS). pseudorandom function (PRF): A function that hashes various inputs to derive a pseudorandom value. In order to ensure liveness of a communication in which a pseudorandom value is used, a nonce is used as one of the inputs to the function. public safety answering point (PSAP): A physical location where emergency calls are received and routed to the appropriate emergency service dispatch center. NOTE 11(#1101)—See NENA 08-002 [B53]. quadrature binary phase shift keying (QBPSK): A binary phase shift keying modulation in which the binary data is mapped onto the imaginary (Q) axis. quality-of-service (QoS) access point (AP): An AP that supports the QoS facility. NOTE- In IEEE Std 802.11, the functions of a QoS AP are a superset of the functions of a non-QoS AP, and thus a QoS AP is able to function as a non-QoS AP to non-QoS stations (STAs). quality-of-service (QoS) basic service set (BSS): A BSS that provides the QoS facility. An infrastructure QoS BSS contains a QoS access point (AP). quality-of-service (QoS) facility: The set of enhanced functions, channel access rules, frame formats, frame exchange sequences and managed objects used to provide QoS. quality-of-service (QoS) independent basic service set (IBSS): An IBSS in which one or more of its stations (STAs) support the QoS facility. quality-of-service management frame (QMF): A (#100)Management frame that is transmitted using the QMF service.(11ae) quality-of-service management frame (QMF) access point (AP): A quality-of-service AP that implements the QMF service.(11ae) quality-of-service management frame (QMF) policy: A policy defining the access category of (#100)Management frames. QMF stations (STAs) transmit their (#100)Management frames using the access category defined by the policy.(11ae) quality-of-service management frame (QMF) service: A service in which the enhanced distributed channel access (EDCA) access category with which a (#100)Management frame is sent is determined according to a configured policy.(11ae) quality-of-service management frame (QMF) station (STA): A quality-of-service STA that implements the QMF service.(11ae) quality-of-service (QoS) station (STA): A STA that implements the QoS facility. NOTE-A QoS STA acts as a non-QoS STA when associated in a non-QoS basic service set (BSS). reassociation service: The service that enables an established association [between access point (AP) and station (STA)] to be transferred from one AP to another (or the same) AP. receive chain: The physical entity that implements any necessary signal processing to provide the received signal to the digital baseband. Such signal processing includes filtering, amplification, down-conversion, and sampling. receive power: Mean power measured at the antenna connector. received channel power indicator (RCPI): An indication of the total channel power (signal, noise, and interference) of a received frame measured on the channel and at the antenna connector used to receive the frame. received power indicator (RPI): A quantized measure of the received power level as seen at the antenna connector. received signal to noise indicator (RSNI): An indication of the signal to noise plus interference ratio of a received frame. RSNI is defined by the ratio of the received signal power (RCPI-ANPI) to the noise plus interference power (ANPI) as measured on the channel and at the antenna connector used to receive the frame. NOTE 12(#1101)—RCPI and ANPI might not be measured simultaneously; see 10.11.9.4 (Noise Histogram report) for details. registered station (STA): A STA for which information needs to be submitted to an appropriate regulatory or coordination authority before it is allowed to transmit. remote request broker (RRB): The component of the station management entity (SME) of an access point (AP) that supports fast basic service set (BSS) transitions over the distribution system (DS). resource information container (RIC): A sequence of elements that include resource request and response parameters. restricted channel: A radio channel in which transmission is restricted to stations (STAs) that operate under the control of licensed operators. roaming consortium: A group of subscription service providers (SSPs) having inter-SSP roaming agreements. (#1181)robust Management frame: A (#100)Management frame that is eligible for protection. scheduled service period (SP): The SP that is scheduled by the quality-of-service (QoS) access point (AP) or the personal basic service set (PBSS) control point (PCP)(11ad). Scheduled SPs start at fixed intervals of time. sector: A transmit or receive antenna pattern corresponding to a Sector ID. (11ad) service interval (SI): The interval between the start of two successive scheduled service -periods (SPs). service period (SP): A contiguous time during which one or more downlink individually addressed frames are transmitted to a quality-of-service (QoS) station (STA) and/or one or more transmission opportunities (TXOPs) are granted to the same STA. SPs are either scheduled or unscheduled. NOTE—A non-access-point (non-AP) STA can have at most one nongroupcast with retries SP (non-GCR-SP)(11aa)(#1124) active at any time.(#1033) service set transition: A STA movement from one BSS to another BSS, i.e., either a BSS transition or an ESS transition. serving AP: The AP to which the STA is associated. sounding: The use of preamble training fields to measure the channel for purposes other than demodulation of the Data portion of the physical layer (#61)(PHY) protocol data unit (PPDU) containing the training fields. NOTE 13(#1101)—These uses include calculation of transmit steering, calculation of recommended MCS, and calculation of calibration parameters. source mesh station (STA): A mesh STA from which a MAC service data unit (MSDU) enters the mesh basic service set (MBSS). A source mesh STA may be a mesh STA that is the source of an MSDU or a proxy mesh gate that receives an MSDU from a STA outside of the MBSS and forwards the MSDU on a mesh path. space-time block coding/spatial multiplexing (STBC/SM): A combination of STBC and SM where one spatial stream is transmitted using STBC and one or two additional spatial streams are transmitted using SM. space-time streams: Streams of modulation symbols created by applying a combination of spatial and temporal processing to one or more spatial streams of modulation symbols. spatial multiplexing (SM): A transmission technique in which data streams are transmitted on multiple spatial channels that are provided through the use of multiple antennas at the transmitter and the receiver. spatial sharing (SPSH): Use of a frequency channel by multiple stations (STAs) located in the same vicinity, and whose directional transmissions may overlap in time. (11ad) spatial stream: One of several streams of bits or modulation symbols that might be transmitted over multiple spatial dimensions that are created by the use of multiple antennas at both ends of a communications link. station (STA): A logical entity that is a singly addressable instance of a medium access control (MAC) and physical layer (PHY) interface to the wireless medium (WM). station service (SS): The set of services that support transport of medium access control (MAC) service data units (MSDUs) between stations (STAs) within a basic service set (BSS). subscription service provider (SSP): An organization (operator) offering connection to network services, perhaps for a fee. subscription service provider network (SSPN): The network controlled by a subscription service provider (SSP). The network maintains user subscription information. Supplicant: An entity at one end of a point-to-point LAN segment that is being authenticated by an Authenticator attached to the other end of that link. (IEEE Std 802.1X-2010(M14)) Supplicant address (SPA): The medium access control (MAC) address of the IEEE Std(#130) 802.1X Supplicant. temporal encryption key: The portion of a pairwise transient key (PTK) or group temporal key (GTK) used directly or indirectly to encrypt data in medium access control (MAC) protocol data units (MPDUs). temporal key (TK): The combination of temporal encryption key and temporal message integrity code (MIC) key. temporal message integrity code (MIC) key: The portion of a transient key used to ensure the integrity of medium access control (MAC) service data units (MSDUs) or MAC protocol data units (MPDUs). time unit (TU): A measurement of time equal to 1024 µs. traffic category (TC): A label for medium access control (MAC) service data units (MSDUs) that have a distinct user priority (UP), as viewed by higher layer entities, relative to other MSDUs provided for delivery over the same link. Traffic categories are meaningful only to MAC entities that support quality of service (QoS) within the MAC data service. These MAC entities determine the UP for MSDUs belonging to a particular traffic category using the priority value provided with those MSDUs at the MAC service access point (MAC_SAP). traffic classification (TCLAS): The specification of certain parameter values to identify a Protocol Data Unit (PDU) or a medium access control (MAC) service data unit (MSDU). The classification process may be performed above the MAC service access point (MAC_SAP), within the MLME, or within the MAC, based on the type of classification.(#78) traffic filter: A set of traffic specifications defined by the use of traffic classification (TCLAS) elements that are utilized by the traffic filtering service (TFS) to identify specific allowed frames. traffic filtering service (TFS): A service provided by an access point (AP) to a non-AP station (STA) to reduce the number of frames sent to the non-AP STA by not forwarding individually addressed frames addressed to the non-AP STA that do not match traffic filters specified by the non-AP STA. traffic identifier (TID): Any of the identifiers usable by higher layer entities to distinguish medium access control (MAC) service data units (MSDUs) to MAC entities that support quality of service (QoS) within the MAC data service. NOTE--There are 16 possible TID values; eight identify TCs, and the other eight identify parameterized TSs. The TID is assigned to an MSDU in the layers above the MAC. traffic indication map (TIM) broadcast: A service that enables a non-access-point (non-AP) station (STA) to request periodic transmission of a TIM frame by the AP. TIM frames have shorter duration than Beacon frames and can be transmitted at a higher physical layer (PHY) rate, which allows the STA to save additional power while periodically checking for buffered traffic in standby mode, relative to the power consumed if the station (STA) were to periodically wake up to receive a (#1565)Beacon frame. traffic specification (TSPEC): The quality-of-service (QoS) characteristics of a data flow to and from a QoS station (STA). traffic stream (TS): A set of medium access control (MAC) service data units (MSDUs) to be delivered subject to the quality-of-service (QoS) parameter values provided to the MAC in a particular traffic specification (TSPEC). TSs are meaningful only to MAC entities that support QoS within the MAC data service. These MAC entities determine the TSPEC applicable for delivery of MSDUs belonging to a particular TS using the priority parameter provided with those MSDUs at the MAC service access point (MAC_SAP). traffic stream identifier (TSID): Any of the identifiers usable by higher layer entities to distinguish medium access control (MAC) service data units (MSDUs) to MAC entities for parameterized quality of service (QoS) [i.e., the traffic stream (TS) with a particular traffic specification (TSPEC)] within the MAC data service. The TSID is assigned to an MSDU in the layers above the MAC. transmission opportunity (TXOP): An interval of time when a particular quality-of-service (QoS) station (STA) has the right to initiate frame exchange sequences onto the wireless medium (WM). NOTE--A TXOP is defined by a starting time and a maximum duration. The TXOP is either obtained by the STA by successfully contending for the channel or assigned by the hybrid coordinator (HC). transmission opportunity (TXOP) holder: A quality-of-service (QoS) station (STA) that has either been granted a TXOP by the hybrid coordinator (HC) or successfully contended for a TXOP. transmission opportunity (TXOP) responder: A station (STA) that transmits a frame in response to a frame received from a TXOP holder during a frame exchange sequence, but that does not acquire a TXOP in the process. transmitted basic service set identifier (BSSID): The BSSID included in the MAC Header transmitter address field of a Beacon frame when the multiple BSSID capability is supported. tunneled direct-link setup (TDLS): A protocol that uses a specific Ethertype encapsulation to TDLS frames through an access point (AP) to establish a TDLS direct link. tunneled direct-link setup (TDLS) direct link: Direct link between two non-AP stations (STAs) that has been established using the TDLS protocol. tunneled direct-link setup (TDLS) initiator station (STA): A STA that transmits a TDLS Setup Request frame or a TDLS Discovery Request frame. tunneled direct-link setup (TDLS) peer power save mode (PSM): A power save mode that is based on periodically scheduled service periods, which can be used between two stations (STAs) that have set up a TDLS direct link. tunneled direct-link setup (TDLS) peer power save mode (PSM) initiator: A station (STA) that transmits a TDLS Peer PSM (#99)Request frame. tunneled direct-link setup (TDLS) peer power save mode (PSM) responder: A station (STA) that transmits a TDLS Peer PSM (#99)Response frame. tunneled direct-link setup (TDLS) peer station (STA): A STA with a TDLS direct link. tunneled direct-link setup (TDLS) peer unscheduled automatic power save delivery (U-APSD) [TDLS peer U-APSD (TPU)]: A power save mode based on unscheduled service periods that can be used between two stations (STAs) that have set up a TDLS direct link. tunneled direct-link setup (TDLS) peer unscheduled automatic power save delivery (U-APSD) [TDLS peer U-APSD (TPU)] buffer station (STA): A TDLS peer STA that buffers traffic for a TPU sleep STA. tunneled direct-link setup (TDLS) peer unscheduled automatic power save delivery (U-APSD) [TDLS peer U-APSD (TPU)] sleep station (STA): A TDLS STA that entered power save mode on a TDLS direct link and that is using TPU for the delivery of buffered traffic. tunneled direct-link setup (TDLS) power save mode (PSM): TDLS peer PSM or peer unscheduled automatic power save delivery (U-APSD). tunneled direct-link setup (TDLS) responder station (STA): A STA that receives or is the intended recipient of a TDLS Setup Request frame or TDLS Discovery Request frame. unauthorized disclosure: The process of making information available to unauthorized individuals, entities, or processes. unauthorized resource use: Use of a resource not consistent with the defined security policy. unicast: See: individually addressed. unicast address: See: individual address. uniform spreading: A regulatory requirement for a channel selection mechanism that provides uniform usage across a minimum set of channels in the regulatory domain. unquenchable flame: A flame that cannot be quenched. unreachable destination: A destination mesh station (STA) for which the link to the next hop of the mesh path to this destination mesh STA is no longer usable.(#1125) unscheduled service period (SP): The period that is started when a quality-of-service (QoS) station (STA) transmits a trigger frame to the QoS access point (AP). user priority (UP): A value associated with an medium access control (MAC) service data unit (MSDU) that indicates how the MSDU is to be handled. The UP is assigned to an MSDU in the layers above the MAC. validated AP: An AP that has either been explicitly configured as a Neighbor or learned through a mechanism like the Beacon (#1294)report. wildcard BSSID: A BSSID value used to represent all BSSIDs. NOTE—In IEEE Std. 802.11, this is represented by all binary 1s. wildcard SSID: A SSID value used to represent all SSIDs. NOTE—In IEEE Std. 802.11, this is represented by the value “null”. wireless distribution system (WDS): Often used as a vernacular term for a mechanism for wireless communication among nonmesh stations (STAs) using a four address frame format. NOTE 15(#1101)—This standard specifies such a frame format. (#301)This standard defines use of this frame format for a mesh basic service set (MBSS). wireless local area network (WLAN) system: A system that includes the distribution system (DS), access points (APs), and portal entities. It is also the logical location of distribution and integration service functions of an extended service set (ESS). A WLAN system contains one or more APs and zero or more portals in addition to the DS. wireless medium (WM): The medium used to implement the transfer of protocol data units (PDUs) between peer physical layer (PHY) entities of a wireless local area network (LAN).

## Comment 2099

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2099 |  |  |  | There are multiple forms of expression used when a MIB variable forms part of a condition. i.e.,* When dot11 (300) o Also note when dot11 ... is set to 1 should refer to the time it is set, not a condition. (102) o All of these uses appear to be correct* If dot11 (397)* For which dot11 (38)* In which dot11 (12)* With dot11 (84)* Has dot11 (29)* Have dot11 (12)I believe such variability is generally unnecessary, and creates confusion for submission authors. | Decide on a preferred syntax and change other uses to suit. |  | GEN |

Proposed resolution:

Rejected.

The comment does not indicate specific changes that would satisfy the commenter.

Note, in reply to the commenter, it was discovered after some research, that the forms in use are too diverse and inconsistent to be readily identified with simple search and replace operations.  That being the case, a fully marked up set of changes is needed.

# Clause 10 comments

| 2125 | 1357.16 | 10.1.3.4 |  | "network initialization" is a misleading term. We provide layers 1 and part of layer 2 of a communications protocol. "networking" is considered a layer 3 term. | Replace all "network initialization" with "the establishment of a BSS" modulo necessary syntactic changes. |
| --- | --- | --- | --- | --- | --- |

Discussion:

The term “network” does exist outside .11ad text,   the term “network initialization” is unique to .11ad text.  So the options are

- Make no changes

- Change all uses of  “network”

- Change all uses of  “network initialization”

I have limited the scope of the changes to the term introduced by .11ad.

Proposed changes:

Revised. Globally change “network initialization” to “establishment of a BSS”.

| 2126 | 1363.30 | 10.1.4.2.2 |  | "If at any time during the scan the DMG STA detects a non-DMG Beacon frame,..." -- "non-DMG Beacon frame" is ambiguous. Should be "frame that is not a DMG Beacon frame" | Replace cited text with: "If at any time during the scan the DMG STA detects a frame that is not a DMG Beacon frame," |
| --- | --- | --- | --- | --- | --- |

Context: 1363.30

If at any time during the scan the DMG STA detects a non-DMG Beacon frame, the DMG STA shall continue to scan the current channel until the scanning timer expires. After scanning one channel, the DMG STA shall initiate scanning in another channel if at least one channel within the ChannelList parameter has not yet been scanned.

Discussion:

This part of the logic is like the non-DMG case.  The receipt of any valid frame causes the scan to prolong.

The same is true here,  except that receipt of a DMG Beacon frame by itself doesn’t cause the scan on that channel to prolong.

Proposed Resolution:

Accepted.

| 2143 | 1405.44 | 10.2.6.3 |  | ""WS" is an abbreviation of Wakeup Schedule." True, that's what 3.3 says. However we generally don't abbreviate a "Wakeup Schedule element" to a "WS element". | Remove cited text. In Figure 10-10 replace any "WS element" by "Wakeup Schedule element" |
| --- | --- | --- | --- | --- | --- |

Context:

NOTE—Figure 10-10 (State Transition Diagram of PCP Power Management Mode) uses the following terminology: “As per T1” means that the transitions are specified in Table 10-3 (Power management statesfor an Awake BI), “As per T2” means that the transitions are specified in Table 10-4 (Power management states for a Doze BI), “WS” is an abbreviation of Wakeup Schedule.

Proposed resolution:

Accepted

# Approved Resolutions

| 2134 | 1377.10 |  | 10.2.2.5.2 |  | "If the non-APSTA wants to terminate use of all QoS services provided by an ADDTSRequest frame including U-APSD Coexistence, it may transmit a DELTS Request frame to the AP" - normative statement based on a STA's wants. | Reword so that the normative statement is based on observable conditions. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.   Replace cited text with:

“A non-AP STA may terminate use of all QoS services (including U-APSD Coexistence) resulting from an ADDTS Request frame by transmitting a DELTS Request frame to the AP”

| 2359 | 1385.50 | 10.2.2.14 |  | "contiguous" means either "sharing a border" or "near another entity". But what is a single time period contiguous with? | Replace "contiguous" with "continuous". |
| --- | --- | --- | --- | --- | --- |

Context: 1385.50

A TDLS Peer PSM service period is a contiguous period of time during which one or more individually addressed frames are transmitted between two TDLS peer STAs when atleast one STA employs TDLS Peer PSM.

Discussion:

Do we really need “continuous?”   Surely a non-continuous period of time is actually multiple periods of time.  And we mustn’t lose sight of the need to delete 300 pages before we can roll in .11ac.

Proposed Resolution:

Revised.  Delete “contiguous”.

| 2172 | 1398.11 | 10.2.3.5 |  | " the STA should retain the buffered BUs and attempt to transmit the ATIM during the next ATIM Window/Awake Window." -- if you read this literally, at the start of the next ATIM window it will transmit both the old (delayed) ATIM and shiny new one.One way to fix this is just to discard the ATIM. However that starts the backoff count from a random number and gives no "priority" to old ATIMs. Might be better to state in a) that new ATIMs are created, except where one is already queued for transmission. | Perhaps exclude destinations that already have an ATIM queued from the generation of new ATIMs in step a). |
| --- | --- | --- | --- | --- | --- |

Context: 1398.11

If a STA is unable to transmit an ATIM during the ATIM Window/Awake Window, for example due to contention with other STAs, the STA should retain the buffered BUs and attempt to transmit the ATIM during the next ATIM Window/Awake Window.

Discussion:

There are several aspects to this.  Certainly it makes no sense to have multiple ATIMs transmitted.  The logic at 1397.51 says:  “during the ATIM Window, the STA shall transmit an individually addressed ATIM frame to each STA for which it has one or more buffered individually addressed BUs”.   It doesn’t separate the act of queing and the act of transmission.

Also, such created ATIMs will have a new backoff.

If a STA fails to transmit an ATIM because its backoff counter does not reach zero,  then the ATIM should resume transmission in the next ATIM window with the backoff counter it had at the end of the previous ATIM window.

I think there are two models:

- Retain the previous ATIM and run a channel access attempt across multiple ATIM windows

- Create the ATIM afresh each ATIM window.

Unfortunately the text in this subclause hedges it bets and kind-of appeals to both.

I think the cleanest way to fix this is to leave the cited text alone,  and change the text at 1397.51 thus:

Following the reception or transmission of the Beacon frame in a non-DMG IBSS, during the ATIM Window, the STA shall queue for transmission an individually addressed ATIM frame to each STA for which it has one or more buffered individually addressed BUs and for which an ATIM is not already queued. Following the BTI or ATI in a DMG BSS, during the Awake Window, the STA shall queue for transmission an individually addressed ATIM management frame to each STA for which it has one or more buffered individually addressed BUs and for which an ATIM is not already queued. If the STA has one or more buffered group addressed MSDUs (excluding those with a service class of StrictlyOrdered) or has one or more buffered group addressed MMPDUs, it shall queue for transmission an appropriately addressed group addressed ATIM frame if such an ATIM is not already queued.

Change 1398.09 as follows:

If a STA is unable to transmit an ATIM during the ATIM Window/Awake Window, for example due to contention with other STAs, the STA should retain the buffered BUs and either discard the ATIM or attempt to transmit the ATIM during the next ATIM Window/Awake Window.

Proposed resolution:

Revised.  Change 1397.51 as shown in <this document> under CID 2172.

These changes avoid multiple queued ATIMs to the same address.

| 2130 | 1367.12 | 10.1.4.3.4 |  | "(set to 0)" - there is no need to say this | Removed cited text (twice) |
| --- | --- | --- | --- | --- | --- |

Context: 1367.08:

Proposed Resolution:

Accepted.

| 2132 | 1367.26 | 10.1.4.3.4 |  | "For the purpose of this MAC address comparison, the first transmitted octet shall be interpreted as the most significant octet (i.e., big endian)."My experience tells me that implementers will still get this wrong. | Refer to 11.6.1, which contains " For the purposes ofcomparison, the MAC address is encoded as 6 octets, taken to represent an unsigned binary number. Thefirst octet of the MAC address shall be used as the most significant octet. The bit numbering conventions in8.2.2 (Conventions) shall be used within each octet. This results in a 48-bit unsigned integer labelled B0(least significant) to B47 (most significant), where the I/G bit is in B40." |
| --- | --- | --- | --- | --- | --- |

Context: 1367.20

For each peer STA reported as part of an MLME-SCAN.confirm primitive or considered as part of a PCP handover, the STA proceeds as follows. If the STA’s value of self_PCP_factor is greater than the value of peer_PCP_factor or if the values are equal and the MAC address of the STA is greater than the MAC address of the peer STA contained in the peer STA’s DMG Capabilities element, the STA becomes a candidate PCP. Otherwise, the STA does not become a candidate PCP. For the purpose of this MAC address comparison, the first transmitted octet shall be interpreted as the most significant octet (i.e., big endian).

The text in 11.6.1 is referenced,  e.g. at 1718.20 for the purpose of comparing MAC addresses:

The IEEE Std 802.1X reauthentication timers in each STA are independent. If the reauthentication timer of the STA with the higher MAC address (see 11.6.1 (Key hierarchy) for MAC comparison) triggers the reauthentication via its Authenticator, its Supplicant shall send an EAPOL-Start frame tothe authenticator of the STA with the lower MAC address (see 11.6.1 (Key hierarchy) for MAC comparison) to trigger reauthentication on the other STA. This process keeps the pair of STAs in a consistent state with respect to derivation of fresh temporal keys upon an IEEE Std 802.1X reauthentication.

Proposed changes:

For each peer STA reported as part of an MLME-SCAN.confirm primitive or considered as part of a PCP handover, the STA proceeds as follows. If the STA’s value of self_PCP_factor is greater than the value of peer_PCP_factor or if the values are equal and the MAC address of the STA is greater than (see 11.6.1 (Key hierarchy) for MAC address comparison) the MAC address of the peer STA contained in the peer STA’s DMG Capabilities element, the STA becomes a candidate PCP. Otherwise, the STA does not become a candidate PCP.

Proposed Resolution:

Revised.  Delete cited sentence.  At 1367.22 insert “(see 11.6.1 for MAC address comparison)” after “greater than”.

| 2356 | 1376.37 | 10.2.2.5.2 |  | "may not" is ambiguous -- does it mean "might not" or "is not permitted" ("shall not")? | Both on this line and line 38 replace "may" with "might". |
| --- | --- | --- | --- | --- | --- |

Context: 1376.35

10.2.2.5.2 U-APSD Coexistence A non-AP STA that uses U-APSD may not be able to receive all AP transmitted frames during the service period due to interference observed at the non-AP STA. Although the AP may not observe the same interference, it is able to determine that the frames were not received correctly by the non-AP STA. The UAPSD Coexistence capability enables the non-AP STA to indicate a requested transmission duration to the AP for use during U-APSD service periods.

Proposed Resolution:

Accepted

Context: 1154.11

Before network initialization (see 10.1.3.5 (Beacon generation in an IBSS)), the value of the TSF timer is delivered by DMG Beacon frames generated at each BTI.

Discussion:

The correct reference should be to 10.1.3.4.  That aside,  the scope of the subclause is infrastructure and PBSS networks.  “Before network initialization” is outside this scope.

Proposed resolution:

Revised.  Delete cited sentence.

| 2117 | 1357.12 | 10.1.3.3.3 |  | "The DMG AP shall assert the dot11MaxLostBeacons attribute value" - assertive, but not informative? | Replace it with something meaningful |
| --- | --- | --- | --- | --- | --- |

Context:

The DMG AP shall assert the dot11MaxLostBeacons attribute value equal to the aMinBTIPeriod parameter value.

Proposed resolution:

Revised.   Replace cited sentence with:

“A DMG AP shall set dot11MaxLostBeacons to the value of the aMinBTIPeriod parameter.”

| 2446 | 846.32 | 8.4.2.87 | Table 8-185 is mis-labeled. | Change the Table heading to "DMS Request Type field" (Change "SCS" to "DMS" and change capitalization) |
| --- | --- | --- | --- | --- |

Context:

Proposed Resolution:

Accepted

| 2469 | 866.21 | 8.4.2.101 | The mesh peering protocol identifier field is indicated as being 2 octets in length in the diagram of the element format, but the table that describes the encoding for this field stops at 255 - so is the field 1 octet or 2 octets? | Fix the discrepancy between the format figure and the table values. |
| --- | --- | --- | --- | --- |

Context:

I asked Kaz for comment.  He confirms that 1 octet was the original intention, but the linux implementation (https://github.com/torvalds/linux/blob/master/net/mac80211/mesh_plink.c#L344) uses 2 octets.  So we have to stick with 2 octets.

Proposed Resolution:

Revised.

At 866.24 insert a row: “256-65535 / Reserved”

| 2032 | 898.02 | 8.4.2.128 | Figure 8-469 has a blank field name | Insert "MinPPDuration" |
| --- | --- | --- | --- | --- |

Discussion:

The proposed field name is the only one in the text following the figure that is not present in the figure,  so the change has to be correct.

Proposed Resolution:

Accepted.

| 2033 | 901.07 | 8.4.2.129 | Mixing bit fields and integers in the same field is a recipe for confusion, as one is represented least significant on the left, and the other least significant on the right. | Describe structure of subfield using a bit-oriented figure. |
| --- | --- | --- | --- | --- |

Context: 901.07

For the MIDC with multiplesector ID (MID) subphase only, indicates the number of sectors that the responder/initiator transmits during the MID subphase. Otherwise, indicates the number of beams in the Sector ID Order subfield for the MIDC subphase with the direction and the TX/RX antenna identification. The 1st bit is set to 0 for the initiator link and to 1 for the responder link. The 2nd bit is set to 0 for the transmitter antenna and to 1 for the receiver antenna. The 3rd to 5th bits represent the number of beams for beam combining. E.g., “01101” stands for Nbeam(I, RX)= 5.

Discussion:

Assaf Kasher (one of the .11ad authors) indicates that this field is not used in the MID suphase.  So we can simply restructure it to indicate

Proposed change.

Change Figure 8-472 as follows:

| … | B46 | B47 | B48 | B49 B51 |
| --- | --- | --- | --- | --- |
| … | Sector ID Order Present | Link Type | Antenna Type | Number of Beams |
| … | 1 | 1 | 1 | 3 |

Change Table 8-209 as shown (only last existing row is shown):

| Field | Meaning |
| --- | --- |
| … | … |
| Link Type | Set to 0 for the initiator link and to 1 for the responder link |
| Antenna Type | Set to 0 for the transmitter antenna and to 1 for the receiver antenna |
| Number of Beams | Indicates the number of beams in the Sector ID Order subfield for the MIDC subphase. |

At 1322.62, after “… is carried out.”  Insert a new paragraph:

“In 9.36.6.3 the following terminology is used:

- Nbeam:  the value of the Number of Beams subfield of the FBCK-TYPE field

- Nbeam(Link Type,Antenna Type):  represents a combination of Nbeam, Link Type (I for Initiator, R for Responder), and Antenna Type (TX for Transmitter, RX for Receiver)”

Replace all “Nbeam subfield” with “Number of Beams subfield”.

Proposed resolution:

Revised.  Make changes in <this-document> under CID 2033.

| 2034 | 903.21 | 8.4.2.131 | "The broadcast AID asserted in the Source AID and the Destination AID fields for an SP allocation indicates that during the SP a non-PCP/non-AP STA does not transmit unless it receives a Poll or Grant frame from the PCP/AP"What does "asserted" mean here? | Rewrite as English. |
| --- | --- | --- | --- | --- |

Discussion:

This is getting perilously close to behavioural description “does not transmit”.

Proposed change:

If the Source AID and the Destination AID fields for an SP allocation contain the broadcast AID, then during the SP a non-PCP/non-AP STA does not transmit unless it receives a Poll or Grant frame from the PCP/AP.

Proposed Resolution:

Revised. Make changes in <this-document> under CID 2034.   These changes clarify the language at the cited location.

| 2338 | 903.29 | 8.4.2.131 | "contiguous" means either "sharing a border" or "near another entity". What border is shared or other entities near a time block? | Replace "contiguous" with "continuous". |
| --- | --- | --- | --- | --- |

Context: 903.29

The Allocation Block Duration field indicates the duration, in microseconds, of a contiguous time block for which the SP or CBAP allocation is made and cannot cross beacon interval boundaries. Possible values range from 1 to 32 767 for an SP allocation and 1 to 65 535 for a CBAP allocation.

Proposed resolution:

Revised.  Change “contiguous time block” to “period of time”

| 2339 | 909.48 | 8.4.2.135 | "contiguous" means either "sharing a border" or "near another entity". What border is shared or other entities near a bunch of time samples that are separated from each other by Tc? | Replace "contiguous" with "consecutive". |
| --- | --- | --- | --- | --- |

Context: 909.46

Each channel measurement contains Ntaps channel impulse taps. The channel impulse response reported for all Nmeasmeasurements correspond to a common set of relative tap delays. If the Tap Delay subfield is not present, then the Ntaps channel taps is interpreted as contiguous time samples, separated by Tc. The delay values in the Tap Delay subfield, when present, correspond to the strongest taps and are unsigned integers, in increments of Tc, starting from 0.Each channel tap is reported as an in-phase and quadrature component pair, with each component value represented as a twos complement number between –128 and 127. Unless all in-phase and quadrature component values are reported as zero, they are scaled such that the two most significant bits for at least one of the component values equal 01 or 10 (binary).

Proposed Resolution:

Accepted.

| 2035 | 918.09 | 8.4.2.144 | "NOTE--A STA that is operating in a band/channel is not required tobe continuously in the Awake state on that band/channel." -- I don't understand why this note is needed, and even more so, I don't understand why it's needed in Clause 8. | Delete cited note. |
| --- | --- | --- | --- | --- |

Context: 918.06:

The Operation subfield is set to 1 to indicate that the STA is operating in the band indicated within the Band ID subfield. Otherwise, it is set to 0. Other values are reserved. NOTE—A STA that is operating in a band/channel is not required to be continuously in the Awake state on that band/channel.

Proposed Resolution:

Accepted

| 2036 | 918.58 | 8.4.2.146 | The sizeof() operator is not defined. | Restructure equation to use short variable names for each of the sizeof terms. In the variable list describe these variables as the "size of the field in octets". |
| --- | --- | --- | --- | --- |

Context: 918.57:

The value of n in Figure 8-498 (Cluster Report element format) is equal to 255 – (2 + sizeof(Reported BSSID field) + sizeof(Reference Timestamp field) + sizeof(Clustering Control field) + sizeof(ECPAC Policy Element field) + sizeof(TSCONST field)).

Proposed resolution:

Accepted

| 2037 | 919.09 | 8.4.2.146 | "Traffic Scheduling Constraint (TSCONST)"So what is the name of the field?1. The redundant "Traffic Scheduling Constraint (TSCONST)"2. "Traffic Scheduling Constraint"3. "TSCONST" | Change name of field so that it doesn't include an embedded abbreviation of itself. Check all references use this form. |
| --- | --- | --- | --- | --- |

Discussion:

There is need to abbreviate the name of the field,  so recommend using the full form to refer to the name of the field and leave TSCONST as part of the name of its subfields.

See 906.37:

The Number of Constraints field indicates the number of TSCONST fields contained in the element. The value of this field ranges from 0 to 15. Other values are reserved. The Traffic Scheduling Constraint (TSCONST) field contains one or more Constraint subfields as illustrated in Figure 8-481 (Traffic Scheduling Constraint field format).

This contains a logical error.   The Number of Constraints field must indicate the number of Constraint subfields,  not the number of TSCONST fields.   The point is that the latter are of variable length and have no way to delimit between them.   There is also no semantics associated with several TSCONST fields.

This error highlights the poor naming of the “Traffic Scheduling Constraint”, because it actually represents a set of constraints.  I asked the original author of this text (Carlos Cordeiro) for comment.  He said:

… Also, please consider changing the name of the field from “Traffic Scheduling Constraint” to “Traffic Scheduling Constraint Set”. This would make its intent much clearer.

Also, there is very little value in trying to define the “format” of the Traffic Scheduling Constraint [Set] field in a separate figure.  Instead whenever a Traffic Scheduling Constraint [Set] field is defined,  it should be described as “containing Constraint subfields as defined in figure 8-482.

Also, at 907.27:

The Interferer MAC Address field is set to the value of the TA field within a frame received during the interval of time indicated by this TSCONST field. If the value is unknown, the Interferer MAC Address field is set to the broadcast MAC address.

I assume that “TSCONST” should be “Constraint”,  because each Constraint has its own notion of an interval of time.

Proposed changes:

At 906.41:

The Traffic Scheduling Constraint field contains one or more Constraint subfields (see Figure 8-482).

At 919.09:

Delete “(TSCONST)”

At 920.18:

The Traffic Scheduling Constraint field contains one or more Constraint subfields as defined in 8.4.2.133 (DMG TSPEC element) and specifies periods of time with respect to the TBTT of the beacon interval of the BSS the STA participates where the STA experiences poor channel conditions, such as due to interference.

At 904.41

Change “TSCONST” to “Traffic Scheduling Constraint”

At 906.37:

The Number of Constraints field indicates the number of Constraint subfields contained in the element. The value of this field ranges from 0 to 15. Other values are reserved.

Delete 906.41 to 50 (including the figure):

At 907.27:

The Interferer MAC Address field is set to the value of the TA field within a frame received during the interval of time indicated by this Constraint subfield. If the value is unknown, the Interferer MAC Address field is set to the broadcast MAC address.

At 918.58:

The value of nin Figure 8-498 (Cluster Report element format) is equal to 255 – (2 + sizeof(Reported BSSID field) + sizeof(Reference Timestamp field) + sizeof(Clustering Control field) + sizeof(ECPAC Policy Element field) + sizeof(Traffic Scheduling Constraint field)).

(note to editor,  merge with change at CID 2036)

At 919.33:

The Cluster Report subfield is set to 1 to indicate that this element contains a cluster report. If this subfield is set to 1, the Reported BSSID, Reference Timestamp and Clustering Control fields are present in this element. Otherwise, if the Cluster Report subfield is set to 0, none of the Reported BSSID, Reference Timestamp, Clustering Control, Extended Schedule Element, and Traffic Scheduling Constraint fields is present in this element

At 919.45:

The TSCONST Present subfield is valid only if the Cluster Report subfield is set to 1; otherwise, it is reserved. The TSCONST Present subfield is set to 1 to indicate that the Number ofConstraints field and the Traffic Scheduling Constraint field are present inthis element. Otherwise, these fields are not present in this element.

At 920.14:

The Number of Constraints field indicates the number of Constraint subfields contained in the Traffic Scheduling Constraint field. The value of this field ranges from 0 to 15. Other values are reserved. The Traffic Scheduling Constraint field contains zero or more Constraint subfields as defined in 8.4.2.133 (DMG TSPEC element) and specifies periods of time with respect to the TBTT of the beacon interval of the BSS the STA participates where the STA experiences poor channel conditions, such as due to interference.

At 1273.61:

The PCP/AP should schedule SPs for a STA such that the scheduled SPs do not overlap in time with the traffic scheduling constraints indicated by this STA in the Traffic Scheduling Constraint field of the associated DMG TSPEC element.

At 1280.53:

A STA that receives an RTS and/or DMG CTS frame that updates the NAV and that overlaps in time with an SP where the STA is destination or source, may report the overlap to the PCP/AP by sending a DMG ADDTS Request frame variant (8.6.3.3.2 (DMG ADDTS Response frame variant)) and including in the DMG TSPEC element (8.4.2.133 (DMGTSPEC element)) the indication of interference in the Traffic Scheduling Constraint field (Figure 8-481 (Traffic Scheduling Constraint field format)). Transmission of the DMG ADDTS Request frame variant shall follow the rules defined in 10.4 (TS operation), with the following exceptions.

(The following changes follow the pattern of changes shown above)

And change “TSCONST” to “Traffic Scheduling Constraint” at: 1281.28, 1281.36, 1298.56, 1299.06, 1618.01, 1618.03, 1634.29.

At 1281.01:

The Traffic Scheduling Constraint field of the DMG TSPEC element may contain one or more Constraint subfields. Each Constraint subfield provides information separately for each overlapping NAV event.

(and finally)

Globally change “Traffic Scheduling Constraint” to “Traffic Scheduling Constraint Set”.

Proposed Resolution:

Revised.  Make changes in < this –document> under CID 2037.

| 2041 | 929.10 | 8.4.2.156 | The term "information element" was changed to "element", but we did not do the same for "information subelement" | Globally replace "Information subelement" with "subelement". |
| --- | --- | --- | --- | --- |

There are 43 instances.  Three of which are “Directional Channel Quality Reporting Information” subelement.

The “information” is useless there too.  We either have to change the name of this subelement, or exclude it from the global change.

Proposed resolution:

Revised.

Change all “Directional Channel  Quality Reporting Information” to “Directional Channel  Quality Reporting”.

Change all “Information subelement” to “subelement”,  with capitalization as required by syntax.

| 2042 | 1031.50 | 8.6.14.15 | Many figures claim to show the format of a frame, when they are showing the format of the action field (e.g. Figure8-613) | Review all figure captions in the action frame subclause and replace "frame format" with "Action field format". |
| --- | --- | --- | --- | --- |

Range of figures is 8-554 to 8-642.

Range of tables is 8-232 to 8-366.

Also included are some “Action fields expressed as tables”.

Proposed Resolution:

Revised:

Review all figure captions in the range 8-554 to 8-642 and replace "frame format" with "Action field format".

Review all table captions in the range 8-232 to 8-366 and replace "frame format" with "Action field format".

| 2044 | 1062.54 | 8.6.20.2 | "The format of the Power Save Configuration Request frame Action field is shown..." -- no it's not. The action field does not include category or DMG action fields. | Remove the Category and DMG Action fields from all the "action field" figures in 8.6.20 and remove accompanying descriptions. |
| --- | --- | --- | --- | --- |

Context: 1062.52

And 1063.01:

Proposed resolution:

Accepted.

| 2046 | 1083.65 | 8.6.22.2 | "Any number of elements can be included within an Announce frame." - is both patently untrue, and doesn't follow style. | Delete cited sentence. |
| --- | --- | --- | --- | --- |

Context: 1083.48

8.6.22.2 Announce frame format The Announce frame is an Action or an Action No Ack frame of category Unprotected DMG. The format of an Announce frame Action field is shown in Table 8-365 (Announce frame Action field format). Announce frames can be transmitted during the ATI of a beacon interval and can perform functions including of a DMG Beacon frame, but since this frame does not have to be transmitted as a sector sweep to a STA, it can provide much more spectrally efficient access than using a DMG Beacon frame. The Category field is set to the category for Unprotected DMG, specified in Table 8-43 (Category values). The Unprotected DMG Action field is set to the value corresponding to Announce specified in Table 8-364 (Unprotected DMG Action field values). Any number of elements can be included within an Announce frame. …

And 1084.01:

Discussion:

The original design is probably underconstrained.   For example,  all the optional elements above could be omitted and then re-introduced under the “multiple elements” position – thus rendering the specified ordering moot.

And then there’s a whole bunch of elements that aparently “can appear” here,  but that would make no sense.  For example,  it makes no sense to include multiple DMG Capabilities elements,  particularly if they are not exact copies.

What .11ad should have done is list the valid elements and their multiplicities.

I suppose the least we can address is the “any number” issue.

Because the Table is normative and definitive,  and specifies the contents of the “last-n” order in a lot more detail than the “Any number of elements can be included within an Announce frame.”,  I propose to delete the latter.

Proposed resolution:

Revised.  At cited location delete: “Any number of elements can be included within an Announce frame.”

| 2095 | 1087.32 | 8.7.2 | "Data MPDUs sent under an HT-immediate Block Ack ... in a DMG BSS, QoS Null MPDUs..."There is no need to conflate together the Data MPDUs and QoS NUll MPDUs into the same row, as it makes the logic unnecessarily complex. | Separate out the DMG QoS Null case into a separate row. |
| --- | --- | --- | --- | --- |

Discussion:  page number is actually 1089.32.

Context: 1089.01

- A-MPDU contents in the data enabled immediate response contextinput.doc. MPDU Description; Conditions. (#1198)Ack(#190); If the preceding PPDU contains an MPDU that requires an (#1198)Ack response, a single (#190)(#1198)Ack frame at the start of the A-MPDU.; In a non-DMG STA:(11ad) at most one of these MPDUs is present. In a DMG STA: at most one (#1198)Ack MPDU is present, and zero or more HT-immediate BlockAck MPDUs are present.(11ad). HT-immediate BlockAck; In a non-DMG STA:(11ad) if the preceding PPDU contains an implicit or explicit Block Ack request for a TID for which an HT-immediate Block Ack agreement exists, at most one (#192)BlockAck frame for this TID, in which case it occurs at the start of the A-MPDU. In a DMG STA: if the preceding PPDU contains an implicit or explicit Block Ack request for a TID for which an HT-immediate Block Ack agreement exists, one or more copies of the same BlockAck for this TID.(11ad). Delayed BlockAcks; BlockAck frames with the BA Ack Policy subfield equal to No Acknowledgment with a TID for which an HT-delayed Block Ack agreement exists.. Delayed Block Ack data; QoS Data MPDUs with a TID that corresponds to a Delayed or HT-delayed Block Ack agreement. These have the Ack Policy field equal to Block Ack.. Action No Ack; Management frames of subtype Action No Ack.. Delayed BlockAckReqs; (#193)BlockAckReq frames with a TID that corresponds to an HT-delayed Block Ack agreement in which the BA Ack Policy subfield is equal to No Acknowledgment.. Data MPDUs sent under an HT-immediate Block Ack agreement (M34)or in a DMG BSS, QoS Null MPDUs with Ack Policy set to No Acknowledgment; QoS Data MPDUs with the same TID, which corresponds to an HT-immediate Block Ack agreement (#202)See NOTE. In addition, in a DMG BSS, QoS Null MPDUs with Ack Policy set to No Acknowledgment.(M34); Of these, at most one of the following is present in a non-DMG BSS:(M34) - One or more QoS Data MPDUs with the Ack Policy field equal to Implicit Block Ack Request - A BlockAckReq frame Of these, at most one of the following is present in a DMG BSS:(M34) - One or more QoS Data MPDUs with the Ack Policy field equal to Implicit Block Ack Request - QoS Null MPDU with Ack Policy set to No Acknowledgment - A BlockAckReq frame with an optional QoS Null MPDU with Ack Policy set to No Acknowledgment. Immediate BlockAckReq; At most one BlockAckReq frame with a TID that corresponds to an HT-immediate Block Ack agreement. This is the last MPDU in the A-MPDU. It is not present if any QoS (#100)Data frames for that TID are present.. (#202)NOTE—These MPDUs all have the Ack Policy field equal to the same value, which is either Implicit Block Ack Request or Block Ack..

Proposed changes:

- A-MPDU contents in the data enabled immediate response contextinput.doc. MPDU Description; Conditions. (#1198)Ack(#190); If the preceding PPDU contains an MPDU that requires an (#1198)Ack response, a single (#190)(#1198)Ack frame at the start of the A-MPDU.; In a non-DMG STA:(11ad) at most one of these MPDUs is present. In a DMG STA: at most one (#1198)Ack MPDU is present, and zero or more HT-immediate BlockAck MPDUs are present.(11ad). HT-immediate BlockAck; In a non-DMG STA:(11ad) if the preceding PPDU contains an implicit or explicit Block Ack request for a TID for which an HT-immediate Block Ack agreement exists, at most one (#192)BlockAck frame for this TID, in which case it occurs at the start of the A-MPDU. In a DMG STA: if the preceding PPDU contains an implicit or explicit Block Ack request for a TID for which an HT-immediate Block Ack agreement exists, one or more copies of the same BlockAck for this TID.(11ad). Delayed BlockAcks; BlockAck frames with the BA Ack Policy subfield equal to No Acknowledgment with a TID for which an HT-delayed Block Ack agreement exists.. Delayed Block Ack data; QoS Data MPDUs with a TID that corresponds to a Delayed or HT-delayed Block Ack agreement. These have the Ack Policy field equal to Block Ack.. Action No Ack; Management frames of subtype Action No Ack.. Delayed BlockAckReqs; (#193)BlockAckReq frames with a TID that corresponds to an HT-delayed Block Ack agreement in which the BA Ack Policy subfield is equal to No Acknowledgment.. Data MPDUs sent under an HT-immediate Block Ack agreement (M34)or in a DMG BSS; QoS Data MPDUs with the same TID, which corresponds to an HT-immediate Block Ack agreement (#202)See NOTE. (M34); Of these, at most one of the following is present in a non-DMG BSS:(M34) - One or more QoS Data MPDUs with the Ack Policy field equal to Implicit Block Ack Request - A BlockAckReq frame Of these, at most one of the following is present in a DMG BSS:(M34) - One or more QoS Data MPDUs with the Ack Policy field equal to Implicit Block Ack Request - QoS Null MPDU with Ack Policy set to No Acknowledgment - A BlockAckReq frame with an optional QoS Null MPDU with Ack Policy set to No Acknowledgment. QoS Null MPDUs with Ack Policy set to No Acknowledgment; In a DMG BSS, QoS Null MPDUs with Ack Policy set to No Acknowledgment.. Immediate BlockAckReq; At most one BlockAckReq frame with a TID that corresponds to an HT-immediate Block Ack agreement. This is the last MPDU in the A-MPDU. It is not present if any QoS (#100)Data frames for that TID are present.. (#202)NOTE—These MPDUs all have the Ack Policy field equal to the same value, which is either Implicit Block Ack Request or Block Ack..

Proposed Resolution:

Revised.  Make changes in <this-document> under CID 2095, which implements the suggested change.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- |
| 2066 | 1261.14 | 9.33.3 |  | (From Editor Panel Review of D1.1) " A non-PCP STA that is a non-AP STA and that receives scheduling information accesses the medium during the scheduled periods using the access rules specific to that period."We have various ways of ways of expressing this relation ship. The term "A non-PCP and non-AP STA" is also correct, but awkward. The terminology used elsewhere in .11ad is non-PCP/non-AP. This terminology should be used consistently or consistently replaced with something avoiding the "slash" conjection - which variously means "or" (as in a PCP/AP) or "and" (as in non-PCP/non-AP). | Discuss the terminology. Do we like non-PCP/non-AP? If so, lets define it (because the definition is non obvious given the ambiguity of the conjunction) and use it to replace instances such as cited.If not, then replace all ambiguous "slash" conjunctions with plain English, or define a new term "e.g., a nurgel", add a definition and use consistently throughout. | MAC |

Proposed Resolution:

Rejected. The proposed change does not provide specific changes to make.

In reply to the commenter,  CID 2115 replaces all “PCP/” terminology with a textual conjunction “and” or “or”,  resulting in “AP or PCP” and “non-AP and non-PCP”.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- |
| 2050 | 1556.47 | 10.24.16.2 |  | The substitutions from .11ad might be better served by creating termS to reflect "AP or DMG STA" and "non-AP or DMG STA" that relate to their roles, and to use those terms throughout this subclause. | Consider defining terms representing these roles and using them in this subclause. | MAC |

Discussion:

Before .11ad,   DMS roles of sender and receiver mapped neatly onto AP and non-AP respectively.

After .11ad,  a DMS STA can provide either role, but only in the context of a PBSS.

The changes made by .11ad were to introduce these substitutions,  but the substitutions were not precise,  because they don’t capture the distinctions between PBSS and DMG infrastructure operation.

So we ended up with “non-AP STA or DMG STA” and “AP or DMG STA” repeatedly in this section.

To address a similar issue, in REVmb days we introducted the term “FTO” to clarify roles and simplify the text related to fast transition.  We could do the same here.

The scope is local to 10.24.16.2.

For discussion:  at 1556.47:

The Directed Multicast Service (DMS) is a service that may be provided by an AP or DMG STA to its associated non-AP STAs or DMG STAs that support the DMS service, where the AP or DMG STA transmits group addressed MSDUs as individually addressed A-MSDUs.

Where does “associated” bind?    Regardless “… provided by a DMG STA to its associated non-AP STAs” only makes sense if the DMG is an AP or PCP, which conflicts with 1556.51:  “In a PBSS, DMS is a service that may be provided by any STA to other STAs associated in the same PBSS”

Also, I originally suspected the “In a PBSS” is wrong here, assuming it to be true for infrastructure DMG BSS and PBSS.  However,  in an exchange with the .11ad editor,  he confirms that the “any STA can originate DMS” is specific to a PBSS.

Also note that the substitution “non-AP STA or DMG STA” is unnecessary and incorrect.  A DMG AP never acts as a DMS client.    A PCP and non-PCP/non-AP STA can.

I believe we gain an increment of clarity by defining a term for the recipient behaviour,  even if it expands to the previous text.

Proposed resolution:

Revised.

At 1556.46 add:

“In this subclause, the following terms are used:

--Directed Multicast Service (DMS) provider: An AP, PCP, or DMG STA associated with a PCP that provides the DMS service.

--DMS recipient: A non-AP STA that uses the DMS service.”

Throughout 10.24.16.2 replace “AP or DMG STA” with “DMS provider”.

Throughout 10.24.16.2 replace “non-AP or DMG STA” with “DMS recipient”.

| 2197 | 744.14 | 8.4.2.30 | I don't understand how a TCLAS can apply a frame classifier of type 6 to an incoming MSDU. But that operation is now permitted. | Add a limitation somewhere that this classifier is used only by the procedures that make sense, i.e. TFS. |
| --- | --- | --- | --- | --- |

Discussion:

A TCLAS is used for these distinct purposes:

- To match an MSDU to TSPEC

- To match an MSDU for transmission using FMS (flexible multicast)

- To match MPDUs or MSDU/MMPDUs for filtering and notification using TFS

- To match MSDUs for transmission using DMS (directed multicast)

- To match MSDUs for SCS (stream classification service, .11aa,  drop eligibility)

All except TFS operate at the level of MSDU/MMPDU.   Upon review,  TFS can clearly do both.

TFS says: “When a traffic filtering agreement is established for a STA, the AP shall discard all individually addressed frames destined for the STA until a frame is found that matches one or more Traffic Filter Sets within the traffic filtering agreement.”

Although TFS may allow a TCLAS that operates on individual MPDUs, it makes no sense to take action on individual MPDUs.   A notification should occur based on the entire MSDU/MMPDU.

This is naturally the outcome of the logic at 1554.18 onwards, unless the TCLAS is looking at the fragment number field.  Given that I can’t see any reason to want to do that,  I think the current specificiation is OK.

Proposed Resolution:

Rejected.   This classifer applies to MPDU headers, not MSDUs, and therefore implicitly applies after the processes the turn an MSDU into an MPDU.

| 2015 | 577.06 | 8.3.4.1 | The Order of "Last -n" is very curious and will be misread as "Last to n". | Change to "2 - (Last - 1)", which mirrors usage in the action frame at 574.37. Make similar change at 1079.06, 1080.17, 1084.28 |
| --- | --- | --- | --- | --- |

Straw poll: (Discussed and results from 2013-11 shown)

Do you want to:

A:  remove the order column - 4

B:  change to 1, 2.., n,  last - 9

B.5:  change to 1, 2, 3, 4, 5  (i.e. no “last”) - 3

B.6:  As above + change column title to “rank” - 1

B.7:  1,2,3,  last-2, last-1, last - 10

C:  use “last” and “penultimate” terminology - 2

D:  reject the comment - 3

E: Resolve this comment by “Last (-n)” – 2

Status 2013-11-26:  Updated the yellow block below.

Issue: Mesh Peering Action frames (see 1047.20) don’t follow the format for Action frame,  and include Vendor Specific.  Can be fixed by adding the AMPE element into the Action frame format in position “Last”.

Proposed Resolution:

Revised.

Replace the “order” column entries at the following locations as follows:

559.52: Last – 1

571.31: Last – 1

571.34: Last

574.14: Last – 1

574.36: Last – 1

577.06: Last – 1

1007.60: 4

1053.50: 3

1056.59: 3

1057.31: 4

1058.31: 4

1059.11: 4

Also copy last row of Table 8-314 (1047.20) to the Action frame format (Table 8-35) as a new Last row, renumber previous Last and Last-1, and add to description “Present only in Self-protected Action frames”.

| 2045 | 1064.24 | 8.6.20.4 | REVmc D1 removed attempts to calculate the order of variable-count fields. This should be propagated to the .11ad tables. | At order 5, change information to DMG Capabilities (optional, repeated), remove the N+4 row, change N+5 to 6, and change information for old N+5 to (optional, repeated), delete 4+N+M row.Make matching changes at 1065.07.Make similar change at 1068.51, 1069.60, 1084.62 |
| --- | --- | --- | --- | --- |

Context: 1064.20:

Discussion.  If this is an action field, it is followed by Vendor Specific, but should not iself include Vendor Specific.

“IE Provided” is not the name of any element and uses “old language”.  Except for the Mesh Peering Action frames,  which have their own problems, all other Action field descriptions don’t use “Last” and don’t have Vendor Specific.    They should be removed from the .11ad action fields.

Discuss: The proposed resolutions don’t introduce a new column.   The alternative would be to follow the pattern of table 8-258 (976.44) and introduce a “Notes” column to convey any additional multiplicity semantics.

Proposed Resolution:

At 1064.20, Number order 5, Replace “DMG Capabilities 1 (optional)” with “Zero or more DMG Capabilities elements”

At 1064.22, delete row

At 1064.24, delete row

At 1064.26, Number order 6, Replace “IE Provided 1 (optional)” with “Zero or more provided elements”

At 1064.28 & 30, delete rows

At 1065.17, Number order 4, Replace “DMG Capabilities 1” with “One or more DMG Capabilities elements”

At 1065.19, delete row

At 1065.21, delete row

At 1065.23, Order = 5

At 1065.25, Number order 6, Replace “IE Provided 1 (optional)” with “Zero or more provided elements”

At 1065.27 & 28, delete rows

At 1068.48, replace “Relay Capable STA 1 Info” with “Zero or more Relay Capable STA Info fields”.

Delete rows at 1068.49 and 1068.51.

At 1069.56, replace “Channel Measurement Info 1” with “One or more Channel Measurement Info fields”

Delete rows at 1069.58 and 1068.60.

At 1079.06, replace “Last – n” with “10”.

Delete row at line 14.

At 1080.17, replace “Last – n” with “11”.

Delete row at line 25.

At 1084.28,  replace “Last – n” with “13”.

Delete row at line 33.

At 1084.57, Replace “Channel Measurement Feedback element 1 (optional)” with “Zero or more Channel Measurement Feedback elements”.

Delete rows at lines 60 and 62.

| 2028 | 827.61 | 8.4.2.71 | Does this para dupliate the previous one? | If so, delete it. |
| --- | --- | --- | --- | --- |

Context: 827.56

When transmitted by a DMG STA, the Nontransmitted BSSID Capability element includes the DMG BSS Control and the Nontransmitted BSSID DMG Capability element fields. These fields are not present if this element is transmitted by non-DMG STAs. The DMG BSS Control field and the Nontransmitted BSSID DMG Capabilities element field are present in this element only when the element is transmitted by a DMG STA and are not present otherwise.

Proposed Resolution:

Revised.  The text is a functional duplicate of the previous para.  Delete cited para.

| 2447 | 835.54 | 8.4.2.78 | 8.4.2.78 on max idle period specifies specific behavior details that aren't in subclause 10.24.13. This is confusing (behavior in clause 8). Move (or copy) this to clause 10. | Replace the sentence "The Max Idle Period field indicates the time period during which a STA can refrain from transmitting frames to its associated AP without being disassociated." with "The Max Idle Period field indicates the idle timeout limit, as described in 10.24.13 (BSS max idle period management)." and delete "A non-AP STA is considered inactive if the AP has not received a Data frame or Management frame of a frame exchange sequence initiated by the STA for a time period equal to or greater than the time specified by the Max Idle Period field value." from the cited location.Add, "The Max Idle Period field of the BSS Max Idle Period element indicates the time period during which a STA can refrain from transmitting frames to its associated AP without being disassociated. A non-AP STA is considered inactive if the AP has not received a Data frame or Management frame (protected or unprotected per the following) of a frame exchange sequence initiated by the STA for a time period equal to or greater than the time specified by the Max Idle Period field value." to the start of the second paragraph of 10.24.13. |
| --- | --- | --- | --- | --- |

Context: 835.54:

The Max Idle Period field indicates the time period during which a STA can refrain from transmitting frames to its associated AP without being disassociated. The Max Idle Period field is a 16-bit unsigned integer. The time period is specified in units of 1000 TUs. The value of 0 is reserved. A non-AP STA is considered inactive if the AP has not received a Data frame or Management frame of a frame exchange sequence initiated by the STA for a time period equal to or greater than the time specified by the Max Idle Period field value.

Changes 835.54

The Max Idle Period field indicates the idle timeout limit, as described in 10.24.13 (BSS max idle period management).

And at 1554.46:

10.24.13 BSS max idle period management If dot11MaxIdlePeriod is a nonzero, the STA shall include the BSS Max Idle Period element in the Association Response frame or the Reassociation Response frame. Otherwise, the STA shall not include the BSS Max Idle Period element in the Association Response frame or the Reassociation Response frame. A STA may send security protocol protected or unprotected keep-alive frames, as indicated in the Idle Options field. The Max Idle Period field of the BSS Max Idle Period element indicates the time period during which a STA can refrain from transmitting frames to its associated AP without being disassociated. A non-AP STA is considered inactive if the AP has not received a Data frame or Management frame (protected or unprotected as specified in this paragraph) of a frame exchange sequence initiated by the STA for a time period equal to or greater than the time specified by the Max Idle Period field value. If the Idle Options field requires security protocol protected keep-alive frames, then the AP may disassociate the STA if no protected frames are received from the STA for a period indicated by the Max Idle Period field of the BSS Max Idle Period element. If the Idle Options field allows unprotected or protected keep-alive frames, then the AP may disassociate the STA if no protected or unprotected framesare received from the STA for a duration indicated by the Max Idle Period field of the BSS Max Idle Period element.

Proposed resolution:

Revised.  Make changes in <this document> under CID 2447.

| 2448 | 836.12 | 8.4.2.78 | In 8.4.2.78 (max idle period) what is "the Idle Timer" - it's not mentioned anywhere else. | Replace"The Protected Keep-Alive Required bit set to 1 indicates that the STA sends an RSN protected frame to the AP to reset the Idle Timer at the AP for the STA, as defined in 10.24.13 (BSS max idle period management). If the Protected Keep-Alive Required bit is 0, the STA sends either an unprotected or a protected frame to the AP to reset the Idle Timer at the AP."with"The Protected Keep-Alive Required bit is set to 1 to indicate that an RSN protected frame is required to show activity. The Protected Keep-Alive Required bit is set to 0 to indicate either an unprotected or a protected frame indicates activity." |
| --- | --- | --- | --- | --- |

Context: 836.12:

The Protected Keep-Alive Required bit set to 1 indicates that the STA sends an RSN protected frame to the AP to reset the Idle Timer at the AP for the STA, as defined in 10.24.13 (BSS max idle period management). If the Protected Keep-Alive Required bit is 0, the STA sends either an unprotectedor a protected frame to the AP to reset the Idle Timer at the AP.

Change proposed by commenter:

The Protected Keep-Alive Required bit is set to 1 to indicate that an RSN protected frame is required to show activity. The Protected Keep-Alive Required bit is set to 0 to indicate either an unprotected or a protected frame indicates activity.

Discussion:

- “bit” is not preferred language.

- “is required” sounds like a normative requirement

Proposed resolution:

Revised.  Replace cited paragraph with:

“The Protected Keep-Alive Required subfield is set to 1 to indicate that only an RSN protected frame indicates activity. The Protected Keep-Alive Required subfield is set to 0 to indicate that either an unprotected or a protected frame indicates activity.”

| 2030 | 839.31 | 8.4.2.80 | "When the value of the Subelement ID field is 1, the TFS Response Subelement is a TFS Status subelement" - this merely duplicates what is stated in table 8-180. | Delete cited para. |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted

| 2330 | 644.31 | 8.4.2.20.4 | Normative verb in a definition. | Replace "may indicate" with "indicates". |
| --- | --- | --- | --- | --- |

| 2335 | 676.15 | 8.4.2.21.4 | Normative verb in a definition. | Replace "may indicate" with "indicates". |
| --- | --- | --- | --- | --- |

Propose resolution:

Revised.  Delete the first three sentences of the paragraph at 644.31.

Delete the first two sentences of the para at 676.15.

Insert a new subclause 10.11.x (at the end):

“10.11.x RPI histogram request and report

The response to an RPI histogram request is an RPI histogram report. A STA may support the generation of an RPI histogram. A STA may generate an RPI histogram report in response to an RPI histogram request.”

| 2329 | 644.06 | 8.4.2.20.3 | Normative verb in a definition. | Replace "may indicate" with "indicates". |
| --- | --- | --- | --- | --- |

| 2334 | 675.40 | 8.4.2.21.3 | Normative verb in a definition. | Replace "may indicate" with "indicates". |
| --- | --- | --- | --- | --- |

Context: 644.04:

8.4.2.20.3 CCA request A Measurement Type in the Measurement Request element may indicate a CCA request. A response to a CCA request is a CCA report. It is optional for a STA to generate a CCA report in response to a CCA request. The Measurement Request field corresponding to a CCA request is shown in Figure 8-127 (Measurement Request field format for a CCA request)

Propose resolution:

Revised.  Delete the first three sentences of the paragraph at 644.06.

Delete the first two sentences of the para at 675.40.

Insert a new subclause 10.11.x (at the end):

“10.11.x CCA request and report

The response to a CCA request is a CCA report. A STA may support the generation of a CCA report. A STA may generate a CCA report in response to a CCA request.”

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2258 | 61.17 |  |  | "A non-PCP/non-AP STA requests a PCP/AP for SPs, which can be used for transmission": confusing at best. How does a STA request a PCP or AP for service periods? What does that mean? We're guessing that it means that the STA requests one or more SPs from a PCP or AP. | Replace "requests a PCP/AP for SPs, which can be used" with "requests SPs from a PCP or AP. These SPs can be used". |  | GEN |

Context: 61.16

The third mechanism, designated the service period (SP) access orservice period channel access (SPCA), is applicable only to DMG STAs and allows for the reservation of channel time by the PCP/AP. A non-PCP/ non-AP STA requests a PCP/AP for SPs, which can be used for transmission to any other STA in the BSS. The request is initiated by the SME of the non-PCP/non-AP STA. The PCP/AP either accepts or rejects the request based on an admission control policy. If the request is accepted, the PCP/AP uses the Extended Schedule element to schedule SPs for communication between the source and destination DMG STAs indicated within the request. Details of this mechanism are provided in 9.34.6.2 (Service period (SP) allocation), 9.34.6.4 (Pseudo-static allocations), 9.34.6.6 (DMG Protected Period), and 10.4 (TS operation).

Discussion:

CID 2115 reworded some of the cited text thus:

A non-AP and non-PCP STA requests an AP or PCP for SPs, which can be used for transmission to any other STA in the BSS.

The improvement proposed by the commenter can be merged into the terminology/ordering changes from CID 2115.

Proposed Resolution:

Revised.  Change cited sentence to read: “… requests SPs from an AP or PCP.  These SPs can be used …”

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2000 | 9.48 | 3.1 |  | The "coordination function" definition includes too many 802.11-specific terms. | Either make it generic. Or move to 802.11-specific subclause |  | GEN |

# Context:  (9.48):

coordination function:The logical function that determines when a station (STA) operating within a basic service set (BSS) is permitted to transmit protocol data units (PDUs) via the wireless medium (WM). The coordination function within a BSS might have one hybrid coordination function (HCF), or it might have one HCF and one point coordination function (PCF) and has one distributed coordination function (DCF). A quality-of-service (QoS) BSS has one DCF and one HCF.In addition, a directional multi-gigabit (DMG) STA has a DMG channel access function that includes the beacon transmission interval (BTI), the association beamforming training (A-BFT), the announcement transmission interval (ATI), and the service period channel access (SPCA).

# Proposed resolution:

Revised.

Turn all but the first sentence into a NOTE--.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2213 | 20.10 | 3.1 |  | A PCP and an AP each are defined to contain a single STA. So how can a STA be one or more PCPs and/or APs? | Replace "is at least one of a PCP or an AP" with "either is contained in an AP or is contained in a PCP". |  | GEN |

Discussion:

The resolution for CID 2115 removes the need for this definition as it splits all occurances of PCP/AP into its component parts.

Proposed Resolution:

Revised.

Delete cited definition.

(Note to editor, this is a subset of the changes for CID 2115)

| 2131 | 927.08 | 8.4.2.154 |  | "availability of the nth Beacon SP" -- in other TGs we have tried to avoid the superscripted "th". For consistency should replace here. | Reword "availability of the Beacon SP n". Review and reword all such uses in the draft where possible. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

While I think “th” is overwhelmingly ugly, the question is whether the change below introduces ambiguity.

If it does, we can resolve this by adding to 1.4 (Word Usage):

‘The expression “index” followed by a number identifies a single occurance of an ordered sequence.   For example, “symbol index 2 of the frame” identifies the second symbol.’

And then prefix each of the insertions below with “index”.

Straw poll:  Do you

Prefer to reject this comment 9

Prefer to make changes to address the comment 3

Proposed resolution:

Rejected.  These expressions are unambiguous, and we prefer not to make any change.

| 2145 | 1225.30 | 9.26.2 |  | "+HTC/DMG" -- the "/" as a conjunction is evil because it sometimes means "and" and sometimes means "or". | Replace all such with +HTC or DMG |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2115 |  |  | A | PCP/AP suffers from a cart-before-the-horse issue. Clearly those who wrote the PCP stuff concentrated on the existence of PCPs. But most readers of the standard will care more about APs, and (like the egg) these did come first. So it's a bit a surprise to see wholesale rebranding of APs as PCP/APs. | Replace PCP/AP with AP/PCP, and similarly the non-PCP/non-AP. | ACCEPTED (GEN: 2013-11-12 21:08:35Z) | GEN |

Discussion:

At TGmc session on 2013-11-12 the general question of / in PCP/AP and non-PCP/non-AP arose during discussion of 2115, and sentiment was expressed that we should address the general issue in the context of comment 2145.  However, I propose we leave 2145 as an accept, and re-visit 2115 as below

Also the definitions of the “PCP/” terms are now no longer needed and should be remove.

The following resolution touches about 1400 locations.

Proposed resolution

Revised.

The CRC decided also to remove “/” from PCP/AP terminology.

Replace globally as follows, with adjustment to surrounding synax as necessary:

PCP/AP -> AP or PCP

non-PCP/non-AP -> non-AP and non-PCP

Non-PCP/Non-AP -> Non-AP and non-PCP

S-PCP/S-AP -> S-AP or S-PCP

member PCP/member AP -> member AP or member PCP

PCP/HC -> HC or PCP

PCP/APs -> APs and PCPs

S-PCP/SAP -> S-AP or S-PCP

PCP/access point (AP) Cluster -> access point (AP) cluster or PCP cluster

PCP/APAP -> AP or PCP

PCP/AP Clustering -> AP or PCP Clustering

PCPs/APs -> APs and PCPs

S-AP/member PCP/  member AP -> member AP, S-AP, or member PCP

member PCP/  AP -> member AP or member PCP

synchronization PCP (S-PCP)/S-AP -> S-AP or synchronization PCP (S-PCP)

control point (PCP)/access point (AP) -> control point (PCP) or access point (AP)

personal basic service set (PBSS)control point (PCP)/AP Cluster -> AP Cluster or personal basic service set (PBSS)control point (PCP)

Delete definitions at 17.55, 20.10, 38.53

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- |
| 2114 |  |  |  | The PCO "tricks" for separating two classes of device don't extend into further classes of device, which means it is not compatible with VHT. There is no point trying to over-manage 2.4 GHz because of the variety of non-802.11 devices present, and it won't be able to manage 5 GHz which will be used generally only when VHT is deployed.It has not been implemented, and based on the above analysis, never will be. | Deprecate PCO. | MAC |

Discussion:

Should we deprecate or mark as obsolete?

We have 13 deprecates outside the MIB.

Some of them:

“The use of priority value of ContentionFree is deprecated at QoS STAs.”

“The use of WEP for confidentiality,authentication, or access control is deprecated.”

“The use of TKIP is deprecated.”

And there are 9 obsoletes:

“Note that the use of the StrictlyOrdered service class is obsolete and the StrictlyOrdered service

class might be removed in a future revision of the standard”

“The PCF mechanism is obsolete. Consequently, this subclause might be removed in a later revision of the

standard.”

It seems that we “deprecate” stuff that we have to leave in the standard,  and we mark as obsolete stuff we hope to remove.   However the word “obsolete” also carries the implication of age that is arguably not so in this case.

Notwithstanding this,  “obsolete” seems the more appropriate choice.

Stock phrase for “phased removal” is as follows: (630.27)

“The PCF mechanism is obsolete. Consequently, this subclause might be removed in a later revision of the standard.”

This statement is present in a small number of locations per feature that has been obsoleted,  generally Clause 8,  Clause 9/10 and Annex B.

Straw poll:   Do you agree to deprecate or mark as obsolete the PCO mechanism?

Yes: 10

No: 1

Abstain: 6

Straw poll:   Do you prefer to deprecate or mark as obsolete?

Deprecate (no indication of future removal) 6

Deprecate and obsolete and might be removed later 4

Mark as obsolete and might be removed later 12

No preference 3

None of the above 1

Proposed resolution:

Revised.  At 777.21 (Extended capabilities) in the Definition column add:

“The PCO mechanism is obsolete. Consequently, this subfield might be reserved in a later revision of this standard.”

At 1008.09 (Set PCO Phase frame format) and 1514.35 (General description of PCO) add:

“The PCO mechanism is obsolete. Consequently, this subclause might be removed in a later revision of this standard.”

At 2340.52 (PICS) add

“The PCO mechanism is obsolete. Consequently, the PCO mechanism might be removed in a later revision of this standard.”

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- |
| 2190 |  |  |  | Dual CTS protection is evil.The issue is that STBC was introduced in .11n as an attempt to extend range, moving "high throughput" goal-posts so far they fell out of the ball-park.It created a bunch of corner conditions related to how you initiate a TXOP and how you truncate it. Although I can't remember any of the specific causes for concern, I do remember doing the analysis and discovering multiple corner cases.And then we have the issue of transmitting all broadcast frames in both STBC and non-STBC variants at the lowest rates.As far as I know, nobody has implemented this feature. | Deprecate Dual CTS protection and related mechanisms (e.g. dual transmission of broadcast frames). | MAC |

Discussion:

Is Dual CTS protection inseperable from Dual Beacon?   I believe so.  Dual Beacon was added to extend the range of the Beacon for 1-antenna devices in .11n.   Dual CTS protection was added to extend the range of protection for these devices by using STBC for control frames.

The proposed resolution comes in two parts, one to deprecate dual-cts, one to deprecate STBC Beacon.

Straw poll, do you:

Agree to deprecate these mechanisms (no notice of removal inserted in text) 15

Agree to mark as obsolete (notice of removal) 6

Want to keep them 1

Proposed Resolution:

Revised.

At 784.43 (HT Operation element) in the Definition column, add:

The use of the dual CTS mechanism is deprecated.”

At 1109.49 (Dual CTS protection procedure) add:

“The use of the dual CTS mechanism is deprecated.”

At 2340.19 (PICS) add:

“The use of the dual CTS mechanism is deprecated.”

At 784.34 (HT Operation element) and 784.56 in the Definition column, add:

“The use of the dual beacon mechanism is deprecated.”

At 1355.38 (Beacon generation in non-DMG infrastructure networks) add:

“The use of the dual beacon mechanism is deprecated.”

At 2340.13 (PICS) add:

“The use of the STBC beacon transmission mechanism is deprecated.”

| 2007 | 509.36 | 8.2.4.1.4 | The insertion by .11ad disallows mesh the use of this encoding (ToDS and FromDS both zero). Is this correct? | If it is not correct, add MBSS somewhere. |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.   The statements at 509.47 and 509.52 exclude To DS=From DS=0 from use by a mesh STA.  So the insertion of “infrastructure” by .11ad at line 38 is correct.

| 2013 | 510.51 | 8.2.4.1.7 | The references introduced by CID 86 are bogus. 10.2.1.2 does not exist. 10.2.2.4 seems wrong. | Check and correct references. |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.  At 510.52 change “10.2.1.2” to “10.2.2.2”.  At 510.62 change “10.2.2.4” to “10.2.3.4”

| 2451 | 511.21 | 8.2.4.1.8 | D1.5 P502.50 How does the AP know that the non-DMG STA has the subfield 1 and APSD enabled? This should talk about signaling the AP has received. Similar at 502.55, but that one might be okay. | Change "has the More Data Ack subfield of its QoS Capability element equal to 1" to "has the More Data Ack subfield equal to 1 in the most recently received Capability Information field"Change, "has APSD enabled" to "is using APSD and is in PS mode" |
| --- | --- | --- | --- | --- |

Context: (511.20)

An AP optionally sets the More Data field to 1 in Ack frames to a non-DMG STA that has the More Data Ack subfield of its QoS Capability element equal to 1 and that has APSD enabled to indicate that the AP has a pending transmission for the STA.

Change proposed by commenter:

An AP optionally sets the More Data field to 1 in Ack frames to a non-DMG STA that "has the More Data Ack subfield equal to 1 in the most recently received Capability Information field and that is using APSD and is in PS mode to indicate that the AP has a pending transmission for the STA.

Comments:

- We have moved away from “most recently received” describing capabilities.

- “is using APSD” is still wooly.

Proposed resolution:

Revised.

Replace cited sentence with:

“An AP optionally sets the More Data field to 1 in Ack frames to a non-DMG STA from which it has received a frame that contains a QoS Capability element in which the More Data Ack subfield is equal to 1 and that has one or more ACs that are delivery enabled and that is in PS mode to indicate that the AP has a pending transmission for the STA.”

| 2014 | 513.06 | 8.2.4.2 | The insertion by .11ad is probably wrong. The insertion is between two conditions that are exclusions, but the .11ad insertion is an inclusion. | Reword so that all terms are inclusions or exclusions. |
| --- | --- | --- | --- | --- |

Context: (513.06),  addition by .11ad highlighted.

Duration value (in microseconds) within all frames other than PS-Poll frames transmitted during the CP, within all frames transmitted by a DMG STA, and under HCF for frames transmitted during the CFP

Discussion:

All frames transmitted by a DMG STA use this encoding.   So we can achieve the intended effect unambiguously by excluding from the exclusion any frames transmitted by a DMG STA.   A DMG does not operate under HCF,  so there is no need to exclude that.

Proposed Resolution:

Revised.

Replace the first row of the “Usage” table with:

“Duration value (in microseconds) within all frames except:

- PS-Poll frames transmitted by a non-DMG STA during the CP

- frames transmitted during the CFP using the HCF”

| 2320 | 513.32 | 8.2.4.3.1 | Normative verb in a defnition | "may not" also is ambiguous here. Replace "may" with "might". |
| --- | --- | --- | --- | --- |

Context: 513.30: (and proposed change)

There are four address fields in the MAC frame format. These fields are used to indicate the basic service set identifier (BSSID), source address (SA), destination address (DA), transmitting STA address (TA), and receiving STA address (RA). Certain frames might not contain some of the address fields.

Proposed resolution:

Accepted.

| 2480 | 519.14 | 8.2.4.5.4 | It is time to reconsider the restriction on a QOS Null frame to "normal ack" ack policy. The original rationale for this restriction was probably something along the lines of "Why send a frame that causes nothing to happen and not receive an acknowledgement?" But i say, why send a frame and receive an acknowledgement if nothing will happen at the recipient that receives that frame? But i digress. The point is that now, with lots of optional fields, take HT Control, for example, now appearing, potentially in the QOS NULL frame, the receipt of a QOS NULL CAN cause something to happen at a recipient. This means that the original rationale becomes valid, and i suppose is not really an argument to support the suggestion herein, which is to allow the NOACK setting for this frame. The rationale to support a NOACK setting is that the QOSNULL carrying HTC can be a valid option for, as an example an RDG decline. It is also possible that the transmission of this frame could be used to stretch the time to create an RDG response in the case of accepting the RDG. | Allow the use of NO ACK policy for the QOS NULL frame. |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.   The proposed change, by itself, is not sufficient.   A new STA that uses QoS Null (NoAck) cannot determine whether its peer supports this or not.  So it cannot determine whether an Ack will be sent or not in this case.

| 2124 | 531.01 | 8.2.5.3 | "NOTE--DMG STAs do not transmit QoS CF-Poll frames".Why is the NOTE necessary?Ditto other similar statements in 8.2.5 | Delete cited note, and at 531.32, 531.46, 532.13, 534.14, 535.03, 535.57, 544.40 |
| --- | --- | --- | --- | --- |

Discussion:

Notes are informative.   They have no effect on implementations of the standard.

Proposed Resolution:

Revised.  Make changes as indicated,  and delete the last two sentences at 537.24.

Delete sentence at 540.62.

| 2452 | 549.54 | 8.3.2.1 | 8.3.2.1, bottom of page 549 has behavioral "shall" text | Remove the paragraph at the bottom of page 549. Split the third paragraph of 9.2.8 into two paragraphs, after the first sentence. Replace the second paragraph now created, with:Address filtering is performed on the Address 1 field of each MPDU contained in a PPDU and on the DA of each MSDU within an A-MSDU. When the Address 1 field or DA field contains a group address, address filtering is performed by comparing the value in the Address 1 field or DA field to all values in the dot11GroupAddressesTable, and the STA also validates the BSSID to verify either that the group addressed frame originated from a STA in the BSS of which the receiving STA is a member, or that it contains the wildcard BSSID value, indicating a Data frame sent outside the context of a BSS (dot11OCBActivated is true in the transmitting STA).A mesh STA also uses the address matching rules described in 9.33.4 (Addressing and forwarding of individually addressed Mesh Data frames), when it receives an individually addressed frame. When a mesh STA receives a frame with the Address 1 field equal to a group address, the mesh STA also checks the TA to determine whether the group addressed frame originated from one of its peer mesh STA; if there is no match, the STA shall discard the frame. A mesh STA also uses the address matching rules described in 9.33.5 (Addressing and forwarding of group addressed Mesh Data frames).If the Address 1 field of an MPDU carrying an A-MSDU does not match any individual address at a receiving STA, then the entire A-MSDU is discarded. |
| --- | --- | --- | --- | --- |

Context: (549.54)

A STA uses the contents of the Address 1 field to perform address matching for receive decisions. A mesh STA also uses the address matching rules described in 9.33.4 (Addressing and forwarding of individually addressed Mesh Data frames), when it receives an individually addressed frame. When a STA other than mesh STA (nonmesh STA) receives a frame with the Address 1 field equal to a group address, the STA also validates the BSSID to verify either that the group addressed frame originated from a STA in the BSS of which the receiving STA is a member, or that it contains the wildcard BSSID value,indicating a Data frame sent outside the context of a BSS (dot11OCBActivated is true in the transmitting STA). When a mesh STA receives a frame with the Address 1 field equal to a group address, the mesh STA also checks the TA to determine whether the group addressed frame originated from one ofits peer mesh STA; if there is no match, the STA shall discard the frame. A mesh STA also uses the address matching rules described in 9.33.5 (Addressing and forwarding of group addressed Mesh Data frames).

Proposed change: (1099.31)

9.2.8 MAC data service The MAC data service provides the transport of MSDUs betweenMAC peer entities as characterized in 5.1.1 (Data service). The transmission process is started by receipt of an MA-UNITDATA.request primitive containing an MSDU and the associated parameters. This might cause one or more Data MPDUs containing the MSDU to be transmitted following A-MSDU aggregation, fragmentation, and security encapsulation, as appropriate. The MA-UNITDATA.indication primitiveis generated in response to one or more received Data MPDUs containing an MSDU following validation, address filtering, decryption, decapsulation, defragmentation, and A-MSDU deaggregation, as appropriate. Address filtering is performed on the Address 1 field of each MPDU contained in a PPDU and on the DA of each MSDU within an A-MSDU. When the Address 1 field or DA field contains a group address, address filtering is performed by comparing the value in the Address 1 field or DA field to all values in the dot11GroupAddressesTable, and the STA also validates the BSSID to verify either that the group addressed frame originated from a STA in the BSS of which the receiving STA is a member, or that it contains the wildcard BSSID value, indicating a Data frame sent outside the context of a BSS (dot11OCBActivated is true in the transmitting STA). A mesh STA also uses the address matching rules described in 9.33.4 (Addressing and forwarding of individually addressed Mesh Data frames), when it receives an individually addressed frame. When a mesh STA receives a frame with the Address 1 field equal to a group address, the mesh STA also checks the TA to determine whether the group addressed frame originated from one of its peer mesh STA; if there is no match, the STA shall discard the frame. A mesh STA also uses the address matching rules described in 9.33.5 (Addressing and forwarding of group addressed Mesh Data frames). If the Address 1 field of an MPDU carrying an A-MSDU does not match any individual address at a receiving STA, then the entire A-MSDU is discarded. In a QoS STA, the TID parameter of the MA-UNITDATA.request primitive results in a TID being specified for the transmitted MSDU. This TID associates the MSDU with the AC or TS queue for the indicated traffic.

Proposed Resolution:

Accepted.

| 2495 | 561.36 | 8.3.3.6 | EDCA Parameter Set is not always present in the (Re-)Association Response frame. | Add the following into the Note column of Table 8-23 and Table 8-25."The EDCA Parameter Set element is present ifdot11QosOptionImplemented is true and dot11MeshActivated is false." |
| --- | --- | --- | --- | --- |

Discussion:

There are a bunch of conditionals for this parameter/element as follows:  154.33 (Associate Confirm), 160.60 (Associate Response), 167.36 (Reassociate confirm),  173.57 (Reassociate response)

Specifies the EDCA parameter set that the STA should use. The parameter is present if dot11QosOptionImplemented is true; otherwise not present.

At 557.20 (Beacon frame body)

The EDCA Parameter Set element is present if dot11QosOptionImplemented is true, and dot11MeshActivated is false, and the QoS Capability element is not present.

At 569.16 (Probe Response)

The EDCA Parameter Set element is present if dot11QosOptionImplemented is true and dot11MeshActivated is false.

The location cited in the comment relates to the Association Response frame.

As far as I can tell the exclusion of the QoS Capability element enables  a Beacon to include an EDCA Parameter Set element in a subset of beacons.  So that shouldn’t affect the Association Response.

The MCF text doesn’t reference this element,  so I think it is clear that it is specific to infrastructure BSS.

The difference between probe response and (re-)associate primitives is correct,  as the latter are non-mesh specific.

Proposed resolution:

Revised.

Add the following into the Note column of Table 8-27 and Table 8-29."The EDCA Parameter Set element is present if dot11QosOptionImplemented is true; otherwise not present."

| 2016 | 578.02 | 8.3.4.1 | "The value of this field is in the range of 1 to 16, with the value being equal to the bit representation plus 1." -- How can the value of the field be different from the value represented by its bits? | Replace para with: "The FSS field specifies the number of SSW frames allowed per sector sweep slot (9.36.5 (Beamforming inA-BFT)) minus one. The range of this field is 0 to 15. For example, the number of SSW frames allowed per sector sweep is 5, the field contains the value 4." |
| --- | --- | --- | --- | --- |

Context: 578.06,  plus change proposed by commenter

The FSS field specifies the number of SSW frames allowed per sector sweep slot minus one (9.36.5 (Beamforming in A-BFT)). The range of this field is 0 to15 For example, when the number of SSW frames allowed per sector sweep is 5, the field contains the value 4.

Proposed resolution:

Revised.

Change cited text to read:

The FSS field specifies the number of SSW frames allowed per sector sweep slot minus one (9.36.5 (Beamforming in A-BFT)). The range of this field is  0 to 15. For example, when the number of SSW frames allowed per sector sweep is 5, the field contains the value 4.

| 2018 | 580.31 | 8.4.1.3 | Why is this a note? It is surely part of the specification of this field. | Promote to body text. |
| --- | --- | --- | --- | --- |

Context:

NOTE—A value of 0 in the Beacon Interval field transmitted by a DMG STA indicates that the TBTT of the next BTI is unknown.

Proposed resolution:

Accepted.

| 2324 | 589.33 | 8.4.1.10 | Normative verb in a definition. | Replace "may" with "can". |
| --- | --- | --- | --- | --- |

Context: 589.30 (and proposed change)

The allocation or TS has not been created because the request cannot be honored; however, a suggested TSPEC/DMG TSPEC is provided so that the initiating STA can attempt to set another allocation or TS with the suggested changes to the TSPEC/DMG TSPEC

Discussion:

“can” is appropriate,  because the mechanism being described explicitly supports that functionality.

Proposed resolution:

Accepted

| 2019 | 592.31 | 8.4.1.11 | "The Column labelling "Action frame" is non-intuitive. | Rename column "Per Action-frame exceptions." |
| --- | --- | --- | --- | --- |

Discussion:

The commenter aparently didn’t read 592.18: “The “Action frame” column in Table 8-43 (Category values) identifies exceptions, if any, that specific frames within a category have with respect to the “Robust” column.”

The change would claim to make the column more useful for general exception.

The one entry in that column at 592.51: “DMG: Link Measurement Request and Link Measurement Report are not robust frames” adequately establishes the nature of the exception.

So,  the change can be made to the column heading,  but the sentence at 592.18 needs also to be modified to reflect the implied semantics.

Propose resolution:

Revised.   Remove the sentence at 592.18.  Remove the “Action frame” column.

In Robust cell for Radio Measurement, replace contents of cell with “See NOTE 1”.   Insert new NOTE 1 in table footer to read:  “Radio measurement frames are Robust, except for Link Measurement Request and Link Measurement Report in a DMG BSS.”  Renumber existing NOTE as appropriate.

| 2325 | 596.52 | 8.4.1.17 | Normative verb in a definition. | On lines 52, 63 and 65, and on page 597 lines 6 and 8, replace "may" with "can". |
| --- | --- | --- | --- | --- |

Context (and proposed changes): 596.51

The Max SP Length subfield is 2 bits in length and indicates the maximum number of total buffered MSDUs, A-MSDUs, and MMPDUs the AP can deliver to a STA during any SP triggered by the STA. This subfield is reserved when the APSD subfield in the Capability Information field is equal to 0. If the APSD subfield in the Capability Information field is equal to1, the settings of the values in the Max SP Length subfield are defined in Table 8-44 (Settings of the Max SP Length subfield).

596.63:

AP can deliver all buffered MSDUs, A-MSDUs, and MMPDUs. AP can deliver a maximum of two MSDUs, A-MSDUs, and MMPDUs per SP.

597.06:

AP can deliver a maximum of fourMSDUs, A-MSDUs, and MMPDUs per SP. AP can deliver a maximum of six MSDUs, A-MSDUs, and MMPDUs per SP.

Discussion:

The normative effect of this field is established at 1379.45:

At each unscheduled SP for a STA, the AP shall attempt to transmit at least one BU, but no more than the value specified in the Max SP Length field in the QoS Capability element from delivery-enabled ACs, that are destined for the STA.

But,  I believe that the whole description of these fields is wrong.   The focus should not be on what the AP is allowed to do (which moves us into “quasi-normative” statements),  but on what the STA is capable of doing.

So,  I propose to reword these on that basis thus:

596.51

The Max SP Length subfield is 2 bits in length and indicates the maximum number of total buffered MSDUs, A-MSDUs, and MMPDUs the STA is prepared to receive during any SP triggered by the STA. This subfield is reserved when the APSD subfield in the Capability Information field is equal to 0. If the APSD subfield in the Capability Information field is equal to1, the settings of the values in the Max SP Length subfield are defined in Table 8-44 (Settings of the Max SP Length subfield).

596.63:

The STA is prepared to receive all buffered MSDUs, A-MSDUs, and MMPDUs. The STA is prepared to receive a maximum of two MSDUs, A-MSDUs, and MMPDUs per SP.

597.06:

The STA is prepared to receive a maximum of four MSDUs, A-MSDUs, and MMPDUs per SP. The STA is prepared to receive a maximum of six MSDUs, A-MSDUs, and MMPDUs per SP.

Proposed resolution:

Make changes in <this-document> under CID 2325.   These changes reword the cited locations to describe what the STA is capable of, not what the AP is limited to doing.

| 2022 | 626.37 | 8.4.2.1 | Name collision: Wakeup Schedule | Rename DMG Wakeup Schedule and adjust all uses in .11ad material. |
| --- | --- | --- | --- | --- |

Discussion:

The comment is correct, and the proposal acceptable.  But it needs specific locations.

Proposed Resolution:

Revised. Change “Wakeup Schedule” to “DMG Wakeup Schedule” at the following locations:

137.31, 626.37, 901.26 (and rest of subclause), 1063.22 (and rest of subclause), 1063.45 (and rest of subclause), 1078.52 (and rest of subclause), 1080.10 (and rest of subclause), 1399.51 (and rest of 10.2.6, including Figure 10-11), 1429.44, 1430.43

| 2453 | 632.31 | 8.4.2.6 | P634L47 (D1.6) "shall" is inappropriate in clause 8. | Change "shall encode" to "encodes" |
| --- | --- | --- | --- | --- |

Context: 632.29:

Based upon its knowledge of the capability of associated stations to support the multiple BSSID capability, as indicated by the corresponding field in the Extended Capabilities element and the content of the traffic indication virtual bitmap, an AP shall encode the Partial Virtual Bitmap and the Bitmap Control field of the TIM element using one of the two following methods. Specifically, an AP uses Method B when it determines that the bit for each associated non-AP STA in the traffic indication virtual bitmap that is reconstructed by each non-APSTA from the received TIM element encoded using Method B is set correctly. Otherwise, an AP uses Method A.

Proposed resolution:

Accepted

| 2326 | 634.02 | 8.4.2.9 | Normative verb in a definition. | Replace "may" with "might". |
| --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 2327 | 639.61 | 8.4.2.18 | Normative verb in a definition. | Replace "may" with "might". |
| --- | --- | --- | --- | --- |

Proposed resolution:

Revised.  Replace “may” with “can”.  This verb is appropriate because the cited text is followed immediately by a reference to text that indicates the circumstances under which it is present.

| 2328 | 643.46 | 8.4.2.20.2 | Normative verb in a definition. | Replace "may indicate" with "indicates". |
| --- | --- | --- | --- | --- |

Context: 643.47:

A Measurement Type in the Measurement Request element may indicate a basic request.

Discussion:

I think this is a really confusing statement.    There is no such thing as “A Measurement Type”.

I believe this sentence intends to say “One of the possible values of the Measurement Type field is basic request.”  Well, that adds nothing over and above Table 8-66.

Propose resolution:

Revised.  Delete the first sentence of the paragraph at the cited location.

| 2331 | 650.08 | 8.4.2.20.7 | Normative verb in a definition. | Replace "may" with "might". |
| --- | --- | --- | --- | --- |

Context: 650.07:

The Beacon Reporting subelement indicates the condition for issuing a Beacon report. The Beacon Reporting subelement may be included in a Beacon Request only for repeated measurements.

Discussion:

I believe “might” is wrong.   I think this is trying to say something stronger than might – “may .. only”.

We can also move a bit closer to preferred style by using “present” rather than “included”.

Proposed resolution:

Revised.  Replace sentence with:

“The Beacon Reporting subelement is optionally present in a Beacon Request for repeated measurements; otherwise not present.”

| 2332 | 674.39 | 8.4.2.21.2 | Normative verb in a definition. | Replace "may indicate" with "indicates". |
| --- | --- | --- | --- | --- |

Propose resolution:

Revised.  Delete the first sentence of the paragraph at the cited location.

| 2333 | 675.20 | 8.4.2.21.2 | Normative verb in a definition. | On lines 20 replace "may indicate" with "indicates" and on line 24 replace "may be set" with "is set". |
| --- | --- | --- | --- | --- |

Context: 675.17: (and proposed changes)

--OFDM preamble bit, which is set to 1 when at least one sequence of short training symbols, as defined in 18.3.3 (PHY preamble (SYNC)), was detected in the channel during the measurement period without a subsequent valid SIGNAL field (see 18.3.4 (SIGNAL field)).This indicates the presence of an OFDM preamble, such as high-performance RLAN/2 (HIPERLAN/2). Otherwise, the OFDM preamble bit is set to 0. — Unidentified Signal bit, which is set to 1 when, in the channel during the measurement period, there is significant power detected that is not characterized as radar, an OFDM preamble, or a valid MPDU. Otherwise, the Unidentified Signal bit is set to 0. The definition of significant power is implementation dependent.

Proposed resolution:

Accepted

| 2026 | 691.64 | 8.4.2.21.9 | Figure 8-185 and surrounding claim to show a subelement, but don't look like a subelement. | Either show the whole structure, (compliant with WG11 style rules) or reword as "payload of the .. subelement" |
| --- | --- | --- | --- | --- |

Discussion:

The changes shown below are purely editorial in nature.

However, we might also want to address 692.44:

“In a nontriggered STA Statistics report, all fields in the Reporting Reason subelement are set to 0.”

The question is what purpose is there in having this subelement present when it conveys no information.  The other question is whether this subelement is always present in the triggered STA Statistics report.  I can find nothing that clarifies this status.

Proposed resolution:

Revised.   Replace “The Reporting Reason subelement” by “The Data field of the Reporting Reason subelement” at 691.47, 691.64 and 691.44.

And at 692.44 change “all fields in the Reporting Reason subelement” to “all subfields of the Data field of the Reporting Reason subelement”.

| 2336 | 715.10 | 8.4.2.24.1 | "contiguous" means either "sharing a border" or "near another entity". What border is shared or other entities near a range of discrete values? In ordinary English a set of whole numbers can be 'continuous', but mathematics (and thus engineering) doesn't use this concept of continuity. | Delete the sentence "The range of Version field values a STA supports is contiguous." If the 'continuity' (in the ordinary sense) of the supported whole numbers is critical, replace that sentence with: "A STA shall support consecutive set of version numbers." |
| --- | --- | --- | --- | --- |

Context: 715.10

The Version field indicates the version number of the RSNA protocol. The range of Version field values a STA supports is contiguous. Values 0 and 2 or higher of the Version field are reserved. RSN Version 1 is defined in this standard.

Discusion:

What a STA supports or does not support is not a Clause 8 concern,   except how to signal the degree of support.  Furthermore, only one value is defined in this standard, so any concern about whether multiple values are “contiguous” is trying to create a constraint on the writers of future amendments.  They may have other ideas…

Proposed resolution:

Revised.   Replace para with:

The Version field indicates the version number of the RSN protocol. Version 1 is defined in this standard. Other values are reserved.

| 2027 | 731.59 | 8.4.2.29 | Basing the format of an element on whether is is transmitted in a particular band is suboptimal from general principles. The general principle is that an MPDU needs to identify its own structure, minimizing the need for additional context.The specific reason why this is bad for a TSPEC is that is a TSPEC is communicated over the DS (e.g. by an FT STA), then the format is ambiguous.Further, the only apparent difference is the existence of the DMG attributes field. | Change to a single figure showing a DMG Attributes field of length 0 or 2 octets. Explain that this field is present in a TSPEC transmitted by a DMG STA, and its presence can be determined from the length of the element. |
| --- | --- | --- | --- | --- |

Dislaimer:  this is my comment.

Proposed Resolution:

Revised.

Delete figure 8-226.   Delete “(non-DMG)” from figure 8-225.   Copy “DMG Attributes” field from figure 8-226 to the end of figure 8-225, and modify size to “0 or 2”.

Delete: “when the element is transmitted in a non-DMG BSS and in Figure 8-226 (TSPEC element format (DMG)) when the element is transmitted in a DMG BSS”

After: “The DMG Attributes field is defined in Figure 8-229 (DMG Attributes field format).”, insert:  “The DMG Attributes field is present in a TSPEC when the BSS to which the TSPEC applies is a DMG BSS;  otherwise absent.”

Notes in text, tables, and figures of a standard are given for information only and do not contain requirements needed to implement the standard.

Information on references can be found in Clause 2 (Normative references).

The numbers in brackets correspond to the numbers of the bibliography in Annex A.

Description of these primitives can be found in 6.3.4 (Synchronization).

Information on references can be found in Clause 2 (Normative references).