IEEE P802.11
Wireless LANs

| 802.11 REVmc Sponsor first recirculation ballot (SB1) - some proposed comments resolutions (Stephens) – Part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-02-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This document contains some proposed resolutions to SB1 comments.

The comments trivial technical comments that are “owned” by EDITOR.

# Comments

## Frame Formats

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7781 |  |  |  | The $Foo frame Action field does not include VSIEs, MICEs or AMPEEs (per 9.3.3.13) | Delete the rows with "Last" in the left-hand column at each of 1209.25, 1210.50, 1211.43 |  | EDITOR |

Discussion:

Agree, these “orders” are already included in the Action frame.  See 646.35.

Proposed Resolution:

Accepted.

| 7779 |  |  |  | "Vendor Specific" should not appear in $Foo frame Action field formats, since the VSIEs are in the Action frame not the Action field (per 8.3.3.13/14) | Delete the rows at 1209.25, 1210.50, 1211.43, 1147.24, 1187.18, 1188.32 |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The Vendor Specific “order” is already included in the Action frame.  See 646.35.

1147.24, 1187.18, 1188.32 are  incorrect because this is a subelement ID definition.

Proposed Resolution:

Revised.  Delete the rows at 1209.25, 1210.50, 1211.43.

No change is made at 1147.24, 1187.18, 1188.32 because these locations do not describe an Action field.

(Note to editor, these changes are a subset of those made in CID 7781)

| 7306 |  |  |  | Not all the "Optional subelement IDs for" tables have an Extensible column | Either add one to all tables, or have a statement that if there is no such column none of the elements are extensible |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

At 1074.10:

9.4.3 Subelements Subelements are defined to have a common general format consisting of a 1-octet element-specific Subelement ID field, a 1-octet Length field, and a variable-length subelement-specific Data field. Each subelement is assigned a subelement ID that is unique within the containing element or subelement. The Length field specifies the number ofoctets in the Data field. See Figure 9-587 (Subelement format). At the location in this standard that a subelement is defined, the definition might indicate if the subelement is extensible, typically using a table column called “Extensible” in the table in which subelement IDs are defined. A subelement that is indicated as extensible (typically with “Yes” in the “Extensible” column) might be extended in future revisions or amendments of this standard. A subelement that is indicated as extensible through subelements (typically with “Subelements” in the “Extensible” column) might be extended in future revisions or amendments of this standard by defining (additional) subelements within the subelement. If the definition of a subelement does not indicate whether it is extensible (e.g. there is no “Extensible” column in a table defining it) that subelement is not extensible. A subelement that is not defined as extensible will not be extended in future revisions or amendments of this standard.

Proposed Resolution:

Revised.

At 1074.26 add a new para: “If the definition of a subelement does not indicate whether it is extensible (e.g. there is no “Extensible” column in a table defining it) that subelement is not extensible.”

| 7003 | 572.57 | 9.2.4.3.2 |  | "Each Address field contains a 48-bit address as defined in 9.2 of IEEE Std 802-2014."Subclause 9.2 of 802-2015 is "Ethertypes", so the reference is wrong.Ditto at 1603.20. | Change reference to "Clause 8", as there is no subclause of Clause 8 that is specifically for 48-bit addresses. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Note, correct Doc is IEEE Std 802-2014.

The TOC looks like this:

Proposed Resolution:  Accepted

| 7047 | 578.41 | 9.2.4.5.4 |  | "There may be a response frame to the frame that is received, but it is neither the Ack frame nor any Data frame of subtype +CF-Ack." - normative verb in Clause 9 | Change "may" to "might" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:  Accepted

| 7048 | 579.18 | 9.2.4.5.6 |  | "The AP may use information contained in the Queue Sizesubfield to determine the TXOP duration assigned to the STA." - normative verb in clause 9 | change "may" to "might" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:  Accepted

| 7050 | 582.11 | 9.2.4.6.1 |  | "The response to a reverse direction grant (RDG) may contain Data frames from any TID" - normative verb in Clause 9. | Change "can" and cite the subclause defining this behavior.Make similar change at line 14. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 582.06:

Proposed Resolution:

Revised.

At 582.11 and 582.14 change “may contain” to “contains”.  At the end of the description, add “, see 10.28.4.”

| 7203 | 582.40 | 9.2.4.6.1 |  | What does the bit after the comma mean in "An RDG is present, as defined by the Duration/ID field"? | Delete ", as defined by the Duration/ID field" |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7557 | 592.33 | 9.2.4.7.3 | J | " If theAddress Extension Mode is "None", the Mesh Address Extension subfield is not present. For values"Address4" and "Address5&6", the Mesh Address Extension subfield is present following the MeshSequence Number subfield. " duplicates the info in the table | Delete the cited sentences |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  There is certainly a flavour of repition here, but in my mind it is acceptable because the two instances are arguably at different levels of abstraction.

Proposed Resolution:

Rejected. The cited text is correct. The cited text describes conditions under which the field is present. The following table describes the contents in more detail.

| 7051 | 593.22 | 9.2.4.7.3 |  | "The active path selection protocol maydefine additional parameters in the forwarding information." - normative verb in Clause 9 | change "may" to "might" in cited text.Make same change at 618.11, 621.41, 623.37 |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited occurences of “may” are actually a xref that went bad.  Comment CID 7116 fixes these.

Proposed Resolution:

Revised. These locations are damaged cross-references.

Replace “(The active path selection protocol may define … in 14.10.8.4 (Forwarding information).)”

with a cross-reference to 9.3.5.

(Note to editor, this is the same change as in CID 7116).

| 7115 | 612.59 | 9.3.1.14 |  | "individual (group) address" - there is no such thing. This is trying to say something more subtle. | Reword to avoid subtlety, describing conditions for individual and group separately. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 612.58

For DMG CTS-to-self frames, the TA field is set to the individual (group) address of the recipient(s) of the frame that the DMG STA intends to transmit after the DMG CTSto-self frame

Proposed Resolution:

Revised.  Change at the cited location “set to the individual (group) address of the recipient(s) of the frame”

to “set to the individual address of the recipient or the group address of the recipients of the frame”

| 7052 | 618.55 | 9.3.2.1 |  | "One or both of these fields may also be present" - normative verb in Clause 8 | Change "may" to "can" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 618.53:

When a Data frame carries an A-MSDU, the DA and SA values related to each MSDU carried by the A-MSDU are carried within the A-MSDU.One or both of these fields may also be present in the Address 1 and Address 2 fields as indicated inTable 9-26 (Address field contents).

Proposed Resolution:

Revised.  Change cited sentence to read:

“Zero, one or both of these fields are present in the Address 1 and Address 2 fields …”

| 7416 | 619.16 | 9.3.2.1 |  | In "The QoS Control field is defined in 8.2.4.5 (QoS Control field)." should add "The presence of the QoS Control field is determined by the Subtype subfield of the Frame Control field, as specified in 9.2.4.1.3." to be consistent with the two similar bits of wording for the HT Control field. | As it says in the comment |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The change is consistent with adjacent wording.

Context 619.16:

The Sequence Control field is defined in 9.2.4.4 (Sequence Control field). The QoS Control field is defined in 9.2.4.5 (QoS Control field). The HT Control field is defined in 9.2.4.6 (HT Control field). The presence ofthe HT Control field is determined by the +HTC/Order subfield of the Frame Control field, as specified in 9.2.4.1.10 (+HTC/Order subfield).

Proposed Resolution:

Revised.

After cited sentence add: “The presence of the QoS Control field is determined by the Subtype subfield of the Frame Control field, as specified in 9.2.4.1.3.”

| 7531 | 622.12 | 9.3.3 |  | The IEEE 802.11 standard defines a single Vendor Specific element (221). However, there are many earlier proprietary elements whose Element ID is listed as Reserved in the standard, e.g. 133. Most of the Management frame formats list that Vendor Specific elements can optionally be included after all of the elements listed in the standard. However, I cannot find any clarification on whether the Reserved Element IDs should be included in that category, or whether it is just Element ID 221 | Clarify |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

A STA that transmits a frame using a reserved value is non-compliant to the standard.

The 802.11 standard does not describe any non-compliant operation (i.e., where a STA might transmit a reserved Element ID). The 802.11 standard does not describe how to respond to non-compliant peers.

Any any manufacturer or group of manufacturers wants to use a value that is reserved for their use,  the onus is surely on them to describe fully how it is used, including relative element transmit ordering.

Proposed Resolution:

Rejected.

The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

| 7628 | 622.33 | 9.3.3.2 |  | "In an MMPDU carried in one or more PPDU(s), none of which are VHT PPDU(s), the maximum unencrypted MMPDU size is 2304 octets." is duplication of T9-19 | Delete this sentence |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 622.25:

The format of a Management frame is defined inFigure 9-57 (Management frame format). The Frame Control, Duration, Address 1, Address 2, Address 3, and Sequence Control fields are present in all management frame subtypes. In an MMPDU carriedin one or more non-VHT PPDUs the maximum MMPDU size is specified in Table 9-19 (Maximum data unit sizes (in octets) and durations (in microseconds)). In an MMPDU carriedin one or more PPDU(s), all of which are VHT PPDU(s), the maximum MMPDU size specified in Table 9-19 (Maximumdata unit sizes (in octets) and durations (in microseconds)) is the maximum MPDUsize supported by the recipient(s) less the shortest management frame MAC header and FCS. In an MMPDU carried in one or more PPDU(s), none of which are VHT PPDU(s), the maximum unencrypted MMPDU size is 2304 octets.

Proposed Resolution:

Rejected.  The cited text extends Table 9-19 to cover also the case when an MMPDU is fragmented, and the fragments are carried in a mixture of VHT and non-VHT PPDUs.

| 7053 | 624.29 | 9.3.3.2 |  | "Gaps may exist in the ordering of fields and elements within frames" - normative verb in Clause 9 | Change "may" to "might" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7338 | 644.21 | 9.3.3.12 |  | Table 9-36---Presence of fields and elements in Authentication frames has "Status" in some cells under "Status Code". This is useless. It should either be "Any" or an explicit list of the permissible statuses. | Change "Status" to "Any" when it appears alone in a cell |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Worth having a discussion on.   I agree with the direction of the change.

However, is “Any” status code truly allowed, for example in a Shared Key sequence number 2 frame,  or only a subset of the available status codes.

The point is that the “Status code” column is trying to indicate two things:

- When the field is reserved or not

- For specific status codes,  how this affects fields 4 onwards

Proposed Resolution:

Accepted.

| 7055 | 647.15 | 9.3.3.15 |  | "One or more vendor-specific elements may appear in this frame" - normative verb in Clause 9 | Change cited text to: "One or more vendor-specific elements are optionally present" |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7056 | 653.01 | 9.3.5 |  | "A source mesh STA may be a meshSTA that is the initial source of an MSDU/MMPDU or a mesh STA that receives an MSDU/MMPDU froma mesh path or from a STA outside the mesh BSS and translates and forwards the MSDU/MMPDU on themesh path." - normative verb in Clause 9 | change "may be" to "is" in cited text.Make same change at line 7. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.  Replace “may be” with “is either” at 653.01 and 653.07.

| 7057 | 699.29 | 9.4.1.49 |  | "may take a value between 2 and 8" - normative verb in Clause 8 | change "may take" to "takes" in cited text |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7058 | 701.03 | 9.4.1.49 |  | " A beamformee may choose to reduce Ns by using a method referred to as grouping" - normative verb in clause 9 | change "may" to "might" in cited text |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7185 | 715.30 | 9.4.1.53 |  | The 10.4.8 is not a hyperlink | Make it a hyperlink, and fix the ref (it's 11.4.8) |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  The corrected reference is 11.40.8 Extended NSS BW Support Signaling, not 11.4.8,  as stated.

Proposed resolution:

Revised. At 715.30 change “10.40.8 (Extended NSS BW Support Signaling)” to be a ‘live’ cross-reference to 11.40.8.

| 7659 | 716.13 | 9.4.1.53 | V | What is "OMN"? | Change to "Operating Mode Notification frame" |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

REVISED. Change "OMN" to "Operating Mode Notification field".

In reply to the commenter, this parallels the language used in the table caption.

| 7252 | 718.32 | 9.4.1.56 | V | "The structure and information fields are little endian, per conventions defined in9.2.2 (Conventions)." -- since this is the default, should not be stated (an a space is missing anyway) | Delete the sentence |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

REVISED. Insert "NOTE--" at the start of the para. Insert the missing space. Move to 718.65 (at the end of the subclause).

| 7004 | 733.24 | 9.4.2.6 |  | There are some flags. | Request values from the 802.11 ANA and insert them in the draft wherever there is an flag. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7117 | 739.15 | 9.4.2.11 |  | "an element ID of 255the value of the Requested Element ID field" - makes no sense | delete "an element ID of 255" |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 739.15

The Requested Element ID Extensionsfield contains a list of 1-octet element ID extension values that, combined with an element ID of 255the value of the Requested Element ID field, identify elements that are requested to be included in the Probe Response or Information Response frame. The values in this field are listed in increasing order.

Proposed resolution:

Accepted.

| 7005 | 776.07 | 9.4.2.21.17 |  | "The Optional Subelements field contains zero or more subelements."I beg to differ. It contains precisely nothing, because no subelements are defined. | Remove the Optional Subelements field from Figure 9-185 and the para at the cited line. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7006 | 776.60 | 9.4.2.21.18 |  | "The Optional Subelements field contains zero or more subelements."No it doesn't, because none are defined that it can contain. | Remove cited para. Remove "Optional Subelements" field from figure 9-186. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7091 | 777.61 | 9.4.2.21.19 |  | "The FTM Range Subelements field shall include a concatenation of at least Minimum AP Count NeighborReport subelements." -- Normative verb in Clause 9 | Change "shall include" to "includes" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7059 | 783.24 | 9.4.2.22.4 |  | "The STA may use this information to assist in the choice of new channel" - normative verb in clause 9 | change "may" to "might" in cited text |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7060 | 827.22 | 9.4.2.23 |  | "Thisinterval may be used to assist inmaking channel measurements without interference from other STAs" - normative verb in clause 9 | change "may" to "might" in cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7063 | 837.44 | 9.4.2.26 |  | "Multiple Vendor Specific elements are optionally present in a single frame." -- this whole para really doesn't belong in a description of the format of the element.Also normative verb in Clause 9. | Move this description to the clauses that define the containing object (i.e. Action frame, Action No Ack frame). And reword to avoid "may".Also search out all locations that include a vendor specific element and ensure they allow zero or more instances. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 837.44:

Multiple Vendor Specific elements are optionally present in a single frame. Each Vendor Specific element might have a different Organization Identifier value.The number of Vendor Specific elements that may appear in a frame is limited only by the maximum frame size.

Context: 646.35: (Action frame format)

One or more vendor-specific elements are optionally present. These elements are absent when the Category subfield of the Action field is Vendor-Specific, Vendor-Specific Protected, or Self-protected or when the Category subfield of the Action field is VHT and the VHT Action subfield of the Action field is VHT Compressed Beamforming.

Discussion:

The “multiple” aspect of this is covered already at 646.35.

The “Each Vendor Specific element might have a different Organization Identifier value” deserves at most to be a NOTE.  Absent a statement that adjacent Vendor Specific elements are constrained in some way, the assumption should be that they are not.

(To “Reduce this to absurdity”,  it is not necessary to state that if a VSIE of length 40 B is present,  it is permitted to have another one of a different length.)

And, finally, “The number of Vendor Specific elements that may appear in a frame is limited only by the maximum frame size.” is wrong.  It is limited by MMPDU size.  And MMPDU max size can, under some circumstances be bigger than max MPDU size.  The limit is MMPDU size,  although packing a frame that would otherwise not be fragmented with VSIE elements, thereby forcing it to fragment would doubtless uncover all kinds of wrong implementation assumptions :0).

Proposed resolution:

Revised.  Delete para at 837.44.

| 7064 | 843.52 | 9.4.2.28 |  | "This element maybe used by the STA for vendor-specific AP selection algorithm when roaming" - normative verb in clause 9 | Change "may" to "might" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 7076 | 845.26 | 9.4.2.29 |  | " The AIFSNsubfield indicates the number of slots after a SIFS a STA should defer before either invoking a backoff orstarting a transmission. The minimum value of the AIFSN subfield is 2." -- unnecessary normative verb in clause 9 | Change "should defer" to "defers" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7065 | 847.28 | 9.4.2.30 |  | "The TSPEC allows a set of parameters more extensive than may be needed, or may be available, for anyparticular instance of parameterized QoS traffic" - normative verb in clause 9 | Change "may" to "might" (twice) in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7067 | 850.22 | 9.4.2.30 |  | "that may elapse without arrival or transfer of an MPDU belonging to the TS before this TS is deleted by the MAC entity at the HC. " - normative verb in clause 9 | change "may" to "can" in cited text.Make same change at 850.26. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 850.20

The Inactivity Interval field is 4 octets long and contains an unsigned integer that specifies the minimum amount of time, in microseconds, that may elapse without arrival or transfer of an MPDU belonging to the TS before this TS is deleted by the MAC entity at the HC. The Suspension Interval field is 4 octets long and contains an unsigned integer that specifies the minimum amount of time, in microseconds, that may elapse without arrival or transfer of an MSDU belonging to the TS before the generation of successive QoS(+)CF-Poll is stopped for this TS.

Proposed resolution:

Accepted.

| 7068 | 850.37 | 9.4.2.30 |  | "This may help" - unnecessary use of normative verb | Change "may" to "might" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 7070 | 861.61 | 9.4.2.32 |  | "The TS Delay element is used in an ADDTS Response frame transmitted by an HC and indicates the timeafter which the ADDTS may be retried." - normative verb in clause 9 | Change "may" to "can" and cite subclause that defines this behaviour. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

There is no normative behaviour associated with the TS Delay element.  I’m tempted to delete it.  But,  assuming we can’t do that we should fix that.

Proposed Resolution:

Revised.  Change cited sentence to read:

“The TS Delay element is used in an ADDTS Response frame transmitted by an HC and indicates the time after which the ADDTS can be retried, see 11.4.7."”

At 1643.55 change “two” to “three”.

At 1643.60 add a new list item “c) An ADDTS Response frame is received that contains the Status Code field equal to REJECTED_FOR_DELAY_PERIOD. In this case, if the Delay field of the TS Delay element of the ADDTS Response frame is non-zero, the STA that transmitted the ADDTS Request frame shall not transmit another ADDTS Request frame to the same AP until the time indicated by the Delay field has elapsed.”

| 7071 | 862.22 | 9.4.2.33 |  | " It may also be present in the ADDTSRequest and ADDTS Response frames if there are no TCLAS elements." - normative verb in Clause 9 | Change "may also be present" to "is optionally present" |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7072 | 862.64 | 9.4.2.34 |  | "The information in this element may be used" - unnecessary normative verb | Change "may" to "might" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7073 | 863.52 | 9.4.2.34 |  | "The HC may set the Service Start Time field and the Service Interval field to 0 (unspecified) for nonpowersaving STAs," | Change "may" to "can" and cite the normative text that defines this behaviour. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7579 | 873.18 | 9.4.2.39 |  | " DCF or EDCAF services are " -- what's a DCF/EDCAF service? | Change to " DCF or EDCAF is " |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.  Change cited text to:  " DCF or EDCAFs are "

| 7009 | 896.36 | 9.4.2.56.5 |  | "May" in clause 9, in the definition column. | Change "may" to "might" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 896.32:

When transmitted by an AP: indicates the PCO Transition Time to be used during PCO operation. The value contained in this field is dynamic when transmitted by an AP, i.e., the value of this field may change at any time during the lifetime of the association of a STA with the AP. See 11.17.3 (Operation at a PCO active non-AP STA).

Proposed Resolution:

Accepted

| 7010 | 896.56 | 9.4.2.56.5 |  | "may" in Clause 9. | Change "may" to "might" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  “can” also works equally well here.

Context: 896.55:

Indicates support for acting as a reverse direction responder, i.e., the STA may use an offered RDG to transmit data to an RD initiator using the reverse direction protocol described in 10.28 (Reverse direction protocol)

Proposed Resolution:

Accepted.

| 7011 | 902.08 | 9.4.2.57 |  | "may" in clause 9, definition column. | Change "may" to "can", as this is reflecting normative behavior described in 11.16.12. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 902.07

Defines the channel widths that may be used to transmit to the STA. See 11.16.12 (Switching between 40 MHz and 20 MHz)

Proposed Resolution:

Accepted

| 7012 | 904.21 | 9.4.2.57 |  | "may" in Clause 9. | change "may" to "might" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 904.13:

Indicates whether PCO is active in the BSS Present in Beacon/Probe Response frames transmitted by an AP. Otherwise reserved. Non-PCO STAs regard the BSS as a 20/40 MHz BSS and may associate with the BSS without regard tothis field. See 11.17 (Phased coexistence operation (PCO)).

Proposed Resolution:

Accepted.

| 7013 | 904.49 | 9.4.2.57 |  | "may" in Clause 9. | Change "may" to either "might" or "can". |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 904.49

A non-HT BSS is overlapping (a non-HT BSS may be detectedby the reception of a Beacon or Probe Response frame that does not include an HT Capabilities element)

Proposed resolution:

Revised.  Change “may” to “might” at cited location.

| 7014 | 926.11 | 9.4.2.69.5 |  | "may" in Clause 9. | Reword to avoid normative language. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 926.11:

The following text describes the various subelements that may be included in Diagnostic Subelements field of a Diagnostic Request element (9.4.2.69) or a Diagnostic Report element (9.4.2.70). The format of a Diagnostic subelement is shown in Figure 9-370 (Diagnostic subelement format).

Proposed Resolution:

Revised.  At 926.11 change “may be” to “are optionally”.

| 7015 | 939.03 | 9.4.2.71.1 |  | "may be identified" - "may" in Clause 9 | Reword to avoid normative verb. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Commenter’s location is off by 10 pages.

The Antenna ID field doesn’t define modes of operation of the Antenna(s).

Modes of operation of the antenna is a concept not defined anywhere.

I think the intended meaning is “The Antenna Type field does not change for a specified Antenna ID if the undefined “mode” of operation changes for that antenna.

Context: 929.01

The Antenna Type field does not change based on different modes of operation of the antenna(s), as may be identified by the Antenna ID field (see 9.4.2.40 (Antenna element)).

Proposed Resolution:

Revised. Delete cited sentence.

| 7016 | 963.04 | 9.4.2.86 |  | "may be grouped" - "may" in Clause 9. | Replace "may" with "can" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7017 | 963.07 | 9.4.2.86 |  | "may be included" - "may" in Clause 9 | Replace "may" with "can" at cited location. |  | EDITOR |

Proposed Resolution:

Accepted.

| 7018 | 969.04 | 9.4.2.89 |  | "may be present" - "may" in Clause 9. | Replace "may be present" with "is optionally present" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7019 | 969.32 | 9.4.2.90 |  | "may use to deliver" - "may" in Clause 9. | Replace "may" with "can" or "might" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 969.31

The Destination URI element contains URI and ESS Detection Interval values from the requesting STA that the responding STA may use to deliver Event or Diagnostic Report frames. The format of the Destination URI element is given in Figure 9-436 (Destination URI element format).

Discussion:  the question is whether this “use” is described here or not.  There are a number of normative statements in 11.24 that cite this element,  so I believe the answer is yes.  This means we can use “can”.

Proposed Resolution:

Revised.  At 969.32,  change “may” to “can”.

| 7020 | 971.53 | 9.4.2.92 |  | "which may employ" - unnecessary use of normative verb | change "may" to "might" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 7021 | 971.60 | 9.4.2.92 |  | "Further information on types of charges may be available" - unnecessary use of normative verb | Change "may" to "might" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 7547 | 983.49 | 9.4.2.101 |  | "The congestion notification duration values are encoded as unsigned integers in units of 100 us." -- they can obviously not be negative | Change the para to "The element contains four Congestion Notification Duration fields for the four EDCA access categories to indicate the estimated congestion duration, in units of 100 s, for each AC at the mesh STA transmitting the congestion notification." |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Rejected.  The statement is correct.  There are many other instances of “unsigned integer” in this context in the standard, so there is no incentive to change this one.

| 7023 | 1004.06 | 9.4.2.121 |  | "This element may be included in ADDTS Request" - normative language in Clause 9 | Change "may be included" to "is optionally present" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 7546 | 1037.44 | 9.4.2.146 |  | "All numeric fields are encoded as unsigned integers." -- none of the fields can take a negative value anyway | Delete this sentence |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Rejected.  The cited statement is correct.  It does no harm to have this explicit statement of encoding, than having one implied by the range of the contents.

| 7186 | 1053.56 | 9.4.2.158.2 |  | None of the subclause references in "9.7.12.1 (Rx Supported VHT-MCS and NSS Set) and 9.7.12.2 (Tx Supported VHT-MCS and NSS Set) and 10.40.8 (Extended NSS BW Support Signaling)" are hyperlinks | Make them hyperlinks and fix the refs (e.g. the last one is 11.4.8). Check all the other subclause references in the change which added Extended NSS BW Support as it looks as if hyperlinking them all might have been overlooked |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.

At 715.10 delete the second “(Setting of the Channel Width subfield and Dynamic Extended NSS BW subfield at a VHT STA transmitting the Operating Mode field)”.  (This is plain text).

At 715.31 replace “10.40.8 (Extended NSS BW Support Signaling)” with a live link to 11.40.8.

At 1053.44 replace “8.4.2.157.3 (Supported VHT-MCS and NSS Set field)” with a live link to 9.4.2.158.3.

At 1053.56 replace “9.7.12.1 (Rx Supported VHT-MCS and NSS Set)” with a live link to 10.7.12.1

At 1053.56 replace “and 9.7.12.2 (Tx Supported VHT-MCS and NSS Set)” with “, “ followed by a live link to 10.7.12.2.

At 1053.58 replace “10.40.8 (Extended NSS BW Support Signaling)” with a live link to 11.40.8.

At 1049.54 repalace “Figure 8-553a” with a live link to Figure 9-558.

| 7024 | 1062.27 | 9.4.2.165 |  | "Furthermore, the QuietChannel element indicates the conditions under which the primary 80 MHz channel of the VHT BSS may beused during the quiet interval." - normative verb in Clause 9 | Change "may" to "can" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 7025 | 1062.31 | 9.4.2.165 |  | "The Quiet Channel element may be included in Beacon frames" - normative verb in Clause 9 | Change "may be included" to "is optionally present" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 7094 | 1072.24 | 9.4.2.172 |  | "When the Block Ack window size expected to be used by the transmitter of the element does not match any of the values shown in the table, the transmitter uses the next lower value in the table.""expected to be used by the transmitter" - who does the expecting? What does it mean for a STA to expect anything? | Reword to avoid anthropomorphic verbs and the passive voice. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

This author can see the commenter’s point.  However, there are 198 instances of “expected” and there is no point fixing up just one.

Proposed resolution:

Rejected.  The term “expected” is reasonable in the context of estimating or predicting throughput.

| 7026 | 1081.18 | 9.4.5.1 |  | "The ANQP-elements that may be configured" - normative verb in clause 9 | Change "may" to "can" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 7027 | 1081.26 | 9.4.5.2 |  | " Each ANQP-element may be returned in response" - normative verb in Clause 9 | Change "may" to "can" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion: The “Each ANQP-element” is meaningless.   The two sentences can be collapsed.

Context: 1081.25:

The Query List ANQP-element provides a list of identifiers of ANQP-elements for which the requesting STA is querying. Each ANQP-element may be returned in response to an Query List ANQP-element using the procedures in 11.25.3.2.2(Query List procedure)).

Proposed resolution:

Revised. Replace the whole para at 1081.25 with:

“The Query List ANQP-element provides a list of identifiers of ANQP-elements for which the requesting

STA is querying, see 11.25.3.2.2(Query List procedure)).”

| 7162 | 1081.48 | 9.4.5.2 |  | The use of the terms "ANQP query response" and "ANQP response" mean the same thing. When these are an action (as opposed to a Noun), the term should be "ANQP response", as the verb 'query' is already within the ANQP acronym - Access Network Query Protocol.The use of the terms "ANQP query", "ANQP Query" and "ANQP query request" all mean the same thing. When these are an action (as opposed to a Noun), the term should be "ANQP request", as the verb 'query' is already within the ANQP acronym - Access Network Query Protocol. | Change all occurances of "ANQP query response" to "ANQP response", when these terms are used as an action and not as Nouns. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

These terms are all used as nouns, so the proposed resolution has no effect.

If we change “ANQP query response”, we should do the same with “ANQP query request”.

So either:

Revised.  Change all “ANQP query request” to “ANQP request” and “ANQP query response” to “ANQP response”.

or:

Rejected. The comment is out of scope:  i.e., it is not on changed text, text affected by changed text or text that is the target of an existing valid unsatisfied comment.

| 7028 | 1082.42 | 9.4.5.4 |  | "Venue Name ANQP-element may be used to provide" - normative verb in Clause 9. | Change "may" to "might" at cited location. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7029 | 1082.42 | 9.4.5.4 |  | "For example, theinformation may be used to assist a user in selecting the appropriate BSS with which to associate. Zero ormore Venue Name fields may be included in the same or different languages." - two normative verbs | Change "may" to "might" at line 43. Change "may" to "can" at line 44. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7030 | 1084.56 | 9.4.5.6 |  | "Each Network Authentication Type Indicator defines additional information that may be communicated." - normative verb in Clause 9. | Change "may" to "can" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7031 | 1085.03 | 9.4.5.6 |  | "Higher layerprotocols on the non-AP STA may indicate to the user that accounts may be created." two normative verbs in Clause 9 in one sentence.Bingo! | Change "may" to "might" (twice) in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7032 | 1085.27 | 9.4.5.7 |  | " This list may be returned in response to a GAS Queryusing procedures in 11.25.3.2.3 (Roaming Consortium procedure). " - normative verb in Clause 9 | Change "may" to "can" in cited text.Make the same change at 1092.28. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7033 | 1093.44 | 9.4.5.16 |  | " the URI at which an EAS messagemay be retrieved as described in 11.25.7" - normative verb in Clause 9 | Change "may" to "can" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7035 | 1113.42 | 9.6.3.2.1 |  | "There may be one or more TCLAS elements in the ADDTS frame." - unnecessary normative verb in Clause 9. | Change "may be" to "are" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion: the TCLAS element is optional (see 1113.07).    “may be one or more” is equivalent to “are zero or more”.

Proposed resolution:

Revised. Change “may be one or more” to “are zero or more”.

| 7036 | 1113.45 | 9.6.3.2.1 |  | " There may be one Expedited Bandwidth Request element" - normative verb in Clause 9. | Replace cited text with "An Expedited Bandwidth Request element is optionally present"Make similar change at line 53. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7078 | 1114.44 | 9.6.3.2.2 |  | "The optional TSPEC element defines a TS that can use the allocation should the allocation be createdsuccessfully." -- unnecessary use of the normative verb "should" | change "should .. be" to "if .. is" |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7037 | 1119.01 | 9.6.3.6 |  | "There may be zero or more Intra-Access Category Priority elements," - normative verb in Clause 9. | Change cited text to "Zero or more Intra-Access Category Priority elements are present," |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7040 | 1147.23 | 9.6.8.17 |  | "The Vendor Specific subelement may be included" - unnecessary use of the normative verb. Also limits to a single copy of this subelement. | Change to "Zero or more Vendor Specific subelements are included"Make the same change at 1187.17, 1188.31. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7041 | 1150.62 | 9.6.8.23 |  | "The optional Avoidance Request field contains a TXOP Reservation field, as defined in 9.4.1.44 (TXOPReservation field) and may be present when the Status Code field is not SUCCESS" - normative verb in Clause 9 | Change "may be present" to "is optionally present" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7042 | 1187.14 | 9.6.14.6 |  | "The Location Indication Options subelement may beincluded" - normative verb in Clause 9.I also note, for completeness, that the WG11 style is "present" not "included" in this context. However, to minimize changes, I'm not proposing any change to "included" here, as it would need changes throughout this other table and similar tables that do not follow WG11 style. | Change "may be included" to "is optionally included".Make matching changes at: 1188.07, 1188.10, 1188.13, 1188.17 |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7044 | 1250.35 | 9.7.1 |  | "The length of an A-MPDU addressed to a particular STA may be further constrained as described in 10.13.2 (A-MPDU length limit rules)" - normative verb in Clause 9 | Change "may" to "can" in cited text. |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7045 | 1251.32 | 9.7.1 |  | "Pattern that may be used to detect an MPDU delimiter when scanning for an MPDU delimiter" - normative verb in Clause 9 | Change "may" to "can" |  | EDITOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

## Other comment groups

These are comments owned by EDITOR,  excluding MAC Operation & MAC Management,  not in the Editorials comment group and not marked submission required.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Comment Group | Ad-hoc Status | Ad-hoc Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7513 |  |  |  | "DSE enablement" (or without space and/or uppercase) is spurious since E stands for enablement | Delete the "enablement" in all cases. Also change "DSE disablement" to "DSD" and define the term just before DSE |  | Terminology |  | Changed to Terminology. Needs some thought. |

Discussion:

There are a bunch of “DSE” related terms.   “DSE” is used as an adjective, not a noun.

1	56	DSE Registered

2	29	DSE Enablement

3	27	DSE Power

4	19	DSE procedures

5	18	DSE power

6	17	DSE Measurement

7	15	DSE Deenablement

8	13	DSE measurement

9	11	DSE LCI

10	8	DSE registered

11	7	DSE bit

12	7	DSE3:M

13	6	DSE functions

14	6	DSE STA

15	5	DSE service

16	4	DSE Dependent

17	4	DSE frequency

18	4	DSE timer

19	4	DSE2:M

20	3	DSE_BAND

21	2	DSE deenablement

22	2	DSE dynamic

23	2	DSE field

24	2	DSE request

25	2	DSE time

26	2	DSE(MDR

27	2	DSE) in

28	2	DSE)). A

29	2	DSE; otherwise

30	1	DSE are

31	1	DSE dependent

32	1	DSE elements

33	1	DSE enablement

34	1	DSE is

35	1	DSE location

36	1	DSE operating

37	1	DSE status

I would not support changing some to DSD and others to DSE.  That way the cohesiveness of the terminology is lost.

This terminology has not changed in D5.0, so this comment is out of scope.

Proposed resolution:

Rejected. The comment is out of scope:  i.e., it is not on changed text, text affected by changed text or text that is the target of an existing valid unsatisfied comment.

| 7804 |  |  |  | "FST" is an action (fast session transfer), so what's an "FST session"? | Change all instances of "FST session" to "FST-capable association" |  | Terminology | Discuss | EDITOR: 2016-01-29 11:44:54Z - The proposed change is clear enough. There are 64 instances, including those in compound nouns such as "FST session setup protocol" and "FST session setup". The question is whether the group wants to make the change. I'd be tempted to say "no" with the rationale: 'Rejected. The "FST session" is a session that permits/supports/enables the action of "fast session transfer". The name is appropriate.' |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

We made changes in the last ballot “FST session transfer” => “FST”.  That probably invalidates a “scope” rejection.

It will require a little work to identify the different uses of “FST session” and exclude those that are used in compound nouns.  Should I do this work?

Straw poll: Prefer to:

A: Accept in principle the change as proposed

B: Reject the comment

| 7392 |  |  |  | There are a handful of "TDLS Peer Key"s | Change all of them to "TDLS PeerKey" |  | Terminology | Discuss | Changed to "Terminology" comment group. There is no such term as "TDLS PeerKey". |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

There are ~180 instances of PeerKey, and 10 of “Peer Key”.  So, clearly “PeerKey” is the name we use for this protocol and related concepts.

This is unfortunate.  Really it should be “peer key”, but let’s grandfather the awkwardness of “PeerKey”.

In the last ballot we changed “TDLS Peer Key” to “TPK”.  We missed one.

Proposed resolution:

Revised.

Change “Peer Key” to “PeerKey” at 61.24, 61.25, 80.54, 2030.41,

Change “TDLS Peer Key” to “TPK” at 2030.48.

| 7487 |  |  |  | The motioned resolution to CID 5235 per 15/0532r30 has not been effected | Implement the proposed resolution, namely "REVISED (EDITOR: 2015-07-17 02:57:38Z) - Replace para at 1709.21 with "A STA with dot11OCBActivated equal to true shall not join or start a BSS".Delete the para at 1709.26." (D4.0 refs, of course) |  | Editing defect from SB0 |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Goodness only knows how this didn’t get edited,  as the edit notes claim to show the change was reviewed.  Anyhow,  the commet is correct.

Proposed resolution: (D5.0 references)

Revised.

Replace para at 1739.21 with "A STA with dot11OCBActivated equal to true shall not join or start a BSS".

Delete the para at 1739.26.

| 7508 |  |  |  | The term "Vendor OUI" is used in 3 places, but not defined | Define it in Clause 3 as being an OUI other than 00-0F-AC |  | Terminology |  | Clarity/consistency (Small scope) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.

Insert in 3.2 in alphabetic order:

“vendor organizationally unique identifier(OUI):  An OUI is that is not the IEEE Std 802.11 OUI (00-0F-AC).”

| 7630 |  |  |  | There are references to "payload" in the MAC but this term is not defined | Change to "frame body" at 578.55, 578.56, 578.58, 857.51, 1307.34, 1752.34, 1755.58, 1946.31, 1950.1, 1956.20, 1958.26Change to "information" at 1020.37, 1037.51, 1842.49Change to "SNAP body" at 1741.56, 1814.56, 1862.53, 2104.48Delete at 1806.44, 1804.47, 1934.56 |  | Terminology |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

“payload” is used with at least these different meanings:

- The plaintext that is encrypted

- The entire frame body

- The entire MPDU or A-MPDU (from PHY perspective)

Regardless, “Frame Body” is well defined (e.g. 565.03).   It is open to question whether we should attempt to replace one colloquial term with another “frame body”.

So, I propose that “Frame Body field” should be used.

The proposed change at 857.51is questionable.

The value of the Filter Offset subfield is the number of octets from the beginning of the MSDU or MMPDU at which the Filter Value is compared. A value of 0 for the Filter Offsetindicates that the Filter Value subfield is to be compared to the first octet of the payload prior to encryption following the MAC header.

This talks about “MSDU or A-MSDU” and “prior to encryption”, so I don’t see how this can be replaced with “frame body”.  I don’t recommend making this change.

At 1946.32:

CCMP encrypts the payload of a plaintext MPDU

Does replacing “payload” with “Frame Body field” create any ambiguity about which fields are included?  I’ll assume not.

At  1842.49:

A STA shall send an Information Response frame with an empty payload in response to a received Information Request frame that solicits information about a single target STA, asidentified by the Subject Address field within the Information Request frame, if

See the Information Response frame format at 1226.63.  This includes a fixed field and a “one or more DMG Capabilities elements” as well as optional elements.

It is unclear to me what an “empty payload” is. It isn’t an empty Information field, to be sure, to be sure.

I don’t propose trying to fix this.

I disagree with these changes: “Change to "SNAP body" at 1741.56, 1814.56, 1862.53, 2104.48” as Annex H (3415.15) defines “Payload” in this context.

Straw poll:  Do you prefer?

- To make changes (as shown below)

- To make no changes

Proposed resolution:

Revised.

Change “payload” to "Frame Body field" at 578.55, 578.56, 578.58,

1307.34, 1752.34, 1755.58, 1946.31, 1950.1, 1956.20, 1958.26

Change to "information" at 1020.37, 1037.51

Delete at 1806.44, 1806.47, 1934.56

| 7631 |  |  |  | "primary channel of the BSS" (and similar things ending "channel of the BSS") is pleonastic, since "primary channel" is defined as a characteristic of the BSS | Delete "of the BSS" in all instances (about 21) |  | Terminology |  | EDITOR: 2016-01-28 14:17:48Z - Changed to "Terminology". |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  This comment is also out of scope.

Proposed Resolution:

Rejected.

The cited text is not incorrect.   It emphasises that the primary channel is an attribute of the BSS and not, for example, of a TDLS link that the STA has.

| 7230 |  |  |  | Change dot11*ProbeDelay* to dot11*NAVSync* (see CID 6568) | As it says in the comment |  | Terminology | Discuss | EDITOR: 2016-01-29 12:04:15Z - The change is small enough. But I'd like to determine group consensus before making the change. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Contexts:

its NAV, or until a period of time equal to the dot11DMGProbeDelay has transpired, whichever is earlier. 10.36.6 C he channel is not the operating channel, wait for dot11RMMeasurementProbeDelay, or until a PHY-RXSTART.indication primitive has NAV, or until a period of time equal to at least dot11TDLSProbeDelay has transpired (this combined event is indicated lue, dot11RadioMeasurementActivated TruthValue, dot11RMMeasurementProbeDelay Unsigned32, dot11RMMeasurementPilotPeriod Unsign meout Unsigned32, dot11OCBActivated TruthValue, dot11TDLSProbeDelay Unsigned32, dot11TDLSDiscoveryRequestWindow Unsi 62 63 64 IEEE P802.11-REVmc/D5.0, Jan 2016 dot11RMMeasurementProbeDelay OBJECT-TYPE SYNTAX Unsigned32 MAX-ACCESS read-w { false } ::= { dot11StationConfigEntry 102 } dot11TDLSProbeDelay OBJECT-TYPESYNTAX Unsigned32 (1..65535) UNITS "mi urationMax dot11DMGEcssClusterReportDurationMin dot11DMGProbeDelay } Unsigned32, Unsigned32, Unsigned32 dot11Ma VAL { 1000 } ::= { dot11DMGOperationEntry 17 } dot11DMGProbeDelay OBJECT-TYPESYNTAX Unsigned32 (0..10000) UNITS "mi ementImplemented, dot11RadioMeasurementActivated, dot11RMMeasurementProbeDelay, dot11RMMeasurementPilotPeriod, dot11RMLinkMeasur ementImplemented, dot11RadioMeasurementActivated, dot11RMMeasurementProbeDelay, dot11RMMeasurementPilotPeriod, dot11RMLinkMeasur ementImplemented, dot11RadioMeasurementActivated, dot11RMMeasurementProbeDelay, dot11RMMeasurementPilotPeriod, dot11RMLinkMeasur ementImplemented, dot11RadioMeasurementActivated, dot11RMMeasurementProbeDelay, dot11RMMeasurementPilotPeriod, dot11RMLinkMeasur urationMax, dot11DMGEcssClusterReportDurationMin, dot11DMGProbeDelay } STATUS current DESCRIPTION "Attributes that , dot11TDLSResponseTimeout, dot11OCBActivated, dot11TDLSProbeDelay, dot11TDLSDiscoveryRequestWindow, dot11TDLSACDe ementImplemented, dot11RadioMeasurementActivated, dot11RMMeasurementProbeDelay, dot11RMMeasurementPilotPeriod, dot11RMLinkMeasur

Proposed Resolution:

Accepted.

| 7268 |  |  |  | It says "-SAP" | Change the hyphens to spaces (about 46 instances). If you really insist, you can keep them for labels in figures (but decide whether they should be hyphens or underscores), I suppose, but I just don't see the point even of that |  | Terminology | Discuss | EDITOR: 2016-01-29 12:20:42Z - We previously rejected a comment to change "_SAP" to " SAP", and the same comment reappeared in this ballot. I dutifully copied the former reject for that comment. But we have not seen this specific comment before. Looking for group input. Is there any harm or benefit in having "*-SAP" vs "* SAP"? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

We resolved comment 6549 thus: “REJECTED (EDITOR: 2015-04-30 14:16:57Z) - X_SAP and X SAP are both valid and do not create any ambiguity.  The former is used as a label for the SAP (usually on a figure showing an architecture).  The lattter names the SAP.”

There are about 50 instances of “*-SAP”,  mainly *=PHY,  but also MLME and MAC.

This comment is also out of scope.

Proposed Resolution:

Rejected.

The comment is out of scope:  i.e., it is not on changed text, text affected by changed text or text that is the target of an existing valid unsatisfied comment.

| 7744 | 8.42 | 3.1 |  | Why are "Authentication Server" and "Authenticator" capitalised? | Lowercaseify |  | Terminology | Discuss | EDITOR: 2016-01-29 13:51:32Z - Likewise is "Supplicate" (noted in another Ad-hoc note). Needs group discussion as we have to determine whether these are proper names from another context (therefore capitalized) or merely REALLY IMPORTANT STUFF we invented ourselves. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:   I thing we could craft a reject indicating these terms are adopted from 802.1x.

The comment is also out of scope.

Proposed Resolution:

Rejected. The comment is out of scope:  i.e., it is not on changed text, text affected by changed text or text that is the target of an existing valid unsatisfied comment.

| 7164 | 108.52 | 4.5.9 |  | The generic advertisement service (GAS) is used for both network discovery and selection purposes. | Change the following sentence "The generic advertisement service (GAS), described in 4.5.10 (Generic advertisement service (GAS)), provides both support for a STA's network selection and a conduit for communication by a non-AP STAwith other information resources in a network before joining the wireless LAN"to"The generic advertisement service (GAS), described in 4.5.10 (Generic advertisement service (GAS)),provides both support for a STA's network discovery and selection and a conduit for communication by a non-AP STAwith other information resources in a network before joining the wireless LAN" |  | Architecture |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed change:

The generic advertisement service (GAS), described in 4.5.10 (Generic advertisement service (GAS)), provides both support for a STA's network discovery and selection and a conduit for communication by a non-AP STAwith other information resources in a network before joining the wireless LAN.

Proposed resolution:

Accepted.

| 7291 | 731.22 | 9.4.2.3 |  | It says "This allows any value in the supported rates set", but there is no such set | Delete "in the supported rates set" |  | Terminology |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

In D4.0 (CID 6483) we replaced references to a “supported rates set” with “operational rates set” and added a definition.  However,  the

reference below is clearly to the contents of the field.

Context: 731.21:

NOTE—Because the BSS membership selector and supported rates are carried in the same field, the BSS membership selector value cannot match the value corresponding to any valid supported rate. This allows any value in the supported rates set to be determined as either a supported rate or a BSS membership selector.

Proposed Resolution:

Accepted.

| 7605 | 1063.50 | 9.4.2.167 |  | It was deemed in D4.0 comment resolution that "FTM Format and Bandwidth" needed the "FTM" because it was otherwise too general (CID 6243). The same is true of "Status Indication" and "Value" | Rename these subfields to "FTM Status Indication" and "FTM Value" respectively |  | Editorials | Discuss | EDITOR: 2016-02-02 12:05:27Z - I see no reason to change this. But we should have a group discussion. Possible rejections are: "Rejected. The names of these field accurately relate to their contents are specific to the Fine Timing Measurement Parameters element." "Rejected. The comment is out of scope: i.e., it is not on changed text, text affected by changed text or text that is the target of an existing valid unsatisfied comment." EDITOR_Q: 2016-02-01 22:23:16Z - A group discussion is needed. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

From ad-hoc notes:

I see no reason to change this. But we should have a group discussion.

Straw poll:  to you agree to:

- Make changes as specified

- Reject the comment

Possible rejections are:
"Rejected. The names of these field accurately relate to their contents are specific to the Fine Timing Measurement Parameters element."

"Rejected. The comment is out of scope: i.e., it is not on changed text, text affected by changed text or text that is the target of an existing valid unsatisfied comment."

| 7271 | 1573.01 | 11.2 |  | "The power management mode of a STA is selected by the PowerManagementMode parameter of the MLME-POWERMGT.request primitive." -- not for a mesh STA | Add "or MLME-MESHPOWERMGT.request" after "request" and make a similar addition to the next sentence |  | MAC Management |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The proposed change breaks the following sentence.

Context:

The power management mode of a STA is selected by the PowerManagementMode parameter of the MLME-POWERMGT.request primitive. Once the STA updates its power management mode, the MLME shall issue an MLME-POWERMGT.confirm primitive indicating the success of the operation

Proposed Resolution:

Revised.  At 1573.02 change “MLME-POWERMGT.request primitive” to “MLME-POWERMGT.request or MLME-MESHPOWERMGT.request primitive”.

At 1573.03 change “MLME-POWERMGT.confirm primitive” to “MLME-POWERMGT.confirm or MLME-MESHPOWERMGT.confirm primitive”.

| 7784 | 1621.01 | 11.3.4.3 |  | This para is duplicated in 11.3.8 | Delete this para |  | MAC Management |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1621.01, 11.3.4.3 (Authentication – Destination STA)

An AP may provide neighbor report information to a STA that requests authentication or association by responding with an Authentication or (Re)Association Response frame that includes the Reason Code field set to REJECTED_WITH_SUGGESTED_BSS_TRANSITION and that includes one or more Neighbor Report elements

Context: 1633.12:

11.3.8 Neighbor report information upon rejection with suggested BSS transition An AP may provide neighbor report information to a STA that requests authentication or association by responding with an Authentication or (Re)Association Response frame that has the Reason Code field set to REJECTED_WITH_SUGGESTED_BSS_TRANSITION and that includes one or more Neighbor Report elements.

Discussion:

I agree there is duplication.  As these are aparently cut & paste,  I see no reason to keep both.

Proposed Resolution:

Accepted.

| 7785 | 1626.19 | 11.3.5.3 |  | This para is duplicated in 11.3.8 | Delete this para |  | MAC Management |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7384 | 1719.55 | 11.14 |  | This last para belongs to the subclauses in subclause 11.3 on receipt of Deauthentication/Disassociation frames | Move the material to subclause 11.3 |  | Editorials | Discuss | EDITOR: 2016-02-02 12:19:44Z - In the past we've spent a lot of time writing and rewriting 11.3. The disadvantage of moving into 11.3 is that this probably gets duplicated. Discussion needed. EDITOR_Q: 2016-02-02 02:46:53Z - a group discussion is needed. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Although classified as an Editorial comment, I believe the scope is larger than that.   The job would be to merge into the descriptions of both association and reassociation in the appropriate location.

This text has been worked on in D5.0, so the comment is, IMHO, in scope.

Proposed Resolution:

Rejected.  Its currently location has the advantage of not duplicating this complex description for both association and re-association.

| 7632 | 1758.27 | 11.24.4.1 |  | "Upon reception of a new Location Configuration Request frame, the STA shall override any previously received Location Configuration Request frame with the new frame. " -- it's not the frame which is overridden but the information contained therein | Change to "Upon reception of a new Location Configuration Request frame, the STA shall override any information from a previously received Location Configuration Request frame with the new frame. " |  | MAC Management |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7445 | 1879.34 | 11.40.7 |  | This (last) para is entirely covered by the previous (penultimate) one | Delete this para |  | MAC Management |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  this is arguably out of scope because no non-editorial changes have been made to the text.

Proposed Resolution:

Rejected.   The previous paragraph refers to the MLME-JOIN.request primitive.  The cited para refers to the process of (re)association.

| 7161 | 1897.04 | 11.46 |  | What is this about "entities wishing" at the start of this paragraph? This wording should be clarified. In addition, it may be useful to suggest to clarify what entities may do with the parameter. | Change the paragraph to read "Entities which may control the traffic steering decision of a device benefit from able to predict the throughput that might potentially be obtained through a link with an STA. Those same entities also need to know what the current expectation for throughput is for network selection purposes (by comparing the expected throughput with an existing throughput). The MLMEESTIMATED-THROUGHPUT.request and MLME-ESTIMATED-THROUGHPUT.confirm primitives together provide an interface to allow such entities, operating through the SME, to obtain an estimate of throughput for MSDUs sent between the STA that corresponds to the PeerMACAddress indicated in the parameter list of the MLME-ESTIMATED-THROUGHPUT.request primitive and this STA." |  | MAC Management |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Cited text: 1897.01: with changes proposed by commenter

Entities which may control the traffic steering decision of a device benefit from able to predict the throughput that might potentially be obtained through a link with an STA. Those same entities also need to know what the current expectation for throughput is for network selection purposes (by comparing the expected throughput with an existing throughput). The MLMEESTIMATED-THROUGHPUT.request and MLME-ESTIMATED-THROUGHPUT.confirm primitives together provide an interface to allow such entities, operating through the SME, to obtain an estimate of throughput for MSDUs sent between the STA that corresponds to the PeerMACAddress indicated in the parameter list of the MLME-ESTIMATED-THROUGHPUT.request primitive and this STA.

Critique:

- Grammar “Entities which”

- Unnecessary normative verb “may”

- Grammar “from able”

- Questionable grammar “an existing throughput”

Changes proposed here:

Entities outside the scope of this standard that might control the traffic steering decision of a device benefit by being able to predict the throughput that might be obtained through a link with a STA. Those same entities also need to know what the current estimate of throughput is for network selection purposes (by comparing an estimated throughput with existing throughput). The MLME-ESTIMATED-THROUGHPUT.request and MLME-ESTIMATED-THROUGHPUT.confirm primitives together provide an interface to allow such entities, operating through the SME, to obtain an estimate of throughput for MSDUs sent between the STA that corresponds to the PeerMACAddress indicated in the parameter list of the MLME-ESTIMATED-THROUGHPUT.request primitive and this STA

Proposed resolution:

Revised.  Make changes in <this-doc> under CID 7161.  These changes address the issues in the change proposed in the comment and correct various grammatical issues.

| 7653 | 1944.14 | 12.5.2.6 |  | We killed most of the "monotonically incrementing"s but left one | Change to "strictly increasing" |  | Security |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  We missed making this change in SB0.  That might be because this is in TKIP, which is deprecated.  But I can’t find anything to support that supposition.

Proposed Resolution:

Accepted.

| 7215 | 2235.10 | 16.2.3.5 |  | In Table 17-1, "Lockedclocks bit0 = not1 = locked" makes no sense | Change to "Lockedclocks bit:0 = not locked1 = locked" |  | Other PHY |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 7247 | 2236.28 | 16.2.3.6 |  | "a length extension bit shall be placed at bit position B7 in the SERVICE field" duplicates information in the table above | Change to "a length extension bit is present in the SERVICE field" and change "service field (b7) bit" to "length extension bit" at line 35 |  | Other PHY |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  cited location should be 2235.28.  There’s an additional instance of ‘service field (b7) bit’ not covered by the commenter’s proposed change.

Proposed Resolution:

Revised.

Change cited text to "a length extension bit is present in the SERVICE field"

and change "service field (b7) bit" to "length extension bit" at lines 35 and 45.

| 7248 | 2236.35 | 16.2.3.6 |  | It is not clear what the length extension bit is set to if the rate is not 11 Mbps | Say it is reserved in this case |  | Other PHY |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.

At 2235.39 add a new para: “The length extension bit is reserved when the data rate is not 11 Mb/s.”

| 7299 | 2240.55 | 16.2.5 |  | "long or short PHY" makes no sense -- there's only one PHY | Append "preamble" |  | Other PHY |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Rejected.  The term “long PHY” without “preamble” is used in about 30 instances.  It is established usage (dating back >13 years).

| 7423 | 2292.11 | 17.3.9.7.4 |  | It says "The relative constellation RMS error, averaged over subcarriers, OFDM PPDUs, and packets, " -- what is a "packet"? | Change to read "The relative constellation RMS error, averaged over subcarriers and OFDM PPDUs, " |  | Other PHY |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Rejected.   There are approximately 400 instances of “packet” in the PHY Clauses.   This is generally taken to refer to a PPDU.  There is no need to change this specific instance.

| 7411 | 2504.51 | 21.2.5.2 |  | "PHY-TXSTART.request(TXVECTOR) primitive is issued" -- to what? There is no OFDM PHY | Use "as if" wording, as above |  | VHT PHY |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Context: 2504.46 with changes proposed here.

The Clause 21 (Very High Throughput (VHT) PHY specification) TXVECTOR parameters in Table 21-1 (TXVECTOR and RXVECTOR parameters) are mapped to Clause 17 (Orthogonal frequency division multiplexing (OFDM) PHY specification) TXVECTOR parameters in Table 17-1 (TXVECTOR parameters) according to Table 21-3 (Mapping of the VHT PHY parameters for NON_HT operation) and the PHY operates as if a Clause 17 (Orthogonal frequency division multiplexing (OFDM) PHY specification) PHYTXSTART.request(TXVECTOR) primitive was issued. NOTE—When the FORMAT parameter is set to NON_HT and the NON_HT_MODULATION parameter is set to NON_HT_DUP_OFDM in a PHY-TXSTART.request(TXVECTOR) primitive, the behavior of the VHT PHY is defined in Clause 21 (Very High Throughput (VHT) PHY specification).

Discussion:

There is a single PHY – the Clause 21 PHY.  Certain behaviours are defined in Clause 17,   but it’s still a Clause 21 PHY.

Proposed Resolution:

Revised.  At 2504.51 insert “PHY operates as if a” before “Clause 17”.  At 2504.52 change “is” to “was”.

| 7475 | 2574.60 | 21.3.11.4 |  | "After the STA's MLME is configured using the PHYCONFIG_VECTOR parameterGROUP_ID_MANAGEMENT" -- the PHYCONFIG_VECTOR does not configure the MLME, it configures the PHY | Change "STA's MLME" to "PHY" |  | VHT PHY |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7236 | 2756.22 | B.4.9 |  | It says "Set SERVICE field bits for locked clocks, and length extension (B0, B2, B3, B5, B6, and B7)" but b0, b3, b5 and b7 are now reserved | Delete the parenthesis |  | PICS |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 7498 | 3401.61 | G.1 |  | "Data type Null Data subtype bit equal to 1." -- which bit is that? | Refer to the specific bit in the Frame Control field |  | Frame Exchange Sequences |  | Editorial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.

At 3401.07 change “Data type CF-Ack subtype bit equal to 1” to “Data type CF-Ack subtype bit (Frame Control field B4) equal to 1”

At 3401.09 change “Data type CF-Poll subtype bit equal to 1” to “Data type CF-Poll subtype bit  (Frame Control field B5) equal to 1”

At 3401.61 change “Data type Null Data subtype bit equal to 1” to “Data type Null Data subtype bit  (Frame Control field B6) equal to 1”

At 3402.07 change “Data type QoS subtype bit equal to 1” to “Data type QoS subtype bit (Frame Control field B7) equal to 1”