IEEE P802.11
Wireless LANs

| Resolution for some GEN comments in SB1 – Part II |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-02-22 |  |  |  |  |
| Author: |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Edward Au | Huawei Technologies | 303 Terry Fox Drive, Suite 400, Ottawa, Ontario K2K 3J1 |  | edward.ks.au@huawei.com |

##### This submission presents proposed resolution to CIDs 7574, 7575, 7127, 7406, 7403, 7407, 7401 and 7402.  Changes indicated by instructions.

##### Revision history:

##### R0 – initial version

R1 – remove CIDs 7701 and 7702 from the document.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 7574 | C.3 | 3074 | 14 | "is a 77 bit subfield" -- what is this doing in the MIB? | Call it a bitmap instead |

Discussion:

Referring to line 3074.14,

and Figure 9-333,

Rx MCS Bitmasks subfield is of length 77 bits, but the commenter is correct that it is not necessary to describe the subfield and its corresponding length in MIB.

Resolution:

### Accept

### TGmc Editor:  Change “a 77 bit subfield” to “a bitmap”.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 7575 | C.3 | 3074 | 14 | "is a 10 bit subfield that" -- what is this doing in the MIB? And why is the range in the DESCRIPTION not the SYNTAX? | Delete the cited text. Move the range info to the SYNTAX |

Discussion:

Referring to line 3074.21,

and Figure 9-333,

Rx Highest Supported Data Rate subfield is of length 10 bits, but the commenter is correct that it is not necessary to describe the subfield and its corresponding length in MIB.  Further, the range is missing in the SYNTAX.

As a related not, “HT Highest Supported Data Rate” should be changed to “HT Rx Highested Supported Data Rate” in order to match the corresponding field in Figure 9-333.

Resolution:

### Revised

### TGmc Editor:  Change lines 3074.21 – 3074.33 as follows.

### dot11RMNeighborReportHTRxHighestSupportedDataRate OBJECT-TYPE

### SYNTAX Unsigned32 (0..9)

### MAX-ACCESS read-create

### STATUS current

### DESCRIPTION

### "This is a status variable.

### It is written by the SME when a measurement report is completed.

### The HT Rx Highest Supported Data Rate defines the

### highest HT PPDU data rate that the STA is able to receive, in units of 1

### Mb/s in the range 1 to 1023. See 9.4.2.56.4 (Supported MCS Set field)"

### ::= { dot11RMNeighborReportEntry 40 }

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 7127 | 17.3.10.3 | 2295 | 23 | "The power difference between the interfering and the desired channel is the corresponding adjacent channel rejection." -- channels don't have powers. | Relate to powers of objects that have a power attribute. Make same changes in other phys and non-adjacent test if this text occurs there too. |

Discussion:

Line 2295.23:

Referring to line 2295.23, the term “adjacent channel rejection” refers to the measure of signal power from the desired and the interfering channels.  A proposed resolution is to change “The power difference between the interfering channel and the desired channel” to “The difference in signal power between the interfering and the desired channel”.

Throughout the draft, there are TBD additional appearances as follows.

Line 2296.6:

A proposed resolution is to change “The power difference between the interfering channel and the desired channel” to “The difference in signal power between the interfering and the desired channel”.

Line 2315.59:

A proposed resolution is to change “The power difference between the interfering channel and the desired channel” to “The difference in signal power between the interfering and the desired channel”.

Line 2400.26:

A proposed resolution is to change “The power” to “The difference in signal power”.

Line 2400.36:

A proposed resolution is to change “The power” to “The difference in signal power”.

Line 2400.57:

A proposed resolution is to change “The power” to “The difference in signal power”.

Line 2400.65:

A proposed resolution is to change “The power” to “The difference in signal power”.

Line 2586.55:

A proposed resolution is to change “The power difference between the interfering channel and the desired channel” to “The difference in signal power between the interfering and the desired channel”.

Line 2587.59:

A proposed resolution is to change “The power difference between the interfering channel and the desired channel” to “The difference in signal power between the interfering and the desired channel”.

Line 2588.16:

A proposed resolution is to change “the power difference” to “the difference in signal power”.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 7406 | 21.2.5.2 | 2505 | 37 | The OPERATING_CHANNEL is used by the OFDM vector | Change "discarded" to "OPERATING_CHANNEL" |

Discussion:

In line 2505.37:

The commenter points out correctly that PHYCONFIG_VECTOR used by OFDM vector also contains an OPERATING_CHANNEL paremeter as shown in clause 17.3.8.4.1:

Resolution:

### Accepted

### TGmc Editor:  In line 2505.37, change “discarded” to “OPERATING_CHANNEL”.s

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 7403 | 21.2.5.2 | 2505 | 01 | There is no SECONDARY_CHANNEL_OFFSET in the vector | Change SECONDARY_CHANNEL_OFFSET to CHANNEL_WIDTH. Also at 2505.39 |

Discussion:

In line 2505.01:

In line 2505.39:

The commenter points out correctly that there is no SECONDARY_CHANNEL_OFFSET in the PHYCONFIG_VECTOR used by VHT vector.  Instead, as shown in clause 21.2.3, the PHYCONFIG_VECTOR used by VHT vector contains the following parameters:

- OPERATING_CHANNEL;

- CHANNEL_WIDTH;

- CENTER_FREQUENCY_SEGMENT_0;  and

- CENTER_FREQURNCY_SEGMENT_1.

Resolution:

### Accepted

### TGmc Editor:  In lines 2505.1 and 2505.39, change “SECONDARY_CHANNEL_OFFSET” to “OPERATING_CHANNEL”.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 7407 | 21.2.5.2 | 2505 | 40 | The CENTER_FREQUENCY_SEGMENTs need to be discarded too | Add two rows, one for CENTER_FREQUENCY_SEGMENT_0 and one for CENTER_FREQUENCY_SEGMENT_1, both with "discarded" and "PHYCONFIG_VECTOR" |

Discussion:

In Table 21-3:

The parameters CHANNEL_FREQUENCY_SEGMENT_0 and CHANNEL_FREQUENCY_SEGMENT_1 are missing here for PHYCONFIG_VECTOR used by VHT PHY.

Resolution:

### Accepted

### TGmc Editor:  At lines 2505.40, add two rows, one for CENTER_FREQUENCY_SEGMENT_0 and one for CENTER_FREQUENCY_SEGMENT_1, both with "discarded" and "PHYCONFIG_VECTOR".

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 7401 | 15.4.4.3 | 2221 | 54 | 2G4 PHYs do not use dot11CurrentFrequency | Change dot11CurrentFrequency to dot11CurrentChannel. Also change "for the" to "for a" |
| 7402 | 16.3.6.3 | 2249 | 52 | 2G4 PHYs do not use dot11CurrentFrequency | Change dot11CurrentFrequency to dot11CurrentChannel. Also change "for the" to "for an" |

Discussion:

In line 2221.54:

In line 2249.52:

The commenter points out correctly that 2.4 GHz channel use dot11CurrentChannel instead of dot11CurrentFrequency:

In addition to make the changes at lines 2221.54 and 2249.52, we need to consider the following changes:

- There is no “Information field”;

- The information on “dot11CurrentChannel” may be confusing because 2.4 GHz use dot11CurrentChannel instead of dot11CurrentFrequency;

- The last sentence is incorrect (and redundant too) because it should the length of the Current Channel field is octet.

A proposed resolution here is to change “The Information field contains a single parameter constaining the dot11CurrentChannel (see 15.4.4.3 (Channel Numbering of operating channels), 16.3.6.3 (Channel Numbering of operating channels), 17.3.8.4.2 (Channel numbering) and 19.3.15 (Channel numbering and channelization for values).  The length of the dot11CurrentChannel parameter is 1 octet” to “The Curent Chanel field contains a single parameter describing the current operating frequency channel”.

Resolution for CID 7401:

### Accepted

### TGmc Editor: In line 2221.54, change “dot11CurrentFrequency” to “dot11CurrentChannel”.  Further, in line 2221.52, change “for the DSSS PHY” to “for a DSSS PHY”.

Resolution for CID 7402:

### Accepted

### TGmc Editor:

### In line 2249.52, change “dot11CurrentFrequency” to “dot11CurrentChannel”.  Further, in line 2249.50, change “for the HR/DSSS PHY” to “for an HR/DSSS PHY”.

In line 731.63, change “The Information field contains a single parameter constaining the dot11CurrentChannel (see 15.4.4.3 (Channel Numbering of operating channels), 16.3.6.3 (Channel Numbering of operating channels), 17.3.8.4.2 (Channel numbering) and 19.3.15 (Channel numbering and channelization for values).  The length of the dot11CurrentChannel parameter is 1 octet” to “The Curent Chanel field contains a single parameter describing the current operating frequency channel”.