IEEE P802.11
Wireless LANs

| Simulation Scenario Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017 - 03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Frank Hsu | MediaTek Inc. |  |  | frank.hsu@mediatek.com |
| James Yee | MediaTek Inc. |  |  | james.yee@mediate.com |

Abstract

This document contains the proposed text for 11ax Simulation Scenario document 11-14/0980r16

Reference:

[1] IEEE 802.11-16/1604r0 PAR Verification Single BSS Simulation

[2] IEEE 802.11-17/0076r1 PAR Verification Multiple BSS Simulation

Single BSS Scenario

| Parameter |  | Value |
| --- | --- | --- |
| Topology |  |  |
| Environment description | An open area with an AP and many STAs. Three different sizes of the area are defined and the STA densities are the same. Three topologies are defined. - 10m x 10m area with 20 STAs - 20m x 20m area with 80 STAs - 30m x 30m area with 180 STAs |  |
| APs location | AP is at the center of the area |  |
| AP Type | HE AP |  |
| STAs location | Place STAs randomly within the area |  |
| STAs type | All STAs are HE or all STAs are VHT |  |
| Channel Model | Fading model TGac channel model D NLOS for all the links. |  |
| Penetration Losses | Pathloss model PL(d) = 40.05 + 20*log10(fc/2.4) + 20*log10(min(d,5)) + (d>5) * 35*log10(d/5) + 18.3*F^((F+2)/(F+1)-0.46) + 5*W - d = max(3D distance [m], 1) - fc = frequency [GHz] - F = number of floors traversed - W = number of walls traversed in x-direction plus number of walls traversed in y-direction Shadowing Log-normal with 5 dB standard deviation, iid across all links |  |
| PHY parameters |  |  |
| Center frequency and BW | Operation Channel: 5GHz with 80 MHz channel |  |
| MCS | Use MCS5 or MCS9 for all transmission |  |
| GI | Long GI |  |
| STA TX power | 15 dBm |  |
| AP TX Power | 20 dBm |  |
| AP #of TX antennas | 1 |  |
| AP #of RX antennas | 1 |  |
| STA #of TX antennas | 1 |  |
| STA #of RX antennas | 1 |  |
| MAC parameters |  |  |
| Access protocol parameters | EDCA with default parameters according to the access category |  |
| UDP packet size | 100/1500 bytes |  |
| Aggregation | 64 MPDU per AMPDU |  |
| Max # of retries | 10 |  |
| RTS/CTS Threshold | No RTS/CTS |  |
| Association | All STAs are associated to the AP |  |

Notes:

- Uplink traffic only

- STAs have full buffer

- VHT STAs are randomly assigned to an access category among AC_VO, AC_VI, or AC_BE.

- In UL OFDMA, for each available RU, HE AP selects an associated STA randomly

- HE STAs are waiting for trigger frames to transmit UL data

Expected Results:

- UL system throughputs and UL 5th percentile throughputs

- In each topology, comparison of UL throughputs between cases of all HE STAs and all VHT STAs

- Throughput comparison among 3 topologies

Basic Multiple BSS Scenario

| Parameter |  | Value |
| --- | --- | --- |
| Topology |  |  |
| Environment description | There is an open area with 3 APs. The locations of 3 APs form a regular triangle and the distance between any two APs is 30 meters. 3 topologies are defined. In each BSS, - 13 STAs are within a 15-meter radius circle (39 STAs total) - 26 STAs are within a 15-meter radius circle (78 STAs total) - 52 STAs are within a 15-meter radius circle (156 STAs total) |  |
| APs location | AP is at the center of 15-meter radius circle. Any two APs are separated by 30 meters. |  |
| AP Type | HE AP |  |
| STAs location | Inside each BSS, place STAs randomly within the circle |  |
| STAs type | Inside each BSS, all STAs are HE or all STAs are VHT |  |
| BSS traffic type | - 3 BSSs are all UL - 1 BSS is DL and the other 2 BSSs are UL - 2 BSSs are DL and the rest 1 BSS is UL |  |
| Channel Model | Fading model TGac channel model D NLOS for all the links. |  |
| Penetration Losses | Pathloss model PL(d) = 40.05 + 20*log10(fc/2.4) + 20*log10(min(d,5)) + (d>5) * 35*log10(d/5) + 18.3*F^((F+2)/(F+1)-0.46) + 5*W - d = max(3D distance [m], 1) - fc = frequency [GHz] - F = number of floors traversed - W = number of walls traversed in x-direction plus number of walls traversed in y-direction Shadowing Log-normal with 5 dB standard deviation, iid across all links |  |
| PHY parameters |  |  |
| Center frequency and BW | Operation Channel: 5GHz with 80 MHz channel |  |
| MCS | Use MCS5 or MCS9 for all UL/DL transmission |  |
| GI | Long GI |  |
| STA TX power | 15 dBm |  |
| AP TX Power | 20 dBm |  |
| AP #of TX antennas | 1 |  |
| AP #of RX antennas | 1 |  |
| STA #of TX antennas | 1 |  |
| STA #of RX antennas | 1 |  |
| MAC parameters |  |  |
| Access protocol parameters | EDCA with default parameters according to the access category |  |
| UDP packet size | 100/1500 bytes |  |
| Aggregation | 64 MPDU per AMPDU |  |
| Max # of retries | 10 |  |
| RTS/CTS Threshold | No RTS/CTS or Enable RTS/CTS |  |
| Association | All STAs are associated to the designated AP |  |

Notes:

- Inside each BSS, VHT STAs are randomly assigned to an access category among AC_VO, AC_VI, or AC_BE.

- In UL OFDMA, for each available RU, HE AP selects an associated STA randomly

- For the BSS with UL traffic

  - STAs have full buffer

  - HE STAs are waiting for trigger frames to transmit UL data

- For the BSS with DL traffic

  - AP is the only device competing the medium and has full buffer

  - STAs only send acknowledgement to AP

Expected Results:

- UL/DL system throughputs and UL/DL 5th percentile throughputs

- In each topology, comparison of UL throughputs between cases of all HE STAs and all VHT STAs

- Throughput comparison among 3 topologies

- For each topology, comparison of the DL throughputs with different traffic neighboring BSSs

  - DL BSS with one DL neighboring BSS and one UL neighboring BSS

  - DL BSS with two UL neighboring BSSs

Straw Poll:

Do you support to add simulation scenarios proposed in this document to the simulation scenario document of TGax (11-14/0980r16)?