IEEE P802.11
Wireless LANs

| Proposed Comment Resolutions 2309, 2310 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-04-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| George Calcev | Futurewei Technologies | Rolling Meadows, Illinois, USA |  | George.Calcev@huawei.com |

Abstract

This document proposes resolutions to REVmd D2.0 for the following CIDs: 2309, 2310

| CID | Clause Number(C) | Page(C) | Line(C) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 2309 | 8.3.5 | 760 | 3 | "The effect of receipt of this primitive by the MAC is unspecified." -- it is not useful to have a primitive whose receipt has unspecified results | Delete subclauses 8.3.5.3/11/12/13/16 |
| 2310 | 8.3.5 | 760 | 3 | "The effect of receipt of this primitive by the MAC is unspecified." -- it is not useful to have a primitive whose receipt has unspecified results | Add something useful (e.g. for the first could talk about adding to nascent PSDU, for third that it potentially updates the overall CS state) |

Discussion

The comments are valid for Clauses: 8.3.5.3, 8.3.5.11, 8.3.5.12, 8.3.5.13, 8.3.5.16. We note that the comments refer to the same issue but offer two different possible resolutions. We reject 2309, which suggests the deletion of the clauses because we consider that the text could be improved to offer some useful alternative as suggested in the proposed resolution of CID2310  .

We revised the proposed resolution of CID 2310 as follows:

Proposed Revised Resolution

CID 2309- Reject

CID 2310- Revised

Proposed Changes

Instruct the editor to replace in Clause 8.3.5.3.4:

“The effect of receipt of this primitive by the MAC is unspecified”

With

“The receipt of this primitive by the MAC entity causes the MAC to collect the data octet provided by the PHY layer and to add it to the newly created A-MPDU in the current MAC RX state.”

Instruct the editor to replace in Clause 8.3.5.11.4:

“The effect of receipt of this primitive by the MAC is unspecified”

With

“The receipt of this primitive by the MAC entity causes the MAC to determine that the PHY CCA state machine was reset followinga PHY-CCARESET.request and the MAC may collect the observed IPI values if they requested in the PHY-CCARESET.request primitive.”

Instruct the editor to replace in Clause 8.3.5.13.4:

“The effect of receipt of this primitive by the MAC is unspecified”

With

“The receipt of this primitive by the MAC entity causes the MAC to change the state from CS/CCA to RX state and prepare for the creation of a new A-MPDU from the octets provided by PHY layer during the MAC RX state.”

Instruct the editor to replace in Clause 8.3.5.16.4:

“The effect of receipt of this primitive by the MAC is unspecified”

With

“The receipt of this primitive by the MAC entity causes the MAC to determine that the PHY has received and successfully applied the parameters in the PHY-CONFIG.request primitive.”