IEEE P802.11
Wireless LANs

| Resolutions for some initial SA ballot comments on 11md/D3.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-01-10 |  |  |  |  |
| Author: |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Edward Au | Huawei Technologies | 303 Terry Fox Drive, Suite 400, Ottawa, Ontario K2K 3J1 |  |  |

##### This submission present proposed resolution for CIDs 4804, 4793, 4791, 4651, 4650, 4312, 4769, 4478, 4195, 4497, 4141, 4335, 4517, 4398, 4397, 4273, 4219, 4257, 4634, 4428, 4324, 4111, 4384, 4307, 4020, 4021.  The proposed changes are based on REVmd/D3.0.

##### Revision history:

##### R0 – initial version

##### R1 – Update resolution of CIDs 4791, 4651, and 4650 based on the discussion on the December 20, 2019, call.

R2 – Add proposed resolution of CIDs 4769, 4478, 4195, 4497, 4141, 4335, 4517, 4398, 4397, 4273, 4219, 4257, 4634, 4428, 4324, 4111, 4384, 4307, 4020, 4021.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4804 | 11.3.5.3 | 2235 | 35 | No such thing as "MLME-ASSOCIATION.response" | Change "MLME-ASSOCIATION.response" to "MLME-ASSOCIATE.response" |

Discussion:

The following is the sentence of interest:

Throughout the clause and the whole draft, it (page 2235, line 35) is the only instance of “MLME-ASSOCIATION”.

Proposed resolution:

Accepted

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4793 | 25.2.2 | 3487 | 47 | Extraneous space | Remove space in "PARTIAL_AI D" |

Discussion:

The following is the sentence of interest:

There is an extra space between “AI” and “D” that is unnecessary.

Proposed resolution:

Accepted

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4791 | 10.3.3 | 1758 | 34 | Since we've cleaned up all the language to talk about a backoff count, and not time, the title of this section being "backoff time", while arguably correct, is confusing. | Change the title of this subclause to "Random backoff" |
| 4651 | 10.3.3 | 1758 | 34 | "Random backoff time" should be "Random backoff" or "Random backoff procedure" | Change to the second option |
| 4650 | 10.3.3 | 1758 | 34 | "Random backoff time" should be "Random backoff" or "Random backoff procedure" | Change to the first option |

Discussion:

These comments are related to the title of the subclause 10.3.3 as follows:

Proposed resolution for CID 4791:

Revised.  Change the title of the subclause from “Random backoff time” to “Random backoff procedure”.

Proposed resolution for CID 4651:

Accepted.

Proposed resolution for CID 4650:

Revised.  Change the title of the subclause from “Random backoff time” to “Random backoff procedure”.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4312 |  |  |  | The names of the following fields should have all initials upper-case, to avoid confusion: "Number of Channel Measurement Info" should be "Of". Also "Number of RX DMG Antennas", "Number of Channels", "Number of Time Blocks", "Normal Number of Frames per Channel", "Number of ANQP OIs", "OI #1 and #2 Lengths" | As it says in the comment |

Discussion:

As per the 802.11 style guide (09/1034r15),

- Initial letters of proper names, which includes:

  - Frame names – e.g. the “Beacon frame”,   but “transmits a beacon” where it is used to represent the concept of a beacon (no caps).

  - Element & subelement names – e.g. “the Capabilities element”,  but “the capabilities of the STA”

  - Field & subfield names – e.g. “the More Data subfield of the Frame Control field”, but “shall set it to 1 if it has more data to send”.

  - Enumerated values of a field or subfield

  - Certain measurement requests and reports (see 8.4.2.20 and 8.4.2.21)

The commenter lists the following:

- Number of Channel Measurement Info

- Number of RX DMG Antennas

- Number of Channels

- Number of Time Blocks

- Normal Number of Frames per Channel

- Number of ANQP OIs

- OI #1 and #2 Lengths

There are at least the following 5 additional fields:

- Number of Points field

- List of 2-Dimension Points field

- Measurement for Time Block fields

- Number of Directions field

- Measurement for Direction List field

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4769 | 12.6.20 | 2642 | 12 | After the first cross-reference in this sentence, the following text starts with "(The", when it should be "the". The following text also all appears to be "hot" (as in a hot link), when it should not be. Finally, this all ends around line 18 with, "frames).)), which". This latter text should be "frames). Which" | Change "(The" to "the". Clean up the hot linkage. Change "frames).)), which" to "frames). Which" |

Discussion:

The comment is related to the following paragraph:

Agree with the commenter that

- “The Protected Dual …” should be replaced by “the Protected Dual”, but the location of “(see 9.6.10 (…))” should be moved before the comma.

- The entire sentence “(The Protected Dual … (see 9.4.1.1(…))” is in the hot linkage that needs to be cleaned up.

- There are two redundant “)” after the end of the sentence about Table 9-403.

- “which” should be replaced by “Which”.

Proposed resolution:

Revised

When a Public Action frame is transmitted for which a Protected Dual of Public Action frame is defined (see 9.6.10 (Protected Dual of Public Action frames), the Protected Dual of Public Action frame is defined to allow robust STA-STA communications of the same information that is conveyed in Action frames that are not robust (see 9.4.1.11 (Action field)). A Public Action field, in the octet immediately after the Category field, differentiates the Protected Dual of Public Action frame formats. The defined Protected Dual of Public Action frames are listed in Table 9-403 (Public Action field values defined for Protected Dual of Public Action frames). Which variant (i.e., protected or not protected) is used depends on the setting of the “Protected” parameter of the corresponding MLME .request or .confirm primitive. Where there is no such parameter, the protected variant is used when management frame protection has been negotiated.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4478 | 19.3.2.1 | 3070 | 1 | Figure 19-25--PHY receive procedure for HT-mixed format PPDU format is cropped top and right | As it says in the comment |

Discussion:

The comment is related to the following figure:

Agree with the commenter that the figure is cropped unintentionally up and right.

Proposed resolution:

Revised.

The figure should be replaced by the following updated figure that the terms “CS/CCA state” and “RX state” are now clearly shown at the top, and the description “Issued at the same time” is clearly shown at the right hand side.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4195 |  |  |  | Spurious underscores are spurious | Change "Tx_Sector" to "Tx Sector" throughout (6x) |

Discussion:

The first 3 appearances of “Tx_Sector” are located in subclause 9.4.2.222 (SSW Report element) because there is a subfield called “Peer Tx_Sector ID” as shown in Figure 9-742 below.

The remaining 3 appearances of “Tx_Sector” are located in subclause 9.4.2.226 () because there is also a subfield called “Peer Tx_Sectior ID” as as shown in Figure 9-748 below.

The 802.11 style guide (09/1034r15) does not have a recommendation on the use of “_” for the element/field name.

Proposed resolution:

Rejected

Spurious underscores are not spurious because it is part of the name of the Peer Tx_Section ID subfield.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4497 | B.4.4.1 | 3593 | 6 | Something terrible has happened to the reference for PC34.1.8.1 | Change to refer to 12.7.8 (this is the ref given at 1108.34) |

Discussion:

The comment is related to the following entry:

As referred to the IEEE 802.11-2016 standard, the reference of PC34.1.8.1 is subclause 12.7.8 (PeerKey handshake).  However, this entire subclause together with another subclause 12.2.5 (RSNA PeerKey Support) are both removed from the P802.11REVmd Draft 0.1 because of CID 59, which is about the removal of DLS and STSL (see https://mentor.ieee.org/802.11/dcn/17/11-17-1518-03-000m-resolution-cids-59-62-remove-dls-stsl.docx).

As the commenter points out, there is a reference of subclause 12.7.8 to 1108.34, which should no longer exist too.  As referred to IEEE 802.11-2016 standards, the description of bit 9 refers to the removed subclause 12.7.8 (PeerKey handshake), and it is not related to AP PeerKey procotol or TDLS PeerKey protocol.

Note that the PeerKey Enabled subfield is cited by the following subclause.

Note that the PeerKey handshake is appeared in subclause 3.2 (Definitions specific to IEEE Std 802.11).

Proposed resolution:

Revised

- Delete the entry of PC34.18.1 and the corresponding protocol capability, references, status, and support from B.4.4.1.

- In Figure 9-289, replace “PeerKey Enabled” with “Reserved”.

- At 1108.33 to 1108.35, replace

“Bits 9: PeerKey Enabled. An AP(#2508) sets the PeerKey Enabled subfield of the RSN Capabilities field to 1 to signal it supports PeerKey handshake (see 12.7.8 (TDLS PeerKey (TPK) security protocol))(#2455). Otherwise, this subfield is set to 0.”

with

“Bit 9:  reserved”.

- At 2686.61, replace

“In the RSN Capabilities field, the No Pairwise subfield shall be set to 0 and the PeerKey Enabled subfield shall be set to 1.”

with

“In the RSN Capabilities field, the No Pairwise subfield shall be set to 0.”

- At 196.48, replace

“Key management that includes the 4-way handshake, the group key handshake, authenticated mesh peering exchange, mesh group key handshake, and the PeerKey handshake.

with

“Key management that includes the 4-way handshake, the group key handshake, authenticated mesh peering exchange, and mesh group key handshake.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4141 | O.1 | 4583 | O.1 | In addition to the VHT and HT matlab reference models there is a matlab model for the 802.11ah PHY which should be referenced in section O.1. The last version of the model is here https://mentor.ieee.org/802.11/dcn/14/11-14-0631-02-00ah-revised-tx-reference-code.pptx. | Add notes to Section O.1 indicating where to find the 802.11ah Matlab model and that the model was based on an early draft of 802.11ah and some details relating to the signal field length subfield were changed subsequently. |

Discussion:

As per the identified document 14/0631r2, the reference code published therein is compatible with draft 1.1 of the IEEE 802.11ah-2016 amendment.

Proposed resolution:

Revised

Annex O

(informative)

Additional HT, VHT, and S1G information

O.1 HT, VHT, and S1G waveform generator tool

As an informative extension to this standard, waveform generator tools have been written to model the PHY transmission process described in Clause 17 (Orthogonal frequency division multiplexing (OFDM) PHY specification), Clause 18 (Extended Rate PHY (ERP) specification), Clause 19 (High-throughput (HT) PHY specification), Clause 21 (Very high throughput (VHT) PHY specification), and Clause 23 (Sub 1 GHz (S1G) PHY specification).

The waveform generator can be downloaded from the public IEEE 802.11 document website. The waveform generator code that includes Clause 17 (Orthogonal frequency division multiplexing (OFDM) PHY specification), Clause 18 (Extended Rate PHY (ERP) specification), and Clause 19 (High-throughput (HT) PHY specification) is contained in document 11-06/1715, and the waveform generator description is contained in document 11-06/1714 (HT code). A description of the waveform generator that includes Clause 17 (Orthogonal frequency division multiplexing (OFDM) PHY specification), Clause 19 (High-throughput (HT) PHY specification), and Clause 21 (Very high throughput (VHT) PHY specification) and the waveform generator code itself may be found in document 11-11/0517 (VHT code).  A description of the waveform generator that includes Clause 23 (Sub 1 GHz (S1G) PHY specification) and the waveform generator code itself is contained in document 11-14/0631 (S1G code).

The purpose of these tools is to promote common understanding of complex PHY algorithms, facilitate device interoperability by providing reference test vectors, and assist researchers in industry and academia to develop next generation wireless solutions.

The code is written in the MATLAB computing language and can be configured to generate test vectors for most PHY configurations, defined by this standard. Instructions on how to configure and run the tools are specified in the referenced documents.

A command line interface is used to configure the VHT code tool. For consistency with this standard, the configuration interface is made very similar to the TXVECTOR parameters defined in 21.2.2 (TXVECTOR and RXVECTOR parameters).

A command line interface and graphic user interface (GUI) exist to configure the HT code tool. For consistency with this standard, the configuration interface is made very similar to the TXVECTOR parameters defined in 19.2.2 (TXVECTOR and RXVECTOR parameters).

A command line interface is used to configure the VHT code tool. For consistency with this standard, the configuration interface is made very similar to the TXVECTOR parameters defined in 21.2.2 (TXVECTOR and RXVECTOR parameters).

A GUI exists to configure the S1G code tool.  For consistency with this standard, the configuration interface is made similar to the TXVECTOR parameters defined in 23.2.2 (TXVECTOR and RXVECTOR parameters).

NOTE–The waveform generator tool was developed based on Draft 1.3 of the IEEE 802.11ah-2016 amendment. Some details relating to the S1G PPDU format may have subsequently changed.

The HT and VHT waveform generator tools produce test vectors for all transmitter blocks, defined in Figure 19-2 (Transmitter block diagram 1) and Figure 19-3 (Transmitter block diagram 2(#1359)), generating reference samples in both frequency and time domains. Outputs of the tool are time domain samples for all transmitting chains.

The S1G waveform generator tool produces test vectors for transmitter blocks that support 1 MHz and 2 MHz channel widths, defined in 23.3.3 (Transmitter block diagram), generating reference samples in both frequency and time domains. Outputs of the tool are time domain samples for 1 and 2 spatial streams.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4335 |  |  |  | MS-SAP should be MS SAP | As it says in the comment |

Discussion:

As referred to P802.11REVmd Draft 3.0, there are 19 appearances of “MS-SAP” and zero appearance of “MS SAP”.

The abbreviation “MS-SAP” is originated from IEEE 802.11ak-2016 amendment and it means “method specific service access point” as defined in subclause 3.4 (Abbreviations and acronyms).

Proposed resolution:

Rejected.

“MS-SAP” is an abbreviation defined in subclause 3.4 (Abbreviations and acronyms) of IEEE 802.11REVmd Draft 3.0.   This abbreviation is defined for the term “method specific service access point” that is used frequently throughout the draft standards.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4517 | 12 |  |  | "SC" stands for single-carrier, so cannot be used in this clause to stand for Sequence Control | As it says in the comment |

Discussion:

There are 5 appearances of “SC” in IEEE 802.11REVmd Draft 3.0.  Two of them are related to the MPDU Sequence Control field of the AAD construction for PV0 MPDUs in subclause 12.5.3.3.3 (Construct AAD):

Another two are related to the MPDU Sequence Control field of the AAD construction for PV1 MPDUs in the same subclause:

The remaining one is related to the test vector in Annex J.11.1 (Test Vector):

Proposed resolution:

Revised.

At 2604.1 (Figure 12-19), 2604.50, 2605.4 (Figure 12-20), 2605.49, and 4527.31, replace “SC” with “SEQC”.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4398 | 14.9.2 | 2802 |  | "iff" is not defined, but here it doesn't seem necessary anyway | Change to "if" |
| 4397 | 14.9.2 | 2802 |  | "iff" is not defined | Expand to "if and only if" (3x) |

Discussion:

There are 3 appearances of “iff” in Draft 3.0 and all of them are located in Figure 14-5 as follows:

“iff” (If and only if) is a biconditional statement.  It means that either both statements are true or both are false.  Therefore, it is essentially and “If” statement that works both ways.  (Only) If is a conditional statement, and it does not require that either both statements are true or both are false.  For the description of “Compariason operator” in the table, it is a conditional, rather than a biconditional statement.

Proposed resolution for CID 4398:

Accepted.

Proposed resolution for CID 4397:

Revised.  Replace “iff” with “if” at 2802.54, 2802.55, and 2802.56 in Draft 3.0.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4273 | 13.8 |  |  | "The RSNE shall be present only if dot11RSNAActivated is true." 4x duplicates Table 13-1--FT authentication elements (and ditto for other fields) | Delete the cited sentence throughout |

Discussion:

As referred to Table 13-1 at 2746.1, the commenter refers to RSNE and Fast BSS Transition (they are present only if dot11RSNAActivated is true), and probably Timeout Interval (it is present only if dot11RSNAActivated is not true).

For RSNE, the 4 appearances of the identified sentence are located at 2746.40 (subclause 13.8.2), 2747.6 (subclause 13.8.3), 2747.33 (subclause 13.8.4), and 2748.34 (subclause 13.8.5).  The snapshot of the first appearance is shown below.  Note that the other 3 appearances follow the same pattern.

For Fast BSS Transition, the 4 appearances of the sentence “The FTE shall be present only if dot11RSNAActivated is true” are located at 2746.55 (subclause 13.8.2), 2747.19 (subclause 13.8.3), 2747.46 (subclause 13.8.4), and 2748.47 (subclause 13.8.5).  The snapshot of the first appearance is shown below.  Note that the other 3 appearances follow the same pattern.

For Time Interval, the sole appearance is located at 2749.36 (subclause 13.8.5) as follows:

While I agree with the commenter that the sentences “The RSNE shall be present only if dot11RSNAActivated is true” and “The FTE shall be present only if dot11RSNAActivated is true” can be removed because of the duplication with the information contained in Table 13-1, I would prefer to keep the sentence describing he Time Interval as the corresponding information contained in Table 13-1 is not sufficient.

Proposed resolution:

Revised

Delete the sentence “The RSNE shall be present only if dot11RSNAActivated is true” at 2746.40, 2747.6, 2747.33, and 2748.34.

Delet the sentence “The FTE shall be present only if dot11RSNAActivated is true” are located at 2746.55, 2747.19, 2747.46, and 2748.47.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4219 |  |  |  | Don't use Ceil except in ASCII-only contexts (e.g. MIB). Use the ceiling glyph instead. Ditto for Floor | As it says in the comment |

Discussion:

As referred to the document, there are 2 appearances of Ceil, namely 1457.55 and 1991.3:

Please note that the appearance of Ceil at 1991.3 is in a two-parameter form that is not typically represented by the ceiling glyphs.  Further note that there are 2 apperances of “ceil” that I have resolved in CID 4664 by replacing “ceil” with the ceiling glyph.

As for “Floor”, there are 5 apperances and all of them are in ASCII-only contexts (c.f., 3941.38, 3960.50, 3965.33, 3966.5, and 4086.31).

In subclause 1.5, there are guidelines in using the floor and ceiling operations at 151.38 and 151.44, respectively:

Proposed resolution:

Rejected

As per subclause 1.5 of Draft 3.0, there is no restriction in using Ceil () and Floor () in non-ASCII only contexts.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4257 | C.3 | 3819 | 46 | The description for dot11CFPMaxDuration should lose the last para as PCF is no longer defined, just like the last para of dot11MediumOccupancyLimit was deleted | As it says in the comment |

Discussion:

The comment is related to the following paragraph:

Proposed resolution:

Rejected

Generally, features that are marked deprecated or obsolete are not maintained.  PCF is obsolete.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4634 | 10.45.3.2.3 | 2077 | 5 | "data frame" should be "Data frame" | Fix in Figure 10-99--Example of data transmission in SP with link cooperation relay |

Discussion:

The comment is related to the following figure:

It is correctly pointed out by the commenter that “data frame” be replaced by “Data frame” at the top right hand corner.

“Pre-determined” should also be replaced by “Predetermined”.

Proposed resolution:

In Figure 10-99, replace “data frame” with “Data frame”, and “Pre-determined time” with “Predetermined time”.

Note to the editors:  The revised figure is given as follows.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4428 | 10 |  |  | WinStartR should be WinStart<sub>R</sub>, but isn't at 1884.62/64, 1885.3(2x)/7/8/12/15. Similarly all the *Os in Figure 10-39--Flow control and its associated parameters as a function of receiver buffer size | As it says in the comment |

Discussion:

The comment is related to the following figure and the following paragraphs:

For the figure, in addition to replacing WinStartO with WinStartO, WinEndO with WinEndO, WinStartO with WinStartO, BufSizeO with BufSizeO, and WinCapacityO with WinCapacityO as suggested by the commenter, there are additional changes to the figure as follows:

- Replace WinHeadO with WinHeadB

- Replace WinTailO with WinTailB

- Replace “Starting Seq. Number” with “Starting sequence number”

- Replace “Receiver Buffer size negotiated” with “Receiver buffer size negotiated”

- Replace “Transmitter”, “Buffer”, and “Originator” with “transmitter”, “buffer”, and “originator”, respectively.

Proposed resolution:

Revised

For Figure 10-39, replace WinStartO with WinStartO, WinEndO with WinEndO, WinStartO with WinStartO, BufSizeO with BufSizeO, and WinCapacityO with WinCapacityO as suggested by the commenter.  In addition, replace WinHeadO with WinHeadB, WinTailO with WinTailB, “Starting Seq. Number” with “Starting sequence number”, “Receiver Buffer size negotiated” with “Receiver buffer size negotiated”, “Transmitter” with “transmitter”,, “Buffer” with “buffer”, and “Originator” with “originator”.

Replace WinStartR with WinStartR at 1884.62, 1884.64, 1885.7, 1885.8, 1885.12, and 1885.15.

Note to the editors:  The revised figure is given as follows.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4324 |  |  |  | "Commit" and "Confirm" should be preceded by "SAE" in the context of SAE | Fix in "shall transmit the new Commit and Confirm to the peer.", "the same as the previously received Commit frame", "transmit its Commit and Confirm (with the new Sc value)", "an integer representing the peer's Confirm", "a new Confirm (with the new Sc value)"; "the Confirm portion of the frame"; also "Commit or Confirmed state" should be "Committed or Confirmed state" |

Discussion:

The comment is related to the following 7 sentences in clause 12.

- At 2584.36 in subclause 12.4.8.6.4 (Protocol instance behavior - Committed state)

The commenter’s suggestion is to replace “new Commit and Confirm” with “new SAE Commit and SAE Confirm”.   Refer to the subclause 12.4, I would replace “new Commit and Confirm” with “SAE Commit message and SAE Confirm message”.

- At 2585.14 in subclause 12.4.8.6.5 (Protocol instance behavior - Confirmed state)

Actually, it is not precisely a Commit frame.  As referred to 2585.7, it is a rejection frame received in Confirmed state.  I would suggest replacing “the previously received Commit frame” with “the previously received rejection frame”.

- At 2584.16 in subclause 12.4.8.6.5 (Protocol instance behavior - Confirmed state) – please see the snapshot above.

Suggest to replace “its Commit and Confirm (with the new Sc value) messages” with “its SAE 	Commit and SAE Confirm (with the new Sc value) messages”.

- At 2578.27 in subclause 12.4.7.5 (Encoding and decoding of SAE Confirm messages)

Suggest to replace “the peer’s Confirm” with “the peer’s SAE Confirm message”.

- At 2585.32 in subclause 12.4.8.6.5 (Protocol instance behavior - Confirmed state)

Suggest to replace “a new Confirm (with the new Sc value) Message” with “a new SAE Confirm (with the new Sc value) message” as there is an abuse of capitalization for the word “message”.

- At 2585.48 in subclause 12.4.8.6.6 (Protocol instance behavior - Accepted state)

Suggest to replace “the Confirm portion of the frame” with “the SAE Confirm message of the frame”.

- At 3881.23 in clause C.3

The commenter is correct that there is no “Commit state”.  Agree to replace “Commit or Confirmed 	state” with “Committed or Confirmed state”.

Proposed resolution:

Revised

At 2584.36, replace “new Commit and Confirm” with “SAE Commit message and SAE Confirm message”.

At 2585.14, replace “the previously received Commit frame” with “the previously received rejection frame”.

At 2584.16, replace “its Commit and Confirm (with the new Sc value) messages” with “its SAE Commit and SAE Confirm (with the new Sc value) messages”.

At 2578.27, replace “the peer’s Confirm” with “the peer’s SAE Confirm message”.

At 2585.32, replace “a new Confirm (with the new Sc value) Message” with “a new SAE Confirm (with the new Sc value) message”.

At 2585.48, replace “the Confirm portion of the frame” with “the SAE Confirm message of the frame”.

At 3881.23, replace “Commit or Confirmed state” with “Committed or Confirmed state”.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4111 | 10.40.1 | 2002 | 13 | "... the DMG AP Or PCP Capability Information field ..." should read "... the DMG AP or PCP Capability Information field ...". | Replace "Or" with "or" in the sentence in the comment. |

Discussion:

The comment is related to the following sentence:

As per Figure 9-549, however, the field is indeed called “DMG AP Or PCP Capability Information”, i.e., “O” is capitalized.

Proposed resolution:

Rejected.

The name of the field is “DMG AP Or PCP Capability Information” as shown in Figure 9-549 of Draft 3.0, instead of “DMG AP or PCP Capability Information” as suggested by the commenter.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4384 |  |  |  | Why is the top of Figure 12-26--Expanded GCMP MPDU emboldened compared with Figure 12-16--Expanded CCMP MPDU? | Embolden the top line of the top row of cells in Figure 12-16. Also, in Figure 12-17 change "Data PDU" to "Data (PDU)<linebreak>>= 1 octet" (with the >= glyph) |

Discussion:

The first portion of the comment is related to the following 2 figures:

These 2 figures are probably drawn by different editors.

For Figure 12-17, the representation of Data (PDU) is not consistent with that in Figures 12-16 and 12-26 as the size of the Data (PDU) is missing.

Proposed resolution:

Accepted

Note to the editors:  The revised Figure 12-16 and Figure 12-17 are shown as follows.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4307 |  |  |  | What does GF stand for? "HT-greenfield format (HT_GF)" suggests it stands for Greenfield format, but then "HT_GF format" is pleonastic | As it says in the comment |

Discussion:

HT_GF means HT-greenfield format as per subclause 19.1.4 (PPDU formats):

There are 3 appearances of “HT_GF format” in Draft 3.0 and all of them are located in clause 19.

At 3065.17 in subclause 19.3.19.5.4 (CCA sensitivity in 20 MHz):

At 3065.41 and 3065.44 in subclause 19.3.19.5.5 (CCA sensitivity in 40 MHz):

Proposed resolution:

Revised.

At 3065.17, 3065.41, and 3065.44, replace “HT_GF format PPDUs” with “HT_GF PPDUs”.

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4020 | 11.22.16.3.4 | 2402 | 35 | Frame exchange operation is described in Annex G. The title should be renamed to reflect its scope regarding the GCR procedures as a whole, not specifically frame exchange. | Change "GCR frame exchange procedures" to "GCR procedures" at line 35 and line 28 and 39. |
| 4021 | 11.22.16.4.2 | 2408 | 45 | Frame exchange operation is described in Annex G. The title should be renamed to reflect its scope regarding the GLK-GCR procedures as a whole, not specifically frame exchange. | Change "GLK-GCR frame exchange procedures" to "GLK-GCR procedures" |

Discussion:

These 2 comments are related to the title of the following 2 subclauses:

Agree with the commenter that the contents of these 2 subclauses are not limited to the frame exchange procedures, but also, e.g., the policy update.  Note that line 28 suggested by the commenter should be line 38.

Proposed resolution for CID 4020:

Revised.

At 2402.35 and 2402.38, replace “GCR frame exchange procedures” with “GCR procedures”.

At 2402.39, replace “GLK-GCR frame exchange procedures” with “GLK-GCR procedures”.

Proposed resolution for CID 4021:

Accepted.