IEEE P802.11
Wireless LANs

| Comment resolution on MIBS and PICS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-07-27 |  |  |  |  |
| Author: |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Edward Au | Huawei Technologies | 303 Terry Fox Drive, Suite 400, Ottawa, Ontario K2K 3J1 |  | edward.ks.au@gmail.com |

This submission present proposed resolutions for CIDs 24210, 24539, 24536, 24533, 24333, 24534, 24531, 24532, 24411, 24412, 24538, 24443, 24444, 24445, 24209.  The proposed changes of the CIDs starting from R3 are based on P802.11ax D6.1.

##### Revision history:

##### R0 – initial version

##### R1 – update to 24212 and 24532 during the CRC call on May 5.

##### R2 – update to 24210 during the CRC call on May 5.

R3 – update to 24351, 24531, 24532, and 24212; include resolutions of the following additional CIDs: 24209, 24411, 24412, 24538, 24443, 24444, 24445.

R4 – more updates based on the comments received during the July 23rd CRC call.

R5 – remove CIDs 24359 and 24212.  Update to CID 24209.

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24210 | B.4.33.1 | 727 | 6 | Why is "Signaling of STA and BSS capabilities in Beacon, Probe Response, (Re)Association Response frames" not Mandatory for CFHE | Clarify or correct | Revised. Agree in principle with the commenter that the entries are related to HE. Instruction to TGax editor: Replace “(CFVHT AND CFAP):M” with “(CFHE AND CFAP):M”. Replace “Signaling of STA and BSS capabilities in Beacon, Probe Response, (Re)Association Response frames” with “Signaling of HE STA capabilities and HE BSS capabilities in Beacon, Probe Response, (Re)Association Response frames’ |

Discussion:

The commenter refers to the following entries in PICS:

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24539 | C.3 | 764 | 15 | The current SMTbase is dot11SMTbase15, per REVmd. SMTbase14 was skipped (for accidental, historical reasons), so 802.11ax _could_ use it. But, that seems very counter-intuitive to have dot11SMTbase14 replace/update dot11SMTbase15. | Suggest changing dot11SMTbase14 to dot11SMTbase16. Also, instructions need to be given to mark dot11SMTbase15 deprecated, and replaced by this new dot11SMTbase (whatever it's called). | Revised. Agree in principle that the numbering of dot11SMTbase should be in ascending order. Instruction to TGax editor: At 764.14, 764.38, and 766.30, replace “dot11SMTbase14” with “dot11SMTbase16”. At 764.21, replace “SMTbase14” with “SMTbase16”. At 764.10, copy the dot11SMTbase15 OBJECT-GROUP from P802.11REV D3.2 and add the following at the beginning of the DESCRIPTION of this object-group: “Superseded by dot11SMTbase16”. |

Discussion:

The commenter refers to the following MIB object-group:

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24536 | C.3 | 747 | 38 | dot11MemberOfColocated6GHzESSOptionActivated appears to be a control variable (would need to be set ("controlled") by some external entity that make the determination about the ESS). It needs to be read-write. | Change the MAX-ACCESS to "read-write" . Add to the DESCRIPTION that it is written by an external management entity. | Revised. Agree in principle with the commenter on both comments (i.e., read-write, and written by external management entity). Revised instead of Accept because of editorial changes. Instruction to TGax editor: At 747.40, replace “read-only” with “read-write”. At 747.44, add the following new paragraph after the first paragraph in the DESCRIPTION: “It is written by an external management entity”. |

Discussion:

The commenter refers to the following MIB object:

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24533 | C.3 | 745 | 35 | dot11SRGAPOBSSPDMinOffset says it is a control variable and writen by external management entity. Then it needs to be "read-write". | Change the MAX-ACCESS to "read-write". Same with the next 4 attributes (through dot11NonSRGAPOBSSPDMaxOffset). | Accepted. Note to the editors: The locations to replace “read-only” with “read-write” are 745.35, 745.49, 745.63, 746.12, and 746.27. |

Discussion:

The commenter refers to the following 5 MIB objects:

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24333 | C.3 | 751 | 50 | dot11MUEDCATableAIFSN's description says "This attribute specifies the number of slots, after a SIFS, that the STA, for a particular AC, senses the medium idle either before transmitting or executing a backoff." but this is not true if the value is 0 | Add ", except that the value 0 indicates that EDCA is disabled for that AC" before the full stop | Accepted |

Discussion:

The commenter refers to the following MIB object:

The commenter is correct that the scenario when the AIFSC field is equal to 0 (c.f., see below) is not discussed in the DESCRIPTION of the MIB object.

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24534 | C.3 | 747 | 27 | dot11UnsolicitedProbeResponseOptionActivated appears to be a simple control variable over the AP's behavior. It needs to be read-write. | Change the MAX-ACCESS to "read-write" . Add to the DESCRIPTION that it is written by an external management entity. | Revised. Agree in principle with the commenter on both comments (i.e., read-write, and written by external management entity). Revised instead of Accept because of editorial changes. Instruction to TGax editor: At 751.36, replace “read-only” with “read-write”. At 751.41, add the following new paragraph after the first paragraph in the DESCRIPTION: “It is written by an external management entity. Changes take effect as soon as practical in the implementation. ”. |

Discussion:

The commenter refers to the following MIB object:

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24531 | C.3 | 742 | 44 | What is the purpose of a read-only attribute holding this duration? If it is a fixed value, just say so in the main text and remove this attribute. If it can be changed, it needs to be read-write and describe who writes it (an external management entity, for example?) and when changes take effect. | Delete dot11BSSColorCollisionAPPeriod and replace it with "50 seconds" in the body text. Similarly, for dot11BSSColorCollisionSTAPeriod, and "5 seconds", and "dot11ObssNbRuToleranceTime" and "1800 seconds". | Revised The MAX-ACCESS of these 3 MIB objects are “read-write” rather than “read-only”. For dot11BSSColorCollisionAPPeriod and dot11BSSColorCollisionSTAPeriod, the changes are made in the resolution of CID 24249 as shown in https://mentor.ieee.org/802.11/dcn/20/11-20-0316-04-00ax-resolution-for-cids-related-to-bss-color.docx. Instruction to TGax editor: At 748.24, replace “read-only” with “read-write”. At 748,27, insert the following sentences at the beginning of the DESCRIPTION: “It is a status variable. It is written by an external management entity. Changes take effect as soon as practical in the implementation. ”. |

Discussion:

The commenter refers to the following 3 MIB objects:

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24532 | C.3 | 742 | 62 | dot11AutonomousBSSColorCollisionReportingImplemented claims to be a capability (and looks like one, in the body text). It should be read-only. | Change the MAX-ACCESS to "read-only" | Revised Instruction to TGax editor: At 7476, change the MAX-ACCESS to "read-only". At 747.13, delete “DEFVAL {false}”. |

Discussion:

The commenter refers to the following MIB object:

As per the document 15/0355r13, a feature is dynamic or it is not (i.e., static).  It cannot be both:

•	dot11<XXX>Implemented:  A static implementation pattern is for a feature that is an inherent capability of a given implementation.  As an “inherent” capability, this pattern is for features that are permanently operational in an instantiation of an implementation that supports it – that is, it is not enabled or disabled dynamically during the lifetime of an instance of the implementation.

•	dot11<XXX>Activated:  This pattern is for a feature that, when present in an implementation, becomes operational or non-operational dynamically within the lifetime of a particular instance of the implementation.  Such dynamic changes occur as a result of behaviors or interactions described within Std 802.11, for example, based on a protocol exchange, or receiving an enablement indication from a peer entity, or as a result of an external entity writing to the MIB attribute.  It is critical to unambiguous description of the behavior that only one entity be able to change the attribute, whether that is an internal or external entity.

A static feature is described in this MIB variable and so, dot11<XXX>Implemented is appropriate.

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24411 | B.4.4.1 | 724 | 35 | The PICS implies that if you support dynamic fragmentation at a certain level, you have to support it for both tx and rx. But this seems unnecessarily restrictive | As it says in the comment | Revised. Instruction to TGax editor: Please incorporate the changes as shown in 20/0665r4 under the CIDs 24211 and 24212. |
| 24412 | B.4.4.1 | 724 | 35 | The PICS implies that if you support dynamic fragmentation at a certain level, you have to support it for both tx and rx. But this seems unnecessarily restrictive | Duplicate the PC46.1 row into PC46.1.1 and PC46.1.2 rows, then in one of the rows make the second cell "Dynamic fragmentation level 1 MSDU/MMPDU transmission" and in the other "Dynamic fragmentation level MSDU/MMPDU reception" | Revised. Instruction to TGax editor: Please incorporate the changes as shown in 20/0665r4 under the CIDs 24211 and 24212. |

Discussion – the followings are the PICS entries the commenter refer to:

Proposed changes for CIDs 24211 and 24212:

| PC46 | Dynamic fragmentation | 10.3 (DCF), 10.4 (MSDU, A-MSDU, and MMPDU fragmentation) |  |  |
| --- | --- | --- | --- | --- |
| PC46.1 | Dynamic fragmentation level 1 | 26.3.2.2 (Level 1 dynamic fragmentation) |  |  |
| PC46.1.1 | Dynamic fragmentation level 1 MSDU/MMPDU transmission | 26.3.2.2 (Level 1 dynamic fragmentation) | CFHE:O | Yes No  |
| PC46.1.2 | Dynamic fragmentation level 1 MSDU/MMPDU reception | 26.3.2.2 (Level 1 dynamic fragmentation) | CFHE:O | Yes No  |
| PC46.2 | Dynamic fragmentation level 2 | 26.3.2.3 (Level 2 dynamic fragmentation) |  |  |
| PC46.2.1 | Dynamic fragmentation level 2 MSDU/MMPDU transmission | 26.3.2.3 (Level 2 dynamic fragmentation) | CFHE:O | Yes No  |
| PC46.2.2 | Dynamic fragmentation level 2 MSDU/MMPDU reception | 26.3.2.3 (Level 2 dynamic fragmentation) | CFHE:O | Yes No  |
| PC46.3 | Dynamic fragmentation level 3 | 26.3.2.4 (Level 3 dynamic fragmentation) |  |  |
| PC46.3.1 | Dynamic fragmentation level 3 MSDU/MMPDU transmission | 26.3.2.4 (Level 3 dynamic fragmentation) | CFHE:O | Yes No  |
| PC46.3.2 | Dynamic fragmentation level 3 MSDU/MMPDU reception | 26.3.2.4 (Level 3 dynamic fragmentation) | CFHE:O | Yes No  |

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24538 | C.3 | 754 | 57 | dot11HEDeviceClass appears to be a status (the current state of operation) not a capability. It is not used in body text, however, so it's hard to confirm. Also, it seems like a bad choice to use a boolean to indicate a state that isn't "Yes"/"No". A two-value enumeration would be better. | If the concept is considered clear enough in body text to just reference "Class A device" and "Class B device" (which seems to be the case), just delete this MIB attribute. Otherwise, add references in the body text where the concept appears, and fix-up the MIB definition as described in the comment. | Revised. Instruction to TGax editor: Please incorporate the changes as shown in 20/0665r4 under the CID 24538. |

Discussion – the followings are the texts that the commenter refers to:

C.3, 759.2:

26.5.2.1, 345.43:

Proposed changes for CID 24538:

At 759.2:

dot11HEDeviceClass OBJECT-TYPE

SYNTAX INTEGER{ClassB(0), ClassA(1)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute specifies the device class of a non-AP STA

."

::= { dot11PhyHEEntry 3}

At 345.43:

If a non-AP STA has dot11HEDeviceClass equal to ClassA(1), then it indicates Class A in the Device Class subfield of the HE PHY Capabilities Information

field of the HE Capabilities element and the HE TB PPDUs it transmits shall meet the Class A requirements in 27.3.15 (Transmit requirements for PPDUs sent in response to a triggering frame). Otherwise, it indicates Class B in the Device Class subfield of the HE PHY Capabilities Information

field of the HE Capabilities element and the HE TB PPDUs it transmits shall meet the Class B requirements in 27.3.15 (Transmit requirements for PPDUs sent in response to a triggering frame).

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24444 | C.3 | 754 | 21 | CID 22364 (previously 20503). It's still broken. The Supported Channel Width Set field is a 7-bit *bitmap*, not an *enumeration* allowing values 0-6 (even ignoring the fact that (b)6 is reserved), However, dot11HECurrentChannelWidthSet is not used anyway. Furthermore, "The PHY shall set dot11HECurrentChannelWidthSet to a value that is obtained from the Supported Channel Width Set subfield of a transmitted HE Capabilities element." is not precise enough -- transmitted by whom? | Change dot11HECurrentChannelWidthSet to "dot11HECurrentChannelWidthSet OBJECT-TYPE SYNTAX Unsigned32 (0..127) MAX-ACCESS read-only STATUS current DESCRIPTION "This is a status variable. This attribute specifies the STA's channel width set." REFERENCE "IEEE Std 802.11ax-202x, Table 9-321b--Subfields of the HE PHY Capabilities Information field, Supported Channel Width Set subfield" ::= { dot11PhyHEEntry 1} In "The PHY shall set dot11HECurrentChannelWidthSet to a value that is obtained from the Supported Channel Width Set subfield of the HE Capabilities element transmitted by the STA." | Accepted. |
| 24445 | C.3 | 754 | 21 | CID 22364 (previously 20503). It's still broken. The Supported Channel Width Set field is a 7-bit *bitmap*, not an *enumeration* allowing values 0-6 (even ignoring the fact that (b)6 is reserved), However, dot11HECurrentChannelWidthSet is not used anyway. Furthermore, "The PHY shall set dot11HECurrentChannelWidthSet to a value that is obtained from the Supported Channel Width Set subfield of a transmitted HE Capabilities element." is not precise enough -- transmitted by whom? And also the PHY has no way of finding out what Supported Channel Width Set subfield was transmitted, so this is not implementable. And dot11HECurrentChannelWidthSet is not used anywhere | In 27.2.4 delete "The PHY shall set dot11HECurrentChannelWidthSet to a value that is obtained from the Supported Channel Width Set subfield of a transmitted HE Capabilities element.". In C.3 delete dot11HECurrentChannelWidthSet (including from Dot11PhyHEEntry and dot11PhyHEComplianceGroup). In Table 27-54--HE PHY MIB attributes delete the dot11HECurrentChannelWidthSet row | Revised. At 758.32 Change dot11HECurrentChannelWidthSet to "dot11HECurrentChannelWidthSet OBJECT-TYPE SYNTAX Unsigned32 (0..127) MAX-ACCESS read-only STATUS current DESCRIPTION "This is a status variable. This attribute specifies the STA's channel width set." REFERENCE "IEEE Std 802.11ax-202x, Table 9-321b--Subfields of the HE PHY Capabilities Information field, Supported Channel Width Set subfield" ::= { dot11PhyHEEntry 1} At 499.45: Change the identified sentence to "The PHY shall set dot11HECurrentChannelWidthSet to a value that is obtained from the Supported Channel Width Set subfield of the HE Capabilities element transmitted by the STA." |
| 24443 | C.3 | 754 | 21 | CID 22364 (previously 20503). It's still broken. The Supported Channel Width Set field is a 7-bit *bitmap*, not an *enumeration* allowing values 0-6 (even ignoring the fact that (b)6 is reserved), However, dot11HECurrentChannelWidthSet is not used anyway. Furthermore, "The PHY shall set dot11HECurrentChannelWidthSet to a value that is obtained from the Supported Channel Width Set subfield of a transmitted HE Capabilities element." is not precise enough -- transmitted by whom? | It needs to be an unsigned integer in the range 0..63 (since b6 is reserved) | Revised At 758.32 Change dot11HECurrentChannelWidthSet to "dot11HECurrentChannelWidthSet OBJECT-TYPE SYNTAX Unsigned32 (0..127) MAX-ACCESS read-only STATUS current DESCRIPTION "This is a status variable. This attribute specifies the STA's channel width set." REFERENCE "IEEE Std 802.11ax-202x, Table 9-321b--Subfields of the HE PHY Capabilities Information field, Supported Channel Width Set subfield" ::= { dot11PhyHEEntry 1} At 499.45: Change the identified sentence to "The PHY shall set dot11HECurrentChannelWidthSet to a value that is obtained from the Supported Channel Width Set subfield of the HE Capabilities element transmitted by the STA." |

Discussion – the followings are the texts that the commenter refers to:

At C.3, 758.32:

Proposed text change of the commenter:

dot11HECurrentChannelWidthSet OBJECT-TYPE
	SYNTAX Unsigned32 (0..127)
	MAX-ACCESS read-only
	STATUS current
	DESCRIPTION
		"This is a status variable.
		This attribute specifies the STA's channel width set."

REFERENCE "IEEE Std 802.11ax-202x, Table 9-321b--Subfields of the HE PHY Capabilities

Information field, Supported Channel Width Set subfield"
::= { dot11PhyHEEntry 1}

At 499.45:

Proposed text change of the commenter:

The PHY shall set dot11HECurrentChannelWidthSet to a value that is obtained from the Supported Channel Width Set subfield of the HE Capabilities element transmitted by the STA."

At 192.5:

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24209 | B.4.3 | 724 | 19 | Is it the correct understanding from lines 19 and 25 that a CFHE6G device has to mandatorily support 2.4 and 5 GHz? Would that make tri-band operation mandatory? | Clarify. If the intention is to support either 2.4 or 5 GHz, use O#n notation. | Revised CF2G4n6G and CF5Gn6G are used only for the out-of-band discovery of a 6 GHz AP when an AP operating in the 2.4 GHz or 5 GHz band is in the same co-located AP set as a 6 GHz AP. It does not mean that a CFHE6G device has to mandatorily support 2.4 and 5 GHz. It also does not mean that tri-band operation is mandatory. Instruction to TGax editor: Please incorporate the changes as shown in 20/0665r5 under the CID 24209. |

Discussion:

Proposed changes for CID 24209:

| CF2G4n6G | An AP that operates in the 2.4 GHz band and that is in the same co-located AP set as one or more 6 GHz APs | 26.17.2.4 (Out of band discovery of a 6 GHz BSS) | (CFHT2G4 or CFHE2G4):O CFHE6G:M | Yes No  N/A  |
| --- | --- | --- | --- | --- |
| CF5Gn6G | An AP that operates in the 5 GHz band and that is in the same co-located AP set as one or more 6 GHz APs | 26.17.2.4 (Out of band discovery of a 6 GHz BSS) | (CFHT5G or CFHE5G):O CFHE6G:M | Yes No  N/A  |