IEEE P802.11
Wireless LANs

| Resolution to CID 4161 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Montemurro | Self |  |  | montemurro.michael@gmail.com |

Background

This contribution proposes a resolution to CID 4161

### Comment

REVmd WG CC Comments

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 4161 | 4383.00 | E |  |  | The Australian channel numbers 28 and 32 in global operating class 69 are not represented in the US set of channel numbers for the same global operating class (crikey!). Is there a reason why channels 30 and 34 aren't used for Australia instead of 28 and 32? Not using a common set of channel numbers for 2 MHz may not be a problem if scanning for networks is done using 1 MHz channels, but somehow I think it would be safer to have a common set of channel numbers. In addition if we managed to get additional spectrum below 915 MHz we would need to renumber anyway to make the channels contiguous. A similar problem occurs for 4 MHz channel numbering and for the same channels in New Zealand. | Review and renumber Australian and New Zealand 2 MHz and 4 MHz channel numbers. |

Discussion

The overall band allocations have been verified through the following documents:

Australia: https://www.legislation.gov.au/Details/F2019C00681

New Zealand: https://gazette.govt.nz/notice/id/2019-go1588

United States: https://www.ecfr.gov/cgi-bin/text-idx?SID=c7a7ec9fb344960247a175e50d92201e&mc=true&node=pt47.1.15&rgn=div5#se47.1.15_1245

Based on the overall channelization and band allocation based on US channels, a graphical representation of the channelization is given below for 2MHz, 4MHz, and 8 MHz channels.

This should actually be shifted to be channel 44

US Channels

900 MHz frequencies

US above, others below

New Zealand Channels

Australia Channels

It appears that for AUS:

OC 22, "28,32" should be "30, 34"

OC 23, "30" should be "32"

For NZ,

OS 27, "28, 32, 36, 40" should be "30, 34,38, 42"

OS 28, "30, 38" should be "32, 40"

OS 29, "34" should be "44”

Proposed Resolution:

The channel numbers in the Australia and New Zealand tables appear to be incorrect (do not match the channelization with other countries.

With reference to D3.0

At 4383.46 (row 22), change "28, 32" to "30, 34"

At 4383.52 (row 23), change “30” to “32”

At 4384.16 (row 27), change “28, 32, 36, 40” should be “30, 34,38, 42”

At 4384.21 (row 28), change “30, 38” to “32, 40”

At 4384.27 (row 29), change “34” to “44”