IEEE P802.11
Wireless LANs

| Proposed Spec Draft Text of EHT PHY TXVECTOR/RXVECTOR/TRIGVECTOR/CONFIG_VECTOR |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-08-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Bo Sun | ZTE | ZTE R&D center, #9 Wuxingduan, Xifeng Rd., Chang’an district, Xi’an, China |  | Sun.bo1@zte.com.cn |
| Youhan Kim | QualComm |  |  |  |

Abstract

This document is a spec draft text proposal for IEEE P802.11be D0.1 regarding sub-clause 34.x.x (TXVECTOR and RXVECTOR), sub-clause 34.x.x (TRIGVECTOR parameters) and sub-clause 34.x.x (PHY CONFIG_VECTOR).

Revision History

- R0, initial draft for group review.

- R1, editorial update.

- R2, remove 80+160 MHz.

- R3, add TTT members

---------------------Below is the proposed sub-clause 34.x.x (TXVECTOR and RXVECTOR)-----------------

- 2 EHT PHY service interface

34.2.1 Introduction

tbd.

34.x.2 TXVECTOR and RXVECTOR

The parameters in Table 34-1 (TXVECTOR and RXVECTOR parameters) are defined as part of the TXVECTOR parameter list in the PHY-TXSTART.request primitive and/or as part of the RXVECTOR parameter list in the PHY-RXSTART.indication and PHY-RXEND.indication primitives.

Table 34-1 TXVEROR and RXVECTOR Parameters

| Parameter | Condition | Value | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- | --- |
| FORMAT |  | Determines the format of the PPDU. Enumerated type: NON_HT indicates Clause 15, Clause 16, Clause 17, Clause 18 or non-HT duplicate PPDU format. In this case, the modulation is determined by the NON_HT_MODULATION parameter. HT_MF indicates HT-mixed format. HT_GF indicates HT-greenfield format. VHT indicates VHT format. HE_SU indicates HE SU PPDU format. HE_MU indicates HE MU PPDU format. HE_ER_SU indicates HE ER SU PPDU format. HE_TB indicates HE TB PPDU format. EHT_MU indicates EHT MU PPDU format. EHT_TB indicates EHT TB PPDU format. | Y | Y |
| NON_HT_MODULATION | See corresponding entry in Table 19-1 (TXVECTOR and RXVECTOR parameters). |  |  |  |
| L_LENGTH | FORMAT is EHT_MU | Not present. | N | N |
|  | FORMAT is EHT_TB | TBD | TBD | TBD |
|  | Otherwise | See corresponding entry in Table 19-1 (TXVECTOR and RXVECTOR parameters), Table 21-1 (TXVECTOR and RXVECTOR parameters), or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| N_TX | FORMAT is EHT_MU or EHT_TB | Indicates the number of transmit chains. | Y | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| EXPANTION_MAT | FORMAT is EHT_MU | TBD | TBD | TBD |
|  | FORMAT is EHT_TB | TBD | TBD | TBD |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| CHAN_MAT | FORMAT is EHT_MU | TBD | TBD | TBD |
|  | FORMAT is EHT_TB | TBD | TBD | TBD |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| DELTA_SNR | FORMAT is EHT_MU | TBD | TBD | TBD |
|  | FORMAT is EHT_TB | TBD | TBD | TBD |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| RCPI | See corresponding entry in Table 19-1 (TXVECTOR and RXVECTOR parameters), Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |  |
| NO_SIG_EXTN | FORMAT is EHT_MU or EHT_TB | Indicates whether signal extension needs to be applied at the end of transmission. Boolean: true indicates that no signal extension is present. false indicates that a signal extension is present. | Y | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| SNR | FORMAT is EHT_MU | TBD |  |  |
|  | FORMAT is EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 19-1 (TXVECTOR and RXVECTOR parameters), Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| CQI | FORMAT is EHT_MU | TBD |  |  |
|  | FORMAT is EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| STBC | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 19-1 (TXVECTOR and RXVECTOR parameters), Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| GI_TYPE | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| MCS_SIG_B | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| DCM_SIG_B | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| SIG_B_COMPRESSION_MODE | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| HE_LTF_TYPE | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| HE_LTF_MODE | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| NUM_HE_LTF | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| HE_SIG_A2_RESERVED | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| TXOP_PS_NOT_ALLOWED | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| RX_START_OF_FRAME_OFFSET | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters). |  |  |  |
| PREAMBLE_TYPE | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| SCRAMBLER_INITIAL_VALUE | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| TRIGGER_RESPONDING | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| HE_PRE_FEC_PADDING_FACTOR | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| HE_TB_PE_DISAMBIGUITY | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| MU_COMPRESSION_MODE | FORMAT is EHT_MU | Indicates whether or not the RU Allocation subfield(s) is included in the Common field of the EHT-SIG. Integer: 0 indicates that the RU Allocation subfield is present 1 indicates that the RU Allocation subfield is not present | Y | N |
|  | Otherwise | Not present |  |  |
| FEC_CODING | FORMAT is EHT_MU or EHT_TB | Indicates the FEC encoding used. Enumerated type: BCC_CODING indicates BCC coding. LDPC_CODING indicates LDPC coding. | MU | MU |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| LDPC_EXTRA_SYMBOL | FORMAT is EHT_TB | Indicates the presence of the LDPC extra symbol segment in an EHT TB PPDU. Integer: 1 indicates that an LDPC extra symbol segment is present. 0 indicates that an LDPC extra symbol segment is not present. | Y | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| TXPWR_LEVEL_INDEX | FORMAT is EHT_MU or EHT_TB | The allowed values for the TXPWR_LEVEL_INDEX parameter are in the range from 1 to numberOfOctets (dot11TxPowerLevelExtended)/2. This parameter is used to indicate which of the available transmit output power levels defined in dot11TxPowerLevelExtended shall be used for the current transmission. | Y | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| RSSI | FORMAT is EHT_MU or EHT_TB | The allowed values for the RSSI parameter are in the range 0 to 255 inclusive. This parameter is a measure by the PHY of the power observed at the antennas used to receive the current PPDU measured during the reception of the EHT-LTF field. RSSI is intended to be used in a relative manner, and it is a monotonically increasing function of the received power. | N | Y |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| RSSI_LEGACY | FORMAT is EHT_MU or EHT_TB | The allowed values for the RSSI_LEGACY parameter are in the range 0 to 255 inclusive. This parameter is a measure by the PHY of the power observed at the antennas used to receive the current PPDU measured during the reception of non-EHT portion of the EHT PPDU preamble. RSSI_LEGACY is intended to be used in a relative manner, and it is a monotonically increasing function of the received power. | N | Y |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| MCS | FORMAT is EHT_MU or EHT_TB | Indicates the modulation and coding schemes used in the transmission of the PPDU. Integer: range 0 to TBD | MU | MU |
|  | Otherwise | See corresponding entry in Table 19-1 (TXVECTOR and RXVECTOR parameters), Table 21-1 (TXVECTOR and RXVECTOR parameters), or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| DCM | FORMAT is EHT_MU or EHT_TB | TBD | MU | MU |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| MCS_EHT_SIG | FORMAT is EHT_MU or EHT_TB | Indicates the modulation and coding scheme used for EHT_SIG field. Integer: TBD | Y | Y |
|  | Otherwise | Not present |  |  |
| REC_MCS | FORMAT is EHT_MU | Indicates the EHT-MCS that the receiver recommends | N | O |
|  | FORMAT is EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| CH_BANDWIDTH | FORMAT is EHT_MU | TBD | Y | Y |
|  | FORMAT is EHT_TB | TBD | Y | Y |
|  | Otherwise | See corresponding entry in Table 19-1 (TXVECTOR and RXVECTOR parameters), Table 21-1 (TXVECTOR and RXVECTOR parameters), or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| INACTIVE_SUBCHANNELS | FORMAT is EHT_MU | TBD |  |  |
|  | FORMAT is EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| DYN_BANDWIDTH_IN_NON_HT | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| CH_BANDWIDTH_IN_NON_HT | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| LENGTH | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| APEP_LENGTH | FORMAT is EHT_MU or EHT_TB | tbd | MU | O |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| PSDU_LENGTH | FORMAT is EHT_MU or EHT_TB | tbd | N | Y |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| USER_POSITION | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| NUM_STS | FORMAT is EHT_MU | Indicates the number of space-time streams. Integer in the range: 1-4 per user per MU-MIMO RU in the TXVECTOR 1-4 per MU-MIMO RU in the RXVECTOR 1-tbd per RU assigned to no more than 1 user in the TXVECTOR and RXVECTOR NUM_STS summed over all users per RU is not greater than tbd. | MU | Y |
|  | FORMAT is EHT_TB | Indicates the number of space-time streams. Integer in the range: 1-4 for a MU-MIMO RU 1-tbd for an RU assigned to no more than 1 user NUM_STS summed over all users per RU is not greater than tbd. | Y | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| GROUP_ID | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| PARTIAL_AID | FORMAT is EHT_MU or EHT_TB | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| TXOP_DURATION | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| SPATIAL_REUSE | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| DOPPLER | FORMAT is EHT_MU or EHT_TB | Set to 1 to indicate that either the midamble is present for the PPDU or that the channel is with high Doppler. Set to 0 otherwise. | Y | Y |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| NUM_USERS | FORMAT is EHT_MU or EHT_TB | Not present |  |  |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| RU_ALLOCATION | FORMAT is EHT_MU | TBD |  |  |
|  | FORMAT is EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| BEAMFORMED | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) or Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| CENTER_26_TONE_RU | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| EHT_LTF_TYPE | FORMAT is EHT_MU or EHT_TB | Indicates the type of EHT-LTF. Enumerated type: 1xEHT-LTF indicates a 1x EHT-LTF 2xEHT-LTF indicates a 2x EHT-LTF 4xEHT-LTF indicates a 4x EHT-LTF See 34.3.X.X (EHT-LTF). | Y | Y |
|  | Otherwise | Not present |  |  |
| EHT_LTF_MODE | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | Not present. |  |  |
| NUM_EHT_LTF | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | Not present. |  |  |
| STARTING_STS_NUM | FORMAT is EHT_TB | Set to the starting spatial stream number minus 1 (spatial streams in a given PPDU transmission are numbered starting from 1) | Y | N |
|  | FORMAT is EHT_MU | Not present |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| NOMINAL_PACKET_PADDING | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| TRIGGER_METHOD | FORMAT is EHT_TB | TBD |  |  |
|  | FORMAT is EHT_MU | Not present |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| DEFAULT_PE_DURATION | FORMAT is EHT_TB | TBD |  |  |
|  | FORMAT is EHT_MU | Not present |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| BEAM_CHANGE | FORMAT is EHT_MU | TBD |  |  |
|  | FORMAT is EHT_TB | Not present |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| BSS_COLOR | FORMAT is EHT_MU or EHT_TB | TBD |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| UPLINK_FLAG | FORMAT is EHT_MU | TBD |  |  |
|  | FORMAT is EHT_TB | Not present |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| STA_ID | FORMAT is EHT_MU | TBD |  |  |
|  | FORMAT is EHT_TB | Not present |  |  |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| NDP_REPORT | FORMAT is EHT_TB and PSDU_LENGTH=0 | TBD | N | Y |
|  | FORMAT is EHT_MU | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| FEEDBACK_STATUS | FORMAT is EHT_TB and APEP_LENGTH=0 | TBD | Y | N |
|  | FORMAT is EHT_MU | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| RU_TONE_SET_INDEX | FORMAT is EHT_TB and APEP_LENGTH=0 | TBD | Y | N |
|  | FORMAT is EHT_MU | Not present | N | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). |  |  |
| MIDAMBLE_ PERIODICITY | FORMAT is EHT_MU or EHT_TB, and DOPPLER is 1 | Indicates the midamble periodicity in number of OFDM symbols in thin the Data field. Set to 10 or 20. | Y | N |
|  | Otherwise | See corresponding entry in Table 27-1 (TXVECTOR and RXVECTOR parameters). | N | N |
| EHT_PRE_FEC_PADDING_FACTOR | FORMAT is EHT_TB | TBD |  |  |
|  | Otherwise | Not present | N | N |
| EHT_TB_PE_DISAMBIGUITY | FORMAT is EHT_TB | TBD | Y | N |
|  | Otherwise | Not present | N | N |
| Note: In the “TXVECTOR” and “RXVECTOR” columns, the following apply: Y = ‘Present’; N = ‘Not present’; O = ‘Optional’; |  |  |  |  |

34.x.3 TRIGVECTOR parameters

The TRIGVECTOR is carried in a PHY-TRIGGER.request primitive and provides the PHY of the AP with the parameters needed to receive an EHT TB PPDU over each assigned RU. The parameters in Table 34-2 (TRIGVECTOR parameters) are defined as part of the TRIGVECTOR parameter list in the PHY-TRIGGER.request primitive.

Table 34-2—TRIGVECTOR parameters

| Parameter | Value |
| --- | --- |
| CH_BANDWIDTH | TBD |
| UL_LENGTH | TBD |
| GI_AND_EHT_LTF_TYPE | TBD |
| MU_MIMO_EHT_LTF_MODE | TBD |
| NUM_EHT_LTF_SYMBOLS | TBD |
| MIDAMBLE_PERIODICITY | TBD |
| LDPC_EXTRA_SYMBOL | TBD |
| PRE_FEC_PADDING_FACTOR | Indicates the pre-FEC padding factor for the expected EHT_TB PPDU. Value range TBD |
| PE_DISAMBIGUITY | Indicates the PE disambiguity of the expected EHT_TB PPDU. Value range TBD |
| DOPPLER | Indicates the Doppler mode for the expected EHT_TB PPDU. Value range TBD |
| AID12_LIST | TBD |
| RU_ALLOCATION_LIST | TBD |
| FEC_CODING_LIST | TBD |
| EHT_MCS_LIST | TBD |
| UL_DCM_LIST | TBD |
| SS_ALLOCATION_LIST | TBD |

34.x.4 PHY CONFIG_VECTOR

The PHYCONFIG_VECTOR carried in a PHY-CONFIG.request primitive for an EHT PHY contains an OPERATING_CHANNEL parameter, which identifies the operating or primary channel. The PHY shall set dot11CurrentPrimaryChannel to the value of this parameter.

The PHYCONFIG_VECTOR carried in a PHY-CONFIG.request primitive for an EHT PHY contains a CHANNEL_WIDTH parameter, which identifies the operating channel width and takes one of the values 20 MHz, 40 MHz, 80 MHz, 160 MHz, 80+80 MHz, 320 MHz, and 160+160 MHz and 80+160 MHz. The PHY shall set dot11CurrentChannelWidth to the value of this parameter. The PHY shall set dot11EHTCurrentChannelWidthSet to a value that is obtained from the Supported Channel Width Set subfield of a transmitted EHT Capabilities element

The PHYCONFIG_VECTOR carried in a PHY-CONFIG.request primitive for an EHT PHY contains a CENTER_FREQUENCY_SEGMENT_0 parameter, which identifies the center frequency of the channel (or of segment 0 if the CHANNEL_WIDTH parameter indicates 80+80 MHz, and 160+160 MHz and 80+160 MHz) and takes a value between 1 and 255. The PHY shall set dot11CurrentChannelCenterFrequencyIndex0 to the value of this parameter. The PHYCONFIG_VECTOR carried in a PHY-CONFIG.request primitive for an EHT PHY contains a CENTER_FREQUENCY_SEGMENT_1 parameter, which takes the value 0 if the CHANNEL_WIDTH parameter does not indicate 80+80 MHz, or 160+160 MHz, or 80+160 MHz, and otherwise identifies the center frequency of segment 1 and takes a value between 1 and 200. The PHY shall set dot11CurrentChannelCenterFrequencyIndex1 to the value of this parameter.

References:

- https://mentor.ieee.org/802.11/dcn/20/11-20-0566-64-00be-compendium-of-straw-polls-and-potential-changes-to-the-specification-framework-document.docx

- https://mentor.ieee.org/802.11/dcn/19/11-19-1755-07-00be-compendium-of-motions-related-to-the-contents-of-the-tgbe-specification-framework-document.pptx

- https://mentor.ieee.org/802.11/dcn/19/11-19-1262-13-00be-specification-framework-for-tgbe.docx