IEEE P802.11
Wireless LANs

| EHT-SIG CR on P802.11be D0.3 – part 5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bo Gong | Huawei Technologies |  |  | gongbo8@hisilicon.com |
| Ross Jian Yu | Huawei Technologies |  |  | ross.yujian@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3.

5 Comments

CIDs 1400, 2811, 2934, 3193, and 3005 are resolved in this submission.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 1400

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 255.61 | 36.3.11.8.3 | Notation "[]" should be defined in a note at the foot of the table | Notation "[]" should be defined in a note at the foot of the table | REVISED Agreed in principle. Reflect the detailed explaintation. Instructions to the editor Please make the changes as shown in 11/21-0338r0 Note to the editor: The resolutions for CID 1400, CID 2811, CID 2934, CID 3193 and CID 3005 in 11/21-0338r0 are the same. |

## CID 2811

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 255.61 | 36.3.11.8.3 | Notation "MRU of []-242-484" is not clear. Similar comments on other lines of the Table. | Improve notation | REVISED Agreed in principle. Reflect the detailed explaintation. Instructions to the editor Please make the changes as shown in 11/21-0338r0 Note to the editor: The resolutions for CID 1400, CID 2811, CID 2934, CID 3193 and CID 3005 in 11/21-0338r0 are the same. |

## CID 2934

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 255.61 | 36.3.11.8.3 | In Table 36-26, there is no definition of what the brackets [] mean, e.g. "MRU of []-242-484". | Please add the definition | REVISED Agreed in principle. Reflect the detailed explaintation. Instructions to the editor Please make the changes as shown in 11/21-0338r0 Note to the editor: The resolutions for CID 1400, CID 2811, CID 2934, CID 3193 and CID 3005 in 11/21-0338r0 are the same. |

## CID 3193

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 255.61 | 36.3.11.8.3 | What does the notation "[]-242-484" mean? | Define the notation. | REVISED Instructions to the editor Please make the changes as shown in 11/21-0338r0 Note to the editor: The resolutions for CID 1400, CID 2811, CID 2934, CID 3193 and CID 3005 in 11/21-0338r0 are the same. |

## CID 3005

| Page. Line |  | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 255.96 | 36.3.11.8.3 |  | Rather than using []-242-484, 242-[]-484, etc, better to use MRU index. | See comment. | REVISED MRU indices mapping between the description in Table 36-x, such as 484+242 tone MRU-1, and Table 36-26, such as []-242-484, is provided. (Please refer to Table 36-x in 11-21-0104-03) Instructions to the editor Please make the changes as shown in 11/21-0338r0 Note to the editor: The resolutions for CID 1400, CID 2811, CID 2934, CID 3193 and CID 3005 in 11/21-0338r0 are the same. |

Instructions to the editor

By supplementing some explanation, part of Table 36-26 (RU Allocation subfield) are revised as follows:

| RU Allocation subfield (B8 B7 B6 B5 B4 B3 B2 B1 B0) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | Number of entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 96–103 (001100y2y1y0) | MRU of []-242-484, including 484+242 tone MRU-1, 5, 9, and 13 within the 1st, 2nd, 3rd, and 4th 80 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 104–111 (001101y2y1y0) | MRU of 242-[]-484, including 484+242 tone MRU-2, 6, 10, and 14 within the 1st, 2nd, 3rd, and 4th 80 MHz subblock in increasing order ,respectively |  |  |  |  |  |  |  |  | 8 |
| 112–119 (001110y2y1y0) | MRU of 484-[]-242, including 484+242 tone MRU-3, 7, 11, and 15 within the 1st, 2nd, 3rd, and 4th 80MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 120–127 (001111y2y1y0) | MRU of 484-242-[], including 484+242 tone MRU-4, 8, 12,and 16 within the 1st, 2nd, 3rd, and 4th 80MHz subblock in increasing order ,respectively |  |  |  |  |  |  |  |  | 8 |
| 128–135 (010000y2y1y0) | MRU of []-484-996, including 996+484 tone MRU-1 and 5 within the 1st and 2nd 160 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 136–143 (010001y2y1y0) | MRU of 484-[]-996, including 996+484 tone MRU-2 and 6 within the 1st and 2nd 160 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 144–151 (010010y2y1y0) | MRU of 996-[]-484, including 996+484 tone MRU-3 and 7 within the 1st and 2nd 160 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 152–159 (010011y2y1y0) | MRU of 996-484-[], including 996+484 tone MRU-4 and 8 within the 1st and 2nd 160 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 160–167 (010100y2y1y0) | MRU of []-996-996-996, including 3×996-tone MRU-1 |  |  |  |  |  |  |  |  | 8 |
| 168–175 (010101y2y1y0) | MRU of 996-[]-996-996, including 3×996-tone MRU-2 |  |  |  |  |  |  |  |  | 8 |
| 176–183 (010110y2y1y0) | MRU of 996-996-[]-996, including 3×996-tone MRU-3 |  |  |  |  |  |  |  |  | 8 |
| 184–191 (010111y2y1y0) | MRU of 996-996-996-[], including 3×996-tone MRU-4 |  |  |  |  |  |  |  |  | 8 |
| 192–199 (011000y2y1y0) | MRU of []-484-996-996-996, including 3×996+484 tone MRU-1 |  |  |  |  |  |  |  |  | 8 |
| 200–207 (011001y2y1y0) | MRU of 484-[]-996-996-996, including 3×996+484 tone MRU-2 |  |  |  |  |  |  |  |  | 8 |
| 208–215 (011010y2y1y0) | MRU of 996-[]-484-996-996, including 3×996+484 tone MRU-3 |  |  |  |  |  |  |  |  | 8 |
| 216–223 (011011y2y1y0) | MRU of 996-484-[]-996-996, including 3×996+484 tone MRU-4 |  |  |  |  |  |  |  |  | 8 |
| 224–231 (011100y2y1y0) | MRU of 996-996-[]-484-996, including 3×996+484 tone MRU-5 |  |  |  |  |  |  |  |  | 8 |
| 232–239 (011101y2y1y0) | MRU of 996-996-484-[]-996, including 3×996+484 tone MRU-6 |  |  |  |  |  |  |  |  | 8 |
| 240–247 (011110y2y1y0) | MRU of 996-996-996-[]-484, including 3×996+484 tone MRU-7 |  |  |  |  |  |  |  |  | 8 |
| 248–255 (011111y2y1y0) | MRU of 996-996-996-484-[], including 3×996+484 tone MRU-8 |  |  |  |  |  |  |  |  | 8 |
| 256–263 (100000y2y1y0) | MRU of []-484-996-996, including 2×996+484 tone MRU-1 and7 within the 240 MHz subblock composed of the 1st, 2nd, and 3rd 80 MHz subblock and the 240 MHz subblock composed of the 2nd , 3rd, and 4th 80MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 264–271 (100001y2y1y0) | MRU of 484-[]-996-996, including 2×996+484 tone MRU-2 and 8 within the 240 MHz subblock composed of the 1st, 2nd and 3rd 80 MHz subblock and the 240 MHz subblock composed of the 2nd , 3rd, and 4th 80 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 272–279 (100010y2y1y0) | MRU of 996-[]-484-996, including 2×996+484 tone MRU-3 and 9 within the 240 MHz subblock composed of the 1st, 2nd, and 3rd 80 MHz subblock and the 240 MHz subblock composed of the 2nd , 3rd, and 4th 80 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 280–287 (100011y2y1y0) | MRU of 996-484-[]-996, including 2×996+484 tone MRU-4 and10 within the 240 MHz subblock composed of the 1st, 2nd, and 3rd 80 MHz subblock and the 240 MHz subblock composed of the 2nd , 3rd, and 4th 80 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 288–295 (100100y2y1y0) | MRU of 996-996-[]-484, including 2×996+484 tone MRU-5 and 11 within the 240 MHz subblock composed of the 1st, 2nd, and 3rd 80 MHz subblock and the 240 MHz subblock composed of the 2nd , 3rd, and 4th 80 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 296–303 (100101y2y1y0) | MRU of 996-996-484-[], including 2×996+484 tone MRU-6 and12 within the 240 MHz subblock composed of the 1st, 2nd, and 3rd 80 MHz subblock and the 240 MHz subblock composed of the 2nd , 3rd, and 4th 80 MHz subblock in increasing order, respectively |  |  |  |  |  |  |  |  | 8 |
| 304–511 | Disregard |  |  |  |  |  |  |  |  | 208 |
| If signaling RUs or MRUs of size greater than or equal to 242 subcarriers, y2y1y0 = 000–111 indicates the number of User fields in the EHT-SIG content channel that contains the corresponding 9-bit RU Allocation subfield. The binary vector y2y1y0 indicates N r c = 22  y + 21  y + y + 1 users multiplexed in the RU. [] indicates one or multiple RU/MRUs that are not overlapped with the RU or MRU indicated by the 9-bit RU allocation subfield and is to help indicate the frequency order of the MRU within an 80/160/240/320 MHz subband. user 2 1 0 |  |  |  |  |  |  |  |  |  |  |