IEEE P802.11
Wireless LANs

| Segment Parser CR on P802.11be D0.3 – part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bo Gong | Huawei Technologies |  |  | gongbo8@hisilicon.com |
| Jianhan Liu | Mediatek |  |  | Jianhan.Liu@mediatek.com |
| Ross Jian Yu | Huawei Technologies |  |  | ross.yujian@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3.

10 Comments,

CIDs 1587, 2442, 2443, 2672, 2673, 2817, 2951, 2952, 2953 and 3072 are resolved in this submission.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 1587

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.1 | 36.3.12 | Frequency domain duplication for EHT-MCS14 block is defined in the transmitter block diagram. A new subsection which adresses this block is also needed in the Data field section. | Add a new subsection for frequency domain duplication into the Data field section and describe its detail. | Revised The details for frequency domain duplication for EHT-MCS 14 block and the related changes in the context are included in PDT 11/21-0139r3. Note to the editor: the proposed changes have been reflected in 11/21-0139r3 and no further changes are needed. |

## CID 2442

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.35 | 36.3.12.5 | correct a parameter in an expression | Correct to k=(Ncbpss,l,u-nl*44*Nbpscs,l,u),...,Ncbpss,l,u-1 (at the end of the expression there should be Ncbpss and not Nbpscs) | Revised Agreed in principle. Reflect the detailed explanation Instructions to the editor Please make the changes as shown in 11/21-0443r0 |

Instructions to the editor:

In Line35 P296, the maximum value of k is  instead of ,the expression

should be revised as

.

## CID 2672

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 295.59 | 36.3.12.5 (36-61) | sum range for getting offset of m with-in each (summation m_l) block should be sum of m for all previous segments | the second term in the equation should be summation m_i from i=0 to i=l-1 and not from i=0 to i=L-1, i.e. summation upto previous segment | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r0 Note to the editor: The resolutions for CID 2672 CID 2817 and CID 2951 in 11/21-0443r0 are the same |

## CID 2817

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 295.59 | 36.3.12.5 | sum over m_i in (36-61) should be from 0 to l-1, not from 0 to L-1. For example for m_i=1, i=0:, .., 3, we should have: y_k,l = x_4k+l. | Correct. Also correct in (36-62) | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r0 Note to the editor: The resolutions for CID 2672 CID 2817 and CID 2951 in 11/21-0443r0 are the same. |

## CID 2951

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 295.59 | 36.3.12.5 | In the equation \sum_{i=0}^{L-1}m_i should be \sum_{i=0}^{l-1}m_i | Correct the equation. | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r0 Note to the editor: The resolutions for CID 2672 CID 2817 and CID 2951 in 11/21-0443r0 are the same. |

Instructions to the editor:

In Line 59 Page 295, the second term of equation (36-61),, should be revised as , in which,  is revised as .

## CID 2443

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.30 | 36.3.12.5 | correct a parameter in an expression | The last sum (sum(mi)) should be until "l-1" and not "L-1" and also for "i!=lo" and not "i!=i0" | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r0 Note to the editor: The resolutions for CID 2443, CID 2673, CID 2953 in 11/21-0443r0 are the same. |

## CID 2673

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.30 | 36.3.12.5 (36-62) | sum range for getting offset of m with-in each (summation m_l) block should be sum of m for all previous segments | same comment as 36-61 but here it applies to third term | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r0 Note to the editor: The resolutions for CID 2443, CID 2673, CID 2953 in 11/21-0443r0 are the same. |

## CID 2953

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.30 | 36.3.12.5 | The last "L-1" should be "l-1" | Correct the equation. | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r0 Note to the editor: The resolutions for CID 2443, CID 2673, CID 2953 in 11/21-0443r0 are the same. |

Instructions to the editor:.

In Line 30 Page 296, the third term of the equation (36-62),, should be revised as , in which,  is revised as  and  is revised as .

- CID 2952

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.20 | 36.3.12.5 | "L-1" should be "l-1" | Correct the equation. | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r0 |

## CID 3072

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.20 | 36.3.12.5 | change "for subblock l=0 " to "for frequency subblock l=0" | as commented | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r0 |

Instructions to the editor:

In Line 20 Page 296, the expression “ for subblock  ” should be revised as

“ for frequency subblock ”, where  is revised as  and “subblock” is changed to “frequency subblock”.