IEEE P802.11
Wireless LANs

| CC36 CR on 36.6 Parameters for EHT-SIG-MCSs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Edward Au |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.01.

1 comment is resolved.

CID 8152

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 8152

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 578.11 | 36.6 | In the Table 36-87, Value of the EHT-SIG field is meant to be Value of the EHT-SIG MCS field. EHT-SIG MCS Index is meant to be EHT-MCS index. | as in comment | REVISED Agreed in principle. Reflect the detailed changes Instructions to the editor: Please make the changes as shown in 11/21-xxxxr0, under CID 8152. |

Instructions to the editor, please make the following changes to P600, L10 of P802.11be D1.01:

# Parameters for EHT-SIG-MCSs

The EHT-SIG-MCSs, defined in Table 36-87 (EHT-SIG MCSs), are used for the EHT-SIG field transmission in the EHT MU PPDU.

Table 36-87—EHT-SIG MCSs

| Value of the EHT-SIG MCS field | EHT- MCS index | Modulation | R | NBPSCS | NSD | NCBPS | NDBPS | EHT-SIG rate (Mb/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | EHT-MCS 0 | BPSK | 1/2 | 1 | 52 | 52 | 26 | 6.6 |
| 1 | EHT-MCS 1 | QPSK | 1/2 | 2 | 52 | 104 | 52 | 13.2 |
| 2 | EHT-MCS 3 | 16-QAM | 1/2 | 4 | 52 | 208 | 104 | 26.0 |
| 3 | EHT-MCS 15 | BPSK-DCM | 1/2 | 1 | 26 | 26 | 13 | 3.3 |
| NOTE—The parameters NSD , NCBPS , and NDBPS are used for the EHT-SIG field transmission in each 20 MHz subchannel. |  |  |  |  |  |  |  |  |