IEEE P802.11
Wireless LANs

| CC36 CR on 36.3.12.8.4 Common field for non-OFDMA transmission |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Stephen McCann |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.01.

9 comments are resolved.

CID 6440, 6441, 4671, 4857, 4858, 5422, 5480, 8119, 8120

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 6440

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 443.25 | 36.3.12.8 | Are the values of Spatial Resue and Number Of Non-OFDMA Users subfields different between 80 MHz frequency subblocks? We may clarify whether the values of Spatial Resue and Number Of Non-OFDMA Users subfields are same in every EHT-SIG content channel or not. | Clarify whether the values of Spatial Resue and Number Of Non-OFDMA Users subfields are same in every EHT-SIG content channel or not. | Revised P802.11be D1.0 already clarifies that those values shall be the same in every EHT-SIG content channels: “B0–B16 of Table 36-36 (Common field for non-OFDMA transmission to a single user and non-OFDMA transmission to multiple users) are U-SIG Overflow bits for non-OFDMA transmission to a single user and non-OFDMA transmission to multiple users. Both the U-SIG Overflow bits and Number Of Non-OFDMA Users subfields are duplicated in each content channel.” Note to the editor: no futher edits are needed. |

## CID 6441

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 445.01 | 36.3.12.8.3 | LDPC Extra Symbol Segment, Pre-FEC Padding Factor, and PE Disambiguity subfields don't exist in common field for EHT sounding NDP. | Delete the sub-bullets of LDPC Extra Symbol Segment, Pre-FEC Padding Factor, and PE Disambiguity subfields in this paragraph and add the sub-bullets of NSS and beamformed subfields | Revised Agreed with the commenter. This has been resolved by CID4670 in 11/21-1057r1. Note to the editor: no futher edits are needed. |

## CID 4671

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 444.16 | 36.3.12.8.4 | Run on sentence "Set to ..., see" | Make two sentences, or encapsulate the run-on in parentheses | Revised Reflect the detailed changes for Tables 36-33, 36-36 and 36-37. Instructions to the editor: Please make the changes as shown in 11/21-1149r0, under CID 4671. |

Instructions to the editor, please make the following changes to P466, L11 of P802.11be D1.01:

Table 36-37—Common field for EHT sounding NDP

| Bit | Subfield | Number of bits | Description |
| --- | --- | --- | --- |
| B0-B3 | Spatial Reuse | 4 | Indicates whether or not spatial reuse modes are allowed during the transmission of this PPDU. Set to 15 from Table 27-22 (Spatial Reuse field encoding for an HE SU PPDU, HE ER SU PPDU, and HE MU PPDU). See 26.11.6 (SPATIAL_REUSE), and 26.10 (Spatial reuse operation). |

Instructions to the editor, please make the following changes to P452, L15 of P802.11be D1.01:

Indicates whether or not spatial reuse modes are allowed during the transmission of this PPDU.

Set to a value from Table 27-22 (Spatial Reuse field encoding for an HE SU PPDU, HE ER PPDU, and HE MU PPDU). See 26.11.6 (SPATIAL_REUSE), and 26.10 (Spatial reuse operation).

Instructions to the editor, please make the following changes to P464, L15 of P802.11be D1.01:

Indicates whether or not spatial reuse modes are allowed during the transmission of this PPDU.

Set to a value from Table 27-22 (Spatial Reuse field encoding for an HE SU PPDU, HE ER PPDU, and HE MU PPDU). See 26.11.6 (SPATIAL_REUSE), and 26.10 (Spatial reuse operation).

## CID 4857

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 443.18 | 36.3.12.8.4 | B13-B16 are Disregards bits and are not U-SIG overflow bits. change B0-B16 with B0-B12 | As in comment | Rejected Disregard bits are also part of U-SIG Overflow bits |

## CID 4858

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 444.58 | 36.3.12.8.4 | B14-B16 are Disregards bits and are not U-SIG overflow bits. change B0-B16 with B0-B13 | As in comment | Rejected Disregard bits are also part of U-SIG Overflow bits |

## CID 5422

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 444.58 | 36.3.12.8.4 | Should specify clearly that other values of NSS are Validate for R1 devices. | Change to "Other values are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true." | Revised This has been resolved by CID5416 in 11/21-1057r1. Note to the editor: no futher edits are needed. |

## CID 5480

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 443.13 | 36.3.12.8.4 | Add description that for SU transmission, other values than 0 is Validate. For non-OFDMA MU-MIMO tranmsission, Value 0 is Validate. | as in comment | Revised This has been resolved by CID5416 in 11/21-1057r1. Note to the editor: no futher edits are needed. |

## CID 8119

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 443.19 | 36.3.12.8.4 | discrepancy between the first paragraph and the second paragraph at P443. U-SIG overflow bits include Spatial Reuse but in the second paragraph, it missed the Spatial Reuse subfield. Moreover the second paragraph itself is redundant. Better to delete it. | as in comment | Revised This has been resolved by CID 4670 in 11/21-1057r1. Note to the editor: no futher edits are needed. |

## CID 8120

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 444.58 | 36.3.12.8.4 | discrepancy between the paragraph atP444L58 and the paragraph at P445L1. U-SIG overflow bits include Spatial Reuse but in the second paragraph, it missed the Spatial Reuse subfield. Moreoverthe paragraph at P445L1 itself is redundant. Better to delete it. | as in comment | Revised This has been resolved by CID 4670 in 11/21-1057r1. Note to the editor: no futher edits are needed. |