IEEE P802.11
Wireless LANs

| CC36 CR on 36.3.12.8.3 Common field for OFDMA transmission – Part 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Edward Au |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.1.

4 comments are resolved.

CID 4672, 4673, 4675, 5419

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 5419

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 433.47 | 36.3.12.8.3 | Before this paragraph, there is no info about how to put the RU allocation subfields to which content channel, e.g., RU allocation subfields corresponding to odd/even-indexed 20 MHz subchannels in the odd/even-indexed content channel. It is weird to talk about how many RU allocation subfields per content channel for a particular size of RU/MRU here without that prior knowledge. | Suggest to have a brief description on the how to put the RU allocation subfields to which content channel and move Table 36-34 before this paragraph. | Rejected Table 36-35 (RUs associated with each RU Allocation subfield for each EHT-SIG content channel and PPDU bandwidth) already specifies the mapping of RU Allocation subfield to the subchannel/groups of tones. It is also shown in the figures in 36.3.12.8.6 (Encoding and modulation). |

## CID 4675

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 440.24 | 36.3.12.8.3 | Apologies but it is not clear to me how the same RU allocation can signal the lower 240MHz or upper 240 MHz of a PPDU. Is the idea is that this is interpreted in conjunction with the Punctured Channel Information field in USIG? | Please provide more explanation; perhaps by way of a note to avoid duplicating the explanation 6x. Try "NOTE - The occupied 240 MHz can be determined from a) XXXX(?), then b) the upper or lower 240 MHz within that 320 MHz from the Punctured Channel Information field in the USIG field." | Revised Propose to add a clarification in 36.3.2.2.3.2 (Large size multiple RUs for OFDMA) that when 2×996+484-tone MRUs exist in a 240 MHz frequency subblock, the other 80 MHz shall be punctured. Instructions to the editor: Please make the changes as shown in 11/21-1269r0, under CID 4675. |

Discussion

In 1935r38, the following motion exists.

The table below defines 12 options for MRU 996×2+484 in 320 MHz BSS.

- 240/160+80 MHz BW entry is TBD.

- NOTE – Shaded area in the table is punctured.

[Motion 122, #SP166, [12] and [13]]

There is a requirement that even in OFDMA transmission, when 2×996+484-tone MRUs exist in a 240 MHz frequency subblock, the other 80 MHz shall be punctured.

If we following this requirement, then the above restriction needs to be added to 36.3.2.2.3.2 (Large size multiple RUs for OFDMA). Whilst the RU Allocation signaling still works without the above restriction.

Regarding the NOTE suggetsted by the commenter, if RU Allocation subfields exist, then it is OFDMA transmission. In this case, the Punctured Channel Information field in the U-SIG field doesn’t tell the puncturing pattern of the whole PPDU, but only the puncturing pattern of the corresponding 80 MHz.

How to determine the lower or upper 240 MHz is implementation specific. For 2×996+484-tone MRU 1 to 6, the entry indicating 2×996+484-tone MRU exists in the first RU Allocation-1 subfield of content channel 1 or the first RU Allocation-1 subfield of content channel 2. For 2×996+484-tone MRU 7 to 12, the entry indicating 2×996+484-tone MRU exists in the second RU Allocation-1 subfield of content channel 1 or the second RU Allocation-1 subfield of content channel 2. If we add the above restriction, an alternative way is to identify the 80 MHz of which the corresponding four RU allocation subfields are with value 26 (Punctured 242-tone RU).

Discussion ends

Instructions to the editor, please make the following changes to P410, L2 of P802.11be D1.01:

(#1296)The 2×996+484-tone MRU is allowed in an OFDMA 320 MHz EHT PPDU. The 2×996+484-tone MRU is obtained by combining two 996-tone RUs and 484-tone RU. The data subcarriers of a 2×996+484-tone MRU consist of the data subcarriers of the two 996-tone RUs and 484-tone RU that make up the 2×996+484-tone MRU. The pilot subcarriers of a 2×996+484-tone MRU consist of the pilot subcarriers of the two 996-tone RUs and 484-tone RU that make up the 2×996+484 tone MRU. The twelve allowed 2×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU are shown in Figure 36-19 (Allowed 2×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU(#1296)). The fourth 80 MHz shall be punctured if any one of the 2×996+484-tone MRU 1 to the 2×996+484-tone MRU 6 exists. The first 80 MHz shall be punctured if any one of the 2×996+484-tone MRU 7 to the 2×996+484-tone MRU 12 exists.

## CID 4673

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 439.18 | 36.3.12.8.3 | Apologies but I cannot make sense of this description. I see MRU of []+242+484 and MRUs 1/5/9/13. I lok at Fig 36-17 which seems to cover MRUs of []+242+484 but only see MRU-1 to MRU-4. Ditto Table 36-13 oonly goes up to MRU-4. What are these MRU-13? I see indices this high in Table 36-15, but how do I know to use that table versus the other table? | Do we need entries for 80, 160 and 320 MHz PPDUs? Anmd for each PPDU bandwidth we should list the MRUs and the associated Table where the MRU is defined? This would probably be a lot clearer if MRUs had globally unique names - e.g. MRU_{320MHz}^{3x996+484+242}-1? | Rejected While it may be clearer to describe the MRU index per BW, it makes the description within one row too long. On the other hand, it is clear enough that MRU index and RU Allocation subfield are both a function of PPDU bandwidth: - Table 36-33 shows that RU allocation-1/2 subfield is a function of PPDU dandwidth. -Table 36-13 to table 36-15 are also a function of PPDU bandwidth. |

## CID 4672

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 440.25 | 36.3.12.8.3 | "in increasing order" is vague | Change to "in increasing frequency order", x6 for the 6 "240 MHz" rows in this table (P440L25 onwards), and also x8 for the 8 "80MHz" rows in this table (P439L19 onwards) | REVISED Reflect the detailed changes Instructions to the editor: Please make the changes as shown in 11/21-1269r0, under CID 4672. |

Instructions to the editor, please make the following changes to P486, L36 of P802.11be D1.1:

Table 36-35—RU Allocation subfield

| RU Allocation subfield (B8 B7 B6 B5 B4 B3 B2 B1 B0) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | Number of entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 (000011010) | Punctured 242-tone RU |  |  |  |  |  |  |  |  | 1 |
| 27 (000011011) | Unassigned 242-tone RU |  |  |  |  |  |  |  |  | 1 |
| 28 (000011100) | 242-tone RU; contributes zero User fields to the User Specific field in the same EHT-SIG content channel as this RU Allocation subfield and is not unallocated. |  |  |  |  |  |  |  |  | 1 |
| 29 (000011101) | 484-tone RU; contributes zero User fields to the User Specific field in the same EHT-SIG content channel as this RU Allocation subfield and is not unallocated. |  |  |  |  |  |  |  |  | 1 |
| 30 (000011110) | 996-tone RU; contributes zero User fields to the User Specific field in the same EHT-SIG content channel as this RU Allocation subfield and is not unallocated. |  |  |  |  |  |  |  |  | 1 |
| 31 (000011111) | Validate |  |  |  |  |  |  |  |  | 1 |
| 56–63 (000111000–000111111) | Validate |  |  |  |  |  |  |  |  | 8 |
| 64–71 (001000y2y1y0) | 242 |  |  |  |  |  |  |  |  | 8 |
| 72–79 (001001y2y1y0) | 484 |  |  |  |  |  |  |  |  | 8 |
| 80–87 (001010y2y1y0) | 996 |  |  |  |  |  |  |  |  | 8 |
| 88–95 (001011y2y1y0) | 2×996 |  |  |  |  |  |  |  |  | 8 |
| 96–103 (001100y2y1y0) | (#1400)MRU of []-242-484, including 484+242-tone MRU-1, 5, 9, and 13 within the first, second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 104–111 (001101y2y1y0) | (#1400)MRU of 242-[]-484, including 484+242-tone MRU-2, 6, 10, and 14 within the first, second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 112–119 (001110y2y1y0) | (#1400)MRU of 484-[]-242, including 484+242-tone MRU-3, 7, 11, and 15 within the first, second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 120–127 (001111y2y1y0) | (#1400)MRU of 484-242-[], including 484+242-tone MRU-4, 8, 12, and 16 within the first, second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 128–135 (010000y2y1y0) | (#1400)MRU of []-484-996, including 996+484-tone MRU-1 and 5 within the first and second 160 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 136–143 (010001y2y1y0) | (#1400)MRU of 484-[]-996, including 996+484-tone MRU-2 and 6 within the first and second 160 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 144–151 (010010y2y1y0) | (#1400)MRU of 996-[]-484, including 996+484-tone MRU-3 and 7 within the first and second 160 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 152–159 (010011y2y1y0) | (#1400)MRU of 996-484-[], including 996+484-tone MRU-4 and 8 within the first and second 160 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 160–167 (010100y2y1y0) | (#1400)MRU of []-996-996-996, including 3996-tone MRU-1 |  |  |  |  |  |  |  |  | 8 |
| 168–175 (010101y2y1y0) | (#1400)MRU of 996-[]-996-996, including 3996-tone MRU-2 |  |  |  |  |  |  |  |  | 8 |
| 176–183 (010110y2y1y0) | (#1400)MRU of 996-996-[]-996, including 3996-tone MRU-3 |  |  |  |  |  |  |  |  | 8 |
| 184–191 (010111y2y1y0) | (#1400)MRU of 996-996-996-[], including 3996-tone MRU-4 |  |  |  |  |  |  |  |  | 8 |
| 192–199 (011000y2y1y0) | (#1400)MRU of []-484-996-996-996, including 3996+484-tone MRU-1 |  |  |  |  |  |  |  |  | 8 |
| 200–207 (011001y2y1y0) | (#1400)MRU of 484-[]-996-996-996, including 3996+484-tone MRU-2 |  |  |  |  |  |  |  |  | 8 |
| 208–215 (011010y2y1y0) | (#1400)MRU of 996-[]-484-996-996, including 3996+484-tone MRU-3 |  |  |  |  |  |  |  |  | 8 |
| 216–223 (011011y2y1y0) | (#1400)MRU of 996-484-[]-996-996, including 3996+484-tone MRU-4 |  |  |  |  |  |  |  |  | 8 |
| 224–231 (011100y2y1y0) | (#1400)MRU of 996-996-[]-484-996, including 3996+484-tone MRU-5 |  |  |  |  |  |  |  |  | 8 |
| 232–239 (011101y2y1y0) | (#1400)MRU of 996-996-484-[]-996, including 3996+484-tone MRU-6 |  |  |  |  |  |  |  |  | 8 |
| 240–247 (011110y2y1y0) | (#1400)MRU of 996-996-996-[]-484, including 3996+484-tone MRU-7 |  |  |  |  |  |  |  |  | 8 |
| 248–255 (011111y2y1y0) | (#1400)MRU of 996-996-996-484-[], including 3996+484-tone MRU-8 |  |  |  |  |  |  |  |  | 8 |
| 256–263 (100000y2y1y0) | (#1400)MRU of []-484-996-996, including 2996+484-tone MRU-1 and 7 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 264–271 (100001y2y1y0) | (#1400)MRU of 484-[]-996-996, including 2996+484-tone MRU-2 and 8 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 272–279 (100010y2y1y0) | (#1400)MRU of 996-[]-484-996, including 2996+484-tone MRU-3 and 9 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 280–287 (100011y2y1y0) | (#1400)MRU of 996-484-[]-996, including 2996+484-tone MRU-4 and 10 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |
| 288–295 (100100y2y1y0) | (#1400)MRU of 996-996-[]-484, including 2996+484-tone MRU-5 and 11 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |

Table 36-35—RU Allocation subfield (continued)

| RU Allocation subfield (B8 B7 B6 B5 B4 B3 B2 B1 B0) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | Number of entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 296–303 (100101y2y1y0) | (#1400)MRU of 996-996-484-[], including 2996+484-tone MRU-6 and 12 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively |  |  |  |  |  |  |  |  | 8 |