IEEE P802.11
Wireless LANs

| CC36 CR on Annex Z EHT-SIG examples-320MHz example |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.09.23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | H3, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Wei Jiang |  |  |  |  |
| Mengshi Hu |  |  |  |  |
| Ming Gan |  |  |  |  |
| Stephen McCann |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.2.

CID 5496 (1 comment) is resolved.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 5496

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 620.01 | Annex Z | Add examples regarding 320MHz PPDU | as in comment | REVISED Reflect the details Instructions to the editor: Please make the changes as shown in 11/21-1578r0, under CID 5496 |

Instruction to the editor: please add the following at the end of Annex Z of P802.11be D1.2:

# Z.11  EHT-SIG Example 7

Examples of the EHT-SIG field with U-SIG Overflow and resource allocation, with signaling for a 320 MHz OFDMA transmission using EHT MU PPDU, are shown in Table Z-34 and Table Z-35 respectively.

Table Z-34 U-SIG Overflow example 7

| U-SIG Overflow | Subfield | Indication | Meaning |
| --- | --- | --- | --- |
|  | Spatial Reuse | 1111 | PSR_AND_NON_SRG_OBSS_PD_PROHIBITED |
|  | GI+LTF Size | 11 | 4x LTF + 3.2 μs GI |
|  | Number Of EHT-LTF Symbols | 010 | 4 EHT-LTF symbols |
|  | LDPC Extra Symbol Segment | 0 | An LDPC extra symbol segment is not present. |
|  | Pre-FEC Padding Factor | 01 | A pre-FEC padding factor of 1 |
|  | PE Disambiguity | 0 | The condition in Equation (36-80) is not met |
|  | Disregard | 1111 |  |

Table Z-35 Resource allocation signaling example 7

| RU/MRU | 2×996-tone RU 1 | 242-tone RU 9 | 242-tone RU 10 | 996+484-tone MRU 5 ([]-484-996) |
| --- | --- | --- | --- | --- |
| SS1 | STA-ID 1441, EHT-MCS10, LDPC | Unassigned | Unassigned | STA-ID 1443, EHT-MCS7, LDPC |
| SS2 | STA-ID 1442, EHT-MCS 4, LDPC |  |  | STA-ID 1444, EHT-MCS 8, LDPC |

In this example, the EHT-SIG content channels per 80 MHz frequency subband are set to the same values by the AP. The EHT-SIG content channels per 80 MHz frequency subband can also be different.

The illustration of an RU Allocation subfield for each of the four 80 MHz frequency subbands is given in Table Z-36 (Resource Allocation subfield illustration for each 80 MHz frequency subband example 7).

Table Z-36 RU allocation subfield illustration for each 80 MHz frequency subband example 7

|  | RU Allocation subfield illustration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Content channel 1 | 2×996 (2 User field) | 996 (0 User field) | 996 (0 User field) | 996 (0 User field) | Unassigned 242 | 484 (0 User field) | 996 (0 User field) | 996 (0 User field) |
| Content channel 2 | 996 (0 User field) | 996 (0 User field) | 996 (0 User field) | 996 (0 User field) | Unassigned 242 | []-484-996, MRU 5 (2 User fields) | 996 (0 User field) | 996 (0 User field) |

The AP can perform a dynamic split of the User fields for the two MU-MIMO STAs on 2×996-tone RU 1, with zero, one, or two User fields assigned to EHT-SIG content channel 1 and correspondingly two, one, or zero User fields assigned to EHT-SIG content channel 2. The AP can also perform a dynamic split of the User fields for the two MU-MIMO STAs on 996+484-tone MRU 5, with zero, one, or two User fields assigned to EHT-SIG content channel 1 and correspondingly two, one, or zero User fields assigned to EHT-SIG content channel 2. In this example, there are two User fields assigned to EHT-SIG content channel 1 and two User fields assigned to EHT-SIG content channel 2, to avoid a disparity in the number of User fields between content channels.

The User fields for STAs 1441 and 1442 are in content channel 1, while the User fields for STAs 1443 and 1444 are in content channel 2. The contents of EHT-SIG field in each 80 MHz frequency subband for this example is shown in Table Z-37 (EHT-SIG content in each 80 MHz frequency subband for example 7).

Table Z-37—EHT-SIG content in each 80 MHz frequency subband for example 7

|  | EHT-SIG content channel 1 |  | EHT-SIG content channel 2 |  |
| --- | --- | --- | --- | --- |
| Common field (U-SIG Overflow+2 RA-1 subfields+CRC+Tail+6 RA-2 subfields+CRC+Tail) | 1111 11 010 0 10 0 1111 100110100 011110000 1100 000000 011110000 011110000 110110000 101110000 011110000 011110000 0111 000000 |  | 1111 11 010 0 10 0 1111 011110000 011110000 0110 000000 011110000 011110000 110110000 100000010 011110000 011110000 1011 000000 |  |
| User Specific field | STA 1441 | 10000101101 0101 1 000000 | STA 1443 | 11000101101 1110 1 000000 |
|  | STA 1442 | 01000101101 0010 1 000000 | STA 1444 | 00100101101 0001 1 000000 |
|  | CRC and Tail | 1101 000000 | CRC and Tail | 0010 000000 |
|  | Padding | 00000 | Padding | 00000 |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11111101 00100111 11001101 00011110 00011000 00000011 11000001 11100001 10110000 10111000 00111100 00011110 00001110 00000100 00101101 01011000 00001000 10110100 10100000 01101000 00000000 |  | 11111101 00100111 10111100 00011110 00001100 00000011 11000001 11100001 10110000 10000001 00111100 00011110 00010110 00000110 00101101 11101000 00000100 10110100 01100000 00010000 00000000 |  |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 10111111 11100100 10110011 01111000 00011000 11000000 10000011 10000111 00001101 00011101 00111100 01111000 01110000 00100000 10110100 00011010 00010000 00101101 00000101 00010110 00000000 |  | 10111111 11100100 00111101 01111000 00110000 11000000 10000011 10000111 00001101 10000001 00111100 01111000 01101000 01100000 10110100 00010111 00100000 00101101 00000110 00001000 00000000 |  |
| EHT-SIG field content in hexadecimal, organized as octets | BF E4 B3 78 18 C0 83 87 0D 1D 3C 78 70 20 B4 1A 10 2D 05 16 00 |  | BF E4 3D 78 30 C0 83 87 0D 81 3C 78 68 60 B4 17 20 2D 06 08 00 |  |