IEEE P802.11
Wireless LANs

| Introduction to the EHT PHY CR on P802.11be D1.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.10.24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bo Gong | Huawei Technologies |  |  | gongbo8@huawei.com |
| Bin Tian | Qualcomm |  |  | btian@qti.qualcomm.com |
| Ross Jian Yu | Huawei Technologies |  |  | ross.yujian@huawei.com |

Abstract

This submission shows

- resolution for a comment received from TGbe comment collection (based on TGbe Draft D1.0)

- the proposed changes based on TGbe Draft D1.2.

The submission provides resolutions to CIDs 4521, 4523, 4940, 4942, 6462, 6464, 6907, 7095, 7958, 7962, 7964.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 4521

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 314.06 | 36.1.1 | Add a bullet on non-OFDMA DL MU MIMO transmission support for AP with <4 spatial streams support | as in the comment | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-1760r0 Note that the resolutions for CID 4521 and 7964 are the same. |

## CID 7964

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 314.05 | 36.1.1 | Why are we explicitly stating that UL MU-MIMO RX can be optionally supported if RX Nss<4, but not explicitly stating that DL MU-MIMO TX can be optionally supported if TX Nss<4? Either do not state both, or state both. | Delete the bullet starting at P314L5 ("MU-MIMO reception ...") | Revised A bullet on non-OFDMA DL MU MIMO transmission support for AP with <4 spatial streams support is added. Instructions to the editor Please make the changes as shown in 11/21-1760r0 Note that the resolutions for CID 4521 and 7964 are the same. |

Instructions to the editor:

Please add the following sentence in Line 60, Page 413 in TGbe Draft D1.2:

An EHT AP may support the following features:

—Transmission of a non-OFDMA EHT MU PPDU utilizing MU-MIMO (DL MU-MIMO) when the AP is not capable of transmitting equal to or more than 4 spatial streams.

—MU-MIMO transmission on an RU or MRU in an EHT MU PPDU where there are multiple RUs or MRUs(#3263) in the PPDU bandwidth (DL MU-MIMO within OFDMA).

—MU-MIMO reception on an RU or MRU in an EHT TB PPDU where the RU or MRU is of size larger than or equal to 242 tones in the supported bandwidth non-OFDMA transmission (UL MU-MIMO) when the AP is not capable of receiving equal to or more than 4 spatial streams.

—MU-MIMO reception on an RU or MRU in an EHT TB PPDU which consists of multiple RUs or MRUs(#3264) in the entire PPDU bandwidth (UL MU-MIMO within OFDMA).

## CID 4523

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 315.21 | 36.1.1 | Add clarification that 1x EHT-LTF and 1.6 μs GI duration is only used for UL MU-MIMO. | as in the comment | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-1760r0 |

Instructions to the editor:

Please make the following change in Line 6, Page 415 in TGbe Draft D1.2:

—Transmission of  an EHT TB PPDU utilizing non-OFDMA UL MU-MIMO with a 1х EHT-LTF and 1.6 μs GI duration on the EHT-LTF and Data field OFDM symbols.

- CID 4940

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 311.40 | 36.1.1 | What is the definition of a "MU-MIMO resource unit"? | as in comment | Rejected. The expression ‘a MU-MIMO resource unit’ means ‘a resource unit utilizing MU-MIMO’, which brings no confusion or misleading, and has been adopted in 11ax. |

- CID 4942

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 314.40 | 36.1.1 | "The non-AP EHT STA shall be able to receive its intended spatial streams in a DL MU-MIMO transmission with a total number of spatial streams across all users of at least four." So no improvement over 11ac or 11ax? Very disappointing | change to 8 | Rejected. We do not have a concensus on this change. |

- CID 6462

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 311.38 | 36.1.1 | Better to write "Both DL MU-MIMO and UL MU-MIMO transmissions..." | as in comment | Revised. Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-1760r0 |

Instructions to the editor:

Please make the following changes in Line 39 Page 411 of TGbe Draft D1.2.

The EHT PHY provides support for DL OFDMA, UL OFDMA, DL MU-MIMO, and UL MU-MIMO. Both DL MU-MIMO and UL MU-MIMO transmissions are supported on portions of the PPDU bandwidth (on resource units greater than or equal to 242 tones). (#1239)In a MU-MIMO resource unit, there is support for up to eight users with up to four spatial streams per user with the total across all users not exceeding eight spatial streams.

- CID 6464

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 311.40 | 36.1.1 | Should be "an MU-MIMO"instead of "a MU-MIMO". | Change from "a MU-MIMO" to "an MU-MIMO". It is recommended to perform a quick search across the spec and correct all similar wording | REVISED Agreed in principle. Reflect the detailed explanation. Instructions to the editor: Please make the changes as shown in 11/21-1760r0 Note that CID 6464, CID 6907, and CID 7095 have the same resolution. |

- CID 6907

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 311.40 | 36.1.1 | "a MU-MIMO resource unit" should be "an MU-MIMO resource unit". | See comment ditto: P324L43 P446L24 P447L19 P447L20 P449L1 P449L5 P549L51 | REVISED Agreed in principle. Reflect the detailed explanation. Instructions to the editor: Please make the changes as shown in 11/21-1760r0 Note that CID 6464, CID 6907, and CID 7095 have the same resolution. |

- CID 7095

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 311.40 | 36.1.1 | Typo. Change "In a MU-MIMO ..." to "In an MU-MIMO ..." | See comment | REVISED Agreed in principle. Reflect the detailed explanation. Instructions to the editor: Please make the changes as shown in 11/21-1760r0 Note that CID 6464, CID 6907, and CID 7095 have the same resolution. |

Instructions to the editor:

Please change ‘a MU-MIMO’ to ‘an MU-MIMO’ in TGbe Draft D1.2.

- In Line 6, Page 34,

Table 36-41—User field format for an  MU-MIMO allocation ..................................................................... 552

- In Line 40, Page 411,

The EHT PHY provides support for DL OFDMA, UL OFDMA, DL MU-MIMO, and UL MU-MIMO. Both DL and UL MU-MIMO transmissions are supported on portions of the PPDU bandwidth (on resource units greater than or equal to 242 tones). (#1239)In an MU-MIMO resource unit, there is support for up to eight users with up to four spatial streams per user with the total across all users not exceeding eight spatial streams.

- In Line 23, Page 424,

(#1531)Indicates the number of spatial streams.

Integer in the range:

1–4 for an  MU-MIMO RU.

1–8 for an RU assigned to no more than 1 user.

NUM_STS summed over all users per RU is not greater than 8.

- In Line 24, Page 548,

The User field format for a non-MU-MIMO allocation is defined in Table 36-40 (User field format for a non-MU-MIMO allocation). The User field format for an MU-MIMO allocation is defined in Table 36-41 (User field format for an  MU-MIMO allocation).

- In Line 21, Page 549,

The User field format for a non-MU-MIMO allocation is defined in Table 36-40 (User field format for a non-MU-MIMO allocation). The User field format for an  MU-MIMO allocation is defined in Table 36-41 (User field format for an  MU-MIMO allocation).

- In Line 1, Page 552,

The User field format for an MU-MIMO allocation is defined in Table 36-41 (User field format for an  MU-MIMO allocation).

- In Line 5, Page 552,

Table 36-41—User field format for an  MU-MIMO allocation

- In Line 1, Page 553,

Table 36-41—User field format for an  MU-MIMO allocation (continued)

## CID 7962

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 313.48 | 36.1.1 | EHT TB PPDU using 1xLTF + 1.6us GI for MU-MIMO - see P470L38. | Change "Reception of a non-OFDMA EHT TB PPDU with a 1x EHT-LTF" to "Reception of non-OFDMA EHT TB PPDUs from multiple users using a 1x EHT-LTF" | ACCEPTED |

- CID 7958

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 313.24 | 36.1.1 | "EHT MU PPDU consisting of a single RU or MRU of size larger than or equal to 242 tones in supported bandwidth non-OFDMA transmission" This is not readable | Change "Transmission of an EHT MU PPDU consisting of a single RU or MRU of size larger than or equal to 242 tones in supported bandwidth non-OFDMA transmission and utilizing MU-MIMO" to "Transmission of a non-OFDMA EHT MU PPDU utilizing MU-MIMO" | ACCEPTED |