IEEE P802.11
Wireless LANs

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Stephen McCann | Huawei Technologies Co., Ltd | Southampton, United Kingdom |  | stephen.mccann@ieee.org |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D2.0.

The following comment resolutions are proposed:

CIDs 10440, 10935, 11170, 11171, 11177, 11816, 11820, 12035, 12217 and 12706

# Revision Notes

| R0 | Initial revision |
| --- | --- |

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 10440 | 51.5 | 3.2 | Can a SP be allocated to a group of STAs? | Please clarify this. | Rejected: The Service Period is determined by a STA and is unique to that STA. A Service Period could possibly be allocated to a group of STAs, although this will require a submission to add normative text on such a feature. The current definition is correct and does not require any changes. |
| 10935 | 54.15 | 3.2 | EMLMR has in contrast to EMLSR no definition | Please add: Baseline could be a generalized EMLSR definition | Revised Agree in principle, although there is no current definition of EMLSR, just “EMLSR operation” that is not very useful. Create new definitions for EMLSR and EMLMR and remove “EMLSR operation”. Instructions to the editor: Please make the changes as shown in 11/22-1196r0, with the reference CID 10935. |
| 11170 | 51.6 | 3.2 | The changes to the definition of SP do not clarify anything and are not correct. An SP is a period of time established by the TXOP holder to transmit frames to a QoS STA and/or allow the QoS STA to transmit frames to the TXOP holder by granting the QoS STA a TXOP during the TXOP holder's TXOP. Which is what the original definition said. Adding the words "(portions of)" and "or allocated" do not provide clarity, just confusion. | Delete the addition to the definition of service period. | Revised. 35.2.1.2 introduces a TXOP sharing procedure, so that a sercice period may be a portion of a TXOP doe one STA. The term “or allocated” is not require. Instructions to the editor: Please make the changes as shown in 11/22-1196r0, with the reference CID 11170. |
| 11171 | 51.15 | 3.2 | As each affiliated STA has independent PS control, and there for listens or not to the WM independently, this mode applies to each of the affiliated STAs independently. Therefore the text should be addressing an affiliated STA not STAs. | Change: "... whereby a non-AP STA or STAs affiliated with a non-AP MLD need not listen for every ..." To: "... whereby a non-AP STA or an affiliated STA of a non-AP MLD need not listen for every ..." | Accept |
| 11177 |  | 3.2 | There is no definition of an NSTR mobile AP, though the term is used in many location in the draft. Please define. | Add a definition for NSTR mobile AP. | Revised: Agree in principle. Instructions to the editor: Please make the changes as shown in 11/22-1196r0, with the reference CID 11177. |
| 11816 | 53.1 | 3.2 | The pair of links is not withing the MLD, but rather STAs affiliated with the MLD operate on the links of this pair. Please fix. | As in comment. | Revised: Agree in principle. The definition of NSTR link pair can be simplified. Instructions to the editor: Please make the changes as shown in 11/22-1196r0, with the reference CID 11816. |
| 11820 | 54.8 | 3.2 | Good to add MLMR, MLSR. eMLMR as well? | As in comment. | Revised: MLMR and MLSR are not actually used in the draft, although it is useful to create a new definition of EMLMR. Instructions to the editor: Please make the changes as shown in 11/22-1196r0, with the reference CID 11820. |
| 12035 | 54 | 3.2 | A definition of EMLMR is missing, while EMLSR is defined. It is still unclear to me what is the difference between EMLSR and EMLMR devices (number of spatial streams?) because reading the definition of EMLSR as it is I could think it applies the same to EMLMR ... a definition would help understanding the differences. | As in comment | Revised Agree in principle, although there is no current definition of EMLSR, just “EMLSR operation” that is not very useful. Create new definitions for EMLSR and EMLMR and remove “EMLSR operation”. Instructions to the editor: Please make the changes as shown in 11/22-1196r0, with the reference CID 12035. |
| 12217 | 54.1 | 3.2 | Why are simultaneous links defined as pairs? There's no reason to have pairs of these links. You could have any number of simultaneous links in an MLD. I think this definition is spurious and it's not simple the opposite of an NSTR | Change the definition to "simultaneous transmit and receive (STR) link: A link that is not part of a nonsimultaneous transmit and receive (NSTR) link pair." | Accept |
| 12706 |  | 3.2 | A definition for EMLMR operation is missing (compared to EMLSR existing definition). Please add a corresponding definition. | As in comment | Revised Agree in principle, although there is no current definition of EMLSR, just “EMLSR operation” that is not very useful. Create new definitions for EMLSR and EMLMR and remove “EMLSR operation”. Instructions to the editor: Please make the changes as shown in 11/22-1196r0, with the reference CID 12706. |

Discussion:

There is no definition for either EMLMR or EMLSR, but only “EMLSR operation”. This definition in clause 3.2 is not very useful and appears to be a cut and paste of text from 35.3.17 page 461 line 58.

New definitions of EMLSR and EMLMR are proposed.

3.2 Definitions specific to IEEE 802.11

TGbe editor: Insert the following paragraph at the appropriate alphabetical position in the clause

enhanced multi-link multiple radio (EMLMR): A non-AP MLD that receives and transmits on more than one link at a time. (#10935, #11820, #12035, #12706)

TGbe editor: Change the following paragraph at the appropriate alphabetical position in the clause

enhanced multi-link single radio (EMLSR): A non-AP MLD that receives on more than one link but transmits frames only on one link at a time. (#10935, #12035, #12706)

Discussion:

The current definition of NSTR link pair is not very useful and is defined in the negative. This also allows a simple definition of NSTR mobile AP to be created.

New definitions of NSTR link pair and NSTR mobile AP are proposed.

3.2 Definitions specific to IEEE 802.11

TGbe editor: Change the following paragraph at the appropriate alphabetical position in the clause

nonsimultaneous transmit and receive (NSTR) link pair: A pair of links from affiliated STAs within a multi-link device (MLD) that support transmission on link 1 concurrent with reception on link 2, but cannot support transmission on link 2 concurrent with reception on link 1.

(#11816)

TGbe editor: Insert the following paragraph at the appropriate alphabetical position in the clause

nonsimultaneous transmit and receive (NSTR) mobile Access Point (AP): An AP multi-link device (MLD) with one NSTR link pair, where one link is designated as a primary link. (#11177)

Discussion:

An updated definition of service period is proposed.

3.2 Definitions specific to IEEE 802.11

TGbe editor: Change the following paragraph at the appropriate alphabetical position in the clause

service period (SP): A period of time during which one or more downlink individually addressed frames are transmitted to a quality-of-service (QoS) station (STA) and/or one or more (portions of) transmission opportunities (TXOPs) are granted to the same STA. SPs are either scheduled or unscheduled. (#11170)