IEEE P802.11
Wireless LANs

| LB266 CR in 36.3.12.8.3 – part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.07.14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | H3 building, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Edward Au |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D2.0. The changes are based on P802.11be D2.1.1.

3 comments are resolved.

11286, 11362, 13581

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 11286, 11362

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 11286 | 667.27 | 36.3.12.8.3 | "MRU of []-242-484" is awkward notation/description. | Change to e.g. "242+484 tone MRU with pattern []-242-484". Similarly in other places. | Revised Reflect the detail changes. Instructions to the editor: Please make the changes as shown in 11/22-1349r0, under CID 11362 |
| 11362 | 668.63 | 36.3.12.8.3 | Notaton "[]" and its definition need to be updated to be more clear. Can use the same notation as in the MRU definition table in 36.3.2.2.3 | as in the comment | Revised Reflect the detail changes. Instructions to the editor: Please make the changes as shown in 11/22-1349r0, under CID 11362 |

Discussion

Example in 36.3.2.2.3

Discussion end

Instructions to the editor, please make the following changes to P683, L27 of P802.11be D2.1.1:

| 96–103 (001100y2y1y0) | MRU of [gap 242]-242-484, specifically 484+242-tone MRU-1, 5, 9, and 13 within the first, second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| --- | --- | --- |
| 104–111 (001101y2y1y0) | MRU of 242-[gap 242]-484, specifically 484+242-tone MRU-2, 6, 10, and 14 within the first, second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| 112–119 (001110y2y1y0) | MRU of 484-[gap 242]-242, specifically 484+242-tone MRU-3, 7, 11, and 15 within the first, second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| 120–127 (001111y2y1y0) | MRU of 484-242-[gap 242], specifically 484+242-tone MRU-4, 8, 12, and 16 within the first, second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| 128–135 (010000y2y1y0) | MRU of [gap 484]-484-996, specifically 996+484-tone MRU-1 and 5 within the first and second 160 MHz subblock in increasing frequency order, respectively | 8 |
| 136–143 (010001y2y1y0) | MRU of 484-[gap 484]-996, specifically 996+484-tone MRU-2 and 6 within the first and second 160 MHz subblock in increasing frequency order, respectively | 8 |
| 144–151 (010010y2y1y0) | MRU of 996-[gap 484]-484, specifically 996+484-tone MRU-3 and 7 within the first and second 160 MHz subblock in increasing frequency order, respectively | 8 |
| 152–159 (010011y2y1y0) | MRU of 996-484-[gap 484], specifically 996+484-tone MRU-4 and 8 within the first and second 160 MHz subblock in increasing frequency order, respectively | 8 |
| 160–167 (010100y2y1y0) | MRU of [gap 996]-996-996-996, specifically 3996-tone MRU-1 | 8 |
| 168–175 (010101y2y1y0) | MRU of 996-[gap 996]-996-996, specifically 3996-tone MRU-2 | 8 |
| 176–183 (010110y2y1y0) | MRU of 996-996-[gap 996]-996, specifically 3996-tone MRU-3 | 8 |
| 184–191 (010111y2y1y0) | MRU of 996-996-996-[gap 996], specifically 3996-tone MRU-4 | 8 |
| 192–199 (011000y2y1y0) | MRU of [gap 484]-484-996-996-996, specifically 3996+484-tone MRU-1 | 8 |
| 200–207 (011001y2y1y0) | MRU of 484-[gap 484]-996-996-996, specifically 3996+484-tone MRU-2 | 8 |
| 208–215 (011010y2y1y0) | MRU of 996-[gap 484]-484-996-996, specifically 3996+484-tone MRU-3 | 8 |
| 216–223 (011011y2y1y0) | MRU of 996-484-[gap 484]-996-996, specifically 3996+484-tone MRU-4 | 8 |
| 224–231 (011100y2y1y0) | MRU of 996-996-[gap 484]-484-996, specifically 3996+484-tone MRU-5 | 8 |
| 232–239 (011101y2y1y0) | MRU of 996-996-484-[gap 484]-996, specifically 3996+484-tone MRU-6 | 8 |
| 240–247 (011110y2y1y0) | MRU of 996-996-996-[gap 484]-484, specifically 3996+484-tone MRU-7 | 8 |
| 248–255 (011111y2y1y0) | MRU of 996-996-996-484-[gap 484], specifically 3996+484-tone MRU-8 | 8 |
| 256–263 (100000y2y1y0) | MRU of [gap 484]-484-996-996, specifically 2996+484-tone MRU-1 and 7 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| 264–271 (100001y2y1y0) | MRU of 484-[gap 484]-996-996, specifically 2996+484-tone MRU-2 and 8 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| 272–279 (100010y2y1y0) | MRU of 996-[gap 484]-484-996, specifically 2996+484-tone MRU-3 and 9 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| 280–287 (100011y2y1y0) | MRU of 996-484-[gap 484]-996, specifically 2996+484-tone MRU-4 and 10 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| 288–295 (100100y2y1y0) | MRU of 996-996-[gap 484]-484, specifically 2996+484-tone MRU-5 and 11 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| 296–303 (100101y2y1y0) | MRU of 996-996-484-[gap 484], specifically 2996+484-tone MRU-6 and 12 within the 240 MHz subblock composed of the first, second, and third 80 MHz subblock and the 240 MHz subblock composed of the second, third, and fourth 80 MHz subblock in increasing frequency order, respectively | 8 |
| 304–511 (100110y2y1y0– 111111y2y1y0) | Disregard | 268 |
| For an RU Allocation subfield with value greater than or equal to 64, y2y1y0 = 000–111 indicates the number of User fields in the EHT-SIG content channel that contains the corresponding 9-bit RU Allocation subfield. The binary vector y2y1y0 indicates N r c = 22  y + 21  y + y + 1 User fields in the EHT-SIG content channel that user 2 1 0 contains the corresponding 9-bit RU Allocation subfield. [Gap 242/484/996] indicates a 242/484/996-tone RU that is not overlapped with the RU or MRU indicated by the 9-bit RU Allocation subfield and is to help indicate the frequency order of the MRU within an 80/160/240/320 MHz subband. |  |  |

## CID 13581

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 661.01 | 36.3.12.8.3 | Define 1 bit of Disregard bit in EHT-SIG as vendor specific field | Will bring a detailed proposal. | Revised Reflect the detail changes Instructions to the editor: Please make the changes as shown in 11/22-1349r0, under CID 13581 |

Discussion:

- Vendors may make use of some reserved fields/entries in preamble for proprietary/vendor specific designs.

  - For example, implementation of 1K QAM for 11ac; implementation of 4KQAM for 11ax.

  - This will make reserved entries difficult for further use because of the proprietary/vendor specific designs.

- In 11be, the spec defines Validate and Disregard. There are:

  - 5 Disregard bits (B20-B24) and 1 Validate bit (B25) for U-SIG-1

  - 1 Validate bit (B2) for potential modes extension, 1 Validate bit (B8) for potential puncture mode extension in U-SIG-2

  - 4 Disregard bits (B13-B16) in U-SIG overflow of EHT-SIG common for OFDMA/non-OFDMA transmission to single user/multiple users, 2 Disregard bits (B14-B15) in U-SIG overflow of EHT-SIG common for EHT sounding NDP.

  - 1 Reserved bit for User field format for a non-MU-MIMO allocation

- If different vendor takes use of different reserved values, it may make the reserved values difficult for future use or make some interoperability issues.

- Propose to choose one Disregard bit (B15 of EHT-SIG common field) as vendor specific subfield.

Discussion end

Instructions to the editor, please make the following changes to P683, L27 of P802.11be D2.1.1:

Table 36-33—Common field for OFDMA transmission (continued)

| Bit | Subfield | Number of subfields | Number of bits per subfield | Description |
| --- | --- | --- | --- | --- |
| B13–B14 | Disregard | 1 | 2 | Set to all 1s. Disregard if dot11EHTBaseLineFeaturesImplementedOn ly equals true. |
| B15 | Vendor specific | 1 | 1 | Vendor specific |
| B16 | Disregard | 1 | 1 | Set to 1. Disregard if dot11EHTBaseLineFeaturesImplementedOn ly equals true. |

Table 36-36—Common field for non-OFDMA transmission to a single user and non-OFDMA transmission to multiple users (continued)

| Bit | Subfield | Number of bits | Description |
| --- | --- | --- | --- |
| B12 | PE Disambiguity | 1 | Indicates PE disambiguity as defined in 36.3.14 (Packet extension). |
| B13–B14 | Disregard | 2 | Set to all 1s. Disregard if dot11EHTBaseLineFeaturesImplementedOnly equals true. |
| B15 | Vendor specific | 1 | Vendor specific |
| B16 | Disregard | 1 | Set to 1. Disregard if dot11EHTBaseLineFeaturesImplementedOn ly equals true. |

Table 36-37—Common field for EHT sounding NDP (continued)

| Bit | Subfield | Number of bits per subfield | Description |
| --- | --- | --- | --- |
| B14 | Disregard | 1 | Set to 1. Disregard if dot11EHTBaseLineFeaturesImplementedOn ly equals true. |
| B15 | Vendor specific | 1 | Vendor specific |