IEEE P802.11
Wireless LANs

| LB266 CR on Annex Z-part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.08.16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | H3 building, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Mengshi Hu |  |  |  |  |
| Oded Redlich |  |  |  |  |
| Shimi Shilo |  |  |  |  |
| Edward Au |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D2.0. The changes are based on P802.11be D2.1.1.

3 comments are resolved.

CID 10764, 10765, 11971

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 10764

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 868.05 | Annex Z | []-242-484 MRU 1 appears in the lower 80 MHz and 484-242-[] MRU 8 appears in the upper 80 MHz, even though the RU Allocation value is the same for either 80 MHz frequency subblock. Having these two MRUs in the same table is confusing. | Add a note to point out that the i_80FS issue (defined in P655L15) may affect the MRU index (with the same RU Allocation value specified in the content channels) below the table or list both lower and upper 80 MHz frequency subband separately. When the latter is adopted, need to care about the correct MRU index in each i_80FS. Only the index from the corresponding i_80FS is used. | Revised 484-242-[] only aims to show the MRU type. The exact location is clear with the MRU index. Whilst it is worthwile to clarify []. Instructions to the editor: Please make the changes as shown in 11/22-1360r0, under CID 10764 |

Instructions to the editor, please make the following changes to P868, L37 of P802.11be D2.1.1:

Table Z-9—Resource allocation signaling example 1(#10762)

| RU or MRU | 484+242-tone MRU 2 (242-[gap 242]-484) | 242-tone RU 2 |
| --- | --- | --- |
| SS0 | STA-ID 1441, EHT-MCS 10, LDPC, no Tx beamforming | STA-ID 1442, EHT-MCS 4, BCC, 2SS, Tx beamforming |
| SS1 | N/A |  |

Table Z-10—Resource Allocation subfield illustration example 1

|  | RU Allocation subfield illustration |  |
| --- | --- | --- |
| Content channel 1 | 242-[gap 242]-484, MRU 2 (1 User field) | 484 (0 User field) |
| Content channel 2 | 242 (1 User field) | 484 (0 User field) |

Table Z-13—Resource allocation signaling example 2(#10762)

| RU or MRU | 484+242-tone MRU 2 (242-[gap 242]-484) | 242-tone RU 2 |
| --- | --- | --- |
| SS0 | STA-ID 1441, EHT-MCS 10, LDPC, 2SS | STA-ID 1442, EHT-MCS 4, BCC, 2SS, Tx beamforming |
| SS1 |  |  |
| SS2 | STA-ID 1443, EHT-MCS 7, LDPC | N/A |
| SS3 | STA-ID 1445, EHT-MCS 5, LDPC |  |

Table Z-14—Resource Allocation subfield illustration example 2

|  | RU Allocation subfield illustration |  |
| --- | --- | --- |
| Content channel 1 | 242-[gap 242]-484, MRU 2 (2 User fields) | 484 (0 User field) |
| Content channel 2 | 242 (1 User field) | 242-[gap 242]-484, MRU 2 (1 User field) |

Table Z-17—Resource allocation signaling example 3

| RU or MRU | 996+484-tone MRU 2 (484-[gap 484]-996) | 484-tone RU 2 |
| --- | --- | --- |
| SS0 | STA-ID 1441, EHT-MCS 10, LDPC, no Tx beamforming | STA-ID 1442, EHT-MCS 4, LDPC, 2SS, Tx beam- forming |
| SS1 | N/A |  |

Table Z-18—Resource Allocation subfield illustration for the lower 80 MHz example 3

|  | RU Allocation subfield illustration |  |  |  |
| --- | --- | --- | --- | --- |
| Content channel 1 | 484-[gap 484]-996, MRU 2 (1 User field) | 484 (0 User field) | 996 (0 User field) | 996 (0 User field) |
| Content channel 2 | 484 (0 User field) | 484 (1 User field) | 996 (0 User field) | 996 (0 User field) |

Table Z-23—Resource allocation signaling example 4

| RU or MRU | 242-tone RU 1 | 484+242-tone MRU 1 ([gap 242]-242-484) | 484+242-tone MRU 7 (484-[gap 242]-242) | 242-tone RU 7 |
| --- | --- | --- | --- | --- |
| SS0 | STA-ID 1441, EHT-MCS 8, BCC | STA-ID 1445, EHT-MCS 10, LDPC, 2SS | STA-ID 1447, EHT-MCS 4, LDPC, 2SS, Tx beamforming | STA-ID 1448, EHT-MCS 7, LDPC |
| SS1 | STA-ID 1442, EHT-MCS 7, BCC |  |  | STA-ID 1449, EHT-MCS 6, LDPC |
| SS2 | STA-ID 1443, EHT-MCS 6, BCC | STA-ID 1446, EHT-MCS 4, LDPC, 2SS | N/A | STA-ID 1450, EHT-MCS 5, LDPC |
| SS3 | STA-ID 1444, EHT-MCS 5, BCC |  |  | STA-ID 1451, EHT-MCS 5, LDPC |

Table Z-24—Resource Allocation subfield illustration for the lower 80 MHz example 4

|  | RU Allocation subfield illustration |  |  |  |
| --- | --- | --- | --- | --- |
| Content channel 1 | 242 (4 User fields) | 484 (0 User field) | 484 (0 User field) | 242 (0 User field) |
| Content channel 2 | [gap 242]-242-484, MRU 1 (2 User fields) | 484 (0 User field) | 484 (0 User field) | 242 (0 User field) |

Table Z-25—Resource Allocation subfield illustration for the upper 80 MHz example 4

|  | RU Allocation subfield illustration |  |  |  |
| --- | --- | --- | --- | --- |
| Content channel 1 | 242 (0 User field) | 484 (0 User field) | 484 (0 User field) | 242 (4 User fields) |
| Content channel 2 | 242 (0 User field) | 484 (0 User field) | 484-[gap 242]-242, MRU 7 (1 User field) | 242 (0 User field) |

Table Z-29—Resource allocation signaling example 5(#10762)

| RU or MRU | 484+242-tone MRU 2 (242-[gap 242]-484) |
| --- | --- |
| SS0 | STA-ID 1441, EHT-MCS 10, LDPC, 2SS |
| SS1 |  |
| SS2 | STA-ID 1443, EHT-MCS 7, LDPC |
| SS3 | STA-ID 1445, EHT-MCS 5, LDPC |

Table Z-32—Resource allocation signaling example 6(#10762)

| RU or MRU | 484+242-tone MRU 2 (242-[gap 242]-484) |
| --- | --- |
| SS0 | STA-ID 1441, EHT-MCS 10, LDPC, 2SS, Tx beamforming |
| SS1 |  |

Table Z-35—Resource allocation signaling example 7(#10762)

| RU or MRU | 2996-tone RU 1 | 242-tone RU 9 | 242-tone RU 10 | 996+484-tone MRU 5 ([gap 484]-484-996) |
| --- | --- | --- | --- | --- |
| SS0 | STA-ID 1441, EHT-MCS 10, LDPC | Unassigned | Unassigned | STA-ID 1443, EHT-MCS 7, LDPC |
| SS1 | STA-ID 1442, EHT-MCS 4, LDPC |  |  | STA-ID 1444, EHT-MCS 8, LDPC |

Table Z-36—RU Allocation subfield illustration for each 80 MHz frequency subblock example 7(#11698)

| RU Allocation subfield illustration |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Content channel 1 | 2996 (2 User fields) | 996 (0 User field) | 996 (0 User field) | 996 (0 User field) | Unas- signed 242 | 484 (0 User field) | 996 (0 User field) | 996 (0 User field) |
| Content channel 2 | 996 (0 User field) | 996 (0 User field) | 996 (0 User field) | 996 (0 User field) | Unas- signed 242 | [gap 484]-484- 996 MRU 5 (2 User fields) | 996 (0 User field) | 996 (0 User field) |

Table Z-39—Resource allocation signaling example 8

| RU or MRU | 242-tone RU 1 | 484+242-tone MRU 1 ([gap 242]-242-484) | 484+242-tone MRU 8 (484-242-[gap 242]) | 106-tone RU 15 | 106+26-tone MRU 16 |
| --- | --- | --- | --- | --- | --- |
| SS1 | Punctured | STA-ID 1441, EHT-MCS 10, LDPC, 2SS | STA-ID 1443, EHT-MCS 8, LDPC, 2SS, Tx beamforming | STA-ID 1444, EHT-MCS 4, BCC, Tx beamforming | STA-ID 1445, EHT-MCS 7, BCC, Tx beamforming |
| SS2 |  |  |  | N/A |  |
| SS3 |  | STA-ID 1442, EHT-MCS 4, LDPC, 2SS | N/A |  |  |
| SS4 |  |  |  |  |  |

Table Z-40—RU Allocation subfield illustration for each 80 MHz frequency subblock example 8(#11698)

| RU Allocation subfield illustration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Content channel 1 | Punctured 242 | 484 (0 User field) | 484-242-[gap 242], MRU 8 (1 User field) | 242 (0 User field) |
| Content channel 2 | [gap 242]-242-484, MRU 1 (2 User fields) | 484 (0 User field) | 484 (0 User field) | 106 + (106+26) RU 15 + MRU 16 (2 User fields) |

## CID 10765

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 868.22 | Annex Z | "In this example, STAs 1441-1445 are operating on the primary 80 MHz channel, which is the lower 80 MHz. The User field for STA 1443 is in content channel ..... No User fields exist in the upper 80 MHz." is not what Table Z-40 shows. | Please clarify and correct it if agreed. | Revised Agree with the commenter. Reflect the changes. Instructions to the editor: Please make the changes as shown in 11/22-1360r0, under CID 10765 |

Discussion

The transmission considered in Example 8 is 160 MHz OFDMA. It first says: “In this example, the EHT-SIG content channels per 80 MHz frequency subblock are set to the same values by the AP.” followed by “The User field for STA 1443 is in content channel 1 while the User fields for STAs 1441, 1442, 1444, and 1445 are in content channel 2 in the lower 80 MHz. No User fields exist in the upper 80 MHz.”

Since the EHT-SIG content channels per 80 MHz frequency subblock are duplicated, the User fields in the upper 80 MHz shall be the same as the lower 80 MHz.

Discussion ends.

Instructions to the editor, please make the following changes to P868, L37 of P802.11be D2.1.1:

In this example, STAs 1441–1445 are operating on the primary 80 MHz channel, which is the lower 80 MHz. The User field for STA 1443 is in content channel 1 while the User fields for STAs 1441, 1442, 1444, and 1445 are in content channel 2 in both the lower and upper 80 MHz frequency subblocks. The contents of the entire EHT-SIG field in each 80 MHz for this example is shown in Table Z-41 (EHT-SIG content in each 80 MHz frequency subblock for example 8(#11698)).

## CID 11971

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 852.20 | Z.6 | It is impossible to have LDPC extra symbol=1 with the configuration given in the example | In Table Z-8 in the 2nd coloum replace '1' with '0' and in the 3rd column replace "An LDPC extra symbol segment is present." with "An LDPC extra symbol segment is not present.". In addition, modify the content of Table Z-11 accordingly | Revised Agree with the commenter, reflect the changes. Instructions to the editor: Please make the changes as shown in 11/22-1360r0, under CID 11971 |

Instructions to the editor, please make the following changes to P868, L23 of P802.11be D2.1.1:

Table Z-8—U-SIG overflow example 1

| Subfield | Indication | Meaning |
| --- | --- | --- |
| Spatial Reuse | 1111 | PSR_AND_NON_SRG_OBSS_PD_PROHIBITED. |
| GI+LTF Size | 11 | 4 EHT-LTF and 3.2 µs GI. |
| Number Of EHT-LTF Symbols | 001 | 2 EHT-LTF symbols. |
| LDPC Extra Symbol Segment | 0 | An LDPC extra symbol segment is not present. |
| Pre-FEC Padding Factor | 01 | A pre-FEC padding factor of 1. |
| PE Disambiguity | 0 | The condition in Equation (36-94) is not met. |
| Disregard | 1111 |  |

Table Z-11—EHT-SIG content for example 1

|  | EHT-SIG content channel 1 |  | EHT-SIG content channel 2 |  |
| --- | --- | --- | --- | --- |
| Common field (U-SIG Overflow, RU Alloca- tion-1 subfield, CRC, Tail) | 1111 11 100 0 10 0 1111 000101100 101110000 1010 000000 |  | 1111 11 100 0 10 0 1111 000000100 101110000 1010 000000 |  |
| User Specific field | STA 1441 | 10000101101 0101 1 0000 0 1 | STA 1442 | 01000101101 0010 1 1000 1 0 |
|  | CRC and Tail | 1010 000000 | CRC and Tail | 0001 000000 |
|  | Padding | 000 | Padding | 000 |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11111110 00100111 10001011 00101110 01010000 00000100 00101101 01011000 0010 00000000 |  | 11111110 00100111 10000001 00101110 01010000 00000010 00101101 00101100 0100 00000000 |  |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 01111111 11100100 11010001 01110100 00001010 00100000 10110100 00011010 00101100 00000000 |  | 01111111 11100100 10000001 01110100 00001010 01000000 10110100 00110100 01000010 00000000 |  |
| EHT-SIG field content in hexa- decimal, organized as octets | 7F E4 D1 74 0A 20 B4 1A 2C 00 |  | 7F E4 81 74 0A 40 B4 34 42 00 |  |