IEEE P802.11
Wireless LANs

| LB266 CR for CID 13582 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.10.12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | H3 building, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D2.0.

1 comment is resolved.

CID 13582

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 13577

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 455.29 | 35.3.16.5 | Add some exaples on how to ensure PPDU alignment, e.g., using padding in EHT-SIG field, additional EHT-LTF, PE etc. | Will bring a detailed proposal. Suggest to discuss this in joint session. | Revised Reflect the detail changes. Instructions to the editor: Please make the changes as shown in 11/22-1757r0, under CID 13577. |

Discussion

“When an AP MLD is required to align the end time of simultaneously transmitted PPDUs, it shall satisfy the following conditions:

—The AP MLD shall ensure that the difference between the end times of simultaneously transmitted PPDUs is less than or equal to 8 μs (see NOTE 2), where the end time of the PPDU is the time of the end of the last OFDM symbol or the time of the end of the packet extension if present, whichever is later.

—The AP MLD shall ensure that the end time of one or more PPDUs that (#13808)carry a frame soliciting an immediate response frame is at most 4 μs (see NOTE 3) earlier than the end time of (#11136)any of the PPDUs containing a Trigger frame with the CS Required subfield set to 1.”

The current examples regarding PPDU alignment include:

“An AP MLD may use any type of padding to align the end time of transmitted PPDUs, such as using the Padding field in a Trigger frame, post-EOF A-MPDU padding, aggregating other MPDUs in the A-MPDU, or a packet extension.”

Additional, padding in HE-SIG-B or EHT-SIG symbol (4us per symbol), additional HE/EHT-LTF symbol (for 1x or 2x symbol) can also be used to fulfil PPDU alignment.

Discussion end

Instructions to the editor, please make the following changes to P470, L52 of P802.11be D2.1.1:

An AP MLD may use any type of padding to align the end time of transmitted PPDUs, such as using the Padding field in a Trigger frame, post-EOF A-MPDU padding, padding in HE-SIG-B or EHT-SIG symbol, HE/EHT-LTF symbol, aggregating other MPDUs in the A-MPDU, or a packet extension.