IEEE P802.11
Wireless LANs

| LB271 CR for 36.3.10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023.06.20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ying Li | Huawei |  |  | liying246@huawei.com |
| Lei Huang | Huawei |  |  | lei.huang1@huawei.com |
| Ross Jian Yu | Huawei |  |  | ross.yujian@huawei.com |
| Bin Tian | Qualcomm |  |  |  |
| Sigurd Schelstraete | MaxLinear |  |  |  |

Abstract

This submission contains proposed comment resolutions to 2 comments related to clause 36.3.10 of P802.11be D3.0.

CID 17196, 17197

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Revised the Resolution for CID 17196 and CID 17197 |

# #17196

| CID | Commenter | Clause Number(C) | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 17196 | Sigurd Schelstraete | 36.3.10 | 746.15 | Table 36-20, column "Description": N_SR is described as the "highest data subcarrier index". This is not correct if the punctured 20/40/80 MHz is at the high end of the bandwidth. | Correct description or add NOTE | Revised It is very clear from (36-9) and (36-10) that for the punctured and OFDMA case, N_SR is NOT used to represent the highest data subcarrier index for the assigned RU or MRU. For clarification, we correct the desctiption. TGbe editor: please incorporate the changes shown in 11-23/1029r1 under tag #17196. |

Background (P746 of D3.0):

Background (P753~756 of D3.0):

Proposed Changes:

Instruction to TGbe Editor:

Change Table 36-20 (P802.11be D3.0 P746) as follows:

Table 36-20—Subcarrier allocation related constants for the EHT-modulated fields in a punctured non-OFDMA EHT PPDU (continued)

| Parameter | CBW80 with 20 MHz puncturi ng 484+ 242- tone MRU | CBW160 with 40 MHz puncturi ng 996+ 484- tone MRU | CBW160 with 20 MHz puncturi ng 996+484+ 242-tone MRU | CBW320 with 120 MHz puncturi ng 2996+ 484-tone MRU | CBW320 with 80 MHz puncturi ng 3996- tone MRU | CBW320 with 40 MHz puncturi ng 3996+ 484-tone MRU | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NSR | 500 | 1 012 | 1 012 | 2 036 | 2 036 | 2 036 | (#17196) Highest data subcarrier index among all punctured cases |
| NDC | 23 | 23 | 23 | 23 | 23 | 23 | Number of null subcarriers at DC |
| NGuard,Left | 12 | 12 | 12 | 12 | 12 | 12 | Number of low frequency guard subcarriers |
| NGuard,Right | 11 | 11 | 11 | 11 | 11 | 11 | Number of high frequency guard subcarriers |

# #17197

| CID | Commenter | Clause Number(C) | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 17197 | Sigurd Schelstraete | 36.3.10 | 748.16 | "NOTE--For LDPC coding, this is the number of data bits per OFDM symbol based on the LDPC coding rate for the MCS.". I have no idea what the purpose of this note is. How is this different when BCC is used? This is a table with definitions. THis is not the place to go into the derivation of N_DBPS. | Delete NOTE | Revised This NOTE helps to clarify that N_DBPS is computed based on the code rate shown in MCS table instead of effective code rate for LDPC. For clarification, we revised the NOTE. TGbe editor: please incorporate the changes shown in 11-23/1029r1 under tag #17197. |

Background (P748 of D3.0):

Proposed Changes:

Instruction to TGbe Editor:

Change Table 36-23 (P802.11be D3.0 P748) as follows:

Table 36-23—Frequently used parameters (continued)

| Symbol | Explanation |
| --- | --- |
| NCBPSS u | Number of coded bits per OFDM symbol per spatial stream for user u, u = 0 1  Nuser total – 1 . |
| NCBPSS l u | Number of coded bits per OFDM symbol per spatial stream for user u in the l-th 80 MHz frequency block, u = 0 1  Nuser total – 1 and l = 0 1  L – 1 . L is the number of 80 MHz frequency subblocks. |
| NDBPS u | Number of data bits per OFDM symbol for user u, u = 0 1  Nuser total – 1 . (#17197) NOTE—For LDPC, N_DBPS,u is derived from N_CBPS,u using R_u, rather than the effective LDPC code rate, which may vary depending on shortening/puncturing/repetition performed during LDPC encoding. |