IEEE P802.11
Wireless LANs

| “Discussion of the normative text understanding of Frame Exchange Sequences” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Harry BIMS | Bims Laboratories, Inc. | Menlo Park, CA, USA. | 650 283 4174 | harrybims@me.com |

Abstract

This document raises for discussion the potential that normative text descriptions of frame exchange sequences may have been based on an understanding of frame exchange sequences that conflict with the current definition.  This conflict may result in a change to the normative text that may need to be resolved in Annex G as the need arises.

# A.  Discussion of frame exchange sequences

From 802.11REVme_D4.0:

According to this definition, the “sequence of frames” is not limited to a set of frame exchanges between a defined set of STAs, nor is “control of the wireless medium” managed by a single STA.  For example, all frame exchanges in the context of a TXOP are “maintaining control of the wireless medium”, and therefore there can only be one frame exchange sequence in a TXOP, even if frames are exchanged with different STAs (without limit) in different portions of the TXOP.

In comparison, the following normative text appears in both the 802.11e-2005 amendment (clause 9.9.1.2), and in the 802.11REVme_D4.0 maintenance draft (clause 10.23.2.3):

The above normative text suggests there can be one or more frame exchange sequences that can complete within a TXOP.  This is confirmed in the following normative text that appears in both 802.11e-2005 (clause 9.9.2.1.2) and 802.11REVme_D4.0 (clause 10.23.3.2.2):

Only when a particular EDCAF (that is within a particular STA) maintains “control of the wireless medium” after a frame exchange sequence has completed, is there an occurrence of a “multiple frame transmission” mode within the TXOP.

Indeed, in both 802.11e-2005 (clause 9.1.3.1) and 802.11REVme_D4.0 (clause 10.2.3.2), the normative text describes multiple frame exchange sequences in an EDCA TXOP.

In both 802.11e-2005 (clause 9.1.3.2) and 802.11REVme_D4.0 (clause 10.2.3.3), the normative text describes multiple frame exchange sequences in a polled TXOP.

The same language appears in 802.11e-2005 (clause 9.9.2) and 802.11REVme_D4.0 (clause 10.23.3.1) normative text:

As the normative text clearly shows in 802.11e-2005 (clause 9.9.2) and 802.11REVme_D4.0 (clause 10.23.3.1), a frame exchange sequence can be interrupted.  When this happens, control of the medium is lost.  Thus, recovery rules for frame exchange sequences (see clause 10.23.3.2.3) do not start a new frame exchange sequence.

According to 802.11e-2005 (clause 9.9.2.1) and 802.11REVme_D4.0 (clause 10.23.3.2.1), it is the protection mechanism that reserves the medium, not the frame exchange sequence itself.

According to 802.11e-2005 (clause 9.9.2.2.1), a frame exchange sequence can be initiated independent of the NAV value, confirming that the understanding of a frame exchange sequence does not include the protection mechanisms that reserve the medium.

Finally, see Doc 1387r1 (currently under discussion in UHR SG) for one example of conflicting understanding within 802.11 that continues to persist.  This proposal on TXOP optimizations is aimed at improving support within 802.11 for VR applications that require frame exchange sequences from multiple STAs to occur within a tight time window.  The proposal allows any STA to share a TXOP with minimal overhead for reverse direction or forwarding traffic.  This shows the current understanding in 802.11 is that the NAV may not be associated with a particular frame exchange sequence, as different STAs can initiate frame exchange sequences within the same TXOP that is “controlling the medium”.