IEEE P802.11
Wireless LANs

| CC50 CR on UEQM CID 372, 374 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2025-04-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jiyang Bai | TCL |  |  | Jiyangbai@tcl.com |

Abstract

This submission contains proposed comment resolutions to comments on P802.11bn D0.1. The changes are based on P802.11bn D0.2.

The submission provides discussion and proposed resolutions for CID 372 and CID 374 in subclause 38.3.15.9.6 and subclause 38.3.11, respectively.

R0 – initial version

2 CID related to UEQM

| 372 | 38.3.15.9.6 | 187.21 | In the User Specific field, when UEQM is applied, there are 2 to 4 MCS for each space stream. How to select the value in MCS subfield (i.e. B11-B15) is not clear. Besides, the value of MCS subfield is restricted due to the limitation of UEQM pattern (Table 38-28) and MCS tables (Table 38-50 to Table 38-65). Therefore, it is necessary to illustrate how to select MCS value or what kind of rule/restriction when selecting the MCS subfield value. | 1. In MCS Description Illustrate that using MCS with the Highest modulation order/ the first space stream as the the MCS subfield value when UEQM is applied. 2. Presenting the available values of MCS subfields. The commentor qwill bring a contribution to address this comment and provide more detialed solutions. | REVISED. Text related to motion 168, 169, 199 and 200 enable to illustrate available MCS selections for the MCS subfield. Please apply changes marked as #372 in this document. |
| --- | --- | --- | --- | --- | --- |
| 374 | 38.3.11 | 134.14 | When selecting the value of UHR-MCS, the corresponding modulation and coding scheme maybe not available for the UEQM scheme. | It is necessary to illustrate that the MCS value selection is restricted, with clear indication to available MCS values when UEQM is applied. | REVISED. Text related to motion 168, 169, 199 and 200 enable to illustrate available MCS selections for the MCS subfield. Please apply changes marked as #374 in this document. |

# Discussion

In the User Specific field, when UEQM is applied, the value in the MCS subfield (i.e. B11-B15) represent the MCS of the first spatial stream. However, according to the motions of UEQM pattern, new MCSs and available MCSs for UEQM (Motion 168, 169, 199, 200), not all MCS are available due to the constraint of UEQM patents (Table 38-28) and available modulation order and coding rate.

For instance, if NSS=2, UEQM patent=1, for the first spatial stream,

- MCS 0, 17, 2, 20 are not available because there is no corresponding MCS for the second spatial stream.

- MCS 1 is not available because MCS 0 is not available in UEQM and there is no available MCS for the second spatial stream.

# Proposed Change

Text to be adopted begins here:

38.3.15.9.6 User Specific field

TGbn editor: Please update User Specific field in 11bn D0.2 to add available MCSs of the first spatial stream as below

In 38-29 (UEQM pattern subfield encoding), “s” is the constellation index value corresponding to the

modulation order of the UHR-MCS used in the first spatial stream, which is defined in Table 9-417u

(Constellation index), and “s-Δ” represents the modulation order(s) used in the rest of the spatial stream that is Δ modulation levels lower than s.

NOTE – The modulation order from the first to the sixth corresponds to QPSK, 16-QAM, 64-QAM, 256-

QAM, 1024-QAM, and 4096-QAM.

Table 38-29—UEQM pattern subfield encoding

| NSS Subfield | Number of Spatial Streams | UEQM Pattern Subfield | UEQM pattern |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Stream 1 | Stream 2 | Stream 3 | Stream 4 |
| 1 | 2 | 0 | s | s-1 | N/A | N/A |
|  |  | 1 | s | s-2 | N/A | N/A |
| 2 | 3 | 0 | s | s | s-1 | N/A |
|  |  |  | s | s | s-2 | N/A |
|  |  |  | s | s-1 | s-2 | N/A |
| 3 | 4 | 0 | s | s | s | s-1 |
|  |  | 1 | s | s | s | s-2 |
|  |  | 2 | s | s | s-1 | s-2 |
|  |  | 3 | s | s-1 | s-1 | s-2 |

In 38-xx (Available MCSs of the first spatial stream), “s” is the constellation index value corresponding to the modulation order of the UHR-MCS used in the first spatial stream, which is defined in Table 9-417u (Constellation index).  “ Ru” is the coding rate of the u-th user.(#372)

Table 38-xx—Available MCSs of the first spatial stream (#372)

| Value in the UEQM pattern subfield | Constellation index s | Modultation | Coding Rate Ru |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1/2 | 2/3 | 3/4 | 5/6 |
| 0 | 2 | 16QAM | MCS 3 | MCS 19 | MCS 4 | N/A |
|  | 3 | 64QAM | N/A | MCS 5 | MCS 6 | MCS 7 |
|  | 4 | 256QAM | N/A | MCS 23 | MCS 8 | MCS 9 |
|  | 5 | 1024QAM | N/A | N/A | MCS 10 | MCS 11 |
|  | 6 | 4096QAM | N/A | N/A | MCS 12 | MCS 13 |
| 1-3 | 3 | 64QAM | N/A | MCS 5 | MCS 6 | N/A |
|  | 4 | 256QAM | N/A | MCS 23 | MCS 8 | MCS 9 |
|  | 5 | 1024QAM | N/A | N/A | MCS 10 | MCS 11 |
|  | 6 | 4096QAM | N/A | N/A | MCS 12 | MCS 13 |

38.3.11 UHR modulation and coding schemes (UHR-MCSs) and Unequal modulation (UEQM)

TGbn editor: Please update UHR-MCSs an UEQM in 11bn D0.2 to add constraint of MCS selection on the first spatial stream as below

For UEQM, the uncoded bits transmitted over all spatial streams for the same user in the Data field of the

PPDU are jointly coded, and the coded bits parsed to at least one of the spatial streams are modulated with a different modulation level from that of the first spatial stream. In the case of UEQM, UHR-MCS is a value that indicates the modulation and coding scheme for the first spatial stream in the Data field of the PPDU. The modulation orders of the subsequent spatial streams are indicated by the UEQM Pattern subfield in the user field of UHR-SIG in Table 38-29 (UEQM pattern subfield encoding) in 38.3.15.9.6 (User Specific field). The available MCSs of the first spatial stream are indicated in Table 38-30 (Available MCSs of the first spatial stream). (#374)

Text to be adopted ends here.

SP: Do you agree to incorporate the proposed text changes for UEQM to the latest TGbn draft?

References:

- 11-24-0171r21: 11-24-0171-21-00bn-tgbn-motions-list-part-1, Alfred Asterjadhi (Qualcomm Inc.)

- 11-25-0014-r7: 11-25-0014-07-00bn-tgbn-motions-list-part-2, Alfred Asterjadhi (Qualcomm Inc.)