IEEE P802.11
Wireless LANs

| Minutes for 802.11 bn PHY ad-hoc in September 2025 Interim session |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2025-09-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dongguk Lim | LG Electronics |  |  | Dongguk.lim@lge.com |

Abstract

This document contains the PHY ad hoc meeting minutes for TGbn held on:

- Tuesday AM2, September 16, 2025

- Thursday AM1, September 18, 2025 (Cancelled)

Tuesday September 16th, 2024 10:30 – 12:30 Hawaii time

Introduction

- The Chair (Tianyu, Apple) calls the meeting to order at 10:30am Hawaii time.

- The Chair follows the agenda in 11-25/1432r1.

- Reminder for registration for the Interim meeting.

- The Chair reminds everyone to report their attendance by using IMAT system and by sending an e-mail to the Co-chair, Dongguk Lim (LGE), Sigurd Schelstraete (MaxLinear) or the Chair himself if unable to record attendance via IMAT system.

- The Chair goes through the IPR policy and asks if anyone is aware of any potentially essential patents. Nobody speaks up.

- The Chair goes through the Copyright policy.

- Discussions on the agenda.

25/1176 Unused tone EVM for DRU (update)					Lin Yang

25/1608 A Simple Procedure to Determine Co-BF Sync-reference and Sync-follower Roles													Shuling Feng

25/1609 2x LDPC Usage at Shared AP for CoBF					Sameer Vermani

25/1534 Discussion on N_UHR-LTF for DRU Transmission			Eunsung Park

25/1621 LTF Design for Co-SR Channel Estimation					Ying Wang

Technical contributions

- 25/1176 Unused tone EVM for DRU (update)			Lin Yang(Qualcomm)

Discussions:

C: In slide 21, it is not clear whether unused tone is a tone within DBW or DRU

A: It is for each DRU. I will fix the text by adding “each”.

SP1

Do you support to include the following?

- Define the transmit constellation error in an UHR DRU TB PPDU for all MCSs to be the same as in an UHR RRU TB PPDU

Results: No objection

SP2

Do you support to include the following?

- Local oscillator leakage and its within ±3 neighboring subcarriers shall be excluded from the computation of the transmitter modulation accuracy test for DRU

Results: No objection

SP3

- Do you support to include the following?

DRU unused tone EVM requirement for the unused tones within DBW is given by

UnusedToneErroriDRU26/52≤max(ε-1,-38dB)

  - Where ε is used tone EVM, iDRU26/52 is index of other DRU26/52 than the transmitted in the same DBW

  - Applicable to all DRUs

  - EVM measurement over unused tones within DBW

  - EVM measurement over each unused DRU26 for DBW20 and DBW40

  - EVM measurement over each unused DRU52 for DBW60 and DBW80

Results: No objection

SP4’ (Cleaner Version)

- Do you support to include the following into the 11bn SFD

DRU unused tone EVM requirement for the unused tones outside DBW but within PPDU is given by

UnusedToneErroriRRU242,  start+m≤maxε – DRU spreading gain, -38dB,maxε-10– DRU spreading gain, -38dB,if -r≤m≤-1if -2r≤m≤-r-1maxε-20– DRU spreading gain, -38dB,-38dB,if -3r≤m≤-2r-1otherwise

UnusedToneErroriRRU242,  end+m≤maxε – DRU spreading gain, -38dB,maxε-10– DRU spreading gain, -38dB,if 1≤m≤rif r+1≤m≤2rmaxε-20– DRU spreading gain, -38dB,-38dB,if 2r+1≤m≤3rotherwise

  - Where ε is used tone EVM, r is number of DBW20 in the transmitting DBW, r=1 for DBW20, r=2 for DBW40, and r=4 for DBW80 and DBW60

  - EVM measurement over unused tones outside of DBW but within PPDU

  - DRU spreading gain is given by

|  | DRU Spreading Gain in dB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DRU26 | DRU52 | DRU106 | DRU242 | DRU484 |
| DBW20 | 9 | 6 | 3 | N/A | N/A |
| DBW40 | 12 | 9 | 6 | 3 | N/A |
| DBW80 | N/A | 12 | 9 | 6 | 3 |
| DBW60 | N/A | 12 | 9 | 6 | N/A |

Results: No objection

SP5

Do you support to add the following to 11bn SFD?

The unused tone constellation error in an UHR DRU TB PPDU is define as:

UnusedToneErrorRMSk= 1Nfif=1Nfis=1NSYMisc∈Ωk(Iu(if,is,isc))2+(Qu(if,is,isc))2NSYM∙|Ωk|∙PS

where Ωk is a set of subcarriers for k-th

-  26-tone DRU (within 20MHz/40MHz DBW)

- 52-tone DRU (within 60MHz/80MHz DBW)

- 242-tone RRU (outside of DBW)

Results: No objection

- 25/1608r0, A Simple Procedure to Determine Co-BF Sync-reference and Sync-follower Roles, 								Shuling (Julia) Feng (Mediatek)

Discussions:

C: Which case do you recommend that the role of sync reference and follower keep?

A: It depends on the implementation.

C: What is an advantage of being a sync follower or reference?

A: For Co-BF sounding/transmission, we need to synchronize the carrier. It can be performed simply.

SP 1

Do you support to use the following procedure to determine MAPC Co-BF sync-reference and sync-follower roles when Co-BF MAPC agreement is established or updated in 11bn?

- A MAPC requesting AP indicates explicitly its sync-reference / sync-follower role in the MAPC Scheme Request Set field of the Co-BF profile in the MAPC Negotiation Request frame.

- A MAPC responding AP assumes the sync-follower / sync-reference role left by the requesting AP if it accepts the agreement establishment or agreement update.

Note: It’s recommended to keep a CoBF AP’s sync-reference/follower role semi-static, only changes it in Co-BF MAPC agreement update when necessary.

Results: No objection

- 25/1609r0, “2x LDPC Usage at Shared AP for CoBF”, 	Sameer Vermani (Qualcomm)

Discussions : No discussion

SP1

Do you support adding the following rules for Co-BF in the TGbn spec in 37.15.2.1.4 Co-BF transmission procedure?

  - If the minimum number of data OFDM symbols of the Co-BF transmission indicated in the Co-BF Invite frame corresponds to Navbits > 3888 bits for a user of the coordinated AP, then the coordinated AP may indicate that 2xLDPC will be used for the user in the Co-BF Response frame.

  - If the minimum number of data OFDM symbols of the Co-BF transmission indicated in the Co-BF Invite frame corresponds to Navbits ≤ 3888 bits for a user of the coordinated AP, then the coordinated AP shall not indicate that 2xLDPC will be used for the user in the Co-BF Response frame.

  - A coordinating AP shall not choose a value for the Length field in the L-SIG field of the Co-BF PPDU in the Co-BF Trigger frame that corresponds to a number of OFDM symbols in the Data field which is smaller than the minimum number of data OFDM symbols the coordinating AP had indicated in the Co-BF Invite frame

Note: These changes for the TGbn editor are reflected in word document below

https://mentor.ieee.org/802.11/dcn/25/11-25-1670-00-00bn-2x-ldpc-related-spec-text-changes-for-cobf.docx

Results: No objection

SP2

- Do you support adding the following recommendation for COBF to the TGbn spec in 37.15.2.1.4 Co-BF transmission procedure?

  - To help the coordinated AP choose an appropriate LDPC codeword length, the minimum number of data OFDM symbols of the Co-BF transmission indicated in the Co-BF Invite frame should be greater than or equal to 0.5 times the maximum number of data OFDM symbols of the Co-BF transmission indicated in the Co-BF Invite frame.

Note: These changes for the TGbn editor are reflected in word document below

https://mentor.ieee.org/802.11/dcn/25/11-25-1670-00-00bn-2x-ldpc-related-spec-text-changes-for-cobf.docx

Results: No objection

- 25/1534 Discussion on N_UHR-LTF for DRU Transmission	Eunsung Park(LG Electronics)

Discussions:

C: I want to support option3 for backward capability with 11be.

A: Okay.

C: I slightly support option 4. The limitation of up to 4 is more consistent.

C: It supports 4 or 8 for hybrid mode, right?

SP1

- Do you support the following for TGbn?

  - 6 N_UHR-LTF is disallowed for a DRU transmission and a hybrid RRU and DRU transmission

Results: No objection

SP2

- Do you support the following for TGbn?

  - For a hybrid RRU and DRU transmission, partial BW MU-MIMO is disallowed

Results: No objection

- 25/1621 LTF Design for Co-SR Channel Estimation		Ying Wang(InterDigital)

Discussions:

C: It seems to require the change of existing EHT and UHR, and is complex.

A: Yes, many change needs to enable this MU-MIMO

C: I think maybe we keep this simple method for now. It can be discussed in the next generation.

A: Thanks for your comment.

Recess

The meeting is Recessed at 12:30pm Hawaii Time.

Thursday September 18th, 2024 08:00 – 10:00 Hawaii time ( Cancelled)