IEEE P802.11
Wireless LANs

| November 2008 Interpretation Response |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 USA | +1 925 474 2273 | msg@trapezenetworks.com |

Abstract

[place document abstract text here]

Designation Number: 15

Document: IEEE Std. 802.11-2007

Classification: Unambiguous

Relevant Clauses: 11.7.1.2

Relevant Figures: not applicable

Topic: DLS setup procedure at the AP

Interpretation Request #15:

In 802.11-2007, section 11.7.1.2 ("Setup procedure at the AP") [for DLS] notes:

"[...] Send DLS Response frame to the STA that sent the DLS Request frame with a result code of Not a STA, if the destination STA [...]"

Is this meant to say

"[...] Send DLS Response frame to the STA that sent the DLS Request frame with a result code of Not QoS STA, if the destination STA [...]"

(note "Not QoS STA")

Interpretation Response #15

IEEE Std. 802.11-2007 contains a mapping of result codes from DLS setup primitives into status codes in Table 11-6.  Table 11-6 contains the result codes NOT_ALLOWED, NOT_PRESENT, and NOT_QOS_STA.  The status codes defined by IEEE Std. 802.11-2007 are found in Table 7-23, and use status code 50 for "The Destination STA is not a QoS STA."

The standard provides no additional detail.  The question of whether additional detail is needed will be forwarded to the 802.11 Working Group for consideration in the next revision of the P802.11 standard.