IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG8 PAC Framework Document (Draft) |  |
| Date Submitted | May 16, 2013 (r0) May 16, 2013 (r2) Nov. 12, 2013 (r4) Nov. 14, 2013 (r7) Nov. 14, 2013 (r8) Jan. 07, 2014 (r9) Jan. 13, 2014 (r10) |  |
| Source | 802.15.8 Technical Editor: Byung-Jae Kwak (ETRI) Seung-Hoon Park (Samsung) | E-Mail: [bjkwak@etri.re.kr] [shannon.park@samsung.com] |
| Re: |  |  |
| Abstract | This is the draft version of 802.15.8 PAC Framework Document. |  |
| Purpose | This document provides the framework from which the draft PAC specification will be developed. The document provides an outline of each the functional blocks that will be a part of the final specification. The document is intended to reflect the working consensus of the group on the broad outline for the draft specification. As such it is expected to begin with minimal detail reflecting agreement on specific techniques and highlighting areas on which agreement is still required. It may also begin with an incomplete feature list with additional features added as they are justified. The document will evolve over time until it includes sufficient detail on all the functional blocks and their inter-dependencies so that work can begin on the draft specification itself. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.15 Working Group or IEEE 802.15.8 Task Group. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table of Contents

1.	Overview	7

2.	Definitions	7

3.	Abbreviations and acronyms	8

4.	General descriptions	9

4.1.	Concepts and architecture	9

4.2.	Topology	11

4.3.	Reference model	11

5.	MAC layer	12

5.0.	Overview	12

5.1.	Frame Structure	12

5.2.	Synchronization	13

5.3.	Discovery	13

5.4.	Peering	14

5.5.	Communications	15

5.5.1.	Unicast	15

5.5.2.	Multicast	16

5.5.3.	Broadcast	16

5.10.1 Multicast Group Management	16

5.10.2 Multicast Data Transmission	25

5.10.3	Prevention Loopback Problem	27

5.10.4	Reliable Multicast	27

5.10.5	Multicast Protocol Using Directional Antenna	28

5.6.	MPDU structure	30

5.6.1.	Packet Format	31

Table 1. ACF (Advertisement Command Frame)	31

Table 2. ARCF (Advertisement Reply Command Frame)	31

Table 3. MGNF (Multicast Group Notification Frame)	31

Table 4. Multicast Data Frame	32

PPDU structure	32

MPDU structure	32

5.7.	Multiple access	33

5.7.1.	Basic Operation	33

5.7.2.	Backoff Procedure	33

5.7.3.	Collision Detection	34

5.7.4.	RTS/CTS Frame Exchange Procedure	34

5.7.5.	Unicast Frame Transmission Procedure	34

5.8.	Synchronization procedure	35

5.8.1.	PAC synchronization modes and procedure	36

5.8.2.	Synchronization procedure for operations in unlicensed band	37

5.9.	Discovery procedure	38

5.9.1.	Low Energy Service Discovery (LESD) protocol	39

5.9.2.	LESD MAC command frames	44

5.9.3.	LESD MAC Primitives	45

5.9.4.	LESD MAC constants and PIB attributes	51

5.10.	Scheduling	54

5.11.	QoS	54

5.12.	Interference management	55

5.13.	Transmit power control	55

5.14.	Multicast ( Multicast already covered in 5.4 Communications)	57

5.15.	Broadcast ( Broadcast already covered in 5.4 Communications)	57

5.16.	Multi-hop operation	57

5.12.1	Multi-hop Unicast Data Transmission	57

5.17.	Relative positioning	59

5.18.	Power management	59

5.19.	Security	59

5.15.1 Security modes	59

5.20.	Coexistence	68

5.20.1.	Interference sensing	68

5.20.2.	Blocking signal	68

5.20.3.	Low power transmission	69

5.21.	Higher layer interaction	69

6.	Physical layer	70

6.1.	Block Diagram	70

6.2.	Channelization	70

6.2.1.	Sub 1GHz band	70

6.2.1.	Channelization of 5.7 GHz band	71

6.2.2.	Channelization of 2.4 GHz band	71

6.2.3.	Channelization of sub-GHz band	71

6.2.4.	6.2.4. UWB band	71

6.3.	Duplex schemes	72

6.3.1.	TDD	72

6.3.2.	FDD	72

6.4.	Multiplex schemes	72

6.5.	Frame PPDU structure	73

6.5.1.	Frame structure in FDD mode	73

6.5.2.	Frame structure in TDD mode	74

6.5.3.	Cyclic prefix	75

6.5.4.	Resource block	75

6.5.5.	Data frame structure	76

6.5.1.	Preamble	77

6.5.2.	Beam Jitter field	80

6.5.1.	Discovery frame structure	81

6.5.2.	Data frame structure	82

6.5.3.	Synchronization region	83

6.5.4.	Discovery region	83

6.5.5.	Peering region	84

6.5.6.	Data region	84

6.5.7.	Discovery frame structure	85

6.6.	Modulation and coding scheme (MCS)	87

6.6.1.	Modulation	87

6.6.2.	Coding Scheme	89

6.6.3.	Pulse shape and duration	90

6.6.4.	Packet structure	90

6.6.5.	SHR Structure	90

6.6.6.	Symbol structure	91

6.6.7.	Data rates	91

6.6.8.	Channel coding and data rates	91

6.6.9.	Symmetrical double-sided two-way ranging	92

6.6.10.	Discovery mode	93

6.7.	Multiple antennas	93

6.7.1.	Layer mapping	93

6.7.2.	Precoding	94

6.8.	Bit interleaver	95

6.9.	Scrambling	95

6.10.	Discovery	96

6.10.1.	Discovery procedure	98

6.11.	Peering	98

6.11.1.	Peering procedure	99

6.12.	Reference signals	100

6.12.1.	Channel time dispersion	100

6.12.2.	Channel frequency dispersion	100

6.12.3.	ZC sequences	100

7.	UWB Physical (PHY) layer specification	101

7.1.	General	101

7.2.	PPDU format	102

7.2.1.	PPDU encoding process	102

7.2.2.	Symbol structure	104

7.2.3.	PSDU timing parameters	105

7.2.4.	Preamble timing parameters	107

7.2.5.	SHR preamble	109

7.2.6.	PHY header (PHR)	112

7.2.7.	Data field	114

7.3.	UWB PHY modulation	114

7.3.1.	Modulation mathematical framework	114

7.3.2.	Spreading	115

7.3.3.	Forward error correction (FEC)	117

7.4.	UWB PHY RF requirements	119

7.4.1.	Operating frequency bands	119

7.4.2.	Channel assignments	119

7.4.3.	Transmitter specification	120

7.5.	Timestamps and time units	122

Appendix A	123

<IMPORTANT>

Editor’s Note:

- Please use the latest version of PFD as a base when you submit text proposals. Otherwise, it makes editing unnecessarily difficult.

- Text proposals with format incompatibility will be IGNORED, and will NOT be accepted.

- When you send an updated text, please MARK CLEARLY what is updated and EXPLAIN how to interpret the markings. Editors need to know what THE EXACT CHANGES are.

</IMPORTANT>

<CHANGES>

Changes from revision 9:

- …

</CHANGES>

# Overview

The 802.15.8 specification shall be developed according to the P802.15.8 Peer Aware Communication (PAC) project authorization request (PAR), document number 15-12-0063r2 and Five Criteria (5c), document number 15-12-0064r1, which were approved by the IEEE-SA in March of 2012.

This standard defines PHY and MAC mechanism for Wireless Personal Area Networks (WPAN) Peer Aware Communications (PAC) optimized for Peer-to-Peer and infrastructure-less communications with fully distributed coordination. A peer can participate in multiple services or applications.

Key features of PAC may include the following:

- Operational in selected globally available unlicensed/licensed bands below 11 GHz capable of supporting these requirements

- Scalable data transmission rates, typically up to 10 Mbps

- Discovery for peer information without association

- Discovery signaling rate typically greater than 100 kbps

- Discovery of the number of devices in the network

- Group communications with simultaneous membership in multiple groups, typically up to 10

- Multi-hop relay

- Relative positioning

- Security

# Definitions

Device ID: Unique PAC device ID, e.g. MAC address

Discovered PD: PD which transmits the response signal to discovering PD  needs re-wording (any volunteer?)

Discovering PD: PD which transmits the request signal to discover other PD(s)  needs re-wording (any volunteer?)

Peer aware communications (PAC) network: A peer-to-peer wireless proximity network of which a device can act as a for the other devices by allowing shared access to various resources such as configuration or control information, location information, sensing data, advertisement, multi-media contents, social contents, etc.  needs re-wording (any volunteer?)

Service group: The set of PDs which became peering each other  needs re-wording (any volunteer?)

Service type ID: Service category for PAC discovery

Burst: group of ultra wide band (UWB) pulses occurring at consecutive chip periods This terminology not limited to UWB?

Mean pulse repetition frequency (PRF): total number of pulses within a symbol divided by the symbol duration  PRF also means pseudo random function; besides, maybe does not belong to PFD

Payload data: contents of a data message that is being transmitted

Peak pulse repetition frequency (PRF): maximum rate at which an ultra-wide band (UWB) physical layer (PHY) emits pulses PRF also means pseudo random function; besides, maybe does not belong to PFD

Ranging frame (RFRAME): ultra wide band (UWB) frame having the ranging bit set in the physical layer (PHY) header (PHR)  maybe does not belong to PFD

Ranging marker (RMARKER): first ultra wide band (UWB) pulse of the first bit of the physical layer (PHY) header (PHR) of a ranging frame (RFRAME).  maybe does not belong to PFD

Symbol: a period of time and a portion of the transmitted signal that is logically considered to be a unit signalling event conveying some defined number of data bits or repeated portion of the synchronization signal.

# Abbreviations and acronyms

AGC		Automatic Gain Control

CAP		contention access period

CCA		clear channel assessment

CFP		contention free period

CP		Cyclic Prefix

CS		Channel Sampling

CTS		Clear To Send

CW		Contention Window

FCS		Frame Check Sequence

GI		Guard Interval

IS		Interference Sensing

LTF		Long Training Field

MLME		MAC sublayer Management Entity

MLSDE		MAC sublayer Service Discovery Entity

OFDM		Orthogonal Frequency Division Multiplexing

PAN		Personal Area Network

PD		PAC Device

PDU		Protocol Data Unit

PID    		Peering Identifier

Peering-REQ	Peering-Request

Peering-RSP	Peering-Response

PLME		PHY layer Management Entity

RTS		Request To Send

SC		Single Carrier

SDU		Service Date Unit

STF		Short Training Field

ULA		Uniform Linear Array

ZC		Zadoff-Chu

BPM		burst position modulation  UWB; maybe does not belong to PFD

BPSK		binary phase-shift keying

CRC		cyclic redundancy check

DPS		dynamic preamble selection  UWB; maybe does not belong to PFD

FEC		forward error correction

LFSR		linear feedback shift register

LSB		least significant bit

MAC		medium access control

MSB		most significant bit

PHR		PHY header

PHY		physical layer

PPDU		PHY protocol data unit

PRBS		pseudo-random binary sequence

PRF		pulse repetition frequency  also means pseudo random function

PSD		power spectral density

PSDU		PHY service data unit

RF		radio frequency

RFRAME	ranging frame  UWB; maybe does not belong to PFD

RMARKER	ranging marker UWB; maybe does not belong to PFD

SHR		synchronization header

SYNC		synchronization

UWB		ultra wide band

# General descriptions

This clause provides the basic framework of PDs. The framework serves as a guideline in developing the functionalities of PDs and their interactions specified in detail in the subsequent clauses.

## Concepts and architecture

PAC provides functionalities optimized for scalable peer to peer communications with fully distributed coordination for decentralized system composed of PDs. PAC also provides mechanisms that enable low power consumption, discovery of peer information before association, efficient resource allocation in a distributed manner, and coexistence with other 802 systems.

Some PDs may be able to connect to infrastructure on an opportunistic basis, which is out of scope for IEEE 802.15.8.

A PAC system may contain the following functions at MAC and/or PHY.

Upper Layer: the layer above PHY/MAC, such as service or application layer for an infrastructure-less P2P wireless system.

PD Management Entity: manages PD information across PHY/MAC and higher layer for P2P communications. Editor: P2P  PAC?

Synchronization Function: performs initial and/or periodic time boundary synchronization with a peer at MAC; maintains frequency and/or clock phase synchronization at PHY.

Discovery Function: discovers peer(s) in proximity by using IDs such as Device ID, Device Group ID, Application type ID, Application-specific ID, Application-specific user ID, and Application-specific group ID, and/or peer context information; sends discovery request messages with IDs such as Device ID, Device Group ID, Application type ID, Application-specific ID, Application-specific user ID, and Application-specific group ID, and/or peer context information for to-be-discovered in proximity.

Peering Function: requests or responds to Peering (i.e. association), Peering Updates, De-peering, or Re-peering by using IDs such as Device ID, Device Group ID, Application type ID, Application-specific ID, Application-specific user ID, and Application-specific group ID, and/ or peer context information.

Channel Management Function: manages the radio resource or channel allocation among P2P networks based on the services or applications; manages channel allocation and/or accessing within a P2P network based on the peer information.

Power Control Function: performs transmitting power control and interference management based on power control information.

Data Transceiving Function: conducts reliable data transmitting and/or receiving based on the QoS required by service or application.

Measurement and Report Function: conducts measurements of channel, QoS, etc., and sends data reports from other logic functions to higher layer.

Scheduler Function: manages or controls the sequence of events in MAC and/or PHY layer, for example, transmission queuing, collision avoidance, etc.

Encoder Function: performs encoding and other data processing such as interleaving, scrambling, etc. at PHY to aid in reliable reception

Modulator Function: performs modulation and other data processing such as mapping, precoding, etc. at PHY

Decoder Function: performs decoding and other data processing, such as de-interleaving, de-scrambling, etc. at PHY to aid in reliable reception

Demodulator Function: performs demodulation and other data processing such as equalization, de-mapping, etc. at PHY

</380r2>

## Topology

Several topologies are considered to support interactions among PDs for various services. One-to-one and one-to-many topologies shall be supported.

IEEE 802.15.8 shall support a PD having simultaneous communication sessions for same or different applications. IEEE 802.15.8 shall support a PD participation in at least two independent one-to-many communications with different peers at the same time.

Mesh topology or star topology may be supported. PAC supports multi-hop relay of at least two hops.

Figure 4.3. An example of concurrent communication

## Reference model

All PDs are internally partitioned into a physical (PHY) layer and a medium access control (MAC) sublayer of the data link layer, in accordance with the ISO/OSI-IEEE Std 802-2001 reference model. Direct communications between PDs are to transpire at the PHY layer and MAC sublayer as specified in this standard; Message security services are to occur at the MAC sublayer, and security operations are to take place inside and/or outside the MAC sublayer.

Within a PD, the MAC provides its service to the higher layer through the MAC service access point (SAP) located immediately above the MAC sublayer, while the PHY provides its service to the MAC through the PHY SAP located between them. On transmission, the higher layer passes MAC service data units (MSDUs) to the MAC sublayer via the MAC SAP, and the MAC sublayer passes MAC frames (also known as MAC protocol data units or MPDUs) to the PHY layer via the PHY SAP. On reception, the PHY layer passes MAC frames to the MAC sublayer via the PHY SAP, and the MAC sublayer passes MSDUs to the higher layer via the MAC SAP.

MAC and PHY SAPs also pass control information between the layers.

Figure 2. Reference model

There may be a logical PD management entity (PDME) that exchanges network management information with the PHY and MAC as well as with other layers.

# MAC layer

  - Overview

This section defines MAC mechanism for Wireless Personal Area Networks (WPAN) Peer Aware Communications (PAC) optimized for peer-to-peer and infrastructure-less communications with fully distributed coordination.

The MAC functions are described in this clause. The following are main functions and their brief explanation.

- Synchronization is the procedure to establish synchronization reference among PDs.

- Discovery is the procedure to find existence and capability information of other PDs that are within discoverable range. Capability information contains not only PHY/MAC layer information but also high layer information (service, application, etc.).

- Peering is the procedure to establish a link between a pair of PDs or links among multiple PDs.

- Communication is the procedure to exchange user data in conjunction with control/management message among PDs.

## Frame Structure

PAC operates in synchronization mode, where PDs share timing reference with other PDs in proximity. The timing reference defines where the frame boundaries are.

The radio resource is comprised of successive frames with fixed time duration. The length of the frame is for further study. A frame is divided into regions as illustrated in the following figure.

Figure: Frame structure

Timing reference signals are transmitted in the Synchronization region. The details of timing reference signal will be determined after further study.

Discovery, peering, and/or data communication region is used for discovery, peering, and/or data communications. The discovery, peering, and/or data communications region can be further divided into smaller regions. Different combinations of the smaller regions can be used to form different frame types to satisfy the requirements for discovery, peering, and data communications. The detailed structure of this region will be determined after further study.

## Synchronization

In synchronized mode, PAC operates in fully distributed synchronization.

PAC currently recognizes two slightly different definitions of fully distributed synchronization.

- [Def. 1] Synchronization among PDs can be achieved by transmitting/receiving synchronization reference signal. When there is already synchronization among PDs, each PD may send synchronization reference signal independently to maintain the synchronization. When there is no synchronization already established, a PD can initiate it by transmitting synchronization reference signal.

- [Def. 2] Synchronization among PDs can be achieved by transmitting/receiving synchronization reference signal. The synchronization reference signal may be transmitted by a single PD elected by a group of PDs. When there is no synchronization already established, a PD can initiate it by transmitting synchronization reference signal.

The merits and demerits of each definition of fully distributed synchronization will be determined based on their corresponding technical contributions.

## Discovery

PAC should support three discovery types of the following:

- Advertisement: In Advertise type discovery, a PD broadcasts its own identity and does not expect responses.

- Publish/Subscribe: In Publish/Subscribe type discovery, a PD broadcasts its own identity and expects responses from PDs that have discovered the broadcast message.

- Query/Reply: In Query/Reply type discovery, a PD broadcasts the identity of the PD or PDs being queried and expects a response or responses from the PD or PDs, accordingly.

For the purpose of discovery of PDs, the discovery signal conveys information that may represent one or more of the following IDs such as Device ID, Device Group ID, Application type ID, Application-specific ID, Application-specific user ID, and Application-specific group ID.

The discovery procedure should support mechanisms to ensure privacy that a PD is not tracked.

The discovery procedure should support protection of identity from impersonation.

## Peering

Peering is the procedure to establish a link between a pair of PDs or links among multiple PDs discovered during the discovery procedure.

Re-peering is the procedure to re-establish a link between a pair of PDs or links among multiple PDs which peered previously. In the re-peering procedure, discovery may be simplified or omitted.

De-peering is the procedure to disconnect the link established by peering. The disconnected link through de-peering may be re-established by re-peering procedure.

Peering Procedure

Figure: Peering Procedure

The peering procedure is initiated by sending a peering request message including requested peering information. Acknowledgement to peering request message may be optional for responder. Responder may send a peering response message to requestor for indicating if the peering request is accepted or not. The response message may include peering information if the request is accepted. Acknowledgement to response message may be optional as well.

Re-peering procedure:

Figure: Re-peering Procedure

Re-peering procedure is similar to peering procedure. The main difference is: 1) the previous peering information should be included in request message since the re-peering is to re-establish a link or links among PDs that peered previously; 2) the PD(s) receives the request need to check the peering log before making decision if accepts the re-peering request. Both acknowledgement may be optional.

De-peering procedure:

Figure: De-peering Procedure

De-peering procedure starts with a de-peering request, which is replied by a de-peering response message. Both acknowledgement and de-peering response may be optional. Accordingly, the peer may triggers peer discovery process with updated parameters after de-peering procedure.

## Communications

Data communication may be conducted as unicast between a pair of PDs, or as multicast within a group of PDs, or as broadcast to any PD.

### Unicast

Unicast is a one-to-one data communication between a pair of PDs. For reliable unicast transmission, ACK may be used for acknowledging a successful data transmitting.

### Multicast

Multicast is a one way one-to-many data communication to a group or groups of PDs which may be addressed by an ID specific to the group or by multiple IDs. To support multicast, the following features are supported:

- Multicast Group Creation: A PD is able to create a multicast group if necessary.

- Joining multicast group: A PD is able to join a multicast group or multiple multicast groups in order to communicate with multicast group members.

- Leaving multicast group: A PD is able to leave a multicast group or multiple multicast groups to stop communication with multicast group members.

Reliable multicast is to provide reliable data transmission in multicast from one PD to multiple or all PDs with a group. For reliable multicast transmission, ACK may be used for acknowledging a successful multicasting.

Figure: Distributed One-Hop Multicast

### Broadcast

Broadcast is a one way one-to-many data communication to any PD.

TWTW

- Tp(Period)+Tj)

T0

- Trdst=Tp+Tjmax+T1-T02+(safety margin).

- TrD

- TrB

T0

## MPDU structure

MPDU consists of MAC header, a variable length frame body, and a fixed length FCS. MAC header comprises data frame type, device IDs, service type IDs, MCS, MPDU length, and other information about the MPDU. Frame body is data information. FCS contains CRC check sequence for error detection.

Figure. PAC MPDU format

## Multiple access

Multiple access schemes allow multiple terminals to share a communication medium. For PAC standard, contention-based as well as contention-free multiple access schemes are under consideration.

In a contention-based multiple access scheme, multiple terminals compete for channel access. Examples of contention-based channel access schemes are Aloha protocol, and CSMA/CA. While contention-based access schemes tend to be simpler, channel access is not guaranteed and depends on probability.

Contention-free multiple access schemes try to guarantee channel access to devices so that the devices do not have to compete for channel access. It is usually easier to provide QoS provisioning with contention-free multiple access schemes, but they require coordination and scheduling among multiple devices.

CW

rIrD

| rI |  |
| --- | --- |
| rD |  |

## Synchronization procedure

IEEE802.15.8 PAC operates in synchronization mode. For PAC standard, three synchronization schemes are under consideration. In an arbitrary order, the three synchronization schemes are as follows:

- Distributed synchronization in PHY layer: PDs participating in the synchronization procedure transmit timing reference signal, which is received by neighbouring PDs. Upon receiving timing reference signals, a PD estimates the arrival time of the timing reference signals to adjust its own timing. Eventually, all PDs participating in the synchronization procedure converge to common reference timing.

- Distributed synchronization with time-stamp: PDs participating in the synchronization procedure transmit timing reference signal with time-stamp, which is received by neighbouring PDs. Upon receiving a timing reference signal, a PD estimates the arrival time of the timing reference signal which is used together with the time-stamp to adjust its own timing. Eventually, all PDs participating in the synchronization procedure will share common reference timing.

- Initiator based synchronization: A PD initiates synchronization by start to transmit timing reference signal. Neighbouring PDs adjust their own timing to the timing of the PD that initiated the synchronization. The PD that initiates the synchronization is elected by a group of PDs.

## Discovery procedure

Editor: Shannon started an e-mail thread to discuss discovery procedure.

Shannon, please provided a high level description of the discovery procedure based on the discussion. Everybody else, please participate in the discussion if you want to be heard.

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

## QoS

The MAC sublayer provides the resource allocation features and the data primitives for supporting multiple grades of service to the next higher layer. The grades of service are categorized with the technical attributes specified in the application matrix for IEEE 802.15.8 (15-12-0684-00-008).

## Interference management

Editor: The text below enclosed by <377r0> is general in nature, but needs some improvement.

Shannon?

<377r0 name=”Shannon, Samsung”>

Interference among multiple links is managed by threshold level which is used to identify neighboring PDs. The number of concurrent link goes many as the threshold level goes low. Conversely, the number of concurrent link goes small as the threshold level goes high.

</377r0>

## Transmit power control

Qing, can you provide a high level general text on transmit power control from Distributed D2D point of view to replace the text below?

## Multi-hop operation

To extent the coverage of a PD or group, a PD or group relays received data to the destination PD or group. To support multi-hop operation, a PD or group members maintain a routing table.

## Relative positioning

Editor: Dr. Joo, could you provide an improved text for relative positioning, not specific to any proposal?

## Power management

Editor: Qing?

## Security

Security layer provides users with privacy, authentication, and authorization across the network.

### Security modes

PAC security layer provides three different security modes on the basis of security requirements of network connections.

#### Security mode 1 (non-secure)

When a PAC device is in security mode 1, it shall never initiate any security procedure.

#### Security mode 2 (service level enforced security)

When a PAC device is in security mode 2, it shall initiate security procedures after a channel establishment request has been received or a channel establishment procedure has been initiated by itself. Whether a security procedure is initiated or not depends on the security requirements of the requested channel of service.

A PAC device in security mode 2 should classify the security requirements of its services using the following attributes.

| Security requirement | Description |
| --- | --- |
| Authentication required | - Before connecting to the application, the remote device must be authenticated |
| Authorization required | - Access is only granted automatically to trusted PAC devices, or untrusted devices after an authorization procedure - Always requires authentication to verify that the device is the right one |
| Encryption required | - The link must be changed to encrypted mode, before access to the service is possible |

Security mode 1 can be considered as a special case of security mode 2 where o service has registered any security requirements.

#### Security mode 3 (link level enforced security)

When a PAC device is in security mode 3, it shall initiate security procedures before the channel is established.

### Security parameters

PAC security layer uses security parameters. Some example security parameters are as follows.

| Parameters | Description |
| --- | --- |
| PAC_ADDR | PAC device address (unique for each device) |
| AUTH_KEY | Authentication key used for authentication purposes |
| ENC_KEY | Encryption key for secure unicast |
| GENC_KEY | Group encryption key for secure group communication |
| RAND | Frequently changing random or pseudo-random number |

### Key Derivation

For secure communications between PAC devices in networks, several key materials are derived using the shared secret information between the devices.

Each key material shall be derived selectively on the basis of the security mode.

### Authentication

Authentication is the process of verifying ‘who’ is at the other end of the link. In PAC security layer, authentication is performed for devices, or services.

#### Infrastructureless authentication

PAC networks are fully distributed, and no coordinator is expected to exist to serve as an AAA (Authentication, Authorization, and Accountability) server. Authentication between PDs may be done using secret information shared between devices, or certificates issued by a trusted authority.

Some applications might require only one-way authentication.

Some applications might require mutual authentication.

#### Infrastructure authentication

In PAC networks where there is an AAA(Authentication, authorization, accountability) server and a dynamic coordinator, which is a PAC device with intermittent connection to the AAA server, authentication between PAC devices may be achieved using symmetric master key, or certificate issued by the AAA server.

Figure 24. Infrastructure architecture

#### Authorization

Authorization is the process of deciding if device X is allowed to have access to service Y.

Authorization always includes authentication, and grants access rights to devices on the basis of their trust levels.

| Device trust level | Description |
| --- | --- |
| Trusted device | - The device has been previously authenticated - An authentication key is stored - The device is marked as “trusted” in the device DB |
| Untrusted device | - The device has been previously authenticated - An authentication key is stored - But, the device is not marked as “trusted” in the device DB |
| Unknown device | - No security information is available for this device - This is also an untrusted device |

Trusted devices (authenticated) are allowed to services, but untrusted or unknown devices may require authorization based on interaction before access to services is granted.

Technically, key would be derived or given (established) to the authorized user during the authorization procedure.

Figure 26. Flowchart for authorization

## Coexistence

Editor: The text below enclosed by <395r1> needs to be improved and enhanced.

Volunteers? Jinyoung?

<395r1 name=”Jinyoung Chun, LG, jiny.chunWlge.com >

PAC shall coexistence systems such as IEEE802 using unlicensed bands in section 7.1.1.

</395r1>

## Higher layer interaction

Qing, can you provide a neural high level text to replace the text below?

# Physical layer

## Block Diagram

Editor: The block diagram below illustrates a transmitter with MIMO. However, TG8 has not reached any agreement on whether MIMO will be supported, optional, or mandatory, and thus the following diagram needs to be changed to make it more general. Also, the general block diagram should not mention any specific transmission mode (e.g., OFDM, DFT-S OFDM). Please comment.

—Schematic diagram of common mode PHY

Note: Diagram is so specific. Modify it more generally. Describe each block. Remove Buffer. MIMO will be discussed. Add schematic diagram for Sub 1GHz band and UWB band.

## Channelization

### Sub 1GHz band

Editor: The text below, enclosed by <Park> and <707r2> needs to be revised to make it more general. Current text proposes specific channelization for each frequency band. But the TG has never discussed what the BW should be.

<Park name=”TJ Park, ETRI, tjpark@etri.re.kr”>

- Proposed channelization

| Band | Central freq (MHz) | n | No of channels | Max Tx power (mW) |
| --- | --- | --- | --- | --- |
| Japan-A | fc=917+n | 0,1,…,10 | 11 channels of 1 MHz | 1 |
| Japan-B | fc=922+n | 0,1,…,5 | 6 channels of 1 MHz | 20 |
| Japan-C | fc=921.5+n | 0,1 | 2 channels of 1 MHz | 250 |
| Japan-D | fc=928.7+n | 0,1 | 2 channels of 500 kHz | 1 |
| Korea-A | fc=917.1+nⅹ0.2 | 1, 3, 4, 6, 7, 9, 10, 12, 13, 15, 16, 18 | 12 channels of 200 kHz | 3 mW |
| Korea-B | fc=917.1+nⅹ0.2 | 2, 5, 8, 11, 14, 17, 19, …, 32 | 20 channels of 200 kHz | 10 mW |

</Park>

<707r2 name=”Marco, NICT”>

Frequency bands of operation are: Sub-GHz, 2.4 GHz and 5.7 GHz bands.

### Channelization of 5.7 GHz band

Such frequency band ranges from 5.725 GHz to 5.875 GHz, which is divided into 14 channels of 10 MHz. By regulation, the maximum transmit power at the input antenna is 1 W. The central frequencies are given by

fc = 5735 MHz + 10n       for n = 0, 1, ..., 13

### Channelization of 2.4 GHz band

Such frequency band ranges from 2.4 GHz to 2.5 GHz, which is divided into 9 channels of 10 MHz. By regulation, the maximum transmit power at the input antenna is 1 W. The central frequencies are given by

fc = 2410 MHz + 10n       for n = 0, 1, ..., 8

### Channelization of sub-GHz band

The basic channelization by regulations in Japan is summarized in the following table:

| Band | Max Tx power (mW) | Frequency band (MHz) | Basic channelization |
| --- | --- | --- | --- |
| A1 | 1 | 915.9 – 928.1 | 61 channels of 200 KHz |
| B1 | 20 | 920.5 – 928.1 | 38 channels of 200 KHz |
| C1 | 250 | 920.5 – 923.5 | 15 channels of 200 KHz |
| D2 | 1 | 928.1 – 929.7 | 16 channels of 100 KHz |

1bandwidth rule tolerance: 200 n  kHz, where n=1,2,3,4,5.

2bandwidth rule tolerance: 100 n  kHz, where n=1,2,3,4,5.

Proposed channelization:

| Band | Central freq (MHz) | n | No of channels | Max Tx power (mW) |
| --- | --- | --- | --- | --- |
| A | fc=917+n | 0,1,…,10 | 11 channels of 1 MHz | 1 |
| B | fc=922+n | 0,1,…,5 | 6 channels of 1 MHz | 20 |
| C | fc=921.5+n | 0,1 | 2 channels of 1 MHz | 250 |
| D | fc=928.7+n | 0,1 | 2 channels of 500 KHz | 1 |

</707r2>

### 6.2.4. UWB band

Editor: The text below enclosed by <382r0> is proposal specific. Since Igor and Billy have been working together to harmonize their proposals, they can provide a new high level text to replace the following. If a new text is not provided, the text below will be included in PFD modified as below.

<382r0 name=”Igor Dotlic, NICT, dotlic@nict.go.jp”>

We are usingPAC uses higher UWB band of 6 – 10.25 GHz as specified in “Technical Guidance Documentfor 802.15.8 Proposals.” We are proposing a single channel in upper UWB band for the system to maximize allowed Tx power level. Channel location and bandwidth are determined by regulation at a given Geo.

</382r0>

## Duplex schemes

Editor: The majority of people support TDD. Marco also wants to have FDD, but there was no support for FDD from other members of TG8. So, If no comment supporting FDD is received, the following is a proposed text for PFD by the editor. The text below, enclosed by <707r2>, <395r1>, and <392r1> will be deleted. Please comment.

PAC uses TDD as a duplex scheme.

<707r2 name=”Marco, NICT”>

Support for TDD and FDD frames.

</707r2>

### TDD

<395r1 name=”Jinyoung Chun, LG, jiny.chunWlge.com >

PAC is TDD system.

</395r1>

<392r1 name=”SK Cho, ETRI, skcho@etri.re.kr”>

PAC should use TDD scheme.

</392r1>

### FDD

## Multiplex schemes

Editor: The majority of people support OFDM or its variant. The following is a proposed text for PFD by editor. The text enclosed by <395r1>, <707r2>, <377r0>, and <392r1> will be deleted.

Please comment.

The multiplexing scheme used by PAC is OFDM. However, any variants of OFDM are also under consideration if they are proved to have clear advantage over the conventional OFDM. The advantage of a certain multiplexing scheme over another may be measured in terms of performance, complexity, power consumption, etc.

For frequency bands with unique characteristics or applications, multiplexing scheme other than OFDM or its variant can be considered. For example, UWB band under 11 GHz or frequency band under 1 GHz may use different multiplexing schemes.

(e.g. CDMA, OFDMA)

PAC should support OFDM.

<395r1 name=”Jinyoung Chun, LG, jiny.chunWlge.com >

PAC multiplexing scheme is OFDM.

</395r1>

<707r2 name=”Marco, NICT”>

Support for time-frequency multiplexing.

</707r2>

<377r0 name=”Shannon, Samsung”>

IEEE802.15.8 PAC should use OFDM to support high efficient control and data transmission.

</377r0>

<392r1 name=”SK Cho, ETRI, skcho@etri.re.kr”>

PAC shall use OFDM scheme.

</392r1>

## Frame PPDU structure

Editor will improve the following text and provide a high level text describing PPDU structure for PFD.

General structure (figure to be added)

- Preamble

Preamble is used to perform AGC, packet detection, timing- and frequency synchronization, and channel estimation. The desired characteristics of a preamble are as follows:

- High timing synchronization performance

- High frequency synchronization performance

- Robustness to frequency offset between the transmitter and receiver

- Good channel estimation performance

- Good mobility support

- Low implementation complexity

The preamble used in PAC shall have a well-balanced combination of the characteristics listed above.

- PHY Header

- MPDU

- Etc.

## PPDU structure

PPDU consists of preamble, physical header, and PSDU. Preamble is sequence(s) for timing offset, frequency offset and channel estimation, etc. Physical header is physical information such as bandwidth, PPDU length, etc.

Editor: The text enclosed by <707r2> is proposal specific and will be deleted.

<707r2 name=”Marco, NICT”>

### Frame structure in FDD mode

One slot contains 7 DTF-S OFDM or OFDM symbols.

### Frame structure in TDD mode

where L1=PD1 transmits transmits and PD2 receives, L2=PD2 transmits and PD1 receives, S-L1=synchronization for L1, RS-L1=reference signals for L1, RA=random access or channel sounding for MIMO or beamforming, GP=guard period.

The GP is computed as GP=(73+1024)Ts=71.42 µsec and satisfies Tp+Tdec+Tsw+Tcomp+Tp=GP, where Tdec=time to detect the last symbol, Tsw=time to switch from Rx to Tx or vice-versa, Tcomp=compensation time to align to GP, and Tp=1 Km/3x108m/s=3.3 µsec (worst case).

The TDD frame configuration is given in the following table:

| TDD configuration | Slot number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 | L1 | S | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | S | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 |
| 1 | L1 | S | L2 | L2 | L2 | L2 | S | L1 | L1 | L1 | L1 | S | L2 | L2 | L2 | L2 | S | L1 | L1 | L1 |
| 2 | L1 | S | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | S | L1 | L1 | L1 | L1 | L1 |
| 3 | L1 | S | L2 | L2 | L2 | L2 | L2 | L2 | S | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 |
| 4 | L1 | S | L2 | L2 | L2 | L2 | S | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 | L1 |
| 5 | L1 | S | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | L2 | S | L1 | L1 | L1 |

### Cyclic prefix

The cyclic prefix is based on the typical RMS delay spread of the considered unlicensed bands:

| Frequency | Scenario | RMS delay spread |
| --- | --- | --- |
| 5.2 GHz | Indoor commercial | 190 nsec |
| 5.2 GHz | Indoor office | 60 nsec |
| 5.2 GHz | Indoor residential | 23 nsec |
| 2.4 GHz | Outdoor | 295 nsec |
| 900 MHz | Indoor | 30.55 nsec |
| 900 MHz | Urban | 1.82 usec |

The CP length is 73Ts=4.75 µsec. For sampling time Ts see clause 6.6.2.1.

### Resource block

A resource block (RB) is a set of time-frequency slots that enables multiplexing in the time and frequency domains.

where 7  DFT-spread OFDM or OFDM symbols (see clause 6.5.2.1),  6 subcarriers, and  170 (2 upper and lower subcarriers are empty).

Transmission bandwidth (BW) is obtained by concatenating RBs as

The proposed bandwidths for a maximum FFT size of M=1024 are given in the table

| BW (MHz) | No of RBs | No subcarriers | FFT size | Sampling rate |
| --- | --- | --- | --- | --- |
| 1 | 12 | 72 | 128 | 1.92 MHz |
| 3 | 33 | 198 | 256 | 3.84 MHz |
| 5 | 56 | 336 | 512 | 7.68 MHz |
| 10 | 111 | 666 | 1024 | 15.36 MHz |
| 15 | 166 | 996 | 1024 | 15.36 MHz |

### Data frame structure

The physical layer protocol data unit (PPDU) is formed by concatenating synchronization header (SHR), Discovery header (DIS), physical layer header (PHR) and physical layer service data unit (PSDU) as illustrated:

Reference signals for demodulation/equalization are embedded in the PSDU.

The MAC protocol data unit (MPDU) is passed to the PHY. Such data is encoded by QC-LDPC codes.

</707r2>

Editor: The text enclosed by <373r1> is proposal specific and will be deleted.

<373r1 name=”BJ Kwak, ETRI, bjkwak@etri.re.kr”>

The transmitted RF signal is generated by modulating the complex baseband signal, which is composed of multifarious fields. The fields are delimited by timing boundaries. The general PHY frame format is shown in Figure 2.

Figure 2 – General PHY frame format

The Preamble field is composed of STF and LTF, and is a part of PPDU that is used for packet detection, AGC, time and frequency synchronization, indication of transmission mode (OFDM or SC), collision detection, and channel estimation. A detailed description of the Preamble field is in 6.2.1

The PHY Header field is used to describe the content in the MPDU as well as the protocol used to transfer it.

The MPDU consists of MAC header, payload, and FCS.

SSF (Self Spatial Filtering) uses a technique called beam jittering. The Beam Jitter field is transmitted at the end of SSF request frame. The presence of Beam Jitter field is indicated in the MAC header in the MPDU, and payload of MPDU includes IE that contains the threshold for the correlation level used in SSF which is defined in 6.2.2.

### Preamble

[Option 1]

The preamble consists of two fields named STF (Short Training Field) and LTF (Long Training Field). Figure 3 is a time-domain illustration of the preamble structure.

Figure 3 – Proposed preamble structure (option 1)

[Option 2]

The preamble consists of two fields named STF (Short Training Field) and LTF (Long Training Field). Figure 4 is a time-domain illustration of the preamble structure.

Figure 4 – Proposed preamble structure (option 2)

#### STF

[Option 1]

The STF is used for AGC (automatic gain control), packet detection, initial timing and frequency synchronization, and implicit transmission mode indication (e.g., OFDM or SC). The length of STF is equivalent to two OFDM symbols, and contains 5 repetitions of a periodic signal. The sign of the 4th period in the STF can be used to indicate the transmission mode.

There are three candidate sequences that can be used to generate the periodic signal used in the STF. The standard shall choose the best sequence considering performance, implementation complexity, and capability. The three candidates are MZC1 (Modified Zadoff-Chu sequence 1), MZC2 (Modified Zadoff-Chu sequence 2), and GSW (Gold Sine Wave), which are described in 6.2.1.3

[Option 2]

The STF is used for AGC (automatic gain control), packet detection, initial timing and frequency synchronization, and implicit transmission mode indication (e.g., OFDM or SC), and also for the first collision detection. The length of STF is equivalent to two OFDM symbols, and contains 5 repetitions of a periodic signal. The sign of the 4th period in the STF can be used to indicate the transmission mode.

There are two candidate sequences that can be used to generate the periodic signal used in the STF. The standard shall choose the best sequence considering performance, implementation complexity, and capability. The two candidates are MZC1 (Modified Zadoff-Chu sequence 1) and MZC2 (Modified Zadoff-Chu sequence 2), which are described in 6.2.1.3.

The last period of the STF is used for 1st stage collision detection. The same collision detection principle described in 6.2.1.2 is used for the STF.

#### LTF

[Option 1]

The LTF consists of two OFDM symbols. The first symbol of LTF is used for refined timing/frequency synchronization and channel estimation.

There are four candidate sequences that can be used to generate the signal used in the first symbol of LTF. The standard shall choose the best sequence considering performance, implementation complexity, and capability. The four candidates are Base Zadoff-Chu sequence, MZC1 (Modified Zadoff-Chu sequence 1), MZC2 (Modified Zadoff-Chu sequence 2), and GSW (Gold Sine Wave) which are described in 6.2.1.3

The second symbol of LTF is used for collision detection. The sub-carriers of the second symbol of LTF are divided into two groups by DC sub-carrier. A PD selects a random sub-carrier in each group of sub-carriers and transmits a busy tone. Figure 5 illustrates an example of the frequency domain structure of the second symbol of LTF.

Figure 5 – The second symbol of LTF for collision detection.

When a PD receives a frame, the PD checks the second symbol of LTF to detect a collision. For example, if one of the groups of sub-carriers contains more than one busy tone, the PD decides a collision occurred and may decide not to process the remainder of the radio frame. When there are P sub-carriers in each group of sub-carriers, the probability of collision detection failure is 1/P2.

[Option 2]

The LTF is two OFDM symbols long, and consists of a long CP followed by two repetitions of a periodic signal, where the combination of the sign of each period of the signal can be used to indicate transmission mode. LTF is used for refined timing/frequency synchronization, collision detection, and channel estimation.

Either Base ZC sequence or MZCs in frequency domain is used for LTF.

The 2nd period of the periodic signal in the LTF is used for 2nd stage collision detection. The collision detection procedure is illustrated in Figure 6.

Figure 6 – Collision detection procedure using preamble sequence

The collision detection is performed in two stages, the 1st stage using the STF and 2nd stage using the LTF. PHY reports collision detection if collision is detected in any of the two stages.

#### Preamble sequences for PAC

In this sub-clause, candidate sequences that can be used to generate the STF and the LTF of the Preamble field of PAC PHY frame.

Base Zadoff-Chu sequence:

A length-P ZC sequence in frequency domain is defined as follows

bu,Pm=e-jπm(m+1)uP

where u is the root index of base ZC sequence, odd P is the sequence length, and the sequence element index is m = 0, 1, …, P-1.

MZC1 (Modified Zadoff-Chu sequence 1):

Modified ZC sequence 1 in frequency domain is obtained by interleaving a base ZC sequence with it negative. Let Su,N[m] be a length-N modified ZC sequence 1, then Su,N[m] can be written as follows

- For odd m,

Su,Nm=bu,Pm-12=bm,1 ≤m< P+1-bm,P+2 ≤ m<N

- For even m,

Su,Nm=-bm,1≤m<P+1bm,P+2≤m<N0,m=0, P+1 (DC sub-carrier)

where N=2P+2.

MZC2 (Modified Zadoff-Chu sequence 2):

Modified ZC sequence 2 in frequency domain is obtained by concatenating a base ZC sequence with its complex conjugate. Let Su,N[m] be a length-N modified ZC sequence 2, then Su,N[m] can be written as follows

Su,Nm=bu,Pm-1,1≤m<P+1bu,P*m-P-2, P+2≤m<N0,m=0, P+1 (DC sub-carrier)

where N=2P+2.

GSW (Gold Sine Wave sequence):

Gold Sine Wave sequence in time domain is obtained by multiplying a Gold sequence with a half sine wave window function. Let su,N[n] be a length-N GSW sequence, then

su,Nn=2(N-2)NWu(n)sinnπN,  0≤n<N,

where

Wun=G1n-1G2n-1+uN,1≤n<P+1Wun=Wun+1-P,P≤n<N1,n=0, N-1

and N=2P+2, G1 and G2 are m-sequences of same order, u is the index of GSW sequence, and [ ]N is a modulo-N operator. The term 2(N-2)/N is applied to maintain the same average power as MZCs.

### Beam Jitter field

SSF request frame includes a Beam Jitter field. Beam Jitter field comprises of a single OFDM symbol. A sequence (TBD) is assigned to the sub-carriers of the OFDM symbol.

When a PD transmits an SSF request frame, the Beam Jitter field is transmitted using beam jittering described in 6.2.2.1.

When a PD is receiving an SSF request frame, the Beam Jitter field is received without channel equalization. On receiving an SSF request frame, a PD calculates the correlation coefficient defined as follows.

ρ=x,rx,x∙r,r

where x is a vector of the known sequence that is assigned to the sub-carriers of the OFDM symbol of Beam Jitter field, and r is a vector the OFDM sub-carriers of the received Beam Jitter field.

The cross-correlation coefficient ρ is compared with the threshold found in the IE received in the SSF request frame. If ρ is larger than the threshold, the PD transmits an SSF response frame to the transmitter of the SSF request frame with the calculated ρ.

Beam jittering can be implemented for single carrier systems with minor modification. The detailed structure of beam jittering for single carrier system is TBD.

#### Beam Jittering.

Beam jittering is an open-loop transmit beamforming technique that uses an array antenna, where each sub-carrier of an OFDM symbol is transmitted with a beam pattern independently selected from a set of K predefined beam patterns.

The predefined beam patterns are designed so that all the beam patterns have an identical array gain in the boresight direction of the array, while the array gains in other directions are random.

The number of predefined beam patterns (K), design of beam patterns, and beam selection pattern are implementation specific, and is outside of the scope of this document.

Figure 7 and Table 2 show plots and array parameters of an example of a set of pre-defined beam patterns, respectively, where K=2. SSF with K=2 shows good performance when the beam patterns are well designed.

| Amplitude of array response | Phase of array response |
| --- | --- |

Figure 7 – An example of pre-defined beam patterns.

Table 2 – An example of array parameters for K pre-defined beam patterns.

| Antenna configuration | 4 antenna ULA |
| --- | --- |
| Antenna spacing | 0.5λ |
| K | 2 |
| Null locations | Beam L: sin-12i-14, i=-1,1,2. |
|  | Beam R: sin-12i-14, i=-1,0,2. |

</373r1>

Editor: The text enclosed by <377r0> is proposal specific and will be deleted.

<377r0 name=”Shannon, Samsung”>

### Discovery frame structure

Discovery frame is comprised of multiple Discovery slots. A Discovery Slot delivers single Peer Discovery Message (PDM).

Figure 5. Discovery Frame Structure

</377r0>

Editor: The text enclosed by <377r0> is proposal specific and will be deleted.

<377r0 name=”Shannon, Samsung”>

### Data frame structure

A Data frame is comprised of one or multiple scheduling subframe and the multiple resource slots (RSs) which are associated to one scheduling subframe.

A RS consists of Preamble Signal duration, Channel Feedback Signal duration, Data Packet duration, and ACK Signal duration.

</377r0>

Editor: The text enclosed by <392r1> is proposal specific and will be deleted.

<392r1 name=”SK Cho, ETRI, skcho@etri.re.kr”>

The ultraframe has a fixed length and appears repeatedly. Ultraframe, superframe, frame has a hierarchical structure.

Figure 13. Hierarchical structure of ultraframe

Every frame consists of a synchronization region, a discovery region, a peering region and a data region. The type of a frame can be the frame type 0 of the frame type 1 according to the configuration. The frame type 0 consists of a synchronization region, a discovery region, a peering region and a data region. Frame type 1 consists of a synchronization region and a data region. The frame type 0 is used if the frame number is 0 and the frame type 1 is used otherwise.

### Synchronization region

The synchronization region is located at the head of a frame. The synchronization signal for the distributed synchronization is sent in the synchronization region. A PD transmits the synchronization signal without interference sensing but the synchronization signal is transmitted by low power for coexistence with heterogeneous devices. The synchronization signal is transmitted repeatedly for reliability. There are two kinds of synchronization signals. One is used at the synchronization region of the first frame in an ultraframe and another is used at other frame.

Figure 14. Synchronization region

### Discovery region

In the discovery region, a discovery signal is transmitted. The discovery region consists of multiple discovery RUs, ISs and GIs. The discovery RU consists of a preamble signal and a discovery signal. The Forward Blocking scheme is used for coexistence. The discovery region consists of 8 blocking unit and one blocking unit consists of 8 discovery RUs. The IS interval is present in front of every blocking unit.

Figure 15. Discovery region

### Peering region

The peering region consists of the Peering-REQ/RSP interval and the PID broadcast interval. The Peering-REQ/ RSP interval consists of multiple PID-REQ RUs, PID-RSP RUs, ISs and GIs. The Peering-REQ RU consists of a preamble signal and a Peering-REQ signal. The Peering-RSP RU consists of a preamble signal and a Peering-RSP signal. Forward & Backward Blocking scheme is applied at Peering-REQ blocking unit, and Forward Blocking scheme is applied at Peering-RSP blocking unit. Four Peering-REQ blocking units and four Peering-RSP blocking units exist in the Peering-REQ/RSP interval. A Peering-REQ blocking unit has four Peering-REQ RUs and a Peering-RSP blocking unit has four Peering-RSP RUs.

In the PID broadcast interval, multiple PID broadcast RUs mapped to PID exist. Forward Blocking scheme is used.

Figure 16. Peering region

### Data region

The data region is comprised of multiple data channels with a fixed size. In case of frame type 0, the data region has 13 data channels because 3 data channels (0~2) are used for the control such as Discovery and Peering. In case of frame type 1, data region has 16 data channels.

A data channel consists of a scheduling interval and a data interval. The scheduling interval consists of DS-REQ RUs, DS-RSP RUs, IS, GIs, and SRI. In case of the DS-REQ RU and the DS-RSP RU, a preamble signal is followed by a DS-REQ signal and a DS-RSP signal, respectively. Forward and Backward Blocking schemes are utilized in both DS-REQ and DS-RSP blocking units. The data interval consists of GIs and multiple pairs of data burst and ACK. The data burst consists of a preamble signal, a BCI (Burst Control Indicator) signal, and a data signal. The BCI signal is used to indicate the modulation order, the code rate, and the length of data signal. ACK consists of a preamble signal and a ACK signal. The number of data bursts and each burst size are determined by the distributed scheduling.

Figure 17. Data region

</392r1>

### Discovery frame structure

Editor: The text enclosed by <384r0> is proposal specific and will be deleted.

<384r0 name=”TJ Park, ETRI, tjpark@etri.re.kr”>

The LESD Mode PPDU shall be formatted as illustrated.

|  |  | Octets |  |
| --- | --- | --- | --- |
|  |  | 1 | Variable |
| Preamble | SFD | PHY Header | PHY Payload |
| SHR |  | PHR | PSDU |

#### Preamble field

The Preamble field shall contain multiples of the 8-bit sequence “01010101”.

#### SFD

The SFD shall be the selected 16-bit sequence selected from the list of values shown in the following Table.

| Message type | SFD value for uncoded (PHR+PSDU) | SFD value for coded (PHR+PSDU) |
| --- | --- | --- |
| Request | 1011 0001 1001 1100 | 1011 0001 1011 0001 |
| Response | 0100 1110 1001 1100 | 0100 1110 0100 1110 |
| Notification | 0110 0011 1001 1100 | 0110 0011 0110 0011 |

Devices that support FEC shall support the SFD associated with coded (PHR + PSDU) and Devices that do not support FEC shall support the SFD associated with uncoded (PHR + PSDU).

#### PHR

The format of the PHR is shown in the following Table.

If the frame type is not “preamble”, the Frame Length field (L5–L0) specifies the total number of octets contained in the PSDU (prior to FEC encoding, if enabled). Otherwise, The field (L5–L0) represents the remaining time of repeated preamble set.

| Bit string index | 0–1 | 2–7 |
| --- | --- | --- |
| Bit mapping | T1–T0 | L5–L0 |
| Field name | Frame Type | Frame Length / Remaining time |

| (T1 T0) | Frame Type |
| --- | --- |
| 0 0 | Request |
| 0 1 | Response |
| 1 0 | Notification |
| 1 1 | Preamble(No PSDU field) |

#### PSDU field

The PSDU field carries the data of the PPDU.

</384r0>

## Modulation and coding scheme (MCS)

Note: NICT’s proposal is so detail for PFD. Members will discuss depth of text for PFD.

### Modulation

Modulation schemes under consideration for PAC include BPSK, QPSK, 16QAM, 64QAM, GFSK, OOK, and 2PPM BPSK. are included in proposals.

64QAM may be considered.

#### Sub 1 GHz band

Modulation schemes under consideration for the frequency band under 1 GHz are GFSK, OFDM, and Filtered FSK.

#### 2.4 GHz and 5 GHz band

Modulation schemes under consideration for 2.4 GHz and 5 GHz unlicensed bands are BPSK, QPSK, 16QAM, 64QAM.

#### UWB band

Modulation schemes under consideration for UWB band are OOK, 2PPM BPSK.

Note: Harmonized text will be provided.

Editor: The text enclosed by <707r2> is proposal specific, and describes specifications of modulation schemes rather than a framework. To be deleted.

<707r2 name=”Marco, NICT”>

In the modulation mapper, the scrambled coded bits bi  for i=0,1,…,n-1 are modulated with either BSPK, QPSK or 16QAM modulations, resulting in the a block of complex modulation symbols di for i=0,1,…,Nsym-1, where di=I+jQ.

The mapping between bits onto symbols are given by

BPSK mapping:

| bi | I | Q |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |

QPSK mapping:

| bi bi+1 | I | Q |
| --- | --- | --- |
| 00 |  |  |
| 01 |  |  |
| 10 |  |  |
| 11 |  |  |

16QAM mapping:

| bi bi+1 bi+2 bi+3 | I | Q |
| --- | --- | --- |
| 0000 |  |  |
| 0001 |  |  |
| 0010 |  |  |
| 0011 |  |  |
| 0100 |  |  |
| 0101 |  |  |
| 0110 |  |  |
| 0111 |  |  |
| 1000 |  |  |
| 1001 |  |  |
| 1010 |  |  |
| 1011 |  |  |
| 1100 |  |  |
| 1101 |  |  |
| 1110 |  |  |
| 1111 |  |  |

#### DFT-S OFDM or OFDM

The Parameters are given by

| Description | Notation |
| --- | --- |
| Total No. of subcarriers | M |
| Transmission bandwidth | BW |
| No. of used subcarriers | N |
| Subcarrier spacing | ∆f=BW/N |
| Sampling time | Ts |
| Clock rate | Rc |
| Frame time | Tframe |
| Slot time | Tslot |

∆f is constant and equal to 15 KHz.

Maximum FFT size M=1024.

Sampling time

Timing based on a common clock at rate

#### Optional GFSK modulation

An optional and very low power PHY based on CP-2FSK modulation is contemplated for the sub-GHz band with no support for MIMO technologies, i.e., layer mapper and precoding are not necessary. The proposed channel encoder, bit interleaver and scrambler are used as well. The modulation mapper is CP-2FSK that is given by

where V is amplitude, S(t)=sin(2πfc t) is the modulating-carrier signal, fc is  the central carrier frequency, Tsym is the symbol time, β=1 is the modulation index, ∆f=β/2Tsym is the peak frequency deviation, and φ0 is the initial phase of the modulating-carrier signal.

The information bearing signal is given by

where gm is information bits, p(t) is a Gaussian pulse shape of bandwidth-symbol duration product of 0.8.

</707r2>

### Coding Scheme

PAC shall use channel coding to protect messages against channel noise or interference from other devices.

Note: NICT will simplify this text for coding scheme.

Note: Another coding scheme may be provided in response to call for contribution. This will be addressed in January meeting.

Editor: The text enclosed by <707r2> will be deleted.

<707r2 name=”Marco, NICT”>

Channel coding is based on quasi-cyclic low density parity check codes (QC-LDPC).

Quasi-cyclic LDPC codes are systematic, linear codes satisfying HcT=0, where c is the codeword and H is the parity check matrix.

H is constructed from the prototype matrix Hp of size MpxNp by replacing each entry of such matrix [Hp]i,j with either a cyclic shift matrix Pc, or identity matrix or null matrices of size ZxZ. The final size of H is MpZxNpZ.

Details of the encoding process can be found in document with DCN 369r1.

QC-LDPC parameters for different coding rates to enable link adaptation are given by

| Coding rate | k | n |
| --- | --- | --- |
| 1/2 | 972 | 1944 |
| 1/2 | 324 | 648 |
| 2/3 | 1296 | 1944 |
| 2/3 | 432 | 648 |
| 3/4 | 1458 | 1944 |
| 3/4 | 486 | 648 |
| 5/6 | 1620 | 1944 |
| 5/6 | 540 | 648 |

The prototype matrices Hp for different coding rates can be found in document with DCN 369r1.

</707r2>

Editor: The text enclosed by <686r0> will be deleted.

<686r0 name=”Igor Dotlic, NICT, dotlic@nict.go.jp”>

### Pulse shape and duration

We do not define a specific pulse shape in order to allow different low-complexity pulse generators.

Furthermore, operating bands are not defined will be different at different Geos.

Pulse shape is constrained

In spectrum by the local regulations.

In duration by the Duty Cycle (DC) of  no more than DC=1/32=3.1%.

### Packet structure

SHR

PHR

PPDU

Fig. 1: Packet structure.

As Fig, 1. shows, packet consists of SHR – Synchronization Header, PHR – PHY Header and PPDU – Physical Layer Protocol Data Unit.

### SHR Structure

Preamble

SFD

Fig. 2. SHR structure.

Preamble consists of M=8 times repetition of the sequence Si. Si is one of the Gold sequences of length 31. Since set of Gold sequences of length 31 has 33 sequences, there are 33 virtual channels available per physical channel. Gold sequences of length 31 have relatively short length with good circular autocorrelation properties.

Synchronization Frame Delimiter (SFD) represents inversion of Si used in the preamble.

### Symbol structure

Fig. 3. Symbol structure

Symbol structure is shown in Fig. 3. The whole packet, is transmitted using the same symbol structure regardless of the data rate used with On-Off Keying (OOK) modulation. For , where  is the symbol index, the same symbol duration of  is used. Transmitted pulse waveform is denoted . Hence, transmitted signal for the -th transmitted symbol is

(1)

where  is transmitted bit and  is pseudo-random sequence used in  to flatten the spectrum of the transmitted signal, while for   is set. Pseudo-random sequence used is the same Gold sequence used in PR and SFD for .

</686r0>

### Data rates

Editor: The text enclosed by <686r0> will be deleted.

<686r0 name=”Igor Dotlic, NICT, dotlic@nict.go.jp”>

### Channel coding and data rates

RS coder

Bit interleaver

Conv. coder

Modulator

Fig. 4.: Concatenated coding scheme used.

Coding is concatenation of outer Reed-Solomon RS6(63,55) codes and inner convolutional codes. Different data rates have different convolutional coding as in Table 1.

| Data rate (Kbps) | 54.56 | 109.12 | 218.25 | 436.51 | 873.02 |
| --- | --- | --- | --- | --- | --- |
| Conv. coding rate | 1/4 | 1/4 | 1/4 | 1/2 | 1/1 |
| Chips per symbol | 4 | 2 | 1 | 1 | 1 |

Table 1.: Data rates used with convolutional coding rates and number of chips per symbol.

Table 2. provides detailed specification of the convolutional codes used. Bit interleaver used is algebraic bit interleaver specified in the UWB PHY of the IEEE 802.15.6-2012 standard for Body Area Networks.

| Conv. coding rate | ½ | ¼ |
| --- | --- | --- |
| Constrained length | 3 | 4 |
| Generators in octal | 5 7 | 13 15 15 17 |
| Free distance | 8 | 13 |

Table 2.: Convolutional codes used.

### Symmetrical double-sided two-way ranging

Tround1

Tprop

Treply1

t

Tprop

Tprop

t

Tround2

Treply2

Device 1

Device 2

Tprop

Fig. 4. Principle of symmetrical double-sided two-way ranging.

As Fig. 4. shows, symmetrical double-sided two-way ranging is achieved by measuring two round trip times: round trip time from device 1 to device 2 denoted  and round trip time from device 2 to device 1 denoted .

denotes propagation time that is being estimated, while  and  represent reply times of device 1 and 2 respectively. Hence,  estimate is calculated as follows

.	(2)

If relative errors of timing at device 1 and 2 are denoted  and  it follows that

(3)

(4)

(5)

(6)

Thus error of  estimation is:

(7)

Hence, relative error of the range estimation is also in the same order of magnitude as  and , which for crystal oscillators is usually around 20 ppm.

</686r0>

### Discovery mode

Editor: The text enclosed by <384r0> will be deleted.

<384r0 name=”TJ Park, ETRI, tjpark@etri.re.kr”>

The modulation for the LESD Mode is a 2-level filtered FSK.

The following Table shows the modulation and channel parameters for the LESD Mode PHY.

| Frequency Band (MHz) | Modulation | Modulation index | Channel spacing (kHz) | Data rate (kb/s) |
| --- | --- | --- | --- | --- |
| Sub-GHz | Filtered 2FSK | 1 | 200 | 50 |

a Data rates shown are over-the-air data rates (the data rate transmitted over the air regardless of whether the FEC is enabled).

</384r0>

## Multiple antennas

PAC supports multiple antenna technologies such as MIMO or beamforming to improve performance or provide new functionalities. For example, MIMO technologies can be used to increase data rate or reduce packet error rate. Beamforming can be used to increase the SNR of the received signal, extend the coverage, or provide spatial filtering capability. PAC may also support other array processing technologies to estimate the angle of arrival of incident signals to provide location based services.

Editor: The text enclosed by <707r2> will be deleted.

<707r2 name=”Marco, NICT”>

### Layer mapping

Two MIMO technologies are supported: open loop spatial multiplexing and transmit diversity space- frequency block codes (SFBC) for 2 and 4 antennas.

The [complex] modulation symbols per codeword di for i=0,1,…,Nsym-1 are mapped onto several layers as (a layer represents an independent stream of symbols in a MIMO configuration, and rank is the number of layers transmitted):

where ν is the number of layers and NLsym is the number of symbols per layer for the qth codeword.

#### Open loop spatial multiplexing

Open loop spatial multiplexing enables the transmission of parallel data streams. The mapping of codeword symbols onto layer streams can be found in document with DCN 369r1.

#### Transmit diversity

Transmit diversity enables to transmit the same information from multiple antennas. The mapping of codeword symbols onto layer streams can be found in document with DCN 369r1.

### Precoding

Precoding allows increasing system performance and robustness by feeding back to the transmitter channel state information (CSI). The schematic diagram of MIMO support with precoding is illustrated as

#### Open loop spatial multiplexing

Precoding for open loop spatial multiplexing increases robustness by feeding back the rank of the wireless channel (RI=rank indicator). The transmitter chooses a pre-fixed codeword according to the RI.

Single antenna mapping is trivial and given by

where pɛ{0,1,2,3}, i=0,1,…,NPsym-1, and NPsym=NLsym.

Multiple antennas mapping is given by

The transmitter chooses a codeword according to reported RI=ν. The codebook for 2 and 4 antennas can be found in document with DCN 369r1.

#### Transmit diversity

Transmit diversity is aimed to increase robustness in scenarios with low SNR, low delay tolerance or no feedback to the transmitter is available or reliable.

In case of 2 antennas:

  - STBC (space-time block code) for DTF-Spread OFDM.

  - SFBC (space-frequency block code) for OFDM.

SFBC for 2 antennas is given by

In case of 4 antennas there is a combination of SFBC (2 antennas) with frequency switch transmission diversity and given by

</707r2>

## Bit interleaver

In general, wireless channels are not memoryless, and thus bit errors usually occur in bursts rather than independently. Interference from other systems is another source of bit errors in bursts. However, modern channel coding schemes usually assume the bit errors are distributed independently. PAC supports bit interleaving to improve the performance of forward error correcting code.

Editor: The text enclosed by <707r2> will be deleted.

<707r2 name=”Marco, NICT”>

In order to minimize latency and integration on parallel architectures within the encoder/decoder implementation in a chip, an algebraic interleaver is proposed.

A maximum contention-free quadratic permutation interleaver is defined as

Short length interleaver:

Long length interleaver:

where i=0,1,…,NI-1. NI is the length of interleaver. Elements of codewords (ci for i=0,1,…,n-1) are interleaved in blocks of NI bits as c∏(j) for j=0,1,…,NI-1.

</707r2>

## Scrambling

PAC shall have a scrambler, or a randomizer, which is used to shape the data spectrum and to reduce interference.

Editor: The text enclosed by <707r2> will be deleted.

<707r2 name=”Marco, NICT”>

A scrambler is used to shape the data spectrum and to randomize data across users in order to reduce interference. A Gold code generator of length 63 is proposed as scrambler.

Such PN sequence with period 263 appears truly random even for long packets. Moreover, different initialization seeds enable a different Gold code per user with low correlation respect to other user using a different seed.

The Gold code generator outputs si for i=0,1,…,263-1, which is used to scramble the interleaved codeword bits ci for i=0,1,…,n-1 prior to the  modulation mapper as

The shift registers initialization at the start of a packet is given by

  - User ID (1st register) and group ID (2nd register).

  - Fast forward both shift registers 100 times to reduce PAPR.

</707r2>

Editor: The text enclosed by <707r2> is proposal specific, and will be deleted.

<707r2 name=”Marco, NICT”>

## Discovery

The discovery preamble (DP) is based on a ZC sequence and a discovery resource block (DRB) from a modified DTF-S OFDM or OFDM signal. The DP and DRB formed the Discovery Signal (DS).

Moreover, we propose to use one channel (from the proposed channelization for sub-GHz, 2.4 GHz and 5.7 GHz bands) for only discovery of devices.

PAC devices can either transmit or receive the DS in this unique channel asynchronously. Such unique channel for discovery is named Shared Discovery Channel (SDCH). The discovery signal (DS) sent over a discovery shared channel (DSCH) is illustrated as

The S consists of a preamble sequence plus a DRB formed by Nfs frequency slots and Nts time slots. Once synchronized, a receiver knows the location of the discovery resource block (DRS) to scan for possible peers or to pick time-frequency slots to transmit its DS.

The proposed ZC sequence for initial synchronization during discovery (or communication mode) is a ZC sequence with the following parameters: Sequence length N = 63, relative prime r = 62 and q = 0.

Such ZC sequence is mapped onto 62 sub-carriers. The first subcarrier and the DC subcarrier are empty. Of course, several repetitions of the DP may be transmitted to improve reliability.

The discovery process is energy intensive. In order to minimize power consumption only one subcarrier of the N-point IFFT is used per user. Consequently, the PAPR is set to the minimum value. Furthermore, across the frequency domain, users are orthogonal (OFDM).

For discovery, the nth symbol transmitted over the kth subcarrier is given by

where l=L+1,…,0,1,…,N-1 and L is the cyclic prefix length.

Contiguous subcarriers are orthogonal as illustrated in the figure:

The DS is transmitted with a predefined duty-cycle (see NICT MAC proposal). From upper layers, terminals pick time-frequency slots in the DRB to transmit the DS.

A set of 126 symbols are transmitted per time slot. Such frame contains information about a given peer (peer ID, group ID, service ID, application ID, etc.) and which channel is used for association or peering (different from the SDCH).

The number of frequency slots Nfs =1024 (IFFT size) and the number of time slots Nts is chosen as 20. Consequently, the DRB can support up to NfsxNts=20,480 users for discovery per group.

The discovery data is formatted as illustrated in the figure:

Append 57 bits for user ID, device ID, Group ID, etc.

Append 6 bits from CRC-6-ITU error detection code.

Append 63 bits from shorten BCH(126,63) code.

### Discovery procedure

Devices are in three possible states:

  - RDM  – receiving discovery mode,

  - TDM  – transmitting discovery mode,

  - Sleep – idle.

Also, devices are equipped with clear channel assessment (CCA).

1) At start up or after idle or after command, a device enters into receiving discovery mode (RDM) and listens for a DS.

The synchronization subsystem detects the preamble (and hence the position of the DRB) and the detection subsystem scans the DRB for DRB usage and detection of peer ID, service ID, etc. Go to Sleep mode.

2) After command, a device enters into transmitting discovery mode (TDM).

Device chooses a free time-frequency slot to transmit its DS over the next DRB transmission. Go to Sleep mode.

Of course, there are intermediate steps like time-out for DRB scanning, miss detection, etc. Here, we present the core algorithm idea only.

## Peering

For association or peering, intended devices that want to establish a communication link, request peering through a random access preamble transmitted in the ACH.

The random access association preambles are named Random Access Preambles (RAPs).

Such RAPs are formed with ZC sequences as well. Hence, a pool of orthogonal RAPs is formed in order to reduce interference from competing terminals for random access.

A unique RAP is assigned to every device in a Group. Such unique RAP is used for fine synchronization and control messages that control how a communication link is granted.

The RAP signal is illustrated as

where GI is guard interval.

Thus, a set of orthogonal preamble sequences of length 1024 can be generated from ZC sequences for random access, which satisfy the maximum round-trip time and coverage performance for a maximum distance of 500m. Details of preamble design can be found in document with DCN 369r1.

The CP and GI lengths are given by

### Peering procedure

Once initial synchronization and detection of discovery RB are achieved by Terminal 2 over Terminal 1, for instance, the peering procedure flow is as follows:

Terminal 2 requests association by a random access procedure based on an orthogonal RAP (the process is initiated and control by the MAC and possibly upper layers):

1) Terminal 2 sends a RAP over the ACH.

The RAP is randomly selected from a pool of orthogonal ZC sequences. It contains finer frequency granularity for Terminal 1 to acquire fine time and frequency synchronization of Terminal 2, plus information about the resources needed to transmit in step 3).

2) Terminal 1 replies with a RA response message.

It contains timing information (round-trip delay), RAP-ID, RB element grant to transmit in step 3), plus Group identifier, etc.

3) Scheduling request

It contains scheduling request information for transmission. If this message is successfully detected in Terminal 1, still contention remains unsolved for other terminals. A quick resolution is needed.

4) Contention resolution

Terminal 1 echoes Terminal 2 ID contained in 3), so that one of the following statements is true:

Terminal 2 detects its ID and sends ACK (RA terminated) a communication link is scheduled and established.

Terminal 2 detects another ID (RA terminated and restarts a new one).

Terminal 2 fails to detect ID (RA terminated and starts a new one).

</707r2>

Editor: The text enclosed by <707r2> is proposal specific and will be deleted.

<707r2 name=”Marco, NICT”>

## Reference signals

Reference signals are required to perform channel estimation in time and frequency domain for detection and equalization.

### Channel time dispersion

Considering a maximum speed of v=100 Km/h (27.78 m/s).

The Doppler spread is given by fd=fc v/c, where fc is the carrier’s central frequency.

The minimum sampling time to reconstruct the channel is Tc=1/2fd.

| Freq. band | fd (Hz) | Tc (msec) |
| --- | --- | --- |
| 5.7 GHz | 527.82 | 0.947 |
| 2.4 GHz | 222.24 | 2.2 |
| 920 MHz | 85.2 | 6 |

The DFT-spread OFDM or OFDM frame structure has a slot time of Tslot=0.5 msec. Then, one reference symbol per slot is needed in the time domain to estimate the channel correctly.

### Channel frequency dispersion

Considering 90% and 50% coherence bandwidth as BC,90=1/50 στ  and  BC,50=1/5 στ ,

where στ is the channel’s  RMS delay spread.

Such RMS delay spread is estimated as . For a distance d=500m, the 90% and 50% coherence bandwidth are given in the table:

| Freq. band | Ca | γa | στ | Bc,90 (KHz) | BC,50 (KHz) |
| --- | --- | --- | --- | --- | --- |
| 5.7 GHz | 10 | 0.51 | 238 nsec | 84 | 840 |
| 2.4 GHz | 55 | 0.27 | 295 nsec | 67 | 678 |
| 920 MHz | 1254.3 | 0.06 | 1.82 usec | 11 | 110 |

If BC,50<BW, then the wireless channel is frequency selective fading and frequency domain equalization is required. We propose that the spacing between 2 reference symbols in the frequency domain in a RB is 30 KHz to resolve the channel’s frequency dispersion.

### ZC sequences

Preambles or beacons for synchronization and random access are formed with Zadoff-Chu (ZC) sequences of length N.

ZC sequences are constant-amplitude zero-correlation (CAZAC) sequences given by

where  is a primitive nth root of unity, r is a relative prime to N, k is the sequence index such that  k = 0, 1, ...,N-1and q is any integer.

ZC sequences have constant amplitude and so its N-point DFT.  Hence, this limits the PAPR and simplifies implementation as only phases have to be generated and stored. Furthermore, ZC sequences have ideal cyclic autocorrelation, i.e., the correlation with its shifted version is a delta function. Consequently, a [large] set of orthogonal preambles are possible to generate.

</707r2>

Editor: As Marco noted, the text enclosed by <Billy> has been published already and is publicly available. A reference to the previously published document can be provided and the following text can be deleted, and any change from the existing text can be mentioned if there is any.

Please comment.

<Billy name=”Billy Verso, DecaWave, billy.verso@decawave.com”>

# UWB Physical (PHY) layer specification

## General

The UWB PHY employs a mean PRF that is nominally 16 MHz or optionally nominally 64 MHz.  The UWB PHY waveform is based upon an impulse radio signaling scheme using band-limited data pulses. The UWB PHY supports two independent bands of operation:

- The low band, which consists of four channels and occupies the spectrum from 3.1 GHz to 4.8 GHz

- The high band, which consists of eleven channels and occupies the spectrum from 6.0 GHz to 10.6 GHz

Within each channel, there is support for at least two complex channels that have unique length 31 SHR preamble codes. The combination of a channel and a preamble code is termed a complex channel.

A combination of burst position modulation (BPM) and binary phase-shift keying (BPSK) is used to support both coherent and non-coherent receivers using a common signaling scheme. The combined BPM-BPSK is used to modulate the symbols, with each symbol being composed of an active burst of UWB pulses. The various data rates are supported through the use of variable-length bursts.

Figure 1 shows the sequence of processing steps used to create and modulate a packet. The sequence of steps indicated here for the transmitter is used as a basis for explaining the creation of the UWB waveform. Note that the receiver portion of Figure 1 is informative and meant only as a guide to the essential steps that any compliant UWB receiver needs to implement in order to successfully decode the transmitted signal.

Figure 1 - signal flow

## PPDU format

Figure 2 shows the format for the UWB frame, which is composed of three major components: the SHR preamble, the PHR, and the PSDU. For convenience, the PPDU packet structure is presented so that the leftmost field as written in this standard shall be transmitted or received first. All multiple octet fields shall be transmitted or received least significant octet first, and each octet shall be transmitted or received LSB first. The same transmission order should apply to data fields.

The SHR preamble is first, followed by the PHR, and finally the PSDU.  The SHR preamble is always sent at the base rate for the preamble code.  The PHR is sent at a nominal rate of 850 kb/s for all data rates above 850 kb/s and at a nominal of 110 kb/s for the nominal data rate of 110 kb/s. The PSDU is sent at the desired information data rate as defined in Table 3 – rate-dependent and timing dependent parameters.

The PSDU contains MAC layer messages.

### PPDU encoding process

The encoding process is composed of many steps as illustrated in Figure 2.  The details of these steps are fully described in later sub-clauses, as noted in the following list, which is intended to facilitate an understanding of those details:

- Perform Reed-Solomon encoding on PSDU as described in 2.3.3.1.

- Produce the PHR as described in 2.2.6.1.

- Add SECDED check bits to PHR as described in 2.2.6.2 and prepend to the PSDU.

- Perform further convolutional coding as described in 2.3.3.2. Note that in some instances at the 27 Mb/s data rate, the convolutional encoding of the data field is effectively bypassed and two data bits are encoded per BPM-BPSK symbol.

- Modulate and spread PSDU according to the method described in 2.3.1 and 2.3.2. The PHR is modulated using BPM-BPSK at 850 kb/s or at 110 kb/s (for the 110 kb/s data rate) and the data field is modulated at the rate specified in the PHR.

- Produce the SHR preamble field from the SYNC field (used for AGC convergence, diversity selection, timing acquisition, and coarse frequency acquisition) and the SFD field (used to indicate the start of frame). The SYNC and SFD fields are described in 2.2.5.1 and 2.2.5.2, respectively.

Figure 2 – PPDU encoding process

Table 1 and Table 2 show how the 19 header bits (H0-H18), N data bits (D0-DN-1), and two tail bits (T0-T1) are mapped onto the symbols. In these tables, the polarity bit column operation is an XOR. The tables also show when the transition from the header bit rate to the data bit rate takes place. Note that the delay line of the convolutional code is initialized to zero. For this reason, the position bit of Symbol 0 shall always be zero. This means that Symbol 0 is always transmitted in the first half of the first header symbol.

Table 1 – mapping of header bits, data bits and tail bits onto symbols with Viterbi rate 0.5

### Symbol structure

In the BPM-BPSK modulation scheme, a symbol is capable of carrying two bits of information: one bit is used to determine the position of a burst of pulses while an additional bit is used to modulate the phase (polarity) of this same burst.

The structure and timing of a symbol is illustrated in Figure 3. Each symbol shall consist of an integer number of possible chip positions, Nc, each with duration Tc. The overall symbol duration denoted by Tdsym is given by Tdsym = NcTc. Furthermore, each symbol is divided into two BPM intervals each with duration TBPM =Tdsym /2, which enables binary position modulation.

A burst is formed by grouping Ncpb consecutive chips and has duration Tburst = NcpbTc. The location of the burst in either the first half or second half of the symbol indicates one bit of information. Additionally, the phase of the burst (either –1 or +1) is used to indicate a second bit of information.

In each symbol interval, a single burst event shall be transmitted. The fact that burst duration is typically much shorter than the BPM duration, i.e., Tburst << TBPM, provides for some multi-user access interference rejection in the form of time hopping. The total number of burst durations per symbol, Nburst, is given by Nburst = Tdsym /Tburst. In order to limit the amount of inter-symbol interference caused by multipath, only the first half of each TBPM period shall contain a burst. Therefore, only the first Nhop= Nburst/4 possible burst positions are candidate hopping burst positions within each BPM interval. Each burst position can be varied on a symbol-to-symbol basis according to a time hopping code as described in 2.3.

Table 2 –mapping of header bits, data bits and tail bits onto symbols with Viterbi rate 1

### PSDU timing parameters

The PSDU rate-dependent parameters and timing-related parameters are summarized in Table 3. Within each channel {0:15}, the peak PRF shall be 499.2 MHz. This rate corresponds to the highest frequency at which a compliant transmitter shall emit pulses. Additionally, the mean PRF is defined as the total number of pulses emitted during a symbol period divided by the length of the symbol duration. During the SHR preamble portion of a UWB frame, the peak and mean PRFs are essentially the same since pulses are emitted uniformly during each preamble symbol. During the data portion of a PPDU, however, the peak and mean PRFs differ due to the grouping of pulses into consecutive chip durations.

There are two possible preamble code lengths (31 or 127) and two mean PRFs (15.6 MHz or 62.4 MHz). A compliant device shall implement support for the preamble code length of 31 and the 15.6 MHz mean PRFs for the PSDU as depicted in Table 3. The use of the length 127 code is optional; when implemented, the mean PRF of the PSDU shall be 62.4 MHz.

UWB channels {4, 7, 11, and 15} are all optional channels and are differentiated from other UWB channels by the larger bandwidth (> 500 MHz) of the transmitted signals. These channels overlap the lower bandwidth channels. The larger bandwidth enables devices operating in these channels to transmit at a higher power (for fixed PSD constraints), and thus they may achieve longer communication range. The larger bandwidth pulses offer enhanced multipath resistance. Additionally, larger bandwidth leads to more accurate range estimates. The admissible data rates, preamble code lengths, PRFs, and modulation timing parameters are listed in Table 3.  Each UWB channel allows for several data rates that are obtained by modifying the number of chips within a burst while the total number of possible burst positions remains constant. Therefore, the symbol duration, Tdsym, changes to obtain the stated symbol rate and bit rates.

Figure 3 – symbol structure

Table 3 – rate-dependent and timing dependent parameters

| Preamble Code Length | Modulation & Coding |  |  | Data Symbol Structure |  |  |  |  |  | Data |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Viterbi Rate | RS Rate | Overall FEC Rate | #Burst Positions per Symbol Nburst | # Hop Bursts Nhop | # Chips Per Burst Ncpb | #Chips Per Symbol | Burst Duration T burst (ns) | Symbol Duration Tdsym (ns) | Symbol Rate (MHz) | Bit Rate Mb/s | Mean PRF (MHz) |
| 31 31 31 31 | 0.5 0.5 0.5 1.0 | 0.87 0.87 0.87 0.87 | 0.44 0.44 0.44 0.87 | 32 32 32 32 | 8 8 8 8 | 128 16 2 1 | 4096 512 64 32 | 256.41 32.05 4.01 2.00 | 8205.13 1025.64 128.21 64.10 | 0.12 0.98 7.80 15.60 | 0.11 0.85 6.81 27.24 | 15.60 15.60 15.60 15.60 |
| 127 127 127 127 | 0.5 0.5 0.5 0.5 | 0.87 0.87 0.87 0.87 | 0.44 0.44 0.44 0.44 | 8 8 8 8 | 2 2 2 2 | 512 64 8 2 | 4096 512 64 16 | 1025.64 128.21 16.03 4.01 | 8205.13 1025.64 128.21 32.05 | 0.12 0.98 7.80 31.20 | 0.11 0.85 6.81 27.24 | 62.40 62.40 62.40 62.40 |

The peak PRF is 499.2 MHz. This is the highest frequency in megahertz at which a compliant transmitter shall emit pulses. The peak PRF is also used to derive the chip duration Tc by the formula Tc = 1/peakPRF. The value of Tc is approximately 2 ns.  The channel center frequencies and bandwidths are given in Table 11.  Note that the bandwidth is not necessarily the inverse of the chip duration Tc. Pulse shape and bandwidth are further defined in 2.4.3.1.

The UWB PHY contains several optional data rates, preamble code lengths, and PRF. Table 3 describes the remaining timing parameters of Figure 3 for each permitted combination of preamble code length and PRF.

The preamble code length parameter denotes the length of the preamble code to be used during the SHR portion of a data frame. The code together with the channel number defines a complex channel. Individual codes to be used on each channel are given in Table 6 (length 31) and Table 7 (length 127).

The Viterbi rate parameter determines the rate of the convolutional code applied to the PSDU data bits. A value of 1 indicates that no convolutional coding is applied while a value of 0.5 indicates that a rate 1/2 code as described in 2.3.3.2 is applied to the PSDU data bits.

The RS rate parameters indicates the (63, 55) Reed-Solomon code rate, which is approximately 0.87. The Reed-Solomon code is applied to all the PSDU data bits that are transmitted by the UWB PHY. Reed-Solomon encoding is further described in 2.3.3.1.

The overall FEC rate is determine by the product of the Viterbi rate and the Reed-Solomon rate and has either a value of 0.44 or 0.87.

The burst-positions-per-symbol parameter is the total number of possible burst positions within the data symbol duration. Nburst has been chosen so that for each mean PRF a data symbol consists of a fixed number of burst durations.

The hop bursts parameter is the number of burst positions that may contain an active burst, that is, a burst containing UWB pulses. The value is computed as Nhop = Nburst/4.

The chips per burst parameter is the number of chip Tc durations within each burst period Tburst. Each burst consists of a multiple number of consecutive chips, as illustrated in Figure 3. Depending on the data rate to be used in the transmission of the PSDU, the number of chips in a burst varies, e.g., for low data rates, the burst consists of more chip periods than for high data rates. Particular, values of Ncpb have been selected so that the following is a valid data rate: (2 × Overall FEC rate)/(Ncpb × Nburst × Tc).

The burst duration is computed as Tburst = Ncpb × Tc.

The symbol duration is the duration of a modulated and coded PSDU symbol on the air and is computed as follows: Tdsym = Nburst × Tburst.

The symbol rate is the inverse of the PSDU symbol duration 1/Tdsym.

The bit rate is the user information rate considering FEC and is computed as follows:

Bit Rate = 2 × (Overall FEC Rate)/Tdsym

The mean PRF is the average PRF during the PSDU portion of a PHY frame and is computed as follows:

Mean PRF = Ncpb/Tdsym

### Preamble timing parameters

Due to the variability in the preamble code length and the PRF, there are several admissible values for the timing parameters of a preamble symbol. These values are summarized in Table 4. In this subclause, a preamble symbol is defined as the waveform consisting of one whole repetition of the modulated preamble code (either length 31 or 127). Details on the construction of the preamble symbol for various code lengths and PRFs are given in 2.2.5. For each target PRF, the preamble is constructed from a preamble code, Ci, by inserting a number of chip durations between code symbols. The number of chip durations to insert is denoted by δL, values for each code length and PRF are given in Table 4, and the chip insertion is detailed in 2.2.5.1.

Table 4 presents the timing parameters during the SHR portion of a UWB PHY frame while Table 3 presents the timing parameters for the PSDU portion of the frame. First, note that the preamble is sent at a slightly higher mean PRF than the data as defined in Table 3. This is due to the fact that length 31 or 127 ternary codes are being used within the SHR, and the number of chips within the SHR is no longer a power of 2. For example, for the 16 MHz PRF in channels {0:3, 5:6, 8:10, 12:14), the peak PRF during the preamble is 31.2 MHz, and the corresponding mean PRF during the preamble is 16.10 MHz. The mean PRF during the data (PSDU) is 15.60 MHz. The remaining peak and mean PRF values for other optional UWB channels and the optional length 127 code are listed in Table 4.

Table 4 – preamble parameters

| Ci Code Length | Peak PRF (MHz) | Mean PRF (MHz) | Delta Length δL | #Chips Per Symbol | Symbol Duration Tpsym(ns) | Base Rate Msymbol/s |
| --- | --- | --- | --- | --- | --- | --- |
| 31 | 31.20 | 16.10 | 16 | 496 | 993.59 | 1.01 |
| 127 | 124.80 | 62.89 | 4 | 508 | 1017.63 | 0.98 |

The base symbol rate is defined as the rate at which the preamble symbols are sent. The base rate corresponding to the (default) mean PRF of 16.10 MHz is 1 Msymbol/s.  This symbol rate corresponds to preamble symbol duration, Tpsym, of 993.59 ns.

Table 5 – frame-dependent parameters

| Parameter | Description | Value |  |
| --- | --- | --- | --- |
| Channel | UWB PHY channel number | {1:15} |  |
| PRFmean | Mean PRF (MHz) | 16.10 | 62.89 |
| Nc | Number of chips per preamble symbol | 496 | 508 |
| Tpsym | Preamble symbol duration (ns) | 993.6 | 1017.6 |
| Nsync | Number of symbols in the packet sync sequence. | 64 to 4096 |  |
| Tsync | Duration of the packet sync sequence (μs) | 63.6 to 4069.7 | 65.1 to 4168.2 |
| Nsfd | Number of symbols in the SFD | 8 (or 64) |  |
| Tsfd | Duration of the SFD (μs) | 7.9 (or 63.6) | 8.1 (or 65.1) |
| Npre | Number of symbols in the SHR preamble | 72 to 4104 (or 128 to 4160) |  |
| Tpre | Duration of the SHR preamble (μs) | 71.5 to 4077.7 (or 127.2 to 4133.3) | 73.3 to 4176.3 (or 319.5 to 4422.6) |
| NCCA_PHR | Number of multiplexed preamble symbols in PHR | 4 or 32 |  |
| NCCA_data | Number of multiplexed preamble symbols in the data field | Tpre/(4 × Tdsym/M) |  |

Note: the values in brackets apply to the 110 Kb/s data rate.

Finally, for each UWB frame consisting of the SHR, SFD, PHR, and a data field, there are four possible durations of the SHR. This is due to the four possible lengths of SYNC field in the SHR, as described in 2.2.5. The SYNC field consists of repetitions of the preamble symbol. The number of preamble symbol repetitions may be 64, 1024, or 4096, with additional optional values of 128, 256, 512, 1536 and 2048. These different SYNC field lengths yield different time durations of the UWB frame. The relationship between SYNC field length and frame duration is shown in Table 5. After the insertion of the SFD (the SFD may be either 8 or 64 preamble symbols long), the total length (in preamble symbols) of the SHR is Npre as shown in Table 5, and this in turn leads to the possible SHR durations denoted as Tpre. After creation of the SHR, the frame is appended with the PHR whose length, Nhdr, is 16 symbols and duration is denoted as Thdr. The values of the frame duration parameters are shown in Table 5.

### SHR preamble

A SHR preamble shall be added prior to the PHR to aid receiver algorithms related to AGC setting, antenna diversity selection, timing acquisition, coarse and fine frequency recovery, packet and frame synchronization, channel estimation, and leading edge signal tracking for ranging.

In this subclause, four different mandatory preambles are defined: a default preamble, a short preamble, a medium preamble, and a long preamble. The preamble to be used in the transmission of a frame is decided by the application layer.

Figure 4 shows the structure of the SHR preamble. The preamble can be subdivided into two distinct portions: SYNC (packet synchronization, channel estimation, and ranging sequence) and SFD (frame delimiter sequence). The duration of these portions are provided in Table 5. Subclauses 2.2.5.1 and 2.2.5.2 detail the different portions of the preamble.

Figure 4 – SHR preamble structure

#### SHR SYNC field

Each network operating on one of the UWB PHY channels {1–15} is also identified by a preamble code. The preamble code is used to construct symbols that constitute the SYNC portion of the SHR preamble as shown in Figure 4.

The UWB PHY supports two lengths of preamble code: a length 31 code and an optional length 127 code. Each preamble code is a sequence of code symbols drawn from a ternary alphabet {-1,0,1} and selected for use in the UWB PHY because of their perfect periodic autocorrelation properties. The length 31 code sequences are shown in Table 6 while the length 127 code sequences are shown in Table 7 where they are indexed from 1–24 (Ci i = 1,2,...24). The first 8 codes (index 1–8) are length 31 while the remaining 16 (index 9–24) are length 127. Which codes may be used in each of the channels is restricted, and the particular code assignments are made in Table 6 and Table 7. Specifically, the last column in each table indicates the set of channel numbers that permit use of the code. This restriction of codes is to ensure that codes with the lowest cross-correlation are used in the same channel. Additionally, 8 of the length 127 codes are reserved for use with the private ranging protocol only and are not used during normal operation. This restriction is indicated in the third column of Table 7 as well.

Table 6 – length 31 ternary codes

| Code index | Code sequence | Channel numbera |
| --- | --- | --- |
| 1 | -0000+0-0+++0+-000+-+++00-+0-00 | 1, 8, 12 |
| 2 | 0+0+-0+0+000-++0-+---00+00++000 | 1, 8, 12 |
| 3 | -+0++000-+-++00++0+00-0000-0+0- | 2, 5, 9, 13 |
| 4 | 0000+-00-00-++++0+-+000+0-0++0- | 2, 5, 9, 13 |
| 5 | -0+-00+++-+000-+0+++0-0+0000-00 | 3, 6, 10, 14 |
| 6 | ++00+00---+-0++-000+0+0-+0+0000 | 3, 6, 10, 14 |
| 7 | +0000+-0+0+00+000+0++---0-+00-+ | 4, 7, 11, 15 |
| 8 | 0+00-0-0++0000--+00-+0++-++0+00 | 4, 7, 11, 15 |
| a- Note code indices 1 through 6 may also be used for channels 4, 7, 11, and 15 (i.e., channels with bandwidth wider than 500 MHz) if interchannel communication is desired. |  |  |

Table 7 – length 127 ternary codes

| Code index | Code sequence | Channel numbera |
| --- | --- | --- |
| 9 | +00+000-0--00--+0+0+00-+-++0+0000++-000+00-00--0-+0+0--0-+++0++000+-0+00-0++-0+++00-+00+0+0-0++-+--+000000+00000-+0000-0-000--+ | 1–3, 5, 6, 8–10, 12–14 |
| 10 | ++00+0-+00+00+000000-000-00--000-0+-+0-0+-0-+00000+-00++0-0+00--+00++-+0+-0+0000-0-0-0-++-+0+00+0+000-+0+++000----+++0000+++0-- | 1–3, 5, 6, 8–10, 12–14 |
| 11 | -+-0000+00--00000-0+0+0+-0+00+00+0-00-+++00+000-+0+0-0000+++++-+0+--0+-0++--0-000+0-+00+0+----000-000000-+00+-0++000++-00++-0-0 | 1–3, 5, 6, 8–10, 12–14 |
| 12 | -+0++000000-0+0-+0---+-++00-+0++0+0+0+000-00-00-+00+-++000-+-0-++0-0++++0-00-0++00+0+00++-00+000+-000-0--+0000-0000--0+00000+-- | 1–3, 5, 6, 8–10, 12–14 |
| 13 | +000--0000--++0-++++0-0++0+0-00-+0++00++-0++0+-+0-00+00-0--000-+-00+0000-0++-00000+-0-000000-00-+-++-+000-0+0+0+++-00--00+0+000 | 1–15; DPS only |
| 14 | +000++0-0+0-00+-0-+0-00+0+0000+0+-0000++00+0+++++-+0-0+-0--+0++--000---0+000+0+0-+-000000+-+-0--00++000-00+00++-00--++-00-00000 | 1–15; DPS only |
| 15 | 0+-00+0-000-++0000---++000+0+-0-+00-+000--0-00--0--+++-+0-++00+-++0+00000+0-0+++-00+00+000-0000+00--+0++0+0+0-00-0-+-0+0++00000 | 1–15; DPS only |
| 16 | ++0000+000+00+--0+-++0-000--00+-0+00++000+++00+0+0-0-+-0-0+00+00+0++----+00++--+0+-0--+000000-0-0000-+0--00+00000+-++000-0-+0+0 | 1–15; DPS only |
| 17 | +--000-0-0000+-00000+000000+--+-++0-0+0+00+-00+++0-++0-00+0-+000++0+++-0--0+0+-0--00-00+000-++0000+0++-+-00+0+0+--00--0-000+00+ | 4, 7, 11, 15 |
| 18 | --0+++0000+++----000+++0+-000+0+00+0+-++-0-0-0-0000+0-+0+-++00+--00+0-0++00-+00000+-0-+0-0+-+0-000--00-000-000000+00+00+-0+00++ | 4, 7, 11, 15 |
| 19 | -0-++00-++000++0-+00+-000000-000----+0+00+-0+000-0--++0-+0--+0+-+++++0000-0+0+-000+00+++-00-0+00+00+0-+0+0+0-00000--00+0000-+-0 | 4, 7, 11, 15 |
| 20 | --+00000+0--0000-0000+--0-000-+000+00-++00+0+00++0-00-0++++0-0++-0-+-000++-+00+-00-00-000+0+0+0++0+-00++-+---0+-0+0-000000++0+- | 4, 7, 11, 15 |
| 21 | +0+00--00-+++0+0+0-000+-++-+-00-000000-0-+00000-++0-0000+00-+-000--0-00+00-0+-+0++0-++00++0+-00-0+0++0-0++++-0++--0000--000+000 | 1–15; DPS only |
| 22 | 0-00-++--00-++00+00-000++00--0-+-+000000-+-0+0+000+0---000--++0+--0-+0-0+-+++++0+00++0000-+0+0000+0+00-0+-0-+00-0+0-0++000+0000 | 1–15; DPS only |
| 23 | 000++0+0-+-0-00-0+0+0++0+--00+0000-000+00+00-+++0-0+00000+0++-+00++-0+-+++--0--00-0--000+-00+-0-+0+000++---0000++-000-0+00-+000 | 1–15; DPS only |
| 24 | +0+-0-000++-+00000+00--0+-0000-0-000000+--0-+0+--++00+----++0+00+00+0-0-+-0-0+0+00+++000++00+0-+00--000-0++-+0--+00+000+0000++0 | 1–15; DPS only |
| a- Note code indices 9 through 13 may also be used for UWB channels 4, 7, 11, and 15 (i.e., channels with bandwidth wider than 500 MHz) if interchannel communication is desired. |  |  |

Note that the assignment of preamble codes to channels has been done to enable interchannel communication. In other words, it is possible that a device operating on a wideband channel {4, 7, 11 or 15} may communicate with a device on a channel with which it overlaps.

When using the ternary code indexed by i, the SYNC field shall consist of Nsync repetitions of the symbol Si, where Si is the code Ci spread by the delta function δL of length L as shown in Table 4. The spreading operation, where code Ci is extended to the preamble symbol duration indicated in Table 4, is described mathematically by

where the operator  indicates a Kronecker product. After the Kronecker operation, a preamble symbol is formed as depicted in Figure 5, where L – 1 zeros have been inserted between each ternary element of Ci.

The spreading factor L, number of chips per symbol, preamble symbol duration Tpsym, and base symbol rate for different channels are given in Table 4.

Figure 5 – construction of symbol Si from code Ci

#### SHR SFD

An SFD shall be added to establish frame timing. The UWB PHY uses a short SFD for default and medium data rates and a long SFD for the optional low data rate of 110 kb/s as shown in Figure 4.  The short SFD shall be [0 +1 0 -1 +1 0 0 -1] spread by the preamble symbol Si, where the leftmost bit shall be transmitted first in time. The optional long SFD shall be obtained by spreading the sequence [0 +1 0 -1 +1 0 0 -1 0 +1 0 -1 +1 0 0 -1 -1 0 0 +1 0 -1 0 +1 0 +1 0 0 0 -1 0 -1 0 -1 0 0 +1 0 -1 -1 0 -1 +1 0 0 0 0 +1 +1 0 0 -1 -1 -1 +1 -1 +1 +1 0 0 0 0 +1 +1] by the preamble sequence Si. Note that the long SFD is eight times longer than the short SFD and consists of 64 preamble symbols, only 32 of which are active, and the other 32 are zeros. The structure of the SHR preamble and the two possible SFDs are shown in Figure 4.

### PHY header (PHR)

A PHR, as shown in Figure 6, shall be added after the SHR preamble. The PHR consists of 19 bits and conveys information necessary for a successful decoding of the packet to the receiver. The PHR contains information about the data rate used to transmit the PSDU, the duration of the current frame’s preamble, and the length of the frame payload. Additionally, six parity check bits are used to further protect the PHR against channel errors.

Figure 6 – PHR bit assignment

The PHR shall be transmitted using the BPM-BPSK modulation outlined in 2.3. The PHR shall be transmitted at the nominal rate of 850 kb/s for all data rates above 850 kb/s and at the nominal rate of 110 kb/s for the nominal low data rates of 110 kb/s.

#### PHR rate, length, ranging, extension, preamble duration fields

The Data Rate field shall consist of two bits (R1, R0) that indicate the data rate of the received PSDU. The bits R1–R0 shall be set according to Table 9. The default value of the bits R1–R0 shall be set to 01 as this is the only mandatory data rate that is supported by a UWB-compliant PHY implementation. Support for other data rates listed in Table 9 is optional.

The Frame Length field, L6–L0, shall be an unsigned 7-bit integer number that indicates the number of octets in the PSDU that the MAC sublayer is currently requesting the PHY to transmit.

The Ranging Packet bit, RNG, indicates that the current frame is an RFRAME if it is set to 1; otherwise, it is set to 0.

The Header Extension bit, EXT, is reserved for future extension of the PHR. This bit shall be set to 0.

The Preamble Duration field, P1–P0, represents the length (in preamble symbols) of the SYNC portion of the SHR. P1–P0 shall be set according to Table 8. The default Preamble Duration setting is 01, which corresponds to a SYNC field of length 64 preamble symbols.

Table 8 – Preamble Duration field values

| P1–P0 | SYNC length (symbols) (Si) |
| --- | --- |
| 01 | 64 |
| 10 | 1024 |
| 11 | 4096 |

The Preamble Duration field is intended for use during ranging operations and is used by a receiver of the PHY frame to help determine at which preamble symbol the UWB PHY acquired and began tracking the preamble. A receiver may use the Preamble Duration field to set the value of its own preamble duration based upon the received value when communicating a ranging ACK packet.   The optional values of 128, 256, 512, 1536 and 2048 cannot be encoded within the PHY header (PHR), but may be ascertained in the receiver by counting the number of preamble symbols received.  Where one of these optional values is used the PHR encoding shall be the nearest smaller length value.

Table 9 – nominal data rates

| R1–R0 | Data rate Mb/s |
| --- | --- |
| 00 | 0.11 |
| 01 | 0.85 |
| 10 | 6.81 |
| 11 | 27.24 |

#### PHR SECDED check bits

The SECDED (single error correct, double error detect) field, C5–C0, is a set of six parity check bits that are used to protect the PHR from errors caused by noise and channel impairments. The SECDED bits are a simple Hamming block code that enables the correction of a single error and the detection of two errors at the receiver. The SECDED bit values depend on PHR bits 0–12 and are computed as follows:

C0 = XOR (R0, R1, L0, L2, L4, L5, EXT, P1)

C1 = XOR (R1, L2, L3, L5, L6, RNG, EXT, P0)

C2 = XOR (R0, L0, L1, L5, L6, RNG, EXT)

C3 = XOR (L0, L1, L2, L3, L4, RNG, EXT)

C4 = XOR (P0, P1)

C5 = XOR (R1, R0, L6, L5, L4, L3, L2, L1, L0, RNG, EXT, P1, P0, C4, C3, C2, C1, C0)

### Data field

The Data field is the last component of the PPDU and is encoded as shown in Figure 7.

Figure 7 – data field encoding process

The data field shall be formed as follows:

- Encode the PSDU using systematic Reed-Solomon block code, which adds 48 parity bits as described in 2.3.3.1.

- Encode the output of the Reed-Solomon block code using a systematic convolutional encoder as described in 2.3.3.2, except in the cases where the Viterbi rate for the modulation is 1.0 in Table 3. In these cases the convolutional encoder is bypassed.

- Spread and modulate the encoded block using BPM-BPSK modulation as described in 2.3.

## UWB PHY modulation

### Modulation mathematical framework

The transmit waveform during the kth symbol interval may be expressed as

This equation describes the time hopping with polarity scrambling, which improves interference rejection capabilities of the UWB PHY. The kth symbol interval carries two information bits  and . Bit  is encoded into the burst position whereas bit  is encoded into the burst polarity. The sequence  is the scrambling code used during the kth symbol interval,  is the kth burst hopping position, and p(t) is the transmitted pulse shape at the antenna input. The burst hopping sequence  provides for multiuser interference rejection. The chip scrambling sequence  provides additional interference suppression among coherent receivers as well as spectral smoothing of the transmitted waveform. Note that equation defines the transmitted signal during the valid burst interval; at all other possible burst positions, no signal shall be transmitted. A reference modulator illustrating the BPM-BPSK modulation is shown in Figure 8.

Figure 8 – reference symbol modulator

Note here that the FEC Encoder is not included if the modulation Viterbi rate is 1.0, as described in 2.2.7. In this case, the FEC encoder is replaced by a multiplexer which shall apply even bits to the position input and odd bits to the polarity input.

### Spreading

The time-varying spreader sequence  and the time-varying burst hopping sequence  shall be generated from a common PRBS scrambler.

The polynomial for the scrambler generator shall be

where D is a single chip delay, Tc, element. This polynomial forms not only a maximal length sequence, but also is a primitive polynomial. By the given generator polynomial, the corresponding scrambler output is generated as

where  denotes modulo-2 addition.

A linear feedback shift register (LFSR) realization of the scrambler is shown in Figure 9. The LFSR shall be initialized upon the transmission of bit 0 of the PHR. Note that Ncpb may change depending on the data rate and PRF in use during the PSDU. The LFSR shall not be reset after transmission of the PHR.

Figure 9 – LFSR implementation of the scrambler

The initial state of the LFSR shall be determined from the preamble code by first removing all the 0s in the ternary code and then replacing all the –1s with a zero. The first 15 bits of the resulting binary state shall be loaded into the LFSR. Table 10 shows an example of the above procedure for preamble code, C6 (length 31, preamble code index 6, see Table 6).  Table 10 shows the initial state as well as the first 16 output bits from the scrambler.

Table 10 – Example LFSR initial state for preamble code 6

| Initial state (s-15, s-14, …, s-1) | LFSR output: First 16 bits s0, s1, …, s15 (s0 first in time) |
| --- | --- |
| 111000101101101 | 0010011101101110 |

Note that even though each device within a network use the same initial LFSR setting, the communication is asynchronous so that the hopping and scrambling provides interference rejection.

The LFSR shall be clocked at the peak PRF of 499.2 MHz as specified in Table 3. During the kth symbol interval, the LFSR shall be clocked Ncpb times, and the scrambler output shall be the kth scrambling code . Furthermore, the kth burst hopping position, shall be computed as follows:

where

As shown in Table 3, the number of hopping burst Nhop is always a power of two, and consequently m is always an integer. Note that for Ncpb < m, the LFSR is clocked Ncpb times, not m times.

For the mandatory mode with mean data PRF of 15.60 MHz, the numbers of hopping bursts is 8, as indicated in Table 3, and consequently m takes on the values 3 and the corresponding hopping sequence is as follows:

h

k





s

k

N

c

p

b

2

s

1

k

N

c

p

b

+

4

s

2

k

N

c

p

b

+

+

+

=

### Forward error correction (FEC)

The FEC used by the UWB PHY is a concatenated code consisting of an outer Reed-Solomon systematic block code and an inner half-rate systematic convolutional code. The inner convolutional code is not necessarily enabled at all data rates; the rows of  Table 3 that have a Viterbi rate of 1 indicate that the inner convolutional code is disabled for the PSDU part of the PHY frame.

The FEC encoding of a block of M PSDU bits, b0, b1, …, bM-1, is shown in Figure 10. The Reed-Solomon encoder shall append 48 parity bits, p0, p1, …, p47, to the original block. This results in a Reed-Solomon encoded block of length M + 48. Where the Viterbi rate is 0.5, a half-rate systematic convolutional encoder shall encode the Reed-Solomon encoded block into a systematic coded block of length 2M + 96 bits. The convolutional systematic bits shall be used to encode the position of the burst whereas the convolutional parity bits shall be used to encode the polarity of the pulses within a burst. Where the Viterbi rate is 1.0, even outputs of the Reed-Solomon encoder (b0, b2,..., bM–2, p0, p2,…, p46) shall be used to encode the position of the burst, and odd outputs (b1, b3,..., bM–1, p1, p3,…, p47) shall be used to encode the polarity of the pulses. Note here that M is always an even number.

A noncoherent receiver cannot see the convolutional parity bits (parity bits), and consequently a noncoherent receiver may use only a Reed-Solomon decoder to improve its performance. A coherent receiver may use either or both Reed-Solomon and convolutional decoding algorithms. Note here that since both the Reed-Solomon and the convolutional codes are both systematic, a receiver (either coherent or noncoherent) may be implemented without an FEC decoder. In this case, the information bits are simply recovered by demodulating the position of the burst. There will be additional parity check bits as a result of the Reed-Solomon encoding, but these may be simply ignored.

Figure 10 – FEC encoding process

#### Reed-Solomon encoding

The systematic Reed-Solomon code is over Galois field, GF(26), which is built as an extension of GF(2). The systematic Reed-Solomon code shall use the generator polynomial

where α = 010000 is a root of the binary primitive polynomial  in GF(26).

In Reed-Solomon encoding RS6(K + 8, K), a block of I bits (with ) is encoded into a codeword of I + 48 bits. The Reed-Solomon encoding procedure is performed in the following five steps:

- Addition of dummy bits. The  block  of  I  information  bits  is  expanded  to  330 bits by adding 330 – I dummy (zero) bits to the beginning of the block. The expanded block is denoted as {d0, d1, ..., d329} where d0 is the first in time.

- Bit-to-symbol conversion. The 330 bits {d0, d1, ..., d329} are converted into 55 Reed-Solomon symbols {D0, D1, ..., D54} having the following polynomial representation:

Resulting 6-bit symbols are presented as , where d6k+5 is the MSB and d6k is the LSB.

- Encoding. The information symbols {D0, D1, ..., D54} are encoded by systematic RS6(63,55) code with output symbols {U0, U1, ..., U62} ordered as follows:

where Pk are parity check symbols added by RS6(63,55) encoder.

The information polynomial associated with the information symbols {D0, D1, ..., D54} is denoted as . The parity check polynomial associated with the parity check symbols is denoted as . The parity check symbols are calculated as:

- Symbol-to-bit conversion. The output symbols {U0, U1, ..., U62} are converted into binary form with LSB coming out first, resulting in a block of 378 bits {u0, u1, ..., u377}.

- Removal of dummy bits. The 330 – I dummy bits added in the first step are removed. Only the last I + 48 bits are transmitted, i.e., {u330-I, u331-I, ..., u377} with u330-I being first in time.

#### Systematic convolutional encoding

The inner convolutional encoder shall use the rate R = ½ code with generator polynomials g0 = [010]2 and g1 = [101]2 as shown in Figure 11. Upon transmission of each PPDU, the encoder shall be initialized to the all zero state. Additionally, the encoder shall be returned to the all zero state by appending two zero bits to the PPDU. Note that since the generator polynomials are systematic, they are also noncatastrophic.

Figure 11 – systematic convolutional encoder

## UWB PHY RF requirements

### Operating frequency bands

The set of operating frequency bands are as defined in Table 11.  Default channel is channel 5.

### Channel assignments

For each of the 15 operating frequency bands defined in Table 11, at the nominal 16 MHz PRF, two preamble codes are assigned, as per Table 6, giving a total of 30 complex channels.  At the optional nominal 64 MHz PRF, an additional 60 complex channels are available by employing the four preamble codes assigned for each channel as per Table 7.

Table 11 – band allocation

| Channel number (decimal) | Center frequency, fc (MHz) | Band width (MHz) |
| --- | --- | --- |
| 1 | 3494.4 | 499.2 |
| 2 | 3993.6 | 499.2 |
| 3 | 4492.8 | 499.2 |
| 4 | 3993.6 | 1331.2 |
| 5 | 6489.6 | 499.2 |
| 6 | 6988.8 | 499.2 |
| 7 | 6489.6 | 1081.6 |
| 8 | 7488.0 | 499.2 |
| 9 | 7987.2 | 499.2 |
| 10 | 8486.4 | 499.2 |
| 11 | 7987.2 | 1331.2 |
| 12 | 8985.6 | 499.2 |
| 13 | 9484.8 | 499.2 |
| 14 | 9984.0 | 499.2 |
| 15 | 9484.8 | 1354.97 |

### Transmitter specification

#### Baseband impulse response

The transmitted pulse shape p(t) of the UWB PHY shall be constrained by the shape of its cross-correlation function with a standard reference pulse, r(t). The normalized cross-correlation between two waveforms is defined as

In the above, Er and Ep are the energies of r(t) and p(t), respectively. The reference r(t) pulse used in the calculation of is a root raised cosine pulse with roll-off factor of β = 0.5. Mathematically this is

In the above equation, Tp is the reciprocal of the chip frequency.  Table 12 shows the required pulse duration for each channel.

Table 12 – required reference pulse durations in each channel

| Channel number | Pulse duration, Tp (ns) | Main lobe width, Tw (ns) |
| --- | --- | --- |
| {1:3, 5:6, 8:10, 12:14} | 2.00 | 0.5 |
| 7 | 0.92 | 0.2 |
| {4, 11} | 0.75 | 0.2 |
| 15 | 0.74 | 0.2 |

In order for a UWB PHY transmitter to be compliant with this standard, the transmitted pulse p(t) shall have a magnitude of the cross-correlation function  whose main lobe is greater or equal to 0.8 for a duration of at least Tw (see Table 12), and any sidelobe shall be no greater than 0.3. For the purposes of testing a pulse for compliance, the following are defined: Let  be the magnitude of the cross-correlation of p(t) and r(t), and let  i = 1,2,... be a set of critical points, i.e. points at which . The maximum of the function occurs at one of these critical points,  where  for all values of . The requirement above thus states that for some continuous set of values that contain the point  the function  is greater than 0.8. In addition, the second constraint on the value of sidelobes may be stated mathematically as  for all .

Figure 12 shows an example UWB-compliant pulse, p(t) (left plot), along with the root raised cosine reference pulse r(t) (middle plot) with Tp = 2.0 ns and the magnitude of the cross-correlation  (right plot). The pulse p(t) is an 8 order butterworth pulse with a 3 dB bandwidth of 500 MHz. The figure is intended to show that this example pulse meets the requirements for compliance. Specifically, the main lobe is above 0.8 for nearly 1 ns, and no sidelobe is greater than 0.3 (in this case, the largest sidelobe peak is 0.2). The pulse p(t) is a compliant pulse for channels {1:3, 5:6, 8:10, 12:14}.

Figure 12 – compliant pulse example

Note that it is not the intention of this standard to imply that pulse shaping shall occur at baseband, only that the measurements described here occur on the pulse envelope if shaping is done at passband.

#### Transmit PSD mask

The transmitted spectrum shall be less than –10 dBr (dB relative to the maximum spectral density of the signal) for  and –18 dBr for . For example, the transmit spectrum mask for channel 4 is shown in Figure 13. The measurements shall be made using 1 MHz resolution bandwidth and a 1 kHz video bandwidth.

Figure 13 – transmit spectrum mask for band 4

#### Chip rate clock and chip carrier alignment

A UWB transmitter shall be capable of chipping at the peak PRF given in Table 3 with an accuracy of ± 20 parts per million. In addition, for each UWB PHY channel, the center of transmitted energy shall be at the values listed in Table 11 also with an accuracy of ± 20 parts per million. The measurements shall be made using 1 MHz resolution bandwidth and a 1 kHz video bandwidth.

## Timestamps and time units

The UWB PHY supports precision ranging and localization through the capability of time stamping, using a ranging counter, the precise instant that RMARKERS are received (and transmitted) at the device antenna.

#### Time units

The time units used for ranging timestamps is defined by the least significant bit (LSB) of the time values which represents 1/128 of a chip time at the mandatory chipping rate of 499.2 MHz.

Note: The LSB of the ranging counter represents a time interval so small that an actual physical counter would have to run at a nominal 64 GHz to produce values with this resolution. An actual physical realization is not expected. Instead it is assumed that computational techniques will be used to generate sufficient of the less significant bits to yield the desired operational precision.

#### Antenna delays

The time of arrival and time of sending events relate to the RMARKER being at the antenna.

The receive timestamp will naturally occur in the digital domain of the receiver some period of time after the RMARKER arrives at the antenna.  To calculate when the RMARKER was at the antenna, and generate an accurate time of arrival, all the system delays between the antenna and the internal digital receive timestamp need to be accounted for. This receive antenna delay then needs to be subtracted from the internal digital receive timestamp to give the time of arrival value.

Similarly in the transmitter all the system delays between the internal digital transmit timestamp and the antenna need to be accounted.  This transmit antenna delay then needs to be added to the internal digital transmit timestamp to give the time of sending value, when the transmit RMARKER is at the antenna.

The mechanisms for determining these antenna delays are beyond the scope of this standard

</Billy>

## Appendix A

Editor: The text enclosed by <395r1> is general in nature, but the TG has not agreed if this is how PAC will classify service types. Please comment.

<395r1 name=”Jinyoung Chun, LG, jiny.chunWlge.com >

Table A. List of Service types

| Value | Meaning |
| --- | --- |
| 0 | All Service Types |
| 1 | real-time streaming |
| 2 | Display |
| 3 | talking (VoIP) |
| 4 | two-way gaming |
| 5 – 254 | Reserved |
| 255 | Vendor Specific |

</395r1>

Editor: The text enclosed by <390r1> is proposal specific and will be deleted.

<390r1 name=” SK Cho, ETRI, skcho@etri.re.kr”>

Table: Payload of discovery signal

| Contents | Size(bits) | Description | Notes |
| --- | --- | --- | --- |
| Type | 3 | Type of discovery signal 0: device advertisement 1: service advertisement 2: service info request 3: service info response 4: peer search request 5: peer search response |  |
| ID | 48 | Identifier of PD Type=0 : (own)device ID (e.g. mac address) Type=1: (own) (app. Type ID + app. specific ID+ app. Specific user ID) Type=2: (target) device ID Type=3: (own) (app. Type ID + app. specific ID+ app. Specific user ID) Type=4:(target) (app. Type ID + app. specific ID+((opt.)app. Specific user ID)) Type=5:(own) (app. Type ID + app. specific ID+ app. Specific user ID) | In case type=4, (target) app. Specific user ID can be included. ID bits are provided from upper layer (or management block) based on information of application layer |
| SIV(Service Information Version) | 5 | Version of service information provided by each PD .value:0~31(modulo 32) .value can be changed due to addition/deletion of application(s) or change of user | In case Type=0, type=2, Provided from upper layer based on information of application layer |
| Request range | 1 | Request Range of service information - 1: Delta with (pervious) service info. Ver. - 0: Full with (received) service info. Ver. | In case Type=2 |
| SN | 5 | Sequence number | In case Type=1or 3 |
| End indicator | 1 | end indication (0: continue, 1:end) | In case Type=1or 3 |
| GI | 1 | Service info. for group communication (0: individual, 1:group) | In case Type=1,3, 4 or 5 |
| Reserved | 7 or 6 | Reserved bits |  |
| total | 61 |  |  |

</390r1>