IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3d Channel Modelling Document (CMD) |  |
| Date Submitted | January 2016 |  |
| Source | Alexander Fricke (editor) | E-mail: fricke@ifn.ing.tu-bs.de |
| Re: |  |  |
| Abstract | The CMD contains descriptions of the propagation characteristics and channel models of the operational environments relevant for the considered applications (e. g. data required to calculate link budgets) |  |
| Purpose | Supporting document for the development of the amendment 3d of IEEE 802.15.3 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Document Overview

The CMD contains descriptions of the propagation characteristics and channel models of the operational environments relevant for the considered applications (e. g. data required to calculate link budgets)

The CMD will support the evaluation of the proposals submitted to P802.15.3d for consideration by the 15.3d task group.

| List of contributors |  |
| --- | --- |
| Ichiro Seto | Toshiba Corporation |
| Ken Hiraga | NTT Corporation |
| Thomas Kürner | TU Braunschweig |
| Alexander Fricke | TU Braunschweig |
| Bile Peng | TU Braunschweig |
| Sebastian Rey | TU Braunschweig |
| Makoto Yaita | NTT Corporation |
| Ho-jin Song | NTT Corporation |
| Akifumi Kasamatsu | NICT |
| Iwao Hosako | NICT |

Table of Contents

1	Definitions:	5

2	Scope	6

3	Methodology	6

3.1	General Structure of the Channel Model	6

3.2	Multipath and Polarization Characteristics	6

3.3	Usage of the Channel Model in System Simulations	6

3.4	General Channel Parameters	6

3.4.1	Operating frequency band(s)	6

3.4.2	Path loss model	6

3.4.3	Antenna gain/pattern	6

3.5	Scenario-Specific Channel Parameters	6

3.5.1	Angular Dispersion	6

3.5.2	Temporal Dispersion	6

3.5.3	Other	6

4	Close Proximity P2P Applications	7

4.1	Environments	7

4.2	Channel Characterization	7

4.2.1	Introductory Measurement Examples	7

4.2.2	Measurement Methodology and Results	8

4.2.3	Potential KIOSK System	13

4.2.2	Path Loss Model	13

4.2.3	Power Delay Profile	13

4.2.4	Fading Model	13

4.2.5	Polarization	13

5	Intra-Device Communication	14

5.1	Operating frequency band(s)	14

5.2	Introductory Measurement Examples	14

5.2.1	Measurement Methodology and General Channel Peculiarities	14

5.2.2	Significance of Scenario Definitions	18

5.3	General Structure of the Channel Model	25

5.4	Generation of the Channel Transfer Functions	26

5.4.1	Cluster Composition	28

5.4.2	Time of Arrival	30

5.4.3	Mean Path Loss	31

5.4.4	Reflection Angles	33

5.4.5	Depolarization	34

5.4.6	Angles of Departure and Arrival	36

5.4.7	Dispersion Functions	37

5.5	Scenario Definitions	40

5.6	Simulation Results	40

5.6.1	Chip-to-Chip Communications	41

5.6.2	Board-to-Board Communications	45

6	Backhaul/Fronthaul	47

6.1	Introductory Remarks	47

6.2	Path loss model	47

6.2.1	Calculation of the Overall Path Loss	47

6.2.2	Specific Attenuation by Atmospheric Gases according to ITU-R P.676-10	47

6.2.3	Calculation Specific Attenuation R due to Rain according to ITU-R P. 838-3	50

6.2.4	Calculation of Attenuation due to Clouds and Fog	52

6.3	Antenna gain/pattern	53

6.4	Scenario Definitions	53

7	Data Center	55

7.1	Propagation Path Types	55

7.2	Selection Between Path Types	56

7.3	Stochastic Channel Modelling	56

7.3.1	Path Numbers	57

7.3.2	Delay distribution	57

7.3.3	Delay-Pathloss Correlation	58

7.3.4	Pathloss-angle Correlation	58

7.3.5	Phase and Frequency Dispersion	61

7.4	Scenario Definitions	61

8	Reference	63

# Definitions:

# Scope

This document details the characteristics of the air interface channels for the suite of applications described in the current revision of the 802.15.3d Application Requirements Document, 15-14-304-16-003d.

# Methodology

Descriptions of the applications and associated channel modeling parameters are listed in paragraphs 4-7.

## General Structure of the Channel Model

Structure of the CIR equation

## Multipath and Polarization Characteristics

Description of the ray-optical propagation paths and the consideration of polarization characteristics by means of the Jones calculus

## Usage of the Channel Model in System Simulations

## General Channel Parameters

### Operating frequency band(s)

### Path loss model

### Antenna gain/pattern

## Scenario-Specific Channel Parameters

### Angular Dispersion

### Temporal Dispersion

### Other

# Close Proximity P2P Applications

## Environments

Regarding to the application requirement document [4.1] and the contribution on application usage [4.2], environments in where IEEE802.15.3d devices shall be operated can be defined. Two environments are characterized in this report. Table x1 summarizes the two characterized environments.

The scenario can be uniformed to line-of-sight (LOS) channel with transmission distance of a quite short range.  Even for LOS scenario, we have to consider the case which metal chassis or metal cover exists on consumer electronics (CE) in which IEEE802.15.3d devices are implemented inside. That metal must be object for the path between the transmitter (TX) and the receiver (RX).

Table 1: A Table

| Channel Model | Scenario | Environment | Description |
| --- | --- | --- | --- |
| CMx | LOS | Kiosk download |  |
| CMx | LOS w/o Metal | File exchange |  |
| CMx | LOS w Metal | File exchange |  |

## Channel Characterization

Close Proximity P2P (300 GHz):

Concerning the usage model of close proximity P2P wireless communications, the channel is assumed to be line-of-sight propagation in millimeterwave, 300 GHz band.

Generally, TSV model is introduced in millimeterwave PAN/LAN systems in IEEE802.15.3c and IEEE802.11ad operating both at 60 GHz. For proximity communications usage, reflections are observed inside terminals and at surface of terminals, etc. The channel model shall be modified to represent such propagation mechanisms and the frequency band at 300 GHz.

The channel model shall apply at least one of the several kinds of propagation depending on the antenna configurations.

### Introductory Measurement Examples

#### Peculiarities of wave propagation in KIOSK

Under KIOSK use case, there are the following peculiarities.

Link is pear to pear and LOS.

Wave propagation distance is relatively short (< 1 meter).

Front cover (window) exists between Tx and Rx.

Rx module uses relatively low-gain antenna because it is equipped with mobile phones.

Case of Rx (case of mobile phone) is occasionally made by metal.

Tx uses relatively high-gain antenna to avoid the propagation loss.

Tx is set in an environment pretty well-controlled as surrounded by wave absorber.

Fig. 4.1 KIOSK use case

Fig. 4.2 Wave propagation in KIOSK

### Measurement Methodology and Results

#### Measurement Methodology

To clarify a channel model of KIOSK use case, some experiments using 2-port calibrated Vector Network Analyzer (VNA) in 220  340 GHz are conducted. In the experiments, commercially available 25-dBi-antennas are used as Tx and Rx antennas. The S-parameters (S21) are measured by changing Tx-Rx distance up to 1.8m using an automatic stage.

Fig. 4.3 Experimental Setup (Upper: VNA with automatic stage surrounded by wave absorber. Lower: two kinds of 25dBi-antennas.)

#### Free space path loss

At first, free space path losses at several frequencies are measured. The Loss exponent of 1.8 ~ 2.2 are derived as shown in Fig. 4.4. This means that there is no obvious fading effect in the link. However, the path losses have power variations at the maximum of ±2.5 dB. It is thought that these come from multiple reflections between Tx and Rx.

Fig. 4.5 Free space path losses at several frequencies (220, 240, 260, 280, 300, 320, 340 GHz)

#### Power delay Profile

To confirm the effect of reflection, metal plates (20x20cm2) are attached to Tx antenna and Rx antenna as shown in Fig. 4.5.

Fig. 4.5 Metal plates attached to Tx antenna and Rx antenna

Figure 4.6 shows the measured power delay profile using these antennas.

Fig. 4.6 Measured power delay profile using metal-plate attached antennas (Fig. 5).

The first peak of the profile is the direct path (t0 = 1.8 ns). After the peak, there are two peaks (5.4 ns, 8.9 ns). The former is delayed 2t0 from the first peak, and the latter is delayed 2t0 from the former. Thus, these are the first multiple reflected path and the second multiple reflected path, respectively.

Figure 7 shows the amplitude (A) and arrival time (B) of the direct path (red square) and the first multiple reflected path (blue triangle) when changing the Tx-Rx distance.

Fig. 4.7 Amplitude (A) and delay (B) of the direct path (red square) and the first multiple reflected path (blue triangle) of power delay profile when changing Tx-Rx distance.

The first multiple reflected path of Fig. 7 (A) shows the drastic decrease at the distance of about 500 mm. This means that a part of the EM wave from the Tx does not reflect from the metal plate due to its limited size at the distance. The arrival time of the direct path (t0) and the difference of arrival times of the reflected and the direct path (2t0) is 1 : 2, respectively. These results indicate that wave propagation in KIOSK is able to be modeled as direct path and multiple reflected paths between Tx and Rx.

#### Suppressing reflection

To suppress the reflection, metal plate at the Rx antenna is tilted. The multiple reflected paths are drastically decreased by tilting the metal plate at the Rx antenna as shown in Fig. 8. Thus it is realized that tilting the metal plate which reflects the EM wave from Tx is effective for suppressing the multiple reflections.

Fig. 4.8 Measures power delay profiles. Metal plate at Rx antenna is not tilted (A), and is tilted (B).

There is a front cover (window) between Tx and Rx in KIOSK as shown in Fig. 4.2. Thus, a 2-mm thick PET (polyethylene-telephthalate) plate is inserted between Tx and Rx, and the power delay profiles are measured.

Figure 4.9 shows the results of the cases that metal plates and PET plate are not tilted (i), only PET plate is tilted (ii), and both PET plate and metal plate at Rx are tilted (iii).

Fig. 4.9 Power delay profiles of the cases that metal plates and PET plate are not tilted (i), only PET plate is tilted (ii), and PET plate and metal plate at Rx are tilted (iii).

Figure 4.9 (i) shows that the new two peaks (A, B) appear and whose delay values depend on the distances from the antennas. These come from multiple reflections between metal plate and the PET plate, which is confirmed by tilting the PET plate in Fig. 4.9 (ii). When both metal plate at the Rx and the PET plate are tilted, reflections are suppressed as shown in Fig. 4.9 (iii).

### Potential KIOSK System

From the above measurements related to wave propagation in KIOSK, potential KIOSK system is able to be thought as shown in Fig. 4.10, namely, Tx is surrounded by the wave absorber, front cover (window) of the Tx is tilted and Rx case touched on the tilted front cover.

Fig. 4.10 Potential Kiosk System

### Path Loss Model

Molecular attenuation can be ignored because transmission distance along application usage is a short range of up to 50 millimeters.

### Power Delay Profile

### Fading Model

### Polarization

# Intra-Device Communication

## Operating frequency band(s)

As envisaged in the ARD, the desired transmission rates for wireless intra-device communication  reach up to almost 100Gbps. Furthermore, the use of frequency-domain and spatial multiplexing shall be possible. The operational environment is restricted to some 10cm and usually trapped by a device casing. Consequently, a huge frequency range might be exploited, for example between 270 GHz and 320 GHz.

## Introductory Measurement Examples

### Measurement Methodology and General Channel Peculiarities

In the following, the peculiarities of the intra- device propagation channel shall be introduced by a set of measurements in a board-to-board communication environment. The transmission channel consists of two antennas mounted on opposing surfaces at close proximity without any obstructions between the antennas. A sketch of this scenario is provided in Figure 1.

Figure 1: Board-to-board communication scenario (top view)
Tx and Rx are mounted on opposing PCB surfaces (green)

With this configuration, a range of exemplary measurements has been performed to get a first insight in the channel characteristics. The measurements have been based on a setup comprising a vector network analyzer along with the necessary frequency extension modules to reach the frequency band between 270 GHz and 320 GHz. Information regarding the setup and mechanical arrangement can be found in [5.1]. As seen in Figure 2 below, four configurations with diagonal antenna positioning have been measured. The measurements comprise two different box sizes d as well as two box setups, one including Printed Circuit Boards (PCB) at front- and backside and one without.

Figure 2: Measured board-to-board scenarios 
two box sizes (first and second row) 
full plastic or PCB-equipped box (left and right column)

In particular, the impact of printed circuit boards and the behaviour of the channel for the possible sub-bands have been investigated. Figure 3 exemplarily shows a measurement result over the full bandwidth along with the effects arising when only a sub-band of the complete channel is evaluated.

Figure 3: Measured channel transfer function (CTF) and channel impulse response (CIR) 
for the full frequency range (left) and two chosen sub-bands (middle and right)

The channel transfer function (CTF) over the complete bandwidth shows the typical profile of a strong propagation path interfering with some attenuated echoes. Its Fourier-Transform, the channel impulse response (CIR), reveals a strong peak corresponding to the direct path between Tx and Rx followed by the expected signal echoes from reflections inside the casing. It must be noted that the CIR is influenced by the leakage-effect introduced by the inverse Fourier Transform. Comparing the CIR of the full bandwidth to the CIR of the sub-bands band 1 between 270 GHz and 280 GHz and band 3 between 290 GHz and 300 GHz, a varying channel can be observed for the two bands. For band 1, the propagation channel seems to be almost free of echoes; the peaks seen in the full-bandwidth CTF are reduced almost to the FFT-leakage floor. In band 3, the reflections appear even stronger than in the original signal. This effect stems from the reflections at the plastic casing of the device. A signal reflected from a thin layer of plastic will interfere with itself due to two reflection processes at front- and backside of the plastic surface. Depending on the absolute frequency of the signal, these two reflection processes may add up constructively or destructively. A detailed investigation of the reflection and transmission behaviour at THz frequencies is found in [5.2] Thus, the same propagation path may lead to varying contributions to the total channel behaviour if different sub-bands are considered.

In the following, the CIRs obtained for the environments introduced in Figure 2 are presented. First, the result for the whole bandwidth is discussed. Subsequently, the results for sub-band 1 and sub-band 3 are presented.

|  | Plastic Walls | PCB Walls |
| --- | --- | --- |
| Small Box |  |  |
| Large Box |  |  |

Figure 4: Channel impulse responses for the full bandwidth between 270 GHz and 320 GHz

As introduced in the generic example in Figure 3, one strong main peak, corresponding to the direct transmission path between Tx and Rx, followed by a range of echoes from the casing walls is observed in all four cases. For the small box with plastic walls, the path loss of the main signal is as low as -20dB. In case of the large box, the path loss rises to about -30dB due to the additional propagation distance; furthermore, the far-field distance of the employed horn antennas is reached in the large box only. It can be observed that the path loss is around -30dB in case of the small box equipped with PCBs as well. This is due to the fact that the direct path between the antennas or, more precisely, the first Fresnel zone has been blocked by the building parts at the PCB surfaces. While the first echos arrive after around 1ns in the small box, the echoes in the large box arrive after 2 or more nanoseconds. The amplitude of the echo paths is only slightly influenced by the size of the box or the presence of PCBs.

|  | Plastic Walls | PCB Walls |
| --- | --- | --- |
| Small Box |  |  |
| Large Box |  |  |

Figure 5: Channel impulse responses for sub-band 1 between 270 GHz and 280 GHz

Comparing the impulse responses at full bandwidth to the impulse responses in sub-band 2 (Figure 5) and sub-band 3 (Figure 6), the lower temporal resolution of the impulse responses due to the smaller bandwidth of the sub-bands can be observed. It leads to a virtual pulse broadening which can be observed when comparing the impulse responses of the large box scenario with plastic walls. This effect is due to the missing temporal synchronization of the pulse delay to the time steps of the impulse responses; i.e. it can be compensated by receiver synchronization in a real transmission system.

|  | Plastic Walls | PCB Walls |
| --- | --- | --- |
| Small Box |  |  |
| Large Box |  |  |

Figure 6: Channel impulse responses for sub-band 2 between 290 GHz and 300 GHz

Apart from this, the behaviour of the main signal remains constant for both sub-bands when compared to the full bandwidth. The amplitude of the reflected paths varies clearly between the sub-bands for transmission inside the plastic boxes. For the small box, the multipath component at about 1ns after the main peak almost vanishes in sub-band 1. The same effect is observed for two multipath components at around 1.5ns after the main peak in the large box. Both multipath clusters are clearly present in sub-band 3. Looking at the scenarios with PCB walls, no significant difference exists between the sub-bands. This backs up the observation that the (systematically) varying channel behaviour is induced by the thin layers of the plastic casing rather than the PCB building parts.

### Significance of Scenario Definitions

It is assumed that the stochastic channel model under development will have varying statistical properties depending on the concrete operational environment. This assumption is based on the following observations from a measurement campaign comprising scenarios from two different operational modes for board-to-board communication. The operational mode Direct Transmission corresponds to the case of communication via a line-of-sight connection between a transmitter and a receiver mounted on two directly opposing surfaces. In the case of directed non-line-of-sight transmission, the signal is guided via a reflection inside the device due to the missing possibility of aligning the antennas. This could be the case if it is not possible to correctly align the antenna main lobes towards each other, for example, because building parts or edges of the casing are blocking the line of sight.

Two scenario realizations have been defined for each of the operational modes as depicted in Figure 7.

Figure 7: Scenario Definitions for the Operational Modes 
Direct Transmission (left) and directed NLOS Transmission (right)

For Direct Transmission, a diagonal positioning of Tx and Rx, corresponding to the scenario direct_1, and a straight connection between directly opposing Tx and Rx, corresponding to scenario direct_2,  have been measured. For the mode of Directed NLOS Transmission, communication between two antennas mounted on the same surface via a guided reflection on the opposing wall, corresponding to scenario dNLOS_1, and transmission between two opposing antennas via a reflection on a wall perpendicular to both antenna mounts, corresponding to scenario dNLOS_2, have been measured.

Analogous to 5.2.1, each scenario has been measured inside a large and a small environment, the dimensions of which can be found in [5.3]. Also, the environment was measured in two different configurations, with the first consisting of a full plastic environment and the second being equipped with two printed circuit boards at the front- and backside. This leads to a total number of four scenario realizations per scenario definition which are summarized exemplarily for scenario direct_1 in Figure 2 in the above sub-chapter.

Figure 8 - Figure 11 show the measured CIRs for the scenario of Direct Transmission for all four scenario realizations. Each figure contains the measurement results from the first direct scenario in the top and the result from the second scenario in the lower sub-figure. Moreover, each scenario has been measured in two measurement runs that are plotted as a red and a green curve. The horizontal lines depict a threshold of -30dB below the strongest signal component; this threshold is used for the later on RMS delay spread calculations.

Figure 8: Measured CIRs of the Large Box with Plastic only 
Scenario direct_1 (top), Scenario direct_2 (bottom)

For the large plastic box, it is observed that one dominant propagation path exists in the case of board-to-board communications with no obstructions. Its amplitude generally lies 20dB over that of the strongest echo path; most multipath components even vanish below the previously defined threshold.

Figure 9: Measured CIRs of the Large Box with PCBs 
Scenario direct_1 (top), Scenario direct_2 (bottom)

When the scenario is equipped with printed circuit boards, it is observed that the general characteristics of the channel do not change. A clearly distinct main pulse remains visible while the amplitudes of the echo paths remain in the order of the -30dB threshold.

Figure 10: Measured CIRs of the Small Box with Plastic only 
Scenario direct_1 (top), Scenario direct_2 (bottom)

In a smaller environment, the echo clusters arrive earlier compared to the more spacious environment, thus the CIR has a temporally more compact form. The amplitudes of the echo paths remain at roughly the same level as observed for the large environment.

Figure 11: Measured CIRs of the Small Box with PCBs 
Scenario direct_1 (top), Scenario direct_2 (bottom)

Again, inserting printed circuit boards into the environment does not much influence the channel behaviour. However, it must be noted that the amplitudes for the diagonal transmission in scenario direct_1 drop from between -20dB and -30dB in Figure 10 to between -30dB and -40dB in Figure 11. This is most likely due to the fact that part of the first Fresnel Zone is blocked by building parts on the PCB surface in case of the narrow environment; however, no additional pulse broadening is observed from this.

Overall, the presence of printed circuit boards does not seem to have a significant impact to the direct line-of-sight communication channel; compared to the effects already observed for the plastic box, the multipath characteristics are not increased due to the insertion of PCBs.

As a figure of merit for the temporal characteristics of the LOS channel, Table 2 summarizes the RMS delay spreads that have been calculated from the measurements with respect to the above defined -30dB threshold.

Table 2: RMS Delay Spreads from the Direct Transmission Measurements

|  | Large ABS | Small ABS | Large PCB | Small PCB |
| --- | --- | --- | --- | --- |
| direct_1, red | 0.241 ns | 0.019 ns | 0.036 ns | 0.126 ns |
| direct_1, green | 0.164 ns | 0.113 ns | 0.020 ns | 0.065 ns |
| direct_2, red | 0.197 ns | 0.097 ns | 0.215 ns | 0.099 ns |
| direct_2, green | 0.089 ns | 0.107 ns | 0.225 ns | 0.110 ns |

One important characteristic of the presented values is their sensitivity regarding the level of the defined threshold. Comparing the delay spread values for scenario direct_1 in the small box with ABS (green rectangle) to the values in the small box equipped with PCBs (red rectangle), it strikes that the value grows by a factor of six for the measurement corresponding to the green curve in FIGURE but shrinks by a factor of two for the measurement corresponding to the red curve when PCBs are inserted. Having a closer look at Figure 10 and Figure 11 reveals that this is due to the fact that some multipath components (marked with blue circles) exceed the defined threshold slightly while others don’t. Even though the overall characteristic of the impulse responses is the same in both cases, the calculated delay spreads suggest strong and also contradicting changes in the temporal channel behaviour. A consequence of this observations is that the channel model under development should be based on ray-tracing simulations and accompanied by verification measurements. Since there is no noise present in the case of simulations and the temporal position of the multipath components is exactly known, the definition of a threshold for e.g. delay spread calculations becomes obsolete.

Figure 12 - Figure 15 show the measured CIRs for the scenarios of Directed NLOS Transmission for all four scenario realizations.

Figure 12: Measured CIRs of the Large Box with Plastic only 
Scenario dNLOS_1 (top), Scenario dNLOS_2 (bottom)

Observing the results for the large environment, it is noticed that the main signal is clearly broadened due to the reflection on the plastic casing of the box. Apart from this significant difference to the LOS scenario, the multipath characteristics remain similar to the direct transmission case; it should however be noted that some rather strong multipath components are present in scenario dNLOS1.

Figure 13: Measured CIRs of the Large Box with PCBs
Scenario dNLOS_1 (top), Scenario dNLOS_2 (bottom)

Inserting printed circuit boards into the environment may change the channel behaviour drastically for directed NLOS communications as seen in the above part of Figure 13. As the guided reflection takes place via a PCB surface now, the pulse broadening becomes more severe for the main pulse. In addition, the echo components increase in amplitude to la level of -5dB below the main signal. For scenario dNLOS_2 the effects are much less significant as the reflection surface (short side-wall of the box) is still an ABS layer.

Figure 14: Measured CIRs of the Small Box with Plastic only 
Scenario dNLOS_1 (top), Scenario dNLOS_2 (bottom)

Looking at the results for the small boxes, it can be seen that, analogous to the case of directed communications, the temporal structure of the multipath components becomes more compact. For the main signal, a slight increase of the pulse broadening of the main pulse is observed compared to the large box measurement. This is due to the fact that a the larger reflection angle, resulting from the reduced distance between antennas and reflecting wall, leads to a longer path difference of the reflection processes at front- and backside of the reflecting plastic layer. Details regarding this behaviour can also be found in [5.2].

Figure 15: Measured CIRs of the Small Box with PCBs
Scenario dNLOS_1 (top), Scenario dNLOS_2 (bottom)

From the measurement results of the small box equipped with PCBs, it becomes obvious that the impact of PCBs to the channel becomes less significant if the propagation environment gets narrower. However, a temporal spread of the main signal that stems from the scattering processes from the building parts throughout the board surface remains a main channel characteristic.

Concludingly, it is observed that the characteristics of directed NLOS communications vary significantly from those of the direct communications case. The guided reflection process impinges a pulse broadening of the main signal for both plastic and PCB guided reflections; moreover, the presence of  scattering PCB surfaces has an impact on the temporal profile of the channel impulse response, especially in spacious environments.

Table 3 provides the results of the RMS delay spread calculations for the directed NLOS communication scenarios.

Table 3: RMS Delay Spreads from the Directed NLOS Transmission Measurements

|  | Large ABS | Small ABS | Large PCB | Small PCB |
| --- | --- | --- | --- | --- |
| dNLOS_1, red | 0.367 ns | 0.099 ns | 0.758 ns | 0.122 ns |
| dNLOS_1, green | 0.245 ns | 0.115 ns | 0.650 ns | 0.047 ns |
| dNLOS_2, red | 0.072 ns | 0.036 ns | 0.026 ns | 0.027 ns |
| dNLOS_2, green | 0.085 ns | 0.129 ns | 0.139 ns | 0.069 ns |

Under consideration of the measurement results presented in Chapters 5.2.1 and 5.2.2 a scientific base for the derivation of a stochastic channel model is established.

- The derivation of channel characteristics from measurements only leads to a number of issues, e.g. the presence of noise, the effects of IFFT leakage and the unknown position of multipath components in the measured signal. Thus, a ray-tracing approach is chosen for creating the channel statistics

- It has been shown that different operational modes lead to varying channel characteristics that need to be accounted for by separate channel statistics for separate use cases.

## General Structure of the Channel Model

The channel model is realized as a set of channel transfer functions which give a complete description of the propagation channel over the whole frequency range under consideration. The Inverse Fourier Transform represents the signal that is received after a single symbol has been transmitted through the channel with respect to the employed antenna characteristics. The channel transfer function in its general structure may be written as

|  | Hf,ϑTx,φTx,ϑRx,φRx,PTx,PRx=iARxϑRx-ϑAoA,i,φRx-φAoA,i,PRx∙CTFif,Hi∙ATxϑTx-ϑAoD,i,φTx-φAoD,i,PTx | (1) |
| --- | --- | --- |

In this, f is the vector of frequencies under consideration, ϑTx and φTx are the elongation and azimuth coordinates of the transmitter antenna radiation pattern, ϑRx and φRx are the elongation and azimuth coordinates of the receiver antenna radiation pattern and PTx and PRx are the polarization vectors of the antennas. The expression to the right is the sum over each propagation cluster i. The contribution of every cluster is its polarimetric channel transfer function CTFi, multiplied with the direction-dependent gain of the antenna radiation patterns ATx and ARx.

The structure of the channel transfer functions of the separate clusters is

| CTFi(f,Hi)=Hi,11(f)Hi,12(f)Hi,21(f)Hi,22(f) | (1) |
| --- | --- |

Here, the Hi,11 to Hi,22 are the entries of the polarimetric channel matrix for every cluster according to the Jones Calculus [5.4]. Note that every entry is a function of frequency, to generate the desired channel transfer functions.

Finally, the terms for the transmitting and receiving antenna are

| ATx/Rx=gTxRx(ϑTx/Rx-ϑi,φTx/Rx-φi)∙Jϑ,Tx/RxJφ,Tx/Rx | (2) |
| --- | --- |

Where gTx/Rx is the direction-dependent gain of the respective antenna evaluated at the angular positions where the ith ray hits the respective antenna radiation pattern and J is the Jones-vector of the antenna according to [5.4].

## Generation of the Channel Transfer Functions

The channel transfer functions (CTFs) introduced in Chapter 0 are generated by a specific tool for generating channel transfer functions which is termed channel realization generator (CRG). The CRG evaluates a set of stochastic interdependencies for the channel properties that underlie the channel transfer function, such as cluster composition, angular profiles at Tx and Rx, path losses, times of arrival and polarization properties. The channel model is derived from a ray-tracing approach that has been developed to account for the peculiarities of close-proximity and intra-device communications in the THz – range. It includes the electromagnetic influence of plastic layers, metals and printed circuit boards. Moreover, the characteristics of Gaussian antenna profiles are included.

b)

a)

Figure 1: Ray tracing weighted with Gaussian antenna profiles
a) Board to Board case, b) Chip to Chip case

From the ray-tracing results, which are exemplarily visualized in Figure 1, the characteristics of the channel regarding cluster composition, path loss and polarization properties as well as angular and temporal profiles are extracted. These characteristics are used to configure the CRG which is utilized to generate a large number of realistic channel realizations (i.e. frequency responses) for the corresponding use cases. The structure of the employed channel statistics is depicted in Figure 2.

Figure 2: Structure of the underlying channel statistics

### Cluster Composition

The most important component of the CRG is the cluster composition of each channel. The cluster composition is the actual set of multipath components that occur in a channel realization. It is obvious, that an incorrect assumption regarding the cluster composition will lead to invalid channel realizations. The cluster composition implicitly holds the geometric structure of the underlying ray-tracing simulations. For example, analyzing a long hallway will lead to a different set of clusters and cluster interrelationships than a square-shaped office room with room dividers that may in some cases block the line of sight.

For the stochastic generation of channel transfer functions, statistics are extracted for every cluster type and reflection order. The considered cluster types are defined by the reflection processes that occur along their respective propagation path. This results in the cluster types LOS (direct path), TMM (reflection on plastic surfaces), METAL (reflection on metal surfaces) and MIXED (different reflection types along the path). To further illustrate the concept, Figure 3 shows one exemplary propagation path, the corresponding wall indices and the resulting matrix-structure (along with two other paths) holding the depicted second-order path. The matrix-structure is in the following referred to as composition tensor.

Figure 3: Derivation of composition tensors

After the evaluation of the clusters in terms of composition tensors, the relative probabilities of occurrence are generated. In the model, this is done by means of Gaussian Mixture Models (GMM). A GMM is capable of modeling multivariate distributions over correlated features. This way, the characteristics of the cluster tensors (e.g. mutually exclusive occurrence of certain reflections) can be accounted for. After the number n of paths with possibility < 1 and > 0 has been determined, a GMM is set up for each cluster type. Each respective GMM has a number of n variables that are drawn from n Gaussian components. Thus, a single draw from such GMM produces a vector of n logical values, expressing the presence or absence of the nth cluster component for a given channel realization.

Figure 4: Cluster composition

### Time of Arrival

After generating the cluster composition, the propagation time of each path (Time of Arrival, ToA), is generated. It is assumed, that the propagation delays of the various cluster types follow normal distributions. For the direct path clusters, the propagation times are modeled directly based on their delay from the ray-tracing results. For the reflected paths, the delay is modeled based on the relative delay to the corresponding direct path; this way, it  is assured that no reflected clusters are generated that have a shorter propagation time than the direct path, which would be physically unrealistic. Figure 5 depicts two exemplary distributions that have been generated for mixed propagation clusters from one specific communication scenario.

Figure 5: Exemplary delay distributions of two second order clusters

As the modeling methodology for the stochastic generation of CTFs is based on the evaluation of probability distributions, for each cluster occurrence one corresponding probability distribution is generated. For the introduced example of clusters, this fact is further illustrated in Figure 6.

Figure 6: Generation of distributions for each observed cluster

Note that each distribution is truncated to the interval of the maximum and minimum observed delay value. This way, it is ensured that the stochastic generation process does not produce any unrealistically high or negative delay values. The above introduced methodology for the derivation of probability distributions for each object of the cluster composition tensors is applied in each of the following steps. This way, the model is further configured with the implicit information on the channel geometry such as interrelationships between cluster arrival times, reflection angles, polarization and so on.

### Mean Path Loss

Based on the cluster arrival times, the mean path loss of each cluster is generated. The mean path loss is defined as the mean absolute value over frequency for the ray-tracing frequency responses of each cluster. The mean path loss is evaluated for both polarizations. It is considered physically meaningful that the path loss is a function of path delay. For the path loss and all other characteristics that are modeled as functional relationships, the underlying form of the functions are second order polynomials. Even though some dependencies may follow different functional shapes on the large scale (e.g. the path loss in a channel is usually considered to follow a negative-exponential relationship with the propagation delay), second-order polynomials are a sufficient way to model the relationships for relatively small value ranges. Since each cluster (e.g. left-wall reflection, ceiling-reflection and so on) is modeled separately, there is always a second-order relationship that has a very good agreement. Figure 7 depicts the path loss approximation of the theta-component of the LOS cluster transmitted through a plastic layer.

Figure 7: Path Lloss of an exemplary LOS component

As it can be seen, the agreement between the second-order polynomial and the sample values from the ray-tracing simulations is very good. The presented relationship is treated as the mean value of the propagation path loss as a function of frequency.

Evaluating the derivation of the sample points from the generated second-order model, another second order relationship is derived to characterize the mean absolute error (MAE) of this approximation. An example is shown in Figure 8.

Figure 8: Mean average  error of the exemplary LOS component

For the channel model, the functions for the mean value and the mean average error are used to parameterize a Gaussian Distribution (GD) from which the values for the stochastic channel transfer functions are generated. This is depicted schematically in Figure 9.

MAE

mean()

Figure 9: Derivation of a Gaussian distribution from mean and MAE values

### Reflection Angles

In the same manner as the path delays, the initial reflection angles of all reflected paths are modeled as functions of the path delay. However, it is not necessary that these functions are monotonically decreasing. Instead, different geometries may lead to varying types of relationships as is illustrated in Figure 10.

a)

b)

Figure 10: Geometrical relationship between path length and reflection angle

In the example, it becomes obvious that a shorter propagation path (green) may lead to a smaller angle of reflection in situation a) while it leads to a larger angle of reflection in situation b). Due to the fact that every cluster is modeled separately in its behavior and a function of this relationship is generated for every cluster, the amount of implicit information on the channel geometry is again increased. In the case of second order-reflections, the reflection angle is modeled as a function of the corresponding first order reflection angle. This choice has been made due to the fact that no clear functional relationship was obvious anymore between path delay and reflection angle for subsequent reflections after the first one.

### Depolarization

To this point, the statistical channel properties have been evaluated for the phi and theta components of the electromagnetic field after transmission through the channel. However, the channel matrix of a polarimetric radio channel consists of four elements to account for the phenomenon of depolarization:

|  |  | (4) |
| --- | --- | --- |
|  |  | (5) |  |

In the above expressions, the elements H12 and H21 lead to a talk-over between the two canonic polarizations that has to be accounted for if antennas other than horizontally or vertically polarized antennas shall be employed in the channel simulations. Thus, after the generation of the mean path losses for theta- and phi-polarization, the depolarization angle in radians of each cluster is derived by

|  | γdepol=atanH12H11=atan⁡(H21H22) | (6) |
| --- | --- | --- |

The depolarization angle is the angle between the initial canonic polarization components at the position of the Tx antenna and the polarization components of the electric field vector after the passage of the propagation channel as illustrated in Figure 11. In the example, only the amplitude of the H-matrix is considered, which is additionally adjusted for the free space path loss. Situation a) depicts the impact from a channel matrix of a directly transmitted cluster without any reflection processes involved. Situation b) depicts the influence from a channel matrix with nonzero elements H12 and H21 due to geometric depolarization. It can be seen that the tilt in the coordinate system is defined by the ratio of H12/H11 and H21/H22, respectively, as stated in (6). It should further be noted that the polarization tilt is not necessarily identical to the angle between the transmitted and received field vectors as this is also influenced by the amplitude of the H11 and H22 matrix elements.

a)

b)

Figure 11: Definition of the polarization angle
a) No geometric depolarization
b) Geometric depolarization due to one or more reflection processes

As no functional dependency (e.g. to the time of arrival) could be observed, all depolarization angles for all reflection processes are modeled as Gaussian distributions.

### Angles of Departure and Arrival

The final component necessary to fully characterize the Terahertz communication channel is the angular profile at the transmitter and the receiver site. With this, the impact of different antenna types with varying radiation patterns and polarization properties can be incorporated into the channel model. As it is considered geometrically meaningful, the angle of departure at the Tx and the angle of arrival at the Rx are modeled jointly for elongation theta and azimuth phi. The following Figure 12 illustrates this assumption.

Figure 12: Illustration of AoD/AoA correlation

In the figure, the transmitter is moved along the path depicted by the green arrow from position Tx1 to position Tx2. The Rx remains at a fixed position and the effect of the movement on the elongation of departure at the transmitter is observed. As it can be seen on the four pictures to the right, the relationship between these angles in the elongation and azimuth domain is correlated for the direct path. Similar but less graphic relations exist for reflected clusters.

As a consequence of the above observations, the AoA and AoD profiles in theta and phi are modeled as correlated probability densities. To extract the parameters of these distributions, the method of copula distributions is applied. In this methodology, the marginal univariate distributions of the AoA and AoD of a certain cluster are generated; in a second step, a copula structure is generated to provide the correlation between the variables. The output in the case of angular profiles is a Gaussian distribution that is not defined over a single value but over both AoA and AoD in a correlated manner.

### Dispersion Functions

From the generated statistics, the clusters of each channel have to be evaluated with dispersion functions as well as antenna profiles to generate the final channel transfer function of each channel realization. The structure of this process is depicted in Figure 13.

Figure 13: Generation of the channel transfer function

Due to the broadband nature of the investigated propagation channels, a single mean path-loss value is not enough information to characterize the propagation paths. Instead, the path loss is always a function of frequency due to dispersion stemming from Friis Transmission Equation as well as from the reflection processes at thin layers and printed circuit boards. To illustrate this behavior, the effects of the three dispersion mechanisms are illustrated in Figure 14.

c)

b)

a)

Figure 14: Exemplary dispersion functions

In figure a), the characteristic slope of an ultra-broadband channel due to the decreasing effective aperture of a given antenna over a large frequency range is observed, while figure b) shows the typical interference pattern from a reflection at a thin plastic surface. Figure c) shows the frequency response of a path that has been reflected by a PCB surface. In the ray-tracing algorithm, relatively complex models of the electromagnetic behavior at reflection processes are employed. Resulting from this, the ray-tracing algorithm has a high computational complexity leading to a simulation time of up to a minute for the generation of a single channel transfer function. The CRG uses parameterized functions instead of the actual physical models to reduce this complexity while maintaining realistic properties of the dispersion processes.

## Scenario Definitions

For Intra-Device Communications, the cases of Chip to Chip Communications under LOS condition, Chip to Chip Communications under NLOS condition and Board to Board Communication are distinguished. They are depicted schematically in Figure 15.

Chip to Chip

Board to Board

Figure 15: Operational modes for Intra-Device Communications

The parameters of the operational modes are summarized in Table 1

Table 1: Parameters of the Intra-Device Operational Modes

## Simulation Results

In this chapter, a summary of the channel characteristics for the defined scenarios with the investigated antenna configurations is presented. The summary comprises an analysis of the mean path loss over distance for the envisaged application cases along with an envelope function of the impulse responses for each case.

### Chip-to-Chip Communications

Figure 16 and Figure 17 present the mean path loss over distance for the two different operational modes (LOS and NLOS) and polarizations. Again, a numerically fitted function describing the relationship between the separation between Tx and Rx and the observed path losses is shown.

The path-loss of the main signal follows a log-distance dependent behaviour under LOS conditions. This behaviour is illustrated in Figure 16.

b)

a)

Figure 16: Mean path loss for Chip to Chip Communications under LOS condition
a) Linear vertical polarization b) Left-Hand circular polarization

For modelling the path loss of chip to chip communications under LOS conditions, the relationship between the path loss and the distance is defined as follows:

|  | PL(d[m])totaldB=PLd0dB+10∙γ∙log10dmd0m+χg | (7) |
| --- | --- | --- |

In this expression, PLtotal is the total path loss, PLd0 is the path loss at the reference distance d0, γ is the path loss exponent, d is the distance at which the expression is evaluated and χg is a normally distributed random variable with zero mean.

The corresponding parameters of the equation are listed in Table 2.

Table 2: Parameters of the path loss model for Chip to Chip communications under LOS conditions

| Scenario | PLd0 | d0 | γ | RMSE(χg) |
| --- | --- | --- | --- | --- |
| Chip to Chip LOS, vertical | 16.5 | 0.05 | 2.007 | 1.1934 |
| Chip to Chip LOS, circular | 16.5 | 0.05 | 1.969 | 1.1020 |

For the directed NLOS operational mode, the log-distance dependency of the path loss is not valid anymore. As seen in Figure 17, the simulated path losses are rather equally distributed over a certain amplitude range incorporating only a weak relationship between the distance between Tx and Rx and the signal path loss. Comparing the results for linear ( Figure a) ) and circular ( Figure b) ) polarization, it can be seen that the directed NLOS signal is around 5dB weaker in case of circular polarization, which is consistent to the expected behaviour of reflected circularly polarized propagation paths.

a)

b)

Figure 17: Mean path loss for Chip to Chip Communications under NLOS condition
a) Linear vertical polarization b) Left-Hand circular polarization

As mentioned above, the path loss of the NLOS configuration does not follow a log-distance dependency regarding the separation between Tx and Rx. This is comprehensible since the length of the propagation path is only indirectly coupled to the separation between Tx and Rx. In addition to the separation, the implicit variation of box size between the maximum size of L and the minimum size of S leads to a rather uniform distribution of the directed NLOS propagation path loss. However, the reflection angle of that path is also influenced by the Tx-Rx separation, leading to a slight variation of the reflection loss over distance. The overall impact on the main signal for the NLOS transmission mode can be modeled by means of a linear equation

|  | PL(d[m])totaldB=PL0dB+ζ∙dm+χg | (8) |
| --- | --- | --- |

In this, PL(d)total is the distance-dependent path loss, PL0 is the path loss when the separation between Tx and Rx becomes zero, ζ is the path-loss coefficient, d is the separation between Tx and Rx and χg is a normal-distributed random variable with zero mean. Note that the path loss does not become zero for zero antenna separation; as explained above, the target NLOS propagation path is still of a certain length that leads to the path loss of PL0. The parameters of equation (2) are listed in Table 3.

Table 3: Parameters of the path loss model for Chip to Chip Communications under NLOS conditions

| Scenario | PL0 | ζ | RMSE(χg) |
| --- | --- | --- | --- |
| Chip to Chip NLOS, vertical | 44.85 | 40.64 | 1.1934 |
| Chip to Chip NLOS, circular | 51.49 | 43.13 | 1.1020 |

Figure 18 and Figure 19 present the envelopes of the channel impulse responses generated for every scenario. Along with the path loss, they can be utilized to create a mask in time domain that represents the worst-case assumption regarding temporal channel dispersion.

b)

a)

Figure 18: Envelope of the channel impulse responses for Chip to Chip communications 
under LOS condition
a) Linear vertical polarization b) Left-Hand circular polarization

From Figure 18, it becomes evident that a multipath profile exists even under the assumption of high-gain antennas for chip-to-chip communications. However, comparing Figure a) and Figure b), the multipath richness can be significantly reduced if circular polarization is applied.

b)

a)

Figure 19: Envelope of the channel impulse responses for Chip to Chip communications
under NLOS condition
a) Linear vertical polarization b) Left-Hand circular polarization

In the case of directed NLOS communications, as depicted in Figure 19, a couple of very strong multipath components might occur for some geometrical configurations. As seen in Figure a), these reflected paths can reach the amplitude of the main signal and thus lead to strong inter-symbol interference when not properly treated e.g. by equalization techniques or forward error coding. Again, the application of circular polarization provides a flatter temporal profile of the channel. In Figure b), the strongest multipath components are already attenuated by around -10dB compared to the case of linear polarization, providing a much better position for deploying a communication system.

### Board-to-Board Communications

Figure 20 shows the mean path loss for Board to Board communications over Tx-Rx separation for the two different polarizations.

b)

a)

Figure 20: Mean path loss for Board to Board communications
a) Linear vertical polarization b) Left-Hand circular polarization

As depicted above, the path-loss characteristics for board to board communications show the same log-distance dependent behavior as for the chip to chip types. The corresponding parameters for the log-distance path loss models of the two polarizations are:

Table 4: Parameters of the path loss model for Board to Board communications

| Scenario | PLd0 | d0 | γ | RMSE(χg) |
| --- | --- | --- | --- | --- |
| Board to Board, vertical | 16.5 | 0.05 | 1.977 | 1.1502 |
| Board to Board, circular | 16.5 | 0.05 | 1.951 | 1.1437 |

Figure 21 presents the envelopes of the channel impulse responses. Compared to the situation of chip to chip communications, the multipath profile features a similar number of multipath components. However, their amplitude does not reach above -20dB below the main signal and is again effectively attenuated by the application of circularly polarized antennas.

b)

a)

Figure 21: Envelope of the channel impulse responses for Board to Board communications
a) Linear vertical polarization b) Left-Hand circular polarization

# Backhaul/Fronthaul

## Introductory Remarks

The mitigation of the high path loss at 300 GHz requires high gain antennas in the order of 40 dBi at both sides of the link for a transmission distance of several hundred meters. It also requires a LOS connection. In addition such high gain antennas are spatial filters, that suppress multi path propagation at large. This is supported by IEEE document 802.15-15-0681-01-003d in which a simple worst case assessment for the required distance of objects to direct transmission path is illustrated. As long as these minimal distances are met the path loss model provide in section 6.2 is sufficient to evaluate the link budget. Fullfillment of these conditions has to be checked during the planning process, which is required for backhaul and fronthaul links anyhow. This is for example also required due to frequency regulation with national regulators.

## Path loss model

The relevant propagation mechanisms in such an environment, which are contributing to increase the free space loss are described in [6.1]:

- Atmospheric gas attenuation

- Cloud and fog attenuation

- Rain attenuation

For terrestrial links it can be assumed that the link is operated below the height of clouds. The situation that a link penerates clouds may happen for example in some alpine regions with one transceiver at a high mountain, but it is unlikely, that ultra-high capacity links are required there. Therefore the attenuation by clouds may be less relevant. However, the influence of fog may be interest also for dense urban area.

### Calculation of the Overall Path Loss

The overall path loss at a distance d and a carrier frequency f can be modelled as:

(1)

where

### Specific Attenuation by Atmospheric Gases according to ITU-R P.676-10

Two methods are described in ITU-R P.676-10 [6.2]:

- A more detailed line –by-line calculation of gaseous attenuation

- A simplified method, based on curve-fitting of the line-by-line calculation agrees with the more accurate calculations to within an average of about 10% at frequencies removed from the centres of major absorption lines. The absolute difference between the results from these algorithms and the line-by-line calculation is generally less than 0.1 dB/km and reaches a maximum of 0.7 dB/km near 60 GHz.

In the following the specific attenuation due to dry air and water vapour, is estimated using the  simplified algorithms, valid for the frequency range 120 to 350 GHz:

The specific attenuation o due to dry air is calculated using the following equations:

(2)

(3)

(4)

Where		f 	frequency (GHz)

rp  =	ptot/1013, where ptot represents total air pressure

rt  	288/(273  t)

p 	pressure (hPa)

t 	temperature (C)

The specific attenuation w due to water vapor is calculated using the following equations:

(5)

(6)

(7)

(8)

where  is the water-vapor density (g/m3).

Exemplary result for the specific attenuation from 1 to 350 GHz at sea-level for dry air (p=1013 hPa, t=15°C) and water vapor with a density of =7.5 g/m3 (from [6.2]) is shown in Figure 6.1.

Figure 6.1: Exemplary results for specific attenuation due to dry air and water vapour

### Calculation Specific Attenuation R due to Rain according to ITU-R P. 838-3

The specific rain attenuation R is calculated according to according to ITU-R P. 838-3 [6.3]:

(9)

where:

R 	rain rate in mm/h

k  	either kH or kV for horizontal and vertical polarization, respectively

either H or V. for horizontal and vertical polarization, respectively

Values for k and a for the frequencies 200, 300 and 400 GHz and horizontal/vertical polarization are given in Table 6.1

Table 6.1: Values for k and  in the frequency range 200-400 GHz

| Frequency (GHz) | kh | H | kV | V |
| --- | --- | --- | --- | --- |
| 200 | 1.6378 | 0.6382 | 1.6443 | 0.6343 |
| 300 | 1.6286 | 0.6296 | 1.6286 | 0.6262 |
| 400 | 1.5860 | 0.6262 | 1.5820 | 0.6256 |

For linear and circular polarization, and for all path geometries, the coefficients in equation (9) can be calculated from the values given the previous table using the following equations

(10)

(11)

where  is the path elevation angle and  is the polarization tilt angle relative to the horizontal ( = 45° for circular polarization).

Typical rain rates for various rain intensities, which are required in equation (9)  are listed in table 6.2.

Table 6.2: Typical rain rates [6.1, 6.4]

| Type of Precipitation | Range of R (mm/h) | Intensity |
| --- | --- | --- |
| Drizzle | R < 0,1 | Light |
| Drizzle | 0,1 < R < 0,5 | Moderate |
| Drizzle | R > 0,5 | Heavy |
| Rain | R < 2,5 | Light |
| Rain | 2,5 < R < 10 | Moderate |

Exemplary results for specific rain attenuation R at  the carrier frequencies 200, 300 and 400 GHz are listed in Table 6.3

Table 6.3: Exemplary results for specific rain attenuation R

| f/GHz | Horizontal Polarisation |  |  | Vertical Polarisation |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | R/ mm/h |  |  | R/mm/h |  |  |
|  | 0,1 | 5 | 50 | 0,1 | 5 | 50 |
| 200 | 0,38 | 4,57 | 19,89 | 0,38 | 4,56 | 19,66 |
| 300 | 0,38 | 4,49 | 19,12 | 0,39 | 4,46 | 18,87 |
| 400 | 0,38 | 4,35 | 18,37 | 0,37 | 4,33 | 18,28 |

### Calculation of Attenuation due to Clouds and Fog

A calculation method is described in ITU-R 840-6 [6.5]:

The specific attenuation within a cloud or fog can be written as:

dB/km	(12)

where:

c :	specific attenuation (dB/km) within the cloud

Kl :	specific attenuation coefficient ((dB/km)/(g/m3))

M :	liquid water density in the cloud or fog (g/m3).

At frequencies of the order of 100 GHz and above, attenuation due to fog may be significant. Typical water content for different fog types are listed in table 6.5.

Table 6.4: Typical liquid water density of fog types [6.5]

| Fog type | Typical liquid water density in g/m3 |
| --- | --- |
| medium fog (visibility of the order of 300 m) | 0.05 |
| thick fog (visibility of the order of 50 m) | 0.5 |

A mathematical model based on Rayleigh scattering, which uses a double-Debye model for the dielectric permittivity  ( f ) of water, can be used to calculate the value of Kl for frequencies up to 1 000 GHz:

(dB/km)/(g/m3)	(13)

where f is the frequency (GHz), and:

(14)

The complex dielectric permittivity of water is given by:

(4)

(15)

where:

0 =  77.66  +  103.3 (  –  1)	(16)

1 = 0.0671ε0	(17)

2 = 3.52	(18)

= 300 / T	(19)

with T the temperature (K).

The principal and secondary relaxation frequencies are:

fp  =  20.20 – 146  ( – 1) + 316 ( – 1)2                GHz	(20)

fs  =  39.8fp                GHz	(21)

In [6.6] some values for the average liquid water content of clouds are given, see table 6.5

Table 6.5: Liquid water content of cloud types [6.6]

| Cloud type | Average water content in g/m3 |
| --- | --- |
| large cumulus | 2.5 |
| fair weather cumulus | 0.5 |
| Stratocumulus | 0.2 |
| Stratus | 0.2-0.3 |
| Altostratus | 0.2 |

## Antenna gain/pattern

In order to mitigate multipath propagation the applied antennas are required to have no side lobes which are attenuated by less than 30 dB compared to the main antenna lobe. High gain antennas with very small HPBWs of some few degrees, e.g. a dish antennas, usually do not have any sidelobes.

## Scenario Definitions

In [6.7] weather conditions in six cities with different climatic conditions are described yielding also different specific attenuations. Table 6.6 lists these weather conditions and the corresponding specific attenuations together with the name of the channel model. The water vapour density is calculated using the online tool provided at [6.8.]

Table 6.6: Definition of Channel Models for Backhaul/Fronthaul [6.6]

| Channel Model Name | Description in [6.7] | Water vapour density [g/m3] | Rain rate [mm/h] | Liquid water density in fog [g/m3] | Liquid water content of a cloud [g/m3] |
| --- | --- | --- | --- | --- | --- |
| CM-BFH 1 | Bangkok, temperature 35° C, relative humidity 90% | 37.5 | n/a | n/a | n/a |
| CM-BFH 2 | Basra, temperature 43° C, relative humidity 30%, dust (10 m visibility) | 28.2 | n/a | 0.5 | n/a |
| CM-BFH 3 | Berkeley, temperature 20° C, relative humidity 60%, fog (100m visibility) | 10.5 | n/a | 0.14 | n/a |
| CM-BFH 4 | Bellingham, temperature 22° C, relative humidity 50%, rain (4mm/h) | 9.8 | 4 | n/a | n/a |
| CM-BFH 5 | Boulder, temperature 20° C, relative humidity 44% | 8.6 | n/a | n/a | n/a |
| CM-BFH 6 | Buffalo, temperature -10° C, relative humidity 30% | 0.5 | n/a | n/a | n/a |
| CM-BFH 7h / 7v | Boulder including clouds (100 m of large cumulus clouds), temperture 20° C, relative humidity 44%; | 8.6 | n/a | n/a | 2.5 |

# Data Center

The wireless data center uses wireless data links to replace/complement the traditional cable connections, which brings various advantages e.g. high flexibility, low maintenance cost and favorable cooling environment. The high data rate requirement makes the THz technology a competitive candidate because of its high available bandwidth up to 50 GHz.

This document provides a realistic THz wireless channel model in a typical wireless data center scenario. The results presented here are based on [7.1] and [7.2].

As shown in Figure 7, the scenario consists of many server chassis (we assume the standard 1U rackmount chassis in this document), 4 walls and a roof (the 2 front walls and the ceiling are set invisible to illustrate the chassis). The stack height is assumed to be 1.8 m whereas the distance between 2 chassis in the x direction is 0 and in the y direction is 0.5 m. The transmitter (Tx) and the receiver (Rx) are marked as blue circles. A ray tracing simulator is applied to generate the THz channel model. Details of this ray tracing simulator are available in [7.3]. In our scenario, the material parameters of the wall and ceiling are taken from [7.3] whereas the chassis is assumed to be a perfect conductor. The floor is believed to absorb the signal. Using the ray tracing simulator calibrated for the frequency 300 GHz, the propagation channel can be obtained. In Figure 7.1, the propagation paths are illustrated as blue lines.

Figure7.1: The data center scenario

## Propagation Path Types

Figure 7.2 illustrates the possible propagation path types. When the antennas are located on the chassis roof, the signal can be transmitted in a Line of Sight (LoS) path (type 1), or reflected on the ceiling (type 2). In case that Tx and Rx are placed on identical or adjacent chassis, the antenna can be mounted below the chassis roof (type 3) and the propagation path is either LoS or via a reflector to reduce the interference on the propagation path type 1 and 2.

Figure7.2: Propagation path types

## Selection Between Path Types

When Tx and Rx are on identical or adjacent chassis, path type 3 would have advantage over type 1 and 2 because the lower antenna position produces less interference on other channels. If Tx and Rx are further departed therefore the antennas have to be placed on the chassis roof, type 2 is favorable if the propagation distance is limited whereas type 1 shows more advantage over a longer range. This selection is based on 2 considerations: 1) a shorter distance results in less free space propagation loss and therefore allows for additional reflection loss, 2) the elevation of path type 2 deviates from the horizontal direction more significantly with a shorter horizontal direction, therefore a vertically directive antenna would cause less interference on the horizontal LoS paths (because all the chassis have the identical size). We make the general suggestion that if the AoD/AoA elevation is at least 2 times the antenna Half Power Beam Width (HPBW) in the vertical direction away from the horizontal direction, type 2 has an advantage over type 1. The criterion should be adapted for every concrete scenario.

## Stochastic Channel Modelling

The stochastic channel modelling is based on massive ray tracing simulations. We choose a Tx position in the room corner (Tx 1) and in the room center (Tx 2) for propagation path type 1/2. For path type 3, we selected Tx and Rx positions randomly on identical or adjacent chassis.

Based on the simulation results, we derive a stochastic channel model in the following approach:

- Determine number of propagation paths.

- Assign a delay to each propagation path.

- Determine the pathloss of each propagation path according to its delay.

- Define the angular difference of each NLoS path to the LoS path.

- Generate uniformly distributed phase for each path.

- Generate frequency dispersion for each path.

In the following sections, we will explain the process step by step to obtain the stochastic channel model.

### Path Numbers

There is always 1 LoS path. The empirical distributions of the numbers of NLoS paths are presented in Table 7.1.

Table 7.1 NLoS Path number distributions

| Type 1/2, Tx 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of paths | 17 |  |  | 18 |  |  | 19 |  |  |  | 20 |  |  | 21 |  |  |
| Probability (%) | 27 |  |  | 35 |  |  | 22 |  |  |  | 15 |  |  | 1 |  |  |
| Type 1/2, Tx 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of paths | 16 |  | 17 |  |  | 18 |  |  | 19 |  |  | 20 |  |  | 21 |  |
| Probability (%) | 32 |  | 29 |  |  | 12 |  |  | 16 |  |  | 8 |  |  | 3 |  |
| Type 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of paths | 3 | 4 |  |  | 5 | 6 |  | 7 |  | 8 |  | 9 | 10 |  |  | 11 |
| Probability (%) | 22 | 13 |  |  | 8 | 15 |  | 8 |  | 17 |  | 8 | 6 |  |  | 3 |

### Delay distribution

Figire 7.3illustrates the delay distributions. Note that the LoS delay is the absolute value whereas the NLoS delay is the relative delay, i.e. the difference between the NLoS delay and the corresponding LoS delay.

| (a) Type 1/2, Tx 1 | (b) Type 1/2, Tx 2 Figure 7.39: Delay distributions | (c) Type 3 |
| --- | --- | --- |

Table 7.2 lists the distribution types and the corresponding parameter values.

Table 7.2 Delay distributions

| Path | Distribution | Parameters |
| --- | --- | --- |
| Type 1/2, Tx 1, LoS | Normal distribution | =2.26x10-8, =8.76x10-9 |
| Type 1/2, Tx 1, NLoS | Negative EXP | =8.76x109 |
| Type 1/2, Tx 2, LoS | Normal distribution | =1.20x10-8, =4.56x10-9 |
| Type 1/2, Tx 2, NLoS | Normal distribution | =2.98x10-8, =1.79x10-9 |
| Type 3, LoS | Normal distribution | =1.80x10-8, =8.60x10-9 |
| Type 3, NLoS | Negative EXP | =4.92x107 |

### Delay-Pathloss Correlation

The delay has a positive correlation with the pathloss, as depicted in Figure 7.40. As in the last section, the pathlosses and delays for the LoS paths are absolute values whereas the NLoS carries relative pathlosses and delays. The definition of the relative pathloss is the pathloss of the considered path divided by the pathloss of the corresponding LoS pathloss.

| (a) Type 1/2, Tx 1 | (b) Type 1/2, Tx 2 Figure7.4: Delay-pathloss distributions | (c) Type 3 |
| --- | --- | --- |

Table 7.3 illustrates the relationship between delay and pathloss. The subscript “r” stands for “relative”. The relationship between delay and path loss for the LoS paths can be completely described by the Friis equation. Therefore the random part is 0. For the NLoS paths, the additional loss due to reflections etc. contributes to the random part.

Table 7.3 Delay-pathloss relationship

| Path | Deterministic part | Random part |
| --- | --- | --- |
| Type 1/2, Tx 1, LoS | p=-20log10(d)-71.52 | =0 |
| Type 1/2, Tx 1, NLoS | pr=-0.294dr-17.44 | =4 |
| Type 1/2, Tx 2, LoS | p=-20log10(d)-71.52 | =0 |
| Type 1/2, Tx 2, NLoS | pr=-0.385dr-17.95 | =4 |
| Type 3, LoS | p=-20log10(d)-71.52 | =0 |
| Type 3, NLoS | pr=-0.429dr-30.30 | =6 |

With delays for every path available from the last section, the pathloss can be derived from Table 3.

### Pathloss-angle Correlation

The simulation shows some certain degree of correlation between pathloss and the angular difference between the considered NLoS path and the corresponding LoS path. This correlation is important because it has impact on the spatial filtering performance of the directive antennas. The relative probabilities of antenna angles as a function of path loss are depicted in Figure 7.5and the numbers are listed in Table 7.4.

| (a) Type 1/2, Tx 1 | (b) Type 1/2, Tx 2 Figure 7.5: Relative Probabilities of Antenna Angle as a Function of Path Loss | (c) Type 3 |
| --- | --- | --- |

Table 7.4: Relative Probabilities of Antenna Angles versus Path Loss

| Type 1/2, Tx 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative pathloss (dB) |  |  |  |  |  |  |  |  |
| Angular difference () | -70 | -60 | -50 | -40 | -30 | -20 | -10 | 0 |
| 0 | 0.000 | 0.000 | 0.054 | 0.062 | 0.065 | 0.014 | 0.257 | 0.000 |
| 10 | 0.000 | 0.000 | 0.023 | 0.029 | 0.082 | 0.067 | 0.274 | 0.360 |
| 20 | 0.000 | 0.000 | 0.000 | 0.020 | 0.052 | 0.061 | 0.031 | 0.360 |
| 30 | 0.000 | 0.118 | 0.008 | 0.058 | 0.113 | 0.082 | 0.120 | 0.280 |
| 40 | 0.000 | 0.000 | 0.000 | 0.031 | 0.084 | 0.055 | 0.067 | 0.000 |
| 50 | 0.000 | 0.000 | 0.023 | 0.036 | 0.039 | 0.084 | 0.036 | 0.000 |
| 60 | 0.000 | 0.118 | 0.078 | 0.016 | 0.030 | 0.131 | 0.016 | 0.000 |
| 70 | 0.000 | 0.059 | 0.085 | 0.062 | 0.047 | 0.178 | 0.000 | 0.000 |
| 80 | 0.000 | 0.000 | 0.109 | 0.102 | 0.131 | 0.090 | 0.000 | 0.000 |
| 90 | 0.000 | 0.059 | 0.132 | 0.122 | 0.080 | 0.027 | 0.023 | 0.000 |
| 100 | 0.000 | 0.059 | 0.070 | 0.049 | 0.049 | 0.033 | 0.031 | 0.000 |
| 110 | 0.249 | 0.059 | 0.078 | 0.049 | 0.039 | 0.027 | 0.029 | 0.000 |
| 120 | 0.000 | 0.059 | 0.062 | 0.067 | 0.026 | 0.023 | 0.020 | 0.000 |
| 130 | 0.249 | 0.176 | 0.101 | 0.084 | 0.026 | 0.020 | 0.022 | 0.000 |
| 140 | 0.249 | 0.000 | 0.047 | 0.067 | 0.025 | 0.019 | 0.015 | 0.000 |
| 150 | 0.249 | 0.235 | 0.062 | 0.062 | 0.032 | 0.017 | 0.013 | 0.000 |
| 160 | 0.000 | 0.059 | 0.039 | 0.027 | 0.027 | 0.027 | 0.015 | 0.000 |
| 170 | 0.000 | 0.000 | 0.016 | 0.029 | 0.029 | 0.027 | 0.019 | 0.000 |
| 180 | 0.000 | 0.000 | 0.016 | 0.026 | 0.024 | 0.019 | 0.013 | 0.000 |

| Type 1/2, Tx 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative pathloss (dB) |  |  |  |  |  |  |  |  |
| Angular difference () | -70 | -60 | -50 | -40 | -30 | -20 | -10 | 0 |
| 0 | 0.053 | 0.000 | 0.026 | 0.017 | 0.054 | 0.015 | 0.021 | 0.000 |
| 10 | 0.053 | 0.000 | 0.000 | 0.017 | 0.070 | 0.027 | 0.041 | 0.051 |
| 20 | 0.053 | 0.000 | 0.053 | 0.044 | 0.093 | 0.045 | 0.079 | 0.039 |
| 30 | 0.053 | 0.000 | 0.053 | 0.048 | 0.075 | 0.123 | 0.100 | 0.000 |
| 40 | 0.053 | 0.000 | 0.026 | 0.065 | 0.074 | 0.089 | 0.108 | 0.100 |
| 50 | 0.053 | 0.000 | 0.026 | 0.072 | 0.050 | 0.065 | 0.121 | 0.248 |
| 60 | 0.053 | 0.000 | 0.053 | 0.075 | 0.060 | 0.056 | 0.137 | 0.129 |
| 70 | 0.053 | 0.000 | 0.132 | 0.140 | 0.055 | 0.035 | 0.165 | 0.000 |
| 80 | 0.053 | 0.000 | 0.184 | 0.130 | 0.051 | 0.117 | 0.085 | 0.003 |
| 90 | 0.053 | 0.748 | 0.145 | 0.106 | 0.093 | 0.065 | 0.034 | 0.039 |
| 100 | 0.053 | 0.000 | 0.066 | 0.072 | 0.039 | 0.054 | 0.023 | 0.071 |
| 110 | 0.053 | 0.000 | 0.000 | 0.027 | 0.043 | 0.053 | 0.020 | 0.058 |
| 120 | 0.053 | 0.000 | 0.053 | 0.041 | 0.056 | 0.056 | 0.012 | 0.058 |
| 130 | 0.053 | 0.000 | 0.092 | 0.068 | 0.045 | 0.034 | 0.013 | 0.039 |
| 140 | 0.053 | 0.000 | 0.039 | 0.041 | 0.046 | 0.035 | 0.012 | 0.045 |
| 150 | 0.053 | 0.250 | 0.026 | 0.010 | 0.030 | 0.029 | 0.013 | 0.026 |
| 160 | 0.053 | 0.000 | 0.000 | 0.000 | 0.028 | 0.041 | 0.005 | 0.058 |
| 170 | 0.053 | 0.000 | 0.000 | 0.014 | 0.023 | 0.031 | 0.007 | 0.019 |
| 180 | 0.053 | 0.000 | 0.026 | 0.014 | 0.016 | 0.027 | 0.006 | 0.019 |

| Type 3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative pathloss (dB) |  |  |  |  |  |  |  |  |
| Angular difference () | -70 | -60 | -50 | -40 | -30 | -20 | -10 | 0 |
| 0 | 0.062 | 0.045 | 0.000 | 0.000 | 0.000 | 0.000 | 0.003 | 0.002 |
| 10 | 0.000 | 0.091 | 0.049 | 0.054 | 0.005 | 0.006 | 0.000 | 0.000 |
| 20 | 0.125 | 0.136 | 0.024 | 0.027 | 0.005 | 0.004 | 0.000 | 0.000 |
| 30 | 0.062 | 0.091 | 0.024 | 0.000 | 0.016 | 0.000 | 0.004 | 0.003 |
| 40 | 0.000 | 0.091 | 0.073 | 0.000 | 0.033 | 0.013 | 0.006 | 0.006 |
| 50 | 0.000 | 0.136 | 0.000 | 0.000 | 0.030 | 0.006 | 0.047 | 0.028 |
| 60 | 0.000 | 0.000 | 0.049 | 0.000 | 0.025 | 0.011 | 0.013 | 0.006 |
| 70 | 0.000 | 0.000 | 0.000 | 0.054 | 0.027 | 0.032 | 0.003 | 0.047 |
| 80 | 0.000 | 0.000 | 0.122 | 0.000 | 0.019 | 0.015 | 0.131 | 0.069 |
| 90 | 0.000 | 0.091 | 0.439 | 0.675 | 0.692 | 0.877 | 0.780 | 0.820 |
| 100 | 0.000 | 0.182 | 0.049 | 0.000 | 0.005 | 0.004 | 0.004 | 0.001 |
| 110 | 0.062 | 0.000 | 0.049 | 0.081 | 0.033 | 0.000 | 0.001 | 0.002 |
| 120 | 0.125 | 0.091 | 0.073 | 0.000 | 0.033 | 0.002 | 0.000 | 0.005 |
| 130 | 0.187 | 0.000 | 0.024 | 0.081 | 0.027 | 0.004 | 0.003 | 0.009 |
| 140 | 0.062 | 0.000 | 0.024 | 0.027 | 0.025 | 0.000 | 0.003 | 0.001 |
| 150 | 0.187 | 0.000 | 0.000 | 0.000 | 0.016 | 0.000 | 0.000 | 0.000 |
| 160 | 0.062 | 0.045 | 0.000 | 0.000 | 0.005 | 0.006 | 0.000 | 0.000 |
| 170 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.019 | 0.000 | 0.000 |
| 180 | 0.062 | 0.000 | 0.000 | 0.000 | 0.003 | 0.000 | 0.001 | 0.000 |

The angular difference can be determined given the pathloss from the last section.

### Phase and Frequency Dispersion

The phase can be safely assumed to be uniformly distributed. The frequency dispersion can be described by

gf=g0f0f

where f0 and g0 are the reference frequency and the channel gain at the reference frequency, respectively.

## Scenario Definitions

In table 7.5 the scenarios for the concrete channel models for simulations in Wireless Data Centers are defined.

Table 7.5: Definition of concrete Channel Models for Wireless Data Centers

| Channel Model Name | Path Type |
| --- | --- |
| CM-WDC 1 | Type 1/2, Tx1, LoS |
| CM-WDC 2 | Type 1/2, Tx1, NLoS |
| CM-WDC 3 | Type 1/2, Tx 2, LoS |
| CM-WDC 4 | Type 1/2, Tx 2, NLoS |
| CM-WDC 5 | Type 3, LoS |
| CM-WDC 6 | Type 3, NLoS |

# Reference

[4.1] Application Requirements Document, IEEE 802.15-15-0304-14-003d

[4.2] Ken Hiraga, Masasih Shimizu, Toshimitsu Tsubaki, Hideki Toshinaga and Tadao Nakagawa, “Real usage of the kiosk downloading,” IEEE80.15-14-0298-00-003d, Beijing, Mar 2014.

[5.1] A. Fricke, “Measuring the Terahertz Intra-Device Propagation Channel”, IEEE 802.15-15-0166-00-003d, Berlin, March 2015

[5.2] A. Fricke, S. Rey, M. Achir, P. Le Bars, T. Kleine-Ostmann, T. Kürner, "Reflection and Transmission Properties of Plastic Materials at THz Frequencies", 38th International Conference on Infrared, Millimeter and Terahertz Waves, Mainz, September 2013

[5.3] A. Fricke, “Direct and Directed NLOS Channel Measurements for Intra-Device Communications”, IEEE 802.15-15-0528-00-003d, Waikoloa Village, July 2015

[5.4] R. Jones, “A new Calculus for the Treatment of Optical Systems”, Journal of the

optical Society of America, vol. 32, no. 7, pp. 500-503, 1941

[6.1] G. A. Siles, J. M. Riera, P. Garcia-del-Pino, Atmospheric Attenuation in Wireless Communication Systems at Millimeter and THz Frequencies, IEEE Antennas and Propagation Magazine, Vol. 57, No. 1, February 2015, pp. 48-59

[6.2] Rec. ITU-R P.676-10, Attenuation by atmospheric gases, 2013

[6.3]Rec. ITU-R P.838-3, Specific attenuation for rain for use in prediction methods, 2005

[6.4] Guide to Meteorological Instruments and Methods of Observation, World Meteorological Organization (WMO), Geneva, Switzerland, 2008.

[6.5] Rec. ITU-R P.840-6, Attenuation due to clouds and fog, 2013

[6.6] H. J. aufm Kampe, Visibility and Liquid Water Content in the free Atmosphere, Journal of Meteorology, Vol. 7, p. 54-57, February 1950

[6.7] M. Rosker,Progress towards a THz imager, IMS 2007, Workshop WFE, “THz Device Characterization and security applications”, 8 June 2007, slide 5.

[6.8] http://www.ib-rauch.de/bautens/formel/abs_luftfeucht.html (visited on November 8, 2015)

[7.1] B. Peng, “A Stochastic THz Channel Model in Wireless Data Centers“ doc.: 802.15-15-0207-003d Stochastic Channel Model for Wireless Data Center

[7.2] B. Peng, T. Kürner, “A Stochastic THz Channel Model for Future Wireless THz Data Centers”, 12th International Symposium on Wireless Communication Systems, Brussels, August 2015

[7.3] S. Priebe, M. Jacob, T. Kürner, “Calibrated broadband ray tracing for the simulation of wave propagation in mm and sub-mm wave indoor communication channels,” in European Wireless, 2012. EW. pp. 1-10, VDE, 2012.

Based on the calculation using [6.8]

Based on table 6.5

The lower limit for the visibility guven in table 6.4 is 50m. The liquid water density assumed here is for a visibility of 50m.

By linear interpolation from the values given in table 6.4

CM-BFH 7h(7v) channel model for horizontal (vertical) polarization;

The assumption on clouds is not taken from [6.7] and instead assumed by TG3d in order to have scenario including clouds