IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4s Technical Guidance Document |  |
| Date Submitted | [21 April, 2015] |  |
| Source | [Shoichi Kitazawa] [ATR] [2-2-2, Hikaridai, Seika, Kyoto Japan] | Voice: [ ] Fax: [ ] E-mail: [ kitazawa@atr.jp ] |
| Re: | [ ] |  |
| Abstract | [Working draft for TG4s technical guidance document] |  |
| Purpose | [Summaries of technical topics discussed at TG4s] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Document Overview

This technical document provides summary of contributions to SG SRU and TG4s regarding Spectrum Resource Measurement and Management.

| List of contributors |  |
| --- | --- |
| Masayuki Ariyoshi |  |
| Shusaku Shimada |  |
| Mineo Takai |  |
| Takeshi Yamamoto |  |
| Mitsuru Iwaoka |  |
| Hidetoshi Yokota |  |

Table of Contents

1	Overview	4

2	Definitions	4

3	Abbreviation and acronyms	4

4	General descriptions	4

4.1	Summary of PAR	4

4.2	General requirement of SRMM	5

4.3	Spectrum Resource Measurement Report Structure	5

4.4	Use Case	6

4.4.1	Use Case1:Multiple PAN	6

4.4.2	Use Case 2: Anomaly Detection	6

4.4.3	Use Case 3: Resource management for Neighborhood Area Network (NAN)	7

4.4.4	Use Case 4: Cluster Tree	8

4.4.5	Use Case 5:	8

5	Functional requirements	9

5.1	Spectrum Resource Measurement	10

6	Measurement metrics	10

6.1	PHY	10

6.2	MAC	11

7	References	12

Appendix A.	Applications	13

A.1	 Hospital/Medical/Healthcare	13

A.2	Industrial Automation	13

A.3	Infrastructure Monitoring	14

A.4	Advanced Metering Infrastructure (AMI)	15

Appendix B.	Related Standards	15

- Overview

This technical guidance document provides summary of MAC related functions to enable spectrum resource measurement and management for 802.15.4. The TG4s aims at building a standard to specify the MAC framework for exchanging information for the better use of spectrum resource usage.

- Definitions

This section provides definitions of terms that discussed in TG4s.

spectrum resource measurement:

spectrum resource measurement and management:

- Abbreviation and acronyms

The following abbreviations are used in this document.

LR-WPAN	Low-Rate WPAN

RFD		Reduced Function Device

SRM		Spectrum Resource Measurement

SRMM		Spectrum Resource Measurement and Management

SRU		Spectrum Resource Utilization

TGD		Technical Guidance Document

WPAN		Wireless Personal Area Network

- General descriptions

This section provides the basic framework for Spectrum Resource Measurement and Management (SRMM).

  - Summary of PAR

Title:

Standard for Low-Rate Wireless Networks: Amendment Enabling Spectrum Resource Measurement Capability

Scope of the project:

This amendment to IEEE Std 802.15.4 defines MAC related functions to enable spectrum resource management.

It specifies

- spectrum resource measurements and network performance metrics, such as packet error ratio, delay, etc,

- information elements and data structures to capture these measurements,

- procedures for collecting and exchanging spectrum resource measurement information with higher layers or other devices.

Need for the Project:

As various wireless systems are deployed in the shared and license exempt frequency bands including 2.4GHz and 915MHz bands, heavy interference has limited performance of the wireless systems. In order for these wireless systems to operate more effectively, a standardized set of spectrum resource measurements is needed that will facilitate management functions in these networks.

Stakeholders for the Standard:

The stakeholders include manufacturers and users of telecom, medical, environmental, energy, and consumer electronics equipment and manufacturers and users of equipment involving the use of wireless sensor and control networks.

The PAR can be found on the IEEE Standard Association Website.

https://development.standards.ieee.org/P875300033/par

  - General requirement of SRMM

This section describes Spectrum Resource Measurement and Management (SRMM)

  - Spectrum Resource Measurement Report Structure

SRM report header should include at least the following:

- Reporting device ID, PAN ID

- Measurement time

- Number of measurements included in the report

SRM report payload can include multiple pairs of measurement metric type and value as long as those measurements were performed by the same device at the same time.

All the measurement metric types and the details (precision, unit etc.) of their associated value(s) are to be defined in TG4s.

Figure 1 SRM report payload

  - Use Case

The group discusses following categories of the SRM use case.

- The Network manager (Next Higher Layer) request to the PAN coordinators about spectrum resource information.

- PAN coordinator report to the Network manager (Next Higher Layer) about anomaly detection

    - Use Case1:Multiple PAN

This use case depicts coexistence with multiple PAN’s.

Each PAN coordinator notifies the spectrum resource usage information to the Network manager. The network manager may use the spectrum resource usage information for controls PHY and MAC parameter.

Figure 2 Multiple PAN

An Initial Proposal of Reference Architecture for TG4s (15-15-027)

    - Use Case 2: Anomaly Detection

- Co-existing networks, e.g. IEEE802.11 and 15.4e (ISA100.11a),  are able to share the alien system information each other.

  - Increased noise energy using energy detection

  - Overlapping network activity using carrier sense

- Jabber or jamming node may be detected.

  - Suspicious behavior or ordinary mobile demeanor of alien node can be estimated.

  - Sometimes notification storm has to be suppressed, e.g. mobile worker carrying wireless gadgets.

- Security threat may be alleviated, for instance;

  - detection of  “replay attack” node

  - detection of  “active spoofing” node

Figure 3 Anomaly detection

## Vigilance of Spectrum Resource Usage in private facilities for network stability and security (15-15-040)

    - Use Case 3: Resource management for Neighborhood Area Network (NAN)

- Large-scale NAN is composed of multiple PANs

  - When a new smart meter (end point) is installed, it will join one of those PANs in their footprints

  - Selection and reselection of the PAN to be joined is determined with certain criteria

  - Structure of RF mesh changes from time to time

- Stable performance among multiple PANs is desired

  - Well-balanced SRU (Spectrum Resource Usage) is desired for each end point to show as much the same performance as possible regardless of the available PANs

  - Instantaneous QoS fluctuation should not affect the behavior of those end points

Figure 4 Neighborhood Area Network

Use case proposal for Technical Guidance Document (15-15-177)

    - Use Case 4: Cluster Tree

- Each PAN Coordinator collect SRM information from device.

  - RFD (Reduced Function Device) should forward SRM information

- The SRM information should include PAN ID.

Figure 5 Cluster Tree

Additional TG4s use case and consideration of SRM information (15-15-199)

    - Use Case 5:

- Functional requirements

This chapter describes functional requirement of SRMM.

Table 1 Spectrum Resource Measurement Requirement

  - Spectrum Resource Measurement

The Spectrum Resource Measurement provides the following

- Measurement results of channel load and/or link quality

- Anomaly detection

Figure 6 LR-WPAN device architecture

Figure 6 shows typical LR-WAPN device architecture.

  - MAC PIB

General MAC PIB attributes for functional organization

| Attribute | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| macSRMcapable | Boolean | TRUE, FALSE | If TRUE, the device is capable of functionality specific to SRM | — |

- Measurement metrics

This chapter describes measurement metrics for TG4s.

  - PHY

  - MAC

# References

- Kitazawa, Establishing a Study Group for a Spectrum Resource Utilization (SRU) through Radio Resource Measurement and Management for WPANs, IEEE 802.15-13-404r1

- Kitazawa, Overview of SG SRU, IEEE 802.15-13-543r0

- Additional use case of temporal and flexible industrial network deployment

- Additional use case of temporal and flexible industrial network deployment, IEEE 802.15-13-654r1

- Yamamoto et al., Simulation Methodology for SRU, IEEE 802.15-13-660r0

- Proposal of radio resource management architecture(15-13-0285r1)

- A Use Case of Self-Organizing Wireless Network for Medical System(15-13-306-0)

- IG SRU Working Draft RRMM-usecases and 5C(15-13-0294r1)

- IG SRU Usecase requirements table(15-13-0293r1)

- An Initial Proposal of Reference Architecture for TG4s (15-15-027)

- Overview of TG4s Spectrum Resource Usage (15-15-028)

- Vigilance of Spectrum Resource Usage in private facilities for network stability and security (15-15-040)

- Metrics used in some Wireless Sensor Network standards (15-15-72)

- Proposed Spectrum Resource Measurement Report Structure (15-15-086)

- Spectrum Resource Measurement and Management requirement table (15-15-089)

- Use case proposal for Technical Guidance Document (15-15-177)

- Additional TG4s use case and consideration of SRM information (15-15-199)

- Applications

## A.1	 Hospital/Medical/Healthcare

Various wireless systems (WLAN, Bluetooth etc.) are deployed in medical environments.

Requirement

- Self-organizing wireless system

- Spectrum sensing of whole band for searching vacant radio resources

- Radio resource assignment and network topology management

Figure 7 Image of medical information system in hospital

## A.2	Industrial Automation

Requirement

- Quick additional deployment

- Quick withdrawal, relocation

- Inexpensive and reusable

Figure 8 Industrial wireless network

## A.3	Infrastructure Monitoring

Requirement

Figure 9

## A.4	Advanced Metering Infrastructure (AMI)

- A number of smart meters construct Neighborhood Area Network (NAN) in the form of RF mesh in order to send meter reading data for billing or event data for management as well as to receive commands for energy and device control

- Requirements

  - Stable network performance under dynamic mesh structure change

  - Centralized and distributed resource usage control mechanism

  - Assured QoS provision as a commercial communication infrastructure

Figure 10

- Related Standards