IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft IG LPWA Report |  |
| Date Submitted | [The date the document is contributed, in the format “21 May, 1999”] |  |
| Source | [Joerg Robert] [FAU Erlangen-Nuernberg] [address] | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: | [If this is a proposed revision, cite the original document.] [If this is a response to a Call for Contributions, cite the name and date of the Call for Contributions to which this document responds, as well as the relevant item number in the Call for Contributions.] [Note: Contributions that are not responsive to this section of the template, and contributions which do not address the topic under which they are submitted, may be refused or consigned to the “General Contributions” area.] |  |
| Abstract | [Description of document contents.] |  |
| Purpose | [Description of what the author wants P802.15 to do with the information in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Draft IG Low Power Wide Area Report

Date: 2017-xx-xx

# Abstract

# Table of Contents

1 Introduction	4

2 Technical Characteristics of Low Power Wide Area Networks	5

Theoretical Bound on Maximum Payload Bit-Rate	6

Technical Challenges of LPWAN	7

3 Potential Use-Cases for Low Power Wide Area Networks	8

Agriculture and Environmental	8

Consumer/Medical	8

Industrial	8

Infrastructure	8

Logistics	9

Smart Building	9

Smart City	9

4 Frequency Regulation and Channel Models	10

4.1 Frequency Regulation	10

FCC (United States)	10

ETSI (Europe)	11

4.2 Propagation Models for LPWAN	12

Indoor Model	12

Outdoor Urban Model / Outdoor Rural Model	13

Outdoor Device-to-Device	13

Thermal Noise	13

4.3 Interference Channel Model	14

4.4 Number of Active Users	17

5 Use-Case Evaluation Process	20

6 Analysis of Existing IEEE Standards / Candidate Technologies	22

6.1 Suitability of Candidate Technologies	22

6.2 Suitability Analysis of Existing IEEE Standards	25

6.3 Qualitative Evaluation of Candidate Technologies	28

6.4 Quantitative Evaluation of Candidate Technologies	29

6.5 Evaluation Summary	30

7 Summary and Recommendation for Future WG Activities	31

Annex	32

Literature	33

# 1 Introduction

LPWAN (Low Power Wide Area Networks) have gained increasing interest over the recent years. These networks allow the transmission of data over long distances with low transmit powers, making them appealing for a variety of different applications. This resulted in a list of different proprietary or quasi standards that can be used in license exempt frequency bands, mainly in the range close to 900MHz. On the other hand, also the mobile network operators developed solutions to target LPWAN applications. The MNO can especially take benefit from their already existing networks, which enables them the fast introduction of these services on a large scale.

However, the aforementioned solutions may not be able to cover all market demands. Potential issues are interoperability, system availability over decades, network coverage also in rural areas, and full control of the network and the data.

Interoperability: LPWAN operators and user are interested to use interoperable hardware solutions. The installation of multiple incompatible LPWAN systems to support different LPWAN applications is not desirable. Furthermore, the use of different parallel systems may lead to interference between these systems, potentially resulting in a significantly reduced system performance for all operated LPWAN systems.

System availability over decades: Many LPWAN use-cases, especially related to infrastructure, may be operated over decades. This naturally requires the availability of hardware for further extensions or replacement for reasonable cost over decades, which may not be guaranteed in case of proprietary systems. In addition, also mobile network operators may not guarantee the support of a specific LPWAN standard over decades. Hence, proprietary systems, or systems using mobile networks, may not be suitable for LPWAN applications with long lifetimes.

Network coverage: LPWAN systems may be operated in rural areas. Hence, LPWAN solutions based on mobile networks may not be available due to the missing network coverage.

Full control of the network and data: Some use-cases may require a high reliability that cannot be guaranteed by public networks, e.g. for industrial use-cases. Additionally, also laws may prohibit the use of public networks for certain applications. Furthermore, full control of the may be import in many use-cases. The user may want to ensure that his data is not stored and analyzed by third parties, which cannot be guaranteed if a public network is used.

In summary, the aforementioned requirements can only be fulfilled if a user has to possibility to operate a private LPWAN network. This ensures full control and a long-lasting operation. However, this also requires long-time availability of the hardware, which cannot be achieved by proprietary systems. Therefore, the availability of an open standard that is essential for the success of LPWAN.

The scope of the Interest Group (IG) LPWA has therefore been the evaluation, whether existing IEEE standards are able to offer a suitable support of typical LPWAN use-cases. Furthermore, the IG LPWA analyzed potential technical improvements of existing IEEE standards to give recommendation to the WG 802.15 for further steps.

# 2 Technical Characteristics of Low Power Wide Area Networks

State-of-the-art cellular communication systems are mainly optimized for high payload bitrates with few users. As a result, they are not able to efficient cover many applications with low bitrate requirements per device, but potentially thousands of devices in a given area. Examples for such applications are water/gas metering or environmental monitoring. These applications require low-cost devices, battery lifetimes of several years, and spectrally efficient protocols that are able to support thousands of devices. On the other hand, there are no strict requirements on parameters such as latency, and the number of messages per day is limited. Therefore, systems based on IEEE 802.15.4 with mesh-topology are mainly used for these applications. Potentially alternative solutions to cover these use-cases are Low Power Wide Area Networks (LPWAN). Currently, a list of proprietary systems is available on the market, and also 3GPP introduced LPWAN functionally called NB-IoT. LPWAN systems are mainly based on a star topology, i.e. all devices transmit their data to a single central node. Therefore, multi-hop transmission is no longer required and the complexity of the sensor nodes may be significantly reduced. In extreme cases, sensor nodes might not be even equipped with a receiver module. Due to the missing multi-hop, the distance between the transmitter and receiver increases significantly for comparable network sizes. Therefore, the base-stations are typically using highly exposed antennas.

Figure 1: General concept of LPWA networks consisting of base-station and node

Figure 1 shows a typical LPWAM configuration. Low-cost nodes with low transmit power (e.g. 10mW) transmit their data to a base-station over a distance of several km. The antennas of the base-station may be mounted on highly exposed sites for increased coverage. However, the still remaining high path loss caused by the large distances can only be compensated by very low payload bit-rates of typically few bits per second. However, LPWAN systems may the support the simultaneous transmission of hundreds of sensor nodes. Therefore, the sum bit-rate of the cell may be very high, even if the bit-rate of an individual node is low. For further understanding, the next sections show the theoretical bounds and the resulting challenges of LPWAN networks.

## Theoretical Bound on Maximum Payload Bit-Rate

In general, the successful transmission over an additive white Gaussian noise channel requires a certain Eb/N0 [1]. Here, Eb is the energy per useful bit in Joule, and N0 is the noise spectral density, mainly resulting from thermal noise. The theoretical work of Claude Shannon has shown that successful transmission is theoretically possible if Eb/N0>-1.59dB. If it is less, a successful communication is impossible. Whereas the N0 is mainly given by the thermal noise, and thus cannot be modified, the Eb can be influenced by the user. It is given by Eb=PRX/R, where PRX is the received signal power and R is the payload bit-rate. In many cases, the PRX cannot be improved significantly, as it depends on the distance and the propagation characteristics between transmitter and receiver. Additionally on the transmit power, which may be limited by frequency regulation. Hence, changing the rate R may be the only possibility to improve the Eb. However, this also does not change the energy consumption per bit of the LPWAN sensor node. Therefore, the required energy per transmitted bit drained from a battery does not change. Hence, LPWAN are low power systems, but not necessarily low energy.

Using the above relations, the minimum required signal power can be expressed as a function of the payload bit-rate R and vice versa [2]:

Pmin[dBm]=N0[dBm/Hz]+10log10R[bit/s] -1.59dB

In case of terrestrial reception a noise temperature of T0=300K is typically assumed. This results in a noise spectral density of at least N0=-174dBm/Hz. A received signal power of PRX=-140dBm  would then allow a maximum payload bit-rate of R=3622bit/s [2]. Figure 2 shows the resulting payload bit-rate as function of the received signal power.

However, this is theoretical figure only. First, the noise figure of the receiver has to be taken into account, which typically takes values from 3 to 6dB. Furthermore, the theoretical bound of -1.59dB assumes a spectral efficiency approaching zero in addition to a perfect forward error correction. It has to be mentioned that spreading (e.g. based on DSSS) generally results in a very low spectral efficiency. However, as this can be expressed as a linear code, it does not improve the performance. To obtain this gain, codes with non-linear decoding would be required. An example would be a convolutional code with a code-rate approaching zero [1].

As a result, realistic systems are not able to obtain the theoretical figures, and a gap of 10 to 15dB compared to the theoretical figures may be realistic. Thus, the achievable payload bit-rate may be significantly lower than the theoretical values shown in Figure 2. However, this does only correspond to the payload bit-rate of a single sensor node. As many LPWAN systems allow for a parallel transmission of many sensor nodes (e.g. using frequency division multiple access with very narrow channels), the sum payload bit-rate at the base-station may be quite high.

Figure 2: Maximum theoretical payload bit-rate as function of the reception power in the AWGN channel

## Technical Challenges of LPWAN

The ultra-low payload bit-rates that are required to obtain a long-distance transmission lead to a list of technical challenges that do not exist in many other data transmission systems that use higher payload bit-rates:

Frequency offset: State-of-the-art crystal oscillators have a frequency tolerances in the order of typically 20ppm, which results in frequency offsets of 18 kHz if a transmit frequency of 900 MHz is assumed. However, this frequency offset may be significantly higher than the signal bandwidth, which may be significantly less than 1 kHz for low bit-rates. Hence, the signal detection and the synchronization complexity may increase significantly.

Hidden node problem: One sensor-node is typically not able to detect the transmissions of other sensor nodes, as the nodes are normally not equipped with exposed antennas. Therefore, means as carrier sense multiple access do not work. The sensor-nodes will sense the channel as free, but the packets will collide at the exposed base-station antenna. Therefore, other means are required to avoid or resolve collision of the signals of multiple nodes.

Frequency regulation: The use of license-exempt frequency bands is commonly coupled to constraints on different parameters such as duty cycle, maximum transmit duration, or transmit power. This may lead to significant mismatches between the uplink and the downlink, where the downlink to the sensor-nodes is the problematic part [2]. Therefore, base-stations may receive the data of thousands of sensor-nodes simultaneously, but they are only able to send few downlink messages.

Interference: License-exempt frequency bands are also used by other systems that may interfere with the LPWAN system. Caused by the low-data rates, transmissions may last over several seconds. This large spectral foot-print increases the probability that the communication is interfered by other systems.

# 3 Potential Use-Cases for Low Power Wide Area Networks

The low-power consumption and the long transmit range of LPWAN offer new applications that may not be covered by other existing standards with the same efficiency. The IG LPWA therefore discussed different potential use-cases for LPWAN coming from different domains [3]. Similar use cases have also been mentioned in e.g. [4, p. 15]. Generally, the number of potential use-cases is not limited to the listed use-cases. However, this list is a cross section of different domains that allows for a later analysis whether LPWAN system are efficiently able to cover specific domains.

### Agriculture and Environmental

The domain agriculture and environmental is characterized by rural areas with a low population density. The aim is the collection of information from distributed sensors that may be spread over large areas. Typical antenna configurations for the base-station are outdoor roof-top antennas.

Cattle Monitoring: Localization and transmission of health information using LPWAN

Field Monitoring: Transmission of sensor data on soil parameters such as humidity, temperature, soil moisture, etc.

### Consumer/Medical

The domain consumer/medical is characterized by used in urban areas with a potentially high population density. The base-stations are used in a similar way as WiFi access points. Depending on the application, the base-station antennas may be placed indoors or outdoors.

Assisted Living: Indoor localization using LPWAN, detection of collapse and monitoring of vital data

Pet Tracking: Tracking of pets (e.g. cats) within the local neighborhood, use of GPS for outdoor and LPWAN for indoor localization

### Industrial

The domain industrial is characterized by multiple base-stations that are equipped inside buildings or that cover large industrial plants.

Asset Tracking: Location and anti-theft of assets using LPWAN

Industrial Plant Condition Monitoring: Monitoring of large industrial plants (e.g. oil refinery) using distributed sensor-nodes, use of licensed frequency bands

Industrial Production Monitoring: Monitoring of machine parameters (e.g. bearing temperature, oil levels, etc.) inside factory buildings

### Infrastructure

This domain is characterized by LPWAN base-stations with highly exposed antennas mounted on high towers that should cover large areas. These networks can be operated in rural or urban environments.

Pipeline Monitoring: Monitoring of pipelines in rural areas over very long distances using licensed frequency bands

Smart Grid – Fault Monitoring: Detection and indication of faults in the medium voltage distribution network

Smart Grid – Load Control: Control of load distribution in the smart grid, use of licensed frequency band

Smart Metering: Automatic readout of gas and water meters, the periodical update of the public encryption key requires is achieved using broadcast transmission

Structural Health Monitoring: Monitoring of the structural health of bridges and other infrastructure using distributed autonomous sensor nodes

### Logistics

This domain is characterized by systems that aim at the tracking and localization of assets. Multiple base-stations are required for localization and the systems have to operate on a world-wide basis. The LPWAN base-stations may be part of a public infrastructure.

Global Tracking: World-wide tracking of goods, e.g. containers

Fast Asset Tracking: Localization of equipment outside of buildings with high update frequency, e.g. tracking of carts on airports

### Smart Building

This domain covers applications that are used inside buildings. For this purpose, one or multiple LPWAN base-stations are located inside the buildings.

Access Control: Access control using keyless systems

Alarms and Security: Monitoring of doors, windows, etc.

Light Switch: Connectivity for battery-operated light-switches

Smoke Detectors: Real time alerts, status of battery, broadcasting of alarms to other smoke detectors

Water Pipe Leakage Monitoring: Monitoring of water pipe leaks inside buildings

### Smart City

This domain covers applications related to the smart city. The LPWAN base-stations are typically mounted highly exposed to cover larger areas.

Public Lighting: Street lights can be monitored and switched on and off on demand

Smart Parking: Available parking space indication in real-time

Vending Machines – General: Monitoring of vending machines, e.g. monitoring of low stock or maintenance

Vending Machines – Privacy: Data verification, e.g. for cashless payment

Waste Management: Garbage bins fill level monitoring

# 4 Frequency Regulation and Channel Models

For the evaluation process of different candidate technologies and existing IEEE standards it is essential to understand the achievable performance of these systems. In chapter 2 the theoretical bounds concerning the maximum payload bit-rate have already been discussed. This chapter adds additional aspects. First, this covers the frequency regulation that may vary significantly between different countries. Next, the channel models are defined that allow the prediction of the achievable transmit range in different scenarios. Finally, models for the interference and the channel use are presented.

## 4.1 Frequency Regulation

LPWAN systems may be either used in licensed or unlicensed frequency spectrum. Generally, licensed spectrum offers significant performance benefits, as it is typically under full control of the license owner. However, it is expected that most LPWAN systems will be operated in the license exempt frequency bands below 1 GHz. In order to guarantee a certain level of interoperability between users in these frequency bands, the responsible frequency regulation authorities have defined limitations to devices that are using these bands. These limitations refer to parameters such as the maximum transmit power, the used signal bandwidth or the duty cycle. The following sub-sections give a brief overview over the regulation in the United States and Europe.

### FCC (United States)

In the United States, the Federal Communications Commission (FCC) is the body responsible for implementing the frequency regulation rules. These rules are documented in Part 15 of Title 47 of the Code of Federal regulations [5]. Relevant for the license exempt sub-GHz bands 902 to 928 MHz are Part 15.247 (Frequency Hopping and Digitally Modulated Intentional Radiators) and Part 15.249 (General Non-Licensed Intentional Regulators).

Part 15.249 does not enforce any restrictions on devices operated in the 902 to 928 MHz band, e.g. on the bandwidth or the maximum transmit duration. However, Part 15.249 limits the maximum field strength to 50 mV/m in a distance of 3 m. This approximately results in an effective transmit power of -1 dBm [1], which may be too low for many applications.

In contrast, Part 15.247 allows for significantly higher transmit powers. However, it enforces additional rules that are mainly related to the mandatory use of frequency hopping.

Frequency Hopping: According to Part 15.247, systems with a 6 dB bandwidth of less than 500 kHz have to be treated as frequency hopping systems. These systems shall use at least 50 hopping channels, and the average time of occupancy per channel shall not be greater than 0.4 s within a 20 s period. If the 20 dB bandwidth of the hopping channels is 250 kHz or greater, the system shall use at least 25 hopping channels, and the average time of occupancy per channel shall not be greater than 0.4 s within a 10 s period.

Transmit Power: The electrical transmit power is limited to 1 W, and the ERP (Effective Radiated Power) is limited to 4 W. For frequency hopping systems with less than 50 channels these values have to be reduced to 0.25 W electrical power and 1 W ERP. Furthermore, non-frequency hopping signals shall not exceed a power spectral density of 8 dBm in any 3 kHz band.

### ETSI (Europe)

The European norm ETSI EN 300 220-2 [6, p. 20] lists the EU wide harmonized national radio frequency bands from 25 MHz to 1,000 MHz. The norm refers to these devices as “Short Range Devices” (SRD). Available frequencies for SRD are close to 27 MHz, 40 MHz, 169 MHz, 433 MHz, and between 863 and 870 MHz. Furthermore, local authorizes may allow additional frequencies, which a not EU wide harmonized. The main restrictions are related to the transmit power, the duty cycle, and the signal bandwidth.

Transmit power: The transmit power is limited to 10 mW or 25 mW ERP (Effective Radiated Power) in most SRD bands. The term ERP indicates that these values already include the antenna gain, which means that the maximum electrical power may be significantly lower. Only few frequency bands allow for a higher transmit power of 500 mW ERP, i.e. 169.400-169.475 MHz, and 869.400-869.650 MHz.

Duty cycle: The ETSI norm limits the channel occupancy of devices in most frequency bands. The duty cycle, i.e. the ratio expressed as a percentage of the cumulative duration of transmission within an observation interval in a given operational frequency band [7, p. 28]. The observation interval is normally defined as 1 h and typical duty cycles are between 0.1% and 10%. Thus, the cumulative duration is typically limited between 3.6 s and 360 s per hour, which may lead to restrictions in some LPWAN applications, especially for the base-stations. Thus, the transmission of a single packet low bit-rate LPWAN packet may be longer than the allowed duty cycle. Furthermore, the use of techniques such as frequency hopping does not increase the allowed transmission time if the hopping takes place in the same operational band, which will be the case for most applications. Only few operational bands do not have any duty cycle limitation, but they have additional restrictions with respect to the transmit power.

The operational bands between 863 and 870 MHz allows for so-called “polite spectrum access” as an alternative mode to the duty cycle operation. The exact definition of this mode is given in [7, p. 55]. This mode mainly adds a clear channel assessment with well-defined timing parameters before each transmission. However, also this mode defines a maximum cumulative on-time which is 100 s / 1 h within a h h200 kHz portion of the spectrum. This corresponds to a maximum duty cycle of 2.7% if only this portion of the spectrum is used. If also other portions of the spectrum are used, e.g. by means of frequency hopping, a duty cycle of more than 50% is feasible (e.g. hopping between 863 and 868 MHz).

Bandwidth: The maximum bandwidth depends on the used operational band. Most bands offer only a maximum signal bandwidth of few kHz. A higher signal bandwidth of more than 1 MHz is only available in the 434 MHz band. Furthermore, the band ranging from 863 to 870 MHz allows for a maximum signal bandwidth of 3 MHz. M

Frequency Regulation Summary

The frequency regulation authorities enforce different frequency regulatory aspects. A system that may be used on a world-wise basis has to take the following aspects into account:

- Maximum transmit duration of 0.4 s within a 20 s period (10 s period for  some configurations), mandatory use of frequency hopping

- Limitation of duty cycle, which is especially critical for the LPWAN base-stations

## 4.2 Propagation Models for LPWAN

The evaluation of different candidate technologies requires the definition of suitable propagation models for the defined use-cases. These channel models have to cover outdoor propagation in urban and rural areas, as well as indoor propagation. The agreed channel models base on a subset of the channel models that have been used during the development of IEEE 802.11ah, as both systems utilize the 900 MHz band. However, slight adaptions were required to include also the use-cases with transmit ranges of several km. The original discussions can be found in [8].

The basic structure of the channel model is defined as:

yt=xt⋅PLd*ht+nt+it,

where x(t) is the transmit signal, and y(t) is the received signal. Furthermore, PL(d) is the path loss as function of the distance d, h(t) models the multi-path effects that may be time-variant, n(t) is the thermal noise, and i(t) is the interference from other frequency users. In addition, * is the convolution product.

Generally, all terms in the equation are statistical values. Therefore, a new channel model realization has to be calculated for each simulation run, and the system performance is then evaluated using thousands of independent simulation runs. The corresponding values will be defined for the different propagation models in the following subsections. Finally, the interference term i(t) will be defined in section 4.4.

Multi-antenna systems are currently not modeled. However, suitable models may be added if required.

### Indoor Model

The indoor model is identical to IEEE 802.11ah channel model A [9, p. 7]. The path loss is given by:

PLd=X+20log104πdfcc0 for d≤dBPPLdBP+35log10ddBPfor d>dBP

where dBP=5m is the so-called break-point distance. Furthermore, d is the distance in m, fc=900MHz is the carrier frequency, and c0=300,000kms is the speed of light. The term X models the log-normal fading, i.e. the standard deviation, by adding a random variable. This term tries to consider effects such as shadowing that varies for different positions. It is derived from a random Gaussian variable with zero mean and standard deviation in dB and has to be generated for each simulation run. The standard deviation is 2dB for d≤dBP and 3dB else. Additional parameters defined in the [9, p. 4] such as the floor index should not be used.

The multi-path propagation to identical to IEEE 802.11ah channel model A [9, p. 7]. The model shall use a single tap only, as the short expected delay spread in indoor channels will not lead to relevant frequency selectivity within a typical LPWAN bandwidth. Again a carrier frequency of 900MHz is assumed. In addition, the channel should model a Doppler component defined by the velocity v. The corresponding velocity is defined for the specific use-cases [3]. A velocity should be modeled even in case of static configurations, as it considers the effects due to movements in the surroundings, e.g. walking persons.

### Outdoor Urban Model / Outdoor Rural Model

Similar to the outdoor model defined by IEEE 802.11ah, the outdoor urban and the outdoor rural model base on the 3GPP Spatial Channel model [10]. The outdoor urban model should use the parameters defined for the 3GPP scenario “Urban Macro”, and the outdoor rural model should use the parameters defined for the 3GPP scenario “Suburban Macro”. Generally, these models define the path loss PL(d) and the multi-path component h(t) for base-stations with exposed antennas and a typical distance of 3km. The channel models are not suitable for short distances. Thus, the minimum distance between transmitter and receiver shall be 500m.

Furthermore, the center frequency should be set to fc=900MHz, the height of the sensor node to hMS=1.5m, and the base-station antenna height to hBS=32m, or  hBS=140m for highly exposed sites. Additionally, only a single polarization and a single antenna on the transmitter and receiver side with a gain of 0dBi shall be used. The velocity to model the Doppler component is again defined for the specific use-cases [3].

### Outdoor Device-to-Device

The outdoor device-to-device model shall be used to model the communication between devices without exposed antennas, e.g. for the comparison with multi-hop structures. The model bases on the 802.11ah device-to-device model [9, p. 3]. Effects due to multi-path should not be considered. The path loss is given by:

PLd=X-6.17+58.6⋅log10d

where d is the distance in m. Furthermore, a carrier frequency of 900MHz is assumed, and X models a log-normal fading component with a standard deviation of 7.5dB.

### Thermal Noise

The thermal noise component n(t) shall be modeled as white Gaussian noise with a noise power spectral density of -174dBm/Hz. Furthermore, a noise figure of F=3dB for base-stations and F=6dB for sensor-nodes shall be added. This results in a noise power spectral density of  -171dBm/Hz for base-stations and  -168dBm/Hz for sensor-nodes  [1].

In order to obtain the final noise power the spectral density of the noise has to be multiplied with the simulation bandwidth. This approach is more flexible than defining a SNR (Signal to Noise Ratio), as the noise term is always a function of the simulation bandwidth that may vary significantly for different systems.

## 4.3 Interference Channel Model

LPWAN focus on long range transmission. The low payload bit-rate that is required to compensate the path loss leads to long packet durations. Caused by this large spectral foot-print, the possibility to be affected by interferes is significantly increased. In addition, the antennas of LPWAN base-stations may be highly exposed. Hence, these stations receive the signals of many different systems in the range of multiple km. In order to model the effects of interferers the IG LPWA defined an interference model. Details on this model are presented in [11]. This document will give a brief introduction.

Figure 3: Measured interference in Erlangen, Germany, in the license exempt band ranging from 868 MHz to 870 MHz, the antenna was located in a height of 140m above ground

Figure 3 shows the measured interference in the frequency range from 868 to 870 MHz, which is a part of the license exempt SRD band (cf. section 4.1). The antenna was mounted highly exposed in a height of 140m. The measurement shows that significant parts of the spectrum are occupied. A zoomed version of this figure is available in [11]. It shows that especially some frequency bands are occupied by potentially hundreds of signals. In addition, it can be expected that the interference will further grow in the future, as more and more systems are using wireless transmission.

For modelling the traffic the model of Haenggi and Ganti has been used [12]. It models from various radio nodes with distinct Poisson-arrival rates, which is a good approximation in case of many independent interferers. This model has been extended by a path-loss model, which is required to model the effects on different antenna configurations and heights. Furthermore, four different interferer types have been defined to model the effects of different interfering systems.

Table 1: Layer defined for the interference model

| Layer | Mean Arrival Rate λ' [1/s/km²/MHz] | Power [dBm] | Bandwidth [kHz] | Length [ms] |
| --- | --- | --- | --- | --- |
| 1 | 0.8 | 10 | 200 | 5 |
| 2 | 0.15 | 10 | 20 | 5 |
| 3 | 0.04 | 10 | 10 | 100 |
| 4 | 0.01 | 10 | 1,000 | 30 |

Table 1 lists the defined layers. They define interferes of different bandwidth and different length. The arrival rate between these interferers differs. However, the sum arrival rate is one. This means, that we can expect one interferer per second per km² per MHz. This means, if we have a cell size of 1km² and a system bandwidth of 1 MHz, we can expect a mean value of one interfering signal per second. The signal level of this interferer than depends on the distance of the interferer to the receiver as well as the used propagation model (cf. section 4.2), while the position of the interferer within the cell is randomly distributed. Details are given in [11].

In order to model different interferer densities, additional multiplication factors have been defined. Table 2 shows the defined classes. The classes scale the mean arrival rate to model different scenarios. Figure 4 shows example realizations for the classes “Low” and “Dense” assuming the outdoor urban propagation model in addition to an antenna height of 140m. The resulting interference for the class “Dense” in addition to the use propagation model can be considered as worst case assumption. The playground of 2 MHz and 2s contains approx. 50,000 interferer signals, which is mainly caused by the highly exposed antennas that were assume within the simulation.

Table 2: Multiplication factors for different interference classes

| Class | Mean Arrival Rate over all layers λ' [1/s/km²/MHz] |
| --- | --- |
| None | 0 |
| Low | 1 |
| Medium | 10 |
| Dense | 50 |

Figure 4: Example realizations for the interference class Low (top) and Dense (bottom). Both realizations assume an antenna height of 140m in addition to the propagation model urban outdoor.

## 4.4 Number of Active Users

The available frequency spectrum is highly limited. Therefore, a significant performance criterion is the number of users a system is able to support in a given configuration. This value is reflected by the number of active users. The original discussions can be found in [13].

In order to derive suitable numbers for the usage in different areas, the population densities in different areas of the world were considered. Table 3 shows the population densities for selected area of the world.

Table 3: Population density in different areas of the world (Source: Wikipedia)

| Region | Population Density (1/km²) |
| --- | --- |
| Paris | 21,000 |
| Paris (metropolitan area) | 722 |
| London | 5,518 |
| London (metropolitan area) | 1,655 |
| Berlin | 4,000 |
| Atlanta (metropolitan area) | 255 |
| Germany | 227 |
| France | 116 |
| USA | 35 |
| Kansas | 13.5 |

For obtaining an indication of the usage, we will assume that on average each person is equipped with 10 LPWAN devices. Furthermore, each of these devices shows an average activity of one transmission per hour. This results in 2.7⋅10-3 transmissions per second. If we combine the population density of the selected areas with the assumed transmissions per second per user we obtain the mean arrival rate λ', i.e. the average number of packets that are transmitted per second per km². Table 4 shows the resulting arrival rates for selected areas of the world. In addition, it shows the mean arrival rate if a base-station covers an area with a radius of 1 and 10km, respectively. It becomes obvious that the mean rate may reach values of 60 packets per second per km² in selected areas of the world.

Table 4: Resulting arrival rates in selected areas of the world

| Area | Mean Arrival Rate λ' [1/s/km²] | 1km-Raduis Mean Arrival Rate [1/s] | 10km-Radius Mean Arrival Rate λ[1/s] |
| --- | --- | --- | --- |
| Paris | 58.3 | 183 | 18,325 |
| Paris (m.a.) | 2.0 | 6.3 | 630 |
| London | 15.3 | 48 | 4,815 |
| London (m.a.) | 4.6 | 14.4 | 1,444 |
| Berlin | 11.1 | 34.9 | 3,490 |
| Atlanta (m.a.) | 0.71 | 2.2 | 223 |
| Germany | 0.63 | 2.0 | 198 |
| France | 0.32 | 1.0 | 101 |
| USA | 0.097 | 0.31 | 31 |
| Kansas | 0.038 | 0.12 | 12 |

The group agreed to compress this list into a set of four different classes that defines the arrival rates. Table 5 shows the classes and the mean arrival rate per second per km². The classes low and medium model rural areas, the class high models urban areas, and the class very high models densely populated urban areas.

Table 5: Classes defining the number of active interfering users

| Class | Mean Arrival Rate λ‘ [1/s/km²] |
| --- | --- |
| Low | 0.1 |
| Medium | 1 |
| High | 10 |
| Very high | 50 |

For simplifying the modelling, we assume that the activity of the nodes is mutually independent.

This again allows the modelling using a Poisson arrival process [12]. Such an assumption will be valid for ALOHA systems. However, this will not hold in fully coordinated networks, for which more sophisticated models have to be defined.

For MAC simulations that are only focusing on the arrival rate λ, the value is obtained by multiplying the cell size area with the rate λ'.

Sophisticated PHY simulations have to follow a different approach, as e.g. the received signal field strength of the other interfering users is an important parameter. The exact procedure is highly similar to the interference model described in section 4.3. Details are given in [13].

# 5 Use-Case Evaluation Process

The main task of the IG LPWA is the identification of use-cases for LPWAN that are not fully covered by existing IEEE standards. The IG LPWA agreed on a three step evaluation as shown in Figure 5.

The first step is the suitability analysis. This step analysis whether existing IEEE standards are able to cover specific use-cases completely, and a new LPWAN standard would not provide any benefit. These use-cases can then be excluded for the further evaluation process. Furthermore, this step also analysis whether technology options (e.g. modulation schemes) exist that are able to cover the use-cases. If use-cases cannot be covered, they are excluded as well.

Use-cases that may offer potential for improvement using a new LPWAN standard are analyzed in the qualitative evaluation. This especially considers the dependency between the different candidate technologies as well as deployment scenarios. As an example, the network type (e.g. mesh or start) highly impacts the height of the base-station antennas, which again has impact on the expected interference levels, and thus on the suitable modulation schemes.

In the last step, the potential of a new LPWAN standard are analyzed quantitatively, which is only possible and useful for selected technologies.

Figure 5: Analysis of different candidate technologies and IEEE standards

For the evaluation process the IG LPWA identified different parameters that characterize the different use-cases. This parameter list is given in Table 6. These parameters cover the channel model, the interference type, but also the battery type to give indication for its use. The use-cases with the parameters agreed in the IG LPWA are listed in [3].

Table 6: Parameters used for the suitability evaluation

| Parameter Name | Description | Parameter Set |
| --- | --- | --- |
| Channel Model | This parameter defines the assumed channel model for the specific use-case (see section 4.2) | Indoor, Outdoor Rural, Outdoor Urban |
| Interference Model | This parameter considers the interference from other systems operated in license exempt frequency bands (see section 4.3) | Dense, Medium, Low, None |
| Active Interfering Users | This parameter considers the intra-system interference, i.e. the interference from other users using the same system (see section 4.4) | Very high, High, Medium, Low |
| Communication Mode | This parameter defines whether the payload communication is uni- or bi-directional | Uplink, Uplink/Downlink, Uplink/Broadcast Downlink |
| Data Period | Type and frequency of the payload data transmission | Occasionally, less than 1/day; Occasionally 1/day; Occasionally 1/hour; Occasionally, more than 1/hour; Periodically 1/day; Periodically 1/hour; Periodically, more than 1/hour |
| Data Length | Number of bytes that form one transmission | <=16bytes; <=64bytes; <=256bytes; >256bytes |
| Availability | The minimum required probability of a successful transmission | Best effort (> 90%), Medium (>99%), High (>99.9%) |
| Latency | Maximum acceptable latency for a given use-case | <0.25s; <1s; <10s; <1min; <10min; <60min; <1day |
| LPWAN Localization Precision | Required localization using only the LPWAN signal (no GPS), e.g. for indoor localization | <10m; <100m; Not required |
| Typical Power Supply | Typical power supply of the node to estimate overall lifetime and max. transmit power | CR2025; 2xAA; Energy Harvesting; External |
| Frequency Regulation | Considers potential limitations due to frequency regulation, e.g. duty cycle | NA; ETSI; FCC; ETSI/FCC |
| Cell Radius | Required size of the network cell | <1km; <5km; <10km; <50km; >50km |
| Data Security | Required type of data security, encryption is required in all cases | Layer-2; Layer-3; End-to-end; Secure Authentication |
| Node Velocity | Max. speed of the sensor-nodes (static configuration use 3 km/h to model movements in the environment) | 3 km/h; 30 km/h; 120 km/h |

# 6 Analysis of Existing IEEE Standards / Candidate Technologies

The creating of a TG to create a new standard or amendment is only useful if the intended use-cases are not sufficiently covered by existing IEEE standards. Furthermore, there have to be significant prospects that a significant gain with respect to the existing standards can be achieved.

## 6.1 Suitability of Candidate Technologies

The purpose of this section is to show that there is at least one technology option that is able to support the specific use-case. This gives an indication whether it is possible to support the different use-cases by defining a new LPWAN standard.

##### Modulation

In [14] different modulation schemes have been evaluated with respect to their suitability for LPWAN. The list of evaluation schemes includes OFDM (Orthogonal Frequency Division Multiplexing), CDMA (Code Division Multiple Access), DSSS (Direct Sequence Spread Spectrum), FCSS (Frequency Chirp Spread Spectrum), FHSS (Frequency Hopping Spread Spectrum), non-coherent and coherent narrow-band modulation.

According to the discussions within the IG LPWA, the following suitability evaluation parameters defined in Table 6 impact the modulation: Channel Model, Interference Model, Active Interfering Users, Frequency Regulation, LPWAN Localization Precession, Power Supply, Cell Radius, and Node Velocity.

The evaluation against the different use-cases [15] shows that all use-cases defined in chapter 3 are supported by at least one modulation scheme. The most suitable schemes are OFDM and FHSS. The schemes CDMA, DSSS, FCSS, and pure narrow-band modulation are only suitable for a limited number of use-cases. The reason is mainly the limited robustness with respect to strong interference and many active users, which especially occur in large network cells employing license exempt frequency bands. This gets even worse due to the very low bit-rates in addition to spreading, which leads to a very large spectral footprint of several seconds and a bandwidth of several kHz. Hence, the probability that the signal is hit by a strong interferer is very high. This is somehow surprising as most proprietary LPWAN systems exactly base on these modulation schemes. Therefore, it can be expected that the performance of such systems will significantly degrade over time when the number of users in these bands will further increase.

##### Forward Error Correction

Document [16] shows the LPWAN suitability evaluation of potential forward error correction (FEC) schemes. The list of evaluated schemes includes the use of uncoded transmission, Reed Solomon or BCH-Codes, Convolutional Codes, Turbo Codes, LDPC Codes, and Polar Codes.

According to the discussions within the IG LPWA, the following suitability evaluation parameters defined in Table 6 impact the FEC: Channel Model, Interference Model, Active Interfering Users, Frequency Regulation, Availability, Data Length, Power Supply, Cell Radius, and Node Velocity.

The evaluation against the use-cases [17] shows that all use-cases defined in chapter 3 are supported by at least one scheme. Generally, un-coded transmission does not support any use-case. This is mainly caused by the high signal-to-noise ratio required for error free decoding and the high sensitivity with respect to any kind of interference that especially occurs in large network cells. State-of-the-art iterative codes such as Turbo Codes offer a high performance. These codes can be encoded with relatively low complexity. However, the iterative decoding leads to significant complexity, and hence, they are not suitable for applications with strict power limitations. Additionally, due to the iterative processing, these codes are not suited for short block lengths of few bytes. For short block length classical convolutional codes are an interesting alternative. They also work closely to the theoretical bounds for short block length and have an acceptable decoding complexity.

##### Medium Access Control (MAC)

The evaluation of potential Medium Access Control (MAC) schemes is presented in document [18]. The list of evaluated MAC schemes includes ALOHA, Slotted ALOHA, Carrier Sense Multiple Access (CSMA), and a fully coordinated network, e.g. by means of beacons.

The evaluation against the use-cases shows that some use-cases are not covered by any of the evaluated schemes [19]. Practically all schemes suffer from strong interference and a high number of active users. Furthermore, also mechanisms as CSMA will not sufficiently work in large network cells due to the hidden node problem. In addition, also coordinated networks, e.g. by means of beacons, may suffer from strong interference of other systems operated in license exempt frequency bands.

In order to further evaluate this problem, detailed simulations were performed that are presented in section 6.3.

##### Connectivity

Document [20] shows the connectivity evaluation and document [21] the evaluation against the use-cases. Analyzed schemes were transparent transmission, un-compressed IPv6, IPv6 with SCHC (Static Context Header Compression), and IPv6 with RFC 6282 header compression.

Again all use-cases are covered by at least one candidate scheme. However, the use of un-compressed IPv6 transmission practically does not work for transmission schemes that only offer few payload bytes per packet. In addition, the resulting overhead will lead to very high energy consumption. Using SCHC the IPv6 overhead can be highly reduced, which allows a limited IP connectivity even for tiny devices. RFC 6282 based header compression offers a higher degree of flexibility, but the resulting overhead is not supported by schemes that offer a payload length of 16 bytes or less.

##### Network Topologies

Document [22] shows the agreed evaluation of possible network topologies. Analyzed topologies were star, extended star, device to device, base-station assisted device to device, unsynchronized mesh, and synchronized mesh. Document [23] shows the suitability evaluation with respect to the use-cases. All use-cases defined in chapter 3 are covered by at least one network topology.

##### Privacy and Encryption

The IG LPWA intensively discussed the requirements of privacy and encryption for LPWAN. The group has consensus that encryption is required for all applications. The group also discussed input coming from the Privacy Recommendation EC Study Group [24]. Privacy may be very significant for LPWAN due to the long range transmission. Monitoring the data transmission of light switches equipped with LPWAN may indicate whether the residents are at home or not, even if the actual payload data is encrypted and thus cannot be decoded. Therefore, privacy by design should be considered seriously for a new LPWAN standard.

An additional issue of LPWAN is the key exchange. First, the transmission of long public keys may lead to a very significant overhead in case of many LPWAN applications. Second, such key exchange may not be possible in case of uni-directional LPWAN devices.

The IG LPWA did not list any candidate technologies to cover the aforementioned challenges as this was beyond the scope of the group that was mainly focusing on the PHY and MAC layer.

In summary, all defined use-cases may be covered by at least one technology options. Therefore, no use-case has to be excluded from the list. However, especially the MAC requires further analysis that will be presented in section 6.4.

## 6.2 Suitability Analysis of Existing IEEE Standards

This section presents the suitability analysis of existing IEEE standards. The aim is the identification of use-cases that are perfectly covered by these systems. Thus, a new LPWAN standard would not be able cover these use-cases more efficiently, and these use-cases do not have to be further considered for the evaluation process.

Two IEEE standards were identified that may are able to cover the identified use-cases. These are IEEE 802.11ah, and IEEE 802.15.4.

##### IEEE 802.11ah

The standards defined by IEEE 802.11 are mainly focusing on higher payload bit-rates in the license exempt frequency bands above 1 GHz. Due to the high payload bit-rate these systems are only intended for local coverage of typically significantly less than 1km. Therefore, they miss the intended LPWAN sensitivity of -140dBm by decades.

Recently the development of IEEE 802.11ah has been completed. This amendment is focusing on battery driven IoT applications and has been optimized for very low energy consumption. Additionally, it operates in the sub-GHz frequency bands and is thus able to exploit the good propagation conditions of sub-GHz bands.

The detailed suitability analysis and the use-case evaluation of IEEE 802.11ah are shown in documents [25], [26]. The suitability evaluation using the parameters defined in Table 6 shows that IEEE 802.11ah fulfills almost all parameters completely. However, there are two main exceptions, i.e. the cell size and the frequency regulation.

Due to the relatively high payload bit-rate, the sensitivity requirement for the most robust mode is -98dBm. The current frequency regulation in Europe limits the maximum transmit power to 25mW ERP. Hence, IEEE 802.11ah is only able to support the cell size parameter set <1km. The use of mesh may be possible, but would require a very high amount of devices to cover large cells. Furthermore, European regulation limits the duty cycle to 0.1% and 1%, or approx. 2.7% if polite spectrum access is used (cf. section 4.1). However, as the amount of data for these use-cases is limited, IEEE 802.11ah would be able to support these use-cases from the duty cycle perspective. As IEEE 802.11ah is not intended for licensed spectrum, use-cases assuming licensed spectrum are not supported.

In summary, the suitability evaluation indicates that IEEE 802.11ah is suited for the defined LPWAN use-cases focusing on indoor applications. However, it is not suited for the defined outdoor use-cases, as IEEE 802.11ah has not been designed for big cell sizes. Table 7 shows the suitability of IEEE 802.11ah for the different use-cases.

Table 7: Suitability evaluation for IEEE 802.11ah, completely covered used-cases cannot be  covered more efficiently by a new LPWAN standard

| Domain | Completely Covered Use-Cases |
| --- | --- |
| Agriculture and Environmental |  |
| Consumer/Medical | Assisted Living |
| Industrial | Asset Tracking, Industrial Production Monitoring |
| Infrastructure |  |
| Logistics |  |
| Smart Building | Access Control, Alarms and Security, Light Switch, Smoke Detectors, Water Pipe Leakage Monitoring |
| Smart City |  |

##### IEEE 802.15.4

Document [27] gives a comprehensive overview of the capabilities of IEEE 802.15.4. Unlike IEEE 802.11, it also offers also lower bit-rates. Therefore, IEEE 802.15.4 is also able to support data transmission over significantly higher distances than IEEE 802.11. Only very low bit-rates are limited due to FCC regulation [28]. In addition, mesh topologies can be used for further increased network cell sizes. Furthermore, 802.15.4 offers low complexity and is therefore perfectly suited for battery driven applications. The available modulation schemes also cover spreading technologies, making it ideal for precise localization.

As a result, IEEE 802.15.4 is able to perfectly cover the same use-cases as IEEE 802.11. Furthermore, it is able to perfectly cover some of the long-range use-cases with long range. Room for further improvement is only left in case of long-range transmission with significant interference levels. Therefore, use-cases with these characteristics will be further analyzed in the following sections.

Table 8: Suitability evaluation of IEEE 802.15.4, completely covered used-cases cannot be covered more efficiently by a new LPWAN standard

| Domain | Completely Covered Use-Cases |
| --- | --- |
| Agriculture and Environmental | Cattle Monitoring, Field Monitoring |
| Consumer/Medical | Assisted Living |
| Industrial | Asset Tracking, Industrial Production Monitoring |
| Infrastructure |  |
| Logistics |  |
| Smart Building | Access Control, Alarms and Security, Light Switch, Smoke Detectors, Water Pipe Leakage Monitoring |
| Smart City |  |

##### Remaining Use-Cases

IEEE 802.11ah and 802.15.4 are already able to perfectly fulfill many of the LPWAN use-cases listed in Table 6. Therefore, all use-cases where no improvement from a new LPWAN standard can be expected are removed from the following considerations. The remaining use-cases that may offer optimization potential by means of a new LPWAN standard are listed in Table 9. These use-cases all require large cell sizes and suffer from strong interference. The next section will qualitatively analyze the potential improvement using a new LPWAN standard.

Table 9: Remaining use-cases that may offer potential for a new LPWAN standard

| Domain | Remaining Use-Cases |
| --- | --- |
| Agriculture and Environmental |  |
| Consumer/Medical | Pet Tracking |
| Industrial | Industrial Plant Condition Monitoring |
| Infrastructure | Pipeline Monitoring – Terrestrial, Smart Grid - Fault Monitoring, Smart Grid - Load Control, Smart Metering, Structural Health Monitoring |
| Logistics | Global Tracking, Fast Asset Tracking |
| Smart Building |  |
| Smart City | Public Lighting, Smart Parking, Vending Machines – general, Vending Machines - Point of Sale, Waste Management |

## 6.3 Qualitative Evaluation of Candidate Technologies

Tbd.

## 6.4 Quantitative Evaluation of Candidate Technologies

Tbd.

## 6.5 Evaluation Summary

Tbd.

# 7 Summary and Recommendation for Future WG Activities

Tbd.

# Annex

# Literature

| [1] | J. Proakis und M. Salehi, Digital Communications, 5th edition, Irwin Electronics & Computer Enginering, 2007. |
| --- | --- |
| [2] | J. Robert und A. Heuberger, „LPWAN Downlink Using Broadcast Transmitters,“ in Broadband Multimedia Systems and Broadcasting (BMSB), 2017 IEEE International Symposium on, Cagliari, Italy, 2017. |
| [3] | J. Robert, „LPWA Use-Cases,“ 15-16/770r5, 2017. |
| [4] | ETSI, „ETSI TR 103 435 V1.1.1,System Reference document (SRdoc); Short Range Devices (SRD); Technical charateristics for Ultra Narrow Band (UNB) SRDs operating in the UHF spectrum below 1 GHz,“ February 2017. [Online]. Available: http://www.etsi.org/deliver/etsi_tr/103400_103499/103435/01.01.01_60/tr_103435v010101p.pdf. |
| [5] | Federal Communications Commission, „Rules & Regulations for Title 47,“ [Online]. Available: https://www.fcc.gov/general/rules-regulations-title-47. |
| [6] | ETSI, „ETSI EN 300 220-2 V3.1.1, Short Range Devices (SRD) operating in the frequency range 25 MHz to 1 000 MHz; Part 2: Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU for non specific radio equipment,“ February 2017. [Online]. Available: http://www.etsi.org/deliver/etsi_en/300200_300299/30022002/03.01.01_60/en_30022002v030101p.pdf. |
| [7] | ETSI, „ETSI EN 300-220-1 V3.1.1, Short Range Devices (SRD) operating in the frequency range 25MHz to 1 000 MHz; Part 1: Technical characteristics and methods of measurement,“ February 2017. [Online]. Available: http://www.etsi.org/deliver/etsi_en/300200_300299/30022001/03.01.01_60/en_30022001v030101p.pdf. |
| [8] | J. Robert, „Proposal for LPWAN Channel Models,“ 15-17/36r1, 2017. |
| [9] | R. Porat, S. Yong und K. Doppler, „TGah Channel Model,“ 11-11/968r4, 2011. |
| [10] | 3GPP, „TR 25.996, V13.0.0, Spatial channel model for Multiple Input Multiple Output (MIMO) simulations (Release 13),“ 2015. |
| [11] | J. Robert, H. Lieske und S. Rauh, „Proposal for sub-GHz Interference Model,“ 15-17/37r1, 2017. |
| [12] | . M. Haenggi und R. K. Ganti, „Interference in Large Wireless Networks,“ Foundations and Trends in Networking, pp. 127-248, 2008. |
| [13] | J. Robert, „Number of Active Interfering Users,“ 15-17/35r0, 2017. |
| [14] | J. Robert, „Suitability Evaluation of Modulation Schemes,“ 15-17/374r1, 2017. |
| [15] | J. Robert, „Use Case Evaluation of Modulation Schemes,“ 15-17/495r0, 2017. |
| [16] | J. Robert, „Suitability Evaluation of FEC Schemes,“ 15-17/375r1, 2017. |
| [17] | J. Robert, „Use Case Evaluation of FEC Schemes,“ 15-17/497r0, 2017. |
| [18] | J. Robert, „Suitability Evaluation of MAC Schemes,“ 15-17/378r1, 2017. |
| [19] | J. Robert, „Use Case Evaluation of MAC Schemes,“ 15-17/496r0, 2017. |
| [20] | J. Robert, „Suitability Evaluation of Connectivity,“ 15-17/376r3, 2017. |
| [21] | J. Robert, „Use Case Evaluation of Connectivitiy,“ 15-17/498r0, 2017. |
| [22] | J. Haapola, J. Robert und P. Thubert, „Suitability Evaluation of Network Topologies,“ 15-17/379r3, 2017. |
| [23] | J. Robert, „Use Case Evaluation of Network Toplogies,“ 15-17/494r0, 2017. |
| [24] | J. C. Zuninga, „802E Privacy Mitigations,“ privecsg-16/2r0, 2016. |
| [25] | J. Robert, „Suitability of IEEE 802.11ah for LPWAN Applications,“ 15-17/162r0, 2017. |
| [26] | J. Robert, „Use Case Evaluation of IEEE Standards,“ 15-17/499r0, 2017. |
| [27] | P. Kinney, „Summary of IEEE Std 802.15.4 LECIM,“ 15-17/248r0, 2017. |
| [28] | J. Robert, „Suitability of IEEE 802.15.4k,“ 15-17/346r1, 2017. |