IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | P802.15.10a proposal for RMA corrections to 802.15.10-2017 |  |
| DCN | 15-18-0436-01-010a- P802.15.10a-D02-2018 |  |
| Date Submitted | [September 10th, 2018] | Voice: [+1-408-330-4586] E-mail: [charliep@computer.org] |
| Source | [Charlie Perkins] [Futurewei] |  |
| Re: | [TG 802.15.10a (RMA) corrections to 802.15.10-2017] |  |
| Abstract | ‘P2P list’ is changed to be a ‘route table entry’ to enable correction |  |
| Purpose | [Economical method to satisfy 802.15.10a correction requirements] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release |  |  |

PvarDesignation™/DvarDraftNumber
DrafttxtTrialUsetxtGorRPorSTD for varTitlePAR
Amendment A

Sponsor

varCommittee Committee
of the
IEEE Computer Society

Approved <XX MONTH 20XX>

IEEE-SA Standards Board

Copyright © 2017 by The Institute of Electrical and Electronics Engineers, Inc.

Three Park Avenue

New York, New York 10016-5997, USA

All rights reserved.

This document is an unapproved draft of a proposed IEEE Standard. As such, this document is subject to change. USE AT YOUR OWN RISK! IEEE copyright statements SHALL NOT BE REMOVED from draft or approved IEEE standards, or modified in any way. Because this is an unapproved draft, this document must not be utilized for any conformance/compliance purposes. Permission is hereby granted for officers from each IEEE Standards Working Group or Committee to reproduce the draft document developed by that Working Group for purposes of international standardization consideration. IEEE Standards Department must be informed of the submission for consideration prior to any reproduction for international standardization consideration (stds.ipr@ieee.org). Prior to adoption of this document, in whole or in part, by another standards development organization, permission must first be obtained from the IEEE Standards Department (stds.ipr@ieee.org). When requesting permission, IEEE Standards Department will require a copy of the standard development organization's document highlighting the use of IEEE content. Other entities seeking permission to reproduce this document, in whole or in part, must also obtain permission from the IEEE Standards Department.

IEEE Standards Department

445 Hoes Lane

Piscataway, NJ 08854, USA

Abstract: This amendment fully defines how the addressing and route information (already defined in the standard) are to be used by the routing modes (also currently defined in the standard), including at least the following:

- E2E acknowledgement from mesh route in non-storing mode

- P2P routing using a combination of up/down routing in non-storing mode

- On-demand P2P routing for E2E acknowledgement in non-storing mode

- On-demand path storing when sending unicast in non-storing mode

Keywords: Discovery and addition of new nodes; Route establishment; Dynamic route reconfiguration; Re-establishment of broken/lost routes; Purging of inactive routes; Real time link status information exchange; Single hop appearance at the networking layer (not breaking standard Layer 3 mechanisms); Support for broadcast; Support for multicast; Frame forwarding

Important Notices and Disclaimers Concerning IEEE Standards Documents

IEEE documents are made available for use subject to important notices and legal disclaimers. These notices and disclaimers, or a reference to this page, appear in all standards and may be found under the heading “Important Notices and Disclaimers Concerning IEEE Standards Documents.” They can also be obtained on request from IEEE or viewed at http://standards.ieee.org/IPR/disclaimers.html.

Notice and Disclaimer of Liability Concerning the Use of IEEE Standards Documents

IEEE Standards documents (standards, recommended practices, and guides), both full-use and trial-use, are developed within IEEE Societies and the Standards Coordinating Committees of the IEEE Standards Association (“IEEE-SA”) Standards Board. IEEE (“the Institute”) develops its standards through a consensus development process, approved by the American National Standards Institute (“ANSI”), which brings together volunteers representing varied viewpoints and interests to achieve the final product. IEEE Standards are documents developed through scientific, academic, and industry-based technical working groups. Volunteers in IEEE working groups are not necessarily members of the Institute and participate without compensation from IEEE. While IEEE administers the process and establishes rules to promote fairness in the consensus development process, IEEE does not independently evaluate, test, or verify the accuracy of any of the information or the soundness of any judgments contained in its standards.

IEEE Standards do not guarantee or ensure safety, security, health, or environmental protection, or ensure against interference with or from other devices or networks. Implementers and users of IEEE Standards documents are responsible for determining and complying with all appropriate safety, security, environmental, health, and interference protection practices and all applicable laws and regulations.

IEEE does not warrant or represent the accuracy or content of the material contained in its standards, and expressly disclaims all warranties (express, implied and statutory) not included in this or any other document relating to the standard, including, but not limited to, the warranties of: merchantability; fitness for a particular purpose; non-infringement; and quality, accuracy, effectiveness, currency, or completeness of material. In addition, IEEE disclaims any and all conditions relating to: results; and workmanlike effort. IEEE standards documents are supplied “AS IS” and “WITH ALL FAULTS.”

Use of an IEEE standard is wholly voluntary. The existence of an IEEE standard does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to the scope of the IEEE standard. Furthermore, the viewpoint expressed at the time a standard is approved and issued is subject to change brought about through developments in the state of the art and comments received from users of the standard.

In publishing and making its standards available, IEEE is not suggesting or rendering professional or other services for, or on behalf of, any person or entity nor is IEEE undertaking to perform any duty owed by any other person or entity to another. Any person utilizing any IEEE Standards document, should rely upon his or her own independent judgment in the exercise of reasonable care in any given circumstances or, as appropriate, seek the advice of a competent professional in determining the appropriateness of a given IEEE standard.

IN NO EVENT SHALL IEEE BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO: PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE PUBLICATION, USE OF, OR RELIANCE UPON ANY STANDARD, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE AND REGARDLESS OF WHETHER SUCH DAMAGE WAS FORESEEABLE.

Translations

The IEEE consensus development process involves the review of documents in English only. In the event that an IEEE standard is translated, only the English version published by IEEE should be considered the approved IEEE standard.

Official statements

A statement, written or oral, that is not processed in accordance with the IEEE-SA Standards Board Operations Manual shall not be considered or inferred to be the official position of IEEE or any of its committees and shall not be considered to be, or be relied upon as, a formal position of IEEE. At lectures, symposia, seminars, or educational courses, an individual presenting information on IEEE standards shall make it clear that his or her views should be considered the personal views of that individual rather than the formal position of IEEE.

Comments on standards

Comments for revision of IEEE Standards documents are welcome from any interested party, regardless of membership affiliation with IEEE. However, IEEE does not provide consulting information or advice pertaining to IEEE Standards documents. Suggestions for changes in documents should be in the form of a proposed change of text, together with appropriate supporting comments. Since IEEE standards represent a consensus of concerned interests, it is important that any responses to comments and questions also receive the concurrence of a balance of interests. For this reason, IEEE and the members of its societies and Standards Coordinating Committees are not able to provide an instant response to comments or questions except in those cases where the matter has previously been addressed. For the same reason, IEEE does not respond to interpretation requests. Any person who would like to participate in revisions to an IEEE standard is welcome to join the relevant IEEE working group.

Comments on standards should be submitted to the following address:

Secretary, IEEE-SA Standards Board

445 Hoes Lane

Piscataway, NJ 08854 USA

Laws and regulations

Users of IEEE Standards documents should consult all applicable laws and regulations. Compliance with the provisions of any IEEE Standards document does not imply compliance to any applicable regulatory requirements. Implementers of the standard are responsible for observing or referring to the applicable regulatory requirements. IEEE does not, by the publication of its standards, intend to urge action that is not in compliance with applicable laws, and these documents may not be construed as doing so.

Copyrights

IEEE draft and approved standards are copyrighted by IEEE under U.S. and international copyright laws. They are made available by IEEE and are adopted for a wide variety of both public and private uses. These include both use, by reference, in laws and regulations, and use in private self-regulation, standardization, and the promotion of engineering practices and methods. By making these documents available for use and adoption by public authorities and private users, IEEE does not waive any rights in copyright to the documents.

Photocopies

Subject to payment of the appropriate fee, IEEE will grant users a limited, non-exclusive license to photocopy portions of any individual standard for company or organizational internal use or individual, non-commercial use only. To arrange for payment of licensing fees, please contact Copyright Clearance Center, Customer Service, 222 Rosewood Drive, Danvers, MA 01923 USA; +1 978 750 8400. Permission to photocopy portions of any individual standard for educational classroom use can also be obtained through the Copyright Clearance Center.

Updating of IEEE Standards documents

Users of IEEE Standards documents should be aware that these documents may be superseded at any time by the issuance of new editions or may be amended from time to time through the issuance of amendments, corrigenda, or errata. A current IEEE document at any point in time consists of the current edition of the document together with any amendments, corrigenda, or errata then in effect.

Every IEEE standard is subjected to review at least every ten years. When a document is more than ten years old and has not undergone a revision process, it is reasonable to conclude that its contents, although still of some value, do not wholly reflect the present state of the art. Users are cautioned to check to determine that they have the latest edition of any IEEE standard.

In order to determine whether a given document is the current edition and whether it has been amended through the issuance of amendments, corrigenda, or errata, visit IEEE Xplore at http://ieeexplore.ieee.org/ or contact IEEE at the address listed previously. For more information about the IEEE SA or IEEE’s standards development process, visit the IEEE-SA Website at http://standards.ieee.org.

Errata

Errata, if any, for all IEEE standards can be accessed on the IEEE-SA Website at the following URL: http://standards.ieee.org/findstds/errata/index.html. Users are encouraged to check this URL for errata periodically.

Patents

Attention is called to the possibility that implementation of this standard may require use of subject matter covered by patent rights. By publication of this standard, no position is taken by the IEEE with respect to the existence or validity of any patent rights in connection therewith. If a patent holder or patent applicant has filed a statement of assurance via an Accepted Letter of Assurance, then the statement is listed on the IEEE-SA Website at http://standards.ieee.org/about/sasb/patcom/patents.html. Letters of Assurance may indicate whether the Submitter is willing or unwilling to grant licenses under patent rights without compensation or under reasonable rates, with reasonable terms and conditions that are demonstrably free of any unfair discrimination to applicants desiring to obtain such licenses.

Essential Patent Claims may exist for which a Letter of Assurance has not been received. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims, or determining whether any licensing terms or conditions provided in connection with submission of a Letter of Assurance, if any, or in any licensing agreements are reasonable or non-discriminatory. Users of this standard are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

Participants

At the time this drafttxtTrialUsetxtGorRPorSTD was submitted to the IEEE-SA Standards Board for approval, the varWorkingGroup Working Group had the following membership:

varWkGrpChair, Chair

varWkGrpViceChair, Vice Chair

Participant1

Participant2

Participant3

Participant4

Participant5

Participant6

Participant7

Participant8

Participant9

The following members of the <individual/entity> balloting committee voted on thistxtTrialUsetxtGorRPorSTD. Balloters may have voted for approval, disapproval, or abstention.

(to be supplied by IEEE)

Balloter1

Balloter2

Balloter3

Balloter4

Balloter5

Balloter6

Balloter7

Balloter8

Balloter9

When the IEEE-SA Standards Board approved thistxtTrialUsetxtGorRPorSTD on <XX MONTH 20XX>, it had the following membership:

(to be supplied by IEEE)

<Name>, Chair

<Name>, Vice Chair

<Name>, Past Chair

<Name>, Secretary

SBMember1

SBMember2

SBMember3

SBMember4

SBMember5

SBMember6

SBMember7

SBMember8

SBMember9

*Member Emeritus

Introduction

The 802.15.10 recommended practice defines a protocol that routes packets in a dynamically changing IEEE 802.15.4 network with minimal impact from route management. The result is an extension of the area of coverage as the number of nodes increases.  This amendment fully defines how the addressing and route information (already defined in 802.15.10) are to be used by the routing modes (also currently defined in 802.15.10), including at least the following:

- E2E acknowledgement from mesh route in non-storing mode

- P2P routing using a combination of up/down routing in non-storing mode

- On-demand P2P routing for E2E acknowledgement in non-storing mode

- On-demand path storing when sending unicast in non-storing mode

Contents

<After draft body is complete, select this text and click Insert Special->Add (Table of) Contents>

DrafttxtTrialUsetxtGorRPorSTD for varTitlePAR
Amendment A

NOTE—The editing instructions contained in this <amendment/corrigendum> define how to merge the material contained therein into the existing base standard and its amendments to form the comprehensive standard.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editing instructions, change markings, and this NOTE will not be carried over into future editions because the changes will be incorporated into the base standard.

- Overview

  - Scope

  - Purpose

- Normative references

The following referenced documents are indispensable for the application of this document (i.e., they must be understood and used, so each referenced document is cited in text and its relationship to this document is explained). For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments or corrigenda) applies.

- Definitions

For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictionary Online should be consulted for terms not defined in this clause.

- L2R functional description

  - Starting and joining an L2R mesh

    - Starting and stopping an L2R mesh

      - Procedure to start an L2R mesh

Change the third paragraph as indicated:

If On-demand P2P route discovery is TRUE, the P2P path list is initialized

as an empty table. The Route table is initialized as an empty table.

  - L2R mesh formation and route establishment

    - L2R mesh construction

Change the following entry in Table 5-1 as indicated:

| Name | Type | Valid range | Description | Retrieved from IE | Condition to record |
| --- | --- | --- | --- | --- | --- |
| P2P path list Route table | List of P2P paths Reachable destination information | List of the elements of a P2P path Route table entry as defined in Table 5-3 | List of P2P paths available through the current neighbor. Omitted if ondemand P2P route discovery or storing mode are disabled. Information needed to route a frame to each reachable destination | P2P-RP IE, P2P-RQ IE, RA IE, SRA IE | PJ |

Change the following entry in Table 5-2 as indicated:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| List of reachable destinations | List of address | Short or extended address as specified by Mesh address mode in the MT in Table 5-1 | List of devices that are reachable through the neighbor by DS routing (Only used in storing mode) |

Change the following entries in Table 5-3 as indicated:

Table 5-3—Elements of a P2P path Route table entry in storing mode

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination address | Address | Short or extended address as specified by Mesh address mode in the MT in Table 5-1 | Address of the P2P destination |
| PSN | Integer | 0x00-0xff | Sequence number of the latest P2P-RQ IE or P2P-RP IE from the destination. It is used only for on-demand P2P. |
| Next hop Route information | Address or address list | Short or extended address as specified by Mesh address mode in the MT in Table 5-1 | Address of the neighbor from which the P2P-RQ IE or P2P RP IE has been received to the destination or intermediate address list to the destination |
| P2P PQM value | Integer Depends on the metric ID | 0x00-0xff Depends on the metric ID, within the range 0x00-0xff | PQM between the current device and the destination. It is used only for on-demand P2P. |

Replace Figure 5-12 with the following figure:

Replace Figure 5-14 with the following figure:

Replace Figure 5-15 with the following figure:

Replace Figure 5-17 with the following figure:

    - L2R route metrics

Change paragraph 4 as indicated: (line 8 on p.48)

If the content of the Link Quality Metric ID and the Mesh Root Address fields   match the corresponding parameters in the MT of an L2R mesh the device has joined, the content of the Value field is recorded as the announced link metric of the neighbor in the global NT.

      - Expected airtime metric

Replace the equation for ExpectedAirtime with the following equation:  (line 6 on p.51)

Change table 5-6 as indicated:

| BO | Beacon order | Beacon order of the PAN |
| --- | --- | --- |
| SO | Superframe order | Superframe order of the PAN |
| SD | Superframe Duration | Duration of the active part of the Superframe |
| BI | Beacon Interval | Duration of time between successive beacons |

Change paragraph 2 as indicated: (line 7 on p.51)

BOSD and SOBI are variables described in Table 5-6. BOSD and SOBI are defined in IEEE Std 802.15.14 and are calculated from  BO and SO, which are found in the beacons transmitted in the PAN.  In case BO = 15, there is no beacon; set BI and SD = 1 in the above equation.

    - L2R routing capabilities

      - DS source route establishment

Change last paragraph in 5.2.4.2 as indicated (p.54):

When the mesh root receives the RA IE or the SRA IE, it delivers the path and the source address therein to the next higher layer with the L2LME-INTERMEDIATE-ADDR-LIST.indication primitive., and stores them as the pair of the address list and the destination address into the ‘Route Table’ in the MT (Table 5-1).

    - Multicast route establishment

Change the last sentence of 5.2.6 as indicated: (line 2 on p.57)

The intermediate devices B, C and G and the mesh root record the multicast address retrieved from the Multicast Subscription field of the RA IE into the list of reachable destinations multicast groups of the neighbor from which they received the RA IE.

    - P2P route establishment

      - Storing mode

Change the first paragraph of 5.2.7.1 as indicated:

When a device receives a P2P-RQ IE, if it has no existing path to the source of the P2P-RQ IE, it stores the information retrieved from the P2P-RQ IE into a new entry in the P2P path list Route Table in the MT. The address of the neighbor from which the P2P-RQ IE is received is recorded in the Next hop Route information of the P2P path Route Table. The device calculates or measures the LQM between itself and the neighbor from the previous hop and adds the result to the PQM value found in the IE. The device then stores the result into the PQM Value of the P2P path Route Table entry.

Change the 4th and 5th paragraph of 5.2.7.1 as indicated:

If the PSN in the latest P2P-RQ IE is equal to the recorded PSN, the device compares the PQM value of the two P2P-RQ IEs. If the PQM value of the latest P2P-RQ IE is the equal to or greater than that of the previous P2P-RQ IE, the latest P2P-RQ IE is discarded. Otherwise, the device replaces the value of Next hop Route information with the address of neighbor from which the latest P2P-RQ IE has been received, and replaces the PQM value with the PQM provided by the current neighbor. The IE is then discarded.

If the PSN of new P2P-RQ IE is newer than the recorded PSN, the device calculates the PQM between the device and the source of the P2P-RQ IE. All elements of the P2P path Route Table are updated with the information from the latest P2P-RQ IE and with the newly calculated PQM value.

Change the 8th to 11th paragraphs of 5.2.7.1 as indicated:

If the Request Intermediate Response field in the P2P-RQ IE is set to 1 and if an intermediate device has a path Route Table entry to the requested destination, it does not propagate the P2P-RQ IE but replies with a P2P-RP IE where the PSN and the PQM Value fields are set to the PSN and PQM value recorded for the path Route Table entry of interest in the P2P path list Route Table.

If the device is the desired destination, it replies with a P2P-RP IE. The TTL field of the P2P-RP IE is set to (l2rDefaultTTL - ttl), where ttl is the value of the TTL field in the received P2P-RQ IE. The PQM Value is set to 0. The PSN field is set to the device's current PSN. The P2P-RP IE is forwarded to the neighbor whose address is recorded in Next hop Route information in the current P2P path Route Table entry.

When a device receives a P2P-RP IE, it processes the information in the IE in the same way as it processes the information in a P2P-RQ IE as described above in 5.2.7.1. A P2P path Route Table entry is created or updated accordingly.

If the address of the device receiving the P2P-RP IE does not match the address found in the P2P SA field, the device forwards the P2P-RP IE through the Next hop Route information recorded for the corresponding P2P path Route Table entry with the Destination address matching the address in the P2P SA field.

Change the 13th paragraph of 5.2.7.1 as indicated:

The original source device may start routing data frames as soon as it receives a P2P-RP IE. When a device receives a new P2P-RP IE for the same destination, if the PQM value therein is lower than the PQM value provided by the current next hop, the P2P path Route Table is updated with the information of the new P2P-RP IE. Otherwise the P2P-RP IE is discarded.

      - Non-storing mode

Change the 2nd and 3rd paragraph of 5.2.7.2 (pg. 64) as indicated:

If the device is the desired destination, it delivers the path and the source address therein to the next higher layer with the L2RLME-INTERMEDIATE-ADDR-LIST.indication primitive. The device also creates a new entry in the P2P path list Route Table of the MT where the Destination address is set to the address retrieved from the P2P SA field of the P2P-RQ IE, the PSN is set to the value retrieved from the corresponding field in the IE, the PQM value is set to the sum of the value of the PQM field and the LQM on the latest hop. The Intermediate address list is set to the inverted list of addresses found in the Intermediate Address List field.  The device generates a P2P-RP IE and copies the Intermediate Address List field of the P2P-RQ IE into the corresponding field in the P2P-RP IE and sets the PSN to its own current PSN. The PQM Value is set to the value of the PQM recorded in the Route Table entry for the P2P path of interest. The P2P-RP IE is then carried forwarded to the source through the path indicated.

When the P2P-RP IE reaches the device with the address found in the P2P SA field, the device delivers the path and the source address therein to the next higher layer with the L2RLME-INTERMEDIATE-ADDRLIST.indication primitive. The device also creates a new entry in the P2P path list Route Table of the MT where the Destination address is set to the address retrieved from the P2P DA field of the P2P-RP IE, the PSN, the PQM value and the Intermediate Address List are set to the values retrieved from the corresponding fields in the IE.

  - L2R US route maintenance

    - L2R US route maintenance

Change the 6th paragraph of 5.3.1 as indicated: (p.65)

If a device misses does not receive consecutive l2rMaxMissedTcIe TC IEs, from a neighbor, the corresponding entry in the NT is erased. Route Table entries in the Mesh Table (MT) using that neighbor are also erased.

    - L2R DS route maintenance

Change the last paragraph of 5.3.2 as indicated: (p.66)

If the L2R mesh is in non-storing mode, route entries Route table entries (or list of reachable destinations in the NT) are not recorded by intermediate hops. The mesh root stores the entire path to the device identified by the SA found in the Intermediate Address List field in the Route Table. The route entries Route table entries are updated if needed when the mesh root receives a new L2R Routing IE, RA IE, SLR IE or SRA IE. The device may optionally delete a Route Table entry for the SA if no frame is received from the SA within a period of time determined by the implementer.

  - L2R routing capabilities

      - Source routing (p.67)

Insert the following two paragraphs at the top of the clause 5.4.1.1:

Source routing is used in DS routing and non on-demand P2P routing by the mesh root. It is also used by the source node in On-demand P2P routing; when non storing mode is used intermediate nodes are indicated.

If the frame is an incoming frame from the PHY layer and if the frame passes the filtering at the MAC layer, the frame is delivered to the L2R sublayer. If the DA matches the device’s address, the frame is delivered to the next higher layer. If the DA does not match the device’s address, the frame is forwarded again.

Change the original 1st paragraph of 5.4.1.1 as indicated:

If the L2R mesh is in non-storing mode, devices other than mesh root only store information about their neighbors but do not otherwise store DS routing information received with RA or SRA IEs. The mesh root stores all information received from the TC IEs and the RA IEs or SRA IEs. If on-demand P2P routing is enabled in non-storing mode, only the end to end source and destination devices of a P2P path store routing information. In either case, source routing is used. The path to the destination received from the IntermediateAddrList parameter of L2R-DATA.request primitive or retrieved from the Route table entry is included in the Intermediate Addresses List of the L2R Routing IE or the SLR IE and data frames are routed on the path indicated. An L2R mesh working in non-storing mode is indicated by Storing Mode set to FALSE in the MT.

Replace Figure 5-23 with following figure (pg.67):

      - Hop-by-hop routing

Change the 3rd paragraph of 5.4.1.2 as indicated:

If on-demand P2P routing is allowed in the L2R mesh, a source device may establish a route to the destination through the procedure described in 5.2.7. If the route is established, the next hop is selected according to the newly found path. If a path has already been established, i.e., there is a path to the desired destination information exists in the P2P path list Route Table, the transmitting device forwards the frame to the device with the address recorded in Next hop Route information in the corresponding P2P path Route Table entry. If there is no existing route recorded and no route is found within l2rP2pRouteDiscoveryTimeout, the frame is forwarded US until the first common ancestor between the source and the destination, and is then forwarded DS to the destination. This algorithm is illustrated in Figure 5-24.

  - L2R security aspects

    - Securing frames

Change the 2nd paragraph of 5.5.2 as indicated:

For a broadcast transmission, the L2R sublayer refers to l2rBCSecSecurityLevel, l2rBCSecKeyIdMode, l2rBCSecKeySource, and l2rBCSecKeyIndex found in Table 7-40.

- L2R extensions to MAC frame formats

  - L2R Long nested IE (LLIE)

    - L2R Routing IE

      - Descriptor field

Change the last paragraph of 6.2.2.1 as indicated:

If P2P is set to 1, the current data frame is a P2P data frame and the forwarding device searches the P2P path list Route Table in the MT to find a next hop. Otherwise, the current data frame is sent US or DS and the forwarding device searches among its neighbors or their list of reachable destinations to find a next hop.

- L2R services

  - L2R Long nested IE (LLIE)

    - L2R Routing IE

      - L2R-DATA.request

Change the following entry in Table 7-32 as indicated:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| IntermediateAddrList | List of addresses | List of short or extended addresses as specified by MeshAddrMode | Addresses to be inserted in the IntermediateAddressList of the L2R Routing IE. When IntermediateAddrList is ‘NULL’, the ‘Address List’ in the Route Table in the MT is used, where the destination address matches to the one in this primitive. Omitted if source routing is not used. |

# (informative)
Bibliography

Bibliographical references are resources that provide additional or helpful material but do not need to be understood or used to implement this standard. Reference to these resources is made for informational use only.

The Institute of Electrical and Electronics Engineers, Inc.

3 Park Avenue, New York, NY 10016-5997, USA

Copyright © 2017 by The Institute of Electrical and Electronics Engineers, Inc.

All rights reserved. Published <XX MONTH 20XX>. Printed in the United States of America.

IEEE is a registered trademark in the U.S. Patent & Trademark Office, owned by The Institute of Electrical and Electronics 
Engineers, Incorporated.

PDF:	ISBN 978-0-XXXX-XXXX-X	STDXXXXX

Print:	ISBN 978-0-XXXX-XXXX-X	STDPDXXXXX

IEEE prohibits discrimination, harassment and bullying. For more information, visit http://www.ieee.org/web/aboutus/whatis/policies/p9-26.html.

No part of this publication may be reproduced in any form, in an electronic retrieval system or otherwise, without the prior written permission of the publisher.

IEEE Standards Dictionary Online subscription is available at: http://dictionary.ieee.org.