IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft D0 of pothole detection from the rear LED shapes of the forwarding vehicle using optical camera communication (OCC). |  |
| Date Submitted | [September 2021] |  |
| Source | Md. Osman Ali and Yeong Min Jang | Voice: [ ] Fax: [ ] E-mail: [yjang@kookmin.ac.kr] |
| Re: | Pothole detection from the rear LED shapes of the forwarding vehicle using optical camera communication (OCC). |  |
| Abstract | If the forwarding vehicle (FWV) goes over a pothole, the alignment of the rear LEDs on the following vehicle’s (FV’s) image sensor will be changed. This unfavorable road condition can be detected using OCC. |  |
| Purpose | To assist in ensuring comfortable driving by providing pothole alerts to the drivers. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Introduction

To ensure a comfortable ride on vehicles, a pothole detection technique from the rear LED shapes of the forwarding vehicles (FWVs) is proposed in this document. A mathematical approach using OCC as well as a more accurate NN-based approach is explained in the following sections.

Figure 1. A general on-road scenario for OCC-integrated intelligent transportation system.

A general on-road scenario for OCC-integrated intelligent vehicles is shown in Figure 1. Four on-road scenarios are considered here, i.e., the vertical distance between FWVs and FLVs can increase or decrease, the FWVs can move laterally, the FWVs can turn either left or right, and any of the wheels of the FWV can run over a pothole. In the first two cases, both rear LED sizes remain equal, but in the last case, they become unequal. The size of the LEDs in the captured image decreases with an increase in distance, whereas the coordinates of the LEDs vary with the lateral movement of the FWVs, FLVs, or both. If one of the wheels runs over a pothole, the vertical coordinates of the rear LEDs of the FWV will no longer remain the same which is shown in Figure 2. From this scenario, we can measure the angle created between two LEDs with respect to the horizontal plane provided that the actual distance between two LEDs is known. Here, the actual distance between the LEDs is obtained using OCC. This system will work provided that the projected LED image on the image sensor (IS) covers a minimum of a one-unit pixel area.

Figure 2. Alignment of the rear LEDs when the FWV goes over a a) plain surface, b) pothole under the left wheel, and c) pothole under the right wheel.

# OCC system architecture

Two types of vehicular data, i.e., the vehicular ID and multimedia data, are encoded as the low-rate and high-rate data streams, respectively. They are simultaneously transmitted from the head or tail lights. The vehicular ID is used to distinguish between different vehicles, whereas the multimedia data are transmitted after establishing a connection with a specific vehicle. The actual distance between the two head or tail lights is transmitted from the FWV to FLV using OCC.

Figure 3. General block diagram for vehicular OCC system.

# Proposed Method

# 3.1 LED detection

We used a convolutional neural network (CNN), which is considered a powerful tool for image
and video processing, pattern and speech recognition, and natural language processing.
A CNN extracts features and learns automatically from the input images. It analyzes a small number of pixels instead of all the pixels for LED detection, thus reducing both
the computational time and complexity. It has several layers, namely, the convolutional
layer, pooling layer, and fully connected layer. The convolutional layer plays a key role
in producing proposed regions and feature maps, whereas the pooling layer’s purpose
is to downsample the images while keeping all the features. Repetition of these blocks
can be used to design deep NNs. The fully connected layer is generally the final layer
of the architecture. The output shows that the class of the input image that is shown in
Figure 4. Here, 1000 images of different road curvatures labeled with different angles were
used to train the model. We used 70% of the total images for the training with the help of
Darknet for our implementation using OpenCV in Python. To train the weight value, we
used yolov3.weights with yolov3_training.cfg and name.txt.

Figure 4. A CNN-based vehicular taillight detection scheme.

# 3.2 Dataset preparation and NN architecture

A CNN with two hidden layers was used to classify the images at different angular
conditions. At first, the NN was trained with 5000 images with their corresponding angles between the LEDs. The dimensions of the input and output vectors were 5000 × 6 and
5000 × 1, respectively, i.e., the dimension of the dataset was 5000 × 7. Numerous light
sources can exist in an image, including several backlights of on-road and off-road vehicles.
If all pixels were given as input to the NN, the time complexity of the network would
be enormously high. Therefore, the RoI was detected, and the interfering light sources
were removed. Next, each pair of vehicular taillights was extracted as individual warped
images. Then, a 4 × 4 convolutional filter was utilized to extract the features, whereas max
pooling with a 2 × 2 filter further reduced the size of the warped images, preserving all
features. This pooled feature map was flattened into one column, which was fed as the
input to the NN.

The road curvature could also be estimated trigonometrically. First, the coordinates of the LEDs were taken from the warped images, as mentioned above. Second, the distance per unit pixel was calculated with the prior knowledge of the distance between the rear LED pair obtained using OCC. Third, the apparent distance between the LEDs was estimated with the previously calculated distance per pixel. Finally, by analyzing the hypothetical triangle formed, as shown in Figure 5 (a) and 5 (b), the angle was calculated using the cosine function as follows

Bending angle,

(1)

Figure 5. Hypothetical triangle for calculation of pothole angle when the FWV goes over a pothole under the (a) left wheel and (b) right wheel.

Figure 6. Flowchart of pothole detection and measurement using OCC.

The flowchart of pothole detection and measurement using OCC is shown in Figure 6. The deep learning-based method is more accurate than the mathematical approach. We have compared both methods to provide a more accurate result. If there is any discrepancy between the mathematical method and the deep learning-based method, we can tolerate it up to a certain level and show the result of the deep learning-based approach as output. However, if the difference is more than the threshold (for instance 5 degrees), we can remove that calculation. For instance, we are using a 30 frame rate per second (fps) camera. Then, we will get 30 images in one second and each image will give one result. So, if we use a high frame rate camera, the number of results obtained in one second will be equal to the frame rate of the camera. So, if some of the results are discarded, it will not create any significant performance degradation. Our proposed method is expected to assist the drivers even in adverse weather conditions, such as rainy, snowy, and foggy. Therefore, pothole detection can play a key role in safe and comfortable driving.