IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | LB-PHY and related MAC to be reinserted |  |
| Date Submitted | 22/01/2022 |  |
| Source | Chong Han | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: |  |  |
| Abstract | Text parts that were removed by the CRG decision previously are modified. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

History:

R0 and R1: present the major sections to be reinserted after the last comment resolutions.

R2: change the high-reliability control header and figures

R3: update the figures 14, 17 and 19; update the Poll ACK field.

R4: update the text

R5: rewrite to accommodate the 802.15 style; update figures; add back the definitions of related elements.

5.7.3 LB-PHY introduction

The LB-PHY, specified in 11, is intended for low date rate applications with data rates in the tens of Mb/s using bit-interleaved coded modulation based on OFDM. It supports efficient utilization of the low-bandwidth resources (up to 32 MHz of single-sided bandwidth) of high-power LEDs as well as low-complexity, high energy efficiency and enhanced reliability.

A DC-biased OFDM is the default waveform. Furthermore, enhanced unipolar OFDM (eU-OFDM) is supported. For modulation of the LED, multiple clock rates are used. The LB-PHY supports MIMO and relaying.

The LB-PHY requires use of the polled channel access, defined in 6.4.

6.4 Polled channel access

    - 6.4.1 Overview

The polled channel access only works with the LB-PHY, which supports full duplex transmissions. The Polled channel access consists of two periods, namely Random Access Polling Period (RAPP) and Dedicated Polling Period (DPP). RAPP starts with the transmission of a frame carrying the Random Access element from the coordinator. During RAPP, devices compete by sending frames carrying the Poll Request element. In the DPP, based on the received Poll Request element the coordinator polls one or multiple devices within one frame transmission. In the DPP, devices are allowed to transmit once they recognize the beginning of the frame with the Poll element. An example of a full polled channel access cycle is given in Figure 13.

Figure 13 An example of one cycle of the polled channel access

The polling mechanism enables very low-complexity synchronization of the transmission slots and enables the coordinator to dynamically adapt and assign the slots for transmissions. The specific polling algorithm and order is outside the scope of this specification and is left as a system designer's choice. The concept is presented in the example in Figure 14.

Figure 14 Polled channel access

    - Random access polling period (RAPP)

At the beginning of a random access polling period, the coordinator shall send a Control frame carrying Random Access element. All stations that have already been associated with the coordinator shall ignore any polling during the RAPP. After receiving the control frame (RA), the device may send a frame carrying the Poll Request element. Then, the coordinator may start polling the device in the next polling round after successfully receiving the Poll Request element.

If a device is not polled during the next dedicated polling period, the device may assume the Poll Request it sent was not received by the coordinator. Then, the random backoff is introduced. The device will randomly select a number N between 1 and 5. The device shall retry to establish a connection after N cycles of RAPPs. Upon a successful polling request (the coordinator has successfully received the device’s request and has polled the station within the next transmission round), a device exchanges the necessary association and authentication information with the coordinator via association and authentication control frames.

    - Dedicated polling period (DPP)

During the dedicated polling period, each device is required to respond to a Poll element received from a coordinator.

When polled by the coordinator, a device replies with a frame carrying Poll Response element to the coordinator to announce its availability and append data frames if any are queued. The device shall transmit only one MPDU. The ACK for any data frame may "piggyback" on the transmission of any management or data frame. If a frame is not acknowledged, retransmission of the frame will be rescheduled by the coordinator. If a polled device does not have any data or ACK to transmit, it simply ignores the polling request. The coordinator will stop polling a device provided it does not receive any response from the device for more than aMacPollTimeout. The concept is given in the flow chart in Figure 16.

Figure 16 Flow chart for contention-free polling period

The coordinator will not start another polling request before the current polled device finishes its transmission. Hence, no collisions on the channel from devices to the coordinator are possible.

If a device has no ACK or information to transmit, it will simply ignore its possibility to transmit. A device wanting to keep link valid with coordinator, shall make sure that it sends a response to the Poll frame at least every aMacPollTime seconds. If there is no information to be transmitted to a device, which is next in the queue, a Data Null frame is used as a polling frame.

    - Poll duration field

The Poll duration field, depicted in Figure 32, contains the duration that a device needs to occupy the channel from devices to the coordinator. The information is specific to the contention-free polling mechanism. It is only present in frames originating from devices when sending the Poll Response frame. This is indicated through the Polled bit in the Frame Control field, defined in 7.2.2.

| Bit 0-14 | Bit 15 |
| --- | --- |
| Duration | reserved |

Poll duration field

Duration: Bits 0 to 14 identify the duration that a device needs to occupy the channel from devices to the coordinator. The information is offered in the MAC header of Poll Response frame, which calculates the total time required for the transmission of the Poll Response frame and any other piggybacked packets.

Reserved: Bit 15 is reserved.

7.6.22 Random Access element

The Random Access element in depicted in Figure 62.

Figure 62 Random Access element

Timestamp: The Timestamp field allows synchronization between the devices in an OWPAN. Each device shall maintain a timing synchronization function timer with modulus 264 16 counting in increments of microseconds. When coordinators prepare to transmit a Random Access frame, the coordinator timer is copied into the frame’s Timestamp field. Devices associated with a coordinator accept the timing value in any received Random Access frames, but they may add a small offset to the received timing value to account for local processing by the transceiver.

Random Access Interval: Each OWPAN may transmit Random Access frames at its own specific interval. The interval provides the duration between two adjacent random access periods, which lasts from the beginning of the current Random Access control frame to the beginning of the next Random Access control frame.

Capability Information: The 16-bit Capability Information field is used to advertise the network’s capabilities. In this field, each bit is used as a flag to advertise a particular function of the network.

OWPAN ID: OWPAN ID field gives the ID for the OWPAN.

LB-PHY MCS element: The LB-PHY MCS element, defined in 7.6.20, containing the supported clock rates and MCS.

7.6.23 Poll element

The Poll frame is a lower MAC frame which is sent by the coordinators to poll devices on downlink. The Poll frame does not have any field except the common management information.

All coordinators shall be capable of transmitting this command, although a coordinator is not required to be capable of receiving it. All devices shall be capable of receiving this command.

The poll frame shall be formatted as illustrated in Table 13.

Table 13 Poll element

| Octets: x | 4 |
| --- | --- |
| MHR | MFR |

7.6.24 Poll response element

The Poll response frame is a lower MAC frame which is sent by the devices to response to the Poll frame from coordinators. The Poll response frame does not have any field except the common management information. The Poll Request frame is depicted in Table 13.

All devices shall be capable of transmitting this command, although a device is not required to be capable of receiving it. All coordinators shall be capable of receiving this command.

7.6.25 Poll request element

The Poll request frame is a lower MAC frame which is sent by the devices to establish the connection with the coordinator via the uplink. The Poll request frame does not have any field except the common management information. The Poll Request frame is depicted in Table 13.

All devices shall be capable of transmitting this command, although a device is not required to be capable of receiving it. All coordinators shall be capable of receiving this command.

- LB-PHY specifications

- 1 General information

    - 11.1.1 Overview

The LB-PHY is intended for low date rate applications with data rates in the tens of Mb/s using OFDM modulation. OFDM specified by LB-PHY enables a highly adaptive modular implementation, which supports efficient utilization of the low-bandwidth resources (up to 32 MHz of single-sided bandwidth) as well as a low-complexity PHY designed to enable high energy efficiency and enhanced transmission reliability.

A DC-biased OFDM is the default waveform. Furthermore, the eU-OFDM waveform is supported. For modulation of the LED, multiple clock rates are used. The LB-PHY supports MIMO and relaying. Table 46 provides an overview over the LB-PHY parameters.

Summary of the LB-PHY

| Modulation |  |  | DC-biased OFDM, eU-OFDM |  |  |
| --- | --- | --- | --- | --- | --- |
| FEC |  |  | Convolutional coding |  |  |
| Code rates |  |  | 1/2, 2/3, 3/4 |  |  |
| Subcarrier spacing FSC |  |  | Clock rate / 32 |  |  |
| Cyclic prefix |  |  | 16 samples |  |  |
| MIMO |  |  | Up to 16 by 16 |  |  |
| Modulation | FEC | Clock rate |  | Data rate |  |
|  |  |  |  | Min. | Max. |
| BPSK | Inner convolutional code (1/2) | 1-32 MHz |  | 0.3 Mb/s | 9.6 Mb/s |
| BPSK | Inner convolutional code (3/4) |  |  | 0.45 Mb/s | 14.4 Mb/s |
| QPSK | Inner convolutional code (1/2) |  |  | 0.6 Mb/s | 19.2 Mb/s |
| QPSK | Inner convolutional code (3/4) |  |  | 0.9 Mb/s | 28.8 Mb/s |
| 16-QAM | Inner convolutional code (1/2) |  |  | 1.2 Mb/s | 38.4 Mb/s |
| 16-QAM | Inner convolutional code (3/4) |  |  | 1.8 Mb/s | 57.6 Mb/s |
| 64-QAM | Inner convolutional code (2/3) |  |  | 2.4 Mb/s | 76.8 Mb/s |
| 64-QAM | Inner convolutional code (3/4) |  |  | 2.7 Mb/s | 86.4 Mb/s |

Clock rates between 1 MHz and 32 MHz are defined, as listed in Table 48. The diagram of the DC-biased OFDM system is illustrated in Figure 75.

Diagram of the DC-biased OFDM system

    - 11.1.2 Base MCS

The base MCS for the LB-PHY shall be binary phase shift keying (BPSK) with code rate 1/2, i.e., MCS ID 000 as defined in Table 49.

    - 11.1.3 PHY constants

Table 47 lists values for the LB-PHY constants.

LB-PHY constants

| Name | Description | Value | Unit |
| --- | --- | --- | --- |
| aPhyMaxPsduSize | The maximum supported PSDU size. This attribute is PHY-specific. | 2036 | octets |
| aPhyMifsDuration | The duration of the MIFS for transmissions using the HB-PHY. | 3 | µs |
| aPhyMinPsduSize | The minimum supported PSDU size. This attribute is PHY-specific. | 20 | octets |
| aPhyTurnaroundTime | Not applicable. No turnaround is required, as the LB-PHY always performs full duplex transmission. | N/A | N/A |
| aPhyClockAccuracy | The minimum accuracy of the PHY reference clock. | +/- 20 | ppm |

    - 11.1.4 PHY PIB attributes

Table 48 lists the PHY PIB attributes for the LB-PHY.

PHY PIB attributes for the LB-PHY

| Name | Description | get/set | Range | Unit |
| --- | --- | --- | --- | --- |
| phyClockRate | The used clock rate | get / set | 1, 2, 4, 8, 16, 20, 25, 32 | Enumeration of MHz values |

  - PPDU format

    - 11.2.1 Overview

| Preamble | Channel Estimation | PHY header | Optional Fields | Payload |
| --- | --- | --- | --- | --- |

PPDU format for LB-PHY

The LB-PHY uses the PPDU format shown in Figure 76.

    - Bit order

The PSDU consists of an ordered sequence of octets. The octets of the PSDU shall be transmitted in the order they were received through the PD-SAP. Within each octet of the PSDU, the LSB of each octet shall be transmitted first.

Header fields that contain numbers shall be transmitted starting with the LSB first to the MSB last.

An exception to this rule is the high reliability control header in 11.2.6.4. This header is treated as a sequence of octets with the LSB of each octet transmitted first.

    - 11.2.3 Preamble

The PHY Preamble field is used for PPDU detection and synchronization. Preamble enables the identification of the existence of a transmission, as well as automatic gain control. It consists of pseudo noise training sequence, which lasts for the duration equivalent of two OFDM symbols. The sequence in preamble field is a time domain sequence and does not have any channel coding or line coding.

The following demonstrates the generation of pseudo noise training sequences.

Step 1: Select two 20-bit pseudo noise sequences:

Other pseudo noise sequences may be used as an option.

pn_seq0 = 20'b01001011000001110111

pn_seq1 = 20'b01101100111101010000

Step 2: Up-sample the above sequences by 8

Step 3: Pulse shape with the following pulse {-479, 416, -10, -409, 67, -409, -10, 416}

Step 4: Flip the sequence at the end of Step 3, e.g., (x1, ..., xn, xn, …, x1), in order to get two sequences for each original pseudo noise sequences (i.e., pn_seq0 and pn_seq1).

    - 11.2.4 Channel estimation

Channel Estimation field consists of two repetitions of a “Hermitian symmetric long training sequence” preceded by a cyclic prefix. The sequence in Channel Estimation field is a time domain sequence and does not have any channel coding or line coding.

A sequence of two identical OFDM training symbols is used to estimate the channel impulse response, as well as for additional fine-timing synchronization. The channel estimation sequence contains the following values modulated on the subcarriers of two identical inverse fast Fourier transforms (IFFTs) (index 0 corresponds to the DC subcarrier modulation value):

E-32…31=0,0,0,0,-1,1,-1,-1,1,1,-1,-1,-1,-1,-1,1,-1,1,1,1,-1,-1,-1,1,1,-1,1,-1,1,1,0,0,0,0,0,1,1,-1,1,-1,1,1,-1,-1,-1,1,1,1,-1,1,-1,-1,-1,-1,-1,1,1,-1,-1,1,-1,0,0,0

Timing parameters for the PHY control information (specification for 20 MHz)

The sequence is Hermitian symmetric in order to obtain a real time-domain signal after the inverse discrete Fourier transformation (IDFT). The sequence also has very good auto-correlation properties. In Figure 77, the channel estimation OFDM symbol is transmitted twice in two identical copies of the time-equivalent signal of the frequency-domain modulation sequence E-32…31. The cyclic prefix is a cyclic extension of this same time-domain signal and has a duration of 32 samples (twice the length of the typical cyclic extension for this PHY mode specification).

    - 11.2.5 PHY header

The PHY header contains all information necessary for demodulating the subsequent frame payload. It is encoded in 1/2 FEC rate (no puncturing is used BPSK modulation using DC-biased OFDM.

      - 11.2.5.1 Basic LB-PHY header

Basic PHY header is compulsory for all frames. The Advanced Modulation Header may be added after the basic PHY header, which is indicated by the basic PHY header.

The basic header contains the minimum information required for demodulating the subsequent payload. In LB-PHY, the header includes information such as the constellation size, the FEC rate and the payload size. The basic header contains 24 bits and fits within two OFDM symbols of the current PHY mode. The basic PHY header defines the fields given in Figure 78.

| Bit 0-2 | Bit 3 | Bit 4-14 | Bit 15 | Bit 16 | Bit 17 | Bit 18-23 |
| --- | --- | --- | --- | --- | --- | --- |
| MCS ID | Reserved | PSDU Length | Advanced Modulation Header | High Reliability Control | Parity | Tail |

Fields in the basic LB-PHY header

The individual fields of the basic header are described as follows.

MCS ID: This field consists of three bits and indicates the QAM constellation size and the FEC rate (achieved with the use of a convolutional encoder and puncturing) used for the subsequent payload. The values specified in Table 49 are valid for the MCS ID field. Data rates for different MCSs depend the clock rate. Table 49 lists example common selections of clock rates. The range of the available clock rates is [1, 32] MHz. The clock rate shall be obtained by dividing the available frequency by eight.

Valid MCS ID values

| MCS ID b0-b2 | Modulation | FEC rate | Data rate at 1 MHz clock rate | Data rate at 16 MHz clock rate | Data rate at 20 MHz clock rate | Data rate at 32 MHz clock rate |
| --- | --- | --- | --- | --- | --- | --- |
| 000 | BPSK | Inner convolutional code (1/2) | 0.3 Mb/s | 4.8 Mb/s | 6 Mb/s | 9.6 Mb/s |
| 001 |  | Inner convolutional code (3/4) | 0.45 Mb/s | 7.2 Mb/s | 9 Mb/s | 14.4 Mb/s |
| 010 | QPSK | Inner convolutional code (1/2) | 0.6 Mb/s | 9.6 Mb/s | 12 Mb/s | 19.2 Mb/s |
| 011 |  | Inner convolutional code (3/4) | 0.9 Mb/s | 14.4 Mb/s | 18 Mb/s | 28.8 Mb/s |
| 100 | 16-QAM | Inner convolutional code (1/2) | 1.2 Mb/s | 19.2 Mb/s | 24 Mb/s | 38.4 Mb/s |
| 101 |  | Inner convolutional code (3/4) | 1.8 Mb/s | 28.8 Mb/s | 36 Mb/s | 57.6 Mb/s |
| 110 | 64-QAM | Inner convolutional code (2/3) | 2.4 Mb/s | 38.4 Mb/s | 48 Mb/s | 76.8 Mb/s |
| 111 |  | Inner convolutional code (3/4) | 2.7 Mb/s | 43.2 Mb/s | 54 Mb/s | 86.4 Mb/s |

Reserved: This bit is reserved for introducing additional transmission rates in future modifications of the standard.

PSDU Length: This value scales from zero up to aPhyMaxPsduSize. The 11-bit field indicates the size of the payload in octets.

The LB-PHY supports only 32 bit aligned data in order to simplify the implementation. Bit 0 and 1 of this field have to be set to zero. If the high reliability control header, as defined in Figure 82, is included, the 32 bit aligned length of the PSDU plus eight is written to this field. If the high reliability control header is not included, only the 32 bit aligned length of the PSDU is written to this field. The length of the FCS, which is four octets, is included in the value of this field.

If the PSDU is an MPDU like in Figure 30, the length of this MPDU is written to this field. The value is incremented by eight depending on if the high reliability control header is present or not. The last four octets of the MPDU have to be cleared before writing them to the PHY in order to allow correct FCS calculation.

Advanced Modulation Header: This bit indicates whether an Advanced Modulation Header follows the basic PHY header. One indicates: Advanced Modulation Header follows the basic PHY header. Zero indicates: Advanced Modulation Header will not appear after the basic PHY header.

High Reliability Header: This bit indicates whether a high-reliability control header is added at the beginning of the payload. One indicates: The payload starts with the high-reliability control header. Zero indicates: The payload does not start with the high-reliability control header.

Parity bit does an even parity check for the information in bits 0 - 16.

Tail consists of six bits, which are set to zero to complete the basic PHY header.

      - 11.2.5.2 The Advanced Modulation Header

The Advanced Modulation Header is encoded in separate OFDM symbols from the basic PHY header. The advanced modulation header is also encoded using 1/2 FEC rate BPSK. The advanced modulation header is an optional field, which contains the information necessary for demodulating the subsequent waveform.

The advanced modulation header defines the fields given in Figure 79.

| Bit 0 | Bit 1 | Bit 2 | Bit 3 | Bit 4-9 | Bit 10 | Bit 11-12 | Bit 13 | Bit 14 | Bit 15 | Bit 16 | Bit 17 | Bit 18-23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| reserved | CQI | eU-OFDM | STR | reserved | Relaying Enabled | Relaying Mode | MIMO Enabled | MIMO Pilot Symbol | MIMO Mode | reserved | Parity | Tail |

Fields in the advanced modulation header

The individual fields of the basic header are described as follows.

CQI bit indicates whether the CQIs should be calculated in the PHY for the current transmission frame.

0b1 indicates that the CQIs should be estimated.

0b0 indicates that no CQIs should be estimated.

The channel estimation symbols preceding the PHY header are used for the estimation of the CQIs if the MIMO mode is not enabled. If MIMO mode is enabled, the MIMO pilot symbols used for CQI estimation is further defined in MIMO Pilot Symbol field in the advanced modulation header.

Upon estimation of the CQIs, the PHY conveys the results to the MAC using a predefined PHY service primitive. The calculation of the bit and power allocation scheme as well as the necessary exchange for updating the bit and power allocation scheme at the transmitter and receiver are handled at the MAC sublayer.

eU-OFDM indicates whether the payload field is encoded using eU-OFDM.

0b1 indicates that the payload is encoded using eU-OFDM.

0b0 indicates that the payload is not encoded using eU-OFDM.

The use of this waveform may be negotiated in advance using control/management frames.

STR bit indicates the number of eU-OFDM streams superimposed in the signal encoding procedure.

0b0 indicates: the number of eU-OFDM streams superimposed in the signal encoding procedure is one.

0b1 indicates: the number of eU-OFDM streams superimposed in the signal encoding procedure is four.

Reserved bits are reserved for future use.

Relaying Enabled bit indicates whether a relaying mode is enabled for the current PHY frame.

Relaying mode specifies the type of relaying mode that should be performed.

0b00 indicates that the relaying and duplexing mode is FD-AF.

0b01 indicates that the relaying and duplexing mode is FD-DF.

0b10 indicates that the relaying and duplexing mode is HD-AF.

0b11 indicates that the relaying and duplexing mode is HD-DF.

MIMO Enabled bit indicates whether a MIMO mode is enabled for the current PHY frame.

0b1 indicates that the MIMO mode is enabled and the subsequent payload is encoded using the MIMO scheme already negotiated between the transmitter and the receiver.

0b0 indicates that the MIMO mode is not enabled and the subsequent payload is encoded using the SISO scheme specified by the parameters in the basic PHY header and the advanced modulation header.

MIMO Pilot Symbols Format bit indicates the format of the pilot symbols used for CQI estimation.

0b0 indicates that the MIMO pilot symbols format I is used.

0b1 indicates that the MIMO pilot symbols format II is used.

The two MIMO pilot symbols formats are specified in 11.2.6.

MIMO Mode bit indicates which MIMO mode will be used.

0b1indicates that the MIMO mode is enabled and the subsequent payload is spatially multiplexed.

0b0 indicates that the MIMO mode is enabled for repetition coding and the subsequent payload is encoded using the SISO scheme specified by the parameters in the basic PHY header and the advanced modulation header.

Reserved bit is reserved for future use.

Parity bit does an even parity check for the information in bits 0 - 16.

Tail consists of six bits, which are set to zero to complete the advanced modulation header.

    - 11.2.6 Optional fields

      - 11.2.6.1 Overview

Optional fields contain pilot symbols for MIMO channel estimation. For MIMO pilot symbols, repetitions, FEC and line coding do not apply. The optional fields include NPS MIMO pilot symbols.

MIMO pilot symbols constitute NMIMO OFDM symbols (or the time-frame equivalent of NMIMO OFDM symbols), where NMIMO is the number of MIMO channels. The MIMO pilot symbol formats are described as follows.

      - 11.2.6.2 MIMO Pilot Symbols Format I

For each OFE, only one OFDM symbol interval is set to the desired channel estimation sequence. All other intervals are set to zero. Thus, the channel estimation sequence transmission intervals never coincide with any other transmitters. Hence, the MIMO pilot symbols for the different transmitters are orthogonal to each other. The format is presented in Figure 80.

MIMO pilot symbols format I

      - 11.2.6.3 MIMO Pilot Symbols Format II

For each OFE, every frame interval is set to the desired channel estimation sequence. In addition, the channel estimation sequences for each transmitter are modified by adjusting the polarity of the individual channel estimation sequence sequences according to a pre-determined set of Walsh sequences, where a value of one in the Walsh sequence corresponds to an unmodified channel estimation sequence while a value of minus one corresponds to a channel estimation sequence with reverse polarity. The format is presented in Figure 81. The channel estimation sequences in white are left unmodified, while the channel estimation sequences in gray are multiplied by -1.

MIMO pilot symbols format II

The Walsh sequences for a MIMO configuration with two transmitters (NMIMO = 2) correspond to the rows of the matrix W2 MIMO:

111-1

The Walsh sequences for a MIMO configuration with four transmitters (NMIMO = 4) correspond to the rows of the matrix W4 MIMO:

111-1111-1111-1-1-1-11

The Walsh sequences for a MIMO configuration with eight transmitters (NMIMO = 8) correspond to the rows of the matrix W8 MIMO:

111-1111-1111-1-1-1-11111-1111-1111-1-1-1-11111-1111-1111-1-1-1-11-1-1-11-1-1-11-1-1-11111-1

As a general rule, the Walsh sequences for a MIMO configuration with 2k transmitters (NMIMO=2k) correspond to the rows of the matrix W2kMIMO:

W2k-1MIMOW2k-1MIMOW2k-1MIMO-W2k-1MIMO

      - 11.2.6.4 High-reliability Control field

Robust transmission of the polling and acknowledgment information helps avoiding unnecessary retransmissions. Furthermore, when this information is encoded separately from the rest of the payload, errors in the payload (especially for long payloads) which cause the PPDU to be discarded (and retransmitted) do not affect the polling and acknowledgment mechanism. The MAC header is encoded using the lowest data-rate (most robust) modulation format 1/2 FEC rate BPSK separately from the data payload. Unlike the other headers, this header is treated as a sequence of octets which are transmitted LSB first.

The high-reliability control defines the subfields given in Figure 82.

| 6 Octets | 6 Octets | Bit 0 | Bit 1-8 | Bit 9-41 | Bit 42-47 |
| --- | --- | --- | --- | --- | --- |
| Polled Device Address | Next Device To Poll | Buffer Status Reporting | CRC | reserved | Tail |

information in high-reliability control field

The individual subfields of the high-reliability control header are described as follows.

Polled Device Address subfield specifies the address of the device, which is being polled by the coordinator in the downlink. On the channel from devices to the coordinator, these bits are left unspecified.

Next Device To Poll subfield specifies the address of the next device, which is being polled by the coordinator in the downlink. On the channel from devices to the coordinator, these bits are left unspecified.

Buffer Status Reporting subfield indicates that the current device has more data in the queue waiting for transmission. The bit is only used by devices. For coordinators, the bit is unspecified.

CRC consists of eight bits CRC for previous sub-fields.

Reserved subfield includes 19 bits reserved for future use.

Tail subfield consists of six bits of zeros to complete the high-reliability control.

    - Payload

      - General

Payload is transmitted at one of the supported data rates. Payload consists of service, PSDU Length, PSDU, tail, and pad fields.

The payload defines the fields given in Figure 83.

| Bit 0-15 | Bit 16-47 | Variable | Variable | variable |
| --- | --- | --- | --- | --- |
| Service | PSDU Length | PSDU | Tail | Pad |

Fields in the payload

The individual fields of the payload are described as follows.

Service: Service bits for scrambler initialization.

PSDU Length: Indicates the octet aligned length of the subsequent PSDU as opposed to the 32 bit aligned PSDU length in 11.2.5.1.

PSDU: Indicates the length of the PHY frame.

Tail: Six zero bits.

Pad: Pad bits are appended to payload field as to ensure the number of bits in the payload field to be a multiple of NCBPS.

      - Service subfield

The Service subfield has 16 bits, which shall be denoted as bits 0-15. The bit zero shall be transmitted first in time. The bits from zero to six of the Service subfield, which are transmitted first, are set to zeros and are used estimate the initial state of the transmitter scrambler and to synchronize the descrambler in the receiver. The remaining nine bits (7-15) of the Service subfield shall be reserved for future use. All reserved bits shall be set to zero. The bit allocation is demonstrated in Figure 84.

Service field bit allocation

      - 11.2.7.3 PSDU Length subfield

The field specifies the octet aligned length of the PSDU in the payload. For a simple hardware implementation it is preferable to have all data 32 bit aligned. However, MPDUs submitted to the PHY may have any length. Therefore, this field is added before the PSDU in order to recover the exact length at the receiver. The PSDU length aligned 32 field gives the 32 bit aligned PSDU length.

      - PSDU field

The PSDU field has a variable length and carries the data of the PHY frame. The last four octets of the PSDU are for the FCS in order to detect transmission errors in the PSDU

      - Tail field

The PPDU Tail field shall be six bits of zeros, which are required to complete the payload.

      - Padding Bits

The number of bits in the payload field shall be a multiple of NCBPS, the number of coded bits in an OFDM symbol (24, 48, 96, or 144 bits). To achieve this, the length of the message is extended so that it becomes a multiple of NDBPS, the number of data bits per OFDM symbol. At least 6 bits are appended to the message, in order to accommodate the Tail bits. The number of OFDM symbols, NSYM; the number of bits in the payload field, NPAYLOAD; and the number of pad bits, NPAD, are computed from the length of the PSDU (LENGTH) as follows:

NSYM=16+8×LENGTH+6NDBPS

NPAYLOAD=NSYM×NDBPS

NPAD=NPAYLOAD-16+8×LENGTH+6

The appended pad bits are set to zero and are subsequently scrambled with the rest of the bits in the payload.

      - PHY DATA scrambler and descrambler

The DATA field shall be scrambled with a length-127 PPDU-synchronous scrambler. The octets of the PSDU are placed in the serial bit stream for transmission, bit zero first and bit seven last. The PPDU synchronous scrambler uses the generator polynomial S(x) as follows and is illustrated in Figure 85.

SX=x7+x4+1

PHY DATA scrambler

The 127-bit sequence generated repeatedly by the scrambler shall be (leftmost used first), 00001110 11110010 11001001 00000010 00100110 00101110 10110110 00001100 11010100 11100111 10110100 00101010 11111010 01010001 10111000 1111111, when the all 1’s initial state is used. The same scrambler is used to scramble transmit data and to descramble receive data.

  - 11.3 Modulation and coding

    - 11.3.1 Interleaving

OFDM is invariably used in conjunction with channel coding (forward error correction), and usually applies frequency and/or time interleaving.

Frequency (subcarrier) interleaving increases resistance to frequency-selective channel conditions such as fading. For example, when a part of the channel bandwidth fades, frequency-interleaving causes the bit errors that would result from those subcarriers in the faded part of the bandwidth to spread out in the bit-stream rather than being concentrated. Similarly, time interleaving causes bits that are originally close together in the bit-stream to be transmitted far apart in time, thus mitigating against severe fading as would happen when travelling at high speed.

However, time interleaving is of little benefit in slowly fading channels, such as for stationary reception, and frequency interleaving offers little to no benefit for narrowband channels that suffer from flat fading (where the whole channel bandwidth fades at the same time).

The reason why interleaving is used on OFDM is to attempt to spread the errors out in the bit-stream that is presented to the error correction decoder, because when such decoders are presented with a high concentration of errors the decoder is unable to correct all the bit errors, and a burst of uncorrected errors occurs.

All encoded data bits shall be interleaved by a block interleaver with a block size corresponding to the number of bits in two OFDM symbols, 2NCBPS. The interleaver is defined by a two-step permutation. The first permutation ensures that adjacent coded bits are mapped onto nonadjacent subcarriers. The second ensures that adjacent coded bits are mapped alternately onto less and more significant bits of the constellation and, thereby, long runs of low reliability bits, i.e. LSBs, are avoided. The index of the coded bit before the first permutation shall be denoted by k; i shall be the index after the first and before the second permutation; and j shall be the index after the second permutation, just prior to modulation mapping.

The first permutation is defined by

i=2NCBPS16k mod 16+floork16, k=0,1,…,2NCBPS-1.

The function floor (.) denotes the largest integer not exceeding the parameter.

The second permutation is defined by the rule,

j=s×flooris+i+2NCBPS-floor16×i2NCBPSmod s, i=0,1,…,2NCBPS-1

The value of s is determined by the number of coded bits per subcarrier, NBPSC, according to,

s=max⁡(NBPSC2,1)

The de-interleaver, which performs the inverse relation, is also defined by two permutations.

Here the index of the original received bit before the first permutation shall be denoted by j; i shall be the index after the first and before the second permutation; and k shall be the index after the second permutation, just prior to delivering the coded bits to the convolutional (Viterbi) decoder. The first permutation is defined by,

i=s×floorjs+j+floor(16×j2NCBPS)mod s, j=0,1,…,2NCBPS-1.

The second permutation is defined by,

k=16×i-(2NCBPS-1)floor(16×j2NCBPS), i=0,1,…,2NCBPS-1.

    - 11.3.2 Forward error correction (FEC)

The FEC is inner convolutional coding. The PPDU shall be encoded with a convolutional encoder of coding rate R = 1/2, 2/3, or 3/4, corresponding to the desired data rate. The convolutional encoder shall use the industry-standard generator polynomials, g0 = 1338 and g1 = 1718, of rate R = 1/2, as shown in Figure 86. The bit denoted as "A" shall be output from the encoder before the bit denoted as "B". The summation operation presented in Figure 86 is a modulo-2 summation, i.e., an XOR operation. A subscript eight denotes octal values.

Higher rates shall be derived from this encoding mechanism by employing "puncturing." Puncturing is a procedure for omitting some of the encoded bits in the transmitter (thus reducing the number of transmitted bits and increasing the coding rate) and inserting a dummy zero metric into the convolutional decoder on the receive side in place of the omitted bits. The puncturing patterns are illustrated in Figure 87. Decoding by the Viterbi algorithm is recommended.

Convolution encoder (1338,1718)

Puncturing (bit stealing) algorithm

    - 11.3.3 OFDM modulator

      - 11.3.3.1 DC-biased OFDM modulator

The real time-domain OFDM signal, generated at the PHY, is used to modulate the light emitting device (an LED or a laser diode), which serves as the transmitter front-end. The modulation is conducted only within the active operational range of the device. In this range, the electrical signal and the light output signal must be positive at all times.

The approach for modulating the LED active range with an OFDM signal is to set a positive operating point, around which the bipolar OFDM signal can be realized. Figure 88 illustrates this principle. The positive bias can be introduced as part of the analog front-end (in the case of AC-coupled LED drivers) or as part of the information signal (in case of DC-coupled drivers). This approach is known as DC-biased OFDM.

DC-biased OFDM

The IDFT size is fixed in the LB-PHY mode to enable lower implementation complexity. The modulation of the different frequency-domain subcarriers is achieved through an IDFT operation, described as follows:

k=063S[k]enj2πk64

where S[k] is the symbol mapped to subcarrier index k. Conventionally, the IDFT is implemented with an IFFT algorithm.

The DFT/IDFT size in the current PHY mode is fixed to 64.

Subcarriers with negative indices -28 to -3 are loaded with 24 data symbols and two pilots. The pilots are located at index -21 and -7. Subcarriers with positive indices 3 to 28 are loaded with the conjugate complex of the data and pilot symbols at the negative indices. The pilot symbols have all value one.

Subcarriers with indices -2, -1, -1, 2 are set to zero in order to avoid possible low-frequency distortion in the system due to baseline wandering and background light interference.

Subcarriers -31, -30, -29, 29, 30, 31 are set to zero because those are near the band edge of the low pass filters in the system and may get attenuated excessively. Subcarriers with index 0, i.e. DC, and -32 are also set to zero. The mapping is illustrated in Figure 89.

IDFT realization by means of an IFFT algorithm

      - 11.3.3.2 Enhanced unipolar OFDM (eU-OFDM)

An optional alternative modulation approach is the eU-OFDM. It constitutes of a digital processing algorithm, which turns the bipolar OFDM signal into a strictly unipolar information signal without the addition of an energy intensive DC component that carries no additional information.

The transmitter signals to the receiver the new transmission PHY mode using the eU and STR bits in the advanced modulation PHY header. For conformance purposes, the preamble and the PHY headers are encoded in a DC-biased OFDM fashion. Following the four BPSK OFDM symbols containing the PHY header, as well as the NMIMO MIMO pilot symbols when applicable, the data field is encoded in a eU-OFDM fashion, as shown in Figure 90. The eU-OFDM algorithm works as follows.

Enhanced Unipolar OFDM (eU-OFDM)

      - 11.3.3.3 Data stream mapping with one stream for eU-OFDM

For a single eU-OFDM stream, specified by STR = '0', two consecutive copies of every OFDM symbol are generated. The polarity of the samples in the second copy is inverted, and finally, all negative samples in the resulting time-domain signal are set to zero. The resulting positive signal is used to modulate the transmitter. The concept is illustrated in Figure 91.

Unipolar OFDM generation (1 stream)

      - 11.3.3.4 Data stream mapping with two streams for eU-OFDM

For two eU-OFDM streams cases, every three OFDM symbols are grouped into an eU-OFDM block, where the first two symbols are assigned to data stream 1 (St1) and the remaining one symbol is assigned to data stream 2 (St2). The two symbols in St1 are modulated using the algorithm described for STR='0'. The symbol in St2 is modulated in a similar manner, but instead of two copies, four consecutive copies are created for the OFDM symbol in St2, where the first two copies are kept unchanged, while the polarity of the samples in the next two copies is inverted. Following this procedure, all negative samples in St1 and St2 are removed, and the two signals are summed. Any time domain oversampling and pulse shaping should be done after the removal of the negative samples. If done before the negative samples are removed, the oversampling and pulse shaping should also be performed at the receiver side during the signal re-modulation process required for the data recovery as explained in the RX algorithm. The resulting positive signal is used to modulate the transmitter. The concept is presented in Figure 92.

Unipolar OFDM generation (2 streams)

      - 11.3.3.5 Data stream mapping with three streams for eU-OFDM

For three eU-OFDM streams, every seven OFDM symbols are grouped into a eU-OFDM block, where the first four symbols are assigned to data stream 1 (St1), the next two symbols are assigned to data stream 2 (St2) and the last symbol is assigned to data stream 3 (St3). The four symbols in St1 and the two streams in St2 are modulated using the algorithm described for 2-stream case. The symbol in St3 is modulated in a similar manner. However, eight consecutive copies of that symbol are generated, where the first four copies are left unchanged, while the polarity of the samples in the following four copies is reversed. Following this procedure, all negative samples in St1, St2 and St3 are removed. Any time-domain oversampling and pulse shaping should be done after the removal of the negative samples. If done before the negative samples are removed, the oversampling and pulse shaping should also be performed at the receiver side during the signal re-modulation process required for the data recovery as explained in the RX algorithm. The signals in the three streams are summed and the resulting positive signal is used to modulate the transmitter. The concept is presented in Figure 93.

Unipolar OFDM generation (3 streams)

      - 11.3.3.6 Data stream mapping with four streams for eU-OFDM

For four eU-OFDM streams, indicated by STR = '1', every fifteen OFDM symbols are grouped into an eU-OFDM block, where the first eight symbols are assigned to data stream 1 (St1), the next four symbols are assigned to data stream 2 (St2), the following two symbols are assigned to data stream 3 (St3) and the last symbol is assigned to data stream 4 (St4).

In the first stream, two consecutive copies of every OFDM symbol are transmitted, where the second copy is multiplied by -1 (the signs of all samples are inverted in the time domain) as described in the cases with one, two, or three streams.

In the second stream, four consecutive copies of every OFDM symbol are transmitted, where the first two copies of the symbol are trans mitted in their original format, while the signs of the time-domain samples of the third and the fourth copy are inverted, i.e., the samples are multiplied by -1 as described in the cases for two streams and for three streams.

In the third stream, eight consecutive copies of every OFDM symbol are transmitted, where the first four copies are conveyed in their original format, while the signs of the time-domain samples of the fifth, sixth, seventh and eighth copy are inverted, i.e., the samples are multiplied by -1 as described in the case for two streams.

In the fourth stream, sixteen consecutive copies of every OFDM symbol are transmitted, where the first eight copies are conveyed in their original format, while the signs of the time-domain samples of the ninth, tenth, eleventh and twelfth, thirteenth, fourteenth, fifteenth and sixteenth copy are inverted, i.e., the samples are multiplied by −1.

At this point, all negative samples in the four streams are removed and the signals from the three streams are added together. Any oversampling and pulse shaping should be done after the removal of the negative samples. If done before the negative samples are removed, the oversampling and pulse shaping should also be performed at the receiver side during the signal re-modulation process required for the data recovery as explained in the RX algorithm. The resulting positive signal is used to modulate the transmitter.