| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of Synchronization Channel for Talk-around Direct Communication |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Seokki Kim, Sungcheol Channg, Miyoung Yun, Eunkyung Kim, Won-Ik Kim, Hyun Lee, Sungkyung Kim, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-0626 E-mail: kimsk0729@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments on the 802.16.1a Draft AWD, IEEE 802.16n-11/0033 |  |
| Abstract | This provides AWD text proposals for modification of synchronization channel for talk-around communication in IEEE 802.16.1a |  |
| Purpose | To be discussed and adopted by TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of Synchronization Channel for Talk-around Direct Communication

Seokki Kim, Sungcheol Chang, Miyoung Yun, Eunkyung Kim, Won-Ik Kim, Hyun Lee, Sungkyung Kim, Chulsik Yoon, Kwangjae Lim

ETRI

Introduction

This contribution proposes modification of synchronization channel for talk-around direct communication because there are some problems in synchronization channel structure. Figure 1 is the current synchronization channel structure.

Figure 1 Synchronization channel structure

• If a HR-MS receives a dedicated channel in the slot 1 after transmitting the synchronization channel, transition time gap is needed to retain Tx/Rx switching time.

• If a HR-MS transmits a dedicated channel in the slot 1 after receiving the synchronization channel, transition time gap is needed to retain Rx/Tx switching time.

Figure 2 Frame structure of talk-around direct communication

But there is no transition time gap in current synchronization channel structure. Figure 3 is our proposal to solve this problem.

Figure 3 Proposed synchronization channel structure

• SYNC-CH message part is followed by SYNC-CH preamble part.

• Transition time gap may be inserted in the end of the preamble if necessary.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Modify the following text in Section 6.12.2.3.2.1.2 in the 802.16.1a AWD]

6.12.2.3.2.1.2 Frame structure for CDMZ

…

The first subframe of the CDMZ logical frame is occupied by synchronization channel. All the HR-MSs receives the synchronization signal on the Sync-CH except HR-MSs transmitting the Sync-CH. The HR-MSs are synchronized to the received synchronization signal if the signal timing has priority to HR-MS’s synchronization timing itself. The details of timing priority is FFS. Some HR-MSs sends the synchronization signal on the Sync-CH at selected subframes. HR-MS selects its slots for sending synchronization timing in distributed way. The details of how to select is FFS. The synchronization channel is composed of two parts: synchronization channel preamblemessage part (P-SCH1) and synchronization messagepreamble part (P-SCH2). The synchronization channel preamble part is used for acquiring time and frequency synchronization, and synchronization sequencemessage part is used for transmitting SYNC-CH IE which includes frame structure information, hop count, transmitter ID et. al. The detailed design of synchronization channel is described in 6.12.2.3.2.2.

…

[Remedy2: Modify the following text in Section 6.12.2.3.2.2.1 in the 802.16.1a AWD]

6.12.2.3.2.2.1 Synchronization channel structure

Figure 918-Synchronization channel for direct communication

Figure 918 describes the synchronization channel structure for direct communication in the time domain. One synchronization channel occupies one subframe composed of six OFDM symbols. The first three OFDM symbols are used for Sync-CH preamblemessage transmission and the last three OFDM symbols includeare used for the Sync-CH messagepreamble transmission. In the frequency domain, 72 contiguous subcarriers are assigned to transmit the synchronization channel for direct communication. The Sync-CH preamble is used for preamble detection, timing offset estimation, frequency offset estimation and channel estimation. In the frequency domain, a preamble sequence with 36 binary codes is mapped to 36 subcarriers and remaining 36 subcarriers are not used. The time domain preamble sequence is obtained by taking IFFT of the frequency domain preamble sequence. In the time domain, sequence 0 is denoted as repetition of a basic pattern with NFFT/2 samples, where NFFT is the FFT size, and sequence 1 is composed of a basic pattern with NFFT/2 samples and the sign reversed version of the basic pattern. The firstfourth Sync-CH symbol is defined by the CP and the time domain preamble sequence. Secondfifth and thirdsixth Sync-CH symbols are defined by the repetition of the time domain preamble sequence without the CP. To limit the preamble length to three OFDM symbols, the time domain preamble sequence is repeated by

Where is the CP length.

If a HR-MS receives a dedicated channel in the slot 1 after transmitting the synchronization channel, transition time gap for switching from Tx to Rx shall be inserted by the HR-MS.

If a HR-MS transmits a dedicated channel in the slot 1 after receiving the synchronization channel, transition time gap for switching from Rx to Tx shall be inserted by the HR-MS.

[-------------------------------------------------End of Text Proposal---------------------------------------------------]

References

[1] IEEE 802.16n-11/0033, “WirelessMAN-Advanced Air Interface for Broadband Wireless Access System; Enhancements to Support Higher Reliability Operations”