| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Additional parameters for multimode HR-MS selection |  |
| Date Submitted | 2012-01-10 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Jaesun Cha, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI Seokjoo Shin Chosun University | E-mail: woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr |
| Re: | “IEEE 802.16n-11/0025,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | This contribution is a proposal related to multimode operation in IEEE 802.16.1a. |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Additional parameters for multimode HR-MS selection

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Jaesun Cha, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

Seokjoo Shin

Chosun University

Introduction

According to IEEE 802.16.1a AWD, a multimode HR-MS may operate as an HR-RS to provide connectivity for multiple out-of-coverage HR-MSs. For this purpose, a superorindate HR-BS may select one among multimode HR-MSs who can act HR-RS by considering their locations as well as their battery levels. (How to select a multimode HR-MS acting as HR-RS is an implementation issue. However, the required parameters for the implementation should be defined at this standard.) One of parameters to estimate the location of the multimode HR-MS is its previous serving BSID.

Upon a serving HR-BS being disabled, the multimode HR-MSs in the disrupted cell shall perform either network reentry or initial network entry to an available HR-BS. During the network reentry, the HR-MS reports its previous serving BSID to the HR-BS by sending an AAI-RNG-REQ message with Ranging Purpose Indication set to the code 0b1000. In case of initial network entry, however, the HR-MS cannot report its previous serving BSID to the HR-BS. This is because there is no consideration on the initial network entry issued by coverage loss in the current standards.

In this contribution, we propose to insert a coverage loss indicator and a previous serving BSID into the AAI-RNG-REQ message field in case Ranging Purpose Indication set to the code 0b0000. If a multimode HR-MS performs initial network entry due to the coverage loss, it shall send to the HR-BS an AAI-RNG-REQ message with its previous serving BSID, with setting coverage loss indicator as 0b1.

Proposed Texts

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Add the following text in new Section 6.2.3.1 in 802.16.1a AWD.]

6.2.3.1 AAI-RNG-REQ

[Change Table 27 in section 6.2.3.1 as indicated:]

Table 27—AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode 0b0011 = Idle mode location update 0b0100 = DCR mode extension 0b0101 = Emergency call setup (e.g., E911) 0b0110 = Location update for updating service flow management encodings of E-MBS flows 0b0111 = Location update for transition to DCR mode from idle mode 0b1000 = Reentry from DCR mode, coverage loss or detection of different ABS restart count. 0b1001 = Network reentry from a Legacy BS 0b1010 = Zone switch to MZONE from LZONE 0b1011 = Location update due to power down. 0b1100 = Interference mitigation request to a CSG Femto ABS when experiencing interference from the CSG Femto ABS 0b1101 = NS/EP call setup 0b1110 = HR multicast service location update 0b1111 = Network reentry for FBIS operation | - |
| If (Ranging Purpose Indication == 0b0000) { |  | // Initial network entry |  |
| If (S-SFH Network Configuration bit == 0b0 and AMSID privacy is enabled) { |  |  |  |
| AMSID* | 48 | The AMSID hash value. Refer to 16.2.5.3.1 |  |
| } else if (S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 | AMS’s real MAC address |  |
| } |  |  |  |
| MAC version | 8 | See 11.1.3 |  |
| Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from –15 dB (0x00) to 16dB (0x1F) with 1dB step The value is determined by AMS after successful initial ranging process |  |
| Coverage loss indicator | 1 | 0b0: Initial network entry 0b1: Initial network entry by coverage loss |  |
| If (Coverage loss indicator == 0b1) { |  |  |  |
| Serving BSID | 48 | The BSID of the HR-MS’s previous Serving HR-BS before incurring a coverage loss |  |
| } |  |  |  |
| … | … | … | … |
| }else if (Ranging Purpose Indication == 0b1101) { |  | //NS/EP call setup |  |
| AMS MAC address | 48 | AMS’s real MAC address |  |
| MAC version | 8 | see 11.1.3 |  |
| Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from -15dB (0x00) to 16dB(0x1F) with 1dB step. The value is determined by AMS after successful initial ranging process. |  |
| }else if (Ranging Purpose Indication == 0b1110) { |  | // HR multicast location update |  |
| action code | 3 | bit0: multicast service flow update bit1: location update due to multicast zone change bit2: multicast security key update |  |
| } else if (Ranging Purpose Indication == 0b1111) { |  | // Network reentry for FBIS operation |  |
| If (STID is not preassigned) { |  |  |  |
| Serving BSID | 48 | The BSID of the AMS's previous S-ABS before incurring a coverage loss, or the BSID of the S-ABS to which the AMS is currently connected (has completed the registration cycle and is in Connected State). |  |
| Previous STID | 12 | The STID which the AMS uses in the previous S-ABS. |  |
| } else { |  |  |  |
| STID | 12 | The Station ID pre-assigned by the T-ABS |  |
| } |  |  |  |
| If (CMAC indicator == 0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented if the AMS has a CMAC Tuple necessary to expedite security authentication |
| } |  |  |  |
| Primary Serving ABS flag | 1 | 0b0 : the AMS shall set its primary serving ABS as S-ABS (Degraded HR-BS) after network reentry 0b1 : the AMS shall set its primary serving ABS as T-ABS (Target HR-BS) after network reentry |  |
| Switched Access Mode | 1 | 0 : Switched Access with fixed Switched Access Windows 1 : Switched Access with variable Switched Access Windows |  |
| If(Switched Access Mode==0 ) { |  |  |  |
| Switched Access Window Size | 8 | The size of fixed Switched Access Window in unit of frame |  |
| } else if ( Switched Access Mode == 1) { |  |  |  |
| Maximum Switched Access Window Size | 8 | Maximum size of Switched Access Window in unit of frame |  |
| } |  |  |  |
| Switched Access Start Time | 8 | The 8 least significant bits of the absolute frame number at the T-ABS where the AMS starts to perform the Switched Access operation. |  |
| } //end of Ranging Purpose Indication |  |  |  |
| … | … | … | … |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]