| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Talk-around Direct Communication |  |
| Date Submitted | 2012-03-12 |  |
| Source(s) | Seokki Kim, Sungcheol Channg, Miyoung Yun, Eunkyung Kim, Won-Ik Kim, Hyun Lee, Sungkyung Kim, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-0626 E-mail: kimsk0729@etri.re.kr |
| Re: | “IEEE 802.16-12-0142,” in response to Letter Ballot #38 on P802.16.1a/D1 |  |
| Abstract | This provides AWD text proposals for clarification on talk-around direct communication in IEEE 802.16.1a |  |
| Purpose | To be discussed and adopted by TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Talk-around Direct Communication

Seokki Kim, Sungcheol Chang, Miyoung Yun, Eunkyung Kim, Won-Ik Kim, Hyun Lee, Sungkyung Kim, Chulsik Yoon, Kwangjae Lim

ETRI

Introduction

This contribution is for clarification on talk-around direct communication.

References

[1] IEEE P802.16.1a/D1, WirelessMAN-Advanced Air Interface for Broadband Access Systems – Draft Amendment: Higher Reliability Networks, Feb. 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Modify the following figures and texts in each Sections in the 802.16.1a AWD]

6.12.2.3.2.1.1 Resources for talk-around direct communication

Figure 244 — An example of talk-around direct communication resource allocation

6.12.2.3.2.1.2 Frame structure for CDMZ

Figure 245 — An example of common direct mode logical frame construction

Figure 246 — An example of CDMZ resource segmentation and construction of mRBs and m-tiles (3 UL subframes / 5 msec frame)

6.12.2.3.2.1.3 Frame structure for CDMZ-E and CSDMZ

Figure 247 — An example of CDMZ-E or CSDMZ resource segmentation and construction of mRBs and m-tiles (3 UL subframes / 5 msec frame)

6.12.2.3.2.1.4 Construction of dedicated subchannels for each TDC frame

Figure 248 —An example of mRB assignment for supplementarydedicated subchannels when Nsubframe_per_fame = 3, Nded-subchannel,1 = 4, and Nded-subchannel,2 = 5

6.12.2.3.2.2.1 Synchronization channel structure

Figure 249 —Synchronization channel for direct communication

6.12.2.3.2.2.3.1 Pilot structure for OFDM symbols transmitting SYNC-CH IE

Figure 250 — Pilot structure for OFDM symbols transmitting SYNC-CH IE

6.12.2.3.2.2.3.2 Resource mapping of SYNC-CH IEc

Figure 251 — Physical processing block diagram for the SYNC-CH IE

6.12.2.3.2.3.1 Pilot structure for mRBs transmitting Ded-CH packet

Figure 252 — Pilot structure for OFDM symbols transmitting SYNC-CH IEDed-CH packet

6.12.2.3.2.3.2 Ded-CH preamble transmission

Figure 253 — Logical channel structure for Ded-CH preamble

6.12.2.3.2.3.3

Figure 254 — Physical processing block diagram for the Ded-CH packet transmission

6.12.2.3.2.4 Supplementary channel

Figure 255 — Supplementary channel structure

6.12.2.3.2.4.1 Ranging channel

Figure 256 — Ranging channel structure

6.12.2.3.2.4.2 CQI channel

Figure 257 Figure 928 — CQI channel structure

Figure 258 — Mapping of information in the CQI channel

[-------------------------------------------------End of Text Proposal---------------------------------------------------]