| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for MM-STAT-REP message in multimode operation |  |
| Date Submitted | 2012-03-13 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI Seokjoo Shin Chosun University | E-mail: woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr |
| Re: | “IEEE 802.16-12-0142,” in response to Letter Ballot #37 on P802.16n/D1 |  |
| Abstract | This contribution is a proposal related to multimode operation in IEEE 802.16n/D1 to be consistent with P802.16.1a/D1 |  |
| Purpose | To discuss and adopt the proposed text in the IEEE 802.16n/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for MM-STAT-REP message in multimode operation

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

Seokjoo Shin

Chosun University

Introduction

IEEE 802.16n/D1 describes MM-STAT-REP (multimode status report) message to inform the multimode HR-MS’ status information such as the battery level to the superordinate HR-BS. In this contribution, we propose the additional fields and operations for the MM-STAT-REP message which is transmitted by multimode HR-MS.

Proposed Texts

Note:

The text in BLACK color: the existing text in the p802.16n/D1

The text in RED color: the removal of existing p802.16n/D1

The text in BLUE color: the new text added to the p802.16n/D1

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Insert the following text in Table 53 of section 6.3.2.3 of p802.16n/D1.]

- Table 53 — MAC management messages

| Type | Message Name | Message Description | Connection |
| --- | --- | --- | --- |
| … | … | … | … |
| TBD | MM-STAT-REP | Multimode Status Report message |  |
| … | … | … | … |

[Remedy2: Insert the following sub-section in section 6.3.2.3.98 of p802.16n/D1.]

6.3.2.3.98.xx MM-STAT-REP (multimode status report) message

Multimode HR-MS transmits MM-STAT-REP message for the purpose as follows:

- when the battery level or the maximum capacity of battery changes

- when the MS is plugged in a charger

- when the trigger condition is met

Table xxx – MM-STAT-REP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MM-STAT-REP message format () { | — | — |
| Management Message Type = [TBD] | 8 | — |
| MS Battery Status | 1 | 0b0: The HR-MS is plugged into a power source. 0b1: The HR-MS is not plugged into a power source. |
| MS Battery Level | 3 | 0b000: Battery level is > 75% and ≤ 100% 0b001: Battery level is > 50% and ≤ 75% 0b010: Battery level is > 25% and ≤ 50% 0b011: Battery level is > 5 % and ≤ 25% 0b100: Battery level is below 5% 0b101–0b111: Reserved |
| Max Capacity of MS Battery | 4 | 0b0000: Reserved 0b0001~0b1110: 0.1Wh~1.4Wh (round off to the nearest whole number) 0b1111: The maximum capacity of battery is more than 1.5Wh or the battery is charging. |
| } |  |  |

[Remedy3: Insert the following text in section 16.1.3.1 of p802.16n/D1.]

16.1.3.1 Proactive Operation

A superordinate HR-BS may select a target HR-MS among its subordinate HR-MSs which are capable of role changing to HR-BS, according to the measured signal power at HR-BS and/or subordinate HR-MS’ status information such as the battery level. The superordinate HR-BS may transmit MM-ADV message with trigger condition for which the subordinate HR-MSs capable of role changing to HR-BS shall report its status information. When the trigger condition is met, the subordinate HR-MS capable of role changing to HR-BS may report its status information to the superordinate HR-BS via MM-STAT-REP message as described in 6.3.2.3.98.xx.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]