| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Security Level of MAC Control Messages |  |
| Date Submitted | 2012-05-03 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Anseok Lee, Wooram Shin, Kwangjae Lim | E-mail: jscha@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot Recirculation on P802.16.1b/D3 |  |
| Abstract | This contribution proposes some modification on security level of some MAC control messages. |  |
| Purpose | For discussion in M2M TG and adoption into 16.1b draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Security Level of MAC Control Messages

Jaesun Cha, Soojung Jung, Eunkyung Kim, Anseok Lee, Wooram Shin, Kwangjae Lim

ETRI

Introduction

This contribution proposes some modification of security level of AAI-MTE-IND and AAI-MGMC messages.

Proposed Texts

----------------- Start of the text proposal --------------------------------------------------------------------------------------

[Modify Table 26 on page 9 as follows]

Table 26 – MAC Control messages

| No. | Functional areas | Message names | Message description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| 70 | RELAY | AAI-ARS-CONFIG-CMD | ARS configuration Command | N/A | Unicast |
| 71 | M2M | AAI-MTE-IND | M2M multicast transmission end indication | <delete> N/A: If Multicast SA is not established for the intended M2M device group (identified by MGID) Encrypted: If Multicast SA is established for the intended M2M device group </delete><insert> N/A: in broadcast null: in multicast when multicast SA is not established Encrypted/ICV: in multicast when multicast SA is established </insert> | Broadcast/Multicast (using MGID) |
| 72 | M2M | AAI-MGMC | M2M device group MAC Control | <delete> N/A: If it carries control information for multiple M2M device groups or if it carries control information for one M2M device group and Multicast SA is not established for that M2M device group. Encrypted: If it carries control information for one M2M device group and Multicast SA is established for the that M2M device group </delete><insert> N/A: in broadcast null: in multicast when multicast SA is not established Encrypted/ICV: in multicast when multicast SA is established </insert> | Broadcast/Multicast (using MGID) |
| 73-255 |  |  | Reserved |  |  |

----------------- End of the text proposal ---------------------------------------------------------------------------------------