| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of AAI-NBR-ADV Message Format |  |
| Date Submitted | 2012-05-03 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Anseok Lee, Wooram Shin, Kwangjae Lim | E-mail: jscha@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot Recirculation on P802.16.1b/D3 |  |
| Abstract | This contribution modifies AAI-NBR-ADV message format and proposes relevant ANS.1 code. |  |
| Purpose | For discussion in M2M TG and adoption into 16.1b draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of AAI-NBR-ADV Message Format

Jaesun Cha, Soojung Jung, Eunkyung Kim, Anseok Lee, Wooram Shin, Kwangjae Lim

ETRI

Introduction

New parameters defined in amendment standard shall be added into a legacy control message independently to keep backward compatibility. However, MGID mapping information is located inside the loop defined by reference standard. Two parentheses shall be added into AAI-NBR-ADV message to resolve this problem.

This contribution proposes some modification of AAI-NBR-ADV message format and relevant ASN.1 code.

Proposed Texts

----------------- Start of the text proposal --------------------------------------------------------------------------------------

[Remedy 1: Modify Table 39 on page 15 as follows]

Table 39 – AAI-NBR-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| } //end of for N-NBR-ABSs |  |  |  |
| For (j=0;j<N-NBR-ABSs; j++) } | 6 | N-NBR-ABSs denotes the number of neighboring ABSs [1..64] |  |
| For (n=0; n<N-M2M-GROUP-ZONE; n++) { | 2 | N-M2M-GROUP-ZONE denotes the number of M2M Group Zones that the neighbor ABS belongs to [1..4]. | Present when N-M2M-GROUP-ZONE > 1 |
| M2M-GROUP-ZONE-ID | 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| For (i=0; i<Num-M2M-Zones; i++) { | 2 | Num-M2M-Zones denotes the number of neighboring M2M Zones [1..4] |  |
| M2M-GROUP-ZONE-ID | 12 | Denotes the neighbor M2M-GROUP-ZONE-ID |  |
| For (m=0; m<Num-MGID-Mapping; m++) { | 12 | Num-MGID-Mapping is the number of mappings of current MGID and new MGID between the serving M2M Zone and the neighbor M2M Zone, where the current MGID is not the same as the new MGID [1..4096]. | Present when the mapping of M2M Service IDs to MGIDs is different in the neighboring M2M Zone |
| Current MGID to New MGID mapping | 24 | The 12 LSBs denote the MGID in current M2M Zone and the 12 MSBs |  |
| } |  |  |  |
| } |  |  |  |
| For (i=0; i<N-NBR-R1-BSs; i++) { |  |  |  |

[Remedy 2: Add the following texts on page, line 59 as follow]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Neighbor Advertisement

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-NBR-ADV ::= 		SEQUENCE {

changeCount 				INTEGER (0..7),

totalNumberOfCellTypes 		INTEGER (1..8),

cellType 				ENUMERATED {

macro,

micro,

macro-hotzone,

femto,

ttrRelay,

r1-lzone,

spare2,

spare1

},

totalNumberOfSegments 		INTEGER (1..16),

segmentIndex 				INTEGER (0..15),

startingABSIndex 			INTEGER (0..255),

nbrABSInfoList 			SEQUENCE (SIZE (1..maxNeighborABSs)) OF NeighborABSInfo,

nbrM2MGroupZoneInfoList	SEQUENCE	(SIZE (1.. maxNeighborABSs)) OF		M2MGROUPZONEID	OPTIONAL, -- M2M devices only

mgidMappingInfoList		SEQUENCE	(SIZE (1..4)) OF		MGIDMappingPerZone		OPTIONAL, -- M2M devices only

nbrR1BSInfoList 			SEQUENCE (SIZE (1..maxNeighborR1BSs)) OF NeighborR1BSInfo OPTIONAL,

-- For ABS type whose system info are not included in AAI_NBR-ADV

cellTypeInfo 				CellTypeInfo 			OPTIONAL,

-- Optional LDM parameters included when they are to be changed

...

}

CellTypeInfo ::= 		SEQUENCE {

rangeIDCell 		SEQUENCE (SIZE (1..maxPhyCarrierIndices)) OF RangeIDCell 	OPTIONAL

}

RangeIDCell ::= 		SEQUENCE {

phyCarrierIndex 		PhyCarrierIndex,

idCellStartEnd 			SEQUENCE {

startIDCell 			IDCell,

endIDCell 			IDCell

}

}

NeighborABSInfo ::= 		SEQUENCE {

bsID 				BSID,

macVersion 			MacProtocolVersion,

cpLength 			CPLength,

carrierInfoList 			SEQUENCE (SIZE (1..maxCarriers)) OF CarrierInfo,

nbrSpecificTrigger 		Triggers 			OPTIONAL

}

NbrM2MGZIDInfo ::=		SEQUENCE (SIZE (1..4)) OF	M2MGROUPZONEID -- M2M devices only

MGIDMappingPerZone::=		SEQUENCE {

m2mGroupZoneID			M2MGROUPZONEID,

mgidMappingInfo			SEQUENCE (SIZE (1..4096)) OF	MGIDMappingInfo

} -- M2M devices only

MGIDMappingInfo::=		SEQUENCE {

newMMGID				BIT STRING (SIZE (12)),

currentMGID			BIT STRING (SIZE (12))

}	 -- M2M devices only

CarrierInfo ::= 		SEQUENCE {

idCell 			IDCell,

phyCarrierIndex 	PhyCarrierIndex,

pgid 			PGID,

sfhChangeCount 	INTEGER (0..15),

sfhEncFmt 		CHOICE {

-- All parameters of SFHSubpacket shall be included

fullSubpkt 			SFHSubpacket,

-- Parameters of SFHSubpacket are partially included

deltaInfoCurrentCxr 		OptSFHSubpacket,

-- Parameters of SFHSubpacket are partially included

deltaInfoPrecedingCxr 		OptSFHSubpacket,

noSFHIncluded 		NULL

}

}

NeighborR1BSInfo ::= 		SEQUENCE {

bsID 				BSID,

r1PreambleIndex 		BIT STRING (SIZE (8)),

phyModeID 			INTEGER (0..65535),

channelBW 			ENUMERATED {

five-mhz,

seven-mhz,

eightPoint75-mhz,

ten-mhz

},

r1BSCenterFreq CenterFreq

}

----------------- End of the text proposal ---------------------------------------------------------------------------------------