| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on talk-around direct communication |  |
| Date Submitted | 2012-05-04 |  |
| Source(s) | Seokki Kim, Sungcheol Channg, Miyoung Yun, Won-Ik Kim, Hyun Lee, Eunkyung Kim, Sungkyung Kim, Chulsik Yoon ETRI | Voice: +82-42-860-0626 E-mail: kimsk0729@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #38a (IEEE P802.16.1a/D2) |  |
| Abstract | This contribution provides clarification on talk-around direct communication in IEEE P802.16.1a/D2 |  |
| Purpose | To be discussed and adopted by TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on talk-around direct communication

Seokki Kim, Sungcheol Chang, Miyoung Yun, Won-Ik Kim, Hyun Lee, Sungkyung Kim, Chulsik Yoon

ETRI

Introduction

This contribution provides clarification on talk-around direct communication in IEEE P802.16.1a/D2.

References

[1] IEEE P802.16.1a/D2, WirelessMAN-Advanced Air Interface for Broadband Access Systems – Draft Amendment: Higher Reliability Networks, Apr. 2012.

Proposed Text

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Modify the following message in each sections in the IEEE P802.16.1a/D2]

6.2.3.65.37 AAI-DC-RTS

…

Table 106kk – AAI-DC-RTS message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Source DCTID | 24 | Indicates a source HR-MS address |  |
| Destination DCTID or DCGID | 24 | Indicates a destination HR-MS (Group) address |  |
| Maximum Index of Burst Size | 8 | Indicates a maximum index of burst size that the sending HR-MS suggests to the receiving HR-MS to ecommend. The receivingsending HR-MS selects the maximum index of burst size that is less than in Table 303. |  |
| Maximum Number of HARQ Retransmission | 2 | Indicates maximum number of PHY burst retransmission for HARQ operation. 0: HARQ retransmission is disabled 1~3: Maximum number of HARQ retransmission is enabled |  |
| Destination Address Type | 1 | Indicates type of destination address. 0: DCTID 1: DCGID |  |
| Piggyback Message Indicator | 1 | Indicates whether a control message is piggybacked or not 0: no piggyback 1: MAC control message |  |
| Transmit Power | 6 | Unsigned integer from 1 to 64 in units of 1 dBm, where 0b000000=1 dBm and 0b111111=64 dBm. |  |
| Reserved | 6 |  |  |
| MAC Control Message | varia ble | MAC control messages in Table 1216 except AAI-DC-RTS and AAI-DC-CTS messages. | Present if Piggyback message indicator is set to 1 |

6.2.3.65.38 AAI-DC-CTS

…

Table 106ll – AAI-DC-CTS message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Source DCTID | 24 | Indicates a source HR-MS address |  |
| Destination DCTID or DCGID | 24 | Indicates a destination HR-MS (Group) address |  |
| Recommended Index of Burst Size | 8 | Indicates a recommended index of burst size for a dedicated channel. The receiving HR-MS selects the index of burst size that is smaller than maximum index of burst size in the AAI-DC-RTS message. |  |
| Maximum Number of HARQ Retransmission | 2 | Indicates maximum number of PHY burst retransmission for HARQ operation. 0: HARQ retransmission is disabled 1~3: Maximum number of HARQ retransmission |  |
| Piggyback Message Indicator | 1 | Indicates whether a control message is piggybacked or not 0: no piggyback 1: MAC control message |  |
| Reserved | 5 |  |  |
| MAC Control Message | varia ble | MAC control messages in Table 1216 except AAI-DC-RTS and AAI-DC-CTS messages. | Present if Piggyback message indicator is set to 1 |

6.2.3.65.39 AAI-DC-MCHG-CMD

…

Table 106mm – AAI-DC-MCHG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| MaximumNew index of burst size | 8 | Indicates a maximumnew index of burst size for a dedicated channel |  |

[Remedy2: Modify the following text in the section 6.12.2.3.2.4 in the IEEE P802.16.1a/D2]

6.12.2.3.2.4 Supplementary Channel

…

As shown in Figure 255 ~ Figure 258, Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel. A supplementary sub-channel is composed of two sub-blocks distributed in the frequency domain. Since all HR-MSs should listen the supplementary channels, to obtain Tx. And Rx. Switching time, no signal is transmitted in the last OFDM symbol of supplementary channel (6-th OFDM symbol). The sub-block of supplementary sub-channel is composed of 32 subcarriers-by- 5 OFDM symbols. The supplementary sub-channels corresponding to the dedicated sub-channels in slot 1 are located in slot 2, and the supplementary sub-channels corresponding to the dedicated sub-channels in slot 2 are located in slot 1. By assigning a dedicated sub-channel and the corresponding supplementary sub-channel in a cross way, the setup time of communication link and the retransmission latency can be minimized. For example, if an HR-MS transmits a packet by using the dedicated sub-channel 1 in slot 1, because the corresponding supplementary sub-channel is located in slot 2, the receiving HR-MS can transmit ACK/NACK signal by using the supplementary sub-channel in the same superframe, and the retransmission packet can be transmitted in the next superframe.

…

Figure 255 describes the proposed Sup-CH structure for TDC. Details of the Sub-CH structure are defined in 6.12.2.3.2.1. One Sup-CH is composed of four distributed mini-tiles, where a mini-tile has (2 subcarriers)Ⅹ(5 symbols) rectangular-shaped resource elements. A Sup-CH includes ranging channel, CQI channel, and feedback channel, which are transmitted in TDM (time division multiplexing) manner.

[-------------------------------------------------End of Text Proposal---------------------------------------------------]