| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 coding for MAC control messages of direct communication over IEEE 802.16.1a |  |
| Date Submitted | 2012-05-08 |  |
| Source(s) | Hyun Lee, Miyoung Yun, Seokki Kim, Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Chulsik Yoon, Sungcheol Chang ETRI | E-mail: hyunlee@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16-12-271,” in response to Letter Ballot Recirc #38a on P802.16.1a/D2 |  |
| Abstract | This provides AWD text proposals for ASN.1 coding of MAC control messages of direct communication in IEEE p802.16.1a |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 coding for MAC control messages of direct communication over IEEE 802.16.1a

Hyun Lee, Miyoung Yun, Seokki Kim, Won-Ik Kim, Eunkyung Kim, Sungkyung Kim,

Chulsik Yoon, Sungcheol Chang

ETRI

Introduction

This document provides ASN.1 coding of MAC control messages of direct communication for HR-Network.

References

[1] IEEE 802.16-12-0132-00, GRIDMAN System Requirement Document including SARM annex, January 2012.

[2] IEEE P802.16nTM/D2, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, April 2012.

[3] IEEE P802.16.1aTM/D2, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, April 2012.

[4] EEE P802.16Rev3/D4, IEEE Draft Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” February 2012.

[5] IEEE P802.16.1TM/D4, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, February 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

Annex A

...

A.2 MAC control message definitions (normative)

Change Annex A.2 as indicated:

WirelessMAN-Advanced-Air-Interface DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- MAC Control Messages

MAC-Control-Message ::= SEQUENCE {

message MAC-Control-Msg-Type,

...

}

[Remedy1: Add the following codes below Line# 61, Page#218 in IEEE P802.16.1a/D2.]

MAC-Control-Msg-Type ::= CHOICE {

...

-- MISC

aaiL2Xfer 			AAI-L2-XFER,

aaiMsgAck 			AAI-MSG-ACK,

aaiResCmd 			AAI-RES-CMD,

-- HR direct communication

aaiDcLestReq 			AAI-DC-LEST-REQ,

aaiDcLestRsp 			AAI-DC-LEST-RSP,

aaiDcMesRep 			AAI-DC-MES-REP,

aaiDcRchgReq 			AAI-DC-RCHG-REQ,

aaiDcRchgRsp 			AAI-DC-RCHG-RSP,

aaiDcTknReq 			AAI-DC-TKN-REQ,

aaiDcTknRsp 			AAI-DC-TKN-RSP,

aaiDcTknHo 			AAI-DC-TKN-HO,

aaiDcLestCmd 			AAI-DC-LEST-CMD,

aaiDcLrelDmd 			AAI-DC-LREL-CMD,

aaiDcDsaCmd 			AAI-DC-DSA-CMD,

aaiDcDscCmd 			AAI-DC-DSC-CMD,

aaiDcDsdCmd 			AAI-DC-DSD-CMD,

aaiDcMesCmd 			AAI-DC-MES-CMD,

aaiDcRchgCmd 			AAI-DC-RCHG-CMD,

aaiDcTknAdv 			AAI-DC-TKN-ADV,

aaiDcRts 			AAI-DC-RTS,

aaiDcCts 			AAI-DC-CTS,

aaiDcMchgCmd 			AAI-DC-MCHG-CMD,

aaiDcRelayAdv 			AAI-DC-RELAY-ADV,

aaiDcRelayReq 			AAI-DC-RELAY-REQ,

aaiDcRelayRsp 			AAI-DC-RELAY-RSP,

-- HR Multicast message

AaiHrMgInd			AAI-HR-MG-IND,

AaiHrMtInd			AAI-HR-MT-IND,

...

}

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Common type definitions

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

...

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- MAC control messages for HR-Networks

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

[Remedy2: Add the following codes below Line# 17, Page#228 in IEEE P802.16.1a/D2.]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link creation request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LC-REQ ::= SEQUENCE {

tWDCAddressForTransmitting 	BIT STRING (SIZE (12)),

tWDCAddressForReceiving 	BIT STRING (SIZE (12)),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link creation response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LC-RSP ::= SEQUENCE {

tWDCAddressForTransmitting 	BIT STRING (SIZE (12)),

confirmationCode 		BIT STRING (SIZE (1)),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link deletion request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LD-REQ ::= SEQUENCE {

tWDCAddressForTransmitting 	BIT STRING (SIZE (12)),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link deletion response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LD-RSP ::= SEQUENCE {

tWDCAddressForTransmitting 	BIT STRING (SIZE (12)),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link report request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LR-REQ ::= SEQUENCE {

tWDCAddressForTransmitting 	BIT STRING (SIZE (12)),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link report response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LR-RSP ::= SEQUENCE {

tWDCAddress 	BIT STRING (SIZE (12)),

linkState 	BIT STRING (SIZE (1)),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link establishment request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LEST-REQ ::= SEQUENCE {

linkChangeCount		INTEGER (0..15),

linkList 		SEQUENCE (SIZE (0..1)) OF SEQUENCE {

fID			FID,

trafficPriority		INTEGER (0..7),

cSSpecificationParameters INTEGER {

g729a (16),

amr (17)

} (0..255),

mACHeaderType		ENUMERATED {

agmh,

spmh

}

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link establishment response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LEST-RSP ::= SEQUENCE {

linkChangeCount		INTEGER (0..15),

confirmationCode	INTEGER {

accept (0),

rejectWithaRecommendedDCH (1),

reject (2)

} (0..15),

directModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3) 		OPTIONAL,

dCHNumber		INTEGER (0..63) 	OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication measurement report

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-MES-REP ::= SEQUENCE {

listOfDCHsForCINRReport  SEQUENCE (SIZE (0..36)) OF SEQUENCE {

directModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

dCHNumber		INTEGER (0..63),

dCHCINRMean		INTEGER (0..255)

} OPTIONAL,

listOfDCHsForNIReport  SEQUENCE (SIZE (0..36)) OF SEQUENCE {

directModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

dCHNumber		INTEGER (0..63),

noiseAndInterferenceLevelMean		INTEGER (0..255),

noiseAndInterferenceLevelVariance	INTEGER (0..15)

} OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication resource change request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-RCHG-REQ ::= SEQUENCE {

listOfDCHsForCINRReport  SEQUENCE (SIZE (0..36)) OF SEQUENCE {

oldDirectModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

oldDCHNumber		INTEGER (0..15),

newDirectModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

newDCHNumber		INTEGER (0..15)

} OPTIONAL,

listOfRecommendedDCHs  SEQUENCE (SIZE (0..36)) OF SEQUENCE {

directModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

dCHNumber		INTEGER (0..63),

noiseAndInterferenceLevelMean		INTEGER (0..255),

noiseAndInterferenceLevelVariance	INTEGER (0..15)

} OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication resource change response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-RCHG-RSP ::= SEQUENCE {

confirmationCode	INTEGER {

acceptWithNewDCHsMapping (0),

reject (1)

} (0..15),

listOfNewDCHs  SEQUENCE (SIZE (0..36)) OF SEQUENCE {

oldDirectModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

oldDCHNumber		INTEGER (0..15),

newDirectModeZoneType INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

newDCHNumber		INTEGER (0..15)

} OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication token request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-TKN-REQ ::= SEQUENCE {

directModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

dCHNumber		INTEGER (0..15),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication token response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-TKN-RSP ::= SEQUENCE {

confirmationCode	INTEGER {

accept (0),

reject (1)

} (0..15),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication token handover

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-TKN-HO ::= SEQUENCE {

dCTID 			BIT STRING (SIZE(24)),

dCFrameNumber		INTEGER (0..15),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link establishment command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LEST-CMD ::= SEQUENCE {

linkChangeCount		INTEGER (0..15),

linkList 		SEQUENCE (SIZE (0..1)) OF SEQUENCE {

fID			FID,

trafficPriority		INTEGER (0..7),

cSSpecificationParameters INTEGER {

g729a (16),

amr (17)

} (0..255),

mACHeaderType		ENUMERATED {

agmh,

spmh

}

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication link release command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-LREL-CMD ::= SEQUENCE {

linkReleaseCommandCode	INTEGER {

linkReleaseCommand (0)

} (0..255),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication dynamic service addition command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-DSA-CMD ::= SEQUENCE {

fIDChangeCount		INTEGER (0..15),

flowList 		SEQUENCE (SIZE (0..12)) OF SEQUENCE {

fID			FID,

trafficPriority		INTEGER (0..7),

cSSpecificationParameters INTEGER {

g729a (16),

amr (17)

} (0..255),

mACHeaderType		ENUMERATED {

agmh,

spmh

}

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication dynamic service change command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-DSC-CMD ::= SEQUENCE {

fIDChangeCount		INTEGER (0..15),

flowList 		SEQUENCE (SIZE (0..12)) OF SEQUENCE {

fID			FID,

trafficPriority		INTEGER (0..7),

cSSpecificationParameters INTEGER {

g729a (16),

amr (17)

} (0..255),

mACHeaderType		ENUMERATED {

agmh,

spmh

}

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication dynamic service delete command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-DSD-CMD ::= SEQUENCE {

fIDChangeCount		INTEGER (0..15),

flowList 		SEQUENCE (SIZE (0..12)) OF SEQUENCE {

fID		FID

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication measurement command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-MES-CMD ::= SEQUENCE {

reportMode	INTEGER {

eventTriggered (0),

onetime (1),

periodic (2)

} (0..3),

reportMetric	BIT STRING {

dCHCINRMean (0),

dCHNIMeanandVariance (1)

} (SIZE (2)),

reportPeriod	INTEGER (0..255),

reportTriggerAveragingParameter INTEGER {

one	(0),

half	(1),

quarter	(2),

one-8th	(3),

one-16th	 (4),

one-32th	 (5),

one-64th (6),

one-128th (7)

} (0..15),

reportTriggerCINRThreshold	INTEGER (0..255),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication resource change command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-RCHG-CMD ::= SEQUENCE {

listOfDCHsChanged  SEQUENCE (SIZE (0..36)) OF SEQUENCE {

oldDirectModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

oldDCHNumber		INTEGER (0..15),

newDirectModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

newDCHNumber		INTEGER (0..15)

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication token advertisement

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-TKN-ADV ::= SEQUENCE {

directModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

dCHNumber		INTEGER (0..15),

pTTTokenStatus	INTEGER {

available (0),

unabailable (1)

} (0..3),

destinationDCGID	BIT STRING(SIZE(24)),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication request to send

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-RTS ::= SEQUENCE {

sourceDCTID			BIT STRING(SIZE(24)),

destinationDCTIDorGCGID	BIT STRING(SIZE(24)),

maxIndexofBurstSize		INTEGER (0..255),

maxNumberofHARQRetx		INTEGER (0..3),

destinationAddressType		ENUMERATED {

dctid,

dcgid

},

piggybackMessageIndicator	ENUMERATED {

nopiggyback,

maccontrolmessage

},

transmitPower			INTEGER (0..63),

reserved				INTEGER (0..63),

macControlMessage		CHOICE {

aaiDcLestReq		AAI-DC-LEST-REQ,

aaiDcMesRep 		AAI-DC-MES-REP,

aaiDcRchgReq		AAI-DC-RCHG-REQ,

aaiDcLestCmd    	AAI-DC-LEST-CMD,

aaiDcRelayReq 		AAI-DC-RELAY-REQ

} OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication clear to send

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-CTS ::= SEQUENCE {

sourceDCTID			BIT STRING(SIZE(24)),

destinationDCTIDorGCGID	BIT STRING(SIZE(24)),

recommendedIndexofBurstSize	INTEGER (0..255),

maxNumberofHARQRetx		INTEGER (0..3),

destinationAddressType		ENUMERATED {

dctid,

dcgid

},

piggybackMessageIndicator	ENUMERATED {

nopiggyback,

maccontrolmessage

},

transmitPower			INTEGER (0..63),

reserved				INTEGER (0..63),

macControlMessage		CHOICE {

aaiDcLestReq		AAI-DC-LEST-RSP,

aaiDcRchgReq		AAI-DC-RCHG-RSP,

aaiDcRelayReq 		AAI-DC-RELAY-RSP

} OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication MCS change command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-MCHG-CMD ::= SEQUENCE {

maxIndexofBurstSize		INTEGER (0..255),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication relay advertisement

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-RELAY-ADV ::= SEQUENCE {

relayDCTID		BIT STRING(SIZE(24)),

relayStatus		ENUMERATED {

available,

relayingPackets

},

relayTrafficPriority	INTEGER (0..7),

listOfReservedDCHs  	SEQUENCE (SIZE (0..36)) OF SEQUENCE {

directModeZoneTypeForReservation	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3),

dCHNumber		INTEGER (0..63)

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication relay request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-RELAY-REQ ::= SEQUENCE {

linkChangeCount		INTEGER (0..15),

flowList 		SEQUENCE (SIZE (0..1)) OF SEQUENCE {

fID			FID,

trafficPriority		INTEGER (0..7),

cSSpecificationParameters INTEGER {

g729a (16),

amr (17)

} (0..255),

mACHeaderType		ENUMERATED {

agmh,

spmh

}

},

targetDCTIDorDCGID		BIT STRING(SIZE(24)),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Direct communication relay response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DC-RELAY-RSP ::= SEQUENCE {

linkChangeCount		INTEGER (0..15),

confirmationCode	INTEGER {

accept (0),

rejectWithaRecommendedDCH (1),

waitNextResponseMessage (2)

} (0..15),

directModeZoneType	INTEGER {

cdmz (0),

cdmze (1),

csdmz (2)

} (0..3) 		OPTIONAL,

dCHNumber		INTEGER (0..63) 	OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Multicast Messages

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

...............

[-------------------------------------------------End of Text Proposal---------------------------------------------------]