| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of MAC control messages on direct communication over IEEE 802.16.1a |  |
| Date Submitted | 2012-05-08 |  |
| Source(s) | Hyun Lee, Miyoung Yun, Seokki Kim, Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Chulsik Yoon, Sungcheol Chang ETRI | E-mail: hyunlee@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16-12-271,” in response to Letter Ballot Recirc #38a on P802.16.1a/D2 |  |
| Abstract | Changes are provided to accommodate Correction on MAC control messages on direct communication over IEEE 802.16.1a |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of MAC control messages on direct communication over IEEE 802.16.1a

Hyun Lee, Miyoung Yun, Seokki Kim, Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Chulsik Yoon, Sungcheol Chang

ETRI

Introduction

This document provides Clarification of MAC control messages on direct communication over IEEE 802.16.1a.

Changes are provided to accommodate Correction on the value/descripion of dirct communication MAC control messges as the following proposed texts.

References

[1] IEEE 802.16-12-0132-00, GRIDMAN System Requirement Document including SARM annex, January 2012.

[2] IEEE P802.16.1aTM/D2, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, April 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-----------------------------------------------Start of Text Proposal----------------------------------------------]

[Remedy #1: Adopt the following proposed modification from page 68, Table 106u]

- 6.2.3.65.21  AAI-DC-LEST-REQ

An HR-MS transmits a AAI-DC-LEST-REQ message to establish a one way peer-to-peer TDC link.

Table 106u – AAI-DC-LEST-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, Link Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) 0 : SPMH (Short-Packet MAC header) 1 : AGMH (Advanced Generic MAC Header) default value is 0. |  |
| } |  |  |  |
| Reserved |  |  |  |

[Remedy #2: Adopt the following proposed modification from page 69, Table 106w]

- 6.2.3.65.23  AAI-DC-MES-REP

An HR-MS transmits a AAI-DC-MES-REP message to report the measurement results if a report trigger condition is met.

Table 106w – AAI-DC-MES-REP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<N_DCH_CINR; i++) { |  | N_DCH_CINR is the number of dedicated channels which are used for receiving packets and selected for CINR report | Present if 0 < N_DCH_CINR |
| Direct Mode Zone Type | 2 | Direct mode zone type for measurement 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| DCH number | 6 | DCH number for measurement |  |
| DCH CINR mean | 8 | CINR mean of the dedicated channel |  |
| } |  |  |  |
| For (i=0; i<N_DCH_NI; i++) { |  | N_DCH_NI is the number of dedicated channels which are not used for receiving and packets and selected for NI report Range [0..36] | Present if 0 < N_DCH_NI |
| Direct Mode Zone Type | 2 | Direct mode zone type for measurement 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. |  |
| DCH Number | 6 | DCH number for measurement |  |
| Noise and Interference Level Mean | 8 | Noise and Interference mean. This is noise plus interference power level that is divided by the number of subcarriers in the frequency domain and averaged over the dedicated channel. -134dBm to -30dBm in units of 1dB. -134 dBm is encoded as 0x00, -30 dB is encoded as 0x69, 0x69 to 0xFF is reserved. |  |
| Noise and Interference Level Variance | 4 | Noise and Interference variance. 0 dB to 15 dB in units of 1dB. |  |
| } |  |  |  |

[Remedy #3: Adopt the following proposed modification from page 70, Table 106x]

- 6.2.3.65.24  AAI-DC-RCHG-REQ

An HR-MS transmits an AAI-DC-RCHG-REQ message to change radio resource for dedicated channel.

Table 106x – AAI-DC-RCHG-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<N_DCH; i++) { |  | N_DCH is the number of dedicated channels Range [0..36] |  |
| Old Direct Mode Zone Type | 2 | Direct mode zone type for old DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. |  |
| Old DCH Number | 4 | Indicates old DCH number. |  |
| New Direct Mode Zone Type | 2 | Direct mode zone type for new DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| New DCH Number | 4 | Indicates new DCH number. |  |
| } |  |  |  |
| For (i=0; i<N_DCH_NI; i++) { |  | N_DCH_NI is the number of recommended dedicated channels for candidates of new DCHs Range [0..36] | Present if 0 < N_DCH_NI |
| Direct Mode Zone Type | 2 | Direct mode zone type of a recommended DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| DCH Number | 6 | DCH number of a candidate |  |
| Noise and Interference Level Mean | 8 | Noise and Interference mean. This is noise plus interference power level that is divided by the number of subcarriers in the frequency domain and averaged over the dedicated channel. -134dBm to -30dBm in units of 1dB. -134 dBm is encoded as 0x00, -30 dB is encoded as 0x69, 0x69 to 0xFF is reserved. |  |
| Noise and Interference Level Variance | 4 | Noise and Interference variance. 0 dB to 15 dB in units of 1dB. |  |
| } |  |  |  |

[Remedy #4: Adopt the following proposed modification from page 71, Table 106y]

        - AAI-DC-RCHG-RSP

An HR-MS transmits a AAI-DC-RCHG-RSP message in response to a received AAI-DC-RCHG-REQ.

Table 106y – DC-RCHG-REQRSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure 0x0: accept with new mapping of DCHs 0x1: reject (continue to use current DCHs) 0x2 to 0xF: reserved | Shall always be present |
| For (i=0; i<N_DCH; i++) { |  | N_ DCH is the number of DCHs which are changed to new DCHs Range [0..36] | Present if Confirmation Code == 0x0 |
| Old Direct Mode Zone Type | 2 | Direct mode zone type for old DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| Old DCH Number | 4 | Indicates old DCH number. |  |
| New Direct Mode Zone Type | 2 | Direct mode zone type for new DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| New DCH Number | 4 | Indicates new DCH number. |  |
| } |  |  |  |

[Remedy #5: Adopt the following proposed modification from page 72, Table 106cc]

- 6.2.3.65.29  AAI-DC-LEST-CMD

An HR-MS transmits an AAI-DC-LEST-CMD message to establish a point-to-multipoint link.

Table 106cc – AAI-DC-LEST-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, Link Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) 0 : SPMH (Short-Packet MAC header) 1 : AGMH (Advanced Generic MAC Header) default value is 0. |  |
| } |  |  |  |

[Remedy #6: Adopt the following proposed modification from page 73, Table 106ee]

        - AAI-DC-DSA-CMD

An HR-MS transmits an AAI-DC-DSA-CMD message to create a new service flow on one-way point-to-point and point-to-multipoint TDC links.

Table 106ee – AAI-DC-DSA-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..12] |  |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) 0 : SPMH (Short-Packet MAC header) 1 : AGMH (Advanced Generic MAC Header) default value is 0. |  |
| } |  |  |  |
| Reserved |  |  |  |

[Remedy #7: Adopt the following proposed modification from page 74, Table 106ff]

        - AAI-DC-DSC-CMD

An HR-MS transmits an AAI-DC-DSC-CMD message to change the parameters of an existing service flow on one-way point-to-point and point-to-multipoint TDC links.

Table 106ff – AAI-DC-DSC-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Chg; i++) { |  | N_Flow_Chg is the number of flows on which the sender of this message sends MAC PDUs. Range [0..12] | Present if 0 < N_Flow_Chg |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) 0 : SPMH (Short-Packet MAC header) 1 : AGMH (Advanced Generic MAC Header) default value is 0. |  |
| } |  |  |  |

[Remedy #8: Adopt the following proposed modification from page 76, Table 106ii]

- 6.2.3.65.35 AAI-DC-RCHG-CMD

An HR-MS transmits an AAI-DC-RCHG-CMD message to change communication resource on one-way point-to-point and point-to-multipoint TDC links.

Table 106ii – AAI-DC-RCHG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<N_DCH; i++) { |  | N_ DCH is the number of DCHs which are changed to new DCHs Range [0..36] |  |
| Old Direct Mode Zone Type | 2 | Direct mode zone type for old DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| Old DCH Number | 4 | Indicates old DCH number. |  |
| New Direct Mode Zone Type | 2 | Direct mode zone type for new DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| New DCH Number | 4 | Indicates new DCH number. |  |
| } |  |  |  |

[Remedy #9: Adopt the following proposed modification from page 77, Table 106jj]

        - AAI-DC-TKV-ADV

An HR-MS transmits an AAI-DC-TKN-ADV message to advertise status of a token for half duplex communication on a point-to-multipoint TDC link.

Table 106jj – AAI-DC-TKN-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Direct Mode Zone Type | 2 | Direct mode zone type of DCH for advertising that the PTT token is available. 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| DCH Number | 4 | Indicates DCH number for the advertisement. |  |
| PTT Token Status | 2 | Zero indicates that the PTT token is available. Nonzero One indicates unavailable 0x0: PTT token is available. 0x1: PTT token is unavailable. 0x2~0x3: Reserved |  |
| Destination DCGID | 24 | Indicates a destination HR-MS (Group) address. |  |

[Remedy #10: Adopt the following proposed modification from page 77, Table 106kk]

- 6.2.3.65.37  AAI-DC-RTS

An HR-MS transmits an AAI-DC-RTS message to reserve a dedicated channel in a distributed way.

Table 106kk – AAI-DC-RTS message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Source DCTID | 24 | Indicates a source HR-MS address |  |
| Destination DCTID or DCGID | 24 | Indicates a destination HR-MS (Group) address. |  |
| Maximum Index of Burst Size | 8 | Indicates a maximum index of burst size that the sending HR-MS suggests the receiving HR-MS to recommend. The receiving HR-MS selects burst size that is less than |  |
| Maximum Number of HARQ Retransmission | 2 | Indicates maximum number of PHY burst retransmission for HARQ operation. 0: HARQ retransmission is disabled 1~3: HARQ retransmission is enabled |  |
| Destination Address Type | 1 | Indicates type of destination address. 0: DCTID 1: DCGID |  |
| Piggyback Message Indicator | 1 | Indicates whether a control message is piggybacked or not 0: no piggyback 1: MAC control message |  |
| Reserved | 4 |  |  |
| MAC Control Message | variable | MAC control messages in Table 1216 except AAI-DC-RTS and AAI-DC-CTS messages is AAI-DC-LEST-REQ, AAI-DC-MES-REP, AAI-DC-RCHG-REQ, AAI-DC-LEST-CMD or AAI-DC-RELAY-REQ. | Present if Piggyback message indicator is set to 1 |

[Remedy #11: Adopt the following proposed modification from page 78, Table 106ll]

- 6.2.3.65.38  AAI-DC-CTS

An HR-MS transmits an AAI-DC-CTS message in response to AAI-DC-RTS message.

Table 106ll – AAI-DC-CTS message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Source DCTID | 24 | Indicates a source HR-MS address |  |
| Destination DCTID or DCGID | 24 | Indicates a destination HR-MS (Group) address. |  |
| Recommended Index of Burst Size | 8 | Indicates a recommended index of burst size for a dedicated channel |  |
| Maximum Number of HARQ Retransmission | 2 | Indicates maximum number of PHY burst retransmission for HARQ operation |  |
| Piggyback Message Indicator | 1 | Indicates whether a control message is piggybacked or not 0: no piggyback 1: MAC control message |  |
| Reserved | 5 |  |  |
| MAC Control Message | variable | MAC control messages in Table 1216 except AAI-DC-RTS and AAI-DC-CTS messages is AAI-DC-LEST-RSP, AAI-DC-RCHG-RSP, or AAI-DC-RELAY-RSP. | Present if Piggyback message indicator is set to 1 |

[Remedy #12: Adopt the following proposed modification from page 79, Table 106nn]

- 6.2.3.65.40  AAI-DC-RELAY-ADV

A relaying HR-MS transmits an AAI-DC-RELAY-ADV message to broadcast its information.

Table 106nn – AAI-DC-RELAY-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Relay DCTID | 24 | DC Terminal Identifier. Indicate the HR-MS which relays packets between two TDC links |  |
| Relay Status | 1 | Indicate that the relaying HR-MS is available for two-hop operation. 0x0: available 0x1: relaying packets |  |
| Relay Traffic Priority | 3 | Indicate the traffic priority for relaying packets 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| For (i=0; i<N_Reserved_DCH; i++) { |  | N_Reserved_DCH is the number of dedicated channels for reservation Range [0..36] |  |
| Direct Mode Zone Type for Reservation | 2 | Direct mode zone type of a reserved DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. |  |
| DCH Number for Reservation | 6 | Reserved DCH number on the zone of Direct Mode Zone Type |  |
| } |  |  |  |

[Remedy #13: Adopt the following proposed modification from page 80, Table 106pp]

- 6.2.3.65.42  AAI-DC-RELAY-RSP

An HR-MS transmits a AAI- DC-RELAY-RSP message in response to a received AAI- DC-RELAY-REQ message.

Table 106pp – AAI-DC-RELAY-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | Link Change Count from the corresponding AAI-DCRELAY-REQ | Shall always be present |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure. 0x0: accept 0x1: reject with a recommended DCH 0x2: wait next response message 0x3 to 0xF: reserved | Shall always be present |
| Direct Mode Zone Type | 2 | Direct mode zone type of a recommended DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. | Present if Confirmation Code == 0x1 |
| DCH Number | 6 | Recommended DCH number on zone of Direct Mode Zone Type | Present if Confirmation Code == 0x1 |

[----------------------------------------------End of Text Proposal------------------------------------------------]