| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 coding for BS mode switching messages of multimode HR-MS over IEEE 802.16.1a |  |
| Date Submitted | 2012-05-07 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Seokki Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Sungcheol Chang ETRI Seokjoo Shin Chosun University | E-mail: woniks@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr |
| Re: | “IEEE 802.16-12-271,” in response to Letter Ballot Recirc #38a on P802.16.1a/D2 |  |
| Abstract | This provides AWD text proposals for ASN.1 coding of AAI-MM-BS-REQ, AAI-MM-BS-RSP and AAI-MM-BS-CMD for multimode operation in IEEE p802.16.1a/D2 |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 coding for BS mode switching messages of multimode HR-MS over IEEE 802.16.1a

Won-Ik Kim, Eunkyung Kim, Seokki Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Sungcheol Chang

ETRI

Seokjoo Shin

Chosun University

Introduction

This document provides ASN.1 coding of AAI-MM-BS-REQ, AAI-MM-BS-RSP and AAI-MM-BS-CMD for HR-Network.

References

[1] IEEE P802.16nTM/D2, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, April 2012.

[2] IEEE P802.16.1aTM/D2, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, April 2012.

[3] EEE P802.16Rev3/D4, IEEE Draft Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” February 2012.

[4] IEEE P802.16.1TM/D4, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, February 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

Annex A

...

A.2 MAC control message definitions (normative)

Change Annex A.2 as indicated:

WirelessMAN-Advanced-Air-Interface DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- MAC Control Messages

MAC-Control-Message ::= SEQUENCE {

message MAC-Control-Msg-Type,

...

}

[Remedy1: Add the following codes below Line# 61, Page#218 in IEEE P802.16.1a/D2.]

MAC-Control-Msg-Type ::= CHOICE {

...

-- HR Multimode operation

aaiMmBsReq			AAI-MM-BS-REQ,

aaiMmBsRsp			AAI-MM-BS-RSP,

aaiMmBsCmd			AAI-MM-BS-CMD,

-- HR Multicast message

AaiHrMgInd			AAI-HR-MG-IND,

AaiHrMtInd			AAI-HR-MT-IND,

...

}

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- MAC control messages for HR-Networks

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

...

[Remedy2: Add the following codes below Line# 18, Page#228 in IEEE P802.16.1a/D2.]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode BS Request message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

SuperordinateBsParameterInfo ::= 	SEQUENCE {

sSFHChangeCount			INTEGER (0..15),

globalCarrierConfigChangeCount	INTEGER (0..7),

scdChangeCount				INTEGER (0..15),

mcChangeCount				INTEGER (0..15)

}

RecommendedParameterInfo ::= 		SEQUENCE {

fAIndex					FAIndex,

bsID					BSID,

macProtocolVersion			MacProtocolVersion,

cpLength				CPLength,

mmCarrierInfoList			SEQUENCE (SIZE (1..maxCarriers)) OF MmCarrierInfo

}

MmCarrierInfo ::= 			SEQUENCE {

saPreambleIndex			PreambleIndex,

phyCarrierIndex			PhyCarrierIndex,

pgid					PGID,

sfhChangeCount				INTEGER (0..15),

sfhEncFmt				CHOICE {

-- All parameters of SFHSubpacket shall be included

fullSubpkt				SFHSubpacket,

-- Parameters of SFHSubpacket are partially included

deltaInfoCurrentCxr			OptSFHSubpacket,

-- Parameters of SFHSubpacket are partially included

deltaInfoPrecedingCxr			OptSFHSubpacket,

noSFHIncluded				NULL

}

}

AAI-MM-BS-REQ ::=			SEQUENCE {

actionCode				ENUMERATED {

requestBsRole,

responseForUnsolicitRequest,

rejectForUnsolicitRequest,

spare1

}

bsConfInd				CHOICE {

noRecommendedParameter			NULL,

superordinateBsParameterInfo		SuperordinateBsParameterInfo,

recommendedParameterInfo		RecommendedParameterInfo

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode BS Response message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

BsModeOperation ::=			ENUMERATED {

retransmitBsReqMsg,

retransmitBsReqMsgAfterSysCfgChangeCountUpdate,

cancelBsConfig,

spare1

}

AAI-MM-BS-RSP ::=			SEQUENCE {

responseCode				CHOICE {

unsolicitedReqByHrMs			NULL,

requestApproval			BsModeOperation,

requestRejection			NULL

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode BS Commend message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-MM-BS-CMD ::=			SEQUENCE {

modeTriggerType			CHOICE {

startBsModeAfterActionTime		NULL,

startBsModeUponReachingCondition	TriggerConditions

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Multicast Messages

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

...