| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on multimode operation over IEEE 802.16.1a |  |
| Date Submitted | 2012-05-04 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Seokki Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Sungcheol Chang ETRI Seokjoo Shin Chosun University | E-mail: woniks@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr |
| Re: | “IEEE 802.16-12-271,” in response to Letter Ballot Recirc #38a on P802.16.1a/D2 |  |
| Abstract | This provides AWD text proposals for clarification on multimode operation over IEEE 802.16.1a |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications on multimode operation over IEEE 802.16.1a

Won-Ik Kim, Eunkyung Kim, Seokki Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Sungcheol Chang

ETRI

Seokjoo Shin

Chosun University

Introduction

This document provides AWD text proposals for clarification on multimode operation in IEEE p802.16.1a.

Remedy1: The dual-role operation of HR-MS acting as RS is required in STR mode as well as in TTR mode. For this, we have corrected some ambiguous sentences and typos in Section 6.12.1.2.5.

Remedy2: For selecting a target HR-MS among subordinate HR-MSs which are capable of role changing to HR-BS, the superordinate HR-BS may refer to HR-MS' status information such as the measured signal power and/or the battery level. However, it could be done through MAC control messages and/or MAC signaling headers described in IEEE P802.16.1TM/D6. Therefore, it is not necessary to send additional control messages such as MM-STAT-REP.

Remedy3: We have corrected a simple typo in IEEE p802.16.1a/D2.

References

[1] IEEE P802.16nTM/D2, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, April 2012.

[2] IEEE P802.16.1aTM/D2, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, April 2012.

[3] EEE P802.16Rev3/D6, IEEE Draft Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” April 2012.

[4] IEEE P802.16.1TM/D6, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, April 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Adopt the following change in Section 6.12.1.2.5 in IEEE P802.16.1a/D2.]

[page# 109, line# 36]

6.12.1.2 Relay function for HR-MS

…

6.12.1.2.5 Dual-role operation of HR-MS

An HR-MS acting as RS may maintain MS functionalities in company with RS functionalities. When an HR-MS in TTR relay mode connected state receives AAI-ARS-CONFIG-CMD message with MS functionality maintenance indication that is the value 0b1, it performs dual-role MS/RS operation. At the time instance specified by “Superframe Number Action” in the AAI-ARS-CONFIG-CMD message, the following procedures shall be performed for dual-role operation of the HR-MS acting as RS.

- The dual-role HR-MS starts TTR relay mode and establishes relay link with the superordinate HR-BS.

- The HR-MS establishes relay link with the superordinate HR-BS and starts the RS mode.

- The MS mode in the dual-role HR-MS is behaving as a subordinate station of the RS mode. For connecting the MS’s service flow(s), the MS mode shall communicate with the RS mode through internal interfaces in the dual-role HR-MS. The details of MS’s reentry process with the RS through the internal interface is are out of scope of this standard.

- The RS mode in the dual-role HR-MS requests to the superordinate HR-BS for switching the data path of the MS mode by sending the AAI-L2-XFER message, as if the MS’ handover process had been performed.

[Remedy2: Adopt the following change in Section 6.12.1.3.1 in IEEE P802.16.1a/D2.]

[page# 110, line# 22]

6.12.1.3 Base station function for HR-MS

…

6.12.1.3.1 Proactive Operation

A superordinate HR-BS may select a target HR-MS among its subordinate HR-MSs which are capable of role changing to HR-BS, according to the measured signal power at HR-BS and/or subordinate HR-MS' status information such as the battery level. The superordinate HR-BS may transmit AAI-MM-ADV message with trigger condition for which the subordinate HR-MSs capable of role changing to HR-BS shall report its status information. When the trigger condition is met, the subordinate HR-MS capable of role changing to HR-BS may report its status information to the superordinate HR-BS via MM-STAT-REP message and/or AMS Battery Level Report header as described in 6.2.2.1.3.5.

…

[Remedy3: Correct the typos or errors in multimode operation in IEEE p802.16.1a/D2.]

[page# 51, line# 5]

6.2.3.65.1 AAMAAI-MM-ADV message